3GPP TSG RAN meeting #82										RP-182510

Sorrento, Italy, December 10-13, 2018

# Source:	CMCC

# Title:	Status Report for SI on remote interference management for NR

# Document for:	Approval

Agenda Item:	9.3.9

Status Report to TSG

Agenda item:			9.3.9

| WI / SI Name | Study on remote interference management for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_RIM |  |  |  |  |
| Unique ID | 800093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181832 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2018 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 100 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur | Name | Dan WU |
|  | Company | CMCC |
|  | Email | wudan@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #94bis

In RAN1 #94 bis, the remaining evaluation assumptions for RIM RS design are aligned for comparing the performance of RIM RS candidates and for comparing evaluation cases. The basic structure of RIM RS, time and frequency domain pattern of the RIM RS are also discussed. Mechanisms to improve network robustness are also listed, whether they have corresponding spec impact are left for further discussion. The detailed agreements are listed as follows.

R1-1811091	Update for TR 38.866 Study on remote interference management for NR	CMCC

Agreements:

- Change the agreement from RAN1#94 related to detection window simulation assumption to: Length of detection window WdetLsymbol: to be provided

- Change the agreement from RAN1#94 related to RS delay model to: Delay of received RS: When one or multiple RSs arrive in the detection window, tThe arrival time of the i-th RS respect to the start of the detection window, △i , is uniformly distributed within [-Lsymbol, Lsymbol], where Lsymbol is the length of UL symbol based on the numerology of RS.

- Clarify that the power of the received RS in case of single RS (Case 1) is set to the reference power P0 and hence is not varying over time.

Agreements:

- For fair comparison of evaluation results, more evaluations assumptions need to be aligned.

  - Performance metrics are evaluated at reference SNR, the reference SNR is defined as follows:

    - SNRref(dB)=P0(dBm)-N(dBm)

  - where P0 is the reference receiver power and N is the noise power both within the length of 1 OFDM symbol.

  - False alarm is defined based on detecting any sequences transmitted in the same time-frequency resource in the network with only AWGN input, i.e. only thermal noise is input to the receiver, and should be limited under [1]% over 2 symbols

    - For simulation, the detector (window length and sliding granularity) should be consistent when calculating false alarm and detection probability

    - For different detectors, the false alarm rate will be scaled proportionally over the detection duration

      - For symbol-level detector, the detection duration is the length of the detection window

        - E.g. For 1 OS symbol-level detection window, the false alarm rate is [0.5%]

      - For sample-level detector, the detection duration is the number of symbols that the detection window is sliding over plus detection window length.

        - E.g. For 1 OS sample-level detection window sliding from symbol 0 to 1, the false alarm rate is [1%]

Agreements:

  - Clarify that the detection probability is defined as the probability of detecting a sequence in a detection window given that the sequence is present in the detection window, i.e,

Pd_k = Prob{sequence k is detected in a detection window | sequence k is present in the detection window}.

  - For symbol-level detection, only sequences present in the detection window are counted for detection probability.  “Sequence k is present in the detection window” means that the power of at least one RS copy using sequence k captured in the detection window is no less than that captured in other detection windows

  - For sample-level sliding detection, all sequences arrived should be counted.

  - Note that symbol-level detection requires less complexity at the expense of lower detection probability

  - For case 2-1, the metric is the minimum SNR required for one-shot detection with 90% detection probability under 1% false alarm requirement.

  - For case 2-2A and 2-2B, the metric is the minimum SNR required for one-shot detection with [90]% detection probability under [1]% false alarm and [1]% error detection requirement.

  - Detection algorithm should be declared, at least including

    - Symbol-level or sample-level sliding detection window, etc

    - how decision variable is calculated (e.g. PAPR or max peak, etc)

Agreements:

- Add Case 2-2B as simulation case, and provide results under various number of copies per sequence to reflect the impact of number of gNBs that share the same set ID on detection probability.

  - Case 2-2B (Multiple sequences and multiple RSs): The number of distinctive sequences received within the window is smaller than the number of RS copies. Multiple RSs copies may use the same sequence. Number of total RS sequences is more than 1.

- Modify the Table 7-1 in TR

Table 7-1 Simulation cases for RIM RS design

|  | Total number of sequences used in the same time-frequency resource in the network (Nseq) |  | Number of sequences arriving within the window (n) | Number of RS copies using the same sequence (m) | Number of total RSs arriving within the window (S) |
| --- | --- | --- | --- | --- | --- |
| Case 1 | 1 |  | 1 | 1 | 1 |
| Case 2-1 | 1 |  | 1 | 10 as starting point , other values are encouraged to be provided | m*n |
| Case 2-2A | 8 as starting point |  | 1,2,4,8 1 | 1 | m*n |
| Case 2-2B | 8 as starting point |  | 1,2,4,8 1 | 10 as a starting point, other values are encouraged to be provided | m*n |
|  |  | NOTE 1: Separate simulation runs |  |  |  |

Agreements:

- Add “Error detection probability”, which is defined as the detected sequence IDs do not match with the sequence IDs actually arrived within the detection window, as the other metric for RIM RS evaluation.

    - The metric is counted as follows: Pe=max_n{1,2,4}Perr,nPe=maxn={1,2,4}Perr,n , where Perr,n Perr,nis the probability of detecting at least one sequence different from all the one(s) that actually arrived within the detection window, and n is the number of sequences arriving within the window given in Table 7.1 in the TR.

Agreements:

- The pseudo-random sequence (length-31 Gold sequence) specified in NR is adopted as the RIM RS sequence

Agreements:

- Time-domain circular characteristics should be satisfied for NR-RIM design. The following alternatives are used for further evaluation.

  - Alt 1: 1 symbol RS using existing CSI-RS with comb-like structure in frequency-domain;

    - Comb factor = 2 and 4;

  - Alt 2: 2 symbol RS, where two copies of the RS sequence are concatenated and one CP is attached at the beginning the concatenated sequences;

  - Alt 3: 2 symbol RS, where the CP is separately added to the front of each OFDM symbol, but in frequency domain, the RIM-RS in different OFDM symbols need to be multiplied with different linear phase rotation factors.

    - Note that Alt 2 and Alt 3 may be identical in terms of performance. It is claimed that Alt 3 can use the same FFT as PDSCH generation. Under proper CP design, Alt 2 can also use the same FFT as PDSCH generation.

Agreements:

- Capture the following updated structure in TR 38.866 to include time-domain, frequency-domain, spatial domain and power-domain solutions.

6.1.1	Solutions by network implementation

6.1.1.1 Time-domain based solutions

6.1.1.2 Frequency-domain based solutions

6.1.1.3 Spatial-domain based solutions

6.1.1.4 Power-domain based solutions

6.1.2	Solutions with specification impact

6.1.2.1 Time-domain based solutions

6.1.2.2 Frequency-domain based solutions

6.1.2.3 Spatial-domain based solutions

6.1.2.4 Power-domain based solutions

Agreements:

- Time domain RIM mitigation include the following:

  - Time-domain Aggressor-side RIM mitigation solutions at least include: DL symbols backoff, i.e., muting DL symbol(s) that cause interference to the Victim.

    - Note that this sacrifices downlink throughput of the aggressor gNB

    - FFS details

  - Time-domain Victim-side RIM mitigation solutions at least include Victim gNB avoids scheduling on UL symbol(s) that are interfered

    - Note that this sacrifices uplink throughput of the victim gNB

    - FFS details

  - Note: frequency domain migitation schemes are separate

Agreements:

- Frequency domain RIM mitigation solutions for study at least include the following. Discuss further on whether they are network implementation solutions or have potential spec impact

  - Partial muting in frequency domain at either aggressor gNB or victim gNB

  - Utilizing different frequency band between aggressor gNBs and victim gNBs by scheduling or activating different BWPs or sub-bands with no overlapped bandwidth between them.

- Note that if the victim UL and the aggressor DL use non-overlapped bandwidths all the time (as in a static manner), the spectral efficiency and UL/DL capacity will be reduced

Agreements:

- Spatial domain RIM mitigation solutions for study at least include the following. Discuss further on whether they are network implementation solutions or have potential spec impact

  - Receive beam nulling at victim gNB, to suppress the remote interference in spatial domain.

  - Scheduling UE transmission that will be received in spatial directions that are less interfered at Victim gNB

  - Controlling transmit beam (e.g., down-tilting) at aggressor gNB

  - Use different beam directions on different DL positions (e.g. choose the beam direction which experiences minimal interference, then according to reciprocity, use this beam to perform transmission in DL resources adjacent to GP)

  - Mounting antennas at lower height, electrical/mechanical down-tilt.

- Note that adjusting the down-tilting or height of the antenna at Aggressor or Victim gNB may reduce corresponding cell coverage.

Agreements:

- Power control mechanism for RIM mitigation for study at least include the following.  Discuss further on whether they are network implementation solutions or have potential spec impact

  - Increase UE transmission power at Victim gNB

  - Reduce the DL transmission power of Aggressor gNB

Agreements:

- Further study PRACH enhancement for RIM mitigation

  - FFS network enhancement and/or UE enhancements

    - Network enhancements include multiple PRACH configurations or PRACH reconfiguration by gNB

    - UE PRACH enhancement include UE adopts autonomous RACH enhancement based on multiple PRACH configurations

Agreements:

- Modify in framework 1 in step 3,

  - Note: it is clarified the victim continues RS-1 transmission if RS-2 is detected.

  - the victim may stop RS-1 transmission if RS-2 is not detected and the IoT going back to certain level.

Agreements:

- Further study the following:

  - OAM enhancements:

    - For NR-RIM framework- 1, 2.1 and 2.2, when atmospheric duct interference is detected by victim gNB, victim gNB reports the remote interference to OAM, OAM indicates the potential aggressor gNBs to start the RIM-RS monitoring.

    - When RS-1 is detected at aggressor, aggressor gNB reports to OAM, OAM may configure mitigation schemes at Victim

    - Note that this depends whether the OAM can support such indication in the whole network

  - Timer-based schemes for terminating RS monitoring/transmission

  - Asymmetric channel conditions between a pair of aggressor-victim gNBs

Agreements:

- The gNB is not expected to receive RS before the DL transmission boundary, and not expected to transmit RS after the UL reception boundary.

Agreements:

- The following requirements are at least considered in the RIM RS design

  - The RIM RS should be distinguished from existing RSs used for other purposes, by resource configurations and/or RS sequence design.

  - The RIM RS should be well designed to handle large path delay

Agreements:

- Strive for unified design of RIM RS to convey information for gNB (or gNB group) identification, irrespective of framework chosen, in terms of sequence type, time and frequence transmission pattern

  - Note that the information conveyed in different frameworks does not need to be the same

- Under unified RS design, FFS whether RS-1 and RS-2 in framework 1 are the same RS or distinguish from each other.

Agreements:

- At least one of the following methods is supported to distinguish RIM-RS resources:

  - TDM method: different time-domain occasions are used to distinguish RIM-RS resource

  - FDM method: different frequency positions are used to distinguish RIM-RS resource

    - FFS: comb offsets if comb-like frequency structure is adopted;

  - CDM method: different RS sequences are used to distinguish RIM-RS resource

    - FFS: the number of sequences transmitted on the same time-frequency resource;

    - FFS: OCC index if frequency-domain OCC is adopted.

  - Other methods are not precluded.

Agreements:

- Transmission position of RIM RS-1 in framework 1 and RS in framework 2 is fixed in the last X symbols before the DL transmission boundary, i.e., the ending boundary of the transmitted RIM-RS aligns with the 1st reference point

  - X is the number of symbols that RIM RS(s) are mapped to.

  - FFS for transmission position of RS-2 in framework 1

Agreements:

- For the time-domain pattern for RIM RS, an RS transmission periodicity is defined

  - The transmission periodicity can be semi-statically configured per network.

  - Within the transmission periodicity, multiple time-domain RIM RS transmission occasions are defined.  One or multiple transmission occasions can be semi-statically configured to distinguish one RIM-RS resources or convey set ID information per network

    - FFS details (especially w.r.t. X symbols)

Agreements:

- Study further potential enhancement to improve RS detection performance including potential spec impact (if any):

  - FFS. power boosting (e.g., symbol-level, etc.)

  - FFS. time-domain repetition including granularity of repetitions

  - FFS. whether additional signaling is necessary

Agreements:

- RIM RS for a given functionality transmitted by a gNB or a gNB set are configured with frequency location(s) known to the receiving gNB

Agreements:

- The bandwidth of RIM-RS can be smaller than the carrier bandwidth.

  - FFS. [20MHz, 10MHz, 5MHz, 20 PRB] as a starting point.

- The RIM RS SCS can be configured by the network.

  - FFS: The candidate set of the RIM RS SCS.

RAN1 #95

In RAN1 #95, the basic RIM RS resource and the RIM RS configurations for a gNB are concluded, recommendations for RIM RS design are made. Mechanisms to improve network robustness are identified with no spec impacts. Evaluation results are summarized to conclude the SI. The detailed agreements are listed as follows.

TP to capture agreements from this meeting (R1-1814292) (including summary of evaluation results), R1-1814360

Agreements:

- The 2OS RS with comb-1 is recommended as the basic RIM RS resource.

  - The 2OS is corresponding to the RIM RS SCS.

  - FFS whether to support additional # of symbols for RIM RS

    - If supported, it is achieved based on the the basic RIM RS resource

  - Time-domain circular RS with 2 OFDM symbols (i.e., Alt2 or Alt 3) is supported for RIM RS.

    - Note: For Alt 2, the length of CP plus the length of the concatenated sequences equal to the length of 2 OFDM symbols

    - Further discussion offline to down-select between Alt 2 vs. Alt 3 during this week

    - Alt 2 is supported, while Alt 3 is not supported

Concerns from Huawei: It’s Huawei’s strong belief that time-domain circularity is lost by using Alt 2 when RS is generated, with mapping with time repetition into more than one consecutive basic RIM RS resources which results in bad detection performance.

Agreements:

- For all time occasions, the sequence candidates for detection should be the same,

- If RS-2 is configured, RS-1 and RS-2 are differentiable, via separate configuration using the following:

  - Time occasion, and

  - RS sequence

Agreements:

- A gNB can be configured with multiple RIM RS configurations in a configured RIM RS periodicity

  - Each RIM-RS configuration is referring to the configuration of the resource in time, frequency and sequence for transmission of a basic RIM RS resource

  - For each gNB, multiple configurations of RIM RS-1 share the same frequency resource and sequence

  - For each gNB, multiple configurations of RIM RS-2 share the same frequency resource and sequence

  - The maximum number of configurations to be decided WI stage

  - For different gNB, it is to be decided in WI stage that different frequency resource for RIM RS-1/RS-2 is allowed or not

- For all gNBs, the configured RIM RS periodicity should be same

- Minimum bandwidth of above configured frequency resource is 20MHz if the carrier bandwidth is larger than 20MHz, otherwise the bandwidth of above configured frequency resource is equal to the carrier bandwidth

  - Maximum bandwidth to be decided in WI stage

- For each gNB, the number of candidate frequency resource for RIM-RS detection is 1 for bandwidth smaller than 40MHz

- For each gNB, the number of candidate frequency resource for RIM-RS detection is up to Y for bandwidth larger or equal to 40MHz

  - Y is to be decided in WI stage dependent on assumption in WI of the number of ID to be conveyed in a given duration

  - It is to be decided if only one of the candidate frequency resource for RIM-RS transmission can be configured for each gNB

Agreements:

- The max number of sequences that one gNB needs to detect in one DL-UL period for interference identification is 8.

Agreements:

- Only one SCS is configured for RIM-RS per carrier per network to reduce gNB detection complexity.

  - The candidate set can be {15kHz, 30kHz, [60kHz]}

    - Down-selection can be revisited during the WI phase

Agreements:

- The RIM-RS transmission periodicity should be a multiple of the periodicity of the TDD DL/UL pattern, or a multiple of the combined periodicity, if two TDD DL/UL patterns are configured.

Agreements:

Capture the following descriptions for time-domain RIM mitigation solutions in the TR, Section 6.4.1:

Time-domain RIM mitigation solutions may be applied at Aggressor-side, Victim-side or at both sides.

Solutions with network implementation:

- At Victim side, the victim can avoid scheduling on UL symbols suffering from remote interference or reconfigure slot format to reduce the number of UL symbols. The uplink throughput is sacrificed in this method. This solution does not have specification impact.

- At Aggressor side, the aggressor can mute/backoff or avoid scheduling on DL symbols that cause remote interference to the victim, or backs off DL symbols by reconfiguring the slot format. The downlink throughput is sacrificed in this method. In this solution, the mitigation scheme can be achieved through network implementation. However, the aggressor needs to know how many UL symbols it causes interference to at the victim gNBs, this can be estimated through the RIM-RS transmitted by the victim.

  - Note that there is a chance that the DL symbols causing interference to victim would overlap with SSB (or other measurement RSs) at the aggressor, however, it can be up to network implementation to handle the case that DL symbols causing interference overlap with SSB (or other measurement RSs) symbols at the aggressor-side.

- Time-domain solution can also be in a static manner. The victim and the aggressor can be statically configured with a TDD pattern with long enough guard period to provide robustness against RI, instead of dynamic DL muting. This solution increases the overhead due to the guard period and therefore the throughput is sacrificed. The overhead of GP can be reduced by minimizing the number of DL/UL switching by using TDD DL/UL configuration with longer period. This solution does not have spec impact.

Agreements:

- It is beneficial that Victim gNB can use RIM RS to convey the following information:

  - “Enough mitigation, no further actions needed”

  - “Not enough mitigation, further actions needed”

  - Note: to decide in the WI phase how/whether to reflect the above

  - To capture in the TR, section 6.3.1

Agreements:

Capture the following descriptions for frequency-domain RIM mitigation solutions in the TR section6.4.1.2:

Frequency-domain RIM mitigation solutions may be applied at Aggressor-side, Victim-side or at both sides.

Solutions by network implementation:

- Victim gNB can avoid scheduling in frequency resources that are interfered. In this solution, the victim can measure the interference per resource block and avoid use the frequency resources suffering from high interference. Note that if the uplink reception in the full bandwidth is interfered, frequency-domain based solutions only applied at the victim side cannot work. This solution does not have specification impact.

- The aggressor DL and the victim UL can be statically or semi-statically configured of orthogonal frequency resources. This can be achieved without specification impact:

- The victim UL and the aggressor DL can be statically configured to use non-overlapping bandwidth all the time. This solution sacrifices the spectral efficiency, even in absence of remote interference.

- The OAM can pre-configure the valid frequency-domain resource used for DL and UL to gNBs for the situation when remote interference is present. In this way, some UL resources can be protected for victim gNB, where critical UL resources, such as resources for initial access can be configured on.

- Based on the detection of the RIM RS, the aggressor gNB can perform muting in partial frequency for bandwidth larger or equal to 40MHz.

Agreements:

To Capture the following descriptions for spatial-domain RIM mitigation solutions in the TR, section6.4.1.3:

Spatial-domain RIM mitigation solutions may be applied at Aggressor-side, Victim-side or at both sides.

Solutions by network implementation:

- Mounting antennas at lower height. This is a static solution and it may sacrifice corresponding cell coverage, even in absence of remote interference.

- The victim gNB performs receive beam nulling or apply interference rejection combining receiver or beam selection to suppress the remote interference in the spatial domain.

- The victim gNB schedules UE transmission that will be received in spatial directions that are less interfered at the victim gNB.

- The victim gNB adjusts the down-tilt so that the remote interference level is tolerable. In this solution, the cell coverage at the victim may be sacrificed.

- The aggressor gNB adjusts the down-tilt in a pre-determined way. In this solution, the cell coverage at the aggressor may be sacrificed.

- The aggressor uses beam directions that would cause minimal remote interference at the victim gNB in the resource adjacent to GP. This beam direction can be obtained according to reciprocity by choosing the beam direction which experiences minimal interference.

Agreements:

Capture the following descriptions for power-domain RIM mitigation solutions in the TR, 6.4.1.4:

Power-domain RIM mitigation solutions may be applied at Aggressor-side, Victim-side or at both sides.

Solutions by network implementation:

- The victim gNB increases the UE transmission power by increasing target receiver power P0, or fractional path-loss compensation factor α or using TPC command. This may cause more interference to neighbor cells and increase UE power consumption.

- The aggressor gNB reduces the DL transmission power on DL symbols potentially causing remote interference by a fixed step. This will impact on the cell coverage and may not ensure that remote interference is completely eliminated at the victim.

Agreements:

Capture the following descriptions for RIM mitigation solutions for improved PRACH enhancements in the TR, Section6.4.1.5:

For PRACH transmission, the configured PRACH resource in the victim cell may collide with UL time duration suffering from remote interference. To improve the PRACH robustness, the following network implementation solutions can be considered:

- RI mitigation solutions for improved PRACH robustness can be applied at the victim-side by zeroing out the interfered part of the received signal and only perform detection steps on the non-interfered part of the receive signal.

- Victim gNB can configure an appropriate PRACH configuration index to ensure that the valid PRACH occasions are always on the UL symbols with enough gaps to the GP. This is a static solution and it may sacrifice initial access latency, even in absence of remote interference.

- Victim gNB can reconfigure the PRACH configuration index to ensure that the valid PRACH occasions are on the UL symbols within a TDD DL/UL periodicity that are less interfered by remote interference when detecting the presence of RI. The PRACH reconfiguration is performed by network in SIB1. This may sacrifice initial access latency.

- Victim gNB can reconfigure the PRACH preamble target received power or power ramping step to ones that are more robust towards RI when detecting the presence of RI. The PRACH reconfiguration is performed by network in SIB1. This may sacrifice initial access latency and may cause more interference to neighbor cells.

Agreements:

- No UE side’s enhancement is to be introduced for RIM of R16

Agreements:

Observations:

- Networking implementation, e.g., network reconfiguration of the parameters for random access, can improve the robustness of the random access of UEs, including legacy UEs.

  - PRACH robustness improvement for RIM can be achieved by network implementation only

- Some new proposals for rach access mechanism can be beneficial for new UE with the cost of additional RACH resource

Agreements:

- It is recommended not to support Framework 2.2 in Rel-16.

- It is recommended to specify components to enable Framework-0, Framework-1 and Framework 2.1 in Rel-16.

Agreements:

- Information reporting to OAM from both aggressor and victim gNB is supported from RAN1 perspective

  - Note: this may or may not have spec impact

Agreements:

The following features and solutions are recommended to be specified as part of a Rel.16 RIM WI from a RAN1 perspective:

- Unified design on RIM RS-1/2 and RIM RS in terms of sequence type, time and frequency transmission pattern, applicable for all frameworks

  - RS-1 and RS-2 are differentiable

  - Each RIM-RS can convey information on gNB (or gNB set) identification,

  - RIM RS can assist the aggressor to identify how many UL OFDM symbols at victim it impacted and/or provide information whether the atmospheric ducting phenomenon exists

    - It is beneficial for RIM RS-1 or RIM RS-2 to be used by victim gNB to convey  information that  “Enough mitigation, no further actions needed” &  “Not enough mitigation, further actions needed”

- Basic RIM-RS resource is recommended be designed based on the following

  - Pseudo-random sequence (length-31 Gold sequence) as in NR Rel-15

  - 2OS RS with comb-1, and time-domain circular RS with 2 OS

  - Support candidate SCS set {15kHz, 30kHz, [60kHz]}

  - Support max number of sequences in one DL-UL periodicity for interference identification is 8

2.1.2	Remaining Open issues

No open issues remained in SI

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

RAN3 #101bis

Agreements:

gNBs can be grouped as Cluster.

RAN3 study the design of IDs encoded into the RS and the mechanism to support the gNB grouping, which are common for all RIM solutions.

The gNB cluster is configured by OAM.

A reply LS to RAN1 on the feasibility of RIM solution frameworks was agreed in R3-186224

R3-186224	Reply LS on RIM solution frameworks

Conclusion 1: RAN3 concluded that all three frameworks (1, 2.1 and 2.2) are feasible from the signalling point of view. Nevertheless, concerns were raised about the benefits of using Framework 2.2. Considering the limited time of the Study Item, and the fact that the full description of Framework 2.2 has not been provided (which would have been necessary, to better understand the potential benefits), it is proposed to classify the Framework 2.2 as feasible from the signaling point of view, but not feasible in the scope of the ongoing work in this Study Item, due to unclear benefits and unclear content of RIM coordination assistance information in step3 of Framework 2.2.

Conclusion 2: RAN3 informs RAN1 that the following RAN3 agreements have been reached:

- Agreement 1: gNBs involved in a remote interference scenario can be grouped into gNB sets, where all gNBs in a set transmit an identical RS. The gNB sets can be victim sets or aggressor sets.

- Agreement 2: The OAM system is responsible for grouping of gNBs into sets, and the assignment of gNB set IDs to gNBs.

- Agreement 3: The design of gNB set IDs and the mechanisms for grouping gNBs into sets should be common for all frameworks.

- Agreement 4: RAN1 is respectfully asked to incorporate the Agreements 1-3 into all RIM candidate frameworks.

- Agreement 5: RAN3 to study the design of gNB set IDs encoded into the RS.

RAN3 #102

Chair to report to RAN that SI can be considered complete from RAN3 point of view, with the following comments:

- there are concerns that some of the assumptions, based on RAN1 work, might need to be further verified;

- down-selection of options could not be performed

Agreements:

SET ID and associated RIM RS resource are configured by OAM, and sent over air in the RIM RS

TPs in R3-187276, R3-187277 and R3-187240 were agreed to be captured in TR 38.866

R3-187276 gNB set configuration

A gNB participating in an RI scenario may be a victim, an aggressor or both. The gNBs involved in an RI scenario can be grouped into sets. Each set is assigned a set ID, and is configured with a RIM Reference Signal (RIM-RS) and the radio resources to send and receive the RIM-RS. The grouping of gNBs into sets, the set ID, the RIM-RS configuration and the associated RIM-RS radio resources for sending and/or receiving the RIM-RS are performed by the OAM system, and these resources may be assigned in static or a non-static manner. A gNB may be assigned several set IDs and RIM-RS configurations simultaneously. Every gNB may have pre-configured set ID(s). Depending on the RI scenario and network deployment, a set can comprise one or more gNBs. The design of RIM-RS, the set IDs and the mechanism for grouping of gNBs into sets should be common for all RIM frameworks. A set ID is encoded inside the RIM-RS and sent over the air. The length of set ID is pending to RAN1 progress. A gNB set ID may be reused inside a PLMN. Every gNB set ID is mapped to backhaul address for routing within the core network, which is used as the destination address for routing of RIM backhaul messages.

R3-187277 Intra-set coordination for RIM

RAN3 analysed the need for intra-set coordination based on the following assumptions: - Victim gNB sets: A TDD network is synchronized, where RIM reference signals (RIM-RS) are sent at predefined symbol positions, as configured by the OAM system. Victims in a set individually decide whether to transmit the RS-1. The judgement on whether or not the remote interference (RI) has stopped should be per-cell and there is no need to coordinate in this case either. - Aggressor gNB sets: Each aggressor cell individually concludes whether or not the applied RIM scheme has led to the cessation of RI, based on the RS-1 it receives. For the normative phase, intra-set gNB-to-gNB coordination inside victim and aggressor gNB sets is not recommended by RAN3.

R3-187240 Inter-cluster coordination for RIM

The main options for inter-cluster backhaul signalling are via Xn interface or via the core network. Signalling through the core network is preferred for the following reasons: - RI aggressor and victim nodes are per definition not direct neighbours, but located at distances up to 300 km apart, which could create issues for routing in Xn transport networks designed for local connections; - RI aggressor and victim nodes may belong to different operators using the same frequency in e.g. different countries; - In RI scenarios there is a high number of potential aggressor-victim combinations. Xn signalling requires the setup of an SCTP connection and may therefore not be scalable, while routing through the core network is performed in a connection-less manner. RAN3 recommends to start the normative work on inter-cluster RIM backhaul signalling via the core network (CN). An end-to-end RIM backhaul communication path between an aggressor and a victim gNB consists of three segments: - Source gNB set to Source gNB set AMF, - Source gNB set AMF to destination gNB set AMF, - Destination gNB set AMF to the destination gNB set. For the RIM normative phase, RAN3 recommends the inter-cluster RIM backhaul signalling framework with the following properties: - The RIM backhaul messages have the following content: - Source ID, - Destination ID, - Information about the detection of the RIM-RS, or disappearance of the RIM-RS: - If the AMF performs aggregation of outgoing messages from the gNBs in the set, the aggregated RIM backhaul message may contain the list of sending gNBs. - The RIM backhaul messages can be sent by aggressors or victims. The level of impact in the 5GC (AMF) and core network OAM system depends on the level of required functionality. Two alternative solutions are identified. Solution 1: The solution involves gNBs registering to the AMF using Set IDs configured by the OAM system. An AMF collects the outgoing RIM backhaul messages from the gNBs in the Source gNB set, aggregates these messages into a single RIM backhaul message, and forwards the aggregated message towards the AMF of the Destination gNB set. The aggregated RIM backhaul message contains the list of all the source gNBs whose messages were merged into the aggregated RIM message. Upon receiving a RIM backhaul message destined to its affiliated gNB set, the Destination AMF distributes the incoming RIM backhaul message to the gNBs constituting the receiving set. The mapping between the gNB set IDs and their corresponding AMF IDs can be e.g. in the form of a mapping table stored at the CN. The mapping could be retrieved by the AMF from a database located in e.g. the core network (CN) (other methods are not precluded). Solution 2: An alternative solution avoiding CN impact uses routing functionality introduced for transfer of SON configuration information, where containers defined in NGAP are transparently transferred through the CN (including inter-AMF signalling). Specific information for NR RIM can be defined within this container without AMF impact. Routing in the CN is based on TAI and Global gNB ID. In this solution, some mechanism would have to enable mapping in gNBs from gNB set ID received in RIM-RS to one or more TAI / Global gNB ID pairs. In order to allow for “dynamically” updated gNB sets, the local RAN OAM system enables mapping in gNBs from received RIM-RS to a globally unique cluster ID, and a DNS solution (out of 3GPP scope) is used to retrieve TAI / Global gNB ID of one or more gNBs of the cluster. RAN3 recommends to specify inter-cluster RIM backhaul signalling via the core network for framework 2.1 in the normative work.

2.3.2	Remaining Open issues

No open issues remained in SI

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #94bis

R1-1811039	Updated Work plan for Rel-16 SI on NR RIM	CMCC

R1-1811091	Update for TR 38.866 Study on remote interference management for NR	CMCC

R1-1811981	Updated summary for NR-RIM CMCC

R1-1812025	Updated summary for NR-RIM CMCC

R1-1812060	Updated summary for NR-RIM CMCC

R1-1810147	Design considerations for RIM framework	Huawei, HiSilicon

R1-1811440	On mechanisms for gNB coordination	Ericsson

R1-1810330	Evaluation Methodology and Preliminary Results for RIM	ZTE

R1-1811210	On evaluation of RIM	Nokia, Nokia Shanghai Bell

R1-1811819	Evaluation results for RIM RS and remaining issues on evaluation methodology for RS evaluations	Ericsson

Revision of R1-1811436

R1-1810290	Discussion on evaluations for NR RIM support	LG Electronics

R1-1810713	Evaluation of reference signals in RIM	Huawei, HiSilicon

R1-1810782	Further considerations on RIM frameworks	Intel Corporation

R1-1811638 	Discussion on remaining evaluation assumptions and initial evaluation results for RIM	CMCC

Revision of R1-1811040

R1-1811269	Remaining details on evaluation methodology	Qualcomm Incorporated

R1-1810148	Discussion on remote interference mitigation mechanisms	Huawei, HiSilicon

R1-1811270	Mechanisms for improving network robustness	Qualcomm Incorporated

R1-1810291	Discussion on mechanism for NR RIM support	LG Electronics

R1-1810331	Consideration on Mechanisms for Improving Network Robustness	ZTE

R1-1810548	On remote interference management mechanism	CATT

R1-1811816	RIM-RS design and evaluation	Intel Corporation

Revision of R1-1810783

R1-1810876	Discussion on remote interference management	Samsung

R1-1811041	Discussion on improving network robustness for RIM	CMCC

R1-1811211	Remote interference management for NR	Nokia, Nokia Shanghai Bell

R1-1811437	On mechanisms for increasing network robustness	Ericsson

R1-1811042	Discussion on RS design for RIM	CMCC

R1-1811419	Discussion on procedure for RIM framework	Hisense Co., Ltd.

R1-1810149	Discussion on reference signal design for identifying remote interference	Huawei, HiSilicon

R1-1810292	Discussion on reference signal design for NR RIM support	LG Electronics

R1-1810332	Consideration on Mechanisms for Identifying Strong gNB Interference	ZTE

R1-1810549	On identification of gNB interferers	CATT

R1-1810877	Discussion on reference signal for remote interference identification	Samsung

R1-1811212	Considerations on identifying strong gNB interferers	Nokia, Nokia Shanghai Bell

R1-1811271	Mechanisms for identifying strong gNB interferers	Qualcomm Incorporated

R1-1811438	On RIM reference signal design	Ericsson

R1-1811537	On remote interference management reference signal design	CEWiT

R1-1811608	Discussion on Reference Signal Design for RIM	China Telecommunications

RAN1 #95

R1-1812902	Text proposal for TR 38.866 Study on remote interference management for NR	CMCC

R1-1814291   Text proposal for TR 38.866 Study on remote interference management for NR	CMCC

R1-1814360	Text proposal for TR 38.866 Study on remote interference management for NR	CMCC

R1-1813170	On mechanisms for gNB coordination	Ericsson

R1-1812218	Evaluation of reference signals in RIM	Huawei, HiSilicon

R1-1812439	Evaluation Results for RIM	ZTE

R1-1812498	Evaluation results for NR RIM	Intel Corporation

R1-1812569	Preliminary evaluation results for NR RIM	LG Electronics

R1-1812625	Evaluation results of reference signal	CATT

R1-1812882	Evaluation results for RIM	CMCC

R1-1813889	Evaluation results for RIM RS	Ericsson

Revision of R1-1813166

R1-1813879	Evaluation of RIM RS design	CEWiT

Revision of R1-1813376

R1-1813429	Evaluation Results	Qualcomm Incorporated

R1-1813465	On evaluation of RIM	Nokia, Nokia Shanghai Bell

R1-1812216	Discussion on remote interference mitigation mechanisms	Huawei, HiSilicon

R1-1812440	Discussion on RIM mechanisms for improving network robustness	ZTE

R1-1812499	Mechanism on remote interference mangement	Intel Corporation

R1-1812570	Discussion on mechanism for NR RIM support	LG Electronics

R1-1812626	On remote interference mitigation mechanism	CATT

R1-1812883	Discussion on improving network robustness for RIM	CMCC

R1-1812992	Discussion on remote interference management	Samsung

R1-1813167	On mechanisms for improving network robustness	Ericsson

R1-1813374	On framework modification for asymmetric remote interference management	CEWiT

R1-1813430	Mechanisms for improving network robustness	Qualcomm Incorporated

R1-1813466	Discussion on NR remote interference mitigation schemes	Nokia, Nokia Shanghai Bell

R1-1812217	Discussion on reference signal design for identifying remote interference	Huawei, HiSilicon

R1-1812441	Discussion on reference signal for RIM	ZTE

R1-1812500	Considerations on RIM RS	Intel Corporation

R1-1812571	Discussion on reference signal design for NR RIM support	LG Electronics

R1-1812627	On identification of aggressor and victim gNBs	CATT

R1-1813899	Discussion on RS design for RIM	CMCC

Revision of R1-1812884

R1-1812993	Discussion on reference signal for remote interference identification	Samsung

R1-1813890	On RIM reference signal design	Ericsson

Revision of R1-1813168

R1-1813375	On remote interference management reference signal design	CEWiT

R1-1813431	Mechanisms for identifying strong gNB interferers	Qualcomm Incorporated

R1-1813467	Determination of RIM-RS resources	Nokia, Nokia Shanghai Bell

R1-1812678	Overview of RIM frameworks and potential specification impacts	Huawei, HiSilicon

R1-1812679	Potential specification impacts for RIM RS design	Huawei, HiSilicon

R1-1812680	Potential specification impacts for remote interference mitigation schemes	Huawei, HiSilicon

R1-1813169	On encoding of gNB set ID in RIM-RS	Ericsson

R1-1813171	System assessment for RIM	Ericsson

R1-1813172	PRACH improvements for network robustness	Ericsson

R1-1813432	UE impact by RIM	Qualcomm Incorporated

RAN3 #101bis

R3-185441	LS on NR-RIM frameworks	3GPP RAN1, China Mobile

R3-185469	Initial consideration on RIM	ZTE

R3-185514	Remote Interference Detection	Qualcomm Incorporated

R3-185515	Configuration and coordination for RS transmission	Qualcomm Incorporated

R3-185516	Remote coordination by backhaul (framework #2)	Qualcomm Incorporated

R3-185680	The scenarios and general principles of RIM solutions	Ericsson

R3-185681	gNB identification and grouping in RIM scenarios	Ericsson

R3-185682	The RIM solution frameworks	Ericsson

R3-185683	[Draft] Reply LS on RIM solution frameworks	Ericsson

R3-185846	Signalling and OAM requirements for NR Remote Interference Management	Nokia, Nokia Shanghai Bell

R3-185847	Xn procedures for inter-gNB signaling of TDD radio frame configuration	Nokia, Nokia Shanghai Bell

R3-185965	Possibility of backhaul signaling without core network involvement	LG Electronics Inc.

R3-185966	Consideration on backhaul signaling with core network involvement	LG Electronics Inc.

R3-186015	Discuss on the RIM support	CATT

R3-186024	Considerations on NR RIM	Huawei

R3-186028	TR 38.866 V0.0.0	CMCC

R3-186029	Work plan for study on RIM	CMCC

R3-186030	Discussion on support of RIM framework	CMCC

R3-186116	NR Remote Interference Management session report	Vice chairman

R3-186224	Reply LS on RIM solution frameworks	Ericsson

RAN3 #102

R3-186621	TR 38.866 V0.1.0	CMCC

R3-186749	Open issues on gNB grouping and set ID design 	LG Electronics Inc.

R3-186569	Discussion on gNB set ID define 	CATT

R3-186462	Dynamic cell group configuration 	Qualcomm Incorporated

R3-186493	gNB Grouping and Set ID Design	Huawei

R3-186505	TP for gNB Grouping and Set ID Design 	ZTE

R3-186510	TP for Intra-cluster coordination 	ZTE

R3-186494	RIM Xn procedure	Huawei

R3-186622	(TP to TR 38.866) Signalling flow without CN involvement for backhaul-based RIM framework	CMCC

R3-186463	Remote Interference Mitigation by Backhaul coordination 	Qualcomm Incorporated

R3-186570	Discussion on message routing for RIM 	CATT

R3-186623	(TP to TR 38.866) Signalling flow with CN involvement for backhaul-based RIM framework	CMCC

R3-186498	TP on Inter-gNB Coordination for RIM With Core Network Involvement	ZTE

R3-186756	[TP for TR 38.866] Overall mechanism for inter- and intra-cluster coordination	Nokia, Nokia Shanghai Bell

R3-186474	TP to TR 38.866: The way forward on RIM backhaul RIM frameworks	Ericsson

R3-186475	TP for TR 38.866 on the realization of RIM backhaul communication 		Ericsson

R3-186476	[Draft] LS on RIM backhaul framework 	Ericsson

R3-186477	TP for TR 38.866: The open issues in RIM	Ericsson

R3-186495	RIM NG procedure		Huawei

R3-186733	TP to TR 38.866: The way forward on RIM backhaul RIM frameworks 	Ericsson

R3-186734	R3-18xxxx TP for TR 38.866 on the realization of RIM backhaul communication 	Ericsson

R3-186735	[Draft] LS on RIM backhaul framework 		Ericsson

R3-186736	TP for TR 38.866: The open issues in RIM	Ericsson

R3-186478	TP to TR 38.866: The RAN3 agreements from RAN3#101bis	Ericsson

R3-186737	TP to TR 38.866: The RAN3 agreements from RAN3#101bis 	Ericsson

R3-187199	NR Remote Interference Management session report, RAN3 vice chairman

R3-187202	Summary of offline discussion of CB #1 SET ID, ZTE

R3-187204	Inter-cluster Signaling Solution options, ZTE

R3-187205	Summary of offline discussion for intra-cluster coordination, Ericsson

R3-187206 	Summary of offline discussion on Inter-cluster signalling, CMCC

R3-187212	TP for gNB Grouping and Set ID Design, ZTE

R3-187240	TP of inter-cluster signalling for TR 38.866, CMCC

R3-187241	Status summary of NR RIM SI, CMCC

R3-187276	TP for gNB Grouping and Set ID Design, ZTE

R3-187277	Summary of offline discussion on Intra-cluster co-ordination, Ericsson

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template