3GPP TSG RAN WG1 Meeting #94bis 	R1-1811703

# Chengdu, China, October 8-12, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0309 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on aperiodic CSI-RS in 36.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eFDMIMO-Core |  |  |  |  |  |  | Date: |  | 2018-10-08 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The higher layer parameter CSI-RS-ConfigZPAperiodic is defined and used in 36.212, which is inconsistent with the higher parameter csi-RS-ConfigZP-ApList defined in 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the parameter CSI-RS-ConfigZPAperiodic to csi-RS-ConfigZP-ApList in section 5.3.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### < Unchanged parts are omitted >

##### 5.3.3.1.2	Format 1

DCI format 1 is used for the scheduling of one PDSCH codeword in one cell.

The following information is transmitted by means of the DCI format 1:

- Carrier indicator – 0 or 3 bits. This field is present according to the definitions in [3].

- Resource allocation header (resource allocation type 0 / type 1) – 1 bit as defined in subclause 7.1.6 of [3]

If downlink bandwidth is less than or equal to 10 PRBs, there is no resource allocation header and resource allocation type 0 is assumed.

- Resource block assignment:

- For resource allocation type 0 as defined in subclause 7.1.6.1 of [3]:

- bits provide the resource allocation

- For resource allocation type 1 as defined in subclause 7.1.6.2 of [3]:

-  bits of this field are used as a header specific to this resource allocation type to indicate the selected resource blocks subset

- 1 bit indicates a shift of the resource allocation span

-  bits provide the resource allocation

where the value of P depends on the number of DL resource blocks as indicated in subclause 7.1.6.1 of [3]

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- HARQ process number – 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

- New data indicator – 1 bit

- Redundancy version – 2 bits

- TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

- Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

- HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

- MUST interference presence and power ratio – 0 or 2 bits as defined in subclause 6.3.3 of [2]. This field is present only when the UE is configured for MUST-near operation and the number of antenna ports for CRS transmission in the serving cell is 2.

- Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

If the UE is not configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 1 is equal to that for format 0/1A, one bit of value zero shall be appended to format 1.

If the UE is configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 1 is equal to that for format 0/1A for scheduling the same serving cell and mapped onto the UE specific search space given by the C-RNTI as defined in [3], one bit of value zero shall be appended to format 1.

If the number of information bits in format 1 carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one or more zero bit(s) shall be appended to format 1 until the payload size of format 1 does not belong to one of the sizes in Table 5.3.3.1.2-1 and is not equal to that of format 0/1A mapped onto the same search space.

Table 5.3.3.1.2-1: Ambiguous Sizes of Information Bits

{12, 14, 16 ,20, 24, 26, 32, 40, 44, 56}

Table 5.3.3.1.2-2: Number of bits for Downlink Assignment Index

| Number of bits |  |
| --- | --- |
| 4 | For UEs configured by higher layers with codebooksizeDetermination-r13 = dai and when a DCI format 1/1A/1B/1D/2/2A/2B/2C/2D/6-1A scheduling PDSCH is mapped onto the UE specific search space given by the C-RNTI as defined in [3], or UEs configured by higher layers with codebooksizeDeterminationSTTI = dai and when a DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G scheduling PDSCH is mapped onto the UE specific search space given by the C-RNTI as defined in [3], the 4-bit DAI consists of a 2-bit counter DAI and a 2-bit total DAI. - Counter DAI – 2 bits as defined in subclause 7.3 of [3] - Total DAI – 2 bits as defined in subclause 7.3 of [3] |
| 2 | For UEs not configured with codebooksizeDetermination-r13 = dai and codebooksizeDeterminationSTTI-r15=dai, or for UEs configured by higher layers with codebooksizeDetermination-r13 = dai and when a DCI format scheduling PDSCH is not mapped onto the UE specific search space given by the C-RNTI as defined in [3], this field is present for FDD or TDD operation, for cases with TDD primary cell. If the UL/DL configuration of all TDD serving cells is same and the UE is not configured to decode PDCCH with CRC scrambled by eimta-RNTI, then this field only applies to serving cell with UL/DL configuration 1-6. If at least two TDD serving cells have different UL/DL configurations or the UE is configured to decode PDCCH with CRC scrambled by eimta-RNTI, then this field applies to a serving cell with DL-reference UL/DL configuration 1-6 as defined in subclause 10.2 of [3]. For UEs configured with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 and a DCI format scheduling PDSCH is mapped onto the UE specific search space given by the C-RNTI as defined in [3], this field is present. |
| 0 | For UEs not configured with codebooksizeDetermination-r13 = dai and codebooksizeDeterminationSTTI-r15=dai and not configured with EN-DC and higher layer parameter subframeAssignment-r15, or for UEs configured by higher layers with codebooksizeDetermination-r13 = dai and when a DCI format scheduling PDSCH is not mapped onto the UE specific search space given by the C-RNTI as defined in [3], this field is not present for FDD or TDD operation, for cases with FDD primary cell. For UEs configured with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 and a DCI format scheduling PDSCH is not mapped onto the UE specific search space given by the C-RNTI as defined in [3], this field is not present. |

##### 5.3.3.1.3	Format 1A

DCI format 1A is used for the compact scheduling of one PDSCH codeword in one cell and random access procedure initiated by a PDCCH order. The DCI corresponding to a PDCCH order can be carried by PDCCH or EPDCCH.

The following information is transmitted by means of the DCI format 1A:

-	Carrier indicator – 0 or 3 bits. This field is present according to the definitions in [3].

-	Flag for format0/format1A differentiation or flag for format0A/format1A differentiation – 1 bit, where value 0 indicates format 0 or format 0A and value 1 indicates format 1A

Format 1A is used for random access procedure initiated by a PDCCH order only if format 1A CRC is scrambled with C-RNTI and all the remaining fields are set as follows:

- Localized/Distributed VRB assignment flag – 1 bit is set to '0'

- Resource block assignment – bits, where all bits shall be set to 1

- Preamble Index – 6 bits

- PRACH Mask Index – 4 bits, [5]

- All the remaining bits in format 1A for compact scheduling assignment of one PDSCH codeword are set to zero

Otherwise,

-	Localized/Distributed VRB assignment flag – 1 bit as defined in 7.1.6.3 of [3]

-	Resource block assignment – bits as defined in subclause 7.1.6.3 of [3]:

- For localized VRB:

bits provide the resource allocation

- For distributed VRB:

- If  or if the format 1A CRC is scrambled by RA-RNTI, P-RNTI, SI-RNTI, SC-RNTI or G-RNTI:

- bits provide the resource allocation

- Else

- 1 bit, the MSB indicates the gap value, where value 0 indicates  and value 1 indicates

-  bits provide the resource allocation,

where  is defined in [2].

-	Modulation and coding scheme – 5bits as defined in subclause 7.1.7 of [3]. The MSB is set to 0 when the UE is configured with blindSubframePDSCH-Repetitions set to TRUE and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3] and the repetition number is greater than 1 and the higher layer parameter mcs-restrictionSubframePDSCH-Repetitions is configured to 1.

-	HARQ process number – 4 bits if higher layer parameter dl-TTI-Length is configured for the cell and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3], otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured and the corresponding DCI is not mapped onto the UE specific search space given by the C-RNTI as defined in [3]), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3])

-	New data indicator – 1 bit

-	If the format 1A CRC is scrambled by RA-RNTI, P-RNTI, SI-RNTI, SC-RNTI or G-RNTI:

-	If  and Localized/Distributed VRB assignment flag is set to 1

-	the new data indicator bit indicates the gap value, where value 0 indicates  and value 1 indicates .

-	Else the new data indicator bit is reserved.

-	Else

-	The new data indicator bit as defined in [5]

-	Redundancy version – 2 bits

-	TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

-	If the format 1A CRC is scrambled by RA-RNTI, P-RNTI, or SI-RNTI:

-	The most significant bit of the TPC command is reserved.

-	The least significant bit of the TPC command indicates column of the TBS table defined of [3].

-	If least significant bit is 0 then = 2 else= 3.

-	Else

-	The two bits including the most significant bit indicates the TPC command

-	Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

-	SRS request – 0 or 1 bit. This field can only be present in DCI formats scheduling PDSCH which are mapped onto the UE specific search space given by the C-RNTI as defined in [3]. The interpretation of this field is provided in subclause 8.2 of [3]. This field is not present when the DCI is used for scheduling PDSCH in a LAA SCell.

-	HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

-	SRS timing offset – 3 bits as defined in [3]. This field is present only when the DCI format is used for scheduling PDSCH in a LAA SCell and the UE is configured with uplink transmission on the LAA Scell.

-	Repetition number – 2 bits as defined in Table 5.3.3.1.17-1, where the number of transmissions for PDSCH is given by higher layer parameter maxNumber-SubframePDSCH-Repetitions for the value of '11'. This field is present only when the UE is configured with higher layer parameter blindSubframePDSCH-Repetitions set to TRUE and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3].

If the UE is not configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI, and the number of information bits in format 1A is less than that of format 0, zeros shall be appended to format 1A until the payload size equals that of format 0.

If the UE is configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 1A mapped onto a given search space is less than that of format 0 for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 1A until the payload size equals that of format 0, except when format 1A assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

If the UE is configured to decode PDCCH or EPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 1A mapped onto a given search space is less than that of format 0A for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 1A until the payload size equals that of format 0A, except when format 1A assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

If the number of information bits in format 1A carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one zero bit shall be appended to format 1A.

When the format 1A CRC is scrambled with a RA-RNTI, P-RNTI, SI-RNTI, SC-RNTI or G-RNTI then the following fields among the fields above are reserved:

- HARQ process number

- Downlink Assignment Index (used for cases with TDD primary cell and either FDD operation or TDD operation, and is not present for cases with FDD primary cell and either FDD operation or TDD operation)

##### 5.3.3.1.3A	Format 1B

DCI format 1B is used for the compact scheduling of one PDSCH codeword in one cell with precoding information.

The following information is transmitted by means of the DCI format 1B:

- Carrier indicator – 0 or 3 bits. The field is present according to the definitions in [3].

- Localized/Distributed VRB assignment flag – 1 bit as defined in subclause 7.1.6.3 of [3]

- Resource block assignment – bits as defined in subclause 7.1.6.3 of [3]

- For localized VRB:

bits provide the resource allocation

- For distributed VRB:

- For

- bits provide the resource allocation

- For

- 1 bit, the MSB indicates the gap value, where value 0 indicates  and value 1 indicates

-  bits provide the resource allocation

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- HARQ process number – 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

- New data indicator – 1 bit

- Redundancy version – 2 bits

- TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

- Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

- TPMI information for precoding – number of bits as specified in Table 5.3.3.1.3A-1

TPMI information indicates which codebook index is used in Table 6.3.4.2.3-1 or Table 6.3.4.2.3-2 of [2] corresponding to the single-layer transmission.

- PMI confirmation for precoding – 1 bit as specified in Table 5.3.3.1.3A-2

- HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

- Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

If PMI confirmation indicates that the eNodeB has applied precoding according to PMI(s) reported by the UE, the precoding for the corresponding RB(s) in subframe n is according to the latest PMI(s) in an aperiodic CSI reported on or before subframe n-4.

Table 5.3.3.1.3A-1: Number of bits for TPMI information

| Number of antenna ports at eNodeB | Number of bits |
| --- | --- |
| 2 | 2 |
| 4 | 4 |

Table 5.3.3.1.3A-2: Content of PMI confirmation

| Bit field mapped to index | Message |
| --- | --- |
| 0 | Precoding according to the indicated TPMI in the TPMI information field |
| 1 | Precoding using the precoder(s) according to PMI(s) indicated in the latest aperiodic CSI report. For aperiodic CSI mode 2-2: - Precoding of scheduled resource blocks belonging to the reported preferred M subband(s), use precoder(s) according to the preferred M subband PMI(s) indicated in the latest aperiodic CSI report; - Precoding of scheduled resource blocks not belonging to the reported preferred M subband(s), precoding using a precoder according to the wideband PMI indicated in the latest aperiodic CSI report. |

If the number of information bits in format 1B is equal to that for format 0/1A for scheduling the same serving cell and mapped onto the UE specific search space given by the C-RNTI as defined in [3], one bit of value zero shall be appended to format 1B.

If the number of information bits in format 1B carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one or more zero bit(s) shall be appended to format 1B until the payload size of format 1B does not belong to one of the sizes in Table 5.3.3.1.2-1 and is not equal to that of format 0/1A mapped onto the same search space.

##### 5.3.3.1.4	Format 1C

DCI format 1C is used for very compact scheduling of one PDSCH codeword, notifying MCCH change [6], notifying SC-MCCH change and direct indication [6] , reconfiguring TDD, and LAA common information.

The following information is transmitted by means of the DCI format 1C:

If the format 1C is used for very compact scheduling of one PDSCH codeword

-	1 bit indicates the gap value, where value 0 indicates  and value 1 indicates

-	For, there is no bit for gap indication

-	Resource block assignment – bits as defined in 7.1.6.3 of [3] where  is defined in [2] and  is defined in [3]

-	Modulation and coding scheme – 5 bits as defined in subclause 7.1.7 of [3]

Else if the format 1C is used for notifying MCCH change and direct indication

-	Information for MCCH change notification – 8 bits as defined in subclause 5.8.1.3 of [6]

-	Direct Indication information – 2 bits provide direct indication of system information update and other fields, as defined in subclause 6.6a [6]. This field is only present for MBMS-dedicated cell or feMBMS/Unicast-mixed cell with  .

-	Reserved information bits are added until the size is equal to that of format 1C used for very compact scheduling of one PDSCH codeword

Else if the format 1C is used for notifying SC-MCCH change

-	Information for SC-MCCH change notification – 8 bits as defined in subclause 5.8a.1.3 of [6]

-	Reserved information bits are added until the size is equal to that of format 1C used for very compact scheduling of one PDSCH codeword

Else if the format 1C is used for reconfiguring TDD

-	UL/DL configuration indication:

UL/DL configuration number 1, UL/DL configuration number 2,…, UL/DL configuration number I

Where each UL/DL configuration is 3 bits, ,  is equal to the payload size of format 1C used for very compact scheduling of one PDSCH codeword. The parameter eimta-UL-DL-ConfigIndex provided by higher layers determines the index to the UL/DL configuration indication for a serving cell.

-	Zeros are added until the size is equal to that of format 1C used for very compact scheduling of one PDSCH codeword

Else

-	Subframe configuration for LAA – 4 bits as defined in clause 13A of [3]

-	UL duration and offset – 5 bits as defined in clause 13A of [3]. The field only applies to a UE configured with uplink transmission on a LAA SCell

-	PUSCH trigger B – 1 bit as defined in subclause 8.0 of [3]. The field only applies to a UE configured with uplink transmission on a LAA SCell

-	COT sharing indication for AUL – 1 bit as defined in 4.1.3 of [8]. The field only applies to a UE configured with uplink transmission on a LAA SCell

-	Reserved information bits are added until the size is equal to that of format 1C used for very compact scheduling of one PDSCH codeword

##### 5.3.3.1.4A	Format 1D

DCI format 1D is used for the compact scheduling of one PDSCH codeword in one cell with precoding and power offset information.

The following information is transmitted by means of the DCI format 1D:

- Carrier indicator – 0 or 3 bits. The field is present according to the definitions in [3].

- Localized/Distributed VRB assignment flag – 1 bit as defined in subclause 7.1.6.3 of [3]

- Resource block assignment – bits as defined in subclause 7.1.6.3 of [3]:

- For localized VRB:

bits provide the resource allocation

- For distributed VRB:

- For

- bits provide the resource allocation

- For

- 1 bit, the MSB indicates the gap value, where value 0 indicates  and value 1 indicates

-  bits provide the resource allocation

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- HARQ process number – 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

- New data indicator – 1 bit

- Redundancy version – 2 bits

- TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

- Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

- TPMI information for precoding – number of bits as specified in Table 5.3.3.1.4A-1

TPMI information indicates which codebook index is used in Table 6.3.4.2.3-1 or Table 6.3.4.2.3-2 of [2] corresponding to the single-layer transmission.

- Downlink power offset – 1 bit as defined in subclause 7.1.5 of [3]

- HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

- Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

Table 5.3.3.1.4A-1: Number of bits for TPMI information

| Number of antenna ports at eNodeB | Number of bits |
| --- | --- |
| 2 | 2 |
| 4 | 4 |

If the number of information bits in format 1D is equal to that for format 0/1A for scheduling the same serving cell and mapped onto the UE specific search space given by the C-RNTI as defined in [3], one bit of value zero shall be appended to format 1D.

If the number of information bits in format 1D carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one or more zero bit(s) shall be appended to format 1D until the payload size of format 1D does not belong to one of the sizes in Table 5.3.3.1.2-1 and is not equal to that of format 0/1A mapped onto the same search space.

##### 5.3.3.1.5	Format 2

The following information is transmitted by means of the DCI format 2:

-	Carrier indicator – 0 or 3 bits. The field is present according to the definitions in [3].

-	Resource allocation header (resource allocation type 0 / type 1) – 1 bit as defined in subclause 7.1.6 of [3]

If downlink bandwidth is less than or equal to 10 PRBs, there is no resource allocation header and resource allocation type 0 is assumed.

-	Resource block assignment:

-	For resource allocation type 0 defined in subclause 7.1.6.1 of [3]:

-	bits provide the resource allocation

-	For resource allocation type 1 as defined in subclause 7.1.6.2 of [3]:

-	 bits of this field are used as a header specific to this resource allocation type to indicate the selected resource blocks subset

-	1 bit indicates a shift of the resource allocation span

-	 bits provide the resource allocation

where the value of P depends on the number of DL resource blocks as indicated in subclause 7.1.6.1 of [3]

-	TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

-	Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

-	HARQ process number - 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

-	Transport block to codeword swap flag – 1 bit

In addition, for transport block 1:

-	Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits

-	MUST interference presence and power ratio – 0 or 2 bits as defined in subclause 6.3.3 of [2]. This field is present only when the UE is configured for MUST-near operation and the number of antenna ports for CRS transmission in the serving cell is 2

In addition, for transport block 2:

-	Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits

-	MUST interference presence and power ratio – 0 or 2 bits as defined in subclause 6.3.3 of [2]. This field is present only when the UE is configured for MUST-near operation and the number of antenna ports for CRS transmission in the serving cell is 2

Precoding information – number of bits as specified in Table 5.3.3.1.5-3

-	HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

-	Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

If both transport blocks are enabled, the transport block to codeword mapping is specified according to Table 5.3.3.1.5-1.

In case one of the transport blocks is disabled as specified in subclause 7.1.7.2 of [3], the transport block to codeword swap flag is reserved and the transport block to codeword mapping is specified according to Table 5.3.3.1.5-2.

Table 5.3.3.1.5-1: Transport block to codeword mapping
(two transport blocks enabled)

| transport block to codeword swap flag value | codeword 0 (enabled) | codeword 1 (enabled) |
| --- | --- | --- |
| 0 | transport block 1 | transport block 2 |
| 1 | transport block 2 | transport block 1 |

Table 5.3.3.1.5-2: Transport block to codeword mapping
(one transport block enabled)

| transport block 1 | transport block 2 | codeword 0 (enabled) | codeword 1 (disabled) |
| --- | --- | --- | --- |
| enabled | disabled | transport block 1 | - |
| disabled | enabled | transport block 2 | - |

The interpretation of the precoding information field depends on the number of enabled codewords according to Table 5.3.3.1.5-4 and Table 5.3.3.1.5-5. Note that TPMI indicates which codebook index is used in Table 6.3.4.2.3-1 or Table 6.3.4.2.3-2 of [2]. For a single enabled codeword, indices 18 to 34 inclusive in Table 5.3.3.1.5-5 are only supported for retransmission of the corresponding transport block if that transport block has previously been transmitted using two layers with closed-loop spatial multiplexing.

If the number of information bits in format 2 carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one zero bit shall be appended to format 2.

Some entries in Table 5.3.3.1.5-4 and Table 5.3.3.1.5-5 are used for indicating that the eNodeB has applied precoding according to PMI(s) reported by the UE. In these cases the precoding for the corresponding RB(s) in subframe n is according to the latest PMI(s) in an aperiodic CSI reported on or before subframe n-4. For aperiodic CSI mode 2-2: Precoding of scheduled resource blocks belonging to the reported preferred M subband(s) use precoder(s) according to the preferred M subband PMI indicated by the latest aperiodic CSI report; Precoding of scheduled resource blocks not belonging to the reported preferred M subband(s) use a precoder according to the wideband PMI indicated by the latest aperiodic CSI report.

Table 5.3.3.1.5-3: Number of bits for precoding information

| Number of antenna ports at eNodeB | Number of bits for precoding information |
| --- | --- |
| 2 | 3 |
| 4 | 6 |

Table 5.3.3.1.5-4: Content of precoding information field for 2 antenna ports

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- | --- | --- |
| Bit field mapped to index | Message | Bit field mapped to index | Message |
| 0 | 2 layers: Transmit diversity | 0 | 2 layers: Precoding corresponding to precoder matrix |
| 1 | 1 layer: Precoding corresponding to precoding vector | 1 | 2 layers: Precoding corresponding to precoder matrix |
| 2 | 1 layer: Precoding corresponding to precoder vector | 2 | 2 layers: Precoding according to the latest PMI report on PUSCH, using the precoder(s) indicated by the reported PMI(s) |
| 3 | 1 layer: Precoding corresponding to precoder vector | 3 | reserved |
| 4 | 1 layer: Precoding corresponding to precoder vector | 4 | reserved |
| 5 | 1 layer: Precoding according to the latest PMI report on PUSCH, using the precoder(s) indicated by the reported PMI(s), if RI=2 was reported, using 1st column multiplied by of all precoders implied by the reported PMI(s) | 5 | reserved |
| 6 | 1 layer: Precoding according to the latest PMI report on PUSCH, using the precoder(s) indicated by the reported PMI(s), if RI=2 was reported, using 2nd column multiplied by of all precoders implied by the reported PMI(s) | 6 | reserved |
| 7 | reserved | 7 | reserved |

Table 5.3.3.1.5-5: Content of precoding information field for 4 antenna ports

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- | --- | --- |
| Bit field mapped to index | Message | Bit field mapped to index | Message |
| 0 | 4 layers: Transmit diversity | 0 | 2 layers: TPMI=0 |
| 1 | 1 layer: TPMI=0 | 1 | 2 layers: TPMI=1 |
| 2 | 1 layer: TPMI=1 |  |  |
|  |  | 15 | 2 layers: TPMI=15 |
| 16 | 1 layer: TPMI=15 | 16 | 2 layers: Precoding according to the latest PMI report on PUSCH using the precoder(s) indicated by the reported PMI(s) |
| 17 | 1 layer: Precoding according to the latest PMI report on PUSCH using the precoder(s) indicated by the reported PMI(s) | 17 | 3 layers: TPMI=0 |
| 18 | 2 layers: TPMI=0 | 18 | 3 layers: TPMI=1 |
| 19 | 2 layers: TPMI=1 |  |  |
|  |  | 32 | 3 layers: TPMI=15 |
| 33 | 2 layers: TPMI=15 | 33 | 3 layers: Precoding according to the latest PMI report on PUSCH using the precoder(s) indicated by the reported PMI(s) |
| 34 | 2 layers: Precoding according to the latest PMI report on PUSCH using the precoder(s) indicated by the reported PMI(s) | 34 | 4 layers: TPMI=0 |
| 35 – 63 | reserved | 35 | 4 layers: TPMI=1 |
|  |  | 49 | 4 layers: TPMI=15 |
|  |  | 50 | 4 layers: Precoding according to the latest PMI report on PUSCH using the precoder(s) indicated by the reported PMI(s) |
|  |  | 51 – 63 | Reserved |

##### 5.3.3.1.5A	Format 2A

The following information is transmitted by means of the DCI format 2A:

- Carrier indicator – 0 or 3 bits. The field is present according to the definitions in [3].

- Resource allocation header (resource allocation type 0 / type 1) – 1 bit as defined in subclause 7.1.6 of [3]

If downlink bandwidth is less than or equal to 10 PRBs, there is no resource allocation header and resource allocation type 0 is assumed.

- Resource block assignment:

- For resource allocation type 0 as defined in subclause 7.1.6.1 of [3]

- bits provide the resource allocation

- For resource allocation type 1 as defined in subclause 7.1.6.2 of [3]

-  bits of this field are used as a header specific to this resource allocation type to indicate the selected resource blocks subset

- 1 bit indicates a shift of the resource allocation span

-  bits provide the resource allocation

where the value of P depends on the number of DL resource blocks as indicated in subclause 7.1.6.1 of [3]

- TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

- Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

- HARQ process number - 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

- Transport block to codeword swap flag – 1 bit

In addition, for transport block 1:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

In addition, for transport block 2:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

- Precoding information – number of bits as specified in Table 5.3.3.1.5A-1

- HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback. - MUST interference presence and power ratio – 0 or 2 bits as defined in subclause 6.3.3 of [2]. This field is present only when the UE is configured for MUST-near operation and the number of antenna ports for CRS transmission in the serving cell is 2.

- Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

If both transport blocks are enabled, the transport block to codeword mapping is specified according to Table 5.3.3.1.5-1.

In case one of the transport blocks is disabled, the transport block to codeword swap flag is reserved and the transport block to codeword mapping is specified according to Table 5.3.3.1.5-2.

The precoding information field is defined according to Table 5.3.3.1.5A-2. For a single enabled codeword, index 1 in Table 5.3.3.1.5A-2 is only supported for retransmission of the corresponding transport block if that transport block has previously been transmitted using two layers with large delay CDD.

For transmission with 2 antenna ports, the precoding information field is not present. The number of transmission layers is equal to 2 if both codewords are enabled; transmit diversity is used if codeword 0 is enabled while codeword 1 is disabled.

If the number of information bits in format 2A carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one zero bit shall be appended to format 2A.

Table 5.3.3.1.5A-1: Number of bits for precoding information

| Number of antenna ports at eNodeB | Number of bits for precoding information |
| --- | --- |
| 2 | 0 |
| 4 | 2 |

Table 5.3.3.1.5A-2: Content of precoding information field for 4 antenna ports

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- | --- | --- |
| Bit field mapped to index | Message | Bit field mapped to index | Message |
| 0 | 4 layers: Transmit diversity | 0 | 2 layers: precoder cycling with large delay CDD |
| 1 | 2 layers: precoder cycling with large delay CDD | 1 | 3 layers: precoder cycling with large delay CDD |
| 2 | reserved | 2 | 4 layers: precoder cycling with large delay CDD |
| 3 | reserved | 3 | reserved |

##### 5.3.3.1.5B	Format 2B

The following information is transmitted by means of the DCI format 2B:

- Carrier indicator – 0 or 3 bits. The field is present according to the definitions in [3].

- Resource allocation header (resource allocation type 0 / type 1) – 1 bit as defined in subclause 7.1.6 of [3]

If downlink bandwidth is less than or equal to 10 PRBs, there is no resource allocation header and resource allocation type 0 is assumed.

- Resource block assignment:

- For resource allocation type 0 as defined in subclause 7.1.6.1 of [3]

- bits provide the resource allocation

- For resource allocation type 1 as defined in subclause 7.1.6.2 of [3]

-  bits of this field are used as a header specific to this resource allocation type to indicate the selected resource blocks subset

- 1 bit indicates a shift of the resource allocation span

-  bits provide the resource allocation

where the value of P depends on the number of DL resource blocks as indicated in subclause [7.1.6.1] of [3]

- TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

- Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

- HARQ process number - 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

- Scrambling identity– 1 bit as defined in subclause 6.10.3.1 of [2]

- SRS request – [0-1] bit. This field can only be present for TDD operation and if present is defined in subclause 8.2 of [3]

In addition, for transport block 1:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

In addition, for transport block 2:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

- HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

- MUST interference presence and modulation (This field is present only when the UE is configured for MUST operation) – 0 or 2 bits. The field is defined in Table 5.3.3.1.5C-3, where the interfering antenna port is in {7,8} excluding the antenna port for transmission. The interfering antenna port has the same scrambling identity as indicated in the "Scrambling identity" field.

- SRS timing offset – 3 bits as defined in [3]. This field is present only when the DCI format is used for scheduling PDSCH in a LAA SCell and the UE is configured with uplink transmission on the LAA SCell.

- Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

If both transport blocks are enabled, the number of layers equals two; transport block 1 is mapped to codeword 0; and transport block 2 is mapped to codeword 1. Antenna ports 7 and 8 are used for spatial multiplexing.

In case one of the transport blocks is disabled, the number of layers equals one; the transport block to codeword mapping is specified according to Table 5.3.3.1.5-2; and the antenna port for single-antenna port transmission is according to Table 5.3.3.1.5B-1.

Table 5.3.3.1.5B-1: Antenna port for single-antenna port transmission (one transport block disabled)

| New data indicator of the disabled transport block | Antenna port |
| --- | --- |
| 0 | 7 |
| 1 | 8 |

If the number of information bits in format 2B carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one zero bit shall be appended to format 2B.

##### 5.3.3.1.5C	Format 2C

The following information is transmitted by means of the DCI format 2C:

- Carrier indicator – 0 or 3 bits. The field is present according to the definitions in [3].

- Resource allocation header (resource allocation type 0 / type 1) – 1 bit as defined in subclause 7.1.6 of [3]

If downlink bandwidth is less than or equal to 10 PRBs, there is no resource allocation header and resource allocation type 0 is assumed.

- Resource block assignment:

- For resource allocation type 0 as defined in subclause 7.1.6.1 of [3]

- bits provide the resource allocation

- For resource allocation type 1 as defined in subclause 7.1.6.2 of [3]

-  bits of this field are used as a header specific to this resource allocation type to indicate the selected resource blocks subset

- 1 bit indicates a shift of the resource allocation span

-  bits provide the resource allocation

where the value of P depends on the number of DL resource blocks as indicated in subclause [7.1.6.1] of [3]

- TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

- Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

- HARQ process number - 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

- Antenna port(s), scrambling identity and number of layers – 3 bits as specified in Table 5.3.3.1.5C-1 where nSCID is the scrambling identity for antenna ports 7 and 8 defined in subclause 6.10.3.1 of [2], or 4 bits as specified in Table 5.3.3.1.5C-2 where nSCID is the scrambling identity for antenna ports 7, 8, 11 and 13 defined in subclause 6.10.3.1 of [2] when higher layer parameter dmrs-tableAlt is set to 1, or 1 bit as specified in Table 5.3.3.1.5C-6 where nSCID is the scrambling identity for antenna ports 7 and 8 defined in subclause 6.10.3.1 of [2] when higher layer parameter semiOpenLoop is configured.

- SRS request – [0-1] bit. This field can only be present for TDD operation and if present is defined in subclause 8.2 of [3]

In addition, for transport block 1:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

In addition, for transport block 2:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

- HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

- MUST interference presence, antenna port, and modulation (this field is only present when the UE is configured for MUST operation) – 2 bits when higher layer parameter dmrs-tableAlt is not configured or is set to 0, 4 bits when higher layer parameter k-max is set to 1 and dmrs-tableAlt =1, or 6 bits when k-max is set to 3 and dmrs-tableAlt =1. For the 2 and 6 bit fields, two bits are defined for each interfering antenna port in Table 5.3.3.1.5C-3, where a single interfering antenna port is in {7,8} excluding the antenna port for transmission, and multiple interfering antenna ports are in {7,8,11,13} excluding the antenna ports for transmission. For the 6 bit field, the two or four LSB are reserved in the case of two or one interfering antenna port, respectively. Each pair of the used bits in 6 bit field from MSB to LSB is associated with one interfering antenna port in increasing order of port index. For the 4 bit field, two MSB are defined for interference presence and antenna port in Table 5.3.3.1.5C-4 where the single interfering antenna port is one of {7,8,11,13} excluding the antenna port for transmission, and two LSB are defined for interference modulation in 5.3.3.1.5C-5. The interfering antenna port(s) have the same scrambling identity and OCC length as indicated in the "Antenna port(s), scrambling identity and number of layers" field.

- SRS timing offset – 3 bits as defined in [3]. This field is present only when the DCI format is used for scheduling PDSCH in a LAA SCell and the UE is configured with uplink transmission on the LAA SCell.

- Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

If both transport blocks are enabled; transport block 1 is mapped to codeword 0; and transport block 2 is mapped to codeword 1. When higher layer parameter semiOpenLoop is configured, antenna ports 7 and 8 are used for spatial multiplexing.

In case one of the transport blocks is disabled; the transport block to codeword mapping is specified according to Table 5.3.3.1.5-2. For the single enabled codeword, Value = 4, 5, 6 in Table 5.3.3.1.5C-1 or Value = 12, 13,14 in Table 5.3.3.1.5C-2 are only supported for retransmission of the corresponding transport block if that transport block has previously been transmitted using two, three or four layers, respectively. When higher layer parameter semiOpenLoop is configured, antenna ports 7 and 8 are used for transmit diversity.

If the number of information bits in format 2C carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one zero bit shall be appended to format 2C.

Table 5.3.3.1.5C-1: Antenna port(s), scrambling identity and number of layers indication

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- | --- | --- |
| Value | Message | Value | Message |
| 0 | 1 layer, port 7, nSCID=0 | 0 | 2 layers, ports 7-8, nSCID=0 |
| 1 | 1 layer, port 7, nSCID=1 | 1 | 2 layers, ports 7-8, nSCID=1 |
| 2 | 1 layer, port 8, nSCID=0 | 2 | 3 layers, ports 7-9 |
| 3 | 1 layer, port 8, nSCID=1 | 3 | 4 layers, ports 7-10 |
| 4 | 2 layers, ports 7-8 | 4 | 5 layers, ports 7-11 |
| 5 | 3 layers, ports 7-9 | 5 | 6 layers, ports 7-12 |
| 6 | 4 layers, ports 7-10 | 6 | 7 layers, ports 7-13 |
| 7 | Reserved | 7 | 8 layers, ports 7-14 |

Table 5.3.3.1.5C-2: Antenna port(s), scrambling identity and number of layers indication

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- | --- | --- |
| Value | Message | Value | Message |
| 0 | 1 layer, port 7, nSCID=0 (OCC=2) | 0 | 2 layer, port 7-8, nSCID=0 (OCC=2) |
| 1 | 1 layer, port 7, nSCID=1 (OCC=2) | 1 | 2 layer, port 7-8, nSCID=1 (OCC=2) |
| 2 | 1 layer, port 8, nSCID=0 (OCC=2) | 2 | 2 layer, port 7-8, nSCID=0 (OCC=4) |
| 3 | 1 layer, port 8, nSCID=1 (OCC=2) | 3 | 2 layer, port 7-8, nSCID=1 (OCC=4) |
| 4 | 1 layer, port 7, nSCID=0 (OCC=4) | 4 | 2 layer, port 11,13, nSCID=0 (OCC=4) |
| 5 | 1 layer, port 7, nSCID=1 (OCC=4) | 5 | 2 layer, port 11,13, nSCID=1 (OCC=4) |
| 6 | 1 layer, port 8, nSCID=0 (OCC=4) | 6 | 3 layer, port 7-9 |
| 7 | 1 layer, port 8, nSCID=1 (OCC=4) | 7 | 4 layer, port 7-10 |
| 8 | 1 layer, port 11, nSCID=0 (OCC=4) | 8 | 5 layer, port 7-11 |
| 9 | 1 layer, port 11, nSCID=1 (OCC=4) | 9 | 6 layer, port 7-12 |
| 10 | 1 layer, port 13, nSCID=0 (OCC=4) | 10 | 7 layers, ports 7-13 |
| 11 | 1 layer, port 13, nSCID=1 (OCC=4) | 11 | 8 layers, ports 7-14 |
| 12 | 2 layers, ports 7-8 | 12 | 3 layers, ports 7, 8,11, nSCID=0 (OCC=4) |
| 13 | 3 layers, ports 7-9 | 13 | 4 layers, ports 7, 8,11,13, nSCID=0 (OCC=4) |
| 14 | 4 layers, ports 7-10 | 14 | Reserved |
| 15 | Reserved | 15 | Reserved |

Table 5.3.3.1.5C-3: Content of MUST interference presence and modulation for an antenna port

| Bit field | Message |
| --- | --- |
| 00 | No interference presence |
| 01 | Interference is present with QPSK |
| 10 | Interference is present with 16QAM |
| 11 | Interference is present with 64QAM or 256QAM |

Table 5.3.3.1.5C-4: Content of MUST interference presence and antenna port

| Bit field | Message |
| --- | --- |
| 00 | No interference presence |
| 01 | First antenna port |
| 10 | Second antenna port |
| 11 | Third antenna port |

Table 5.3.3.1.5C-5: Content of MUST interference modulation

| Bit field | Message |
| --- | --- |
| 00 | QPSK |
| 01 | 16QAM |
| 10 | 64QAM |
| 11 | 256QAM |

Table 5.3.3.1.5C-6: Antenna port(s), scrambling identity and number of layers indication

| One Codeword:Codeword 0 enabled, Codeword 1 disabled Or Two Codewords:Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- |
| Value | Message |
| 0 | 2 layer, port 7-8, nSCID=0 |
| 1 | 2 layer, port 7-8, nSCID=1 |

##### 5.3.3.1.5D	Format 2D

The following information is transmitted by means of the DCI format 2D:

- Carrier indicator – 0 or 3 bits. The field is present according to the definitions in [3].

- Resource allocation header (resource allocation type 0 / type 1) – 1 bit as defined in subclause 7.1.6 of [3]

If downlink bandwidth is less than or equal to 10 PRBs, there is no resource allocation header and resource allocation type 0 is assumed.

- Resource block assignment:

- For resource allocation type 0 as defined in subclause 7.1.6.1 of [3]

- bits provide the resource allocation

- For resource allocation type 1 as defined in subclause 7.1.6.2 of [3]

-  bits of this field are used as a header specific to this resource allocation type to indicate the selected resource blocks subset

- 1 bit indicates a shift of the resource allocation span

-  bits provide the resource allocation

where the value of P depends on the number of DL resource blocks as indicated in subclause [7.1.6.1] of [3]

- TPC command for PUCCH – 2 bits as defined in subclause 5.1.2.1 of [3]

- Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2.

- HARQ process number - 4 bits if higher layer parameter dl-TTI-Length is configured for the cell, otherwise 3 bits (for cases with FDD primary cell not configured with EN-DC and higher layer parameter subframeAssignment-r15), 4 bits (for cases with TDD primary cell, or for cases with EN-DC with FDD primary cell and higher layer parameter subframeAssignment-r15 configured)

- Antenna port(s), scrambling identity and number of layers – 3 bits as specified in Table 5.3.3.1.5C-1 (or Table 5.3.3.1.5D-1 for two codewords when higher layer parameter qcl-Operation is set to 'typeC') where nSCID is the scrambling identity for antenna ports 7 and 8 defined in subclause 6.10.3.1 of [2], or 4 bits as specified in Table 5.3.3.1.5C-2 (or Table 5.3.3.1.5D-2 for two codewords when higher layer parameter qcl-Operation is set to 'typeC') where nSCID is the scrambling identity for antenna ports 7, 8, 11 and 13 defined in subclause 6.10.3.1 of [2] when higher layer parameter dmrs-tableAlt is set to 1, or 1 bit as specified in Table 5.3.3.1.5C-6 where nSCID is the scrambling identity for antenna ports 7 and 8 defined in subclause 6.10.3.1 of [2] when higher layer parameter semiOpenLoop is configured.

- SRS request – [0-1] bit. This field can only be present for TDD operation and if present is defined in subclause 8.2 of [3]

In addition, for transport block 1:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

In addition, for transport block 2:

- Modulation and coding scheme – 5 bits if higher layer parameter altMCS-Table is not configured, 6 bits otherwise, as defined in subclause 7.1.7 of [3]

- New data indicator – 1 bit

- Redundancy version – 2 bits

- PDSCH RE Mapping and Quasi-Co-Location Indicator – 2 bits as defined in subclauses 7.1.9 and 7.1.10 of [3]

- HARQ-ACK resource offset (this field is present when this format is carried by EPDCCH. This field is not present when this format is carried by PDCCH) – 2 bits as defined in subclause 10.1 of [3]. The 2 bits are set to 0 when this format is carried by EPDCCH on a secondary cell, or when this format is carried by EPDCCH on the primary cell scheduling PDSCH on a secondary cell and the UE is configured with PUCCH format 3 for HARQ-ACK feedback.

- MUST interference presence, antenna port, and modulation (this field is only present when the UE is configured for MUST operation) – 2 bits when higher layer parameter dmrs-tableAlt is not configured or is set to 0, 4 bits when higher layer parameter k-max is set to 1 and dmrs-tableAlt =1, 6 bits when k-max is set to 3 and dmrs-tableAlt =1. For the 2 and 6 bit fields, two bits are defined for each interfering antenna port in Table 5.3.3.1.5C-3, where a single interfering antenna port is in {7,8} excluding the antenna port for transmission, and multiple interfering antenna ports are in {7,8,11,13} excluding the antenna ports for transmission. For the 6 bit field, the two or four LSB are reserved in the case of two or one interfering antenna port, respectively. Each pair of the used bits in 6 bit field from MSB to LSB is associated with one interfering antenna port in increasing order of port index. For the 4 bit field, two MSB are defined for interference presence and antenna port in Table 5.3.3.1.5C-4 where the single interfering antenna port is one of {7,8,11,13} excluding the antenna port for transmission, and two LSB are defined for interference modulation in 5.3.3.1.5C-5. The interfering antenna port(s) have the same scrambling identity and OCC length as indicated in the "Antenna port(s), scrambling identity and number of layers" field.

- SRS timing offset – 3 bits as defined in [3]. This field is present only when the DCI format is used for scheduling PDSCH in a LAA SCell and the UE is configured with uplink transmission on the LAA SCell.

- Aperiodic zero-power CSI-RS resource indicator for PDSCH RE Mapping – 2 bits as defined in subclauses 7.1.9 and 7.2.7 of [3]. This field is present only when the UE is configured with csi-RS-ConfigZP-ApList.

If both transport blocks are enabled; transport block 1 is mapped to codeword 0; and transport block 2 is mapped to codeword 1. When higher layer parameter semiOpenLoop is configured, antenna ports 7 and 8 are used for spatial multiplexing.

In case one of the transport blocks is disabled; the transport block to codeword mapping is specified according to Table 5.3.3.1.5-2. For the single enabled codeword, Value = 4, 5, 6 in Table 5.3.3.1.5C-1 or Value = 12, 13,14 in Table 5.3.3.1.5C-2 are only supported for retransmission of the corresponding transport block if that transport block has previously been transmitted using two, three or four layers, respectively. When higher layer parameter semiOpenLoop is configured, antenna ports 7 and 8 are used for transmit diversity.

If the number of information bits in format 2D carried by PDCCH belongs to one of the sizes in Table 5.3.3.1.2-1, one zero bit shall be appended to format 2D.

### < Unchanged parts are omitted >