3GPP TSG RAN WG1 Meeting #95 	R1-1814276

# Spokane, USA, November 12th-16th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1205 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on resource exclusion procedure for V2X Phase 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  |  | Date: |  | 2018-11-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following agreements were made in RAN2. This document is to include the procedure of additional resource exclusion based on Option 1-1 in Section 14.1.1.6. - Agreemnt in RAN2#103 - For option1-1, RAN2 assumes PHY will indicate available resources to MAC after exclusion - Agreemnt in RAN2#103bis - We will send LS to RAN1 to ask whether option1-2 is considered as an additional resource reselection triggering condition or whether it is taken into account when the resource is selected but it will not trigger resource reselection procedure. We will also inform RAN2 agreement on option1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of additional resource exclusion procedure based on Option 1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The resource selection procedure of UE with limited TX capability will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### < Unchanged parts are omitted >

# 14 UE procedures related to Sidelink

### < Unchanged parts are omitted >

#### 14.1.1.6	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink transmission mode 4

When requested by higher layers in subframe n for a carrier, the UE shall determine the set of resources to be reported to higher layers for PSSCH transmission according to the following steps. Parameters  the number of sub-channels to be used for the PSSCH transmission in a subframe,  the resource reservation interval, and  the priority to be transmitted in the associated SCI format 1 by the UE are all provided by higher layers.  is determined according to Subclause 14.1.1.4B.

If partial sensing is not configured by higher layers then the following steps are used:

1)	A candidate single-subframe resource for PSSCH transmission  is defined as a set of  contiguous sub-channels with sub-channel x+j in subframe  where . The UE shall assume that any set of  contiguous sub-channels included in the corresponding PSSCH resource pool (described in 14.1.5) within the time interval  corresponds to one candidate single-subframe resource, where selections of  and  are up to UE implementations under  and , if  is provided by higher layers for , otherwise . UE selection of  shall fulfil the latency requirement. The total number of the candidate single-subframe resources is denoted by.

2)	The UE shall monitor subframes ,, …,  except for those in which its transmissions occur, where  if subframe n belongs to the set , otherwise subframe is the first subframe after subframe n belonging to the set . The UE shall perform the behaviour in the following steps based on PSCCH decoded and S-RSSI measured in these subframes.

3)	The parameter  is set to the value indicated by the i-th SL-ThresPSSCH-RSRP field in SL-ThresPSSCH-RSRP-List where .

4)	The set  is initialized to the union of all the candidate single-subframe resources. The set  is initialized to an empty set.

5)	The UE shall exclude any candidate single-subframe resource  from the set  if it meets all the following conditions:

-	the UE has not monitored subframe  in Step 2.

-	there is an integer j which meets  where j=0, 1, …, , , k is any value allowed by the higher layer parameter restrictResourceReservationPeriod and q=1,2,…,Q. Here,  if  and , where  if subframe n belongs to the set , otherwise subframe  is the first subframe belonging to the set  after subframe n; and  otherwise.

6)	The UE shall exclude any candidate single-subframe resource  from the set  if it meets all the following conditions:

-	the UE receives an SCI format 1 in subframe , and "Resource reservation" field and "Priority" field in the received SCI format 1 indicate the values  and , respectively according to Subclause 14.2.1.

-	PSSCH-RSRP measurement according to the received SCI format 1 is higher than .

-	the SCI format received in subframe or the same SCI format 1 which is assumed to be received in subframe(s)  determines according to 14.1.1.4C the set of resource blocks and subframes which overlaps with  for q=1, 2, …, Q and j=0, 1, …, . Here, if  and , where  if subframe n belongs to the set , otherwise subframe is the first subframe after subframe n belonging to the set ; otherwise .

7)	If the number of candidate single-subframe resources remaining in the set  is smaller than , then Step 4 is repeated with  increased by 3 dB.

8)	For a candidate single-subframe resource  remaining in the set, the metric  is defined as the linear average of S-RSSI measured in sub-channels x+k for  in the monitored subframes in Step 2 that can be expressed by  for a non-negative integer j if , and  for a non-negative integer j otherwise.

9)	The UE moves the candidate single-subframe resource  with the smallest metric  from the set  to . This step is repeated until the number of candidate single-subframe resources in the set  becomes greater than or equal to ,

10) When the UE is configured by upper layers to transmit using resource pools on multiple carriers, it shall exclude a candidate single-subframe resource  from  if the UE does not support transmission in the candidate single-subframe resource in the carrier under the assumption that transmissions take place in other carrier(s) using the already selected resources due to its limitation in the number of simultaneous transmission carriers, its limitation in the supported carrier combinations, or interruption for RF retuning time [10].

The UE shall report set  to higher layers.

If partial sensing is configured by higher layers then the following steps are used:

1)	A candidate single-subframe resource for PSSCH transmission  is defined as a set of  contiguous sub-channels with sub-channel x+j in subframe  where . The UE shall determine by its implementation a set of subframes which consists of at least  subframes within the time interval  where selections of  and  are up to UE implementations under  and , if  is provided by higher layers for , otherwise . UE selection of  shall fulfil the latency requirement and  shall be greater than or equal to the high layer parameter minNumCandidateSF. The UE shall assume that any set of  contiguous sub-channels included in the corresponding PSSCH resource pool (described in 14.1.5) within the determined set of subframes correspond to one candidate single-subframe resource. The total number of the candidate single-subframe resources is denoted by.

2)	If a subframe  is included in the set of subframes in Step 1, the UE shall monitor any subframe  if k-th bit of the high layer parameter gapCandidateSensing is set to 1. The UE shall perform the behaviour in the following steps based on PSCCH decoded and S-RSSI measured in these subframes.

3)	The parameter  is set to the value indicated by the i-th SL-ThresPSSCH-RSRP field in SL-ThresPSSCH-RSRP-List where .

4)	The set  is initialized to the union of all the candidate single-subframe resources. The set  is initialized to an empty set.

5)	The UE shall exclude any candidate single-subframe resource  from the set  if it meets all the following conditions:

-	the UE receives an SCI format 1 in subframe , and "Resource reservation" field and "Priority" field in the received SCI format 1 indicate the values  and , respectively according to Subclause 14.2.1.

-	PSSCH-RSRP measurement according to the received SCI format 1 is higher than .

-	the SCI format received in subframe or the same SCI format 1 which is assumed to be received in subframe(s)  determines according to 14.1.1.4C the set of resource blocks and subframes which overlaps with  for q=1, 2, …, Q and j=0, 1, …, . Here, if  and , where  is the last subframe of the  subframes , and  otherwise.

6)	If the number of candidate single-subframe resources remaining in the set  is smaller than , then Step 4 is repeated with  increased by 3 dB.

7)	For a candidate single-subframe resource  remaining in the set, the metric  is defined as the linear average of S-RSSI measured in sub-channels x+k for  in the monitored subframes in Step 2 that can be expressed by  for a non-negative integer j.

8)	The UE moves the candidate single-subframe resource  with the smallest metric  from the set  to . This step is repeated until the number of candidate single-subframe resources in the set  becomes greater than or equal to .

9) When the UE is configured by upper layers to transmit using resource pools on multiple carriers, it shall exclude a candidate single-subframe resource  from  if the UE does not support transmission in the candidate single-subframe resource in the carrier under the assumption that transmissions take place in other carrier(s) using the already selected resources due to its limitation in the number of simultaneous transmission carriers, its limitation in the supported carrier combinations, or interruption for RF retuning time [10].

The UE shall report set  to higher layers.

If transmission based on random selection is configured by upper layers and when the UE is configured by upper layers to transmit using resource pools on multiple carriers, the following steps are used:

- A candidate single-subframe resource for PSSCH transmission  is defined as a set of  contiguous sub-channels with sub-channel x+j in subframe  where . The UE shall assume that any set of  contiguous sub-channels included in the corresponding PSSCH resource pool (described in 14.1.5) within the time interval  corresponds to one candidate single-subframe resource, where selections of  and  are up to UE implementations under  and , if  is provided by higher layers for , otherwise . UE selection of  shall fulfil the latency requirement. The total number of the candidate single-subframe resources is denoted by.

- The set  is initialized to the union of all the candidate single-subframe resources. The set  is initialized to an empty set.

- The UE moves the candidate single-subframe resource  from the set  to .

- The UE shall exclude a candidate single-subframe resource  from  if the UE does not support transmission in the candidate single-subframe resource in the carrier under the assumption that transmissions take place in other carrier(s) using the already selected resources due to its limitation in the number of simultaneous transmission carriers, its limitation in the supported carrier combinations, or interruption for RF retuning time [10].

The UE shall report set  to higher layers.