3GPP TSG RAN Meeting #82	RP-182586

Sorrento, Italy, December 10 – 13, 2018

Agenda item:		9.6

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining aspects of NR Rel-15 UE features

Document for:		Discussion and Decision

# Introduction

RAN1#95 updated the layer 1 UE features list [1], but many cases are still lack decisions on whether the feature is mandatory or optional, and what are the minimum values the UE is required to report on each component. In this document we provide views on a number of unfinished UE features, and in addition we indicate the minimum supported values for selected parameters, which are deemed necessary to support upcoming NR deployments.

# Beam management in FR1 and FR2

RAN1#95 had a joint session with RAN4 on the topic of Beam Correspondence where the pending aspects have been clarified. RAN4#89 also agreed on requirements for Beam Correspondence, which then clarifies the main uncertainty from RAN1 and RAN4 points of view, as described in [2]. As for component 2 in 2-20, The beam correspondence is defined as per band capability (Type 1); thus, we can focus only on intra-band case. For intra-band carrier aggregation, the beam formulation both in downlink reception and uplink transmission are not independent among CCs. The same beam correspondence relationship always exits for the entire RF bandwidth per band of the UE, and hence UEs supporting beam correspondence need to support component 2 as well.

| # | FG | Components | Consequences if not supported by the UE | Note | RAN WG recommendation | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-20 | Beam correspondence | 1. Support Beam correspondence 2. When CA is configured, whether the same beam correspondence relationship for beam management is supported across CCs in the same band. Note: RAN4 to check the feasibility for component-2, e.g. intra-band or inter-band | Beam correspondence is not supported | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports | [Mandatory/optional] with capability signaling Component-2, candidate value: {Yes, No} | [Mandatory/optional] with capability signaling | FR2: Mandatory with capability signalling, which shall be set to ‘1’ Component 2: it is mandatory to indicate ‘Yes’ |

Regarding features 2-21 and 2-22, beam-based transmission schemes envisioned for FR2 can also be employed in FR1 deployments. As a result, the beam reporting features for beam refinement that are currently mandatory for FR2 should also be made mandatory for FR1. SRS-based beam refinement can be seen as an alternative to beam reporting, but has disadvantages such as poor cell edge performance, sensitivity to interference, and relatively poor performance in FDD. It is expected that beam reporting would be much more robust than beam selection based on SRS due to the significantly higher transmit power on the downlink. Simulation results showing the disadvantages of SRS even in TDD and especially for cell edge UEs are shown in the Annex. The finding justifies ensuring beam management features to be mandatory with the support of sufficiently large number of beams:

| # | FG | Components | Consequence if not supported by UE | RAN WG recommendation | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- |
| 2-21 | Periodic beam report | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot | No support of periodic L1-RSRP report | Mandatory with UE capability at least for FR2 FFS: for FR1 | Mandatory with UE capability for FR2 [Mandatory/optional] with UE capability for FR1 | Mandatory with UE capability for FR1&FR2 |
| 2-22 | Aperiodic beam report | 1. Support report on PUSCH | No support of aperiodic L1-RSRP report | Mandatory with UE capability at least for FR2 | Mandatory with UE capability for FR2 [Mandatory/optional] with UE capability for FR1 | Mandatory with UE capability for FR1&FR2 |

For 2-24 SSB/CSI-RS for beam measurement

- Component 1 defines how many SSB/CSI-RS 1Tx resources the UE can track within a duration of one slot, 8 SSBs within a slot suffices for 4 carriers in a CA setup (2 SSBs per slot per carrier), but is very limiting from CSI-RS configuration perspective.

- Component 1a is redundant if component 2 is covering both 1Tx and 2Tx and is defined sufficiently large

- Component 2 defines how many CSI-RS can be configured across all CCs altogether. Thus this would need to be sufficiently large for a CA-capable device to be able to be configured to operate in a CA setup.

- Component 2 is not needed for SSB, as the # of SSBs to support is directly derived from the # of carriers and the number of SSBs on those carriers.

For 2-31 Beam failure recovery

- Component 1 is essentially the same as component 2 of 2-24

- Component 2 is an extension of component 1 of 2-24, covering SSBs across time. Too limiting number corresponds to the UEs inability to actually operate CA.

- Component 3 seems redundant to component 1 and 2. The reason a particular SSB or CSI-RS is being measured (beam identification or PDCCH quality or beam management) should not have any relation to the UEs ability to measure the SSB or CSI-RS.

| # | FG | Components | Consequence if not supported by UE | RAN WG recommendation | Nokia comment |
| --- | --- | --- | --- | --- | --- |
| 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 1a. The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS 4. The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | RSRP measurement is not supported | Mandatory with capability signaling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signaling. Component-1a, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“not supported”, “1 only”, “3 only”, “both 1 and 3”} On FR2, UE is mandated to signal either “3 only” or “both 1 and 3” On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signaling. Component-4, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For both FR1 and FR2, UE is mandated to report at least 4 | Component 1a: Support Mc_1 =64 is mandatory for FR2 Component 2: Support MB_2 =8 is mandatory for FR1 Support MB_2 =64 is mandatory for FR2 Component 3: On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signaling. |
| 2-31 | Beam failure recovery | 1. Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2. Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3. Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. | Beam failure recovery is not supported | Component-1 candidate value set: {from 1 to 16} Component-2 candidate: {from 1 to 16} Component-3: Candidate value set is: {from 1 to 128} [UE is mandate to support at least 64.] | Component-1: FR1: at least 8 is mandatory FR2: at least 16 is mandatory Component-2: FR1: at least 8 is mandatory FR2: at least 16 is mandatory Component-3: FR1: at least 32 is mandatory FR2: at least 256 is mandatory |

2-29 Non-group based beam reporting; for group-based beam reporting, 2 values are always repored per reporting instance. It seems pointless to debate on the number of RSRP reports that can be non-grouped when the measurement support is anyway in place.

| # | FG | Components | Consequence if not supported by UE | RAN WG recommendation | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- |
| 2-29 | Non-group based beam reporting | 1. Support of non-group based RSRP reporting with N_max RSRP values reported | Non-group based beam reporting is not supported | candidate value set is {1, 2,4} | Mandatory with capability for both FR1 and FR2 candidate value set is {1, 2,4} | Minimum value for N_max = 4 for both FR1 and FR2 |

2-51 TRS: The UE should be able to be configured with as many TRS as there are beams in the cell. In case of CA, the number of TRS needed would be the number of TRS per cell * number of cells, and thus preferably the component 5 could be obsoleted and component 4 would be applicable to all cells supported for CA. UE-specific TRS has a scalability problem when larger number of users are in active state in a cell .

| # | FG | Components | Consequence if not supported by UE | RAN WG recommendation | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- |
| 2-51 | TRS (CSI-RS for tracking) | 2. TRS burst length (X), 3. Max # of TRS resource sets (per CC) UE is able to track simultaneously 4. Max # of TRS resource sets configured to UE per CC 5. Max # of TRS resource sets configured to UE across CCs | TRS is not supported | Component-1: candidate values set: {BWP, min(52,BWP), both} Mandatory with capability signaling Component-2: candidate values {1 , ”both 1 and 2”}. UE is mandated to report “both 1 and 2” Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and [8 or 16 or 32] for FR2. To be decided in plenary. Component-5: Candidate value set: {1 to 128} to be decided in plenary UE is mandated to report at least X for FR1 and Y for FR2. X and Y are to be decided in plenary. |  | Component-4: FR2: at least 64 is mandatory Component-5: FR1: at least 8*#CCs is mandatory FR2: at least 64*#CCs is mandatory |

Feature 2-30 deals with UL beam management, but the current description of its components makes it ambiguous whether the feature is applicable only to UL beam management at the UE or if it includes UL signals used to aid DL beam management as well. We propose the following clarification to eliminate the ambiguity and properly scope the feature:

| # | FG | Components | RAN1 recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- |
| 2-30 | Uplink beam management | 1 Support of SRS based beam management of the UE transmitted beam 2. Supported max number of SRS resource per set (SRS set use is configured as for beam management of the UE transmitted beam). 3. Supported max number of SRS resource sets (SRS set use is configured as for beam management of the UE transmitted beam). | Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} | [Mandatory/Optional] with capability signaling Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |

2-53 SRS: UE should be able to store the number of SRSs needed, and there should be no differentiation in UE capabilities. In principle UEs should support the maximum value supported by the specification, but if that is not considered reasonable, a smaller value can be considered.

| # | FG | Components | Consequence if not supported by UE | RAN WG recommendation | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- |
| 2-53 | SRS resources | 1. Maximum number of aperiodic SRS resources (configured to UE) per BWP 2. Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3. Maximum number of periodic SRS resources (configured to UE) per BWP 4. Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5. Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6. Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7. Maximum number of SRS port per resource | No more than one periodic and one aperiodic SRS resources per CC are supported and no SP-SRS is supported | Mandatory with capability signaling Component-1: candidate value: {from 1 , 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1 , 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 1 , 2, 4, 8, 16} } Component-6 candidate value: {1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} Support SP-SRS is mandatory with capability |  | Mandatory with capability, with the capability bit always set to 1. Component-1: At least 4 is mandatory Component-2: At least 4 is mandatory Component-3: At least 4 is mandatory Component-4: At least 4 is mandatory Component-5: At least 4 is mandatory Component-6: At least 4 is mandatory Component-7: At least the number of ports supported for PUSCH is mandatory |

RAN1#94 introduced a set of new features 2-59/60/61, and the proposed minimum values for the parameters are indicated below.

| # | FG | Components | Consequences if not supported by the UE | Note | RAN WG recommendation | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-59 | Configured spatial relations | Maximum number of configured spatial relations per CC for PUCCH and SRS | Only one configured spatial relation for UL signals can be supported | Note: Only applicable for FR2 | candidate value set: {4, 8, 16, 32, 64, 96} |  | UE is mandated to signal at least 64 |
| 2-60 | Active spatial relations | Maximum total number of {unique DL RS (except for aperiodic NZP CSI-RS) and SRS without spatial relation configured, and, TCI states available for DCI triggering of aperiodic NZP CSI-RS}, for indicating spatial domain transmit filter for PUCCH and SRS for PUSCH, per BWP per CC | Only one active spatial relation for UL control and data can be supported | Note: Only applicable for FR2 FFS whether DL RS in the active TCI states and the active spatial relation info can be different if 2-60 is set to 1 and if different whether or not new UE behavior is needed | Candidate value set: {1, 2, 4, 8, 14} |  | UE is mandated to signal at least 4 |
| 2-61 | Additional active spatial relation for PUCCH | Support one additional active spatial relation for PUCCH | No additional active spatial relation for control | Note: Only applicable if 2-20b is set to 1 | Mandatory with capability signaling | Mandatory with capability signaling | Note: Only applicable if 2-60 is set to 4 |

# Other MIMO features

The feature 2-3 ‘PDSCH MIMO layers” TSG-RAN decision was updated in RAN#80 [1] to state:

For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases above means mandatory with capability signaling. It is not expected that there is a signaling change resulting from this (i.e. signaling remains to be defined as {1, 2, 4, 8} in every band, including FR1 and FR2.

RAN#80 discussed also other cases like EN-DC, CA and if relaxations could be considered for these cases. Unfortunately, RAN#80 was not able to complete this discussion and RAN4 and RAN1 were tasked to continue the discussion before the September RAN#81 meeting. In our view on the bands where 4Rx is mandatory, UE should also be mandated to support 4 MIMO layers for all operating scenarios including EN-DC, CA, non-ENDC/non-CA. On all other bands, UE should be mandated to support 2 MIMO layers. We propose that the feature 2-3 ‘PDSCH MIMO layers” is further updated as follows to avoid any confusion on the applicability of 4 MIMO layer UE requirement in different scenarios:

Update the feature 2-3 ‘PDSCH MIMO layers” in the RAN1 NR UE Feature as follows

| 2-3 | PDSCH MIMO layers | 1. Supported maximal number of MIMO layers | Candidate values: {1,2,4,8} | For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers for all operating scenarios (EN-DC, CA, non-EN-DC/non-CA) in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2 for all operating scenarios (EN-DC, CA, non-EN-DC/non-CA). Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signaling. It is not expected that there is a signaling change (i.e. signaling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
| --- | --- | --- | --- | --- |

RAN1 was tasked to clarify 2-38 whether it depends on SRS Tx switch. CSI report without PMI should be supportable by all UEs, and regardless of the ability to do SRS Tx switch or not, the gNB can determine the precoder according to the information at its disposal. We would warmly welcome SRS Tx switch to be mandatory, if applicable, but assume this to be a commercial decision requiring HW implementation and something beyond 3GPP.

| # | FG | Components | Consequence if not supported by UE | TSG-RAN decision | Nokia comment |
| --- | --- | --- | --- | --- | --- |
| 2-38 | CSI report without PMI | Support CSI report without PMI | CSI report without PMI is not supported | Mandatory/Optional] with capability signaling | Mandatory with capability signaling, which shall be set to ‘1’. Note: This does not mandate SRS Tx port switch 2-55 |

# BWP features

As discussed in [3], the features 6-2 and 6-3, seems to be the next implementation delta from the mandatory feature 6-1. And we think that it should be clarified what scenarios should be supported with this optional feature. To our understanding, feature 6-2 supports 2 dedicated BWPs plus initial active BWP, i.e. switching DCI-based switching between 3 BWPs total is supported by feature 6-2 (see RAN2 configuration Option 1 in Section 2). It should be clarified that numerology of initial active BWP should be the same as numerology of 2 UE-specific RRC configured DL BWPs for feature 6-2 and similarly for features 6-3. Feature 6-4 maintains the capability for switching between BWPs of different SCS.

| # | FG | Components |
| --- | --- | --- |
| 6-2 | Type A BWP adaptation with same numerology | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP CORESET#0 (if CORESET#0 is present) and SSB for PCell/PScell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |
| 6-3 | Type B BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP CORESET#0 (if CORESET#0 is present) and SSB for PCell/PScell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |
| 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP CORESET#0 (if CORESET#0 is present) and SSB for PCell/PScell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |

# Rate matching features

The dynamic CORESET rate matching should be supported as mandatory to be able to properly exploit the resource sharing between control and data:

| # | FG | Components | RAN WG recommendation | TSG RAN Decision | Nokia comment |
| --- | --- | --- | --- | --- | --- |
| 5-26 | Semi-static rate-matching resource set configuration for DL | - Bitmap 1/2/3 - controlResourceSet |  | Mandatory with capability signaling |  |
| 5-27 | Dynamic rate-matching resource set configuration for DL | - Bitmap 1/2/3 |  | Optional with capability signaling |  |
| 5-27a | Dynamic rate-matching control resource set for DL |  | [Mandatory with capability signaling] |  | Remove square brackets: Mandatory with capability signaling |

# References

[1] R1- 1814277, RAN1 NR UE features, Nov 2018

[2] RP-182580, UE support for NR Beam Correspondence

[3] R1-1808953, On remaining details of BWPs and CA, Nokia, Nokia Shanghai Bell