3GPP TSG RAN meeting #82										RP-182591

Sorrento, Italy, December 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.2

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_eMTC4 |  |  |  |  |
| Unique ID | 750059 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-172811 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2018 | Performance part: 12/2018 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 100% | Testing part: |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Johan BERGMAN |
|  | Company | Ericsson |
|  | Email | johan.bergman@ericsson.com |
| Rapporteur | Name | Alberto RICO |
|  | Company | Qualcomm |
|  | Email | albertor@qti.qualcomm.com |
| Rapporteur | Name | FANG Huiying |
|  | Company | ZTE |
|  | Email | fang.huiying@zte.com.cn |
| Rapporteur | Name | YE Qiaoyang |
|  | Company | Intel |
|  | Email | qiaoyang.ye@intel.com |
| Rapporteur | Name | SUN Feifei |
|  | Company | Samsung |
|  | Email | feifei.sun@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

#### 2.4.1	Agreements

Core part:

RAN4#88bis discussed A-MPR requirement for PUSCH sub-PRB allocation and made the following agreements (from WF in R4-1813809):

- Companies are encouraged to provide all A-MPR result for remaining NS value for Cat-M1 and Cat-M2

- A-MPR table need further clarify the carrier center frequency range where the A-MPR value apply

- Use the Wideband/Narrowband index to indicate the UE LO frequency for Cat-M2 device; if use Narrowband index, it means the uplink transmission bandwidth start at the NB index.

RAN4#89 discussed A-MPR requirement for PUSCH sub-PRB allocation and made the following agreement:

- Agreed CR (R4-1816600) for Cat-M1 and CR (R4-1815772) for Cat-M2

RAN4#89 discussed edge PRB restriction for flexible starting PUSCH PRB and made the following agreement:

- Agreed LS (R4-1816249) to RAN1 to restrict the edge PRB resource for flexible PRB

Based on the above-mentioned LS from RAN4, RAN1#95 agreed to adopt the restriction requested by RAN4.

Performance part:

RAN4#88bis discussed UE RRM performance requirements and made the following agreement:

- Agreed on a first set of test cases corresponding to phase 1 in agreed test case list in R4-1811704

- Agree applicability rules for new measurement gap patterns

RAN4#89 discussed UE RRM performance requirements and made the following agreement:

- Agreed on all remaining test cases corresponding to phase 2 in agreed test case list in R4-1811704

RAN4#88bis discussed UE demodulation and CSI requirements and made the following agreements:

- Alignment of simulation results of PDSCH with 64QAM and high-speed scenario

- Endorsed draft 36.101 CRs for PDSCH demodulation requirements and CQI reporting test requirements

RAN4#89 discussed UE demodulation and CSI requirements and made the following agreements:

- Alignment of simulation results of PDSCH with 64QAM and high-speed scenario

- Agreed on the 36.101 CRs for PDSCH demodulation requirements and CQI reporting test requirements

RAN4#88bis discussed BS demodulation requirements and made the following agreements:

- Alignment of simulation results of PUSCH with high speed scenario

- Agreed on the 36.104/36.141 CRs for PUSCH demodulation for high speed scenario

- Endorsed the 36.104/36.141 CRs for PUSCH demodulation with sub-PRB transmission

RAN4#89 discussed BS demodulation requirements and made the following agreements:

- Alignment of simulation results of PUSCH with sub-PRB transmission

- Agreed on the 36.104/36.141 CRs for PUSCH demodulation with sub-PRB transmission

CR pack:

RAN4 CR pack is submitted in RP-182365 to RAN#82.

#### 2.4.2	Remaining Open issues

None

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

## 4.	References

RAN4#88bis

57 submitted contributions (for details see agenda item 6.5 in Tdoc list)

RAN4#89

57 submitted contributions (for details see agenda item 6.5 in Tdoc list)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template