3GPP TSG RAN meeting #82										RP-182593

Sorrento, Italy, December 10-13, 2018

## Status Report to TSG

Agenda item:			10.4.1

| WI / SI Name | Additional MTC enhancements for LTE |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_eMTC5 |  |  |  |  |
| Unique ID | 800083 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181878 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2019 | Performance part: 06/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 25% | Performance Part: 0% | Testing part: |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Johan BERGMAN |
|  | Company | Ericsson |
|  | Email | johan.bergman@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

To RAN1#94bis, 102 contributions were submitted (for details see agenda item 6.2.1 in Tdoc list). An updated WI work plan was provided and presented in R1-1810184. After the meeting, a RAN1 agreement summary document was provided in R1-1811736.

To RAN1#95, 91 contributions were submitted (for details see agenda item 6.2.1 in Tdoc list). After the meeting, an updated RAN1 agreement summary document was provided in R1-1813795.

RAN1 discussed UE-group wake-up signal, with the following agreements:

Agreement UE grouping is based on at least UE ID or some function of UE ID Agreement The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled - Performance impact on legacy WUS should be carefully considered Agreement Group WUS is based on at least legacy WUS and UE-group ID. Agreement Configuration of group WUS is at least signaled in SI Agreement A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS

Agreement For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options - TDM - FDM - single-seq CDM - single-seq CDM+TDM - single-seq CDM+FDM - FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs. Agreement For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options - single-seq CDM - FDM - single-seq CDM+TDM - single-seq CDM+FDM Note: At least the maximum number of UE groups should be considered. Agreement The number of UE groups is configurable and broadcasted in SIB. - FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO Agreement UE group ID is used as a parameter to generate WUS UE group sequence(s). Agreement One group WUS is designed as a single sequence Agreement Further study false detection (cross/auto correlation) performance properties for the following designs: - legacy WUS + cover codes, - legacy WUS + shifted scrambling codes, - legacy WUS + phase shift + cover code + scrambling bits - Including combinations of phase shift, cover code, and/or scrambling bits Other designs are not precluded. Agreement Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM. - No new gap higher layer signaling will be introduced for TDM

RAN1 discussed transmission in preconfigured UL resources, with the following agreements:

Agreement In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed): - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell RSRP changes (serving cell refers the cell that the UE is camping on) - FFS Other attributes: - Neighbour cell RSRP change - TDOA of >=2 eNBs - TA History - Subscription based UE differentiation - Others not precluded (for example, attributes that need to be considered for high mobility UEs) Note that UE power consumption should be taken into account for the FFS attributes Agreement Dedicated preconfigured UL resource is defined as an PUSCH resource used by a single UE - PUSCH resource is time-frequency resource - Dedicated PUR is contention-free Contention-free shared preconfigured UL resource (CFS PUR) is defined as an PUSCH resource simultaneously used by more than one UE - PUSCH resource is at least time-frequency resource - CFS PUR is contention-free Contention-based shared preconfigured UL resource (CBS PUR) is defined as an PUSCH resource simultaneously used by more than one UE - PUSCH resource is at least time-frequency resource - CBS PUR is contention-based (CBS PUR may require contention resolution) Agreement In IDLE mode, HARQ is supported for transmission in dedicated PUR - A single HARQ process is supported, - FFS whether more than one HARQ processes are supported - FFS: The design of the corresponding MPDCCH search space Agreement In idle mode, dedicated PUR is supported. - Support for CFS PUR is FFS. - Support for CBS PUR is FFS. Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported. Agreement For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria Agreement Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported.

Agreement For dedicated PUR in idle mode, the UE may skip UL transmissions. - FFS: Resource release mechanism - FFS: Whether or not to support mechanism to disable skipping by eNB Agreement The following evaluation parameters for CFS PUR link level simulation are agreed. Parameter; Value/Description. Channel model; ETU 1Hz. Resource Bandwidth(s); 1, 2, 3 or 6 PRBs (contiguous) for eMTC Proponent to provide details on resource allocation Same bandwidth for all UEs. Traffic Pattern; Fixed (N UEs transmit in the same resource) or (Optional) Bursty (N UEs may transmit in the same resource, with activity factor p for each of them) Note: For all traffic models, it is assumed that a UE has a dedicated DMRS Note: Final decision can be based on the results for both fixed and bursty traffic model. Number of repetitions in a resource; To be declared by proponent. Transport Block Size and Modulation Order; To be declared by proponent. Number of eNB Rx antennas; 2 as baseline, other values are not precluded. Metrics; BLER before HARQ Number of resources (PRB x subframes) used by the technique to serve a given number of UEs Baseline for comparison is dedicated PUR. MCL; 164, 144, 124dB MCL. UE grouping; All UEs in the same MCL level.. Processing Receiver at eNB; No SIC/joint decoding of UEs is assumed. MMSE single user decoding. Not NOMA.. DMRS collision; Each company to provide details on their DMRS assumption. Channel Estimation CFO Timing Error Power Imbalance; Realistic Uniform +/-50 Hz, +/-100 Hz (optional) +/-0.5 CP uniform, +/-1 CP uniform (optional) Uniform +/- [5 or 10] dB. The evaluated proposals should be within the scope of the eMTC WI. Agreement If multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and single HARQ process - FFS: if multi-TB grant is enabled/supported Agreement In idle mode, at least the following TA validation attributes are supported: - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell RSRP changes (serving cell refers the cell that the UE is camping on) - Based on RSRP measurement definition in existing Rel-15 TS36.214 Include in LS to RAN2, RAN4 to consider in their work. Agreement The UE can be configured to use at least these TA validation attributes: - Time Alignment Timer for idle mode - Serving cell RSRP changes - Note: the configuration shall support disabling of the TA validation attributes Include in LS to RAN2, RAN4 For further study: TA validation attributes: - Subscription based UE differentiation (or Stationary indication in held in subscription) - Cell specific indication where TA is valid within that cell Agreement Include in LS to RAN2, RAN4: RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode. Agreement For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in MPDCCH search space - FFS: Details on the search space (for example USS, CSS) Agreement For dedicated PUR in idle mode, upon successful decoding by eNB of a PUR transmission, the UE can expect an explicit ACK FFS: if ACK is sent on MPDCCH (layer 1) and/or PDSCH (layer 2/3) Include in LS to RAN2, RAN4. Agreement For dedicated PUR in idle mode, upon unsuccessful decoding by eNB of a PUR transmission, the UE can expect - an UL GRANT for retransmission on the MPDCCH, or - FFS: a NACK, or - FFS: no explicit ACK Include in LS to RAN2, RAN4. Agreement Send LS to RAN2, RAN4 to consider in their work. All the agreements with ‘include in LS to RAN2, RAN4’ for NB-IoT and eMTC should be captured in this LS. The LS is endorsed in R1-1813778

RAN1 discussed scheduling of multiple DL/UL transport blocks, with the following agreements:

Agreement Confirm the working assumption that - For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, each transport block corresponds to a unique HARQ process. Agreement For CE mode A, the maximum number of scheduled transport blocks with one single DCI is [8] for CE mode A for UL. For CE mode A, the maximum number of scheduled transport blocks with one single DCI is [8] for CE mode A for DL. Agreement The maximum number of scheduled transport blocks with one single DCI for CE mode A for either UL or DL is fixed to [8] Working Assumption For CE mode B, the maximum number of scheduled transport blocks with one single DCI is 4 in the UL, and 4 for the DL. Agreement The maximum number of scheduled transport blocks with one single DCI for CE mode B for either UL or DL is fixed to 4 (working assumption) For further study until next meeting: How to efficiently handle retransmissions when scheduling multiple DL/UL transport blocks. Agreement The UE should only monitor one DCI size in the UE specific search space For further consideration: Optimization of the DCI size and the impact of aspects including number of transport blocks, scheduling pattern (interleaving and scheduling gap), resource assignment, modulation and coding scheme, retransmissions. Agreement Using one DCI to schedule multiple TBs for SC-MTCH is supported, and it is configured and enabled per SC-MTCH via SC-PTM configuration message in SC-MCCH. FFS the maximum number of TBs can be scheduled by one DCI. Agreement For CE mode A, HARQ ACK/NACK feedback bundling or multiplexing on PUCCH can be enabled or disabled by [RRC and/or DCI], when multiple DL transport blocks are assigned by a single DCI. If the network does not enable it, each TB has its own separately encoded HARQ ACK/NACK feedback, i.e., no HARQ ACK/NACK feedback bundling or multiplexing. - RAN1 further compare the performance between HARQ ACK/NACK feedback bundling and multiplexing and down-select between the two options. For CE mode B, further study if there is a benefit for HARQ ACK/NACK feedback bundling or multiplexing on PUCCH. If there is a benefit identified, same configuration principle as CE mode A can be applied, i.e., this feature can be enabled or disabled by [RRC and/or DCI].

Agreement For CEmodeA, the maximum number of scheduled transport blocks with one single DCI is 8 in the UL, 8 in the DL Agreement The following working assumption is confirmed For CEmodeB, the maximum number of scheduled transport blocks with one single DCI is 4 in the UL, 4 in the DL Agreement For both UL and DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI. - Above applies only for valid subframes within the consecutive resource allocation in time - FFS: Whether time gaps between two TBs is also supported For future meetings in Rel-16 eMTC: Further consider which Rel-14/15 features that should be possible to configure together with scheduling of multiple DL-UL transport blocks. Agreement For the DL unicast for a UE, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs: - Frequency-hopping flag, PMI confirmation (TM6-specific), Precoding information (TM6-specific), DM-RS scrambling / antenna ports (TM9-specific), Downlink assignment index (TDD-specific), PUCCH power control - FFS: MCS, RV, Resource assignment, Number of PDSCH repetitions Agreement For the UL unicast, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs: - Frequency-hopping flag, TPC command - FFS: MCS, RV, Resource assignment, Repetition number, Downlink assignment index (TDD-specific) For next meeting For unicast when multi-TBs are scheduled, companies are encouraged to bring in DCI designs which can support - scheduling of initial and retransmission TBs within one DCI - scheduling of initial TBs within one DCI, and retransmissions with one DCI - scheduling of initial TBs within one DCI, and retransmission can only be scheduled by individual DCI RAN1 tries to make a decision on which case is specified in the next meeting based on the trade-off between DCI overhead and scheduling flexibility comparisons of the three cases. Agreement For the case of single DCI scheduling multiple transport blocks with repetitions, scheduling of transport blocks repetitions is down selected between: - Option 1: All the repetitions for one transport block are contiguously scheduled in valid UL/DL subframes - Option 2: The repetitions for one transport block are interleaved with repetitions of all the other transport blocks - Option 3: Option 1 and 2 are supported and eNB configures among them. Agreement The maximum number of TBs for multicast is 8.

RAN1 discussed coexistence of LTE-MTC with NR, with the following agreements:

For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements of subcarrier and resource block alignment to help determine whether the gains are significant enough to motivate the impacts. For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements of resource configuration (e.g. reservation of LTE-MTC resources) to help determine whether the gains are significant enough to motivate the impacts. Agreement RAN1 clarifies that the enhancements introduced by the WI objective on usage of the LTE DL control channel region for MPDCCH/PDSCH transmissions to LTE-MTC UEs do not only apply to LTE-MTC stand-alone deployments but also to the case when LTE-MTC is deployed within an NR carrier. For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements through CRS reduction to help determine whether the gains are significant enough to motivate the impacts. - Also consider backwards compatibility aspects. For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements of frequency hopping to help determine whether the gains are significant enough to motivate the impacts. - Also consider backwards compatibility aspects.

Agreement - RAN1 continues to study the following techniques for performance improvements of resource block alignment until the next meeting: - Puncturing of resource elements at the outlying subcarrier - Rate-matching around the outlying subcarrier - Exploitation of a portion of the NR guard band (this would also require RAN4 study) - RAN1 continues to consider all combinations of LTE-MTC system bandwidths and NR system bandwidths when discussing potential co-existence performance improvements. Agreement - RAN1 continues to study the following techniques for performance improvements of LTE-MTC resource allocation until the next meeting: - Resource reservation at symbol level/slot level/subframe level/subcarrier level - Whether the resource reservation is dynamic or semi-static (if supported) - Whether and how to support LTE-MTC transmission in a portion of the subframe - Impact of resource reservation to legacy UEs - Whether LTE-MTC transmission is postponed or dropped in reserved resources Agreement RAN1 studies LTE-MTC transmission outside the legacy LTE system bandwidth (for reduced NR reserved resource cost for CRS, SIB1-BR, paging, etc.) until the next meeting

RAN1 discussed support of quality report in Msg3, with the following agreements:

Agreement For CE mode A (PRACH CE level 0, 1), the downlink channel quality is down-selected among the following in RAN1#95: - CQI - The repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% - Support both CQI and repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% Agreement For CE Mode B, the downlink channel quality reported in Msg3 is denoted as the repetition number that the UE recommends to achieve a hypothetical MPDCCH decoding BLER of 1% Agreement CRS may be used as the reference signal for measurement of DL quality metric for measurement report in Msg3. Agreement Enabling of DL quality report is indicated in SIB.

Agreement For CE mode A (PRACH CE level 0, 1), the downlink channel quality is the repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% Agreement DL quality report is transmitted via higher layer signaling, e.g. MAC CE or RRC message

RAN1 discussed MPDCCH performance improvements, with the following agreements:

Agreement CRS for improving channel estimation on MPDCCH in idle mode is supported. FFS the details. Agreement For localized MPDCCH, the following mappings are supported between CRS and DMRS: - Predefined mapping - Mapping based on CSI report FFS the details. Agreement For distributed MPDCCH, mapping between CRS and DMRS is defined in specifications. - FFS the details (e.g. number of PRBs over which mapping applies, fixed or configurable) Agreement Power offset between CRS and DMRS antenna ports of MPDCCH is signaled by eNB. FFS the signaling details (e.g. provisioning for default power offset value).

Agreement For distributed MPDCCH, predefined mapping between CRS ports and MPDCCH DMRS ports is supported. Agreement For localized/distributed MPDCCH, the predefined mapping between CRS ports and MPDCCH ports is based on precoder cycling across time and frequency domain. FFS the details. Agreement Reuse existing LTE Rel-8 codebooks for precoding of MPDCCH is the starting point. - Unless performance gains are idenfied further optimization on top of Rel-8 codebook will not be introduced Agreement For UE in Connected/Idle mode, the power offset between CRS and DMRS antenna ports of MPDCCH is indicated by SIB. Agreement - For UE in Idle mode, the configuration of CRS for MPDCCH performance improvement is indicated via SIB/SI. FFS the details of configuration. - For UE in Connected mode, the configuration of CRS for MPDCCH performance improvement is indicated via UE-specific RRC signalling. FFS the details of configuration.

RAN1 discussed CE mode A and B improvements for non-BL UEs, with the following agreements:

Agreement Dual-layer transmission is not supported for non-BL UEs in CE mode B Email discussion for evaluation parameters for dual layer transmission simulations - Until 22th of October (Changhwan Park, LGE) Agreement CSI-RS based CSI feedback is not supported for non-BL UEs at least in CE mode B

Agreement from RAN1 email discussion - T-put performance metric based on Rank 1 and Rank 2 will be shown by Hull-curve against SNR according to the simulation parameters given in Table 1 - The SNR range is to be discussed, aiming to conclude in RAN1#95 - In proposing the SNR range, companies are encouraged to check the WID and perform appropriate analysis considering relevant aspects related to non-BL & CE mode A Table 1: Link-level simulation parameters for Dual-layer transmission for non-BL UEs in CE mode A Parameter; Value. Frequency; 700MHz. Channel; EPA with low correlation according to Table B.5.2-2 in TS 36.104. eNB Antenna configurations; 4 Tx, Cross-polarization: +/-45 degrees. UE configurations; Speed: 1km/h. 2 Rx with X-polarized: 0/+90 degrees. Traffic load; Full Buffer. Transmission scheme; TM9 with fixed rank. PDSCH; 6 RBs with 1, 2, 4, and 8 repetitions. corReceiver; Non-Ideal DMRS channel estimation and interference estimation. LMMSE-IRC receiver. Overhead; 2 symbols for DL CCHs, 2 CRS ports and 2 DM-RS ports. CSI/Precoding; No rank adaptation (Rank 1 and Rank 2) Fixed CQI and feedback wideband PMI with periodicity of 10msec (Channel reciprocity property in TDD system can be used for DL PMI adjustment). Rate control; Target 10% BLER after 1st transmission. Cyclic prefix; Normal CP.

Agreement Companies are encouraged to evaluate DL performance with CSI-RS based CSI feedback for non-BL UE in CE mode A including by comparing downlink throughput performances based on - CRS-based CSI feedback - The current CSI-RS based CSI feedback (detailed configuration of CSI-RS is up to company, including new CSI-RS design, if any) under the simulation assumption in Table 1 used for the performance comparison between Single- and Dual-layer transmissions with the following updates - Rank 1 - CQI adaptions are enabled - Periodic CSI feedback Mode 1-1 is applied with periodicity of 10msec - Aperiodic CSI feedback

RAN1 discussed use of LTE control channel region for DL transmission, with the following agreements:

Agreement Support PDSCH broadcast transmission in LTE control channel region Agreement PDSCH are rate-matched in a backward compatible manner on all available OFDM symbols. FFS on RE mapping details.

Agreement LTE control channel can be used for MPDCCH that is used to schedule broadcast PDSCH channel Agreement For MPDCCH transmission in LTE control region, part of the MPDCCH are mapped into the LTE DL control region. Agreement For PDSCH transmission in LTE control region, rate-matching is done using legacy mapping then wrap around starting with OFDM symbol 0.

RAN1 discussed use of RSS for measurement improvements, with the following agreements:

Agreement In using RSS for measuring RSRP of neighbour cells, the following parameters are signalled to the UE for each cell: - ce-rss-periodicity-config: RSS periodicity {160, 320, 640, 1280} ms - ce-rss-duration-config : RSS duration {8, 16, 32, 40} subframes - ce-rss-freqPos-config: RSS frequency location (lowest physical resource block number) - ce-rss-timeOffset-config: RSS time offset in number of radio frames - ce-rss-powerBoost-config : RSS power offset relative to LTE CRS {0, 3, 4.8, 6} dB Agreement The parameters related in the use of RSS for measuring RSRP of cells are signalled in the SI. Agreement Send an LS to RAN4: - Request to evaluate the benefits and feasilibity of using RSS for RSRP measurements compared to using LTE CRS - For measurement purposes, assess the benefits and feasibility of linking RSS to CRS LS is endorsed in R1-1811934

#### 2.1.2	Remaining Open issues

Specify air interface physical layer aspects of the following improvements for BL/CE UEs:

- Specify support for UE-group wake-up signal (WUS)

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast

- Conclude on additional potential specification enhancement for improving the performance of coexistence of LTE-MTC with NR

- Specify quality report in MSG3 at least for EDT

- Specify MPDCCH performance improvement by using CRS at least for connected mode

- Specify CE mode A and B improvements for non-BL UEs

- Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs

- Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS

## 2.2	RAN2

#### 2.2.1	Agreements

To RAN2#103bis, 55 contributions were submitted (for details see agenda items 12.1, 12.2.3 and 12.2.4 in Tdoc list). A WI work plan was provided and presented in R2-1814350. After the meeting, a RAN2 agreement summary document was provided in R2-1815744.

To RAN2#104, 66 contributions were submitted (for details see agenda items 12.1, 12.2.3 and 12.2.4 in Tdoc list). After the meeting, an updated RAN2 agreement summary document was provided in R2-1818633.

RAN2 discussed mobile-terminated (MT) early data transmission (EDT) jointly with NB-IoT, with the following agreements:

RAN2#103bis agreements: RAN2 intends to support MT-initiated EDT for both CP and UP solutions. The intention to use MT-EDT is for user data, i.e. not for NAS signalling. RAN2#104 agreements: MT EDT are evaluated at least based on battery life, network resource efficiency, security, reliability and potential impact on core network. MT-EDT is intended for DL data which can be transmitted in one transport block. Use cases that require DL data transmission with or without UL data transmission as a response should be supported for MT-EDT. - [104#49][eMTC & NB-IoT R16] MT EDT (Huawei) To evaluate the solution options based on the agreed criteria (Huawei) Intended outcome: Email discussion report to next meeting Deadline: Thursday 2019-02-07

RAN2 discussed UE-group wake-up signal jointly with NB-IoT, with the following agreements:

RAN2#103bis agreements: The aim of UE grouping for WUS is reducing the false alarm probability. At least UE_ID based grouping is supported for UE-Group based WUS. This doesn’t exclude other options. Further discuss whether the following are supported: - Service based grouping - DRX/eDRX based grouping - Gap based grouping. - Coverage based grouping RAN2#104 agreements: Further discuss the benefit and feasibility of using service based parameters for grouping in addition to UE-ID. Can discuss group distribution further, including Rel-15/16 mechanism interaction, once we know more about number of groups and more about the grouping solution (e.g. service based parameters). RAN2 will decide on the UE to WUS group mapping. - [104#42][eMTC & NB-IoT R16] Analyse the benefit and feasibility of service based group WUS solutions (Nokia) Companies can provide solutions, with benefit analysis (e.g. based on reference traffic model) and feasibility. Intended outcome: email discussion report Deadline: Thursday 2019-02-07

RAN2 discussed transmission in preconfigured uplink resources (PUR) jointly with NB-IoT, with the following agreements:

RAN2#103bis agreements: Transmission in dedicated preconfigured uplink resources in IDLE mode is supported for UEs with a valid timing advance. Initially we will focus on dedicated preconfigured uplink resources in idle mode - Shared resources can also be discussed RAN2#104 agreements: The eNB configures the dedicated preconfigured uplink resources via RRC dedicated signaling. Methods for eNB to obtain information used to help configuring the dedicated preconfigured uplink resource to the UE is FFS. Periodic D-PUR with duration is supported FFS if one shot D-PUR is supported. Release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS. - [104#43][eMTC & NB-IoT R16] To progress on D-PUR (Sierra Wireless) Primary scope is to progress the FFS captured for D-PUR Secondary scope, to progress further details on solutions addressing the agreements Intended outcome: email discussion report Deadline: Thursday 2019-02-07

RAN2 discussed scheduling multiple DL/UL transport blocks jointly with NB-IoT, with the following agreements:

RAN2#103bis agreements: Working assumption: For MTCH in SC-PTM, configuration for multiple scheduling is transmitted in MCCH. Backwards compatibility is FFS. SPS is not supported for NB-IoT in Release 16. Enhancements for “SR with SPS for BSR” can be considered. RAN2#104 agreements: RAN2 intends to support separate/shared SC-MTCH transmission. Multiple TB scheduling is supported for UEs in connected mode. It is FFS if it is supported for EDT. UEs in connected mode are configured with multiple TB scheduling via dedicated RRC signalling assuming that a new DCI format is introduced. This is pending RAN1 agreement. UEs report capability to indicate support of multiple TB scheduling in connected mode.

RAN2 discussed quality report in Msg3, with the following agreements:

RAN2#103bis agreements: Quality report in Msg3 is introduced for EDT. FFS for non-EDT. RAN2#104 agreements: RAN2 waits for progress on the discussion of MT-EDT before deciding on whether channel quality report in Msg3 is introduced for EDT. Channel quality report in Msg3 is introduced for non-EDT. - [104#50][eMTC R16] Quality report in Msg3 (Huawei) To progress the discussion on: - which narrowband(s) to measure and report - how to provide the report, i.e. MAC or RRC Intended outcome: Email discussion report to next meeting Deadline: Thursday 2019-02-07

RAN2 discussed MPDCCH performance improvement using CRS, with the following agreements:

RAN2#103bis agreements: eNB enables CRS for improving channel estimation on MPDCCH via broadcasted RRC signalling. RAN2#104 agreements: CRS for improving channel estimation on MPDCCH is configured via SIB2. This agreement can be revisited if RAN2 identifies that it would be more reasonable to configure it via SIB1-BR. It is FFS whether configuration would implicitly mean to enable the feature. UEs report capability to indicate support of using CRS for improving MPDCCH channel estimation. It is FFS if enabling/disabling is needed for the UE.

RAN2 discussed improvements for non-BL UEs, with the following agreements:

RAN2#103bis agreements: RAN2 intends to support CMAS/ETWS for non-BL UEs in CE mode in connected mode.

RAN2 discussed stand-alone deployment, with the following agreements:

RAN2#103bis agreements: The standalone deployment supports legacy operations for legacy BL UEs or UEs in CE mode. LTE control channel region for DL transmission is supported for Rel-16 BL UEs and UEs in CE mode.

RAN2 discussed mobility enhancements, with the following agreements:

RAN2#104 agreements: An LS is sent (in R2-1818662) to inform RAN1 that the overhead introduced by signalling agreed parameters per cell is excessive, and ask which parameters are carrier specific for potential optimization. Relaxation of RRM measurements for UEs using WUS: RAN2 will wait for RAN4 to progress.

#### 2.2.2	Remaining Open issues

Specify air interface higher layer aspects of the following improvements for BL/CE UEs:

- Specify support for UE-group wake-up signal (WUS)

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast

- Conclude on additional potential specification enhancement for improving the performance of coexistence of LTE-MTC with NR

- Specify quality report in MSG3 at least for EDT

- Specify MPDCCH performance improvement by using CRS at least for connected mode

- Specify CE mode A and B improvements for non-BL UEs

- Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs

- Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS

- Specify support for mobile-terminated (MT) early data transmission (EDT)

- Specify relaxations of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs

## 2.3	RAN3

## 2.4	RAN4

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

## 4.	References

RAN1#94bis

102 contributions (for details see agenda item 6.2.1 in Tdoc list)

RAN1#95

91 contributions (for details see agenda item 6.2.1 in Tdoc list)

RAN2#103bis

55 contributions (for details see agenda items 12.1, 12.2.3 and 12.2.4 in Tdoc list)

RAN2#104

66 contributions (for details see agenda items 12.1, 12.2.3 and 12.2.4 in Tdoc list)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template