3GPP TSG-RAN WG2 Meeting #104	R2-1819186

# Spokane, , USA, 12 - 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1672 | rev | - | Current version: | 13.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | n1PUCCH-AN-CS-ListP1-r13 ASN.1 error correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh_b5C-Core |  |  |  |  |  |  | Date: |  | 2018-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In release 13 we created -r13 version of PUCCH-Config. During the procedure we combined n1PUCCH-AN-CS-ListP1-r11 and n1PUCCH-AN-CS-List-r10 into n1PUCCH-AN-CS-r13. Unfortunately -r13 asn.1 structure mandates to configure both of -r10 and -r11 IEs if either of parameter needs to be configured. It should be possible to configure any of the parameter separately (or together). This needs to be corrected to allow separately configuring these parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explain the changes: - UE supporting txDiv-PUCCH1b-ChSelect needs to support PUCCH-ConfigDedicated-v13c0 in order to be able to configure the feature without mandating r10 part of PUCCH configuration to be configured (i.e. n1PUCCH-AN-CS-List-r13). Impact analysis Impacted functionality: PUCCH configuration for PCell/PSCell/SCells. Inter-operability: - If the network is implemented according to the CR and the UE supporting txDiv-PUCCH1b-ChSelect is not it is possible that there is misunderstanding between UE and NW what is current RRC configuration. If UE does not support txDiv-PUCCH1b-ChSelect there is no interoperability problem. - If the UE supporting txDiv-PUCCH1b-ChSelect is implemented according to the CR and the network is not it is possible that there is misunderstanding between UE and NW what is current RRC configuration. If UE does not support txDiv-PUCCH1b-ChSelect there is no interoperability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One cannot configure n1PUCCH-AN-CS-List and n1PUCCH-AN-CS-ListP1 separately in release 13 (and later) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 36.331 CR 3736 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 4.3.4.22	txDiv-PUCCH1b-ChSelect-r11

This field defines whether the UE supports transmit diversity for PUCCH format 1b with channel selection if the UE supports carrier aggregation and two-AntennaPortsForPUCCH-r10. UE supporting txDiv-PUCCH1b-ChSelect shall support configuration of PUCCH-ConfigDedicated-v13c0.