3GPP TSG-CT Meeting #83	CP-190170

# Shenzhen, P. R. China; 18th – 19th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.274 | CR | 1940 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to Configuration Transfer Tunnel over S10 and N26 for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-03-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enhancements to the Configuration Transfer Tunnel procedure are required to support TNL Address discovery via the S1 interface or via inter-system signalling for EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ the support of the Configuration Transfer Tunnel procedure is extended to the N26 interface for TNL address discovery of a candidate en-gNB via inter-system signalling. 2/ The Configuration Configuration Transfer Tunnel procedure is extended (for the S10 and N26 interfaces) with a new en-gNB target type for TNL address discovery of a candidate en-gNB via the S1 interface or via inter-system signalling. In addition, for discovery of a candidate en-gNB via the S1 interface, a new Connected Target eNodeB ID IE is defined to encode the ID of the target eNodeB connected to the target en-gNB when known by the source eNodeB. Also extensions are defined to enable the Configuration Transfer Tunnel procedure to include a new EN-DC Container and to allow the receiving GTP entity to identify the type of container included in the message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TNL Address discovery for EN-DC is not supported across the S10 and N26 interfaces. Misalignments with RAN requirements and procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.0, 7.3.18, 8.48, 8.51 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.413 CR 1662 TS 36.300 CR 1229 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.1.0	Message Type

GTP defines a set of messages between two associated EPC network elements. The messages to be used shall be as defined in Table 6.1-1.

Table 6.1-1: Message types for GTPv2

| Message Type value (Decimal) | Message |  |  |  |  | Reference | Initial | Triggered |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Reserved |  |  |  |  |  |  |  |
| 1 | Echo Request |  |  |  |  |  | X |  |
| 2 | Echo Response |  |  |  |  |  |  | X |
| 3 | Version Not Supported Indication |  |  |  |  |  |  | X |
| 4 to 16 | Reserved for S101 interface |  |  |  |  | TS 29.276 [14] |  |  |
| 17 to 24 | Reserved for S121 interface |  |  |  |  | TS 29.276 [14] |  |  |
| 25 to 31 | Reserved for Sv interface |  |  |  |  | TS 29.280 [15] |  |  |
|  | SGSN/MME/ TWAN/ePDG to PGW (S4/S11, S5/S8, S2a, S2b) |  |  |  |  |  |  |  |
| 32 | Create Session Request |  |  |  |  |  | X |  |
| 33 | Create Session Response |  |  |  |  |  |  | X |
| 36 | Delete Session Request |  |  |  |  |  | X |  |
| 37 | Delete Session Response |  |  |  |  |  |  | X |
|  | SGSN/MME/ePDG to PGW (S4/S11, S5/S8, S2b) |  |  |  |  |  |  |  |
| 34 | Modify Bearer Request |  |  |  |  |  | X |  |
| 35 | Modify Bearer Response |  |  |  |  |  |  | X |
|  | MME to PGW (S11, S5/S8) |  |  |  |  |  |  |  |
| 40 | Remote UE Report Notification |  |  |  |  |  | X |  |
| 41 | Remote UE Report Acknowledge |  |  |  |  |  |  | X |
|  | SGSN/MME to PGW (S4/S11, S5/S8) |  |  |  |  |  |  |  |
| 38 | Change Notification Request |  |  |  |  |  | X |  |
| 39 | Change Notification Response |  |  |  |  |  |  | X |
| 42 to 63 | For future use |  |  |  |  |  |  |  |
| 164 | Resume Notification |  |  |  |  |  | X |  |
| 165 | Resume Acknowledge |  |  |  |  |  |  | X |
|  | Messages without explicit response |  |  |  |  |  |  |  |
| 64 | Modify Bearer Command (MME/SGSN/ TWAN/ePDG to PGW – S11/S4, S5/S8, S2a, S2b) |  |  |  |  |  | X |  |
| 65 | Modify Bearer Failure Indication (PGW to MME/SGSN/ TWAN/ePDG – S5/S8, S11/S4, S2a, S2b) |  |  |  |  |  |  | X |
| 66 | Delete Bearer Command (MME/SGSN to PGW – S11/S4, S5/S8) |  |  |  |  |  | X |  |
| 67 | Delete Bearer Failure Indication (PGW to MME/SGSN – S5/S8, S11/S4)) |  |  |  |  |  |  | X |
| 68 | Bearer Resource Command (MME/SGSN/TWAN/ePDG to PGW – S11/S4, S5/S8, S2a, S2b) |  |  |  |  |  | X |  |
| 69 | Bearer Resource Failure Indication (PGW to MME/SGSN/TWAN/ePDG – S5/S8, S11/S4, S2a, S2b) |  |  |  |  |  |  | X |
| 70 | Downlink Data Notification Failure Indication (SGSN/MME to SGW – S4/S11) |  |  |  |  |  | X |  |
| 71 | Trace Session Activation (MME/SGSN/ TWAN/ePDG to PGW – S11/S4, S5/S8, S2a, S2b) |  |  |  |  |  | X |  |
| 72 | Trace Session Deactivation (MME/SGSN/ TWAN/ePDG to PGW – S11/S4, S5/S8, S2a, S2b) |  |  |  |  |  | X |  |
| 73 | Stop Paging Indication (SGW to MME/SGSN – S11/S4) |  |  |  |  |  | X |  |
| 74 to 94 | For future use |  |  |  |  |  |  |  |
|  | PGW to SGSN/MME/ TWAN/ePDG (S5/S8, S4/S11, S2a, S2b) |  |  |  |  |  |  |  |
| 95 | Create Bearer Request |  |  |  |  |  | X | X |
| 96 | Create Bearer Response |  |  |  |  |  |  | X |
| 97 | Update Bearer Request |  |  |  |  |  | X | X |
| 98 | Update Bearer Response |  |  |  |  |  |  | X |
| 99 | Delete Bearer Request |  |  |  |  |  | X | X |
| 100 | Delete Bearer Response |  |  |  |  |  |  | X |
|  | PGW to MME, MME to PGW, SGW to PGW, SGW to MME, PGW to TWAN/ePDG, TWAN/ePDG to PGW (S5/S8, S11, S2a, S2b) |  |  |  |  |  |  |  |
| 101 | Delete PDN Connection Set Request |  |  |  |  |  | X |  |
| 102 | Delete PDN Connection Set Response |  |  |  |  |  |  | X |
|  | PGW to SGSN/MME (S5, S4/S11) |  |  |  |  |  |  |  |
| 103 | PGW Downlink Triggering Notification |  |  |  |  |  | X |  |
| 104 | PGW Downlink Triggering Acknowledge |  |  |  |  |  |  | X |
| 105 to 127 | For future use |  |  |  |  |  |  |  |
|  | MME to MME, SGSN to MME, MME to SGSN, SGSN to SGSN, MME to AMF, AMF to MME (S3/S10/S16/N26) |  |  |  |  |  |  |  |
|  |  | S3 | S10 | S16 | N26 |  |  |  |
| 128 | Identification Request | X | X | X | X |  | X |  |
| 129 | Identification Response | X | X | X | X |  |  | X |
| 130 | Context Request | X | X | X | X |  | X |  |
| 131 | Context Response | X | X | X | X |  |  | X |
| 132 | Context Acknowledge | X | X | X | X |  |  | X |
| 133 | Forward Relocation Request | X | X | X | X |  | X |  |
| 134 | Forward Relocation Response | X | X | X | X |  |  | X |
| 135 | Forward Relocation Complete Notification | X | X | X | X |  | X |  |
| 136 | Forward Relocation Complete Acknowledge | X | X | X | X |  |  | X |
| 137 | Forward Access Context Notification | - | X | X | - |  | X |  |
| 138 | Forward Access Context Acknowledge | - | X | X | - |  |  | X |
| 139 | Relocation Cancel Request | X | X | X | X |  | X |  |
| 140 | Relocation Cancel Response | X | X | X | X |  |  | X |
| 141 | Configuration Transfer Tunnel | - | X | - | X |  | X |  |
| 142 to 148 | For future use |  |  |  |  |  |  |  |
| 152 | RAN Information Relay | X | - | X | - |  | X |  |
|  | SGSN to MME, MME to SGSN (S3) |  |  |  |  |  |  |  |
| 149 | Detach Notification |  |  |  |  |  | X |  |
| 150 | Detach Acknowledge |  |  |  |  |  |  | X |
| 151 | CS Paging Indication |  |  |  |  |  | X |  |
| 153 | Alert MME Notification |  |  |  |  |  | X |  |
| 154 | Alert MME Acknowledge |  |  |  |  |  |  | X |
| 155 | UE Activity Notification |  |  |  |  |  | X |  |
| 156 | UE Activity Acknowledge |  |  |  |  |  |  | X |
| 157 | ISR Status Indication |  |  |  |  |  | X |  |
| 158 | UE Registration Query Request |  |  |  |  |  | X |  |
| 159 | UE Registration Query Response |  |  |  |  |  |  | X |
|  | SGSN/MME to SGW, SGSN to MME (S4/S11/S3) SGSN to SGSN (S16), SGW to PGW (S5/S8) |  |  |  |  |  |  |  |
| 162 | Suspend Notification |  |  |  |  |  | X |  |
| 163 | Suspend Acknowledge |  |  |  |  |  |  | X |
|  | SGSN/MME to SGW (S4/S11) |  |  |  |  |  |  |  |
| 160 | Create Forwarding Tunnel Request |  |  |  |  |  | X |  |
| 161 | Create Forwarding Tunnel Response |  |  |  |  |  |  | X |
| 166 | Create Indirect Data Forwarding Tunnel Request |  |  |  |  |  | X |  |
| 167 | Create Indirect Data Forwarding Tunnel Response |  |  |  |  |  |  | X |
| 168 | Delete Indirect Data Forwarding Tunnel Request |  |  |  |  |  | X |  |
| 169 | Delete Indirect Data Forwarding Tunnel Response |  |  |  |  |  |  | X |
| 170 | Release Access Bearers Request |  |  |  |  |  | X |  |
| 171 | Release Access Bearers Response |  |  |  |  |  |  | X |
| 172 to 175 | For future use |  |  |  |  |  |  |  |
|  | SGW to SGSN/MME (S4/S11) |  |  |  |  |  |  |  |
| 176 | Downlink Data Notification |  |  |  |  |  | X |  |
| 177 | Downlink Data Notification Acknowledge |  |  |  |  |  |  | X |
| 179 | PGW Restart Notification |  |  |  |  |  | X |  |
| 180 | PGW Restart Notification Acknowledge |  |  |  |  |  |  | X |
|  | SGW to SGSN (S4) |  |  |  |  |  |  |  |
| 178 | Reserved. Allocated in earlier version of the specification. |  |  |  |  |  |  |  |
| 181 to 199 | For future use |  |  |  |  |  |  |  |
|  | SGW to PGW, PGW to SGW (S5/S8) |  |  |  |  |  |  |  |
| 200 | Update PDN Connection Set Request |  |  |  |  |  | X |  |
| 201 | Update PDN Connection Set Response |  |  |  |  |  |  | X |
| 202 to 210 | For future use |  |  |  |  |  |  |  |
|  | MME to SGW (S11) |  |  |  |  |  |  |  |
| 211 | Modify Access Bearers Request |  |  |  |  |  | X |  |
| 212 | Modify Access Bearers Response |  |  |  |  |  |  | X |
| 213 to 230 | For future use |  |  |  |  |  |  |  |
|  | MBMS GW to MME/SGSN (Sm/Sn) |  |  |  |  |  |  |  |
| 231 | MBMS Session Start Request |  |  |  |  |  | X |  |
| 232 | MBMS Session Start Response |  |  |  |  |  |  | X |
| 233 | MBMS Session Update Request |  |  |  |  |  | X |  |
| 234 | MBMS Session Update Response |  |  |  |  |  |  | X |
| 235 | MBMS Session Stop Request |  |  |  |  |  | X |  |
| 236 | MBMS Session Stop Response |  |  |  |  |  |  | X |
| 237 to 239 | For future use |  |  |  |  |  |  |  |
|  | Other |  |  |  |  |  |  |  |
| 240 to 247 | Reserved for Sv interface (see also types 25 to 31) |  |  |  |  | TS 29.280 [15] |  |  |
| 248 to 255 | For future use |  |  |  |  |  |  |  |

* * * Next Change * * * *

### 7.3.18	Configuration Transfer Tunnel

A Configuration Transfer Tunnel message shall be used to:

-	tunnel eNodeB Configuration Transfer messages from a source MME to a target MME over the S10 interface. The purpose of the eNodeB Configuration Transfer is to transfer information from an eNodeB to a target eNodeB or an en-gNB connected to a target eNB in unacknowledged mode (see 3GPP TS 36.413 [10]);

-	tunnel eNodeB Configuration Transfer messages from a source MME to a target AMF, or to tunnel Uplink RAN Configuration Transfer messages from a source AMF to a target MME, over the N26 interface. This is to transfer RAN configuration information between an eNodeB and a gNB in unacknowledged mode (see 3GPP TS 36.413 [10] and 3GPP TS 38.413 [84]).

Table 7.3.18-1 specifies the presence requirements and conditions of the IEs in the message.

Table 7.3.18-1: Information Elements in a Configuration Transfer Tunnel Message

| Information elements | P | Condition / Comment | IE Type | Ins. |
| --- | --- | --- | --- | --- |
| E-UTRAN Transparent Container / EN-DC Container | M | This IE shall contain: - the "SON Configuration Transfer" as specified in 3GPP TS 36.413 [10]. In this case, the Container Type shall be set to 3. or - the "EN-DC SON Configuration Transfer" as specified in 3GPP TS 36.413 [10]. In this case the Container Type shall be set to 5. (NOTE) | F-Container | 0 |
| Target eNodeB ID / en-gNB ID | M | This IE shall contain the ID of the target eNodeB or en-gNB. This IE shall encode a target en-gNB ID over the S10 or N26 interface during: - a TNL address discovery request of a candidate en-gNB via the S1 interface or via inter-system signalling. This IE shall encode a target eNodeB ID: - over the S10 interface, when no EN-DC SON Configuration Transfer IE is received from the source eNB; - over the S10 or N26 interface, during a TNL address discovery response of a candidate en-gNB via the S1 interface or via inter-system signalling. See subclauses 22.3.6.x and 22.3.6.y of 3GPP TS 36.300 [19]. | Target Identification | 0 |
| Connected Target eNodeB ID | CO | This IE shall be present during an TNL address discovery request of a candidate en-gNB via the S1 interface (see subclauses 22.3.6.x of 3GPP TS 36.300 [19]) if the source eNB provided the ID of a target eNB connected to the target en-gNB ID. | Target Identification | 1 |
| NOTE: An EN-DC Container shall only be sent towards MMEs known to support TNL address discovery of a candidate en-gNB via the S1 interface or via inter-system signalling (see subclauses 22.3.6.x and 22.3.6.y of 3GPP TS 36.300 [19]). |  |  |  |  |

* * * Next Change * * * *

## 8.48	Fully Qualified Container (F-Container)

Fully Qualified Container (F-Container) is coded as depicted in Figure 8.48-1.

All Spare bits are set to zeros by the sender and ignored by the receiver.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 | Type = 118 (decimal) |  |  |  |  |  |  |  |  |
|  | 2 to 3 | Length = n |  |  |  |  |  |  |  |  |
|  | 4 | Spare |  |  |  | Instance |  |  |  |  |
|  | 5 | Spare |  |  |  | Container Type |  |  |  |  |
|  | 6 to (n+4) | F-Container field |  |  |  |  |  |  |  |  |

Figure 8.48-1: Full Qualified Container (F-Container)

The F-Container field shall contain one of the following information, depending of the contents of the container transported by the specific GTP Information Element:

-	transparent copy of the corresponding IEs (see subclause 8.2.2):

-	the "Source to Target Transparent Container" or the "Target to Source Transparent Container" as specified in 3GPP TS 25.413 [33]; or

-	the "SON Configuration Transfer" as specified in 3GPP TS 36.413 [10] or "EN-DC SON Configuration Transfer" as specified in 3GPP TS 36.413 [10]; or

-	the "eNB Status Transfer Transparent Container" as specified in 3GPP TS 36.413 [10]; or

-	"Source BSS to Target BSS Transparent Container" or "Target BSS to Source BSS Transparent Container" as specified in 3GPP TS 48.018 [34] or 3GPP TS 25.413 [33], which contains the value part of the "Source BSS to Target BSS Transparent Container" IE or the value part of the "Target BSS to Source BSS Transparent Container" IE defined in 3GPP TS 48.018 [34], i.e. octets 3 to n, excluding octet 1 (Element ID) and octet 2, 2a (Length); or

-	transparent copy of the value part of the "NBIFOM Container" as specified in 3GPP TS 24.161 73].

-	transparent copy of the octets of the encoded OCTET STRING of the "Source to Target Transparent Container" or the "Target to Source Transparent Container" specified in 3GPP TS 36.413 [10] and 3GPP TS 38.413 [84]; or

-	transparent copy of the BSSGP RIM PDU as specified in 3GPP TS 48.018 [34]; or

-	the Packet Flow ID, Radio Priority, SAPI, PS Handover XID parameters as specified in figure 8.42-2.

NOTE 1:	Annex B.2 provides further details on the encoding of Generic Transparent Containers over RANAP, S1-AP and GTP. See also Annex C of 3GPP TS 36.413 [10] for further details on how the MME constructs the F-Container field from the Source to Target Transparent Container or Target to Source Transparent Container IEs received from S1-AP.

Container Type values are specified in Table 8.48-2.

# Table 8. 48-2: Container Type values

| Container Types | Values (Decimal) |
| --- | --- |
| Reserved | 0 |
| UTRAN Transparent Container | 1 |
| BSS Container | 2 |
| E-UTRAN Transparent Container | 3 |
| NBIFOM Container | 4 |
| EN-DC Container | 5 |
| <spare> | 6-255 |

NOTE 2:	For any other new future F-Container content types, new Container Type values may be needed, although use of RAT agnostic containers should be used whenever possible.

The BSS Container IE in the Bearer Context IE in Forward Relocation Request and Context Response messages is coded as depicted in Figure 8.48-3.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 6 | Spare |  |  |  | PHX | SAPI | RP | PFI |  |
|  | a | Packet Flow ID |  |  |  |  |  |  |  |  |
|  | b | SAPI |  |  |  | Spare | Radio Priority |  |  |  |
|  | c | XiD parameters length |  |  |  |  |  |  |  |  |
|  | d to n | XiD parameters |  |  |  |  |  |  |  |  |

Figure 8.48-3: BSS Container

The flags PFI, RP, SAPI and PHX in octet 6 indicate the corresponding type of parameter (Packet FlowID, Radio Priority, SAPI and PS handover XID parameters) shall be present in a respective field or not. If one of these flags is set to "0", the corresponding field shall not be present at all. The Spare bit shall be set to zero by the sender and ignored by the receiver.

If PFI flag is set, Packet Flow ID shall be present in Octet a.

If RP flag is set, Radio Priority shall be present in Octet b.

If SAPI flag is set, SAPI shall be present in Octet b.

If PHX flag is set:

- XiD parameters length is present in Octet c.

- XiD parameters are present in Octet d to n.

* * * Next Change * * * *

## 8.51	Target Identification

### 8.51.1	General

The Target Identification information element is coded as depicted in Figure 8.51-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 | Type = 121 (decimal) |  |  |  |  |  |  |  |  |
|  | 2 to 3 | Length = n |  |  |  |  |  |  |  |  |
|  | 4 | Spare |  |  |  | Instance |  |  |  |  |
|  | 5 | Target Type |  |  |  |  |  |  |  |  |
|  | 6 to (n+4) | Target ID |  |  |  |  |  |  |  |  |

Figure 8.51-1: Target Identification

Target Type values are specified in Table 8.51-1.

# Table 8.51-1: Target Type values and their meanings

| Target Types | Values (Decimal) |
| --- | --- |
| RNC ID | 0 |
| Macro eNodeB ID | 1 |
| Cell Identifier | 2 |
| Home eNodeB ID | 3 |
| Extended Macro eNodeB ID | 4 |
| gNodeB ID | 5 |
| Macro ng-eNodeB ID | 6 |
| Extended ng-eNodeB ID | 7 |
| en-gNB ID | 8 |
| <spare> | 9 to 255 |

* * * Next Change * * * *

### 8.51.3	Macro eNodeB ID

The Target Type is Macro eNodeB ID for:

-	handover to E-UTRAN Macro eNodeB;

-	RAN Information Relay towards E-UTRAN; and

-	a TNL address discovery response of a candidate en-gNB via the S1 interface or via inter-system signalling (see subclauses 22.3.6.x and 22.3.6.y of 3GPP TS 36.300 [19]).

In this case the Target ID field shall be coded as depicted in Figure 8.51-2.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 6 | MCC digit 2 |  |  |  | MCC digit 1 |  |  |  |  |
|  | 7 | MNC digit 3 |  |  |  | MCC digit 3 |  |  |  |  |
|  | 8 | MNC digit 2 |  |  |  | MNC digit 1 |  |  |  |  |
|  | 9 | Spare |  |  |  | Macro eNodeB ID |  |  |  |  |
|  | 10 to 11 | Macro eNodeB ID |  |  |  |  |  |  |  |  |
|  | 12 to 13 | Tracking Area Code (TAC) |  |  |  |  |  |  |  |  |

Figure 8.51-2: Target ID for Type Macro eNodeB

The Macro eNodeB ID consists of 20 bits. Bit 4 of Octet 9 is the most significant bit and bit 1 of Octet 11 is the least significant bit. The coding of the Macro eNodeB ID is the responsibility of each administration. Coding using full hexadecimal representation (binary, not ASCII encoding) shall be used.

* * * Next Change * * * *

### 8.51.5	Extended Macro eNodeB ID

The Target Type is Extended Macro eNodeB ID for:

-	handover to E-UTRAN Macro eNodeB;

-	RAN Information Relay towards E-UTRAN; and

-	a TNL address discovery response of a candidate en-gNB via the S1 interface or via inter-system signalling (see subclauses 22.3.6.x and 22.3.6.y of 3GPP TS 36.300 [19]).

In this case the Target ID field shall be coded as depicted in Figure 8.51-4.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 6 | MCC digit 2 |  |  |  | MCC digit 1 |  |  |  |  |
|  | 7 | MNC digit 3 |  |  |  | MCC digit 3 |  |  |  |  |
|  | 8 | MNC digit 2 |  |  |  | MNC digit 1 |  |  |  |  |
|  | 9 | SMeNB | Spare |  | Extended Macro eNodeB ID |  |  |  |  |  |
|  | 10 to 11 | Extended Macro eNodeB ID |  |  |  |  |  |  |  |  |
|  | 12 to 13 | Tracking Area Code (TAC) |  |  |  |  |  |  |  |  |

Figure 8.51-4: Target ID for Type Extended Macro eNodeB

The Extended Macro eNodeB ID consists of 21 bits. The coding of the Macro eNodeB ID is the responsibility of each administration. Coding using full hexadecimal representation (binary, not ASCII encoding) shall be used.

If the SMeNB flag is not set the Extended Macro eNodeB ID contains a Long Macro eNodeB ID with a length of 21 Bits. Bit 5 of Octet 9 is the most significant bit and bit 1 of Octet 11 is the least significant bit.

If the SMeNB flag is set the Extended Macro eNodeB ID contains a Short Macro eNodeB ID with a length of 18 Bits. Bits 5 to 3 of Octet 9 shall be set to 0 by the sender and shall be ignored by the receiver. Bit 2 of Octet 9 is the most significant bit and bit 1 of Octet 11 is the least significant bit.

* * * Next Change * * * *

### 8.51.x	en-gNB ID

The Target Type is an en-gNB ID for:

-	a TNL address discovery request of a candidate en-gNB via the S1 interface or via inter-system signalling (see subclauses 22.3.6.x and 22.3.6.y of 3GPP TS 36.300 [19]).

In this case, the Target ID field shall be coded as depicted in Figure 8.51.x-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 6 | MCC digit 2 |  |  |  | MCC digit 1 |  |  |  |  |
|  | 7 | MNC digit 3 |  |  |  | MCC digit 3 |  |  |  |  |
|  | 8 | MNC digit 2 |  |  |  | MNC digit 1 |  |  |  |  |
|  | 9 | 5TAC | ETAC | en-gNB ID Length |  |  |  |  |  |  |
|  | 10 to 13 | en-gNB ID |  |  |  |  |  |  |  |  |
|  | p to (p+1) | Tracking Area Code (TAC) |  |  |  |  |  |  |  |  |
|  | q to (q+2) | 5GS Tracking Area Code (TAC) |  |  |  |  |  |  |  |  |

Figure 8.51.x-1: Target ID for Type en-gNB ID

The en-gNB ID Length field, in bits 1 to 6 of octet 9, indicates the length of the en-gNB ID in number of bits.

Bits 7 and 8 of octet 9 shall be encoded as follows:

-	Bit 7 - ETAC: when set to "1", octets p to (p+1) shall be present and encode a TAC (EPS); otherwise, octets p to (p+1) shall be absent.

-	Bit 8 - 5TAC: when set to "1", octets q to (q+2) shall be present and encode a 5GS TAC; otherwise, octets q to (q+2) shall be absent.

The en-gNB ID consists of 22 to 32 bits. The en-gNB ID shall be coded as the gNodeB ID field of the gNodeB ID in subclause 8.51.7.

When present, the TAC shall consist of 2 octets and shall be coded as the TAC field of the Macro eNodeB ID in subclause 8.51.3.

When present, the 5GS TAC shall consist of 3 octets and shall be coded as the 5GS TAC field of the gNodeB ID in ubclause 8.51.7.

* * * End of Changes * * * *