3GPP TSG RAN meeting #84										RP-190983

Newport Beach, USA, June 3-6, 2019

Status Report to TSG

Agenda item:			9.4.8

| WI / SI Name | 5G V2X with NR sidelink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | 5G_V2X_NRSL |  |  |  |  |
| Unique ID | 830078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190766 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 03/2020 | Performance part: 09/2020 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 30% | Performance Part: 0% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Hanbyul Seo |
|  | Company | LG Electronics |
|  | Email | hanbyul.seo@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

282 contributions were submitted to RAN1#96bis. 278 contributions were submitted to RAN1#97.

2.1.1	Agreements

RAN1#96bis

For sidelink design, RAN1 made the following agreements:

- Agreements on channel coding for PSCCH/PSSCH

  - Polar code adopted for Rel-15 NR DCI is applied to PSCCH.

  - LDPC codes used for Rel-15 NR DL-SCH is applied to a transport block delivered by PSSCH.

- Agreements on RX UE assumption for PSCCH decoding

  - The starting symbol and the number of symbols for a PSCCH are assumed to be known to the receiving UE before decoding the PSCCH.

- Agreements on SCI payload size for evaluation

  - For the purpose of evaluation of PSCCH design, RAN1 assumes 60 bits, 90 bits, 120 bits as the total SCI sizes including 24 bits CRC.

    - Other sizes are not precluded.

- Agreements on modulation order for PSCCH

  - QPSK is used for PSCCH.

- Working assumption on TB number for PSSCH

  - Transmission of 1 TB with up to 2 layers in a PSSCH is supported.

- Agreements on multiplexing of PSCCH/PSSCH and PSFCH

  - At least for transmission perspective of a UE in a carrier, at least TDM between PSCCH/PSSCH and PSFCH is allowed for a PSFCH format for sidelink in a slot.

    - FFS the details of the corresponding PSFCH format

    - FFS whether it is also applicable from system/resource pool perspective or not

    - i.e., in this case, there is no simultaneous transmission of PSCCH and PSFCH and there is no simultaneous transmission of PSSCH and PSFCH.

    - FFS FDM between PSCCH/PSSCH and a PSFCH format which uses last symbol(s) available for sidelink in a slot

    - FFS TDM/FDM between PSCCH/PSSCH and other PSFCH format(s), if supported, which is/are different from the PSFCH format which uses last symbol(s) available for sidelink in a slot

- Agreements on sidelink HARQ feedback in groupcast

  - Confirm the following working assumption:

    - Working assumption:

      - When HARQ feedback is enabled for groupcast, support (options as identified in RAN1#95):

        - Option 1: Receiver UE transmits only HARQ NACK

        - Option 2: Receiver UE transmits HARQ ACK/NACK

  - Note: RAN1 has not concluded the respective applicability of option 1 vs. option 2 yet

  - An LS was sent to RAN2 (R1-1905906)

- Agreements on PSFCH resource of RX UE in groupcast

  - In HARQ feedback for groupcast,

    - When Option 1 is used for a groupcast transmission, it is supported

      - All the receiver UEs share a PSFCH

      - FFS: a subset of the receiver UEs share a PSFCH

      - FFS: all or a subset of receiver UEs share a pool of PSFCH.

    - When Option 2 is used for a groupcast transmission, it is supported

      - Each receiver UE uses a separate PSFCH for HARQ ACK/NACK.

      - FFS: all or a subset of receiver UEs share a PSFCH for ACK transmission and another PSFCH for NACK transmission

    - FFS on which entity and how to allocate PSFCH resource to the receiver UE(s)

    - FFS whether or not to additionally support a mixture of option 1 and option 2 for a groupcast transmission

  - Note: Each PSFCH is mapped to a time, frequency, and code resource.

- Working assumption on TX-RX distance based sidelink HARQ feedback for groupcast

  - Regarding the use of TX-RX geographical distance and/or RSRP in determining whether to send HARQ feedback for groupcast

    - Support at least the use of TX-RX geographical distance

      - FFS whether or not to additionally use L1-RSRP

  - Companies are encouraged to perform additional evaulations/analysis

- Agreements on PSFCH resource (pre)configuration

  - It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s)

    - N is configurable, with the following values

      - 1

      - At least one more value >1

        - FFS details

    - The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled

  - HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool

- Agreements on sidelink CSI measurement

  - Support at least Sidelink CSI-RS for CQI/RI measurement

    - Sidelink CSI-RS is confined within the PSSCH transmission

- Agreements on dynamic/configured grant for mode 1

  - A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB.

  - A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions.

    - UE decides which TB to transmit in each of the occasions indicated by a given configured grant.

    - FFS: whether different transmissions of a TB can take place across multiple configured grants.

    - Other restrictions on what can be transmitted in a given configured grant (e.g., based on QoS, destination UE, etc.) are up to RAN2.

- Agreements on resource reservation for mode 2

  - NR V2X supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure

  - NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure

    - This functionality can be enabled/disabled by (pre-)configuration

  - FFS Standalone PSCCH transmissions for resource reservations are supported in NR V2X

- Agreements on PAPR assumption for NR SLSS

  - RAN1 will consider the design of NR SLSS where

    - S-PSS of LTE and NR candidates have similar PAPR.

    - S-SSS of LTE and NR candidates have similar PAPR.

    - SLSS design should take into consideration PAPR difference between S-PSS and S-SSS.

- Agreements on S-SSB structure

  - In NR V2X, S-SSB bandwidth is 11RBs.

    - PSBCH spans 11RBs.

  - The S-SSB is designed following combination 1.

    - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS

    - Two symbols are used for each of S-PSS and S-SSS, respectively.

- Agreements on PSBCH payload size for evaluation

  - For the evaluation of PSBCH performance, the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC.

For NR Uu controlling LTE sidelink, RAN1 made the following agreements:

- Agreements on consideration points for RRC/DCI based activation/release of LTE sidelink

  - Regarding RRC-based versus DCI-based activation/release of LTE sidelink SPS, RAN1 agrees to make the choice on the basis of at least:

    - Spec impact

    - Flexibility

    - Performance, including latency

    - Implementation complexity

    - Timing of the activation/deactivation

For coexistence, RAN1 made the following agreements:

- Conclusion on long-term TDM solutions

  - RAN1 does not see any specification impact for support of Long Term Time-Scale TDM for coexistence of NR and LTE sidelinks

- Working assumption on short-term TDM solutions

  - For Tx/Tx overlap,

    - If packet priorities of both LTE and NR sidelink transmissions are known to both RATs prior to time of transmission subject to processing time restriction, then the packet with a higher relative priority is transmitted

      - In case the priorities of LTE and NR SL transmissions are the same, then it is up to UE implementation as to which transmission is chosen (e.g., taking into account congestion, etc.)

    - If packet priorities of both LTE and NR sidelink transmissions are not known to both RATs prior to time of transmission subject to processing time restriction, then it is up to UE implementation to manage Tx/Tx overlaps (e.g., LTE transmissions are always prioritized, etc.)

    - RAN1 does not assume any impact to LTE physical layer specifications

For QoS management, RAN1 made the following agreements:

- Agreements on sidelink congestion metric

  - Support at least NR CBR as congestion metric for NR sidelink congestion control.

    - LTE CBR is the baseline for defining NR CBR.

For AS level link management, RAN1 made the following agreements:

- Agreements on reference signal for sidelink RLM/RLF

  - No new reference signal dedicated to SL RLM is introduced.

    - Existing SL RS is reused for SL RLM/RLF

      - Note: CSI-RS is not precluded

    - RAN1 has no intention to introduce RS transmitted in a periodic manner only for SL RLM purposes

    - FFS:

      - Whether SL RS is transmitted in a stand-alone manner for SL RLM/RLF

- Agreements on metric for sidelink RLM/RLF

  - Regarding metric for SL RLM/RLF declaration, RAN1 discussed the following (to be further studied):

    - Reuse IS/OOS metric in Uu RLM as much as possible but considering the condition that RAN1 has no intention to introduce RS transmitted in a periodic manner only for SL RLM purposes

    - Other metrics, e.g., congestion control metric (similar to CBR in LTE), consecutive HARQ-NACKs, etc.

    - Note: RAN1 expects further input from RAN2 to further progress on this topic

  - An LS was sent to RAN2 (R1-1905863)

For additional simulation assumption, RAN1 made the following agreements:

- Agreements on Tx/Rx antenna parameter for evaluation

  - For further evaluation of NR V2X,

    - 2Tx/2Rx is taken as an additional simulation assumption for FR1.

RAN1#97

RAN1 endorsed TR 37.985 v0.0.1 (R1-1906663).

For sidelink design, RAN1 made the following agreements:

- Agreements on PSFCH format

  - A sequence-based PSFCH format with one symbol (not including AGC training period) is supported.

    - This is applicable for unicast and groupcast including options 1/2.

    - Sequence of PUCCH format 0 is the starting point.

    - FFS: 1 PRB or multiple PRBs is/are used for this PSFCH format

    - FFS: feasible number of HARQ-ACK bits, mapping of HARQ-ACK bit

  - FFS whether to support the following formats

    - X-symbol PSFCH format with a repetition of the one-symbol PSFCH format (not including AGC training period).

      - E.g. X=2

    - A PSFCH format based on PUCCH format 2

    - A PSFCH format spanning all available symbols for sidelink in a slot

- Agreements on frequency resource of PSSCH transmission

  - Transmission of PSSCH is mapped onto contiguous PRBs only

- Agreements on sub-channel size

  - Sub-channel size is (pre)configurable.

    - FFS details (e.g., possible sizes, a minimum size etc.)

- Conclusion on two-stage SCI

  - If two-stage SCI is supported, the following details are used.

    - Information related to channel sensing is carried on 1st-stage.

    - 2nd-stage is decoded by using PSSCH DMRS.

    - Polar coding used for PDCCH is applied to 2nd-stage

    - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool.

    - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage.

    - FFS other details

  - Companies are encouraged to perform analysis (e.g., flexibility, complexity, forward compatibility, overhead, spec impact, latency, robustness, reliability, etc.)/evaluations with details of the SCI contents comparing single-stage vs. two-stage SCI. Aim to conclude in RAN1#98

- Working assumption on frequency pattern of PSSCH DMRS

  - Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS.

    - FFS whether to support either one or both types

    - FFS details on multiplexing of different ports for PSSCH DMRS

- Agreements on sidelink transmit power

  - For sidelink transmit power control,

    - Total sidelink transmit power is the same in the symbols used for PSCCH/PSSCH transmissions in a slot.

      - FFS whether/how to handle simultaneous transmission of sidelink and uplink

    - The maximum SL transmit power is (pre-)configured to the TX UE.

      - FFS on details (e.g., whether the maximum power is dependent of parameters such as the priority of PSCCH/PSSCH)

- Agreements on sidelink open-loop power control

  - For the SL open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss.

  - When the SL open-loop power control is configured to use both DL pathloss and SL pathloss,

    - The minimum of the power values given by open-loop power control based on DL pathloss and the open-loop power control based on SL pathloss is taken.

      - (Working assumption) P0 and alpha values are separately (pre-)configured for DL pathloss and SL pathloss.

- Agreements on TX-RX distance based sidelink HARQ feedback for groupcast

  - For at least option 1 based TX-RX distance-based HARQ feedback for groupcast,

    - A UE transmits HARQ feedback for the PSSCH if TX-RX distance is smaller or equal to the communication range requirement. Otherwise, the UE does not transmit HARQ feedback for the PSSCH

      - TX UE’s location is indicated by SCI associated with the PSSCH.

        - Details FFS

      - The TX-RX distance is estimated by RX UE based on its own location and TX UE location.

      - The used communication range requirement for a PSSCH is known after decoding SCI associated with the PSSCH

        - FFS implicit or explicit

      - FFS how to define location

  - An LS was sent to SA2 (R1-1907908)

- Agreements on PSFCH resource period

  - For the period of N slot(s) of PSFCH resource, N=2 and N=4 are additionally supported.

- Agreements on sidelink HARQ feedback timeline

  - For a PSSCH transmission with its last symbol in slot n, when the corresponding HARQ feedback is due for transmission, it is expected to be in slot n+a where a is the smallest integer larger than or equal to K with the condition that slot n+a contains PSFCH resources.

    - FFS details of K

- Agreements on mechanism for determining PSFCH resource

  - At least for the case when the PSFCH in a slot is in response to a single PSSCH:

    - Implicit mechanism is used to determine at least frequency and/or code domain resource of PSFCH, within a configured resource pool. At least the following parameters are used in the implicit mechanism:

      - Slot index (FFS details) associated with PSCCH/PSSCH/PSFCH

      - Sub-channel(s) (FFS details) associated with PSCCH/PSSCH

      - Identifier (FFS details) to distinguish each RX UE in a group for Option 2 groupcast HARQ feedback

      - FFS detailed applicability of the above parameters

      - FFS: Other parameters (e.g. SL-RSRP/SINR, Layer-1 source ID, location information, etc.)

- Conclusion on handling of multiple PSFCHs

  - Study further whether/how to handle/avoid the following cases for PSFCH transmission and reception:

    - Case 1 (PSFCH TX/RX overlap): A UE transmitted a PSSCH and received SCI scheduling another PSSCH where PSFCH resources corresponding the two PSSCHs appear in the same slot.

    - Case 2 (PSFCH TX to multiple UEs): A UE received SCI from different UEs and the associated PSFCHs appear in the same slot.

    - Case 3 (PSFCH TX with multiple HARQ feedback to the same UE): A UE received multiple SCI from the same UE and the associated PSFCHs appear in the same slot.

- Agreements on sidelink HARQ feedback report to gNB

  - Sidelink HARQ ACK/NACK report from transmitter UE to gNB is supported with details FFS.

    - Note: this reverts the following agreement from RAN1#96:

      - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

  - SR/BSR report to gNB for the purpose of requesting resources for HARQ retransmission is not supported.

  - An LS was sent to RAN2 (R1-1907905)

- Agreements on transmissions of a TB across mode 1 configured grants

  - NR sidelink does not support performing different transmissions of a TB using different configured grants.

- Agreements on dynamic grant in mode 1

  - For mode 1:

    - A dynamic grant by the gNB provides resources for transmission of PSCCH and PSSCH.

- Agreements on resource reservation for feedback-based retransmissions in mode 2

  - NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB

    - FFS impact on subsequent sensing and resource selection procedures

    - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported

      - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

      - FFS the behavior of the receiver UE(s) of this TB and other UEs

- Conclusion on maximum number of retransmissions/reserved resources for retransmissions

  - RAN1 to discuss further the following

    - Maximum number of blind retransmissions supported for one TB

    - Maximum number of reserved blind retransmission

    - Maximum number of HARQ feedback-based retransmissions supported for one TB

    - Maximum number of reserved HARQ feedback-based retransmission

- Agreements on resource reservation for blind retransmissions in mode 2

  - RAN1 to further select between the following options of sidelink resource reservation for blind retransmissions:

    - Option 1: A transmission can reserve resources for none, one, or more than one blind retransmission

    - Option 2: A transmission can reserve resource for none or one blind retransmission

- Agreements on resource selection window in mode 2

  - Resource selection window is defined as a time interval where a UE selects sidelink resources for transmission

    - The resource selection window starts T1 ≥ 0 after a resource (re-)selection trigger and is bounded by at least a remaining packet delay budget

    - FFS T1 value, whether it is measured in slots, symbols, ms, etc.

    - FFS other conditions

- Agreements on sensing granularity for PSSCH resource selection in mode 2

  - Support a sub-channel as the minimum granularity in frequency domain for the sensing for PSSCH resource selection

    - No additional sensing for other channels

- Working assumption on SLSS sequence

  - For the NR SLSS,

    - Same sequence is used for both symbols of S-PSS

    - Same sequence is used for both symbols of S-SSS

- Agreements on evaluation assumption for checking impact of transient period

  - The impact on detection probability performance of having or not having a transient period between S-PSS and S-SSS symbols is used to evaluate the following:

    - Alt 1: S-PSS symbols and S-SSS symbols are adjacent.

    - Alt 2: S-PSS symbols and S-SSS symbols are not adjacent.

  - FFS (aim to conclude this week – see below)

    - The power difference between S-PSS and S-SSS symbols.

    - The transient duration.

- Agreements on evaluation assumption for MPR values of S-PSS/S-SSS

  - The following parameters are assumed for evaluation:

    - Power Difference for S-PSS and S-SSS symbols:

      - Opt.1) MPR values: S-PSS = 0 dB, S-SSS = 3 dB;

      - Opt.2) MPR values: S-PSS = 3 dB, S-SSS = 3 dB

      - Opt.3) companies to report the assumed MPR values

    - Transient period is

      - 10us for FR1; 5us for FR2

      - Waveform puncturing during the transient period

    - S-PSS detection search window: 80ms and 160ms

- Agreements on number of S-SSB transmissions within period

  - In NR V2X, from transmitter perspective, the period (P1 in unit of ms) of S-SSB(s) transmission is the same for all SCS, for further down-selection:

    - Alt 1: the number of S-SSB(s) transmitted within P1is (pre-)configurable.

    - Alt 2: the number of S-SSB(s) transmitted is fixed within P1 per SCS.

    - Alt 3: only one S-SSB for all SCS is transmitted within P1.

- Agreements on S-SSB transmission period

  - At least for evaluation, one S-SSB transmission with at least the following periodicity:

    - 160ms period at least for 15kHz SCS.

    - FFS other value(s)

For NR Uu controlling LTE sidelink, RAN1 made the following agreements:

- Agreements on RRC/DCI based activation/release of LTE sidelink

  - RRC-based activation/deactivation is not supported

  - DCI-based activation/deactivation is supported

    - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

    - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

    - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

      - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

    - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

      - Z is the same timing offset in current LTE V2X specs

      - X>0. FFS value(s) of X, and if one or multiple values of X are possible

  - An LS was sent to RAN (R1-1907909)

For coexistence, RAN1 made the following agreements:

- Agreements on Tx/Tx overlap for short-term TDM solutions

  - For Tx/Tx overlap,

    - Confirm the working assumption made in RAN1#96bis

    - UE capability is defined for short-term time-scale TDM for in-device coexistence

- Agreements on Rx/Rx overlap for short-term TDM solutions

  - For Rx/Rx overlap,

    - Up to UE implementation to manage receptions of LTE and NR sidelinks.

For QoS management, RAN1 made the following agreements:

- Agreements on sidelink congestion control

  - LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control.

- Agreements on CBR report to gNB

  - Higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs.

For AS level link management, RAN1 made the following agreements:

- Agreements on reference signal for sidelink RLM/RLF

  - No standalone RS dedicated to SL RLM/RLF in Rel-16

For additional simulation assumption, RAN1 made the following agreements:

- Agreements on simulation assumption for high congestion scenarios

  - For the high congestion scenario, UE dropping option A with vehicle speed 10 km/h for Highway is taken as an additional simulation assumption.

    - 100% vehicles generate packets, and companies can select the traffic model(s) from the following two models in TR 37.885:

      - Periodic traffic model 1 (low traffic intensity)

      - Aperiodic traffic model 1 (medium traffic intensity)

    - Company to provide details of congestion control operation, if it is modelled

2.1.2	Remaining Open issues

- Open issues in “Support of sidelink signals, channels, bandwidth part, and resource pools” include

  - RS design for physical sidelink channels

  - Resource mapping and multiplexing of physical sidelink channels

  - Detailed configuration and procedures for sidelink bandwidth part and resource pools

- Open issues in “Resource allocation” include

  - DL signalling for sidelink resource grant and UL signalling for sidelink HARQ feedback report in mode 1

  - Details of signalling and UE behaviour for sensing, resource reservation, resource selection in mode 2

  - Transmitter UE behaviour for simultaneous configuration of Mode 1 and Mode 2 for a UE (to be discussed after the design of mode 1 only and mode 2 only)

- Open issues in “Sidelink synchronization mechanism” include

  - Procedures selecting synchronization reference

  - S-PSS and S-SSS sequences and S-SSB structure

  - Procedures of transmitting and receiving S-SSB

  - Use of RS for sidelink synchronization if specification impact is identified

- Open issues in “Solutions for ‘not co-channel’ in-device coexistence between LTE and NR sidelinks” include

  - Detailed priority handling in Tx/Tx overlap

  - How to handling Tx/Rx overlap

- Open issues in “Sidelink physical layer procedures” include

  - PSFCH resource determination and handling of multiple PSFCHs

  - CSI acquisition using sidelink CSI-RS and CSI reporting channel/procedure

  - Detailed sidelink power control formula and calculation of sidelink pathloss

- Open issues in “congestion control” include

  - Definition of CBR and adaptation of LTE sidelink congestion control to NR sidelink

- Open issues in “AS level link management for unicast” include

  - Definition of physical layer criterion on the radio failure

- Open issues in “support for NR Uu to provide control for LTE sidelink” include

  - Definition of NR DL signalling to control LTE sidelink and the related UE behaviour

- Open issues in “UE report to assist gNB scheduling” include

  - Agreement on UE report information beneficial from the physical layer perspective

- Open issues in “support for QoS management” include

  - Use of the priority information in the physical layer operation

2.2	RAN2

224 contributions were submitted to RAN2#105bis. 278 contributions were submitted to RAN2#106.

2.2.1	Agreements

RAN2#105bis

For NR V2X, RAN2 made the following agreements:

- Agreements on SR

  - For NR Uu controlling NR SL, whether/how to configure separate SR resources and SR configurations for UL and SL is up to gNB implementation (e.g. associating UL LCHs and SL LCHs with different SR configuration IDs respectively)

  - As in NR Uu, the mapping between SR configurations and SL LCHs can be achieved by including in each SL LCH configuration the ID of its associated SR configuration, which is associated with a set of SR resources.

- Agreements on BSR and UAI

  - For SL BSR, at least adopt buffer size (bit size is FFS), destination index (bit size is FFS) and LCG ID (detailed format and bit size is FFS).

  - As in NR Uu, there is a mapping between SL LCH and SL LCG.

  - As in NR Uu, for mode1 the mapping between radio bearer and SL logical channel is provided as part of SL RLC bearer configurations (added or modified).

  - NR SL BSR triggering condition should be based on LTE V2X at least.

  - NR SL BSR cancelling conditions should be based on LTE V2X at least.

  - For SL buffer status, reuse the current definition of buffer status as in NR Uu.

  - Support UE assistance information reporting on traffic pattern, including information on periodicity, time offset, message size, QoS info (details are FFS), and destination id.

- Agreements on SL configured grant

  - Multiple active configured sidelink grants should be supported in NR sidelink.

  - A confirmation for activation/deactivation of SL configured grant type-2 is needed. Details are FFS.

  - The type 1 and 2 configured SL grant should be specified for NR SL mode 1.

- Agreements on validity area mechanism to SL:

  - NR SIB validity mechanism is reused to enable validity area for SL resource pool configured via broadcasted system information.

- Agreements on PC5-RRC message exchange:

  - PC5-RRC connection is needed to establish SL UE context. Synchronization of SL UE context between two UEs is supported by the concept of PC5-RRC connection.

    - Need for PC5-RRC state is FFS.

      - Option 1: Define PC5-RRC state for unicast operation.

      - Option 2: Refer to PC5-S state for unicast operation

    - SL UE context may include at least SL UE capability of the destination UE.

      - FFS whether AS configuration information can be also stored in SL UE context.

    - UE context is per destination UE.

      - It is considered that UE may store UE capability of the destination UE for a newly coming service between UEs in unicast.

      - It may depend on SA2 discussion related to layer-2 ID allocation. RAN2 will come back if there is a problem based on SA2 progress.

    - FFS whether explicit PC5-RRC connection establishment procedure is needed or not.

  - Security aspect comes back after SA3 progress (if there is any issue/problem).

- Agreements on PC5-RRC signalling flow:

  - Separate RRC messages are defined capability transfer and for AS-layer configuration. FFS on whether the two messages can be transmitted together in the same MAC PDU.

  - Set the following a, b and c as RAN2 working assumption:

    - (a) Do not encapsulate PC5-S message related to link setup into PC5-RRC message for AS-layer configuration.

    - (b) PC5-RRC message for AS-layer configuration is not to be sent unprotected, so is not to be sent together with PC5-S messages like Direct Communication Request.

    - (c) Do not encapsulate PC5-S message related to link setup into PC5-RRC message for capability information.

- Agreements on inter-RAT resource allocation:

  - For scheduling LTE SL UEs, the gNB uses RRC messages to deliver the SPS grant configuration.

  - LTE Uu supports configuration for NR mode-2 operation through RRC signaling and through SIB signaling. However, on demand or per valid area mode-2 configuration via SIB signaling is not supported.

  - New system information block type should be designed to support NR sidelink Tx and Rx resource pool configuration via LTE Uu. New system information will be defined as container (OCTET STRING) and actual information will follow what is defined in NR RRC.

  - eNB should be able to configure the NR V2X mode 2 sidelink resource pool or type 1 configured grant for NR V2X sidelink communication via dedicated signalling.

  - NR Sidelink UE information and/or NR UE Assistance information will be transmitted as container (OCTET STRING) and actual information will be defined in NR RRC.

  - Separate system information block should be designed to support LTE resource pool configuration via NR Uu. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

  - gNB should be able to configure the LTE V2X mode 4 sidelink resource pool via dedicated signalling. In addition, gNB should be able to configure mode3 SL resources via dedicated signaling. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

- Agreements on MAC

  - SL-DCH is not needed in NR V2X.

  - Restrictions to SL LCP procedure may be considered at least based on different casting modes. FFS whether destination id can distinguish casting mode.

RAN2#106

For NR V2X, RAN2 made the following agreements:

- Agreements on LCP

  - As, in release 16, only single carrier is used for SL transmission, RAN2 assumes mapping restriction between SCS and Sidelink LCH should not be considered in SL LCP procedure.

  - Configured grant Type 1 is considered as SL LCP mapping restriction for Sidelink LCH.

  - LCP restriction for Sidelink LCH is configured by NW for UE in IC. FFS on the need of preconfiguration option for UE in OOC.

  - Uu like starvation avoidance mechanism is applied to LCP.

  - For Sidelink broadcast, different destinations (i.e. each Destination Layer 2 ID targeting specific broadcast service) are not multiplexed into the same MAC PDU. For Sidelink groupcast, different destinations (i.e. each Destination Layer 2 ID targeting specific group or groupcast service) are not multiplexed into the same MAC PDU. FFS for unicast case.

- Agreements on LCH priority

  - For unicast for IC connected UE, logical channel priority level is configured by NW. Mapping between PQI/PFI, LCH and SLRB is also configured by NW (e.g. by dedicated RRC).

  - For unicast for IC idle/inactive UE, logical channel priority level is configured by NW. Mapping between PQI/PFI, LCH and SLRB is also configured by NW (e.g. by SIB).

  - For unicast for OOC UE, logical channel priority level is configured by NW. Mapping between PQI/PFI, LCH and SLRB is also configured by NW (e.g. by preconfiguration).

  - FFS on groupcast and broadcast cases.

- Agreements on MAC PDU format

  - Length of LC id is 6bits for NR SL.

  - Number of LC id for SL DRB (for a given destination id) is 16.

  - MAC PDU for SL-SCH will follow interleaved structure like NR Uu.

  - Reuse the existing NR UL MAC subheader format for SL-SCH, including R/F/LCID/L MAC subheader with 8-bit L field, R/F/LCID/L MAC subheader with 16-bit L field and R/LCID MAC subheader.

- Agreements on UL/SL prioritization

  - For NR UL and NR SL prioritization, the QoS requirement of both SL and UL transmissions can be used to judge whether the SL transmission is to be prioritized over UL or not, FFS on how the QoS requirement of SL and UL transmission can be taken into account.

  - For NR UL and NR SL prioritization, MSG1/3 for RACH procedure and PUSCH for emergency PDU connection are always prioritized over SL transmission.

  - LTE-solution should be applied to LTE UL and NR SL cross-RAT case (if needed). FFS on the need of this prioritization.

  - For NR UL and LTE SL cross-RAT case, RAN2 aims at no change to LTE SL protocol, and LTE-solution is the baseline (if needed). FFS on the need of this prioritization.

  - The priority value based solution can be applied to PC5-RRC messages as well, and default value can be defined in the spec, and allows (pre-)configuration to override it.

  - RAN2 does not consider the scenario where SL is controlled/configured by SN in Rel-16 NR-V2X.

  - For UL/SL prioritization, RAN2 further discuss the need/impact to consider SCG UL for UL/SL prioritization.

  - RAN2 aims at a general solution for UL/SL prioritization for different cast types.

- Agreements on MAC Header

  - With a given full source/destination ID, if a portion of it is used as L1 source/destination ID in SCI, the rest portion is conveyed in MAC header as L2 source/destination ID.

- Agreements on HARQ feedback support for groupcast

  - In order to support Option1, no additional AS layer co-ordination or signalling for HARQ feedback resource allocation within the group is required.

- Agreements on single carrier operation:

  - Multiple carriers configuration is not supported in Rel-16 to RAN2 understanding on WID.

  - Carrier reselection and PDCP duplication are not supported in Rel-16.

- Agreements on SDAP

  - No need of reflective QoS.

  - FFS on the need of RX UE awareness of QFI.

- Agreements on cell (re)selection

  - For cell selection, NR V2X UE can follow the current NR cell selection procedure.

  - For cell reselection, NR V2X UE who interested in SL V2X communication needs to consider at least the SL RAT(s) supported by the cell. Moreover, the supported SL RAT(s) of the UE capability or the selected SL RAT(s) by the upper layer for the V2X service(s) may also need to be considered.

- Agreements on exceptional TX resource pool:

  - As in LTE V2X, NR V2X adopts the concept of exception pool.

  - As in LTE V2X, when configured in mode 1, UE use exceptional pool in the following cases:

    - When UE detect Uu physical layer problems or radio link failure.

    - Before UE finish the initiated connection (re)establishment.

    - During handover

  - As in LTE V2X, if UE is not configured in mode 1, UE use exceptional pool when the sensing results for the normal TX resource pool is not available.

- Agreements on SL BSR for mode1:

  - LCG ID shall be included in the NR SL BSR MAC CE instead of bitmap.

  - NR SL BSR of 5-bit destination index, 3-bit LCG ID and 8-bit buffer size.

  - All the SL BSR triggers in LTE V2X are adopted for NR V2X SL.

  - As in LTE V2X, all SL BSRs shall be cancelled, if the remaining configured SL grant(s) valid can accommodate all pending data available for transmission in V2X SL communication, or if the MAC entity has no data available for transmission for any of the SL logical channels, or if UE is reconfigured to work from mode-1 to mode-2 (w/o consideration of simultaneous modes).

  - Like NR Uu, all SL BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a SL BSR MAC CE (except for Truncated SL BSR)  which contains buffer status up to (and including) the last event that triggered an SL BSR prior to the MAC PDU assembly.

  - Support the periodic BSR timer and the retx BSR timer, the SR-delay timer for regular BSR can be configured for NR SL BSR operation.

  - No need to explicitly include cast type information in SL BSR. The UE can report the cast type of each destination in the SidelinkUEInformation.

  - In NR V2X, Regular SL BSR, Periodic SL BSR and padding SL BSR are defined.

  - SL BSR and truncated SL BSR are supported in NR.

- Agreements on SR for mode1:

  - Each SR configuraiton can corredpond to one or more SL LCH, and each SL LCH is mapped to zero or one SR configuration (either those dedicated to SL or those shared by UL and SL).

  - For a SL LCH mapped to zero SR configuration, if the SL BSR is triggred by this SL LCH and the SR is triggred accordingly, the UE shall initiate the RA procedure.

  - As for NR Uu, an SR configuration dedicated for NR SL only includes the paramters SchedulingRequestID, sr-ProhibitTimer and sr-TransMax.

  - For NR SL, if a Regular SL BSR has been triggered, the SR shall be triggred if either of the following conditions is satisfied:

    - The UE has neither dynamic UL resource allocated for new transmission nor configured UL grant;

    - The UE has a UL resource allocated for new transmission but the UL resource cannot accommodate the SL BSR MAC CE plus its subheader.

- Agreements on HARQ reTX REQ

  - RAN2 progresses SR/BSR design focusing on initial transmission scenario.

- Working assumption on resource pool configurations:

  - Synchronization reference resource needs to be indicated for each resource pool.

- Agreements on SL configurations

  - For idle/inactive UE, NR V2X SI can be provided on-demand. It is a network decision whether the NR V2X SI is broadcast at a given time. FFS for connected UE.

- Agreements on PC5-RRC

  - Need bi-directional procedure for capability transfer procedure for bi-directional SL traffic.

  - Working assumption: both bi-directional one-way procedure and two-way procedure for capability transfer are allowed. FFS on how to support in details.

  - Need bi-directional procedure for AS-layer configuration procedure for bi-directional SL traffic.

  - Apply the two-way procedure to bi-directional AS-layer configuration, but no need for figure in RRC specification correspondingly.

  - Need to handle failure case for AS-layer configuration. Explicit failure message is used as baseline. Timer-based solution is also needed on top of explicit failure message.

- Agreements on PC5 RLM/RLF

  - Even though transmission of sidelink signal occur irregularly, RAN2 assumes that the physical layer provides periodic indications of IS/OOS to the upper layer as in Uu RLM.

  - From RAN2 perspective, both side UEs perform RLM/RLF detection mechanism. FFS on whether periodic indications of IS/OOS based RLM/RLF is reused or any additional new mechanism is needed.

- Agreements on NR SL QoS and SLRB configurations

  - Stick to SI phase conclusion that SLRB configurations should be NW-configured and/or pre-configured for NR SL.

  - For an RRC_CONNECTED UE, for transmission of a new PC5 QoS flow, it may report the QoS information of the PC5 QoS flow via RRC dedicated signalling to the gNB/ng-eNB. FFS on the exact timing about when UE initiates.

  - For an RRC_CONNECTED UE, the gNB/ng-eNB may provide SLRB configurations and configure the mapping of PC5 QoS flow to SLRB via RRC dedicated signalling, based on the QoS information reported by the UE. The UE can establishes/reconfigures the SLRB only after receiving the SLRB configuration. FFS when the UE establishes/reconfigures the SLRB.

  - FFS what the reported QoS information is (e.g. PFI, PC5 QoS profile, etc.) and what is used to realize the PC5 QoS flow to SLRB mapping (e.g. PFI to SLRB mapping, QoS profile to SLRB mapping, etc.), depending on SA2 conclusion on how PFI is assigned.

  - For RRC_IDLE/INACTIVE UEs, the gNB/ng-eNB may provide SLRB configurations and configure the PC5 QoS profile to SLRB mapping via V2X-specific SIB. When an RRC_IDLE/INACTIVE UE initiates the transmission of a new PC5 QoS flow, it establishes the SLRB associated with the PC5 QoS profile of that flow based on SIB configuration.

  - FFS how to describe each PC5 QoS profile in the SIB, pending SA2’s final conclusion on what PC5 QoS parameters are included in a PC5 QoS profile.

  - For OoC UEs, SLRB configurations and the mapping of PC5 QoS profile to SLRB are pre-configured. When an OoC UE initiates the transmission of a new PC5 QoS flow, it establishes the SLRB associated with the flow based on pre-configuration.

  - FFS what is used to realize for PC5 QoS flow to SLRB mapping in pre-configuration (e.g. PFI to SLRB mapping, QoS profile to SLRB mapping, etc.), depending on SA2 conclusion on how PFI is assigned.

  - For SL unicast of a UE, the NW-configured/pre-configured SLRBs configurations include the SLRB parameters that are only related to TX, as well as the SLRB parameters that are related to both TX and RX and need to be aligned with the peer UEs.

  - For SL unicast, the initiating UE informs the peer UE of SLRB parameters that are related to both TX and RX and need to be aligned with the peer UEs. FFS on the detailed parameters.

  - For SL unicast, do not allow a UE to configure “SLRB parameters only related to TX” for the peer UE in SL via PC5 RRC message. FFS how to handle SRLB parameters only related to RX.

  - For SL groupcast and/or broadcast, the NW-configured/preconfigured SLRBs include the SLRB parameters that are only related to TX.

  - Those SLRB parameters which are related to both TX and RX and thus need to be aligned between a UE and all its peer UE(s) should be fixed in the Spec for SL groupcast and broadcast.

  - For SL broadcast, how to set SLRB parameters only related to RX is up to UE implementation. FFS for groupcast case.

  - SLRB configurations should be (pre-)configured for SL unicast, groupcast/broadcast separately (e.g. SLRB-ConfigForUnicast, SLRB-ConfigForGroupcast, SLRB-ConfigForBroadcast). FFS on the need of separate SLRB configurations between groupcast and broadcast.

2.2.2	Remaining Open issues

- Open issues in “resource allocation ” include

  - Details of signalling and UE behaviour for retransmission scheduling in mode 1

  - Details of signalling and UE behaviour for Resource pool configuration and selection in mode 2

  - Transmitter UE behaviour for simultaneous configuration of Mode 1 and Mode 2 for a UE (to be discussed after the design of mode 1 only and mode 2 only)

- Open issues in “Sidelink synchronization mechanism” include

  - RRC support for SL synchronization

- Open issues in “Sidelink L2/L3 protocols and signalling” include

  - Details of messages and procedures for Uu RRC and PC5-RRC

  - Details of Layer 2 procedures and PDU formats

- Open issues in “congestion control” include

  - Details of CBR reporting for NR SL

  - Details of NR MAC procedures related to CBR for NR SL

- Open issues in “support for NR Uu to provide control for LTE sidelink” include

  - Definition of NR DL signalling to control LTE sidelink and the related UE behaviour

- Open issues in “UE report to assist gNB scheduling” include

  - Agreement on UE report information beneficial from the L2/L3 perspective

  - RRC support of UE report

- Open issues in “support for QoS management” include

  - Details of SL QoS framework and SLRB configuration

2.3	RAN3

The work will start from August in RAN3.

2.3.1	Agreements

2.3.2	Remaining Open issues

- Network solutions to support NR sidelink

  - V2X service authorization

  - F1 signalling for support of NR V2X

  - Resource coordination between NG-RAN nodes for V2X sidelink communication, taking into consideration previous RAN3 discussions

- RAN3 aspects of support for QoS management

2.4	RAN4

29 contributions were submitted to RAN4#90bis. 35 contributions were submitted to RAN4#91.

2.4.1	Agreements

RAN4#90bis RF

- Agreed to Work plan for 5G NR V2X to complete RF/RRM core requirements until March 2020

- Agreed TR skeleton to capture the coexistence evaluation scenarios, parameters and conclusion. Also to specify the detail RF requirements for NR V2X

- Agreed to WF on coexistence scenarios for NR V2X

- In device coexistence

- (LTE SL and NR SL operation at ITS spectrum) without (LTE or NR Uu in another band) on the same device

  - TDM operation in 5.9 GHz ITS spectrum.

    - Based on approved WID, no self interference issues are identified for the IDC

    - Switching times for TDM operation needs to be studied

  - FDM operation

    - Clarification on the sentence as below

    - FDM-based solutions with static power allocation as per the study outcome

    - This will not consider the case where LTE and NR sidelinks are in the same frequency band.

  - The inter-band FDM operation includes a band for LTE SL (at 5.9GHz) and a band for NR sidelink operation.

- RAN4 defers the coexistence discussion to May meeting. Some feedback from 5GAA and operator expected. Final decision on band is discussed in RAN4.

- * There is no self interference problem in ITS spectrum

- UE-to-UE coexistence

- 1) 5.9 GHz ITS spectrum

  - Aggressor-to-Victim UE coexistence

    - NR SL-to-DSRC, NR SL-to-LTE SL

    - DSRC-to-NR SL, LTE SL-to-NR SL

  - In LTE SL WI, RAN4 already analyze the DSRC impact

  - For the new unlicensed ITS spectrum, RAN4 will discuss when it is available

- 2) Licensed spectrum

  - Aggressor-to-Victim UE coexistence

    - NR SL-to-NR/LTE Uu

    - LTE/NR Uu-to-NR SL

  - RAN4 need to UE-to-UE coexistence parameters and assumptions for these scenarios.

  - However, RAN4 defer the coexistence discussion at May meeting. Some feedback from 5GAA and operator expected. Final decision on bands are discussed in RAN4.

- Agreed reply LS to RAN1 for NR UE RF parameters

- AGC settling time

  - No consensus reached on the AGC settling time values. RAN4 will continue to investigate the AGC settling time values

  - Below observations that impact AGC setting time in RAN4 study are:

    - Different companies have different assumptions of AGC settling time.

      - Some parameters that impact AGC settling time are waveform, RB size, SCS, number of carriers, number of receiver chains, and adjacent channel blocking interference. AGC settling time reported to RAN1 shall also consider the implementation and practical aspects.

    - Based on different assumptions:

      - The AGC settling time observed by Qualcomm, Ericsson, Intel, Interdigital is

        - AGC time :  35us for all SCS

      - The AGC settling observed by Mediatek, Huawei, LGE, Spreadtrum is

        - AGC time :  35us for 15kHz  SCS

18us for 30kHz SCS

9us for 60kHz SCS

    - If multiple carriers are configured, AGC performance could be degraded/further evaluated.

    - Any further clarification on RAN1 design/ assumptions regarding the above observations could benefit RAN4 to further evaluate AGC performance.

- Timing error between a UE and its synchronization reference

  - FR1:

    - gNB synchronization reference source: reuse NR UE UL transmission timing error defined in Table 7.1.2-1 in TS38.133

    - GNSS synchronization reference source: Time error is 12*64Tc for 15kHz SCS. It is starting point, RAN4 will further analyse the timing error by GNSS synchronization source and subject to change during NR V2X WI phase. Other SCSs are FFS

    - UE synchronization reference source: FFS

  - FR2:

    - gNB synchronization reference source: reuse NR UE UL transmission timing error defined in Table 7.1.2-1 in TS38.133. It is starting point, RAN4 will further analyse the timing error by gNB synchronization source and subject to change during NR V2X WI phase.

    - GNSS synchronization reference source: FFS

    - UE synchronization reference source: FFS

- Frequency error between a UE and its synchronization reference

  - FR1: ±0.1 ppm for gNB/GNSS/NR UE synchronization reference source

  - FR2: ±0.1 ppm for gNB/GNSS/NR UE synchronization reference source.

- Agreed reply LS to 5GAA for preferred NR V2X operating bands for both FR1&FR2

RAN WG4 kindly asks 5GAA’s feedback on the following:

- Which 3GPP (sub-6 GHz and/or mm-wave) NR bands should be considered for NR V2X sidelink operation in the licensed band?

- What frequency ranges and channel positions should be considered for NR V2X sidelink operation in 5.9GHz ITS spectrum and other potential ITS spectrum?

- What channel bandwidths required for NR V2X sidelink operation?

- What are the assumptions when operating NR V2X sidelink in the licensed band? These might include but not limited to:

  - Single operator scenario where licensed band spectrum is allocated by the operator

  - Multi operator scenario where licensed band spectrum is allocated by each operator

  - Multi operator scenario where a common licensed band spectrum allocated by national regulator

RAN4#90bis RRM

For sidelink transmit timing requirement, RAN4 agreed the following WF:

- RAN4 starts to study V2X UE sidelink transmit timing requirement based on RAN1/2’s agreement for synchronization reference source

- Other RRM requirements is not precluded if identified

And, Specification structure and expected RRM requirements were discussed.

RAN4#91 RF

- Agreed to WF on coexistence scenarios and parameters for NR V2X

  - FR1

  - ITS spectrum(5.9GHz)

    - Case1: NR V2X UE-to-DSRC UE (REF : DSRC UE-to-DSRC UE)

    - Case2: DSRC UE-to-NR V2X UE (REF : DSRC UE-to-DSRC UE)

    - Case3: NR V2X UE-to-LTE V2X UE (REF: LTE V2X UE-to-LTE V2X UE)

    - Case4: LTE V2X UE-to-NR V2X UE (REF: LTE V2X UE-to-LTE V2X UE)

  - Licensed spectrum(2GHz FDD, 3.5GHz TDD)

  - Case5: NR V2X UE-to-NR Uu BS (REF: NR UE-to-NR BS)

  - Case6: NR Uu UE-to-NR V2X UE (REF: victim PRR without aggressor)

  - Case7: NR V2X UE-to-LTE Uu BS (REF : LTE UE-to-LTE UE)

  - Case8: LTE Uu UE-to-NR V2X UE (REF: victim PRR without aggressor)

  - FR2

  - Licensed spectrum(28GHz TDD)

  - Case9: NR V2X UE-to-NR Uu BS (REF : NR Uu UE-to-NR Uu BS)

  - Case10: NR Uu UE-to-NR V2X UE (REF: victim PRR without aggressor)

- General simulation parameters

- Only outdoor deployment will be considered

- Deployment scenarios: Urban grid in Annex I, Highway (optional)

- Simulation Block Size :

  - Urban : Manhattan grid model: 3*433m, 3*250m

- Absolute vehicle speed for urban

  - Urban : 15km/h, 60km/h

    - Used only to decide vehicle density

    - Follow TR36.885 for LTE V2X and TR37.885 for NR V2X

  - Fixed location will be considered for adjacent channel coexistence evaluation

  - All vehicle UE has both LTE V2X and NR V2X,

    - The Tx between LTE V2X and NR V2X is TDM, while Rx between LTE V2X and NR V2X is simultaneous.

- Traffic model:

  - Dependent on UE velocity (LTE ref.:36.785, NR ref.: TR37.885)

    - For the NR traffic, medium intensity aperiodic traffic model will be used.

    - LTE traffic model

      - Option1: UE activation rate: 1%

      - Option2: 15km : 0.1% or 60km: 0.33%

      - Other options are not precluded

- RAN1 dependent parameter

  - Power control scheme, RAN4 can revisit the PC according to RAN1 decision, but following is initial

    - In licensed spectrum, OLPC in TR36.786 is used

    - In ITS spectrum, worst case of no power control is used

  - For licensed band, NR SL operation in Uplink band in FDD, UL opportunity in TDD is considered.

- Simulation Parameters Specific to ITS

- Pathloss models

  - NR V2X UE-to-DSRC UE : Follow TR 37.885

  - NR V2X UE-to-LTE UE : Follow TR 37.885

- ACLR and ACS for DSRC UE (10MHz)

  - UE ACLR= 38 dB

  - UE ACS = 22/25/29dB.

- ACLR and ACS for LTE V2X UE (20MHz)

  - UE ACLR= 30 dB

  - UE ACS = 27dB

- Simulation Parameters Specific to Licensed spectrum at FR1

- Pathloss models

  - NR/LTE Uu UE-to-NR/LTE BS: Table 5.4.2.1-1 in TR37.885

  - NR/LTE Uu UE-to-NR V2X UE: Table 5.4.3.1-1 in TR37.885

- Used ACLR and ACS for legacy LTE & NR BS/UE (20MHz)

  - BS ACLR=45dB, BS ACS=46dB

  - LTE/NR UE ACLR=30dB, LTE/NR UE ACS=27dB

- Simulation Parameters Specific to Licensed spectrum at FR2

- Pathloss models

  - NR V2X UE-to-NR gNB : Follow TR 37.885

  - NR Uu UE-to-NR V2X UE: Follow TR37.885 (P2V model)

- Used ACLR and ACS for legacy NR gNB/UE (200MHz)

  - gNB ACLR=28dB, gNB ACS=23.5dB

  - NR UE ACLR=17dB, NR UE ACS=23dB

- Evaluation metric

- PRR degradation due to adjacent channel interferer(s)  for both DSRC and NR/LTE V2X system in 5.9GHz

- Legacy system throughput degradation due to adjacent channel interferer(s) in both licensed FR1 and FR2 spectrum

- Link level simulation mapping model for LTE V2X UE is referred in Annex A in TR36.785

  - It will be updated at 2GHz/3.5GHz and 28GHz for NR V2X UE. Detail simulation parameters are FFS.

- Agreed to NR V2X operating band in ITS spectrum

| NR operating band | Sidelink (SL) Transmission operating band F_low – F_high | Sidelink (SL) Reception operating band F_low – F_high | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
| n47 | 5855 MHz – 5925 MHz | 5855 MHz – 5925 MHz | TDD | PC5 |

- Agreed to additional reply LS for AGC settling time

- AGC settling time :

- For single CC using CP-OFDM waveform and at least 10 RB allocation

  - 35 usec for 15 kHz SCS

  - 35 usec for 30 kHz SCS

  - 18 usec for 60 kHz SCS

- RAN4 has not studied the multicarrier cases.

RAN4#91 RRM

For sidelink transmit timing requirement, RAN4 agreed the following WF:

- RRM requirements should be specified based on the scenarios agreed in the RAN4 RF group

- For NR V2X SL Tx timing error

  - Synchronization reference sources such as GNSS, gNB, eNB and NR UE should be considered based on RAN1 agreement.

    - RAN4 shall study the impact of synchronization source selection if any of eNB and gNB can be used as sync source based on the conclusion of RAN4 RF or RAN1

- RAN4 shall study impact of BWP switching in NR Uu (DL/UL) on NR SL

And, impact on RRM core requirements was discussed.

2.4.2	Remaining Open issues

RF Open issues

- Detailed simulation parameters and analysis on the adjacent channel coexistence evaluations based on the agreed scenarios and parameters

- The priority of operating scenarios should be discussed based on coexistence evaluation results

- General RF impact to introduce NR V2X UE RF requirements

- V2X UE General requirements (operating bands, operating channel bandwidths)/Transmitter/Receiver requirements

- MPR for TDM allocation between PSSCH and PSCCH

- A-SE requirement at operating band and A-MPR simulation assumptions

- Specify V2X UE Transmitter/Receiver requirements

- Initial simulation results on the MPR/A-MPR simulation from interested companies

- Initial simulation assumption for REFSENS of NR V2X UE

- Other RF requirements

RRM Open issues

- NR V2X SL Tx timing error Requirement

- Other RRM requirements

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SA2

3.1.0	SA2 eV2XARC status – general

FS_eV2XARC (Study on architecture enhancements for 3GPP support of advanced V2X services) has been 100% complete. One remaining key issue, Key Issue #15 (Enhancements to assist Application Adjustment) was concluded in SA2#133 (May 2019).

eV2XARC (Architecture enhancements for 3GPP support of advanced V2X services) has been 75% complete. In SA2#133, SA2 approved the following documents to submit them to SA#84:

-	TS 23.287 "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services" for Information : S2-1906815.

-	Exception sheet to extend the completion target date for Rel-16 eV2XARC to September 2019 (SA#85) : S2-1906816.

3.1.1	Agreements with cross-TSG impacts

In Q2 2019, the following agreements were mainly made under eV2XARC and the details can be seen in TS 23.287v0.4.0.

Regarding QoS model for NR PC5, it was agreed in SA2 to use the bearer based QoS model (i.e. Per-Flow QoS model) for unicast, groupcast and broadcast operations. In SA2#132 (April 2019), SA2 sent the following Reply LS to inform the related RAN WGs of this agreement.

-	S2-1904823: LS response on unicast, groupcast and broadcast in NR sidelink (To: RAN1, RAN2 / Cc: RAN3)

Regarding Unicast mode of V2X communication over NR PC5, some principles were agreed related to the granularity of the PC5 unicast link and etc.

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

Regarding QoS for V2X communication over Uu that has RAN dependency, SA2 sent the following Reply LS to RAN WGs in order to provide answer on Core network PDB value asked by RAN1.

S2-1906340: Reply on Combinations of Uu QoS characteristics values for V2X services (To: RAN1, RAN3 / Cc: RAN2) in order to get final feedback on the following questions asked to RAN1 and RAN2 by sending LS S2-1813386 in SA2#129bis (November 2018):

SA WG2 concluded that for some services (e.g., Collision Avoidance, Platooning with high LoA) new combination of QoS characteristics values with PDB = 5ms, PER = 10-4 and MDBV = 1354 bytes is needed to fulfil the performance requirements defined in TS 22.186. In addition, other services (e.g., Emergency Trajectory Alignment and Sensors information Sharing with high LoA) require another new combination of QoS characteristics values with ultra-low PDB ~1.5 ms, PER=10-5 and MDBV ~1300 bytes. Question: SA WG2 would like to ask RAN WG2 and RAN WG1 whether, for Uu over E-UTRA and NR, the two new combinations of QoS characteristics values indicated above are feasible or not.

The following is SA2 meeting in Q3 2019:

-	SA2#134	June 24 – 28, 2019 (Sapporo, Japan)

RAN WGs related to the above LS will have one meeting in Q3 2019, i.e. August 26 – 30, 2019. Therefore, in order to finalize the pending issue related to the above LS, it is considered that timely feedback on the SA2 questions from RAN WGs is needed.

※ NOTE: In RAN1, the LSs described above were discussed under NR_L1enh_URLLC because the issue is about Uu QoS.

※ All the related LSs are as below:

S2-1813386 @SA2#129bis: Combinations of Uu QoS characteristics values for V2X service (To: RAN1, RAN2 / Cc: RAN3) R1-1905904 @RAN1#96bis: Response to LS on Combinations of Uu QoS characteristics values for V2X services (To: SA2, RAN3 / Cc: RAN2) S2-1906340 @SA2#133: Reply on Combinations of Uu QoS characteristics values for V2X services (To: RAN1, RAN3 / Cc: RAN2) R3-193258 @RAN3#104: Reply LS on Combinations of Uu QoS characteristics values for V2X services (To: RAN1 / Cc: SA2, RAN2)

3.2	SA3

3.2.0	SA3 FS_eV2X_Sec status – general

FS_eV2X_Sec (Study on security aspects of 3GPP support for advanced V2X services) was approved in SA#83 (March 2019). In SA3#95 (May 2019), several key issues were agreed, including security and privacy of unicast/groupcast over PC5, and protection of UE authorization (available in TR 33.836 v0.1.0).

3.2.1	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

Regarding security aspects for PC5 unicast communication, SA3 sent the following Reply LS to RAN2 and SA2 in order to make inquiries about the related information and get answers because SA3’s progress on these aspects are dependent on procedures and information elements to be agreed upon in RAN2 and SA2.

S3-191622: Reply LS on protection of PC5-RRC messages for sidelink unicast communication (To: RAN2, SA2)

3.3	CT WGs

3.3.0	CT WGs eV2X status – general

eV2XARC (CT aspects of architecture enhancements for 3GPP support of advanced V2X services) was approved in CT1#116 (April 2019) – C1-192460 and then the revision was approved in CT1#117 (May 2019) – C1-193208. This work item have impacts on all CT WGs, i.e. CT1, CT3, CT4 and CT6. The work has been started in CT1#117 (May 2019).

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#96bis

- R1-1903943	Sidelink physical layer structure for NR V2X	Huawei, HiSilicon

- R1-1903944	Sidelink physical layer procedures for NR V2X	Huawei, HiSilicon

- R1-1903945	Sidelink reference signal design for NR V2X	Huawei, HiSilicon

- R1-1903946	Sidelink synchronization mechanisms for NR V2X	Huawei, HiSilicon

- R1-1903947	Sidelink resource allocation mode 2  for NR V2X	Huawei, HiSilicon

- R1-1903948	System level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1903949	Link level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1903950	Sidelink resource allocation mode 1	Huawei, HiSilicon

- R1-1903951	NR Uu control for LTE sidelink	Huawei, HiSilicon

- R1-1903952	QoS management for NR sidelink	Huawei, HiSilicon

- R1-1903953	In-device coexistence between LTE and NR sidelinks	Huawei, HiSilicon

- R1-1904072	Physical layer structure for NR sidelink	vivo

- R1-1904073	Discussion on mode 1 resource allocation mechanism	vivo

- R1-1904074	Discussion on mode 2 resource allocation mechanism	vivo

- R1-1904075	Sidelink synchronization mechanism	vivo

- R1-1904076	In-device coexistence between NR and LTE sidelinks	vivo

- R1-1904077	Physical layer procedure for NR sidelink	vivo

- R1-1904078	QoS management for sidelink	vivo

- R1-1904079	Support of NR Uu controlling LTE sidelink	vivo

- R1-1904080	Discussion on RLM in NR sidelink	vivo

- R1-1904226	V2X antenna model and pathloss correction	Fraunhofer HHI, Fraunhofer IIS, Huawei, HiSilicon

- R1-1904255	Discussion on physical layer structure for NR sidelink	Sony

- R1-1904256	Discussion on NR V2X mode 2 resource allocation	Sony

- R1-1904257	Discussion on HARQ feedback for NR V2X communication	Sony

- R1-1904294	Sidelink physical structure design for NR V2X communication	Intel Corporation

- R1-1904295	Network controlled sidelink resource allocation design for NR V2X communication	Intel Corporation

- R1-1904296	Sidelink resource allocation design for NR V2X communication	Intel Corporation

- R1-1904297	Sidelink synchronization design for NR V2X communication	Intel Corporation

- R1-1904298	In-device coexistence for LTE V2X and NR V2X sidelink communication	Intel Corporation

- R1-1904299	Design of physical layer procedures for NR V2X sidelink communication	Intel Corporation

- R1-1904300	QoS aware congestion control for NR V2X sidelink communication	Intel Corporation

- R1-1904301	Design of NR Uu controlling LTE sidelink	Intel Corporation

- R1-1904328	Design of NR V2X Physical Layer Structures	Fraunhofer HHI, Fraunhofer IIS

- R1-1904329	Resource Allocation for Mode 1 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-1904330	Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-1904331	Physical Layer Procedures for NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-1904419	On Physical Layer Structures for NR V2X	Samsung

- R1-1904420	On Resource Allocation for NR V2X Mode 1	Samsung

- R1-1904421	On Resource Allocation for NR V2X Mode 2	Samsung

- R1-1904422	System Level Evaluations for NR V2X Sidelink Resource Allocation	Samsung

- R1-1904423	Considerations on Channel Structures in Sensing Slots for NR V2X Sidelink	Samsung

- R1-1904424	On Synchronization Mechanisms for NR V2X	Samsung

- R1-1904425	On Coexistence between LTE and NR PC5	Samsung

- R1-1904426	On Physical Layer Procedures for NR V2X	Samsung

- R1-1904427	On QoS Management for NR V2X	Samsung

- R1-1904428	On NR Uu controlling LTE sidelink	Samsung

- R1-1904429	On AGC settling issues with evaluation results for NR V2X	Samsung

- R1-1904430	On Sidelink RLM	Samsung

- R1-1904431	On DMRS Design for NR V2X	Samsung

- R1-1904432	On Sidelink Feedback Channel Format	Samsung

- R1-1904433	On Sidelink HARQ Procedure	Samsung

- R1-1904434	On Sidelink CSI Procedure	Samsung

- R1-1904435	On Sidelink Power Control	Samsung

- R1-1904492	On sidelink physical layer structure	MediaTek Inc.

- R1-1904493	On NR SL mode-1 resource allocation	MediaTek Inc.

- R1-1904494	On sidelink resource allocation mechanism	MediaTek Inc.

- R1-1904495	Discussion on sidelink based synchronization mechanism	MediaTek Inc.

- R1-1904496	In-device coexistence of NR V2X and LTE V2X	MediaTek Inc.

- R1-1904497	Discussion on V2X Physical layer procedure	MediaTek Inc.

- R1-1904498	Discussion on NR Uu configuring LTE sidelink	MediaTek Inc.

- R1-1904499	Enhancements to UE assistance information	MediaTek Inc.

- R1-1904575	Discussion on physical layer structure for NR sidelink	Lenovo, Motorola Mobility

- R1-1904576	Discussion on resource allocation for NR sidelink Mode 1	Lenovo, Motorola Mobility

- R1-1904577	Discussion on resource allocation for NR sidelink Mode 2	Lenovo, Motorola Mobility

- R1-1904578	In-device coexistence between LTE and NR sidelinks	Lenovo, Motorola Mobility

- R1-1904579	Discussion on physical layer procedures for NR sidelink	Lenovo, Motorola Mobility

- R1-1904580	Discussion on support of NR Uu controlling LTE sidelink	Lenovo, Motorola Mobility

- R1-1904652	Physical layer structure for NR sidelink	NEC

- R1-1904653	Mode 1 resource allocation mechanism for NR sidelink	NEC

- R1-1904654	Mode 2 resource allocation mechanism for NR sidelink	NEC

- R1-1904655	Synchronization mechanism for NR sidelink	NEC

- R1-1904656	Physical layer procedures for NR sidelink	NEC

- R1-1904657	Discussion the prioritization between NR and LTE sidelinks	NEC

- R1-1904666	Discussion on NR V2X Sidelink Retransmission	ETRI

- R1-1904667	Discussion on NR V2X sidelink synchronization	ETRI

- R1-1904679	Discussion on Mode 1 Resource Allocation	ASUSTEK COMPUTER (SHANGHAI)

- R1-1904680	Discussion on sidelink physical layer procedure on NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1904687	Frame and slot structure for sidelink	Huawei, HiSilicon

- R1-1904688	Sidelink CSI	Huawei, HiSilicon

- R1-1904689	Design and contents of PSCCH and PSFCH	Huawei, HiSilicon

- R1-1904690	BWP operation for V2X sidelink	Huawei, HiSilicon

- R1-1904691	Power control and power sharing for V2X sidelink	Huawei, HiSilicon

- R1-1904692	Sidelink measurements	Huawei, HiSilicon

- R1-1904729	Discussion on sidelink resource allocation mechanism	CMCC

- R1-1904730	Discussion on HARQ feedback for NR V2X	CMCC

- R1-1904731	Discussion on support of NR Uu controlling LTE sidelink	CMCC

- R1-1904791	Considerations on sidelink QoS management	Spreadtrum Communications

- R1-1904792	Discussion on synchronization mechanism for NR V2X	Spreadtrum Communications

- R1-1904793	Discussion on physical layer structure for sidelink	Spreadtrum Communications

- R1-1904794	Discussion on physical layer procedures for sidelink	Spreadtrum Communications

- R1-1904795	Discussion on NR sidelink mode 1 resource allocation	Spreadtrum Communications

- R1-1904796	Discussion on NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-1904805	Physical layer structure for sidelink	TCL Communication Ltd.

- R1-1904806	Resource allocation for NR sidelink Mode 1	TCL Communication Ltd.

- R1-1904807	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-1904808	Sidelink synchronization mechanism	TCL Communication Ltd.

- R1-1904809	Physical layer procedures for sidelink	TCL Communication Ltd.

- R1-1904814	NR sidelink physical layer structure	ZTE, Sanechips

- R1-1904815	Discussion on PHY procedures for sidelink	ZTE, Sanechips

- R1-1904816	Discussion on synchronization mechanism in NR V2X	ZTE, Sanechips

- R1-1904817	Discussion on QoS management	ZTE, Sanechips

- R1-1904818	Discussion on UE reporting	ZTE, Sanechips

- R1-1904821	Resource Allocation Mode 2 Mechanism	Kyocera

- R1-1904822	Physical Layer Procedures for CSI Acquisition in Unicast	Kyocera

- R1-1904906	On Physical Layer Structure for NR sidelink	China Telecommunications

- R1-1904917	Physical layer structure for NR-V2X	OPPO

- R1-1904918	Mode 1 resource allocation for NR SL	OPPO

- R1-1904919	Discussion of synchronization mechanism for NR-V2X	OPPO

- R1-1904920	Physical layer procedure for NR-V2X sidelink	OPPO

- R1-1904921	Discussion of RLM for Unicast	OPPO

- R1-1904922	Support of mode 2a resource allocation for NR-V2X	OPPO

- R1-1904923	Discussions on NR and LTE V2X in-device coexistence	OPPO

- R1-1904924	Considerations on congestion control for NR-V2X	OPPO

- R1-1904925	On support of NR Uu controlling LTE sidelink	OPPO

- R1-1904975	Considerations on NR V2X mode 2 resource allocation mechanism	Apple Inc.

- R1-1904976	Considerations on NR V2X synchronization mechanism	Apple Inc.

- R1-1904977	Considerations on NR V2X physical layer procedures	Apple Inc.

- R1-1904978	Considerations on NR V2X scheduling mechanism	Apple Inc.

- R1-1905008	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1905009	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated

- R1-1905010	Synchronization design for NR V2X	Qualcomm Incorporated

- R1-1905011	Co-existence aspects for NR-V2X and LTE-V2X	Qualcomm Incorporated

- R1-1905012	Physical layer procedures for sidelibk	Qualcomm Incorporated

- R1-1905013	QoS management and congestion control for sidelink	Qualcomm Incorporated

- R1-1905014	NR Uu control of LTE sidelibk	Qualcomm Incorporated

- R1-1905072	Discussion on sidelink resource allocation	Asia Pacific Telecom co. Ltd

- R1-1905076	NR Sidelink Resource Allocations using Mode 1	ITRI

- R1-1905077	Discussion on NR Sidelink Resource Allocation for Mode 2	ITRI

- R1-1905078	On Sidelink Synchronization Issues and Mechanisms for NR V2X	ITRI

- R1-1905079	Discussion on NR V2X Sidelink Physical Procedures	ITRI

- R1-1905080	Discussion on QoS management for sidelink	ITRI

- R1-1905081	The discussions on NR-Uu assisted LTE sidelink in V2X communications	ITRI

- R1-1905084	Discussion on Mode 2 resource allocation in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1905088	Discussion on sidelink structure in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1905099	Physical layer structure for NR V2X	ITL

- R1-1905101	Physical layer procedure for NR V2X	ITL

- R1-1905123	Views on physical layer design for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1905124	Discussion on sidelink synchronization mechanism for NR V2X	ITL

- R1-1905155	Consideration on Mode 2 resource allocation for NR sidelink	KT Corp.

- R1-1905257	On synchronization procedures for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1905298	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1905299	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated

- R1-1905300	Synchronization design for NR V2X	Qualcomm Incorporated

- R1-1905301	Co-existence aspects for NR-V2X and LTE-V2X	Qualcomm Incorporated

- R1-1905302	Physical layer procedures for sidelibk	Qualcomm Incorporated

- R1-1905303	QoS management and congestion control for sidelink	Qualcomm Incorporated

- R1-1905304	NR Uu control of LTE sidelibk	Qualcomm Incorporated

- R1-1905329	Discussion on AS level link management for unicast	ZTE, Sanechips

- R1-1905332	Discussion of physical layer structure for sidelink	Nokia, Nokia Shanghai Bell

- R1-1905333	Discussion of Resource allocation for sidelink - Mode 1	Nokia, Nokia Shanghai Bell

- R1-1905334	Discussion of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

- R1-1905335	Discussion of Sidelink synchronization mechanism	Nokia, Nokia Shanghai Bell

- R1-1905336	Discussion of In-device coexistence between LTE and NR sidelinks	Nokia, Nokia Shanghai Bell

- R1-1905337	Discussion of Physical layer procedures for sidelink	Nokia, Nokia Shanghai Bell

- R1-1905338	Discussion of QoS management for sidelink	Nokia, Nokia Shanghai Bell

- R1-1905339	Discussion of Support of NR Uu configuring LTE sidelink	Nokia, Nokia Shanghai Bell

- R1-1905340	Discussion on Sidelink groupcast HARQ	Nokia, Nokia Shanghai Bell

- R1-1905341	Mode 1 resource allocation schemes on sidelink	ZTE, Sanechips

- R1-1905342	Mode 2 Resource allocation schemes on sidelink	ZTE, Sanechips

- R1-1905343	Simulation of mode 2 Resource allocation schemes	ZTE, Sanechips

- R1-1905344	In-device coexistence between NR V2X and LTE V2X	ZTE, Sanechips

- R1-1905345	Discussion on NR Uu controlling LTE sidelink	ZTE, Sanechips

- R1-1905351	Discussion on physical layer structure in NR V2X	CATT

- R1-1905352	Discussion on resource allocation mechanism for sidelink Mode 1 in NR V2X	CATT

- R1-1905353	Discussion on resource allocation mechanism for sidelink Mode 2 in NR V2X	CATT

- R1-1905354	On design and evaluation of S-SSB for NR V2X Sidelink	CATT

- R1-1905355	Discussion on in-device coexistence of LTE and NR sidelink in NR V2X	CATT

- R1-1905356	Discussion on physical layer procedures for sidelink in NR V2X	CATT

- R1-1905357	Discussion on QoS management for sidelink in NR V2X	CATT

- R1-1905358	Discussion on support of NR Uu controlling LTE sidelink in NR V2X	CATT

- R1-1905374	Discussion on physical layer structure for NR sidelink	Fujitsu

- R1-1905375	Geographic Information based Dynamic TFRP Resource Selection Procedure in NR-V2X	Fujitsu

- R1-1905376	Discussion on Reservation and Sensing based Resource Selection Methods for NR-V2X Sidelink Communication	Fujitsu

- R1-1905377	Dynamic Sidelink Bi-mode Transmission under gNB Configuration	Fujitsu

- R1-1905378	Further Study on TX-RX Distance based HARQ Feedback	Fujitsu

- R1-1905379	Other aspects on physical layer procedures for NR-V2X	Fujitsu

- R1-1905380	Discussion on physical layer structure for sidelink in NR V2X	Panasonic

- R1-1905381	Discussion on resource allocation mechanism of mode 2 in NR V2X	Panasonic

- R1-1905382	Discussion on physical layer procedures for sidelink in NR V2X	Panasonic

- R1-1905387	Sidelink synchronization mechanism for NR V2X	Sequans Communications

- R1-1905388	On HARQ procedure for NR sidelink	Sequans Communications

- R1-1905389	On TR 37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR”	Huawei

- R1-1905391	Physical layer structure for NR sidelink	Sharp

- R1-1905392	Resource allocation mode 1 for NR sidelink	Sharp

- R1-1905393	Resource allocation mode 2 for NR sidelink	Sharp

- R1-1905394	Synchronization mechanism for NR sidelink	Sharp

- R1-1905395	Physical layer procedures for NR sidelink	Sharp

- R1-1905400	On Physical Layer Structure for NR V2X Sidelink	InterDigital, Inc.

- R1-1905401	NR Sidelink Resource Allocation for Mode 1	InterDigital, Inc.

- R1-1905402	NR Sidelink Resource Allocation Mechanism for Mode 2	InterDigital, Inc.

- R1-1905403	Synchronization Signals Evaluations and Procedures for NR V2X Sidelink	InterDigital, Inc.

- R1-1905404	In-device Coexistence of LTE and NR V2X Sidelinks	InterDigital, Inc.

- R1-1905405	On Physical Layer Procedures for NR V2X Sidelink	InterDigital, Inc.

- R1-1905406	Congestion control and QoS Management for NR-V2X	InterDigital, Inc.

- R1-1905407	Discussion on NR Uu Controlling LTE Sidelink	InterDigital, Inc.

- R1-1905408	Sidelink RLM/RLF for NR V2X	InterDigital, Inc.

- R1-1905412	Discussion on physical layer structure for 5G V2X	Xiaomi Communications

- R1-1905413	On Mode 2 resource allocation of V2x communications	Xiaomi Communications

- R1-1905414	On Synchronization for 5G V2X	Xiaomi Communications

- R1-1905415	On HARQ operation for 5G V2x communications	Xiaomi Communications

- R1-1905416	System-level evaluation about HARQ feedback in NR V2X groupcast	ZTE, Sanechips

- R1-1905420	Considerations on transmission type and HARQ feedback timing for multiple SL grants	CAICT

- R1-1905421	NR Sidelink Physical Layer Structure	NTT DOCOMO, INC.

- R1-1905422	NR Sidelink Resource Allocation Mechanism Mode 1	NTT DOCOMO, INC.

- R1-1905423	NR Sidelink Resource Allocation Mechanism Mode 2	NTT DOCOMO, INC.

- R1-1905424	NR Sidelink Synchronization Mechanism	NTT DOCOMO, INC.

- R1-1905425	NR Sidelink Physical Layer Procedure	NTT DOCOMO, INC.

- R1-1905426	Congestion Control for NR Sidelink	NTT DOCOMO, INC.

- R1-1905427	Support of NR Uu Controlling LTE Sidelink	NTT DOCOMO, INC.

- R1-1905435	Support of NR Uu controlling LTE sidelink	Panasonic Corporation

- R1-1905436	Work plan for WI on 5G V2X with NR sidelink	LG Electronics, Huawei

- R1-1905437	Discussion on physical layer structure for NR sidelink	LG Electronics

- R1-1905438	Discussion on resource allocation for NR sidelink Mode 1	LG Electronics

- R1-1905439	Discussion on resource allocation for NR sidelink Mode 2	LG Electronics

- R1-1905440	Discussion on simultaneous configuration of Mode 1 and Mode 2 for a UE	LG Electronics

- R1-1905441	Discussion on NR sidelink synchronization mechanism	LG Electronics

- R1-1905442	Discussion on in-device coexistence between LTE and NR sidelinks	LG Electronics, General Motors, Volkswagen AG

- R1-1905443	Discussion on physical layer procedure for NR sidelink	LG Electronics

- R1-1905444	Discussion on QoS management for NR sidelink	LG Electronics

- R1-1905445	Discussion on NR Uu controlling LTE sidelink	LG Electronics

- R1-1905446	Discussion on AS level link management for unicast	LG Electronics

- R1-1905447	Simulation profile update for including 2 RX antenna and high congestion scenario	LG Electronics, General Motors, Volkswagen AG, Hyundai Motors

- R1-1905450	Details of LTE Uu configuration of NR sidelink	Huawei, HiSilicon

- R1-1905475	PHY layer structure for NR sidelink	Ericsson

- R1-1905476	Uu-based sidelink resource allocation	Ericsson

- R1-1905477	Resource allocation for Mode-2 transmissions	Ericsson

- R1-1905478	On Synchronization procedures for V2X	Ericsson

- R1-1905479	In-device coexistence between LTE and NR sidelinks	Ericsson

- R1-1905480	Physical layer procedures for sidelink	Ericsson

- R1-1905481	On congestion control for NR SL	Ericsson

- R1-1905482	NR Uu controlling LTE sidelink transmissions	Ericsson

- R1-1905490	On SCI and SFCI formats	Ericsson

- R1-1905491	Pre-emption in resource allocation	Ericsson

- R1-1905492	On Mode-1 HARQ indication to gNB	Ericsson

- R1-1905493	Link level evaluations of NR PSCCH	Ericsson

- R1-1905494	Link level evaluations of NR PSSCH	Ericsson

- R1-1905495	On data–DMRS collision for SL	Ericsson

- R1-1905496	On L1 measurements for NR sidelink	Ericsson

- R1-1905497	On NR sidelink resources and short-slot format	Ericsson

- R1-1905498	On link adaptation for sidelink groupcast	Ericsson

- R1-1905499	Detailed system-level evaluation results	Ericsson

- R1-1905500	Simultaneous operation of Mode 1 and Mode 2	Ericsson

- R1-1905501	On Layer 1 IDs	Ericsson

- R1-1905502	UE reports for NR V2X	Ericsson

- R1-1905503	On Details of S-SSB design	Ericsson

- R1-1905504	[Draft] LS on inter-RAT mapping of QoS parameters for in-device coexistence	Ericsson

- R1-1905505	[Draft] Reply LS on SL RLM / RLF in NR V2X for unicast	Ericsson

- R1-1905506	On SL RLM / RLF in NR V2X for unicast	Ericsson

- R1-1905507	Updated simulation assumptions	Ericsson

- R1-1905620	Feature Lead Summary for NR-V2X AI - 7.2.4.8 Others	InterDigital

- R1-1905622	Feature Lead Summary for 7.2.4.6 QoS management	Nokia

- R1-1905623	Feature lead summary on AI 7.2.4.3 Sidelink synchronization mechanism	CATT

- R1-1905624	Feature lead summary for agenda item 7.2.4.5 Physical layer procedures for sidelink	LG Electronics

- R1-1905632	Feature lead summary for 7.2.4.1 Physical layer structure for side link	Samsung

- R1-1905650	Feature lead summary on Resource allocation for NR sidelink. Mode 1	Ericsson

- R1-1905652	Summary for AI 7.2.4.2.2 Mode-2 Resource Allocation	Intel Corporation

- R1-1905659	Feature lead summary for AI 7.2.4.7: NR control for LTE sidelink	Huawei

- R1-1905665	Feature lead summary for 7.2.4.1 Physical layer structure for side link	Samsung

- R1-1905711	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1905717	Summary for AI 7.2.4.2.2 Mode-2 Resource Allocation	Intel Corporation

- R1-1905722	Summary of Coexistence Aspects in NR-V2X	Qualcomm

- R1-1905723	Evaluation on number of SSID	Huawei

- R1-1905725	Feature lead summary#2 for 7.2.4.1 Physical layer structure for side link	Samsung

- R1-1905754	Simulation profile update for including 2 RX antenna and high congestion scenario	LG Electronics, General Motors, Volkswagen AG, Hyundai Motors, TOYOTA InfoTechnology Center

- R1-1905761	System-level evaluations on sidelink for NR V2X with 2Rx and 4Rx	Huawei, HiSilicon

- R1-1905762	Feature lead summary for AI 7.2.4.7: NR control for LTE sidelink	Huawei

- R1-1905763	Feature Lead Summary#2 for NR-V2X AI - 7.2.4.8 Others	InterDigital

- R1-1905788	[Draft] Reply LS on SL RLM / RLF in NR V2X for unicast	InterDigital

- R1-1905790	[Draft] LS on sidelink HARQ feedback for groupcast	LG Electronics

- R1-1905824	On Synchronization Mechanisms for NR V2X	Samsung

- R1-1905834	Feature lead summary#2 on Resource allocation for NR sidelink. Mode 1	Ericsson

- R1-1905851	Proposal for additionnal agreement on HARQ feedback in groupcast	LG Electronics

- R1-1905853	L1 source and destination IDs for V2X	Huawei, HiSilicon

- R1-1905854	On RLM and RLF in V2X	Huawei, HiSilicon

- R1-1905863	Reply LS on SL RLM / RLF in NR V2X for unicast	RAN1, InterDigital

- R1-1905877	Feature lead summary#3 for 7.2.4.1 Physical layer structure for side link	Samsung

- R1-1905892	Feature lead summary#2 for agenda item 7.2.4.5 Physical layer procedures for sidelink	LG Electronics

- R1-1905897	On antenna management for NR V2X	Huawei, HiSilicon

- R1-1905898	Simultaneous Mode 1 and Mode 2 operation for NR sidelink	Huawei, HiSilicon

- R1-1905899	PSFCH formats for NR V2X	Huawei, HiSilicon

- R1-1905906	LS on sidelink HARQ feedback for groupcast	RAN1, LG Electronics

- R1-1905913	V2X antenna model and pathloss correction	Fraunhofer HHI, Fraunhofer IIS, Huawei, HiSilicon

RAN1#97

- R1-1906007	Sidelink physical layer structure for NR V2X	Huawei, HiSilicon

- R1-1906008	Sidelink physical layer procedures for NR V2X	Huawei, HiSilicon

- R1-1906009	Sidelink reference signal design for NR V2X	Huawei, HiSilicon

- R1-1906010	Sidelink resource allocation mode 1	Huawei, HiSilicon

- R1-1906011	Sidelink resource allocation mode 2  for NR V2X	Huawei, HiSilicon

- R1-1906012	Sidelink synchronization mechanisms for NR V2X	Huawei, HiSilicon

- R1-1906013	System level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1906014	Link level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1906015	In-device coexistence between LTE and NR sidelinks	Huawei, HiSilicon

- R1-1906016	QoS management for congestion control in NR V2X	Huawei, HiSilicon

- R1-1906074	Discussion of physical layer structure for sidelink	Nokia, Nokia Shanghai Bell

- R1-1906075	Discussion of Resource allocation for sidelink - Mode 1	Nokia, Nokia Shanghai Bell

- R1-1906076	Discussion of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

- R1-1906077	Discussion of Sidelink synchronization mechanism	Nokia, Nokia Shanghai Bell

- R1-1906078	Discussion of In-device coexistence between LTE and NR sidelinks	Nokia, Nokia Shanghai Bell

- R1-1906079	Discussion of Physical layer procedures for sidelink	Nokia, Nokia Shanghai Bell

- R1-1906080	Discussion of QoS management for sidelink	Nokia, Nokia Shanghai Bell

- R1-1906081	Discussion of Support of NR Uu configuring LTE sidelink	Nokia, Nokia Shanghai Bell

- R1-1906137	Physical layer structure for NR sidelink	vivo

- R1-1906138	Discussion on mode 1 resource allocation mechanism	vivo

- R1-1906139	Discussion on mode 2 resource allocation mechanism	vivo

- R1-1906140	Sidelink synchronization mechanism	vivo

- R1-1906141	In-device coexistence between NR and LTE sidelinks	vivo

- R1-1906142	Physical layer procedure for NR sidelink	vivo

- R1-1906143	QoS management for sidelink	vivo

- R1-1906144	Support of NR Uu controlling LTE sidelink	vivo

- R1-1906145	Discussion on RLM in NR sidelink	vivo

- R1-1906205	NR Sidelink Physical Layer Structure	NTT DOCOMO, INC.

- R1-1906206	NR Sidelink Resource Allocation Mechanism Mode 1	NTT DOCOMO, INC.

- R1-1906207	NR Sidelink Resource Allocation Mechanism Mode 2	NTT DOCOMO, INC.

- R1-1906208	NR Sidelink Synchronization Mechanism	NTT DOCOMO, INC.

- R1-1906209	NR Sidelink Physical Layer Procedure	NTT DOCOMO, INC.

- R1-1906210	Congestion Control for NR Sidelink	NTT DOCOMO, INC.

- R1-1906267	Discussion on physical layer structure for NR sidelink	Lenovo, Motorola Mobility

- R1-1906268	Discussion on resource allocation for NR sidelink Mode 1	Lenovo, Motorola Mobility

- R1-1906269	Discussion on resource allocation for NR sidelink Mode 2	Lenovo, Motorola Mobility

- R1-1906270	In-device coexistence between LTE and NR sidelinks	Lenovo, Motorola Mobility

- R1-1906271	Discussion on physical layer procedures for NR sidelink	Lenovo, Motorola Mobility

- R1-1906272	Discussion on support of NR Uu controlling LTE sidelink	Lenovo, Motorola Mobility

- R1-1906314	Sidelink physical layer structure in NR V2X	CATT

- R1-1906315	On Mode 1 Resource allocation in NR V2X	CATT

- R1-1906316	On Mode 2 Resource allocation in NR V2X	CATT

- R1-1906317	Sidelink synchronization mechanism in NR V2X	CATT

- R1-1906318	In-device coexistence of LTE and NR sidelink	CATT

- R1-1906319	Sidelink physical layer procedures in NR V2X	CATT

- R1-1906320	QoS management in NR V2X	CATT

- R1-1906321	On NR Uu controlling LTE sidelink	CATT

- R1-1906322	Radio link monitoring of sidelink unicast communication	CATT

- R1-1906361	Discussion on physical layer structure for sidelink	Spreadtrum Communications

- R1-1906362	Discussion on physical layer procedures for sidelink	Spreadtrum Communications

- R1-1906363	Discussion on NR sidelink mode 1 resource allocation	Spreadtrum Communications

- R1-1906364	Discussion on synchronization mechanism for NR V2X	Spreadtrum Communications

- R1-1906365	Considerations on sidelink congestion control	Spreadtrum Communications

- R1-1906368	Discussion on NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-1906390	Physical layer structure for NR sidelink	NEC

- R1-1906391	Mode 1 resource allocation mechanism for NR sidelink	NEC

- R1-1906392	Mode 2 resource allocation mechanism for NR sidelink	NEC

- R1-1906393	Synchronization mechanism for NR sidelink	NEC

- R1-1906394	Physical layer procedures for NR sidelink	NEC

- R1-1906395	In-device coexistence between LTE and NR sidelinks	NEC

- R1-1906396	Discussion on Qos management	NEC

- R1-1906402	Discussion on physical layer structure for sidelink in NR V2X	Panasonic

- R1-1906403	Discussion on resource allocation mechanism of mode 2 in NR V2X	Panasonic

- R1-1906404	Discussion on physical layer procedures for sidelink in NR V2X	Panasonic

- R1-1906436	Discussion on physical layer structure for NR sidelink	Fujitsu

- R1-1906437	Discussion on mode 1 resource allocation for NR V2X	Fujitsu

- R1-1906438	Reservation based Resource Allocation for NR Sidelink	Fujitsu

- R1-1906439	Discussion on Reservation and Sensing based Resource Selection Methods for NR-V2X Sidelink Communication	Fujitsu

- R1-1906440	Dynamic Sidelink Bi-mode Transmission under gNB Configuration	Fujitsu

- R1-1906441	Geographic Information based Dynamic TFRP Resource Selection Procedure in NR-V2X	Fujitsu

- R1-1906442	Further Study on TX-RX Distance based HARQ Feedback	Fujitsu

- R1-1906443	Discussion on QoS management for NR sidelink	Fujitsu

- R1-1906444	Other aspects on physical layer procedures for NR V2X	Fujitsu

- R1-1906457	NR sidelink physical layer structure	ZTE, Sanechips

- R1-1906462	Discussion on synchronization mechanism in NR V2X	ZTE, Sanechips

- R1-1906465	Evaluation on DMRS patterns for PSBCH	ZTE, Sanechips

- R1-1906469	Discussion on PHY procedures for sidelink	ZTE, Sanechips

- R1-1906471	Discussion on QoS management	ZTE, Sanechips

- R1-1906472	Physical layer structure for NR-V2X	OPPO

- R1-1906473	Mode 1 resource allocation for NR SL	OPPO

- R1-1906474	Discussion of synchronization mechanism for NR-V2X	OPPO

- R1-1906475	Physical layer procedure for NR-V2X sidelink	OPPO

- R1-1906476	Discussion of RLM for Unicast	OPPO

- R1-1906477	Discussion on mode 2a resource allocation	OPPO

- R1-1906478	Discussion on short time scale TDM for in-device coexistence	OPPO

- R1-1906479	Discussion on congestion control metrics	OPPO

- R1-1906480	Discussion on NR Uu controlling LTE sidelink	OPPO

- R1-1906481	Discussion on AS level link management for unicast	ZTE, Sanechips

- R1-1906482	Discussion on UE reporting	ZTE, Sanechips

- R1-1906515	Discussion on sidelink resource allocation mechanism	CMCC

- R1-1906516	Discussion on HARQ feedback for NR V2X	CMCC

- R1-1906517	Discussion on support of NR Uu controlling LTE sidelink	CMCC, Orange, China Telecom, China Unicom

- R1-1906553	Discussion On sidelink physical layer structure	MediaTek Inc.

- R1-1906554	NR sidelink mode-1 resource allocation	MediaTek Inc.

- R1-1906555	On sidelink mode-2 resource allocation	MediaTek Inc.

- R1-1906556	Discussion on sidelink based synchronization mechanism	MediaTek Inc.

- R1-1906557	On in-device coexistence between LTE and NR sidelinks	MediaTek Inc.

- R1-1906558	Discussion on V2X Physical layer procedure	MediaTek Inc.

- R1-1906559	Discussion on NR Uu controlling LTE sidelink	MediaTek Inc.

- R1-1906560	Discussion on SL RLM	MediaTek Inc.

- R1-1906571	Discussion on physical layer structure for 5G V2X	Xiaomi Communications

- R1-1906572	On Mode 2 resource allocation of V2x communications	Xiaomi Communications

- R1-1906573	On Synchronization for 5G V2X	Xiaomi Communications

- R1-1906574	On HARQ operation for 5G V2x communications	Xiaomi Communications

- R1-1906593	NR Uu control for LTE sidelink	Huawei, HiSilicon

- R1-1906594	Frame and slot structure for sidelink	Huawei, HiSilicon

- R1-1906595	Sidelink CSI	Huawei, HiSilicon

- R1-1906596	Design and contents of PSCCH and PSFCH	Huawei, HiSilicon

- R1-1906597	BWP operation for V2X sidelink	Huawei, HiSilicon

- R1-1906598	Power control and power sharing for V2X sidelink	Huawei, HiSilicon

- R1-1906599	Details of LTE Uu configuration of NR sidelink	Huawei, HiSilicon

- R1-1906600	Antenna aspects for NR V2X	Huawei, HiSilicon

- R1-1906649	Design of NR V2X Physical Layer Structures	Fraunhofer HHI, Fraunhofer IIS

- R1-1906650	Resource Allocation for Mode 1 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-1906651	Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-1906653	Physical Layer Procedures for NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-1906663	TR 37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR”	Huawei

- R1-1906794	Physical structure for NR V2X sidelink communication design	Intel Corporation

- R1-1906795	Design aspects for NR V2X sidelink communication in resource allocation Mode-1	Intel Corporation

- R1-1906796	Sidelink resource allocation Mode-2 design for NR V2X communication	Intel Corporation

- R1-1906797	Synchronization design for NR V2X sidelink communication	Intel Corporation

- R1-1906798	In-device coexistence issues for eV2X communication	Intel Corporation

- R1-1906799	Physical layer procedures for NR V2X sidelink communication	Intel Corporation

- R1-1906800	QoS and congestion control for NR V2X sidelink communication	Intel Corporation

- R1-1906801	Design of NR Uu controlling LTE sidelink	Intel Corporation

- R1-1906839	Discussion on NR V2X mode 2 resource allocation	Sony

- R1-1906840	Discussion on HARQ feedback for NR V2X communication	Sony

- R1-1906880	On Physical Layer Structure for NR sidelink	China Telecommunications

- R1-1906881	On Physical Layer Procedure for NR sidelink	China Telecommunications

- R1-1906934	On Physical Layer Structures for NR V2X	Samsung

- R1-1906935	On Resource Allocation for NR V2X Mode 1	Samsung

- R1-1906936	On Resource Allocation for NR V2X Mode 2	Samsung

- R1-1906937	System Level Evaluations for NR V2X Sidelink Resource Allocation	Samsung

- R1-1906938	Considerations on Channel Structures in Sensing Slots for NR V2X Sidelink	Samsung

- R1-1906939	On Synchronization Mechanisms for NR V2X	Samsung

- R1-1906940	On Coexistence between LTE and NR PC5	Samsung

- R1-1906941	On Physical Layer Procedures for NR V2X	Samsung

- R1-1906942	On QoS Management for NR V2X	Samsung

- R1-1906943	On NR Uu controlling LTE sidelink	Samsung

- R1-1906944	On AGC Settling Issues with Evaluation Results for NR V2X	Samsung

- R1-1906945	On Sidelink RLM/RLF	Samsung

- R1-1906946	On DMRS Design for NR V2X	Samsung

- R1-1906947	On Sidelink Feedback Channel Format	Samsung

- R1-1906948	On Sidelink HARQ Procedure	Samsung

- R1-1906949	On Sidelink CSI Procedure	Samsung

- R1-1906950	On Sidelink Power Control	Samsung

- R1-1907012	Discussion on physical layer structure for NR sidelink	LG Electronics

- R1-1907013	Discussion on resource allocation for NR sidelink Mode 1	LG Electronics

- R1-1907014	Discussion on resource allocation for NR sidelink Mode 2	LG Electronics

- R1-1907015	Discussion on simultaneous configuration of Mode 1 and Mode 2 for a UE	LG Electronics

- R1-1907016	Discussion on NR sidelink synchronization mechanism	LG Electronics

- R1-1907017	Discussion on in-device coexistence between LTE and NR sidelinks	LG Electronics

- R1-1907018	Discussion on physical layer procedure for NR sidelink	LG Electronics

- R1-1907019	Discussion on QoS management for NR sidelink	LG Electronics

- R1-1907020	Discussion on NR Uu controlling LTE sidelink	LG Electronics

- R1-1907021	Discussion on AS level link management for unicast	LG Electronics

- R1-1907022	Proposal for simulation assumption in the congested scenario	LG Electronics

- R1-1907024	Physical layer design for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1907025	On synchronization procedures for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1907037	Discussion on NR V2X Sidelink Retransmission	ETRI

- R1-1907038	Sidelink synchronization mechanism	ETRI

- R1-1907047	Physical layer structure for sidelink	TCL Communication Ltd.

- R1-1907048	Resource allocation for NR sidelink Mode 1	TCL Communication Ltd.

- R1-1907049	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-1907050	Sidelink synchronization mechanism	TCL Communication Ltd.

- R1-1907051	Physical layer procedures for sidelink	TCL Communication Ltd.

- R1-1907082	Sidelink synchronization mechanism for NR V2X	Sequans Communications

- R1-1907083	On HARQ procedure for NR sidelink	Sequans Communications

- R1-1907091	Discussion on Physical Layer Structure for NR V2X Sidelink	InterDigital, Inc.

- R1-1907092	On NR Sidelink Mode 1 Resource Allocation	InterDigital, Inc.

- R1-1907094	NR Sidelink Resource Allocation Mechanism for Mode 2	InterDigital, Inc.

- R1-1907095	Discussion on In-device Coexistence between LTE and NR V2X Sidelinks	InterDigital, Inc.

- R1-1907096	On Physical Layer Procedures for NR V2X Sidelink	InterDigital, Inc.

- R1-1907098	Congestion control and QoS Management for NR-V2X	InterDigital, Inc.

- R1-1907099	On support of NR Uu Controlling LTE Sidelink	InterDigital, Inc.

- R1-1907100	On Sidelink RLM/RLF for NR V2X	InterDigital, Inc.

- R1-1907101	Resource Allocation Mode-2	Kyocera Corporation

- R1-1907103	Physical Layer Procedures for CSI Acquisition in Unicast transmissions	Kyocera Corporation

- R1-1907129	Mode 1 resource allocation schemes on sidelink	ZTE, Sanechips

- R1-1907130	Mode 2 resource allocation schemes on sidelink	ZTE, Sanechips

- R1-1907131	Discussion on NR Uu controlling LTE sidelink	ZTE, Sanechips

- R1-1907132	In-device coexistence between NR V2X and LTE V2X	ZTE, Sanechips

- R1-1907133	Simulation of mode 2 resource schemes	ZTE, Sanechips

- R1-1907134	PHY layer structure for NR sidelink	Ericsson

- R1-1907135	Uu-based sidelink resource allocation	Ericsson

- R1-1907136	Resource allocation for Mode-2 transmissions	Ericsson

- R1-1907137	On CSI report for Mode-1	Ericsson

- R1-1907138	On Mode-1 HARQ indication to gNB	Ericsson

- R1-1907139	Simultaneous operation of Mode 1 and Mode 2	Ericsson

- R1-1907140	UE reports for NR V2X	Ericsson

- R1-1907141	S-SSB design and synchronization protocol for NR SL	Ericsson

- R1-1907142	In-device coexistence between LTE and NR sidelinks	Ericsson

- R1-1907143	Physical layer procedures for sidelink	Ericsson

- R1-1907144	On congestion control for NR SL	Ericsson

- R1-1907145	NR Uu controlling LTE sidelink transmissions	Ericsson

- R1-1907146	On SL RLM / RLF in NR V2X for unicast	Ericsson

- R1-1907147	Link level evaluations of NR PSCCH	Ericsson

- R1-1907148	Link level evaluations of NR PSSCH	Ericsson

- R1-1907149	On data–DMRS collision for SL	Ericsson

- R1-1907150	On L1 measurements for NR sidelink	Ericsson

- R1-1907151	On NR sidelink resources and short-slot format	Ericsson

- R1-1907152	On Layer 1 IDs	Ericsson

- R1-1907153	On SCI contents	Ericsson

- R1-1907164	Physical layer structure design for sidelink	AT&T

- R1-1907165	Resource allocation mode-1 for NR sidelink	AT&T

- R1-1907166	Resource allocation mode-2 for NR sidelink	AT&T

- R1-1907167	Physical layer procedures for sidelink	AT&T

- R1-1907175	Minimum set of ProSe D2D services in Release-16	FirstNet, AT&T

- R1-1907216	Physical layer structure for NR sidelink	Sharp

- R1-1907217	Resource allocation mode 1 for NR sidelink	Sharp

- R1-1907218	Resource allocation mode 2 for NR sidelink	Sharp

- R1-1907219	Synchronization mechanism for NR sidelink	Sharp

- R1-1907220	Physical layer procedures for NR sidelink	Sharp

- R1-1907229	Discussion on Mode 1 Resource Allocation	ASUSTEK COMPUTER (SHANGHAI)

- R1-1907230	Discussion on sidelink physical layer procedure on NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1907231	Considering on NR Sidelink Resource Allocations using Mode 1	ITRI

- R1-1907232	Resource Allocation for NR Sidelink Mode 2	ITRI

- R1-1907233	Discussion on Synchronization Issues and Mechanisms for NR V2X Sidelink	ITRI

- R1-1907234	Physical Layer Procedures for NR V2X Sidelink	ITRI

- R1-1907235	QoS management for sidelink discussion	ITRI

- R1-1907236	The discussions on NR-Uu controlling to LTE sidelink mode-3 and mode-4.	ITRI

- R1-1907269	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1907270	Mode 1 Resource allocation for NR-V2X	Qualcomm Incorporated

- R1-1907271	Sidelink resource allocation m\echanism for NR V2X	Qualcomm Incorporated

- R1-1907272	Synchronization design for NR V2X	Qualcomm Incorporated

- R1-1907273	Co-existence aspects for NR-V2X and LTE-V2X	Qualcomm Incorporated

- R1-1907274	Physical layer procedures for sidelink	Qualcomm Incorporated

- R1-1907275	QoS management and congestion control for sidelink	Qualcomm Incorporated

- R1-1907276	NR Uu control of LTE sidelink	Qualcomm Incorporated

- R1-1907335	Considerations on NR V2X mode 2 resource allocation mechanism	Apple Inc.

- R1-1907336	Considerations on NR V2X synchronization mechanism	Apple Inc.

- R1-1907337	Considerations on NR V2X physical layer procedures	Apple Inc.

- R1-1907338	Considerations on NR V2X scheduling mechanism	Apple Inc.

- R1-1907355	Discussion on sidelink synchronization mechanism for NR V2X	ITL

- R1-1907361	Discussion on sidelink resource allocation mode 2	Asia Pacific Telecom co. Ltd

- R1-1907364	Discussion on NR Sidelink Resource Allocation for Mode 2	National Taiwan University

- R1-1907367	Discussion on sidelink structure in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1907368	Discussion on Mode 2 resource allocation in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1907396	Physical layer structure for NR V2X	ITL

- R1-1907414	Physical layer procedure for NR V2X	ITL

- R1-1907465	Discussion on Sidelink Procedures	Convida Wireless

- R1-1907477	QoS Management for NR sidelink	Fraunhofer IIS, Fraunhofer HHI

- R1-1907480	NR Uu controlling LTE sidelink	Lenovo, Motorola Mobility, Deutsche Telekom

- R1-1907494	On RLM and RLF in NR V2X	Huawei, HiSilicon

- R1-1907558	Discussion on simultaneous configuration of Mode 1 and Mode 2 for a UE	LG Electronics

- R1-1907654	Feature Lead Summary for 7.2.4.6 QoS management	Nokia

- R1-1907655	Feature lead summary on Resource allocation for NR sidelink Mode 1	Ericsson

- R1-1907668	Minimum set of ProSe D2D services in Release-16	FirstNet, AT&T, The UK Home Office, SyncTechno Inc., Softil,  Suomen virveverkko Oy, French MoI, UIC  (201 member consortium), The Police of Netherlands, BMWi

- R1-1907669	Feature lead summary#1 for agenda item 7.2.4.1 Physical layer structure for sidelink	Samsung

- R1-1907682	Feature lead summary for agenda item 7.2.4.5 Physical layer procedures for sidelink	LG Electronics

- R1-1907686	Feature lead summary for AI 7.2.4.7: NR control for LTE sidelink	Huawei

- R1-1907689	Sidelink synchronization mechanism	vivo

- R1-1907690	Discussion on NR sidelink synchronization mechanism	LG Electronics

- R1-1907692	Proposal for simulation assumption in the congested scenario	LG Electronics

- R1-1907699	Summary for AI 7.2.4.2.2 Mode-2 Resource Allocation	Intel Corporation

- R1-1907714	Feature Lead Summary #1 for NR-V2X AI - 7.2.4.8 Others	InterDigital

- R1-1907720	Summary of in-device coexistence aspects in NR-V2X	Qualcomm

- R1-1907721	Feature lead summary on AI 7.2.4.3 Sidelink synchronization mechanism	CATT

- R1-1907755	Feature lead summary#2 for agenda item 7.2.4.1 Physical layer structure for sidelink	Samsung

- R1-1907771	Summary#2 for AI 7.2.4.2.2 Mode-2 Resource Allocation	Intel Corporation

- R1-1907774	Feature lead summary#2 on Resource allocation for NR sidelink Mode 1	Ericsson

- R1-1907789	Discussion on resource allocation for NR sidelink Mode 2	LG Electronics

- R1-1907796	Feature lead summary#2 on AI 7.2.4.3 Sidelink synchronization mechanism	CATT

- R1-1907801	Feature lead summary#2 for AI 7.2.4.7: NR control for LTE sidelink	Huawei

- R1-1907822	Feature lead summary#3 for agenda item 7.2.4.1 Physical layer structure for sidelink	Samsung

- R1-1907823	[DRAFT] Reply LS to SA2 on use of Range parameter in NR sidelink	LG Electronics

- R1-1907826	Draft LS on NR control for LTE sidelink	Huawei

- R1-1907854	Feature lead summary#3 on Resource allocation for NR sidelink Mode 1	Ericsson

- R1-1907888	Summary#3 for AI 7.2.4.2.2 Mode-2 Resource Allocation	Intel Corporation

- R1-1907889	[DRAFT] Reply LS to RAN2 on mode-1 retransmission indication	Ericsson

- R1-1907905	Reply LS to RAN2 on mode-1 retransmission indication	RAN1, Ericsson

- R1-1907906	Physical structure for NR V2X sidelink communication design	Intel Corporation

- R1-1907908	Reply LS to SA2 on use of Range parameter in NR sidelink	RAN1, LG Electronics

- R1-1907909	LS on NR control for LTE sidelink	RAN1, Huawei

- R1-1907916	Feature lead summary#4 on Resource allocation for NR sidelink Mode 1	Ericsson

- R1-1907924	Feature lead summary#4 for agenda item 7.2.4.1 Physical layer structure for sidelink	Samsung

- R1-1907926	Feature lead summary#3 on AI 7.2.4.3 Sidelink synchronization mechanism	CATT

- R1-1907947	Feature lead summary#5 on Resource allocation for NR sidelink Mode 1	Ericsson

RAN2#105bis

- R2-1903002	LS on NR V2X spectrum allocation 	Huawei

- R2-1903040	LS on QoS Support for eV2X over Uu interface 	Huawei

- R2-1904867	Discussion on SA2 LS on QoS support for eV2X over Uu interface	Huawei, Qualcomm incorporated, HiSilicon

- R2-1904868	Draft Reply LS to SA2 on QoS support for eV2X over Uu interface	Huawei

- R2-1905046	Work Plan for 5G V2X with NR Sidelink	LG Electronics Inc

- R2-1904093	SR procedure and configuration for Mode 1	Huawei, HiSilicon

- R2-1903876	On sidelink SR	MediaTek Inc.

- R2-1903209	Discussion on BSR and UAI for mode-1	OPPO

- R2-1903874	On sidelink BSR	MediaTek Inc.

- R2-1903632	Sidelink Configured Grant in NR V2X	vivo

- R2-1904714	Support of Configured SL Grant in Mode 1	Ericsson

- R2-1904276	Coordination of SL Mode 1 resource allocation for NR V2X	Nokia, Nokia Shanghai Bell

- R2-1903165	Resource allocation mode configuration	CATT

- R2-1903168	HARQ retransmission in Mode1	CATT

- R2-1903169	Sidelink SR/BSR in Uu Interface	CATT

- R2-1903273	Impact of Mode 1 Resource Allocation on Uu BWP Operation	Samsung Electronics Co., Ltd

- R2-1903432	Resource Pool Sharing between Mode 1 and Mode 2 UEs	Fraunhofer HHI, Fraunhofer IIS

- R2-1903626	Discussion on mode 1 resource allocation	ZTE, Sanechips

- R2-1903631	Discussion on SR BSR for NR Sidelink mode 1	vivo

- R2-1903699	Discussion on sidelink retransmission indication over Uu	ASUSTeK

- R2-1903722	Prioritization of UL and SL transmission	MediaTek Inc.

- R2-1903788	Discussion on HARQ procedure for mode-1 Sidelink Communication	Spreadtrum

- R2-1903792	Connection establishment for V2X	Spreadtrum Communications

- R2-1903804	Consideration of BSR in NR V2X	Spreadtrum Communications

- R2-1903877	Support indication of the need for TB retransmission	MediaTek Inc.

- R2-1904089	Discussion on the SL BSR for Mode 1	Huawei, HiSilicon

- R2-1904133	UE assistance information for gNB scheduling	vivo

- R2-1904159	SL HARQ operation	Lenovo, Motorola Mobility

- R2-1904216	On Mode 1 Scheduling for NR V2X	InterDigital

- R2-1904457	RRC framework for Sidelink	Lenovo, Motorola Mobility

- R2-1904702	gNB-Scheduled Resource Allocation for Sidelink	Ericsson

- R2-1904713	Scheduling of sidelink retransmission in mode-1	Ericsson

- R2-1904803	UE report to assist gNB scheduling	Samsung R&D Institute UK

- R2-1905038	Sidelink HARQ operation for NR V2X	ITL

- R2-1905047	HARQ Retransmission for NR Sidelink Mode 1	LG Electronics Inc

- R2-1903604	Discussion on NR V2X mode 2 resource allocation	ZTE, Sanechips

- R2-1904875	Considerations on resource management for NR V2X	Huawei, HiSilicon

- R2-1903225	Zone-based resource allocation for NR V2X	MediaTek Inc.

- R2-1903700	Resource pool selection for Mode 2	ASUSTeK

- R2-1904215	Zone Design for NR V2X	InterDigital

- R2-1904217	Resource Pool Aspects for NR V2X	InterDigital

- R2-1904275	Validity area for SL Mode 2 resource allocation in NR V2X	Nokia, Nokia Shanghai Bell

- R2-1904703	Resource configuration per validity area	Ericsson

- R2-1903170	Carrier/Resource (Re-) selection Function in NR V2X Sidelink	CATT

- R2-1903171	Cell (Re-) selection Function in NR V2X Sidelink	CATT

- R2-1903226	Resource pools per cast type	MediaTek Inc.

- R2-1903660	Need for admission control	Intel Corporation

- R2-1904134	Resource pool configuration and selection	vivo

- R2-1904279	Resource announcement and reservation procedure for Mode 2 NR V2X	Nokia, Nokia Shanghai Bell

- R2-1905048	TX resource reselection and carrier reselection for NR SL	LG Electronics Inc

- R2-1903211	Discussion on PC5-RRC for unicast	OPPO

- R2-1905049	Support of multiple PC5-S connections over a PC5-RRC connection	LG Electronics Inc

- R2-1904080	Analysis on connection establishment procedure for SL unicast	Huawei, HiSilicon

- R2-1905052	Protection of PC5-RRC messages	LG Electronics Inc

- R2-1905053	Proposed LS on protection of PC5-RRC messages	LG Electronics Inc

- R2-1903229	Security issues for V2X control plane	MediaTek Inc.

- R2-1903227	Contents and handling of PC5-RRC configuration	MediaTek Inc.

- R2-1903657	UE sidelink configuration for NR V2X	Intel Corporation

- R2-1903172	PC5 RRC for UE capability exchange	CATT

- R2-1903210	Discussion on assistance information for interface selection	OPPO

- R2-1903212	Left issues for group-cast	OPPO

- R2-1903623	Consideration on PC5 RRC procedure for unicast	ZTE, Sanechips

- R2-1903629	Consideration on AS level sidelink management	ZTE, Sanechips

- R2-1903633	Sidelink Groupcast and Broadcast in NR	vivo

- R2-1903634	Signaling procedure details for sidelink unicast connection setup	vivo

- R2-1903656	UE capability information exchange for NR V2X	Intel Corporation

- R2-1903658	On groupcast operation over NR sidelink	Intel Corporation

- R2-1903679	Discussion on RRM of unicast for NR-V2X	OPPO

- R2-1903693	Discussion on SL RRC procedure	Apple

- R2-1903701	AS security on PC5-RRC messages	ASUSTeK

- R2-1903742	PC5 L2/L3 protocols for unicast and groupcast	Kyocera

- R2-1903787	Negotiation of TX resource pool(s) or Carrier(s) for SL unicast	Spreadtrum Communications

- R2-1904332	Further considerations on RAT/interface selection	CMCC

- R2-1904431	Views on the need for PC5 AS Connection	Convida Wireless

- R2-1905176	Views on the need for PC5 AS Connection	Convida Wireless

- R2-1904456	PC5 RRC procedures	Lenovo, Motorola Mobility

- R2-1904500	On the need for PC5-RRC connection establishment	Intel Corporation

- R2-1904717	On the PC5 RRC for unicast NR V2X	Ericsson

- R2-1904721	Discussion on PC5-RRC and PC5-S	Qualcomm Incorporated

- R2-1904795	Further Details on Link Establishment Procedure	Interdigital Asia LLC

- R2-1904876	Considerations on RLM for NR V2X unicast	Huawei, HiSilicon

- R2-1904886	Discussion on SL-RSRP measurement and report procedures in NR SL	LG Electronics

- R2-1904889	Discussion on unicast connection establishment	Beijing Xiaomi Software Tech

- R2-1904903	Discussion on coordination of measurement and report in NR SL	LG Electronics

- R2-1905039	PC5-RRC connection establishment for unicast	ITL

- R2-1905050	Initial PC5-RRC procedure for unicast	LG Electronics Inc

- R2-1905051	Reliable transmission of PC5-RRC messages	LG Electronics Inc

- R2-1905054	Unicast-assisted groupcast transmissions	LG Electronics Inc

- R2-1905060	Consideration on groupcast RRC signalling over NR SL	Huawei, HiSilicon

- R2-1905129	Discussion on PC5-RRC based UE capability transfer procedure for NR V2X unicast	Samsung Electronics

- R2-1905130	Discussion on PC5-RRC based AS-layer configuration procedure for NR V2X unicast	Samsung Electronics

- R2-1905196	QoS Support for NR V2X	Qualcomm Incorporated

- R2-1903174	SLRB configuration procedure	CATT

- R2-1903213	LS on QoS modeling for SL unicast in EPC	OPPO

- R2-1903214	Discussion on QoS for NR-V2X	OPPO

- R2-1903275	SL RBs LCHs and SL RB Configurations for NR Sidelink	Samsung Electronics Co., Ltd

- R2-1903276	MAC PDU Format for NR Sidelink	Samsung Electronics Co., Ltd

- R2-1903280	Discussion on L2/3 protocols procedures and information for QoS support	ITRI

- R2-1903625	Discussion on QoS management for NR V2X	ZTE, Sanechips

- R2-1903635	Congestion control in NR V2X	vivo

- R2-1903636	Discussion on QoS management for NR V2X	vivo

- R2-1903669	UL/SL prioritization for NR V2X	Intel Corporation

- R2-1903702	Discussion on UE assistance information for SLRB configuration	ASUSTeK

- R2-1903723	Admission control for NR V2X	MediaTek Inc.

- R2-1903795	Logical Channel Prioritization Consideration	Spreadtrum Communications

- R2-1904084	Consideration on the admission control in NR SL	Huawei, HiSilicon

- R2-1904088	Discussion on the default DRB and default QoS rule for NR SL	Huawei, HiSilicon

- R2-1904092	Relative QoS handling between NR SL and NR Uu	Huawei, HiSilicon

- R2-1904094	Support of RLC AM for unicast and related SL RB configuration	Huawei, HiSilicon

- R2-1904124	Discussion on QoS management for NR V2X	Lenovo, Motorola Mobility

- R2-1904218	QoS Management for NR V2X	InterDigital	discussion	Rel-16	FS_NR_V2X

- R2-1904274	[DRAFT] Response LS on QoS characteristics for NR V2X over Uu	Nokia, Nokia Shanghai Bell

- R2-1904277	Mobility challenges for NR V2X platooning	Nokia, Nokia Shanghai Bell

- R2-1904278	HARQ feedback for groupcast in the light of minimum required communication range	Nokia, Nokia Shanghai Bell

- R2-1904503	QoS support for NR V2X	Intel Corporation

- R2-1904705	On the sidelink QoS flow and radio bearer	Ericsson

- R2-1904708	Criteria to establish and release radio bearer	Ericsson

- R2-1904709	Impact of per-packet QoS model on RAN	Ericsson

- R2-1904711	On the use of communication range at access stratum	Ericsson

- R2-1904716	Congestion control		Ericsson

- R2-1904804	Unicast SLRB configuration for RRC_IDLE or RRC_INACTIVE UEs	Samsung R&D Institute UK

- R2-1904808	Congestion Control in NR-V2X	Samsung R&D Institute UK

- R2-1904810	SDAP Layer for NR-SL Unicast	Samsung R&D Institute UK

- R2-1904877	QoS monitoring and reporting for NR Sidelink V2X Communications	Huawei, HiSilicon

- R2-1904890	Discussion on sidelink admission control	Beijing Xiaomi Software Tech

- R2-1905032	Considerations on Congestion Control in NR Sidelink for V2X	ITL

- R2-1905055	Priority handling for transmission of a PC5-RRC message		LG Electronics Inc

- R2-1904701	On cross-RAT scheduling	Ericsson

- R2-1903463	Discussion on cross RAT V2X sidelink resource allocation	ZTE Corporation, Sanechips

- R2-1905178	NR Uu control of LTE sidelink	Qualcomm Incorporated

- R2-1903166	System Information Design for V2X	CATT

- R2-1903215	Discussion on Inter-RAT Control for NR-V2X	OPPO

- R2-1903637	Mode 3 SPS grant content and timing	vivo

- R2-1904219	MultiRAT CP Aspects in NR V2X	InterDigital

- R2-1904455	Cross RAT Resource allocation	Lenovo, Motorola Mobility

- R2-1904710	Discussions on NR V2X in MR-DC Scenarios	Ericsson

- R2-1904807	Cross-RAT resource allocation/configuration in MR-DC	Samsung R&D Institute UK

- R2-1904878	The enhancement of Uu to control inter-RAT V2X sidelink	Huawei, HiSilicon

- R2-1904885	Cross-RAT resource scheduling/configuration	Intel Corporation

- R2-1905056	Reporting of Sidelink UE Information for cross-RAT resource allocation	LG Electronics Inc

- R2-1903173	LCP procedure for NR sidelink	CATT

- R2-1903175	SDAP open issues	CATT

- R2-1903176	New triggers for RRC connection establishment and resume	CATT

- R2-1903180	MAC PDU format in PC5	CATT

- R2-1903216	Left issues on MAC for NR-V2X	OPPO

- R2-1903217	Left issues on RLC for NR-V2X	OPPO

- R2-1903218	Left issues on PDCP for NR-V2X	OPPO

- R2-1903219	Left issues on SDAP for NR-V2X	OPPO

- R2-1903220	Discussion on Uu-RRC for NR-V2X	OPPO

- R2-1903274	RRC Connection Initiation Trigger for V2X Sidelink Communication	Samsung Electronics Co., Ltd

- R2-1903277	BWP Aspects of Sidelink and Uu Prioritisation	Samsung Electronics Co., Ltd

- R2-1903278	On Demand SI Acquisition in RRC CONNECTED	Samsung Electronics Co., Ltd

- R2-1903433	Resource Allocation Procedures for NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R2-1903627	Consideration on multi-mode co-existence	ZTE, Sanechips

- R2-1903661	Enhancements to UE assistance and UE sidelink information	Intel Corporation

- R2-1903703	Discussion on UL/SL TX prioritization	ASUSTeK

- R2-1903704	Discussion on scenarios for applying exceptional pool	ASUSTeK

- R2-1903724	Use of communication range on HARQ operation	MediaTek Inc.

- R2-1903765	Different destination service multiplexing in MAC	vivo

- R2-1903777	Discussion on unicast, groupcast and broadcast	Fujitsu

- R2-1903872	On prioritization of sidelink resources	MediaTek Inc.

- R2-1903873	On resource allocation mode	MediaTek Inc.

- R2-1903875	On sidelink LCP	MediaTek Inc.

- R2-1904081	Cell reselection for V2X SL communication in NR	Huawei, HiSilicon

- R2-1904083	Consideration on miscellaneous MAC aspects for NR SL design	Huawei, HiSilicon

- R2-1904086	Discussion about sidelink LCP procedure	Huawei, HiSilicon

- R2-1904087	Discussion on mobility enhancement for mode-1 and mode-2	Huawei, HiSilicon

- R2-1904091	On reception of MAC PDU in NR SL	Huawei, HiSilicon

- R2-1904502	Switching between Mode1 and mode2 for NR V2X	Intel Corporation

- R2-1904698	On the Support of HARQ feedbacks Over Sidelink	Ericsson

- R2-1904700	Miscellaneous MAC components for sidelink	Ericsson

- R2-1904706	On the support of RLC AM for sidelink unicast	Ericsson

- R2-1904707	On lower layer IDs	Ericsson

- R2-1904712	Cell reselection for NR V2X	Ericsson

- R2-1904715	On UE reports for V2X	Ericsson

- R2-1904725	Considerations for sidelink resource pool design for NR V2X	Samsung

- R2-1904805	Sidelink HARQ Configuration	Samsung R&D Institute UK

- R2-1904806	Sidelink RAT Selection	Samsung R&D Institute UK

- R2-1904809	Priority Handling for Sidelink Logical Channel Prioritization	Samsung R&D Institute UK

- R2-1904879	Discussion on HARQ support for NR sidelink	Huawei, HiSilicon

- R2-1904880	Reliability enhancements for NR sidelink broadcast	Huawei, HiSilicon

- R2-1904881	AS coordination for SL groupcast	Huawei, HiSilicon

- R2-1904888	Discussion on resource allocation	Beijing Xiaomi Software Tech

- R2-1904912	RLC for Sidelink	Samsung

- R2-1904913	PDCP Functions for Sidelink	Samsung

- R2-1905125	Reselection priorities handling for NR V2X SL communication	Samsung Electronics

- R2-1905126	Interaction between RRC Connection Resume Condition and RNAU for NR V2X SL Communication	Samsung Electronics

- R2-1905128	Discussion on Sidelink UE Information Initiation Trigger for NR V2X Sidelink Communication	Samsung Electronics

- R2-1904493	Motivation for logical UE split in V2X applications	Autotalks Ltd

- R2-1904494	Motivation for the standalone NR PC5 mode	Autotalks Ltd

- R2-1903177	Support for simultaneous configuration of Mode 1 and Mode 2	CATT

- R2-1903178	Uu and PC5 avalilitity/unavalitity	CATT

- R2-1903179	RLM/RLF declaration in NR V2X Sidelink	CATT

- R2-1903221	Discussion on RRM of unicast for NR-V2X	OPPO

- R2-1903222	Discussion on resource allocation mode for NR-V2X	OPPO

- R2-1903231	Support of VRUs	MediaTek Inc.	discussion	Rel-16

- R2-1903615	Discussion on NR V2X UE operations under different RRC states	ZTE, Sanechips

- R2-1903638	AS level link management for unicast	vivo

- R2-1903639	Discussion on support of simultanous mode 1 and mode 2	vivo

- R2-1903659	On SL interface availability for NR V2X	Intel Corporation

- R2-1903692	Discussion on SL RLM	Apple

- R2-1903705	Discussion on association between sidelink data and resource allocation modes	ASUSTeK

- R2-1904082	Channel mapping for NR SL	Huawei, HiSilicon

- R2-1904085	Discussion about mode existence for NR sidelink	Huawei, HiSilicon

- R2-1904090	On PC5 Availability and Unavailability for NR SL unicast	Huawei, HiSilicon

- R2-1904122	AS level link management for NR V2X unicast	Lenovo, Motorola Mobility

- R2-1904214	Discussion on simultaneous use of mode 1 and mode 2	KT Corp.

- R2-1904220	RAN2 Aspects of Simulataneous Configuration of Mode 1 and Mode 2		InterDigital

- R2-1904221	AS-Level Link Management for NR V2X	InterDigital

- R2-1904223	Discussion on transmitter UE side RLM and RLF in NR SL	LG Electronics France

- R2-1904226	Discussion on RLF Indication from UE to gNB in NR SL	LG Electronics France

- R2-1904458	SL and UL BWP Numerology Mismatch	Lenovo, Motorola Mobility

- R2-1904501	Simultaneous operation of mode1 and mode2 for NR V2X	Intel Corporation

- R2-1904506	AS Level Link Management for Unicast	Convida Wireless

- R2-1904699	Simultaneous configuration of mode 1 and mode 2		Ericsson

- R2-1904704	On the sidelink unicast connection management		Ericsson

- R2-1904718	In-device coexistence betweeen LTE SL and NR SL	Ericsson

- R2-1904727	Considerations for support mobility for NR V2X	Samsung

- R2-1904882	AS-related group communication for platooning	Huawei, HiSilicon

- R2-1905035	Prioritization on the scenarios to be discussed in Rel-16 WI phase	ITL

- R2-1905111	Further consideration on Tx carrier (re)selection in NR V2X	ITL

- R2-1905127	Interaction between AS and Upper layers upon SL RLM/RLF Declaration for NR V2X Unicast	Samsung Electronics

- R2-1905131	Discussion on Support of Vulnerable Road User	Samsung Electronics

- R2-1903603	Discussion on NR V2X congestion control	ZTE, Sanechips

RAN2#106

- R2-1905511	Reply LS on SL RLM / RLF in NR V2X for unicast	 InterDigital

- R2-1905513	LS on sidelink HARQ feedback for groupcast		 LGE

- R2-1905522	Reply LS on QoS Support for eV2X over Uu interface Huawei

- R2-1905536	LS response on unicast, groupcast and broadcast in NR sidelink 	 Ericsson

- R2-1907155	Draft reply LS on Combinations of Uu QoS characteristics values for V2X services	Huawei

- R2-1907157	TP on RAN2 skeleton for TR 37.985 (LTE V2X Part)	Huawei

- R2-1907945	Introduction of 5G V2X with NR Sidelink	LG Electronics France

- R2-1905846	Report of [105bis#33] [NR/V2X] LCP (vivo)	vivo

- R2-1907823	Priority Handling for Sidelink Logical Channel Prioritization	Samsung R&D Institute UK

- R2-1905568	Left issues on MAC for NR-V2X	OPPO

- R2-1907347	On lower layer IDs	Ericsson

- R2-1905847	Uplink and Sidelink transmission prioritization in NR V2X	vivo

- R2-1906541	Discussion on UL/SL TX prioritization	ASUSTeK

- R2-1906337	Prioritization of UL and SL transmission	MediaTek Inc.

- R2-1906427	On HARQ feedback support for groupcast	Intel Corporation

- R2-1906786	RLC for Sidelink	Samsung

- R2-1907445	Single carrier operation versus multiple carrier operation in Rel-16 NR V2X SL	Huawei, HiSilicon

- R2-1905850	PC5 PDCP protocol in NR V2X	vivo

- R2-1905571	Left issues on SDAP for NR-V2X	OPPO

- R2-1907226	Discussion on mode 1 resource allocation	Qualcomm Incorporated

- R2-1906483	Discussion on mode 1 resource request	ZTE, Sanechips

- R2-1905839	Remaining issues on BSR and UAI for NR Sidelink mode 1	vivo

- R2-1907345	On UE reports for V2X	Ericsson

- R2-1905799	Cell (Re-) selection Function in NR V2X Sidelink	CATT

- R2-1906816	Usage of exceptional pool	MediaTek Inc.

- R2-1905569	Left issues on RLC for NR-V2X	OPPO

- R2-1905570	Left issues on PDCP for NR-V2X	OPPO

- R2-1905572	Discussion on Uu-RRC for NR-V2X	OPPO

- R2-1905677	Negotiation of TX resource pool(s) for SL unicast	Spreadtrum Communications

- R2-1905680	Logical Channel Prioritization Consideration	Spreadtrum Communications

- R2-1905723	BWP Aspects of SL and Uu Prioritisation	Samsung Electronics Co., Ltd

- R2-1905725	MAC PDU Format for NR Sidelink	Samsung Electronics Co., Ltd

- R2-1905726	On Demand SI Acquisition in RRC CONNECTED	Samsung Electronics Co., Ltd

- R2-1905727	RRC Connection Initiation Trigger for V2X Sidelink Communication	Samsung Electronics Co., Ltd

- R2-1905801	LCP procedure for NR sidelink	CATT

- R2-1905807	New triggers for RRC connection establishment and resume	CATT

- R2-1905808	MAC PDU format in PC5	CATT

- R2-1905849	PC5 RLC protocol in NR V2X	vivo

- R2-1905851	PC5 SDAP protocol in NR V2X	vivo

- R2-1905852	On the need of exceptional pool in NR V2X	vivo

- R2-1906241	Discussion on resource allocation	Beijing Xiaomi Software Tech

- R2-1906253	Discussion on unicast, groupcast and broadcast	Fujitsu

- R2-1906297	Considerations on layer-2 Destination ID	Intel Corporation

- R2-1906428	Remaining aspects on groupcast operation	Intel Corporation

- R2-1906481	Consideration on HARQ feedback for groupcast	ZTE, Sanechips

- R2-1906487	Consideration on NR V2X UEAssistanceInformation	ZTE, Sanechips	discussion

- R2-1906492	Discussion on NR V2X LCP	ZTE, Sanechips

- R2-1906494	Discussion on NR V2X congestion control	ZTE, Sanechips

- R2-1906542	Discussion on scenarios for applying exceptional pool	ASUSTeK

- R2-1906733	SL HARQ operation	Lenovo, Motorola Mobility

- R2-1906787	PDCP Functions for Sidelink	Samsung

- R2-1906810	On prioritization of sidelink resources	MediaTek Inc.

- R2-1906811	On sidelink LCP	MediaTek Inc.

- R2-1906814	Indication of the cast mode for packet reception	MediaTek Inc.

- R2-1906815	On cell reselection	MediaTek Inc.

- R2-1907111	Resource Allocation Procedures for NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R2-1907145	Apparent contradiction in Zone Configuration	Lenovo, Motorola Mobility

- R2-1907205	Support of HARQ Feedback Options for Groupcast		LG Electronics Inc

- R2-1907207	Proposed LS Response on sidelink HARQ feedback for groupcast	LG Electronics Inc

- R2-1907344	HARQ procedure for SL groupcast	Ericsson

- R2-1907346	Cell reselection for NR V2X	Ericsson

- R2-1907348	On the support of RLC AM for sidelink unicast	Ericsson

- R2-1907349	Miscellaneous MAC components for sidelink	Ericsson

- R2-1907350	On support of SL CSI report	Ericsson

- R2-1907351	On the Support of HARQ feedbacks Over Sidelink	Ericsson

- R2-1907403	Discussion on logical channel prioritization in NR V2X	Qualcomm Incorporated

- R2-1907412	Discussion HARQ mechanism of SL groupcast	Huawei, HiSilicon

- R2-1907414	Discussion on HARQ support for NR sidelink	Huawei, HiSilicon

- R2-1907415	Reliability enhancements for NR sidelink broadcast	Huawei, HiSilicon

- R2-1907416	Discussion on HARQ feedback enable and disable	Huawei, HiSilicon

- R2-1907446	[DRAFT] LS on multiple carrier configuration for Rel-16 NR V2X SL	Huawei

- R2-1907447	Consideration on miscellaneous MAC aspects for NR SL design	Huawei, HiSilicon

- R2-1907461	On reception of MAC PDU in NR SL	Huawei, HiSilicon

- R2-1907818	SL HARQ Configuration	Samsung R&D Institute UK

- R2-1907819	Sidelink RAT Selection	Samsung R&D Institute UK

- R2-1907821	Congestion Control in NR-V2X	Samsung R&D Institute UK

- R2-1907857	Discussion on assistance information for resource allocation in NR SL		LG Electronics

- R2-1907932	Sidelink HARQ operation for NR V2X	ITL

- R2-1907962	Reselection Priorities Handling for NR V2X SL communication	Samsung Electronics

- R2-1907964	Interaction between RRC Connection Resume Condition and RNAU for NR V2X SL Communication	Samsung Electronics

- R2-1907966	Discussion on Sidelink UE Information Initiation Trigger for NR V2X Sidelink Communication	Samsung Electronics

- R2-1907449	Discussion on the SL BSR for Mode 1	Huawei, HiSilicon

- R2-1907450	Further discussion on SR configuration and procedure and for SL Mode-1	Huawei, HiSilicon

- R2-1907413	Discussion on Sidelink Configured Grant support	Huawei, HiSilicon

- R2-1905795	Leftover issues for sidelink configured grant	CATT

- R2-1907204	HARQ Retransmission for NR Sidelink Mode 1	LG Electronics Inc

- R2-1907656	Left issues on retransmission scheduling for NR-V2X	OPPO, Ericsson

- R2-1905796	SR/BSR based HARQ retransmission	CATT

- R2-1906812	Support indication of the need for TB retransmission	MediaTek Inc.

- R2-1907817	The indication for HARQ retransmission to gNB	Samsung R&D Institute UK

- R2-1905560	Discussion for mode-1	OPPO

- R2-1905678	Discussion on HARQ procedure for mode-1 Sidelink Communication	Spreadtrum Communications

- R2-1905679	Connection establishment for V2X	Spreadtrum Communications

- R2-1905681	Consideration of BSR in NR V2X	Spreadtrum Communications

- R2-1905724	Impact of Mode 1 Resource Allocation on Uu BWP Operation	Samsung Electronics Co., Ltd

- R2-1905797	Resource allocation mode configuration	CATT

- R2-1905838	Remaining issues on sidelink configured grant	vivo

- R2-1905840	Remaining issues on SR for NR Sidelink mode 1	vivo

- R2-1906384	Multiple UL/SL Configured Grants and UE Assistance Information	InterDigital

- R2-1906429	MAC related aspects for NR V2X	Intel Corporation

- R2-1906543	Discussion on sidelink retransmission indication over Uu	ASUSTeK

- R2-1906813	Content of UE assistance information	MediaTek Inc.

- R2-1907050	Collision of SL SR and UL-SCH transmission for NR SL Mode 1	SONY

- R2-1907075	Resource Pool Sharing between Mode 1 and Mode 2 UEs	Fraunhofer HHI, Fraunhofer IIS

- R2-1907279	On the deficiencies of indicating the buffer size in SL BSR	Nokia, Nokia Shanghai Bell

- R2-1907281	Coordination of SL Mode 1 resource allocation for NR V2X	Nokia, Nokia Shanghai Bell

- R2-1907282	LS on the inter-NG-RAN node resource coordination for NR V2X	Nokia, Nokia Shanghai Bell

- R2-1907352	Support of Configured SL Grant in Mode 1	Ericsson

- R2-1907353	Draft LS on mode-1 retransmission indication	Ericsson

- R2-1907354	gNB-Scheduled Resource Allocation for Sidelink	Ericsson

- R2-1907448	Discussion on mobility enhancement for mode-1	Huawei, HiSilicon

- R2-1907654	Discussion on resource allocation of mode 1 configured grant for NR V2X	LG Electronics France

- R2-1907711	Discussion on PC5 radio link status based mode 1 resource allocation for NR V2X	LG Electronics France

- R2-1907803	UE report to assist gNB scheduling	Samsung R&D Institute UK

- R2-1907820	LS to RAN1 on NR configured sidelink grant	CATT

- R2-1907858	Discussion on confirmation message for activation/deactivation of configured grant type 2 in NR SL	LG Electronics

- R2-1906480	Discussion on configured grant resource allocation	ZTE, Sanechips

- R2-1906495	Summary of 105bis#31 NR V2X Resource pool configuration and selection (ZTE)	ZTE, Sanechips

- R2-1906385	Resource Pool Aspects for NR V2X	InterDigital

- R2-1907280	Remaining issues on validity area for SL Mode 2 resource allocation in NR V2X	Nokia, Nokia Shanghai Bell

- R2-1905583	Left issues on SIB for NR-V2X	OPPO

- R2-1905798	Resource (Re-) selection Function in NR V2X Sidelink	CATT

- R2-1905848	Tx and Rx resource pool definition for NR V2X Sidelink	vivo

- R2-1906022	Zone-based resource allocation for NR V2X	MediaTek Inc.

- R2-1906023	Resource pools per cast type	MediaTek Inc.

- R2-1906383	Consideration for Geographical Zone Design for NR V2X	InterDigital

- R2-1906430	Need of admission control	Intel Corporation

- R2-1906488	Discussion on NR V2X UE operation under different RRC states	ZTE, Sanechips

- R2-1906490	Discussion on NR V2X resource pool configuration	ZTE, Sanechips

- R2-1906544	Resource pool selection for Mode 2	ASUSTeK

- R2-1906827	NR V2X Mobility Considerations	LG Electronics

- R2-1906837	Resource announcement and reservation procedure for Mode 2 NR V2X	Nokia, Nokia Shanghai Bell

- R2-1907080	Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R2-1907113	Discussion on mobility enhancement for mode-2	Huawei, HiSilicon

- R2-1907206	Reliable transmission of PC5-RRC messages	LG Electronics Inc

- R2-1907208	Unicast-assisted groupcast transmissions	LG Electronics Inc

- R2-1907209	On-demand SI delivery for V2X SIB	LG Electronics Inc

- R2-1907355	On configuration of resource pool	Ericsson

- R2-1907356	Resource configuration per validity area	Ericsson

- R2-1907418	Considerations on handling for intermittent coverage	Huawei, HiSilicon

- R2-1907712	Considerations for sidelink resource pool design for NR V2X	Samsung

- R2-1907826	SL resource pool configuration	Samsung R&D Institute UK

- R2-1907861	Discussion on zone based resource pool configuration in NR SL	LG Electronics

- R2-1907967	Discussion on handling of PC5-S messages for NR V2X unicast	Samsung Electronics

- R2-1905585	Summary of [105bis#32] PC5-RRC signaling		OPPO

- R2-1905562	Discussion on PC5-RRC for unicast	OPPO

- R2-1907658	Discussion on RAN1 Reply LS on SL RLM_RLF for NR V2X	LG Electronics France

- R2-1906387	RLM/RLF for NR V2X	InterDigital

- R2-1906477	Consideration on sidelink RLM management	ZTE, Sanechips

- R2-1905561	Discussion on assistance information for interface selection	OPPO

- R2-1905563	Left issues for group-cast	OPPO

- R2-1905573	Discussion on RRM for NR-V2X	OPPO

- R2-1905574	Discussion on RLM for NR-V2X	OPPO

- R2-1905580	Discussion on network involvement in unicast link establishment	OPPO

- R2-1905582	Discussion on single vs. multiple PC5-RRC connection	OPPO

- R2-1905584	Left issues on SLRB configuration for NR-V2X	OPPO

- R2-1905800	Discussion on AS layer unicast connection setup	CATT

- R2-1905841	Discussion on RAN1 LS  for Sideink radio link management	vivo

- R2-1905842	[Draft] Reply LS to RAN1 on SL RLM/RLF in NR V2X for unicast	vivo

- R2-1905843	Remaining issues on PC5-RRC message exchange	vivo

- R2-1905844	Sidelink Groupcast and Broadcast in NR	vivo

- R2-1905845	Open issues for Sidelink link failure and release	vivo

- R2-1906024	Contents and handling of PC5-RRC configuration	MediaTek Inc.

- R2-1906025	Remaining security issues for V2X control plane	MediaTek Inc.

- R2-1906027	RLM and state modelling based on PC5-S connection	MediaTek Inc.

- R2-1906028	Validity areas based on cell lists	MediaTek Inc.

- R2-1906127	Discussion on PC5 RRC state	Lenovo, Motorola Mobility

- R2-1906128	AS level link management for NR V2X unicast	Lenovo, Motorola Mobility

- R2-1906242	Discussion on unicast connection establishment	Beijing Xiaomi Software Tech

- R2-1906296	PC5-RRC considerations for NR V2X	Intel Corporation

- R2-1906390	Draft Response LS on SL RLM/RLF in NR V2X for unicast	InterDigital

- R2-1906431	UE sidelink configuration for NR V2X	Intel Corporation

- R2-1906479	Consideration on sidelink RRM measurement	ZTE, Sanechips

- R2-1906497	Consideration on PC5 RRC procedure for unicast	ZTE, Sanechips

- R2-1906498	Impact of multiple PC5-S connections	ZTE, Sanechips

- R2-1906659	PC5 L2/L3 protocols for unicast and groupcast	Kyocera

- R2-1906773	Discussion on SL RLM	Apple

- R2-1906774	Discussion on SL RRC procedure	Apple

- R2-1907146	PC5-S and Identification on PC5 RRC and RRC Procedures	Lenovo, Motorola Mobility

- R2-1907180	PC5-RRC state considerations	Kyocera

- R2-1907185	AS Level Link Management for Unicast	Convida Wireless

- R2-1907210	Priority handling for transmission of a PC5-RRC message		LG Electronics Inc

- R2-1907357	On PC5 interface availability	Ericsson

- R2-1907358	On UE capability exchange	Ericsson

- R2-1907359	On the sidelink unicast link management	Ericsson

- R2-1907419	Considerations on RLM for NR V2X unicast	Huawei, HiSilicon

- R2-1907451	On PC5 Availability/Unavailability for NR SL unicast	Huawei, HiSilicon

- R2-1907452	Relationship between PC5-RRC connection and PC5-S connection	Huawei, HiSilicon

- R2-1907644	Discussion on bi-directional PC5-RRC procedure	OPPO	discussion

- R2-1907859	Discussion on coordination of measurement and report in NR SL	LG Electronics

- R2-1907860	Discussion on SL-RSRP measurement and report procedures in NR SL	LG Electronics

- R2-1907946	Discussion on the remaining issues for PC5-RRC and PC5-S	Qualcomm Incorporated

- R2-1907963	Remaining Issues on SL RLM/RLF Declaration for NR V2X Unicast	Samsung Electronics

- R2-1907968	Discussion on SL AS-layer Configuration Failure	Samsung Electronics

- R2-1907629	Discussion on transmitter UE side RLM and RLF in NR SL	LG Electronics France

- R2-1907643	Discussion on RLF Indication from UE to gNB in NR SL	LG Electronics France

- R2-1907454	Details about NR SL QoS handling	Huawei, HiSilicon

- R2-1907364	On the sidelink QoS flow and radio bearer	Ericsson

- R2-1907363	Criteria to establish and release SL QoS flow and radio bearer	Ericsson

- R2-1907579	QoS Support for NR V2X	Qualcomm Incorporated

- R2-1907457	Support of RLC AM for unicast and related SL RB configuration	Huawei, HiSilicon

- R2-1907552	Discussion on the usage of range parameter in NR V2X	Qualcomm Incorporated

- R2-1907362	On the use of communication range at access stratum	Ericsson

- R2-1905564	LS on QoS modeling for SL unicast in EPC	OPPO

- R2-1905565	Discussion on QoS modelling for NR-V2X	OPPO

- R2-1905566	Discussion on QoS mapping for NR-V2X	OPPO

- R2-1905728	SL RBs/LCHs and SL RB Configurations for NR Sidelink	Samsung Electronics Co., Ltd

- R2-1905802	SLRB configuration procedure	CATT

- R2-1905803	UL/SL prioritization	CATT

- R2-1905804	Admission control	CATT

- R2-1905805	SDAP open issues	CATT

- R2-1905853	PC5 RATs prioritization in NR V2X	vivo

- R2-1905854	Draft LS to SA2 on PC5 RATs prioritization in NR V2X	vivo

- R2-1905855	Congestion control in NR V2X	vivo

- R2-1905856	Discussion on QoS management for NR V2X	vivo

- R2-1906139 Discussion on QoS management for NR V2X	Lenovo, Motorola Mobility

- R2-1906243 Discussion on sidelink admission control	Beijing Xiaomi Software Tech

- R2-1906295 QoS support for NR V2X	Intel Corporation

- R2-1906338 Admission control for NR V2X	MediaTek Inc.

- R2-1906339 Use of range parameter	MediaTek Inc.

- R2-1906386 QoS Management for NR V2X	InterDigital

- R2-1906433 UL/SL prioritization for NR V2X	Intel Corporation

- R2-1906496 Discussion on QoS support for NR V2X	ZTE, Sanechips

- R2-1906545 Discussion on UE assistance information for SLRB configuration	ASUSTeK

- R2-1906636 Considerations on Congestion Control in NR Sidelink for V2X	ITL

- R2-1906742 QoS Management for NR V2X	Fraunhofer IIS, Fraunhofer HHI

- R2-1906804 Discussion on L2/3 protocols procedures and information for QoS support	ITRI

- R2-1906826 Discussion on SA2’s LS reply on QoS and range	Nokia, Nokia Shanghai Bell

- R2-1906831 HARQ feedback for groupcast in the light of minimum required communication range	Nokia, Nokia Shanghai Bell

- R2-1907148 HARQ feedback impact on RAN2	Lenovo, Motorola Mobility

- R2-1907211 Support of Per-Flow QoS model		LG Electronics Inc

- R2-1907283 Mobility challenges for NR V2X groupcast/platooning	Nokia, Nokia Shanghai Bell

- R2-1907360 Draft Reply LS on LS response on unicast, groupcast and broadcast in NR sidelink	Ericsson

- R2-1907361 Congestion control	Ericsson

- R2-1907420 QoS monitoring and reporting for NR Sidelink V2X Communications	Huawei, HiSilicon

- R2-1907453 Consideration on the admission control in NR SL	Huawei, HiSilicon

- R2-1907455 Discussion on the default DRB for NR SL	Huawei, HiSilicon

- R2-1907456 Relative QoS handling between NR SL and NR Uu	Huawei, HiSilicon

- R2-1907576 [DRAFT] Reply LS to SA2 on unicast, groupcast and broadcast in NR sidelink	Qualcomm Incorporated

- R2-1907827 Unicast SLRB configuration for RRC_IDLE or RRC_INACTIVE UEs	Samsung R&D Institute UK

- R2-1907834 SDAP Layer for NR-SL Unicast		Samsung R&D Institute UK

- R2-1905567 Discussion on Inter-RAT Control for NR-V2X		OPPO

- R2-1905806 Cross-RAT scheduling	CATT

- R2-1905857 Mode 3 SPS grant content and timing	vivo

- R2-1906340 Inter-RAT BSR	MediaTek Inc.

- R2-1906388 MultiRAT Aspects in NR V2X for IDLE/INACTIVE	InterDigital

- R2-1906432 Cross-RAT scheduling for NR V2X SL		Intel Corporation

- R2-1907150 Cross RAT Resource allocation	Lenovo, Motorola Mobility

- R2-1907365 Discussions on NR V2X in MR-DC scenarios		Ericsson

- R2-1907421 The enhancement of Uu to control inter-RAT V2X sidelink	Huawei, HiSilicon

- R2-1907458 Cell reselection for V2X SL communication in NR	Huawei, HiSilicon

- R2-1907578 NR Uu control of LTE sidelink	Qualcomm Incorporated

- R2-1905950 Motivation for logical UE split in V2X applications	Autotalks Ltd, OnStar Europe, Volkswagen AG

- R2-1905951 Motivation for the standalone NR PC5 mode	Autotalks Ltd, MediaTek, Volkswagen AG

- R2-1905575 Discussion on resource allocation mode for NR-V2X	OPPO

- R2-1905809 Uu and PC5 availability	CATT

- R2-1905810 Support for simultaneous configuration of Mode 1 and Mode 2	CATT

- R2-1905811 RLM/RLF declaration in NR V2X Sidelink	CATT

- R2-1905858 Discussion on support of simultaneous mode 1 and mode 2	vivo

- R2-1906026 Support of VRUs	MediaTek Inc.

- R2-1906299 Simultaneous operation of mode1 and mode2	Intel Corporation

- R2-1906300 Switching between Mode1 and mode2		Intel Corporation

- R2-1906341 RLF in sidelink	MediaTek Inc.

- R2-1906389 RAN2 Aspects of Simulataneous Configuration of Mode 1 and Mode 2		InterDigital

- R2-1906476 Discussion on criteria for PC5 availability and unavailability	ZTE, Sanechips

- R2-1906486 Discussion on multi-mode co-existence	ZTE, Sanechips

- R2-1906546 Discussion on association between sidelink data and resource allocation modes	ASUSTeK

- R2-1906775 Discussion on Cross RAT QoS handling	Apple

- R2-1907152 SL and UL BWP Numerology Mismatch	Lenovo, Motorola Mobility

- R2-1907366 In-device coexistence between LTE SL and NR SL		Ericsson

- R2-1907367 Simultaneous configuration of mode-1 and mode-2		Ericsson

- R2-1907417 Remaining issue on sidelink LCP procedure	Huawei, HiSilicon

- R2-1907422 AS-related group communication for platooning	Huawei, HiSilicon

- R2-1907459 Channel mapping for NR SL in MAC	Huawei, HiSilicon

- R2-1907460 Discussion about mode coexistence for NR sidelink	Huawei, HiSilicon

- R2-1907660 Simultaneous use of mode 1 and mode 2	KT Corp.

- R2-1907713 Considerations for support mobility for NR V2X	Samsung

- R2-1907965 Discussion on Support of Vulnerable Road User	Samsung Electronics

RAN4#90bis

[1063] R4-1902990 On NR V2X RRM characteristics	Intel Corporation

[1064] R4-1903031 Initial coexistence simulation assumptions of NR V2X	LG Electronics France

[1065] R4-1903096 AGC settling time for V2X	Qualcomm Inc.

[1066] R4-1903118 On NR V2X in-band emission model	Qualcomm Inc.

[1067]	 R4-1903125 On NR V2X IBE	Intel Corporation

[1068]	 R4-1903177 Discussion on AGC issue for NR V2X	Huawei, HiSilicon

[1069]	 R4-1903178 Coexistence issue between sidelink and Uu interface in licensed spectrum	Huawei, HiSilicon

[1070]	 R4-1903179 In-device coexistence issue between LTE and NR sidelinks	Huawei, HiSilicon

[1071]	 R4-1903224 Discussion on NR V2X RRM impact	CATT

[1072]	 R4-1903477 List of RRM core requirements for NR V2X	LG Electronics

[1073]	 R4-1903478 RRM specification structure for NR V2X	LG Electronics

[1074]	 R4-1903662 On AGC settling time	MediaTek inc.

[1075]	 R4-1903998 On AGC settling time and IBE for NR V2X	LG Electronics Finland

[1076]	 R4-1904108 NR V2X Band Plan and RF Requirements	Huawei Technologies (Korea)

[1077]	 R4-1904190 AGC settling time analysis	Huawei, HiSilicon

[1078]	 R4-1904256 AGC settling time discussion	Ericsson

[1079]	 R4-1904257 On IBE requirement discussion	Ericsson

[1080]	 R4-1904258 In-device co-existing	Ericsson

[1081]	 R4-1904259 R16 NR v2x scope discussoin	Ericsson

[1082]	 R4-1904337 TP on the operating scenarios for NR V2X Service	LG Electronics France

[1083]	 R4-1904659 Dicussion on NR V2X	vivo

[1084]	 R4-1905084 Work plan of NR V2X	LG Electronics France

[1085]	 R4-1905096 TR skeleton TR38.886 v0.0.1: V2X Services based on NR; User Equipment (UE) radio transmitssion and reception	LG Electronics France

[1086]	 R4-1905097 LS on NR V2X Sidelink Operation	Vodafone

[1087]	 R4-1905098 WF on coexistence scenarios for NR V2X	Qualcomm

[1088]	 R4-1905211 Meeting minutes on AH sessions for NR V2X UE	LG Electronics France

[1089]	 R4-1905218 WF on analysis of V2X RRM requirement	CATT

[1090]	 R4-1905238 WF on RF parameters for NR V2X UE	LG Electronics France

[1091]	 R4-1905241 Reply LS on NR V2X UE RF parameters for NR V2X service	LG Electronics France

RAN4#91

[1092]	 R4-1905367 Discussion on V2X UE transmit timing requirement	CATT

[1093]	 R4-1905368 WF on analysis of V2X RRM requirement	CATT

[1094]	 R4-1905387 Discussion on NR V2X UE RF requirements	CATT

[1095]	 R4-1905484 Coexistence issue between sidelink and Uu interface in licensed spectrum	Huawei, HiSilicon

[1096]	 R4-1905501 Discussion on NR V2X UE RF requirements	vivo

[1097]	 R4-1905726 AGC settling time for NR V2X	Intel Corporation

[1098]	 R4-1905727 On transmit timing requirement for NR V2X	Intel Corporation

[1099]	 R4-1905809 Discussion on NR V2X RRM requirement scope	MediaTek inc.

[1100]	 R4-1905840 UE transmit timing Error for NR V2X	LG Electronics Inc.

[1101]	 R4-1905841 Reply LS on NR V2X UE SL transmit timing Error	LG Electronics Inc.

[1102]	 R4-1906061 TR38.886 v0.1.0 for 5G V2X with NR sidelink	LG Electronics France

[1103]	 R4-1906062 Reply LS on NR V2X UE RF parameters for NR V2X service	LG Electronics France

[1104]	 R4-1906063 TP on the operating scenarios for NR V2X Service	LG Electronics France

[1105]	 R4-1906064 Initial coexistence scenarios and simulation assumptions	LG Electronics France

[1106]	 R4-1906065 Down scope of NR V2X coexistence scenarios	LG Electronics France

[1107]	 R4-1906387 Considerations on 5G V2X with NR SL	Dish Network

[1108]	 R4-1906465 On AGC settling time	MediaTek inc.

[1109]	 R4-1906521 Discussion on UE sidelink timing requirements for NR V2X	Huawei, HiSilicon

[1110]	 R4-1906522 WF on UE sidelink timing requirements for NR V2X	Huawei, HiSilicon

[1111]	 R4-1906626 In-device co-existing	Ericsson

[1112]	 R4-1906845 On AGC settling time for NR V2X	LG Electronics Finland

[1113]	 R4-1906848 Analysis of In-device coexistence scenarios	Huawei, HiSilicon

[1114]	 R4-1906849 Analysis of adjacent channel coexistence scenarios	Huawei, HiSilicon

[1115]	 R4-1906850 On RF parameters for NR V2X	Huawei, HiSilicon

[1116]	 R4-1906851 TP on In-device Coexistence Scenarios	Huawei, HiSilicon

[1117]	 R4-1906942 On UE to UE Coexistence study of NR V2X at 5.9GHz	Qualcomm Inc.

[1118]	 R4-1906943 On Band Definition of NR V2X	Qualcomm Inc.

[1119]	 R4-1906944 AGC settling time for V2X	Qualcomm Inc.

[1120]	 R4-1906945 NR V2X IBE	Qualcomm Inc.

[1121]	 R4-1906953 On NR V2X RRM Core Requirement	Qualcomm Incorporated

[1122]	 R4-1907607 TP on Editorial Corrections to TR38.886	Huawei, HiSilicon

[1123]	 R4-1907731 Way forward on NR V2X transmit timing error	LGE

[1124]	 R4-1907822 Meeting minutes on AH sessions for NR V2X UE	LG Electronics France

[1125]	 R4-1907823 WF on framework for V2X operation in licensed band operation	LG Electronics France

[1126]	 R4-1907826 "WF on Coexistence Scenarios and Simulation Parameters"	LG Electronics France

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template