3GPP TSG RAN Meeting #84								RP-191108

New Port Beach, US, June 3-6, 2019

Agenda Item:	9.4.6

Source:	Apple Inc.

Title:	View on PDCCH skipping

Document for:	Discussion/Decision

# Introduction

The UE power saving SI in RAN1 and RAN2 were concluded in RAN1#96 and RAN2#106 and outcome has been/is to be captured in [1]. Among many different power saving schemes studied in RAN1, PDCCH skipping has been identified as one of the most effective method for UE power saving providing power saving gain up to 83%. However, there has been some concern from two companies about potential duplication with CDRX operation during RP#83. During RAN2 SI, RAN2 has studied the issue and the conclusion is to be reported in RP#84.

In this contribution, we briefly summarize discussions on PDCCH skipping in RAN1 and RAN2 and provide further details to address concerns raised during RAN2 SI. The motivation and proposal is briefly described in Section 6.

# RAN1 SI and WI Discussion

During RAN1 SI, 14 companies have evaluated the performance of PDCCH skipping and made following observation which is captured in Section 5.1.5[1].

Reduction of PDCCH monitoring can also be achieved with explicit/implicit information for UE to skip monitoring PDCCH. A related approach, such as power saving signal/channel triggering/enabling PDCCH monitoring or "go-to-sleep" signaling where the gNB can signal the UE to go to the DRX OFF state or skip the PDCCH monitoring, is considered. Power saving gain in the range of 9%~83% is observed, with latency increase in the range of 0.1%~75% and overhead of 0% - 1% with one source showing 124% latency increase. Generally, the gain is higher for scenarios with CDRX configuration with larger DRX inactivity timer duration and for lower traffic load, and higher latency increase is associated with higher gain.

It is worth to emphasize that power saving gain of PDCCH skipping is as high as 83% and PDCCH skipping is the technique used in RRC_CONNECTED state which has the highest power consumption among different states. It has been reported in RAN1 that PDCCH monitoring without grant is the biggest contributor to UE power consumption inside RRC_CONNECTED state.

The increase of latency in above observation comes from the tradeoff between the two metrics: power consumption and delay, which can be engineered/controlled by the design/configuration of PDCCH skipping technique. The increased delay should not be a concern because increased delay is expected by design/configuration and should be within delay budget of traffic when PDCCH skipping is applied in real network.

WID scope

After UE PS SI is finished in RAN1, WID was approved in RP#83[3] with RAN1 centric scope. The scope of WI was controversial but was determined with being open for potential power saving techniques studied during RAN1 SI.

- Specify power saving techniques with UE adaptation with focus in RRC_CONNECTED mode [RAN1, RAN4] - Specify the power saving techniques with power saving signal/channel - Specify the PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED - Note: this objective shall not duplicate DRX operation and impact to DRX is studied at RAN2 - Note: Any change of PDCCH channel coding and payload interleaver is not in the scope - Specify the procedure of cross-slot scheduling power saving techniques - Note: The procedure is in addition to Rel-15 cross-slot scheduling procedure - Evaluate the required switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers [RAN4] - Note: Switching on/off the RF is part of the evaluation Note: - These objectives are RAN1/RAN4 focus and do not consider RAN2 impact. - The objectives are subject to further update in RAN#84. The update will be based on recommendations from the completion of RAN2 study and remaining RAN1 recommendations based on the conclusion of RAN1 study.

The focus of WID scope was on designing PDCCH-based power saving signal/channel for potential power saving techniques in RRC_CONNECTED mode. The WID scope didn’t explicitly spelled out PS schemes since there was no clear consensus which need to be supported except cross slot scheduling technique. Related to PDCCH skipping technique, there was some concern from a company that it could potentially duplicate DRX operation in MAC layer. So, “Note” was added and consequently RAN2 studied following two aspects of PDCCH skipping.

- Whether PDCCH skipping duplicates DRX operation or not.

- If/how PDCCH skipping could have impact MAC layer.

Current WID scope is to be revised in RP#84 based on RAN2 SI outcome.

UE PS WI Progress in RAN1

RAN1 started its corresponding WI from RAN1#96b. Focus was on the topics that are less controversial such as PDCCH-based power saving signal/channel design and cross slot scheduling technique. The discussion of PDCCH-based power saving signal/channel has been done in two separate domain : inside Active Time and outside Active Time. The discussion of outside of Active Time is around the use case as wake up signal. The discussion of inside Active Time is around the cross slot scheduling technique and other potential techniques including PDCCH skipping.

Among progresses, RAN1 made a following agreement related to power saving technique inside Active Time.

Agreements: The indication of at least one power saving technique(s) is supported at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time. - It applies to UE-specific search space. - It is FFS for the common search space. - The at least one power saving technique(s) includes at least “Cross-slot scheduling” - FFS: Which existing DCI formats includes the power saving information - Whether power saving information is not included in the fallback DCI(s) (e.g., DCI format 0_0, DCI format 1_0) - Use of non-scheduling DCI formats. - It is FFS which field(s) is used to be repurposed for the indication of the power saving technique if the repurpose of existing field(s) is used. - FFS: New DCI format with size aligned with existing DCI format

It is worth to note again that RAN1 agreed that scheduling DCI is to be enhanced to support power saving technique(s) inside Active Time. The main motivation for this choice is that during Active Time there are UE specific dynamic traffic arrival and the purpose of PS scheme is to make UE to adapt to such traffic arrival. Thus it make sense to support enhancing scheduling DCI to support potential PS schemes. But we think new DCI format could be also considered to extend its usability in Active Time.

# RAN2 Discussion

RAN2 SI had specific agenda of “DCI-based power saving adaptation” for PDCCH skipping and PDCCH monitoring periodicity switch.

During the RAN2 SI, there was an email discussion to identify the impact of PDCCH skipping on MAC layer which was captured in [2]. After online/offline discussion in RAN2#106, most recent version of TP from rapporteur of UE PS captures agreements made during RAN2 SI[4] as follows.

### 6.3.2 DCI-based PDCCH monitoring skipping (Section 5.1.5) The higher layer procedure in support of DCI-based PDCCH monitoring skipping (which can include DCI-based PDCCH monitoring periodicity adaptation) was studied to achieve PDCCH monitoring/decoding reduction (Section 5.1.5). If enabled, it is assumed that DCI-based PDCCH monitoring skipping could be configured with or without DRX. However, RAN2 did not reach consensus on the benefit of using PDCCH skipping without c-DRX. DCI-based PDCCH monitoring skipping is aimed to operate on a short time scale (i.e. shorter time scale then the L2 DRX). Under this condition, it has not been identified that DCI-based PDCCH monitoring skipping duplicates the DRX functionality. The common understanding is that such feature should be a physical layer procedure with minimal impact, if any, on the MAC layer. The MAC timers are not affected by the DCI-based PDCCH monitoring skipping command, except for timers related to UL triggered activities (e.g. RA, SR and BFR), which should be addressed in the WI phase, namely how/if the DCI-based PDCCH monitoring skipping command impacts L2 operation while a Random Access or scheduling request procedure is on-going. Further simulation results involving more elaborated scenarios (short DRX, DRX MAC CE) compared to those shown in Section 5.1.5, are provided in Annex.

DCI-based PDCCH monitoring skipping (Section 5.1.5) RAN2 studied the DCI-based PDCCH monitoring skipping, with or without DRX. For this scheme to bring most value, it is recommended to consider skipping durations shorter than the L2 DRX cycles. The final recommendation on whether to support the DCI-based PDCCH monitoring skipping scheme, and with or without DRX, in the WI is left to the plenary. In general, the above feature could and should be designed to have minimal impact on the MAC layer.

The RAN2 concluded the SI with no consensus whether this feature should be supported or not due to the objection from one company. Now it is up to RAN plenary to make final decision.

During RAN2 online/offline discussion there were questions from companies regarding PDCCH skipping. We list them and provide detailed answers.

Q1) Does PDCCH skipping duplicate DRX?

- RAN2 SI concluded that “RAN2 has not identified that PDCCH skipping duplicate DRX functionality." CDRX operation is governed by arrival of traffic and timers. Whereas, PDCCH skipping works purely based on explicit network indication, which means it can be used selectively only when it is needed. It is well understood that the main use case of PDCCH skipping is to allow UE’s PDCCH skipping between two dynamic traffic arrivals. It is not intended to be used to skip multiple DRX on durations, which makes the skipping duration naturally shorter than a DRX cycle. Figure 1 shows the typical use case of PDCCH skipping with CDRX operation.

Figure 1 Typical use case of PDCCH skipping with CDRX

Q2) Can PDCCH skipping be used without CDRX?

- Originally the PDCCH skipping was proposed as RAN1 feature, which has no impact to MAC layer. This naturally allows gNB to configure it independently from MAC layer operations such as DRX. This intention is also well captured in [4] as “If enabled, it is assumed that DCI-based PDCCH monitoring skipping can be configured with or without DRX.”

- Note that PDCCH skipping could be used without CDRX simply because it is PHY feature which is independent of L2 procedures. Although it is possible, this is not a main use case and is not a right use of the mechanism. We don’t think network will configure it in this fashion. For UE power saving in RRC_CONNECTED mode, the first mechanism kicks in will be CDRX mechanism in live network. But, this is up to network configuration.

- PDCCH skipping is to supplement CDRX mechanism for further adaptation to dynamic traffic arrival. In live networks, it is hard for gNB to optimally configure CDRX parameters since related traffic pattern information is in the application layer. Especially for bursty traffic which requires longer inactivity timer value, PDCCH skipping could effectively take advantage gaps between traffic arrivals.

Q3) Does DRX short cycle provide similar performance as that of PDCCH skipping?

- This issue has been well studied in [5]. In general, the use of DRX short cycle increases power consumption since it makes UE monitors PDCCH more often than the case of using DRX long cycle only. The delay could be reduced due to shorter cycle. The evaluation results in Figure 2 from [5] shows power consumptions for following four cases :

  - Case blue : Long DRX only

  - Case red : Long DRX + PDCCH skipping (GTS)

  - Case green : Long DRX + Short DRX

  - Case purple : Long DRX + Short DRX + PDCCH skipping (GTS)

- Results shows that the case green (Long DRX + Short DRX) increases 15~20% of power consumption compared to case blue (Long DRX only). The power consumption could be reduced by ~27% (averaged across UE cell location) by using PDCCH skipping (case red/purple) compared to case blue. The case green (Long DRX + Short DRX) increases power consumption by 18.4% compared to case blue (Long DRX only). We can see the similar trend for both Long DRX cycle of 80ms and 40ms.

- From this evaluation, it is clear that using DRX with short cycle does not provide enough power saving gain as that of PDCCH skipping.

Figure 2 The power consumption for long DRX cycle 80ms and 40ms [5]

Q4) Is PDCCH skipping L1-version of DRX Command MACCE?

- PDCCH skipping is different from DRX Command MACCE in many aspects.

- First, the use case of PDCCH skipping is different from that of MACCE. The main use case of PDCCH skipping is for skipping unnecessary PDCCH monitoring between grant arrivals for short time duration. Whereas the main use case of MACCE is to make UE enter DRX OFF state early by stopping inactivity timer, if running.

- Second, since PDCCH skipping technically keep UE stay in Active Time, UE can perform periodically or semi-statically scheduled activities such as CSI-RS monitoring, CSI/SRS transmission. Thus, the power consumption opportunity depends on such activities. Whereas, DRX Command MACCE makes UE stop inactivity timer and enter DRX OFF state, where UE is not supposed to perform such operation, in general.

- Third, PDCCH skipping is transmitted by DCI, which is much faster than MACCE which is sent through PDSCH. The MACCE requires further processing time in L2, which could be at least 3ms after PDSCH decoding finishes. Thus, if there is any time gap K0 between PDCCH and corresponding PDSCH for MACCE, then the total delay could be as much as K0+3m. Whereas DCI based PDCCH skipping could be used as soon as PDCCH decoding finishes which is at most 1ms.

- Fourth, DRX Command MACCE can certainly save power. However there is large delay increase. It has been reported that DRX Command MACCE could introduce 77% and 59% of delay increase for FTP/Video and VoIP [6]. It has been shown that 24% ~ 30% of delay could be further reduced by PDCCH skipping from DRX + MAC CE (purple) as shown in Figure 3 from [5].

Figure 3 The delay performance of DRX + PDCCH skipping and DRX + MACCE[5]

Q5) How much battery life could be increased if PDCCH skipping is enabled?

- As shown in [7] it is very hard to directly answer this question since power consumption in wireless modem depends on traffic arrival, which depends on applications running in handset and again users preference. Different user preference results in different usage pattern and different amount of power consumption at modem.

- However, to get an high level idea of the impact of this scheme, we can make certain assumptions.

  - Assume that total power consumption of handset without PDCCH skipping is given as 1.

  - Assume that modem consumes 20 or 30% of total power (X=0.2 or 0.3 respectively). This gives power consumption of all other parts including AP, RAM, screen, etc as 1-X.

  - From the evaluation results we know that Y=9%~83% of power can be saved by PDCCH skipping[1].

  - This can reduce modem power consumption to X’=X*(1-Y/100).

  - Then, the total power consumption with PDCCH skipping is given as Z=1-X+X’.

  - Then, the battery life with PDCCH skipping is given as 1/Z.

Figure 4 Power consumption reduction and total power consumption in handset

| Total power consumption without PDCCH skipping | Power consumption by modem : X | Power saving gain by PDCCH skipping : Y (%) | power consumption of modem with PDCCH skipping : X’=X*(1-Y/100) | Total power consumption with PDCCH skipping : Z=1-X+X’ | Battery life : 1/Z | Battery life increase in % |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.2 | 0 % | 0.2 | 1 | 1 | 0% |
|  |  | 9 % | 0.182 | 0.982 | 1.018 | 1.8% |
|  |  | 83 % | 0.034 | 0.834 | 1.199 | 19.9% |
|  | 0.3 | 0 % | 0.3 | 1 | 1 | 0% |
|  |  | 9 % | 0.273 | 0.973 | 1.027 | 2.7% |
|  |  | 83 % | 0.051 | 0.751 | 1.332 | 33.2% |

Q6) When and how long can gNB indicate UE to skip PDCCH monitoring?

- The PDCCH skipping could be indicated by gNB. When to send skipping indication and how long to skip PDCCH monitoring could be determined based on following factors.

- DL/UL queue length : Empty queue indicates that there would be no data to transmit. If there is no data, gNB can indicate UE to skip PDCCH monitoring for a certain duration.

- Number of UEs in a cell : If a gNB serves multiple UEs, then the inter-scheduling time for each UE would increase. This could be taken into account in determining skipping duration.

- Grant inter-arrival time statistics : The measured statistics of inter scheduling time could be used in determining skipping duration.

- Traffic arrival statistics : The measured traffic inter-arrival statistics could be also used for skip duration determination.

- Scheduling Algorithm : If gNB uses a predictable scheduling algorithm, such as round-robin scheduling policy, then the inter-scheduling time for a UE becomes a function of number of UEs being served and queue length. In this case, even queue is not empty, if scheduler knows when to serve the UE in advance, then skipping duration could be determined based on scheduling algorithm.

Q7) Will signaling overhead increase if gNB uses PDCCH skipping?

- No. In RAN1#87, scheduling DCI is agreed to be used for signaling power saving feature inside Active Time. The scheduling DCI(s) could carry valid grant information for DL/UL and additional indication for PDCCH skipping monitoring. Since the signal for PDCCH skipping is piggybacked in scheduling DCI, the increased signaling overhead should be minimal.

Q8) Does PDCCH skipping have impact on RAN2 procedures (RA, SR, BFR)?

- RAN2 SI gives design guideline of PDCCH skipping such that it should be a L1 procedure with minimal impact on MAC layer as shown below [4].

The common understanding is that such feature should be a physical layer procedure with minimal impact, if any, on the MAC layer. The MAC timers are not affected by the DCI-based PDCCH monitoring skipping command, except for timers related to UL triggered activities (e.g. RA, SR and BFR), which should be addressed in the WI phase, namely how/if the DCI-based PDCCH monitoring skipping command impacts L2 operation while a Random Access or scheduling request procedure is on-going.

- During RA, SR, and BFR procedures, UE is waiting for response from network e.g., RAR, UL grant. In this case, if there is any PDCCH skipping indicated, it could be simply ignored in PHY layer to make sure there is no impact to on-going MAC layer procedure. This is one design direction ensuring that PDCCH skipping has no impact to MAC layer. Note that this is impact from MAC layer to PHY layer. The consensus on this design approach has been captured in RAN2 email discussion on power saving. Majority of companies want to minimize impact to MAC layer procedure[2].

# Conclusion

In this contribution, we discussed current status of PDCCH monitoring skipping issue in RAN1, RAN2 and RAN Plenary. Additionally we provided details of PDCCH skipping to address potential concerns raised during WG discussion.

PDCCH monitoring without grant is the biggest contributor to UE power consumption. In that sense, we think the direct solution making UE skipping PDCCH is the most straightforward approach to save power. It has been shown through extensive study that this approach, by allowing UE to enter deeper sleep state, has the highest impact among potential  techniques.

Proposal

- Support PDCCH skipping in R16 UE PS WI scope.

# References

- 38.840 Study on UE Power Saving

- R2-1908072 Report of [105bis#27][NR/Power Saving] - PDCCH skipping (CATT), April 2019

- RP-190727 UE Power Saving in NR, March 2019

- R2-1908509 TP capturing RAN2 agreements and recommendations, CATT, May 22 2019

- R2-1908127 RAN2 impacts when introducing a DCI-based mechanism to skip the PDCCH monitoring, Intel, April, 2019

- R1-1901803 Evaluation Results of NR Power Saving Designs, MediaTek, Athens, Greece, February 2019

- Aaron Carroll, Gernot Heiser, An Analysis of Power Consumption in a Smartphone, 2010, USENIX ATC’10

# Annex : Motivation for PDCCH Monitoring Skipping

It has been noted many times that one of the biggest contributor to UE power consumption is the PDCCH monitoring without grant in CONNECTED mode. When a UE in RRC CONNECTED mode is scheduled by gNB to receive DL/UL data, the grant transmission time to the UE of interest could be random. This is because that traffic arrival for the UE from application server is random and the gNB should serve multiple UEs in the cell. Figure 5 shows an exemplary CDF of grant inter arrival time of video and web-browsing in LTE. It is observed the mean grant-inter arrival time is 5ms for video traffic and 6.2ms for web-browsing. Note that the tail of distribution reaches to ~ 20ms for video and ~40ms for web-browsing. This indicates that there could be time gap of order of ms to tens of ms during which UE can enter sleep state for power saving.

Figure 5 CDF of grant inter-arrival time for video (left) and web-browsing (right)

To take advantage of such time gap between grants, we could think of explicit sleep indication based UE power saving mechanism as shown in Figure 6. Due to the dynamic change of traffic arrivals, configuring static PDCCH monitoring period or CDRX parameters may not be enough to effectively handle dynamic data transmission / reception in power efficient manner. With explicit signaling approach, network can dynamically handle such situation while maximizing power saving gain.

Figure 6 PDCCH skipping in scheduling DCI