3GPP TSG-TSG RAN WG1 Meeting #97	R1-1907616

# Reno, USA,  13th - 17th May, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1261 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on timing relationship for UE configured with EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2019-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - “else if…” sentence has an indentation error. - Companies have different understandings on the condition ‘the UE is not configured with higher layer parameter edt-parameters’. It is not clear whether this condition includes the case that the UE is configured with higher layer edt-parameters but a NPUSCH transmission is not for Msg3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - to de-indent “else if” sentence - Changing the condition ‘the UE is not/is configured with higher layer parameter edt-parameters’ to ‘the UE is not/is using higher layer parameter edt-parameters’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The indentation error may cause misunderstanding. The condition ‘the UE is not/is configured with higher layer parameter edt-parameters’ is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6, 16.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 16.6   Narrowband physical downlink control channel related procedures

<Unchanged parts are omitted>

If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and

-	for FDD, if the corresponding NPRACH transmission starts from subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3;

else if the UE is not using higher layer parameter edt-parameters or if the UE is using higher layer parameter edt-parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n’+1 to subframe n+3.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

<Unchanged parts are omitted>

### 16.3.3	Narrowband random access response grant

<Unchanged parts are omitted>

The redundancy version for the first transmission of Msg3 is 0.

If the UE is not using higher layer parameter edt-parameters, or the UE is using higher layer parameter edt-parameters and ,

-	the TBS, modulation, and number of RUs for Msg3 are determined according to Table 16.3.3-1

otherwise,

-	if the UE is configured with higher layer parameter edt-smallTBS-Enabled set to ‘false’,

-	the TBS is given by higher layer parameter edt-TBS

-	otherwise,

- the UE selects a TBS from the allowed TBS values according to Table 16.3.3-2

- the repetition number for Msg3 is the smallest integer multiple of L value that is equal to or larger than where  is the selected TBS for Msg3, and  is given by higher layer parameter edt-TBS

-	 if ,  otherwise, is used in subclause 16.5.1.2

-	the number of RUs for Msg3 are determined according to Table 16.3.3-3

- π/4 QPSK modulation for  or  and ; QPSK modulation for and

<Unchanged parts are omitted>