3GPP TSG-TSG RAN WG1 Meeting # 96bis	R1-1905546

# Xi'an, China,  8th - 12th April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1248 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to CSI feedback with 64-QAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2019-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For wideband CSI report and 64QAM configuration, if MPDCCH hopping is configured the CSI reference resource comprises more than one SF due to the CSI reference resource being at least subframes regardless of the configuration of csi-NumRepetitionCE-r13. However, 64QAM is supported only for non-repeated unicast PDSCH transmission. Therefore, UE shall not consider 64QAM for wideband CSI feedback if MPDCCH frequency hopping is configured. Current specification only includes this restriction for CQI reporting based on Table 7.2.3-6 but not for Table 7.2.3.-5. Secondly, when a UE is configured both with csi-NumRepetitionCE-r13 > 1 and 64QAM, CQI is reported according to Table 7.2.3-6, which includes a predefined number of repetitions for each CQI entry. Therefore, the UE shall use the repetition column in the CQI table as the csi-NumRepetitionCE-r13 for determining the CSI reference resource for CQI reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct that the reported CQI is less than 11 when UE reports the wideband CQI according to Table 7.2.3-5. For the second issue, clarify that for the CSI reference resource is given by the repetition column in the CQI table when UE is configured to report CQI according to Table 7.2.3-6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI feedback with 64QAM is not correctly implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 7.2	UE procedure for reporting Channel State Information (CSI)

<Unchanged parts are omitted>

For a BL/CE UE, if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the higher layer parameter csi-NumRepetitionCE-r13 indicates more than one subframe, or if the UE is configured with higher layer parameter ce-CQI-AlternativeTableConfig-r15,

-	the UE shall report CQI according to Table 7.2.3-6.

-	if the UE is not capable of supporting 64QAM in PDSCH, or the UE is configured with higher layer parameter mpdcch-pdsch-HoppingConfig-r13 set to 'on' and the UE is calculating CQI for a wideband CSI report, the reported CQI < 13.

Else if the higher layer parameter ce-PDSCH-64QAM-Config-r15 is configured,

-	the UE shall report CQI according to Table 7.2.3-5.

-	if the UE is configured with higher layer parameter mpdcch-pdsch-HoppingConfig-r13 set to 'on' and the UE is calculating CQI for a wideband CSI report, the reported CQI < 11.

Else

-	the UE shall report CQI according to Table 7.2.3-3 with CQI index between 1 and 10.

<Unchanged parts are omitted>

### 7.2.3	Channel Quality Indicator (CQI) definition

<Unchanged parts are omitted>

-	In the time domain and for a BL/CE UE, the CSI reference resource is defined by a set of BL/CE downlink or special subframes where the last subframe is subframe n-nCQI_ref,

-	where for periodic CSI reporting nCQI_ref is ≥ 4;

-	where for aperiodic CSI reporting nCQI_ref is ≥ 4;

where each subframe in the CSI reference resource is a valid downlink or valid special subframe;

-	where for wideband CSI reports:

-	The set of BL/CE downlink or special subframes is the set of the last  subframes before n-nCQI_ref used for MPDCCH monitoring by the BL/CE UE in each of the narrowbands where the BL/CE UE monitors MPDCCH, where  is the number of narrowbands where the BL/CE UE monitors MPDCCH.

-	where for subband CSI reports:

-	The set of BL/CE downlink or special subframes is the set of the last subframes used for MPDCCH monitoring by the BL/CE UE in the corresponding narrowband before n-nCQI_ref ;

-	where is given by the “repetition” column when UE is configured to report the CQI according to Table 7.2.3-6, otherwise by the higher layer parameter csi-NumRepetitionCE.

<Unchanged parts are omitted>

Table 7.2.3-5: 4-bit CQI Table 5

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | 64QAM | 466 | 2.7305 |
| 12 | 64QAM | 567 | 3.3223 |
| 13 | 64QAM | 666 | 3.9023 |
| 14 | 64QAM | 772 | 4.5234 |
| 15 | 64QAM | 873 | 5.1152 |

Table 7.2.3-6: 4-bit CQI Table 6

| CQI index | modulation | code rate x 1024 | repetition |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 56 | 32 |
| 2 | QPSK | 207 | 16 |
| 3 | QPSK | 266 | 4 |
| 4 | QPSK | 195 | 2 |
| 5 | QPSK | 142 | 1 |
| 6 | QPSK | 266 | 1 |
| 7 | QPSK | 453 | 1 |
| 8 | QPSK | 637 | 1 |
| 9 | 16QAM | 423 | 1 |
| 10 | 16QAM | 557 | 1 |
| 11 | 16QAM | 696 | 1 |
| 12 | 16QAM | 845 | 1 |
| 13 | 64QAM | 651 | 1 |
| 14 | 64QAM | 780 | 1 |
| 15 | 64QAM | 888 | 1 |