3GPP TSG-RAN WG1 Meeting #97 	R1-1906990

Reno, USA, May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1256 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EPRE for MWUS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification does not state the EPRE for MWUS. In RAN1#96bis meeting, we have following agreements related to EPRE for MWUS: - A higher layer parameter wus-PowerBoost with values of {0dB, 1.8dB, 3dB, 4.8dB} is indicated in SIB for MWUS power boosting. - The MWUS EPRE relative to CRE EPRE is defined as wus-PowerBoost+10log10p×min⁡(ρAd,ρBd), where ρAd and ρBd are the default values for ρA and ρB relative to CRS EPRE assuming the same transmitted power for symbols with or without CRS, and p is the number of CRS ports. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To reflect the RAN1 agreements related to EPRE for MWUS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot assume correct power for MWUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 4023 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.2	Downlink power allocation

<Unchanged parts are omitted>

For a BL/CE UE, if the UE is configured with higher layer parameter RSS-Config, the ratio of resynchronization signal EPRE to cell-specific RS EPRE is given by higher layer parameter powerBoost in RSS-Config +10log10p×min⁡(ρAd,ρBd), where ρAd and ρBd are the default values for ρA and ρB in Table 5.2-1a assuming the same transmitted power for symbols with or without CRS, and p is the number of CRS ports.

For a BL/CE UE, if the UE is configured with higher layer parameter WUS-Config, the ratio of MWUS EPRE to cell-specific RS EPRE is given by higher layer parameter powerBoost in WUS-Config+10log10p×min⁡(ρAd,ρBd), where ρAd and ρBd are the default values for ρA and ρB in Table 5.2-1a assuming the same transmitted power for symbols with or without CRS, and p is the number of CRS ports.

The ratio of PDSCH EPRE to cell-specific RS EPRE among PDSCH REs (not applicable to PDSCH REs with zero EPRE) for each OFDM symbol is denoted by either  or according to the OFDM symbol index as given by Table 5.2-2 and Table 5.2-3. In addition, and are UE-specific.

For a UE in transmission mode 8 - 10 when UE-specific RSs are not present in the PRBs upon which the corresponding PDSCH is mapped or in transmission modes 1 – 7, the UE may assume that for 16 QAM, 64 QAM, 256QAM, or 1024QAM spatial multiplexing with more than one layer or for PDSCH transmissions associated with the multi-user MIMO transmission scheme, or for a UE in transmission modes 2-4 and configured with higher layer parameter must-Config-r14 the UE may assume that for QPSK,

-	 is equal to [dB] when the UE receives a PDSCH data transmission using precoding for transmit diversity with 4 cell-specific antenna ports according to Subclause 6.3.4.3 of [3];

-	 is equal to [dB] otherwise

whereis 0 dB for all PDSCH transmission schemes except multi-user MIMO as described in Subclause 7.1.5 and where is a UE specific parameter provided by higher layers. If the UE is configured with higher layer parameter must-Config-r14, and if the UE is configured with higher layer parameter p-a-must-r14, and if the PDCCH/EPDCCH DCI of the corresponding PDSCH transmission indicates MUST interference is present [4], the UE shall use the higher layer parameter p-a-must-r14 for determining.

For a UE configured with higher layers parameter servCellp-a-r12, and the UE in transmission modes 8-10 when UE-specific RSs are not present in the PRBs upon which the corresponding PDSCH is mapped or in transmission modes 1-7, the UE may assume that for QPSK and transmission with single-antenna port or transmit diversity transmission schemes or spatial multiplexing using a single transmission layer, and the PDSCH transmission is not associated with the multi-user MIMO transmission scheme, and the PDSCH is scheduled by a PDCCH/EPDCCH with CRC scrambled by C-RNTI,

-	 is equal to  [dB] when the UE receives a PDSCH data transmission using precoding for transmit diversity with 4 cell-specific antenna ports according to Subclause 6.3.4.3 of [3];

-	 is equal to [dB] otherwise

and where is given by the parameter servCellp-a-r12. If the UE is also configured with higher layer parameter must-Config-r14, and if the UE is configured with higher layer parameter p-a-must-r14, and if the PDCCH/EPDCCH DCI of the corresponding PDSCH transmission indicates MUST interference is present [4], the UE shall use the higher layer parameter p-a-must-r14 for determining .

For a cell supporting SC-PTM, the UE may assume that for the PDSCH scrambled by G-RNTI,

-	 is equal to  [dB] when the UE receives a PDSCH data transmission using precoding for transmit diversity with 4 cell-specific antenna ports according to Subclause 6.3.4.3 of [3];

-	 is equal to  [dB] otherwise

where  is configured per SC-MTCH and is given by higher layer parameter p-a-r13. If  is not configured, the UE may assume that  [dB].

For transmission mode 7, if UE-specific RSs are present in the PRBs upon which the corresponding PDSCH is mapped, the ratio of PDSCH EPRE to UE-specific RS EPRE within each OFDM symbol containing UE-specific RSs shall be a constant, and that constant shall be maintained over all the OFDM symbols containing the UE-specific RSs in the corresponding PRBs. In addition, the UE may assume that for 16QAM, 64QAM, 256QAM, or 1024QAM this ratio is 0 dB.

For transmission mode 8, if UE-specific RSs are present in the PRBs upon which the corresponding PDSCH is mapped, the UE may assume the ratio of PDSCH EPRE to UE-specific RS EPRE within each OFDM symbol containing UE-specific RSs is 0 dB.

For transmission mode 9 or 10, if UE-specific RSs are present in the PRBs upon which the corresponding PDSCH is mapped, the UE may assume the ratio of PDSCH EPRE to UE-specific RS EPRE within each OFDM symbol containing UE-specific RS is 0 dB for number of transmission layers less than or equal to two and -3 dB otherwise.

A UE may assume that downlink positioning reference signal EPRE is constant across the positioning reference signal bandwidth and across all OFDM symbols that contain positioning reference signals in a given positioning reference signal occasion [10].

For the purpose of RSRP and RSRQ measurements on CSI-RS of a discovery signal the UE may assume that the EPRE of CSI-RS is constant across the downlink system bandwidth and constant across all subframes with discovery signal transmissions for each CSI-RS resource.

If a serving cell is not configured for a UE as a LAA SCell, and if CSI-RS is configured in the serving cell then the UE shall assume downlink CSI-RS EPRE is constant across the downlink system bandwidth and constant across all subframes for each CSI-RS resource.

If a serving cell is configured for a UE as a LAA SCell, the UE may assume that EPRE of CSI-RS in subframe n2 is same as EPRE of CSI-RS in earlier subframe n1, if all OFDM symbols of subframe n1 and all subframes between subframe n1 and subframe n2, are occupied.

The cell-specific ratio  is given by Table 5.2-1 according to cell-specific parameter  signalled by higher layers and the number of configured eNodeB cell specific antenna ports.  is given by higher layer parameter p-b-r13 for PDSCH scrambled by G-RNTI and by higher layer parameter p-b otherwise. In case PDSCH is scrambled by G-RNTI, if  is not configured, the UE may assume that .

Table 5.2-1: The cell-specific ratio  for 1, 2, or 4 cell specific antenna ports

|  | One Antenna Port | Two and Four Antenna Ports |
| --- | --- | --- |
| 0 | 1 | 5/4 |
| 1 | 4/5 | 1 |
| 2 | 3/5 | 3/4 |
| 3 | 2/5 | 1/2 |

Table 5.2-1a: ρAd and ρBd for 1, 2, or 4 cell specific antenna ports 
assuming the same transmitted power for symbols with or without CRS

| PB | One Antenna Port |  | Two and Four Antenna Ports |  |
| --- | --- | --- | --- | --- |
|  | ρAd | ρBd | ρAd | ρBd |
| 0 | 1 | 1 | 1 | 5/4 |
| 1 | 1/2 | 2/5 | 1/2 | 1/2 |
| 2 | 1/3 | 1/5 | 1/3 | 1/4 |
| 3 | 1/4 | 1/10 | 1/4 | 1/8 |

<Unchanged parts are omitted>