3GPP TSG RAN WG1 Meeting #96bis	R1-1905887

# Xian, China, April 8-12, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0032 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.214 clarifying calculation of DataRate and DataRateCC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The text in current specification related to the calculation of the variables DataRate and DataRateCC in subclauses 5.1.3 and 6.1.4 needs to be updated to reflect that the computation includes dependency on band combination and feature set. For DataRate, the intention of existing text in 38.214 is to reflect that data rate (DataRate) is calculated as the maximum data rate summed over all the carriers in the frequency range in the band combination signaling and feature set consistent with serving cells, but the existing text for DataRate in 38.214 does not reflect this clearly. For DataRateCC, the intention of existing text in 38.214 is to reflect that data rate (DataRateCC) is calculated as the maximum data rate over one carrier in the frequency band of the serving cell in the band combination signaling and feature set consistent with the serving cell, but the existing text for DataRateCC in 38.214 does not reflect this clearly, especially given the lack of ordering of elements in the feature set as described below. featureSetListPerDownlinkCC (from 38.331, f40) Indicates which features the UE supports on the individual carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-bandwidthClassDL. The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 5.1.3 (for downlink), and 6.1.4 (for uplink), Clarify that DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Subclause 4.1.2 in [13, TS 38.306] including the scaling factor f(i). Clarify that DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Subclause 4.1.2 in [13, TS 38.306] including the scaling factor f(i). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current specification text can lead to mismatch in gNB/UE interpretation of the UE throughput computation and incorrect TBS restriction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3, 6.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has isolated impact and only affects the UE data rate calculation used for the TBS restriction. |  |  |  |  |  |  |  |  |

<  Start of Change 1 >

### 5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

To determine the modulation order, target code rate, and transport block size(s) in the physical downlink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field (IMCS) in the DCI to determine the modulation order (Qm) and target code rate (R) based on the procedure defined in Subclause 5.1.3.1, and

-	read redundancy version field (rv) in the DCI to determine the redundancy version..

and second

-	the UE shall use the number of layers (ʋ), the total number of allocated PRBs before rate matching (nPRB) to determine to the transport block size based on the procedure defined in Subclause 5.1.3.2.

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.95, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH.

The UE is not expected to handle any transport blocks (TBs) in a 14 consecutive-symbol duration for normal CP (or 12 for extended CP) ending at the last symbol of the latest PDSCH transmission within an active BWP on a serving cell whenever

2max⁡(0,μ-μ').i∈SCi'Lixi.Fi>1RLBRM.TBSLBRM

where, for the serving cell,

-	S is the set of TBs belonging to PDSCH(s) that are partially or fully contained in the consecutive-symbol duration

-	for the ith TB

-	Ci' is the number of scheduled code blocks for as defined in [5, 38.212].

-	Li is the number of OFDM symbols assigned to the PDSCH

-	xi is the number of OFDM symbols of the PDSCH contained in the consecutive-symbol duration

-	Fi=maxj=0,...,J-1(min(k0,ij+Eij, Ncb,i)) based on the values defined in Subclause 5.4.2.1 [5, TS 38.212]

-	k0,ij is the starting location of RV for the jth transmission

-	Ei j=min(Er) of the scheduled code blocks for the jth transmission

-	Ncb,i is the circular buffer length

-	J-1 is the current (re)transmission for the ith TB

-	μ' corresponds to the subcarrier spacing of the BWP (across all configured BWPs of a carrier) that has the largest configured number of PRBs

-	in case there is more than one BWP corresponding to the largest configured number of PRBs, µ' follows the BWP with the largest subcarrier spacing.

-	μ corresponds to the subcarrier spacing of the active BWP

-	RLBRM = 2/3 as defined in Subclause 5.4.2.1 [5, TS 38.212].

-	TBSLBRM as defined in Subclause 5.4.2.1 [5, TS 38.212].

If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded.

Within a cell group, a UE is not required to handle PDSCH(s) transmissions in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate

where,

-	J is the number of configured serving cells belonging to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot sj.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PDSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Subclause 7.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Subclause 5.2.2 [5, TS 38.212].

-	C' is the number of scheduled code blocks for the transport block as defined in Subclause 5.4.2.1 [5, TS 38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Subclause 4.1.2 in [13, TS 38.306]  including the scaling factor f(i).

For a j-th serving cell, if higher layer parameter processingType2Enabled of PDSCH-ServingCellConfig is configured for the serving cell and set to enable, or if at least one IMCS > W for a PDSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS table 5.1.3.1-2, the UE is not required to handle PDSCH transmissions, if the following condition is not satisfied:

m=0M-1Vj,mL×Tsμ≤DataRateCC

where

-	L is the number of symbols assigned to the PDSCH

-	M is the number of TB(s) in the PDSCH

-	Tsμ=10-32μ∙Nsymbslot where  is the numerology of the PDSCH

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Subclause 7.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Subclause 5.2.2 [5, TS 38.212]

-	C' is the number of scheduled code blocks for the transport block as defined in Subclause 5.4.2.1 [5, TS 38.212]

-	DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Subclause 4.1.2 in [13, TS 38.306] including the scaling factor f(i).

<  End of Change 1 >

<  Unchanged text omitted >

<  Start of Change 2 >

### 6.1.4	Modulation order, redundancy version and transport block size determination

To determine the modulation order, target code rate, redundancy version and transport block size for the physical uplink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field in the DCI scheduling PUSCH or provided in a DCI activating a configured grant Type 2 PUSCH, or as provided by mcsAndTBS as described in Subclause 6.1.2.3 for a configured grant Type 1 PUSCH to determine the modulation order  and target code rate (R) based on the procedure defined in Subclause 6.1.4.1

-	read redundancy version field (rv) in the DCI to determine the redundancy version for PUSCH scheduled by DCI, or determine the redundancy version according to Subclause 6.1.2.3.1 for configured grant Type 1 and Type 2 PUSCH,

and second

-	use the number of layers , the total number of allocated PRBs  to determine the transport block size based on the procedure defined in Subclause 6.1.4.2.

Within a cell group, a UE is not required to handle PUSCH(s) transmissions in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate,

where

-	J is the number of configured serving cells belong to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot-sj.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PUSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Subclause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Subclause 5.2.2 [5, TS 38.212].

-	C'is the number of scheduled code blocks for the transport block as defined in Subclause 5.4.2.1 [5,38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Subclause 4.1.2 in [13, TS 38.306] including the scaling factor f(i).

For a j-th serving cell, if higher layer parameter processingType2Enabled of PUSCH-ServingCellConfig is configured for the serving cell and set to enable, or if at least one IMCS > W for a PUSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS tables 5.1.3.1-2, 6.1.4.1-1, and 6.1.4.1-2, the UE is not required to handle PUSCH transmissions, if the following condition is not satisfied:

m=0M-1Vj,mL×Tsμ≤DataRateCC

where

-	L is the number of symbols assigned to the PUSCH

-	M is the number of TB in the PUSCH

-	Tsμ=10-32μ∙Nsymbslot where  is the numerology of the PUSCH

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Subclause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Subclause 5.2.2 [5, TS 38.212]

-	C' is the number of scheduled code blocks for the transport block as defined in Subclause 5.4.2.1 [5, TS 38.212]

-	DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Subclause 4.1.2 in [13, TS 38.306] including the scaling factor f(i).

<  End of Change 2 >