3GPP TSG-RAN2 WG2 Meeting #106                                  R2-1905671

Reno, USA, 13th - 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0120 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE behavior on cell reselection if rangeToBestCell is configured_Option 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-05-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#105 meeting, RAN2 agreed that rangeToBestCell in SIB2 is applicable to both intra-frequency and inter-frequency cell reselection, but absThreshSS-BlocksConsolidation can be configured per NR frequency in SIB2 or SIB4. According to 38.304, if rangeToBestCell is configured, the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. But, it seems not crystal clear how to address the threshold if absThreshSS-BlocksConsolidation is absent for that frequency |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In case rangeToBestCell is configured but absThreshSS-BlocksConsolidation is not configured on a frequency, a UE considers a cell on that frequency with one beam above the threshold. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: Cell reselection criteria. If the UE is implemented according to the CR, in case rangeToBestCell is configured and absThreshSS-BlocksConsolidation is not configured on a specific frequency, the UE may perform cell reselection to the highest ranked cell. If the UE is not implemented according to the CR, in case rangeToBestCell is configured and absThreshSS-BlocksConsolidation is not configured on a specific frequency, a UE may not perform cell reselection which may treat all cells on that frequency are not-suitable. Inter-operability: There is no inter-operability issue since this is a UE only change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to perform cell reselection if rangeToBestCell is configured but absThreshSS-BlocksConsolidation is not configured for concerned frequency(ies). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

<Start of modification 1>

#### 5.2.4.6	Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

If rangeToBestCell is configured, then the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. If there are multiple such cells, the UE shall perform cell reselection to the highest ranked cell among them. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

NOTE:	If rangeToBestCell is configured but absThreshSS-BlocksConsolidation is not configured on an NR frequency, the UE considers that there is one beam above the threshold for each cell on that frequency.

<End of modification 1>