3GPP TSG-RAN WG2 Meeting #106	R2-1906095

Reno, USA,  May 13th – 17th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1027 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on description of tci-PresentInDCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the field description of tci-PresentInDCI, the definition relies on the condition “If at least spatial QCL is configured/indicated”. The current field description of tci-PresentInDCI is as follows: tci-PresentInDCI If at least spatial QCL is configured/indicated, this field indicates if TCI field is present or not present in DL-related DCI. When the field is absent the UE considers the TCI to be absent/disabled. In case of cross carrier scheduling, the network sets this field to enabled for the ControlResourceSet used for cross carrier scheduling in the scheduling cell (see TS 38.214 [19], clause 5.1.5). However, according to TS 38.213 specified below, there is no such condition “ If at least spatial QCL is configured/indicated” that need to be satisfied in advance to include TCI field in DCI. For each CORESET, the UE is provided the following by ControlResourceSet: …… - An indication for a presence or absence of a transmission configuration indication (TCI) field for DCI format 1_1 transmitted by a PDCCH in CORESET, by TCI-PresentInDCI. The original motivation of tci-PresentInDCI is to indicate whether TCI field is present or not, it doesn’t need to rely on any condition, it should remove the condition from the corresponding description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the condition “If at least spatial QCL is configured/indicated”. Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: TCI indication. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may ingore the parameter “tci-PresentInDCI” if the condition “If at least spatial QCL is configured/indicated” is not satisfied, which may lead to scheduling failure when it receives a DCI including TCI field. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue, since the network may only include TCI field in DL-related DCI when spatial QCL is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may ingore the parameter “tci-PresentInDCI” if the condition “If at least spatial QCL is configured/indicated” is not satisfied, which may lead to scheduling failure when it receives a DCI including TCI field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

6.3.2	Radio resource control information elements

tci-PresentInDCI This field indicates if TCI field is present or not present in DL-related DCI. When the field is absent the UE considers the TCI to be absent/disabled. In case of cross carrier scheduling, the network sets this field to enabled for the ControlResourceSet used for cross carrier scheduling in the scheduling cell (see TS 38.214 [19], clause 5.1.5).

CHANGE END