3GPP TSG-RAN WG2 Meeting #106	R2-1908510

# Reno, USA,  13th - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1016 | rev | 2 | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Handover from NR to EUTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When handover procedure is triggered from NR to E-UTRAN, the source gNB will send MobilityFromNRCommand including targetRAT-MessageContainer to UE. It is specified in 38.331 that the targetRAT-MessageContainer contains the message of RRCConnectionReconfiguration defined in 36.331. However, the handoverCommandMessage contains an entire DL-DCCH-Message including the RRCConnectionReconfiguration message, which is included in HandoverCommand message and sent from target eNB to the source gNB. HandoverCommand-r8-IEs ::= SEQUENCE { handoverCommandMessage OCTET STRING (CONTAINING DL-DCCH-Message), nonCriticalExtension SEQUENCE {} OPTIONAL } In order to assemble the MobilityFromNRCommand, the soure gNB (NR) has to decode the DL-DCCH-Message (LTE) contained in the HandoverCommand to obtain the RRCConnectionReconfiguration, i.e. the gNB has to decode inter-RAT message. It is agreed that the source cell should transmit the handover command transparently without modification. In order to align with the principle and avoid source gNB decoding the inter-RAT message, the message structure of the targetRAT-MessageContainer contained in the MobilityFromNRCommand should be the same with the message structure of the handoverCommandMessage contained in the HandoverCommand from the target eNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The source gNB should send the handover command from the target eNB transparently without modification. Change the message targetRAT-MessageContainer contained in MobilityFromNRCommand from “RRCConnectionReconfiguration” to “DL-DCCH-Message” to align with the container of handoverCommandMessage in Handovercommand from the target eNB. - Change the content of the targetRAT-MessageContainer from the “RRCConenctionReconfiguration” to “DL-DCCH-Message including the RRCConnectionReconfigurtation” in 6.2.2. Impact analysis Impacted architecture options: Impacted 5G architecture: standalone Impacted functionality: Inter-RAT handover Inter-operability: If the Network is implemented according to the CR and the UE is not, the UE wil decode the targetRAT-MessageContainer in MobilityFromNRCommand according to RRCConnectionReconfiguration, however, the network will use the DL-DCCH-Message to encode the targetRAT-MessageContainer , which will lead in ASN.1 decoding error for UE and then cause handover failure. If the UE is implemented according to the CR and the Network is not, the network may encode the targetRAT-MessageContainer in MobilityFromNRCommand using RRCConnectionReconfiguration, however, handover failure will happen as the UE has considered the targetRAT-MessageContainer is a DL-DCCH-Message, which will lead in ASN.1 decode error for UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Upon handover from NR to eNB, the source cell can’t transmit the handover command from target cell transparently. The source cell needs to decode the inter-RAT handover command based on current specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This change is a non-backward compatible change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### –	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA (connected to EPC or 5GC).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

-- TAG-MOBILITYFROMNRCOMMAND-START

MobilityFromNRCommand ::=       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

mobilityFromNRCommand               MobilityFromNRCommand-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=   SEQUENCE {

targetRAT-Type                          ENUMERATED { eutra, spare3, spare2, spare1, ...},

targetRAT-MessageContainer              OCTET STRING,

nas-SecurityParamFromNR                 OCTET STRING                                OPTIONAL,   -- Cond HO-ToEPC

lateNonCriticalExtension                OCTET STRING                                OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                 OPTIONAL

}

-- TAG-MOBILITYFROMNRCOMMAND-STOP

-- ASN1STOP

MobilityFromNRCommand-IEs field descriptions

nas-SecurityParamFromNR This field is used to deliver the key synchronisation and Key freshness for the NR to LTE/EPC handovers and a part of the downlink NAS COUNT as specified in TS 33.501 [11].

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. NOTE 1. A complete message is included, as specified in the other standard.

targetRAT-Type Indicates the target RAT type.

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply, and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra | 3GPP TS 36.331 [10] (clause 5.4.2) | DL-DCCH-Message including the RRCConnectionReconfiguration |

| Conditional Presence | Explanation |
| --- | --- |
| HO-ToEPC | This field is mandatory present in case of inter system handover. Otherwise it is absent. |

END OF CHANGE