3GPP TSG RAN Meeting #85	RP-191835

# Newport Beach, USA, Sep. 16-19, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.811 | CR | 0005 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR TR38.811 Section 6.9.2 and 5.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Fraunhofer HHI, Fraunhofer IIS, Thales |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_nonterr_nw, FS_NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2019-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Two figures were added with the last CR in subclause 6.9.2 by mistake. - Unclear definition of theta in subclause 5.3.4.3 - Two different track speeds are given for the same height right before subclause 5.3.4.3.1 - The vector from terminal to satellite is incorrect in subclause 5.3.4.4. - The formula for the angular velocity is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed upper figure - Clarified angle theta - Corrected the second height from 600km to 10000km - Replaced cos by sin and vice versa. Additionally, T for transposed vector was added. - The angular velocity equals the track speed divided by the radius. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Confusion about why there are two figures showing almost the same. - Confusion about what theta actually is. - Confusion about which value is the correct one. - Derivation of Doppler shift is not comprehensible. - The incorrect equation leads to wrong values for the Doppler shift. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.2, 5.3.4.3, 5.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

### 6.9.2	TDL models

The Tapped Delay Line (TDL) models are filtered from the CDL models according to the section 7.7.4 of TR 38.901 [12] by assuming isotropic UE antenna. Two TDL models, namely NTN-TDL-A and NTN-TDL-B are constructed to represent two different channel profiles for NLOS, while NTN-TDL-C and NTN-TDL-D are constructed for LOS. The parameter of these models can be found in Tables 6.9.2-1 to 6.9.2-4.

Table 6.9.2-1a: Void

Table 6.9.2-1b: Void

The Doppler spectrum for each tap is defined as described in subclause 7.7.2 of TR 38.901. Each TDL model can be scaled in delay to achieve desired RMS delay spread according to the procedure specified in subclause 7.7.3 of TR 38.901. For LOS channel models, the K-factor of NTN-TDL-C and NTN-TDL-D can be set to a desired value following the procedure described in subclause 7.7.6 of TR 38.901.

Additional Doppler shift due to satellite motion should be taken into account according to the following formula:

,

Where  denotes the satellite speed, c denotes the speed of light, R denotes the earth radius, h denotes the satellite altitude,  denotes the satellite elevation angle, and  denotes the carrier frequency.

This additional Doppler shift should be applied to all taps of the TDL model.

The satellite speed, satellite elevation angle and UE speed should be considered to be constant during the simulation duration, if limited to few TTIs.

An illustration of the effect of additional Doppler shift due to satellite motion on the Doppler power spectrum is displayed on the next figure.

Figure 6.9.2-1: Illustration of Doppler power spectrum in NTN in LOS conditions

Table 6.9.2-1. NTN-TDL-A at elevation αmodel=50°

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 1.0811 | -4.675 | Rayleigh |
| 3 | 2.8416 | -6.482 | Rayleigh |

Table 6.9.2-2. NTN-TDL-B at elevation αmodel=50°

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 0.7249 | -1.973 | Rayleigh |
| 3 | 0.7410 | -4.332 | Rayleigh |
| 4 | 5.7392 | -11.914 | Rayleigh |

Table 6.9.2-3. NTN-TDL-C at elevation αmodel=50°

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.394 | LOS path |
|  | 0 | -10.618 | Rayleigh |
| 2 | 14.8124 | -23.373 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 10.224 dB and a mean power of 0 dB. |  |  |  |

Table 6.9.2-4. NTN-TDL-D at elevation αmodel=50°

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.284 | LOS path |
|  | 0 | -11.991 | Rayleigh |
| 2 | 0.5596 | -9.887 | Rayleigh |
| 3 | 7.3340 | -16.771 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 11.707 dB and a mean power of 0 dB. |  |  |  |

<Unchanged parts are omitted>

### 5.3.4	Non geostationary satellites

The Non-Geostationary case comprises three examples

-	LEO  at 600 km

-	LEO at 1500 km

-	MEO at 10000 km

#### 5.3.4.1	Propagation delay

In the case of bent pipe satellites, one way propagation delay is the sum of feeder link propagation delay and user link propagation delay, thus the propagation delay between Gateway and UE.

In the case of regenerative satellite, one way propagation delay is the satellite to UE propagation delay.

In both cases the transit time and/or processing time are not taken into account.

In the case of bent pipe satellite, the Round Trip Time is the physical path duration of the path : Gateway-Satellite-UE-Satellite-Gateway, that is in fact twice the one way propagation delay.

In the case of regenerative satellite, the round trip delay is the delay corresponding to the following path :satellite-UE-satellite.

In the computation, Gateway is set at 5° (TBC) elevation angle, and terminal can be set at various elevation angles, but we consider that the reference case is 10° elevation angle for the propagation delay computation.

The following table summarises the different situations and the different distances in km and the different propagation delays in ms.

The results for the three cases of NGSO satellites are summarized in the next table.

Table 5.3.4.1-1: Propagation delays for different NGSO satellites (altitude and payload types)

|  |  | LEO at 600 km |  | LEO at 1500 km |  | MEO at 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation angle | Path | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) |
| UE: 10° | satellite - UE | 1932.24 | 6,440 | 3647.5 | 12,158 | 14018.16 | 46.727 |
| GW: 5° | satellite - gateway | 2329.01 | 7.763 | 4101.6 | 13.672 | 14539.4 | 48.464 |
| 90° | satellite - UE | 600 | 2 | 1500 | 5 | 10000 | 33.333 |
| Bent pipe satellite |  |  |  |  |  |  |  |
| One way delay | Gateway-satellite_UE | 4261.2 | 14.204 | 7749.2 | 25.83 | 28557.6 | 95.192 |
| Round Trip Delay | Twice | 8522.5 | 28.408 | 15498.4 | 51.661 | 57115.2 | 190.38 |
| Regenerative satellite |  |  |  |  |  |  |  |
| One way delay | Satellite -UE | 1932.24 | 6.44 | 3647.5 | 12.16 | 14018.16 | 46.73 |
| Round Trip Delay | Satellite-UE-Satellite | 3864.48 | 12.88 | 7295 | 24.32 | 28036.32 | 93.45 |

#### 5.3.4.2	Differential delay

In this clause, we compute the differential delays between some specific positions : for instance at nadir and Edge of Coverage.

The path to gateway is supposed to be the same for all terminals.

Table 5.3.4.2-1: Differential Delay for LEO satellite

|  | LEO 600 km |  | LEO 1500 km |  | MEO 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Delta Distance | Delta Delay | Delta Distance | Delta Delay | Delta Distance | Delta Delay |
| Differential One way delay between nadir and EOC paths | 1332.2 km | 4.44 ms | 2147.5 km | 7.158 ms | 4018.16 km | 13.4 ms |
| Percentage of the maximum delay (bent pipe) |  | 31.26 % |  | 27.8 % |  | 14.1 % |
| Percentage of the maximum delay (regenerative satellite) |  | 67 % |  | 58.9 % |  | 28.7 % |

#### 5.3.4.3	Doppler Shift and variation rate

The following picture summarizes the methodology used for Non Geostationary systems. We evaluate the Doppler shift which is maximum when the UE is located in the orbital plane.

Figure 5.3.4.3-1: System Geometry for Doppler computation

The picture describing the system geometry shows the satellite "S" on its circular orbit. The vector V corresponds to the orbital speed vector.

The Doppler shift is computed for a Mobile Terminal "M" located in the orbital plan, and corresponds to the maximum value.

One of the impacting factors on the Doppler shift value is the angle between SM  SM   and the speed vector VV. Angle called θ.

-	The satellite is at an altitude h, and R is the earth radius.

-	The satellite has the velocity V, and the transmitted frequency is Fc.

-	The Doppler shift value Fd  due to satellite motion is expressed by the formula

-	Fd=Fcc×V×cosθ=Fcc×V×sinu1+γ²-2γcosu

-	Where

-	θ is the angle between satellite velocity and SM.

-	u the angle between OM and OS

-	OM is the vector between earth centre and point on earth

-	OS is the vector between earth centre and the satellite

-	angle u is varying with the satellite motion: u(t) = V*t/(R+h) with t the time

-	α is the elevation angle to the satellite of the UT in M.

-	γ can be computed as follow:

-	γ=R+hR

-	The Doppler formula is obtained after some computation.

At this altitude the speed of the satellite in circular orbit is 7.5622 km.s-1. So we can use non-relativistic approximation to compute Doppler shift.

Also at first order we neglect the speed of earth which is 327 m/s at 45° latitude and 464 m/s at the equator.

For all Non GSO cases, the satellite speeds are the following

-	At 600 km : V =7.5622 km.s-1

-	At 1500 km : V =7.1172 km.s-1

-	At 10000 km : V =4.9301 km.s-1

##### 5.3.4.3.1	Case at 2 GHz

Both (Downlink) D/L and (Uplink)  U/L the signal is around 2GHz and we limit the curves at 2 GHz.

If we consider now a moving UE at 1000 km/h, and moving in the same direction than the satellite, we have determined the worst case impact in the following graph. We can define the bounds by adding the Doppler shift due to the satellite motion and the Doppler shift due to the UE motion.

All the curves are gathered in the next graphs, showing clearly the boundaries of the Doppler shift depending on the sense of motion between the satellites and the UE.

Three NON GSO satellites cases are provided.

Figure 5.3.4.3.1-1: Case with 2 GHz signal at 600 km on D/L and U/L: fixed UE and UE in motion

Figure 5.3.4.3.1-2: Case with 2 GHz signal at 1500 km: fixed UE and UE in motion

Figure 5.3.4.3.1-3: Case with 2 GHz signal at 10000 km: fixed UE and UE in motion

##### 5.3.4.3.2	Case in Ka band

There are two cases

-	Downlink at 20 GHz

-	Uplink at 30 GHz

If we consider now a moving UE at 1000 km/h, and moving in the same direction than the satellite, we have determined the worst case impact in the following graph.

This impact is at maximum 18 kHz in one sense or the other at 20 GHz, and 27 kHz at 30 GHz, all the curves are gathered in the next graph, showing clearly the boundaries of the Doppler shift depending on the sense of motion between the satellite and the UE.

Figure 5.3.4.3.2-1: Case with 20 GHz signal at 600 km on D/L: fixed UE and UE in motion

Figure 5.3.4.3.2-2 Case with 30 GHz signal at 600 km on D/L: fixed UE and UE in motion

Figure 5.3.4.3.2-3: Case with 20 GHz signal at 1500 km on D/L: fixed UE and UE in motion

Figure 5.3.4.3.2-4: Case with 30 GHz signal at 1500 km on D/L: fixed UE and UE in motion

Figure 5.3.4.3.2-5: Case with 20 GHz signal at 10000 km on D/L: fixed UE and UE in motion

Figure 5.3.4.3.2-6: Case with 30 GHz signal at 10000 km on D/L: fixed UE and UE in motion

The different cases are summarized here below with the ratio of maximum Doppler shift in absolute value to the central frequency.

Table 5.3.4.3.2-7: Summary of Doppler shift and shift variation for different altitudes

| Frequency (GHz) | Max Doppler | Relative Doppler | Max Doppler shift variation |  |
| --- | --- | --- | --- | --- |
| 2 | +/- 48 kHz | 0.0024 % | - 544 Hz/s | LEO at 600 km altitude |
| 20 | +/- 480 kHz | 0.0024 % | -5.44 kHz/s |  |
| 30 | +/- 720 kHz | 0.0024 % | -8.16 kHz/s |  |
| 2 | +/- 40 kHz | 0.002 % | -180 Hz/s | LEO at 1500 km altitude |
| 20 | +/- 400 kHz | 0.002 % | -1.8 kHz/s |  |
| 30 | +/- 600 kHz | 0.002 % | -2.7 kHz/s |  |
| 2 | +/- 15 kHz | 0.00075 % | -6 Hz/s | MEO at 10000 km altitude |
| 20 | +/- 150 kHz | 0.00075 % | -60 Hz/s |  |
| 30 | +/- 225 kHz | 0.00075 % | -90 Hz/s |  |

Note that the Maximum Doppler shift variation in absolute value is always negative and observed when the Doppler shift is zero.

#### 5.3.4.4	Doppler Shift and variation rate

The following picture summarizes the methodology used to compute Doppler shift for Non Geostationary satellite systems. It assumes a Cartesian coordinate system such that the moving satellite and the receiver are on the y-z plane. The Doppler shift experienced by a stationary receiver can be computed as follows as a function of time:

fd(t)=f0cd(t)d(t)∂xSAT(t)∂t

where f0 is the carrier frequency, d(t) is the distance vector between the satellite and the receiver, and xSAT(t) is the vector of the satellite position. These vectors can be expressed as:

dt=[0, RE+hcosωSATt, RE+hsinωSATt-RE]

dt=0RE+hsinωSATtRE+hcosωSATt-RET

xSATt=[0, RE+hsinωSATt, RE+hcosωSATt]

xSATt=0RE+hsinωSATtRE+hcosωSATt T

where RE is the Earth radius, h is the satellite altitude, and ωSAT is the satellite angular velocity.

Figure 5.3.4.4-1: System geometry for Doppler computation (Satellite moves in the Y-Z plane)

After some mathematical manipulation [7], the Doppler shift as a function of the elevation angle is computed in a closed-form expression as follows:

fd(t)=f0cωSATREcos[θt]

where the angular velocity is ωSAT=GMERE+hωSAT=GMERE+h3, with G the gravitational constant and ME the Earth mass.

If the receiver is placed on board an aircraft or a high speed train, there will be an additional term of Doppler shift resulting from its own velocity. In case of Non Geostationary satellites, the Doppler shift due to satellite movement is much higher than the one caused by UE movement. For GEO and HAPS, the Doppler shift component is mainly caused by the UE movement.

<Unchanged parts are omitted>