3GPP TSG RAN Meeting #85	RP-191869

Newport Beach, USA, September 16-20, 2019

Agenda Item:	8.2.3

Source:	Futurewei

Title:	Rel-17 work scope on NR MIMO

Document for:	Discussion and Decision

# Introduction

NR Rel-15 was developed towards ITU 5G scenarios and performance requirements and has laid a solid foundation for further 5G standards evolution. During the current release, i.e., Rel-16, new features are introduced to support new and critical use cases and scenarios such as 5G V2X with NR sidelink, Industrial IoT, NR positioning, NR-based access to unlicensed spectrum, IAB, and so on. In the meantime, enhancements are being specified to further improve performance in terms of spectrum efficiency (e.g., MIMO enhancement), reliability and latency (e.g., URLLC enhancement), power saving, mobility, spectrum utilization (e.g., MR-DC and CA enhancements), etc.

NR Rel-17 can potentially take 4 main and yet different directions:

- Fix issues from deployments

With the initial wave of CRs for Rel-15 reduced significantly and the deployment of NR networks in the field, it is likely to see more and more issues identified in the field that requires to be addressed by the standards. Though it should be treated case by case, the field issues can be generally small and urgent and should be fixed by Rel-15/16 CRs instead of Rel-17 work item(s). In the case that a larger fix is needed, a short and focused TEI may be used. Of course, field deployments could also provide guidance on major standards work topics such as MU-MIMO performance enhancement for mobility, FR2 beam management overhead reduction, in which cases such standards work topics may be given higher priority in Rel-17.

- Support new use cases and frequency bands

The ongoing studies on non-terrestrial network (NTN) and >52.6 GHz spectrum are targeting to apply (and enhance) NR air interface (and/or architecture) for new use case and new frequency bands. A few other use cases include broadcast/multicast, WTTx, extremely long-distance coverage, etc. The extent of impacts from these use cases/spectrums on NR physical layer design and on NR architecture can be quite different. Considering that NR is a very freshly designed system and just went through some long and painful standardization process for the past two releases, and that implementation and deployment are not yet mature, major and fundamental architecture and physical layer changes should be avoided if possible when supporting these use cases/spectrums. From MIMO perspective, WTTx is a unique scenario where stationary channel can be utilized for higher spectrum efficiency and lower overhead.

- Enhance Rel-16 features

Though features that are being developed in Rel-16 can all benefit from certain enhancements, a few of them really need another release to become more complete with more solid and deployable features, which include sidelink/V2X, multi-carrier operation, and IAB. Note that some MIMO features, e.g. CSI feedback, have been worked on for multiple releases (of both LTE and NR) and further enhancement along the same technical direction needs justification.

- Introduce new techniques

To further improve NR performance significantly, new techniques should also be considered in Rel-17. For example, in order to motivate re-farming of sub 3 GHz FDD bands to NR, performance of FDD MIMO needs to be further improved and one new technology to consider is how to utilize channel partial reciprocity there. In addition, except for multi-TRP NCJT, CoMP technologies have not been explored for NR. Based on the latest research progresses, TDD reciprocity can be utilized to realize advanced interference avoidance/mitigation between the TRPs and yield significant performance benefits. Some of these new techniques may need study first before being specified in standards.

Overall, Rel-17 scope needs to strike the right balance between urgent and short-term fixes/evolution and longer-term new features and selectively focus on a few key items. In this contribution, we focus on the work scope of NR MIMO in Rel-17 including new technologies for TDD cooperative MIMO and FDD partial reciprocity, potential deployment issues of FR2 beam management, and new scenarios with stationary channel.

# TDD cooperative MIMO enhancement via sounding based interference probing

TDD Massive MIMO can provide high SE and is widely considered as a key technology for 5G NR. However, the current 5G NR TDD system provides only moderate performance benefit over 4G LTE TDD system with the same antenna setting. Further enhancement of TDD Massive MIMO for even higher SE is desirable for future releases of NR. In this contribution, we propose a new mechanism that enhances UL SRS to probe and mitigate DL interference (intra-cell and inter-cell) in a TDD Massive MIMO system, resulting in significant SE gains on top of current Rel-16 compatible mechanisms. It is suggested that RAN1 and RAN support this new mechanism in Rel-17.

## Cooperative Massive MIMO: potential and issues

Massive MIMO is considered as the major technique to achieve very high SE performance. One of the main issues in Massive MIMO is how to obtain accurate knowledge of the channels to enable spatial multiplexing of transmissions to multiple users without strong inter-user interference. For FDD systems, a relatively smaller number of antennas in the system limits the potential of performance, and in addition, without channel reciprocity, CSI acquisition needs to rely on CSI feedback which can lead to higher overhead, longer latency, lower accuracy, and hence limited SE performance. For TDD systems, Massive MIMO works relatively well thanks to accurate CSI acquisition through channel UL-DL reciprocity and generally provides higher SE than FDD.

However, channel UL-DL reciprocity is so far only utilized for single-cell (non-cooperative) MU-MIMO for TDD system and the gain is still far from what is promised with the massive number of antennas.

On the other hand, cooperative MIMO (e.g., CoMP in LTE) has not been found successful in practice. Here are some insights on the causes:

- Reciprocity in TDD is utilized (only) for channel information estimation for desired signals.

- Intra-cell interference (from MU-pairing) is handled relatively well, but for most scenarios with Massive MIMO inter-cell interference becomes the dominant limiting factor.

- Handling inter-cell interference using channel information of neighboring cells is not successful even for TDD system:

  - Each individual interfering link may not be strong enough to obtain reliable channel information, by CSI feedback or by SRS.

  - The number of interfering links to consider is generally large, resulting in overhead, complexity, and robustness issues.

  - Each cell (or cell group) uses a centralized approach, which leads to significant complexity, robustness, backhaul, and edge-effect issues.

To resolve these issues and obtain the promised gain from the massive number of antennas, a new approach is proposed here. Instead of trying to obtain channel state information of interfering links and then determine precodings jointly at a centralized entity, SRS is enhanced to directly reflect DL interference spatial information (utilizing UL-DL reciprocity). Each gNB measures the corresponding SRS resources to obtain such information and adjusts its precoding to achieve interference coordination/avoidance. More details are provided in the next subsection.

## SRS Enhancements for Cooperative MIMO in TDD System

Here we introduce a new mechanism to obtain spatial information of DL inter/intra-cell interference through cooperative UL sounding based on UL-DL reciprocity.

Traditionally, DL interference and UL transmissions are not directly related even in a TDD system, and hence UL-DL reciprocity is not applicable for interference acquisition purposes. To utilize TDD UL-DL reciprocity for interference measurement, the key is to tie some sounding activities to scheduled DL transmissions, referred to as DL Interference Probing. In other words, after gNBs make DL scheduling decisions including resource allocation and MU pairing, the system first goes through a step in which the gNBs trigger the to-be-scheduled UEs to sound on resources corresponding to the DL scheduling. That is, only (and all) the UEs/layers prescheduled for PDSCH will send SRS, and the SRS reflects the PDSCH resource/layer allocation. Then by TDD reciprocity, a gNB seeing strong UL interference from a certain spatial direction on the SRS resources (for example, via estimating the UL spatial covariance of interference signal) implies that in DL transmission the gNB will cause strong interference in that direction. The gNB can then adjust the precoding for DL interference avoidance. Finally the gNBs can perform the actual DL transmissions with resource allocations and MU pairing the same as the prescheduling. The gNBs in the network can coordinate the probing SRS resources and the time delay between the prescheduling and actual DL transmission a priori, but essentially no inter-gNB information exchange is required on the fly. To summarize, DL Interference Probing from UL is possible if the network controls the UEs to transmit SRS in a way that best reflects prospective DL interference.

The above approach is illustrated in Figure 1, in which the sounding from UEs is based on prescheduling and reflects UE receive beamforming capabilities. As a result, the gNB can acquire DL interference spatial information. The gNB adjusts the precoders and thus DL SINR and SE can be improved. Furthermore, depending on the network implementation, more users/layers may be paired for MU-MIMO (such as via less conservative scheduling), yielding even higher SE gains. This approach of interference probing and mitigation can be used to enable a variety of communication schemes to be implemented, generally in the category of Cooperative Massive MIMO.

Figure 1: TDD DL SE performance enhancement via interference probing and mitigation. To convey spatial information about the interference (both intra-cell and inter-cell) to the network, SRS based on prescheduling may be used

The above approach is effective to suppress both inter-cell and intra-cell interference. As discussed above, in Massive MIMO systems, the MU interference or intra-cell interference can be effectively controlled by existing solutions such as zero-forcing or WMMSE, leaving inter-cell interference the main bottleneck for further SE improvement. Thus precoding against both intra-cell and inter-cell interference is highly effective in achieving optimized system level performance. This approach also has the following advantages:

- Distributed across gNBs (or TRPs) in the network, with low computational complexity for each gNB

- No or little channel information exchange among gNBs

- Can adapt to inter-cell interference, including unknown interference from non-cooperative gNBs or outside of the network (e.g., from other service providers, small cells, etc.)

- No need to estimate element-wise channel, reducing the complexity and overhead.

One specific way to implement is called bi-directional training (BiT); see, e.g., [1][2] for details on algorithms for narrowband systems and wideband systems, in which iterative interference probing and mitigation are carried out starting from random initial gNB precoding, i.e., the UL sounding and DL transmission with adjusted precoders shown in Figure 1 will be executed multiple times until convergence to optimal/sub-optimal precoders, and then the PDSCH is transmitted with the optimal/sub-optimal precoders. Alternatively, BiT without multiple iterations (i.e., only one shot of interference probing before the PDSCH transmission, as shown in Figure 1) can be performed, in which beamformed sounding based on conventional DL channel measurement and interference measurement is used. The latter is more practical for typical wireless communications, whereas the former can be applied to scenarios with long-term channel stationarity (see also Section 5).

## Performance Evaluation of Cooperative MIMO via Sounding Based Interference Probing

Simulation results are provided to evaluate the performance of BiT with one-shot interference probing in a multi-cell wideband TDD system. The deployment scenarios are based on 3GPP models (see Table 1 in Appendix for more details). One baseline is ZF based Massive MIMO. With one-shot BiT, (much) more than 50% SE gain is achieved over ZF for a number of deployments as shown in Figure 2 (some results are omitted for brevity). It can be seen that generally the SE gains are higher with more gNB antennas, which has more degrees of freedom to perform interference avoidance. In addition, significant cell-edge performance gains are also observed. The gains primarily come from increased DL SINR and also the increased total number of paired layers due to effective inter-cell interference avoidance as illustrated in Figure 3.

Figure 2: Performance comparison of BiT (with wideband and one-shot interference probing) and ZF-based Massive MIMO under different antenna settings. Antenna configuration of 4x8x2 stands for UPA of 4x8 with cross-pol (see Table 1 for details). The performance metrics are: average cell SE (bps/Hz), 10%ile UE throughput (Mbps), and 5%ile UE throughput (Mbps)

(a) 	            								(b)

Figure 3: (a) SINR CDF distributions for BiT (with wideband and one-shot interference probing) and ZF. Each CDF is based on the collection of per-layer post-combining receive SINR and includes all potential layers (in the middle figure, as ZF supports 30% less number of total layers, then ZF CDF has a step at 0.3). (b) Number of layers supported in BiT and ZF.

We remark that the proposed scheme with prescheduling and interference probing bears certain resemblance to the NZP CSI-RS based interference probing scheme described in [4] (Sec. 3.1) and [5]. Both Cooperative Massive MIMO schemes have a step of DL interference probing following prescheduled outcome. However, in the NZP CSI-RS based scheme, the DL interference probing is via DL CSI-RS, and the gNB adjusts MCS (but not precoding), whereas in the currently proposed scheme, the DL interference probing is via UL SRS, and the gNB adjusts precoding for interference avoidance.

We also point out the proposed scheme may have increased latency (depending on the time gap between prescheduling and DL transmission), and potentially increased SRS resource overhead. However, the improved SE offsets the probing latency and overhead, for example, the overall packet-level latency may be reduced (also observed in NZP CSI-RS based schemes). Additionally, the latency and overhead can be limited by various techniques via implementation and standardization, and the scheme is well justified given its substantial gains.

## Objectives for Rel-17 standard support

In order to effectively convey information about dynamic interference conditions to the network, a gNB can indicate UEs how the SRS should be transmitted, including the time/frequency resource allocation and port selection for the SRS corresponding to the prospective PDSCH. This means that the network needs to dynamically adjust more SRS transmission parameters (PRB allocations, port selection) than with conventional SRS transmission (comb, offset, cyclic shift, etc., and on all ports of a SRS resource).

The proposed new Cooperative Massive MIMO mechanism mostly relies on network implementation, but some steps also require standard support:

- Operations requiring standard impact

  - SRS transmission with parameters tied to DL transmission, including PRB allocation and port allocation

    - SRS trigger enhancement to dynamically signal parameters with low overhead

    - SRS beamforming based on DL channel and interference measurement

- Operations requiring no standard impact

  - gNB scheduling/prescheduling, MU pairing, interference measurement on SRS resources, generating interference covariance matrix, precoding adjustment, time gap between prescheduling and actual DL transmission, how to tie the signalled SRS transmission parameters with parameters for DL transmission, etc.

The following are proposed based on above discussions:

Proposal 1: Support TDD cooperative MIMO via DL interference probing and mitigation based on SRS enhancements in Rel-17.

Proposal 2: Include the following objective in Rel-17 MIMO item for TDD MIMO:

- SRS enhancements with dynamically indicated parameters associated with corresponding DL transmissions

  - SRS transmissions with dynamically indicated PRB allocation and port allocation

  - SRS trigger signal enhancement to include PRB allocation and port allocation

  - SRS beamforming based on DL channel and interference measurement

# Sub-3 GHz FDD MIMO Enhancement via Partial Reciprocity

## Sub-3 GHz FDD MIMO: the significance and challenges

Lower bands, especially sub-3 GHz bands, are considered precious for wireless systems due to its wide coverage. FDD prevails in the lower bands. Therefore, to ensure both wide coverage and high SE in wireless systems, it is critical to further enhance sub-3 GHz FDD MIMO in Rel-17. On the other hand, FDD MIMO is considered as challenging (much more challenging than TDD) due to the lack of full UL-DL reciprocity in FDD, and thus spatial information crucial for accurate DL beamforming cannot be fully obtained from UL sounding.

## Partial reciprocity in FDD

Though full UL-DL reciprocity does not hold in FDD, partial reciprocity has been observed in FDD, has found theoretic support, and has been utilized in the literature.

- FDD partial reciprocity in measurement results

In various measurement campaigns carried by the academia and industry, it is observed that the number of clusters, angular domain parameters (such as AoAs and AoDs for each cluster), delay domain parameters (such as the delay for each cluster), and power domain parameters (such as cluster gains) are reciprocal in UL and DL. See [6, 7, 8], to list only a few. Parameters that are not reciprocal mainly include antenna array response vectors (which depend on the carrier frequency) and path phases.

- FDD partial reciprocity theoretic support

Most sources point out the theoretic foundation of partial reciprocity is due to the widely assumed fact that the UL transmissions and DL transmissions in frequency bands not too far from each other experience the same physical propagation environment and conditions [9, 10, 11].

- FDD partial reciprocity utilized in the literature

Recently there are a large number of publications utilizing FDD partial reciprocity to improve FDD MIMO, especially FDD Massive MIMO performance, see, e.g., [9, 10, 12] and references therein.

Hence, the FDD partial reciprocity is broadly accepted in the academia and industry, and it is found beneficial if the partial reciprocity is exploited.

## Objectives for Rel-17

FDD partial reciprocity can be utilized in FDD MIMO for higher SE performance. To this goal, 3GPP should first conduct study to conclude FDD partial reciprocity, and then provide standard support to allow the partial reciprocity to be exploited. It is thus suggested to have the following two phases in Rel-17 MIMO work item:

- Study phase

  - Study FDD partial reciprocity, and conclude to which extend reciprocity holds. For example, for different UL/DL frequency separations, how much correlation there is between the UL AoAs/AoDs/delays/gains/phases and the DL ones, respectively.

  - Establish FDD channel models consistent with above partial reciprocity conclusions.

  - Evaluate FDD MIMO SE performance when the partial reciprocity is utilized via different proposed approaches.

- Work phase

  - Support for CSI acquisition given FDD partial reciprocity. For example, DL and/or UL RS may be enhanced so that UE can better estimate non-reciprocal channel parameters (to be fed back) based on the DL RS and gNB can better estimate reciprocal channel parameters based on the UL RS.

  - Support for feedback enhancement given FDD partial reciprocity. For example, CSI feedback may be enhanced to mainly reflect non-reciprocal channel properties, and CSI feedback overhead reduction in FDD may be possible when the partial reciprocity is utilized.

The following are proposed based on above discussions:

Proposal 3: Include the following objective in Rel-17 MIMO item for FDD MIMO:

- Sub-3GHz FDD enhancement via partial reciprocity

  - Study FDD partial reciprocity and establish FDD channel models consistent with partial reciprocity

  - Support DL and/or UL RS enhancement for CSI acquisition based on partial reciprocity

  - Support CSI feedback enhancement based on partial reciprocity

# FR2 Beam Management Enhancement

FR2 beam management was developed in Rel-15 and then enhanced in Rel-16. Further enhancements should be based on real issues from field deployments instead of leftovers from Rel-16 or even Rel-15 discussions. Some issues may gradually surface as operators roll out FR2 deployment in the field. If any key FR2 beam management issues are identified from the current field deployment and practical operations, such issues should be prioritized over other potential enhancements. To this aim, companies should first review FR2 beam management field performance and key issues. Such issues, if identified and urgent, can be addressed with a TEI.

We have the following proposal:

Proposal 4: For FR2 beam management enhancement

  - Identify potential field problems from current deployment first and then address the urgent issues (if any) with TEI

# Enhancement for Scenarios with Stationary Channels

There are many deployment scenarios in which a network operates with high-capacity wireless connections from network nodes (e.g., gNBs) to stationary devices (which can further communicate with end users via other links), such as WTTx, video surveillance, small cell backhauling, etc. The stationary channels have zero Doppler, which are quite different from most typical wireless channels. Another distinctive property of these deployment scenarios is that the channels are stationary in very large time scales, such as in terms of hours, days, months, or even years. Channels at these time scales are generally too long for conventional wireless systems and hence rarely studied, but become very important to these particular scenarios.

Exploiting the stationarity of wireless channels can enable non-conventional approaches to be developed and used. For example, iterative bi-directional training described in Section 2.2 can be adopted. For another example, precise information of the channels, including both serving channels and interfering channels, can be learned over time, and then advanced transmission and reception schemes can be developed and utilized based on the high-precision channel information. Impairments (including even minor impairments) that prohibit very high spectrum efficiency to be achieved in typical wireless networks can be estimated with high accuracy and compensated, leading to a practical system that can approach the theoretic limit.

We have the following proposal:

Proposal 5: Include the following objective in Rel-17 MIMO item for stationary channels:

- Enhancement for scenarios with stationary channels

  - Study distinctive properties of stationary channels

  - Support high-precision CSI acquisition and advanced high-precision transmission schemes for stationary channels

# Conclusion

In this contribution, we have discussed three categories for MIMO enhancements, i.e., TDD MIMO, FDD MIMO, and FR2 MIMO. We have suggested the following for NR Rel-17 MIMO work item scope:

Proposal 1: Support TDD cooperative MIMO via DL interference probing and mitigation based on SRS enhancements in Rel-17.

Proposal 2: Include the following objective in Rel-17 MIMO item for TDD MIMO:

- SRS enhancements with dynamically indicated parameters associated with corresponding DL transmissions

  - SRS transmissions with dynamically indicated PRB allocation and port allocation

  - SRS trigger signal enhancement to include PRB allocation and port allocation

  - SRS beamforming based on DL channel and interference measurement

Proposal 3: Include the following objective in Rel-17 MIMO item for FDD MIMO:

- Sub-3GHz FDD enhancement via partial reciprocity

  - Study FDD partial reciprocity and establish FDD channel models consistent with partial reciprocity

  - Support DL and/or UL RS enhancement for CSI acquisition based on partial reciprocity

  - Support CSI feedback enhancement based on partial reciprocity

Proposal 4: For FR2 beam management enhancement

  - Identify potential field problems from current deployment first and address the urgent issues (if any) with TEI

Proposal 5: Include the following objective in Rel-17 MIMO item for stationary channels:

- Enhancement for scenarios with stationary channels

  - Study distinctive properties of stationary channels

  - Support high-precision CSI acquisition and advanced high-precision transmission schemes for stationary channels

# References

H. Zhou, J. Liu, Q. Cheng, D. Maamari, W. Xiao, and A. C. K. Soong, "Bi-Directional Training with Rank Optimization and Fairness Control," in 2018 IEEE 88th Vehicular Technology Conference (VTC-Fall), 2018.

J. Liu, Q. Cheng, W. Xiao, D. Maamari, and A. C. K. Soong, "Bi-directional Training for Wideband Systems," in 2019 IEEE 89th Vehicular Technology Conference (VTC-Fall), 2019.

3GPP TR 36.873, "Study on 3D channel model for LTE," v12.7.0, January 2018.

R1-1709932, "Channel and interference measurement for CSI acquisition," Huawei, HiSilicon, 3GPP TSG RAN WG1 NR Ad Hoc Meeting, Qingdao, China, June 27 – 30, 2017.

R1-1719424, "Channel and interference measurement for CSI acquisition," Huawei, HiSilicon, 3GPP TSG RAN WG1 Meeting#91, Reno, USA, November 27 – December 1, 2017.

K. Hugl, K. Kalliola, and J. Laurila, "Spatial Reciprocity of Uplink and Downlink Radio Channels in FDD Systems", in Proc. COST 273 Technical Document TD (02), vol. 66, 2002.

S. E. Foo and M. Beach, "Uplink based downlink beamforming in UTRA FDD", in Proc. COST 273 Technical Document TD (02), vol. 66, 2002.

S. Imtiaz, G. S. Dahman, F. Rusek, and F. Tufvesson, "On the directional reciprocity of uplink and downlink channels in Frequency Division Duplex systems," in IEEE 25th Annual International Symposium on Personal, Indoor, and Mobile Radio Communication (PIMRC), pp. 172-176, 2014.

D. Vasisht, S. Kumar, H. Rahul, and D. Katabi, "Eliminating channel feedback in next-generation cellular networks," in Proc. ACM SIGCOMM Conference, pp. 398–411, ACM, 2016.

L. Miretti, R. L. G. Cavalcante, and S. Stanczak, "FDD massive MIMO channel spatial covariance conversion using projection methods," in Proc. IEEE ICASSP, pp. 3609–3613, April 2018.

3GPP TR 36.897, "Study on elevation beamforming / Full-Dimension (FD) Multiple Input Multiple Output (MIMO) for LTE", v13.0.0, 2015.

M. Arnold, S. Dörner, S. Cammerer, S. Yan, J. Hoydis, and S. ten Brink, "Enabling FDD Massive MIMO through Deep Learning-based Channel Prediction," arXiv:1901.03664 https://arxiv.org/pdf/1901.03664.

# Appendix

Table 1: Simulation settings used in numerical evaluation.

| Items | Parameters |
| --- | --- |
| Carrier | 3.5 GHz with 10 MHz bandwidth |
| BS Deployment | 3GPP 36.873 UMi, ISD = 200 m (with wraparound) [3] |
| UE distribution | 5 per cell/sector, 80% indoor UEs |
| Channel | 3D UMi wideband channel |
| Traffic statistics | Full buffer |
| Antenna configurations | BS: 4x8x2 (8 columns, 4 rows, cross polarization, totally 64 ports) 2x8x2 (8 columns, 2 rows, cross polarizations, totally 32 ports) 2x4x2 (4 columns, 2 rows, cross polarizations, totally 16 ports) UE: 2x1x2 (2 columns, 1 row, cross polarization, 4 ports) 0.5 wavelength spacing |
| UE power | 23 dBm |
| BS power | 47 dBm |
| Precoding granularity | Subband (5 resource blocks, 60 tones) |
| Noise figure | 3 dB at BS, 6 dB at UE |
| Channel/covariance estimation | For scheduler: ideal serving cell channel information, ideal interference covariance estimation at the gNB For UE demodulation: realistic channel/covariance estimation |