3GPP TSG RAN meeting #85										RP-191920

Newport Beach, USA, Sep. 16-19, 2019

Status Report to TSG

Agenda item:			9.3.7

| WI / SI Name | Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_MIMO_OTA_test |  |  |  |  |
| Unique ID | 801001 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181402 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: December / 2019 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 45% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ruixin WANG, Jinqiang XING, He WANG |
|  | Company | CAICT, OPPO, Samsung |
|  | Email | wangruixin@caict.ac.cn, xingjinqiang@oppo.com, h0809.wang@samsung.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	T hen please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #92 (Aug 2019, Ljubljana)

- General

  - NR MIMO OTA SI ad-hoc meeting notes was approved [1]

  - TR38.827 v0.3.0 NR MIMO OTA was approved [2]

  - TPs to TR38.827 v0.3.0 on DUT positioning guidelines and Abbreviations were approved [3][4]

  - WF on NR MIMO OTA was approved [5]

  - LS on RSRPB definition was approved [6]

- Testing methodologies

  - The Environmental condition for FR1

    - Continue the discussion on the open issues for UE noise-limited and SNR controlled environmental condition until Nov RAN4 meeting.

    - Select one of metrics above by the Nov RAN4 meeting

    - SI outcome will recommend the selected metric for test cases metric in the WI phase

  - Test methods for FR1

    - Probes configuration

      - Option 1: 16 probes with 8 uniformly spaced probes (to guarantee LTE backward compatibility) and remaining 8 probes aligned on a 2D sector for NR FR1

      - Option 2: 16 probes ring (uniformly spaced) for NR FR1 MPAC system

    - Weighted RMS spatial correlation error

      - Weighted-RMS Spatial correlation error of LTE MIMO OTA channel model (i.e. UMi and UMa) is considered as a reference (i.e. the error between ideal channel model correlation and  theoretical LTE 8 probes correlation within 1l)

      - Use the weighted RMS correlation error for NR FR1

      - UE vendors are encouraged to study the impact of RMS correlation error on TP performance

  - Test methods for FR2

    - Measurement distance and PSP

      - Focus on 0.75cm and 1m study for corresponding PSP and system dynamic range analysis;

      - PSP limit shall be decided next RAN4 meeting;

    - Radiated verification of the DL signal EVM per probe is needed for FR2 MIMO OTA

      - A pass-through mode in the fader is needed to verify the EVM under the same waveform scaling conditions that are used when fading emulation is enabled

    - Orientations for FR2 MIMO OTA static testing

      - Companies are encouraged to provide the analysis to justify proposals for orientation;

        - Total azimuthal positions for FR2 static testing is FFS

      - Study the vertical rotation for FR2 MIMO OTA

- Channel Models related agreements

  - Channel models for NR MIMO OTA

    - Down-selection of channel model is agreed

      - FR1: UMi CDL-A, Uma CDL-C.

      - FR2: InO CDL-A, UMi CDL-C.

  - Direction of Travel (DoT) for FR1 was approved [7]

  - TP to TR38.827 on Channel models for FR1 and FR2 was approved [8]

2.4.2	Remaining Open issues

- Define the system layout for NR FR1 and FR2 MIMO OTA testing

- Make decision on the orientations and rotations for NR FR2 MIMO OTA testing

- Define channel model validation procedure to ensure correct implementation

- Develop the preliminary uncertainty assessment for the methodology

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #92 (Aug 2019, Ljubljana)

- R4-1909932 “NR MIMO OTA Ad-hoc meeting notes”, CAICT

- R4-1909936 “TR38.827 v0.3.0 NR MIMO OTA”, CAICT

- R4-1910396 “TP to TR 38.827 v0.3.0 on DUT positioning guidelines”, ROHDE & SCHWARZ, CAICT, Keysight

- R4-1909938 “TP to TR38.827 v0.3.0 on Abbreviations”, CAICT

- R4-1910609 “WF on NR MIMO OTA” CAICT

- R4-1910610 “LS on RSRPB definition”, Huawei, HiSilicon

- R4-1907926 “Direction of Travel (DoT) for FR1” Spirent Communications

- R4-1910399 “TP to 38.827 on Channel Models” Keysight Technologies UK Ltd, Spirent

- R4-1909935 “WF on NR MIMO OTA” CAICT

- R4-1909083 “About MIMO OTA SI speed up” OPPO, CAICT

- R4-1908065 “TP to TR 38.827 v0.3.0 on DUT positioning guidelines” ROHDE & SCHWARZ, CAICT

- R4-1909084 “TP to TR 38.827 v0.3.0 on environmental requirements” OPPO

- R4-1909086 “TP to TR 38.827 v0.3.0 on averaging of throughput curves” OPPO

- R4-1909100 “On RSRPB and RSARP” Huawei, HiSilicon

- R4-1909085 “About MIMO OTA environmental condition in FR1” OPPO

- R4-1907991 “NR FR1 MIMO OTA environment condition” MediaTek Beijing Inc.

- R4-1908019 “Further views on FR1 MIMO OTA testing conditions” Apple Inc

- R4-1908870 “Discussion on environmental condition for NR MIMO OTA in FR1” NTT DOCOMO, INC.

- R4-1909579 “Environmental conditions for NR FR1 MIMO OTA” Qualcomm Incorporated

- R4-1909728 “System implementation of FR1 2D MPAC” Keysight Technologies UK Ltd

- R4-1908421 “On NR FR1 MPAC Probe Layout” vivo

- R4-1909087 “About MIMO OTA system layouts” OPPO

- R4-1909088 “TP to TR 38.827 v0.3.0 on test zone size” OPPO

- R4-1907948 “Effect of range length to PSP” Spirent Communications

- R4-1907949 “On the probe layout on FR2 MPAC systems” Spirent Communications

- R4-1908020 “Verification of emulated DL signal quality for FR2 MIMO OTA” Apple Inc

- R4-1908064 “Usage of UBF for FR2 NR MIMO OTA testing” ROHDE & SCHWARZ

- R4-1909089 “About MIMO OTA FR2 UE positions” OPPO

- R4-1909172 “Views on the UE orientation and number of test points in FR2 MIMO OTA test” Sony, MVG Industries

- R4-1909729 “System Implementation of FR2 3D MPAC” Keysight Technologies UK Ltd

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template