3GPP TSG RAN WG # 85									   RP-192115

Newport Beach, CA, USA, Sep.16-20, 2019

Agenda Item:   9.4.11

Source:            AT&T

Title:                 IAB RRM and Demod Requirements for Rel 16

Document for: Discussion/Approval

# Introduction

As part of the Work Item on Integrated Access and Backhaul for NR [1], 3GPP has agreed to support solutions for efficient operation of integrated access and wireless backhaul for NR. The specification of RF and RRM core requirements for IAB nodes is already captured in the IAB WID [1]:

- Specification of RF and RRM requirements [RAN4-led]:

  - Define RF requirements for both backhaul and access links of an IAB-node including requirements for co-existence (e.g. ACLR, ACS). This may include defining a new power class for MT.

  - Define RRM core requirements for both backhaul and access links of IAB node.

  - Define latency requirements for switching between communication over parent backhaul link (i.e. MT) and child backhaul/access links (i.e. DU).

  - Define timing requirements for IAB specific network synchronization. This may include (a) requirement for “case 1” timing (e.g. accuracy of DL transmission timing alignment between an IAB-node and its parent node), and (b) cell phase synchronization accuracy for multi-hop IAB network.

In addition, for the performance part of the WID the following is defined:

- Define RRM performance requirements.

- Define demodulation performance requirements for both backhaul and access links of an IAB-node. [RAN4]

This document provides an overview of the key RRM requirements for IAB nodes as well as the need for demodulation requirements.

# IAB RRM Requirements

One of the key areas of work in RAN1 is to support IAB inter-node discovery and measurement which has to be enhanced compared to the Rel-15 RRM framework design for access UEs due to the half-duplex constraint:

    - Specification of extensions to Rel. 15 to support the use of SSBs orthogonal to SSBs used for UEs (via TDM and/or FDM), for inter-IAB-node discovery and measurements, including additional SMTC periodicities and time-domain mapping of SSB locations (e.g. enable muting patterns to deal with half-duplex constraint).

Figure 1 provides an example of SSB transmission configurations (STC) between access and two backhaul link hop orders.

Figure 1: TDM multiplexing of initial access signals between access and backhaul links (same periodicity)

The access synchronization signal transmissions can overlap in time across hop orders since they are intended for UEs and therefore not subject to the half-duplex constrain requirement. However, orthogonal STCs are required for the transmission of the synchronization signal transmissions intended for backhaul link discovery and maintenance with a possibly larger periodicity of transmission and separately configured from the parameters/resources used by access UEs. As a result, RAN1 agreed to support up to 4 SMTCs for IAB-node MTs, which is optimized for 3 hop orders and additional periodicities than supported by access UEs in Rel-15 (320 and 640 subframes).

Observation 1: The starting point for the definition of RRM requirements for IAB nodes can be those already defined for access UEs, but should also include the additional periodicities defined in Rel-16 (320 and 640 subframes) and up to 4 configured SMTCs at an IAB-node MT.

Additionally, frequent transmission of SSBs can result in excessive overhead and result in undesirable scheduling restrictions on the IAB node DU function since data transmissions cannot be scheduled when the IAB node UE function is performing measurements. During RAN1#94bis, CSI-RS was studied as an alternative for inter-IAB node discovery and measurement, and the following agreement was made:

- CSI-RS can be used for inter-IAB detection in synchronous network

  - Extended CSI-RS periodicities can be considered.

As a result, instead of relying on SSB-based RRM, the network may utilize CSI-RS for topology and route management measurements since the CSI-RS can have lower time/frequency resource overhead and can be UE-specifically configured with finer granularity than SSB-based measurements.

Observation 2: CSI-RS based RRM measurements for inter IAB-node discovery and measurement can have a significant benefit compared to SSB based RRM measurements.

Of course, certain parameters should be optimized to support IAB deployments, considering the higher capability of IAB node MTs compared to UEs and the less strict latency requirements for measurements in Rel-16 which focuses on fixed deployments. However, it is not acceptable to not define RRM requirements for IAB nodes, since IAB node setup, integration, and topology management are functions which are heavily dependent on backhaul link quality measurements at the Donor CU, which would naturally be based on RRM measurements. As a result, it would be impossible for an operator to effectively plan an IAB network deployment, since the performance of a potential topology could not be bounded without minimum RRM performance requirements for the IAB MTs. In addition, a network deployment may include a mix of IAB node types which may even be from different vendors, in this case testing and inter-operability is very challenging without the definition of RRM requirements for IAB nodes.

Proposal 1: Core and performance requirements should be defined for both SSB and CSI-RS based RRM measurements at the IAB-MT, based on the existing requirements for access UEs and including the new periodicities and additional SMTCs configured for IAB-MTs.

# IAB Demod Requirements

As noted in the introduction, besides RRM requirements, the performance section of the IAB WI also covers the definition of demodulation performance requirements for access and backhaul links. This is a straightforward exercise since the backhaul link is based on the existing Rel-15 signals and channels defined for the access link. As a result, demodulation performance requirements for IAB nodes should at least include the requirements defined in the existing 38.101-4 and 38.104 specifications in the new 38.116 specification for IAB (e.g. PUSCH/PUCCH/PRACH for the IAB-DU, and PDSCH/PDCCH/PBCH/ CSI reporting requirements for the IAB-MT).

In addition, in RAN1#98, additional TDD configurations other than those supported in Rel-15 are introduced for IAB-DUs and IAB-MTs to enable more efficient multiplexing of access and backhaul links:

Agreements:

Slot formats with the sequence order UL-Flexible-DL are supported for IAB-node DU resource configurations (F1-AP) and IAB-node MT resource configurations provided in dedicated RRC signaling (e.g. enhancements to the existing TDD-UL-DL-SlotConfig in TDD-UL-DL-ConfigDedicated – details up to RAN2).

- FFS: Support of dynamic indication of UL-Flexible-DL

The same justification for RRM requirements applies to demod requirements. It is critical for network planning and operation that the backhaul link quality can be tested and ensured using performance requirements in the same manner as done for access links. It should be further discussed in RAN4 whether the requirements should differ from those existing requirements defined for wired BS nodes and access UEs.

Proposal 2: Performance requirements are defined for access and backhaul links of an IAB node, taking existing BS and UE demodulation performance requirements as a starting point for the IAB-DU and IAB-MT respectively, and additionally considering new slot formats defined in Rel-16 for IAB-DUs and IAB-MTs.

# Conclusion

This contribution analyzed the RRM and demod performance requirements for IAB. The following proposals were made:

Proposal 1: Core and performance requirements should be defined for both SSB and CSI-RS based RRM measurements at the IAB-MT, based on the existing requirements for access UEs and including the new periodicities and additional SMTCs configured for IAB-MTs.

Proposal 2: Performance requirements are defined for access and backhaul links of an IAB node, taking existing BS and UE demodulation performance requirements as a starting point for the IAB-DU and IAB-MT respectively, and additionally considering new slot formats defined in Rel-16 for IAB-DUs and IAB-MTs.

# Reference

[1] RP-190712, Revised WID: Integrated Access and Backhaul for NR, Qualcomm