3GPP TSG-RAN2 Meeting #107	R2-1908885

# Prague, Czech Republic, 26th - 30th Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1127 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on FailureInformation report for NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#AH2 2017 meeting, it was agreed that for NE-DC case CA duplication can only be configured for MCG bearer, the agreement was as following: Agreements 1: CA packet duplication is not applied to LTE CA of EN-DC. 2: In the EN-DC and NG-EN-DC case, CA packet duplication can only be configured for SCG bearer. In the NE-DC case, CA packet duplication can only be configured for the MCG bearer. 3: In the NR-NR DC case, CA packet duplication can only be configured for non-split bearer.. And in RAN2#103 meeting, it was agreed LTE CA duplication is not supported in combincationwith NR PDCP in Rel-15 for EN-DC, this applies for NE-DC, Option5 and Option7. => LTE CA duplication is not supported in combination with NR-PDCP in Rel-15. This applies for EN-DC, Option 5 and Option 7. (Hence LTE CA duplication could be used in combination with EN-DC but only on an MCG bearer using LTE PDCP).. And the restriction has been captured in the field description of pdcp-Duplication within PDCP-Config in 38.331:”PDCP duplication is not configured for CA packet duplication of LTE RLC bearer.” So from the agreements, it is clear that for NE-DC, it doesn’t support EUTRAN CA duplication in Rel-15. Since NE-DC doesn’t support EUTRAN CA duplication, NE-DC doesn’t support FailureInformation report for SCG RLC bearer failure via NR ULInformationTransferMRDC, either. However, the field description of the ul-DCCH-MessageEUTRA within the ULInformationTransferMRDC supports report of EUTRAN FailureInofrmation: “Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport and FailureInformation messages”, which doesn’t align with the previous agreements. |
| Summary of change: |  | Clarify that the NE-DC doesn’t support CA duplication for SCG in Rel-15, and the NR ULInformationTransferMRDC can’t include the EUTRAN FailureInformation message in Rel-15. - Modify the related procedure, so that ul-DCCH-MessageEUTRA within the ULInformationTransferMRDC can only be used to transfer EUTRAN RRC MeasurementReport message. - Modify the field description of ul-DCCH-MessageEUTRA, so that in this version of the specification it can only be used to transfer the EUTRAN RRC MeasurementReport message. Impact analysis Impacted architecture options: Impacted 5G architecture: NE-DC Impacted functionality: FailureInformation report Inter-operability: If the Network is implemented according to the CR and the UE is not, there is no inter-operability, as the Network will not configure EUTRAN CA duplication for UE configured with NE-DC. If the UE is implemented according to the CR and the Network is not, the network may confuse whether the EUTRAN CA duplication can be configured for UE for NE-DC, if the NW configure EUTRAN CA duplication for UE configured with NE-DC, failure will occur due to the UE doesn’t support it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | As the mismatching between the field descrption for NE-DC doesn’t support EUTRAN CA duplication and the ul-DCCH-MessageEUTRA for supporting EUTRAN FailureInformation report, it is not clear whether CA duplication can be configured for SCG bearer for NE-DC case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2a.3, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.7.2a.3	Actions related to transmission of ULInformationTransferMRDC message

The UE shall set the contents of the ULInformationTransferMRDC message as follows:

1>	if there is a need to transfer MR-DC dedicated information related to NR:

2>	set the ul-DCCH-MessageNR to include the NR MR-DC dedicated information to be transferred (e.g., NR RRC MeasurementReport and FailureInformation message);

1>	else if there is a need to tranfer MR-DC dedicated information related to E-UTRA:

2>	set the ul-DCCH-MessageEUTRA to include the E-UTRA MR-DC dedicated information to be transferred (e.g., E-UTRA RRC MeasurementReport);

1>	submit the ULInformationTransferMRDC message to lower layers for transmission, upon which the procedure ends;

NEXT OF CHANGE

#### –	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR-DC dedicated information  (e.g. for transferring the NR or E-UTRA RRC MeasurementReport message or the FailureInformation message).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

ULInformationTransferMRDC message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFERMRDC-START

ULInformationTransferMRDC ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

ulInformationTransferMRDC                   ULInformationTransferMRDC-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransferMRDC-IEs::=           SEQUENCE {

ul-DCCH-MessageNR                           OCTET STRING                    OPTIONAL,

ul-DCCH-MessageEUTRA                        OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport and FailureInformation messages.

ul-DCCH-MessageEUTRA Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport message.

END OF CHANGE