3GPP TSG-RAN2#107	R2-1911610

Prague, Czech Republic, August 26-30, 2019

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3986 | rev | 3 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on handling of SCell(s) during Make Before Break handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMob-Core |  |  |  |  |  | Date: |  |  | 2019-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | makeBeforeBreak feature was introduced in Rel-14 where the UE keeps the connection with the source eNB while it processes a HO command and attempts access at the target eNB. Per specification, when to stop the connection with the source eNB is up to UE implementation while satisfying the RAN4 requirement for interruption. At the time, it was not explicitly discussed whether the UE can operate at full capability with the source eNB after receiving HO command until it stops the communication with the source eNB. The current specification says that the UE will keep all the serving cells without any mention of any other capability reduction. Since it can’t be expected for all the UEs to operate at full capability during the mentioned transition time, a capability coordination between UE and source eNB would be needed. However, given that this will introduce significant changes to the specification and already being discussed in Rel-16 for mobility enhancements, an easier solution will be to only keep the PCell with the source eNB. Therefore, the CR proposes to leave the handling of SCells to UE implementation. It is also important to note that RAN4 requirements (36.133 5.1.2.1) and RAN5 tests for MBB are for PCell only. Therefore, this change will bring RAN2 specification in line with them. The same state also applies to SN Change procedure where only PSCell should be kept with the source SN when the UE adds a target SN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change “cell(s)” to “PCell” during RRC configuration for handover in 5.3.5.4 and add a Note that the handling of SCells is up to UE implementation. - Change “cell(s)” to “PSCell” during RRC configuration for SN change in 5.3.10.10 and add a Note that the handling of SCells is up to UE implementation. Impact Analysis: Impacted functionality: E-UTRAN Handover, E-UTRAN DC SN Change Interoperability issue: - if the network is implemented according to the CR and the UE is not: there is no inter-operability issue. - if the UE is implemented according to the CR and the network is not: the UE will not receive downlink transmissions from the eNB on SCells. Implementation of this CR from Rel-14 will not cause additional interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior for makeBeforeBreak feature will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.4, 5.3.10.10, Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r2 in R2-1909747 r1 in R2-1908239 r0 in R2-1906455 |  |  |  |  |  |  |  |  |

Start of 1st change

#### 5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	stop timer T370, if running;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	if BL UE or UE in CE:

2>	if sameSFN-Indication is not present in mobilityControlInfo:

3>	acquire the MasterInformationBlock in the target PCell;

1>	if makeBeforeBreak is configured:

2>	perform the remainder of this procedure including and following resetting MAC after the UE has stopped the uplink transmission/downlink reception with the source PCell;

NOTE 1a:	It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source PCell to initiate re-tuning for connection to the target cell, as specified in TS 36.133 [16], if makeBeforeBreak is configured.

NOTE 1x: It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source SCell(s) after receiving RRCConnectionReconfiguration message.

1>	reset MCG MAC and SCG MAC, if configured;

1>	release uplinkDataCompression, if configured;

1>	re-establish PDCP for all RBs configured with pdcp-config that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 2a:	At handover the pdcp-reestablish flag will be set for all RBs configured with NR PDCP in nr-RadioBearerConfig TS 38.331 [82] which will cause the PDCP entity to be re-established also for these RBs.

1>	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

1>	for each SCell configured for the UE other than the PSCell:

2>	if the received RRCConnectionReconfiguration message includes sCellState for the SCell and indicates activated:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else if the received RRCConnectionReconfiguration message includes sCellState for the SCell and indicates dormant:

3>	configure lower layers to consider the SCell to be in dormant state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	if the received RRCConnectionReconfiguration message includes the rach-Skip:

2>	configure lower layers to apply the rach-Skip for the target MCG, as specified in TS 36.213 [23] and 36.321 [6];

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellGroupToReleaseList:

2>	perform SCell group release as specified in 5.3.10.3d;

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the securityConfigHO (without suffix) is included in the RRCConnectionReconfiguration:

2>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

3>	update the KeNB key based on the KASME key taken into use with the latest successful NAS SMC procedure, as specified in TS 33.401 [32];

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

NOTE 2b:	If the UE needs to update the S-KeNB key as specified in 5.3.10.10, the UE updates the S-KeNB after updating the KeNB key.

2>	store the nextHopChainingCount value;

2>	if the securityAlgorithmConfig is included in the securityConfigHO:

3>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	if connected as an RN:

4>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	if connected as an RN:

4>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	else if the securityConfigHO-v1530 is included in the RRCConnectionReconfiguration:

2>	if the nas-Container is received:

3>	forward the nas-Container to upper layers;

2>	if the keyChangeIndicator-r15 is received and is set to TRUE:

3>	update the KeNB key based on the KAMF key, as specified in TS 33.501 [86];

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the received nextHopChainingCount-r15, as specified in TS 33.501 [86];

2>	store the nextHopChainingCount-r15 value;

2>	if the securityAlgorithmConfig-r15 is received:

3>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	if the received RRCConnectionReconfiguration includes the nr-Config and it is set to release; or

1>	if the received RRCConnectionReconfiguration includes endc-ReleaseAndAdd and it is set to TRUE:

2>	perform MR-DC release as specified in TS 38.331 [82], clause 5.3.5.10;

1>	if the received RRCConnectionReconfiguration includes the sk-Counter:

2>	perform key update procedure as specified in in TS 38.331 [82], clause 5.3.5.7;

1>	if the received RRCConnectionReconfiguration includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3.

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6.

1>	if connected as an RN:

2>	configure lower layers to apply the integrity protection algorithm and the KUPint key, for current or subsequently established DRBs that are configured to apply integrity protection, if any;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	if the received RRCConnectionReconfiguration includes the sCellGroupToAddModList:

2>	perform SCell group addition or modification as specified in 5.3.10.3e;

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2>	perfom the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.10.9;

1>	if the RRCConnectionReconfiguration message includes the sl-DiscConfig or sl-CommConfig:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.10.15;

1>	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2>	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1>	if handoverWithoutWT-Change is not configured:

2>	release the LWA configuration, if configured, as described in 5.6.14.3;

1>	release the LWIP configuration, if configured, as described in 5.6.17.3;

1>	if the RRCConnectionReconfiguration message includes rclwi-Configuration:

2>	perform the WLAN traffic steering command procedure as specified in 5.6.16.2;

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

1>	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

1>	if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated or mobilityControlInfoV2X:

2>	perform the V2X sidelink communication dedicated configuration procedure as specified in 5.3.10.15a;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

3>	include logMeasAvailableMBSFN;

2>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include logMeasAvailableBT;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include logMeasAvailableWLAN;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

2>	if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest:

3>	include perCC-GapIndicationList and numFreqEffective;

2>	if the frequencies are configured for reduced measurement performance:

3>	include numFreqEffectiveReduced;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82], clause 5.3.5.3;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

2>	release rach-Skip;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	if the UE is configured to provide IDC indications:

3>	if the UE has transmitted an InDeviceCoexIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2>	if the UE is configured to provide power preference indications, overheating assistance information, SPS assistance information, delay budget report or maximum bandwidth preference indications:

3>	if the UE has transmitted a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has transmitted a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType21 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of V2X sidelink communication related parameters relevant in target PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

3>	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell, except for BL UEs or UEs in CE when sameSFN-Indication is not present in mobilityControlInfo.

Next change

#### 5.3.10.10	SCG reconfiguration

The UE shall:

1>	if makeBeforeBreakSCG is configured:

2>	stop timer T313, if running;

2>	start timer T307 with the timer value set to t307, as included in the mobilityControlInfoSCG;

2>	start synchronising to the DL of the target PSCell, if needed;

2>	perform the remainder of this procedure including and following resetting MAC after the UE has stopped the uplink transmission/downlink reception with the source PSCell;

NOTE 0a:	It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source SCG PSCell to initiate re-tuning for the connection to the target cell, as specified in TS 36.133 [16], if makeBeforeBreakSCG is configured.

NOTE 0x: It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source SCG SCell(s) after receiving mobilityControlInfoSCG.

1>	if scg-Configuration is received and is set to release or includes the mobilityControlInfoSCG (i.e. SCG release/ change):

2>	if mobilityControlInfo is not received (i.e. SCG release/ change without HO):

3>	reset SCG MAC, if configured;

3>	if the UE is not configured with NE-DC:

4>	for each drb-Identity value that is part of the current UE configuration:

5>	if the DRB indicated by drb-Identity is an SCG DRB:

6>	re-establish the PDCP entity and the SCG RLC entity or entities;

5>	if the DRB indicated by drb-Identity is a split DRB:

6>	perform PDCP data recovery and re-establish the SCG RLC entity;

5>	if the DRB indicated by drb-Identity is an MCG DRB; and

5>	drb-ToAddModListSCG is received and includes the drb-Identity value, while for this entry drb-Type is included and set to scg (i.e. MCG to SCG):

6>	re-establish the PDCP entity and the MCG RLC entity or entities;

3>	configure lower layers to consider the SCG SCell(s), except for the PSCell, to be in deactivated state;

1>	if scg-Configuration is received and is set to release:

2>	release the entire SCG configuration, except for the DRB configuration (i.e. as configured by drb-ToAddModListSCG);

2>	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3>	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

2>	stop timer T313, if running;

2>	stop timer T307, if running;

1>	else:

2>	if scg-ConfigPartMCG is received and includes the scg-Counter:

3>	update the S-KeNB key based on the KeNB key and using the received scg-Counter value, as specified in TS 33.401 [32];

3>	derive the KUPenc key associated with the cipheringAlgorithmSCG included in mobilityControlInfoSCG within the received scg-ConfigPartSCG, as specified in TS 33.401 [32];

3>	configure lower layers to apply the ciphering algorithm and the KUPenc key;

2>	if scg-ConfigPartSCG is received and includes the radioResourceConfigDedicatedSCG:

3>	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

2>	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3>	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

2>	if scg-ConfigPartSCG is received and includes measConfigSN:

3>	for measConfigSN perform the actions as specified in 5.5.2 for measConfig unless explicitly stated otherwise;

2>	if scg-ConfigPartSCG is received and includes the sCellToReleaseListSCG:

3>	perform SCell release for the SCG as specified in 5.3.10.3a;

2>	if scg-ConfigPartSCG is received and includes the pSCellToAddMod:

3>	perform PSCell addition or modification as specified in 5.3.10.3c;

NOTE 0:	This procedure is also used to release the PSCell e.g. PSCell change, SI change for the PSCell.

2>	if scg-ConfigPartSCG is received and includes the sCellToAddModListSCG:

3>	perform SCell addition or modification as specified in 5.3.10.3b;

2>	configure lower layers in accordance with mobilityControlInfoSCG, if received;

2>	if rach-SkipSCG is configured:

3>	configure lower layers to apply the rach-SkipSCG for the target SCG, as specified in TS 36.213 [23] and TS 36.321 [6];

2>	if scg-ConfigPartSCG is received and includes the mobilityControlInfoSCG (i.e. SCG change):

3>	resume all SCG DRBs and resume SCG transmission for split DRBs, if suspended;

3>	stop timer T313, if running;

3>	start timer T307 with the timer value set to t307, as included in the mobilityControlInfoSCG, if makeBeforeBreakSCG is not configured;

3>	start synchronising to the DL of the target PSCell;

3>	initiate the random access procedure on the PSCell, as specified in TS 36.321 [6], if rach-SkipSCG is not configured:

NOTE 1:	The UE is not required to determine the SFN of the target PSCell by acquiring system information from that cell before performing RACH access in the target PSCell.

3>	the procedure ends, except that the following actions are performed when MAC successfully completes the random access procedure on the PSCell or when MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-skipSCG is configured:

4>	stop timer T307;

4>	release rach-SkipSCG;

4>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PSCell, if any;

4>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PSCell (e.g. periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PSCell;

NOTE 2:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

Next change

Annex G (normative): List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table G-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-181233: Successful acknowledgement of RRCConnectionRelease for BL and CE UE | 3324 | 1 | Release 13 | RRCConnectionRelease message, for which the poll bit is not set, can be considered succesfully acknowledged when UE has sent HARQ ACK feedback. |
| RP-182674: CR for T312 on LTE HetNet mobility | 3506 | 5 | Release 12 | Remove T312 in leaving condition for event trigger. |
| RP-182671: Corrections on paging monitoring and SI acquisition in RRC_CONNECTED for BL UEs and UEs in CE | 3647 | 2 | Release 13 |  |
| RP-190548: Update description of ack-NACK-NumRepetitions | 3899 | 2 | Release 13 |  |
| RP-190548: Corrections of NB-IoT Access Barring | 3900 | 2 | Release 13 |  |
| RP-191382: SI update notification and access barring in NB-IoT | 4020 | 2 | Release 13 |  |
| RP-19xx : Correction on handling of SCell(s) during Make Before Break handover | 3986 | 3 | Release 14 |  |
| NOTE 1: In case a CR has mirror CR(s), the mirror CR(s) are not listed. NOTE 2: The Additional Information column briefly describes the content of a CR in cases where the CR title may not be descriptive enough. If the CR title is descriptive enough, then the Additional Information column may be left blank. |  |  |  |  |

End of changes