3GPP TSG RAN meeting #85									RP-192278

Newport Beach, USA, September 16-20, 2019				revision of RP-191884

## Status Report to TSG

Agenda item:			9.4.9

| WI / SI Name | NR Mobility Enhancements |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | NR_Mob_enh |  |  |  |
| Unique ID | 800087 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190489 |  |  |  |
| Target Completion Date (indicate if changed) | Core part: 03/2020 |  | Performance part: 09/2020 |  |
| Overall Completion level | Core part: 40% |  | Performance Part: 0% |  |

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Yi Guo |
|  | Company | Intel Corporation |
|  | Email | yi.guo@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

RAN3 TU is reallocated to move 1 TU at RAN3#107bis to RAN3#106 (from 0.5 TU to 1 TU) and RAN3 #107 (from 1 TU to 1.5 TU) to correct misallocation.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

### 2.1.1	Agreements

#### 2.1.1.2	RAN1#98 Agreements

Conclusion on MBB:

Agreements:

- Support separate UE capability for NR-DC/CA and dual active protocol stack (DAPS) based HO interruption reduction solution.

Conclusion:

- Companies are encouraged to provide views and proposals to complete the physical layer specification in the next meeting for dual active protocol stack (DAPS) based HO solution agreed in RAN2.

  - The following are list of potential physical layer aspects that may be relevant for discussion:

    - How to leverage features supported by Multi-TRP WI

    - Procedures related to DL/UL operation

      - PDCCH monitoring, CORESET, and Search Space configuration for source and target cells

      - PDSCH resource allocation and transmission for source and target cells

        - How the simultaneous reception is performed, e.g. TDM

      - PUSCH resource allocation and transmission

        - How the simultaneous transmission is performed, e.g. TDM

        - Multi-beam PUSCH transmission (e.g. repetition of PUSCH)

        - Physical layer functionality needed to support RAN2 agreement, “Simultaneous UL PUSCH transmission does not need to be supported for the HO interruption solution.”.

      - HARQ-ACK, CSI, SR feedback

      - Uplink TA adjustments

      - Power control for PUSCH/PUCCH/SRS

      - Any other Tx/Rx beam related aspects

      - Physical layer aspects required to support DAPS based HO solution in FR2 (including determining feasibility and whether or not support feature for FR2)

    - UE capability aspects

### 2.1.2	Remaining Open issues

To specify the following solutions and agreements agreed during the study phase. [RAN2/RAN1/RAN3/RAN4]:

- To reduce interruption time during HO/SCG change:

  - Dual active protocol stack based HO interruption time reduction solution;

- To improve HO/SCG change reliability and robustness:

  - Conditional handover for NR PCell change;

  - Conditional handover based NR PSCell addition/change for any architecture option with NR PSCell;

  - T312 based fast failure recovery (similar to LTE)

## 2.2	RAN2

### 2.2.1	Agreements

#### 2.2.1.2 RAN2#107 Agreements

Reduction in user data interruption during handover or SCG change:

Regarding DC based solution vs Non-DC based solution, RAN2 concluded in study phase:

-	Introduce a solution for HO interruption time reduction based on dual active protocol stack.

Regarding support of RACHLess HO, RAN2 concluded in study phase:

-	We will not work on RACHless HO any further in Rel16 (Can be revisited if CFRA is not agreed to be part of 2 Step RACH in Rel-16)

Handover robustness improvements:

For CHO beam specific aspects, based on email discussion R2-1911500, RAN2 agreed:

1	For the scenario of multiple CHO cells being triggered the cell selected by the UE considering beams and beam quality. We will not specify normative requirements for the selection process but can be captured in an informative note in stage 3 spec.

2	No additional optimizations are introduced to improve RACH performance for CHO completion with multi-beam operation.

For CHO execution condition, based on email discussion R2-1909536, RAN2 agreed:

1  For FR1, we will leave it up to UE implementation to select the target cell if more than one candidate cell meets the triggering condition (same as for FR2).

2  Do not introduce “bye” message from UE to the source cell for CHO.

3  If UE receives conventional handover command, it will execute the handover command regardless of stored (configured) conditional handover command. This applies if the HO cmd is received before any CHO triggering condition is satisfied. FFS how HO failure is handled.

4 	The UE can’t receive and perform RRC configuration from source cell while executing CHO command (which means from the time when the UE starts synchronization with target cell).

FFS whether simultaneous connectivity and CHO can work simultaneously.

5	UE is not required to continue evaluating the triggering condition of other candidate cell(s) during CHO execution.

6	We will not change cell selection procedure due to CHO (T310 expiry, T304(-like) expiry, etc.)

7	CHO is optional feature for UEs and networks.

FFS: The working assumption remains a working assumption to be confirmed or otherwise at the next meeting

FFS: whether CHO candidates should be removed after the successful HO completion.

For CHO configuration, based on email discussion R2-1911733, RAN2 agreed:

1	As part of CHO configuration to be sent to the UE, RRC container is used to carry target cell configuration and source cell is not allowed to alter any content of configuration from the target cell.

2	Use add/mod list + release list to configure multiple CHO candidate cells. CHO execution condition can be updated by modifying the existing CHO configuration, Target cell configuration can be updated by modifying the existing CHO configuration.

3	Reuse the RRCReconfiguration/RRCConnectionReconfiguration procedure to signal CHO configuration to UE.

4	A RRC complete message is required for UE to confirm receipt and proper comprehension of CHO configuration (execution condition, FFS target cell configuration) to the source eNB/gNB.

FFS whether the UE is required to check the compliance of the target cell configuration within CHO configuration upon reception or whether it is allowed to check upon execution.

FFS whether different RRC processing requirements are defined for the reconfiguration with CHO command.

5	After CHO configuration has been sent to the UE, source configuration can be updated.

FFS whether CHO commands need to be updated after source reconfiguration.

6	Delta configuration for CHO commands is based on latest source configuration

7	Allow having multiple triggering conditions (using “and”) for CHO execution of a single candidate cell. Only single RS type per CHO candidate is supported. At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultnaeously. FFS on UE capability.

8	TTT is supported for CHO condition (as per legacy configuration)

In addition, RAN2 agreed:

1	Support conditional NR PSCell addition/change and reusing the conditional HO solution being developed. Supported for any architecture option with NR PSCell.

2	From RAN2 perspective conditional NR PSCell change can be supported for both intra-SN and inter-SN

3	S1/N2-based CHO is not supported for Release 16. RAN3 impacts to be discussed in RAN3. If we want to support this, the WID has to be updated and SA2 impacts identified.

RAN2 has sent LS R2-1911845 to trigger RAN3 work on CHO based PSCell change.

For fast recovery, RAN2 concluded in study phase:

-	Support T312 mechanism (similar to LTE). At least applicable for PCell

FFS Whether this is applicable for PSCell.

After the meeting, 5 email discussions are allocated:

1 [107#13][NR/Mob-enh] Running stage 2 CR  (Intel)

2 [107#18][LTE/NR/feMOB]  LS to SA3 on security impacts of multiple CHO targets (Sharp)

3  [107#29][NR/Mob-enh] CP for DAPS (Vivo)

4 [107#30][NR/LTE/Mob-enh] Configuration of CHO and execution condition (Intel)

5 [107#44][LTE and NR /feMOB] Discussion on PDCP details for RUDI HO (MediaTek/Huawei)

### 2.2.2	Remaining Open issues

To specify the following solutions and agreements agreed during the study phase. [RAN2/RAN1/RAN3/RAN4]:

- To reduce interruption time during HO/SCG change:

  - Dual active protocol stack based HO interruption time reduction solution;

- To improve HO/SCG change reliability and robustness:

  - Conditional handover for NR PCell change;

  - Conditional handover based NR PSCell addition/change for any architecture option with NR PSCell;

  - T312 based fast failure recovery (similar to LTE)

## 2.3	RAN3

### 2.3.1	Agreements

#### 2.3.1.1. RAN3#105 Agreements

Following CRs have been endorsed as baseline:

-	38.300 (R3-193359, Intel Corporation)

-	38.420 (R3-193361, Intel Corporation)

-	38.423 (R3-193360, Nokia, Nokia Shanghai Bell, Intel Corporation)

Regarding HO interruption reduction:

RAN3 agreed:

-	The PDCP SN and HFN are reset in the target eNB, unless DL and UL status are received from source eNB in the SN STATUS TRANSFER message.

-	WA: Include MBB HO indicator in XnAP Handover Request

-	WA: Add MBB HO indicator in NGAP: Handover Required

Handover robustness improvements:

RAN3 agreed:

-	CHO preparations for multiple target cells toward the same target node for the same UE are allowed;

-	TP R3-194641 for NR_Mob_enh BL CR for TS 38.423: Enabling efficient RRC reconfiguration and CHO cancel;

-	CHO Cancel procedure from the target node shall not be used in legacy HO

-	Both source and target nodes are allowed to initiate per-target cell CHO cancel; target node is only allowed to cancel a prepared CHO;

-	TP R3-194643 for NR Mob BL CR for TS 38.423: Conditional Handover - HO modification;

-	TP R3-194378 for NR_Mob BL CR for TS 38.300: Conditional Handover - Data Forwarding

-	Concentrate further discussion on the following 2 options: early forwarding, and late forwarding initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node

-	We agree to specify both “early forwarding” and “late forwarding” according to definitions in agreed St2 TPs; whether to use early data forwarding for CHO is FFS

### 2.3.2	Remaining Open issues

To specify the following solutions and agreements agreed during the study phase. [RAN2/RAN1/RAN3/RAN4]:

- To reduce interruption time during HO/SCG change:

  - Dual active protocol stack based HO interruption time reduction solution;

- To improve HO/SCG change reliability and robustness:

  - Conditional handover for NR PCell change;

  - Conditional handover based NR PSCell addition/change for any architecture option with NR PSCell;

  - T312 based fast failure recovery (similar to LTE)

## 2.4	RAN4

### 2.4.1	Agreements

#### 2.4.1.2. RAN4#92 Agreements

The way forward on R16 NR mobility enhancement in R4-1908234 has been approved:

The reply LS in R4-1909992has been approved;

Applicability of handover with simultaneous Tx/Rx:

Agreement: Feasibility of handover with simultaneous connectivity

–	Interband HO with DC enhancement allows different SCS to be configured for source and target cell.

Conditional handover && DC-enhancement based HO solutions

Agreement: Conditional handover does not add additional monitoring frequency layers, cells, and SSBs on top of existing Rel-15 UE measurement capability.

### 2.4.2	Remaining Open issues

Evaluations on solutions agreed in RAN2, dual active protocol stack (handover with simultaneous connectivity), conditional handover and T312 based fast recovery;

Specify the necessary UE performance requirements for the specified enhancements.

Applicability of handover with simultaneous Tx/Rx:

–	FFS on whether or not interfrequency intraband HO with DC enhancement supports different SCS to be configured for source and target cell

–	FFS on whether or not intrafrequency HO with DC enhancement supports different SCS to be configured for source and target cell

Handover delay for handover with simultaneous Tx/Rx:

–	The legacy handover delay definition may not be suitable for simultaneous connectivity handover.

–	The starting point of handover delay is defined as the time when UE receives RRC indicating handover. The ending point of handover delay is FFS.

–	RAN4 needs to discuss if delay and interruption requirement is needed for source cell release.

Handover delay for CHO:

The definition of when the CHO delay starts is FFS

- Exact delay equation is FFS

UE RRC processing time is broken into two segments:

- First segment is immediately after RRC HO command reception

- Second segment is after UE realizes the condition is met and identity of target cell is determined

## 2.5	RAN5

### 2.5.1	Agreements

### 2.5.2	Remaining Open issues

### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

### 2.6.1	Agreements

### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

### 3.1.1	Agreements with cross-TSG impacts

### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1908075	Physical layer aspects for NR mobility enhancements	Huawei, HiSilicon

- R1-1908246	Discussion on NR Mobility Enhancements in Physical Layer	ZTE

- R1-1908392	Physical layer feasibility considerations for mobility enhancement	MediaTek Inc.

- R1-1908513	Physical Layer Aspects for Mobility Enhancements	Samsung

- R1-1908663	Physical layer aspects of NR Enhanced mobility	Intel Corporation

- R1-1909226	Physical Layer Aspects for Mobility Enhancements	Ericsson

- R1-1909284	On NR mobility enhancements	Qualcomm Incorporated

- R1-1909343	Mobility enhancements physical layer aspects	Nokia, Nokia Shanghai Bell

- R1-1909702	Summary of discussions on physical layer aspects of enhanced mobility	Intel

- R1-1909227	Details on lower-layer mobility enhancements	Ericsson

- R1-1909312	Discussion on eMBB HO without packet duplication or simultaneous UL PUSCH transmission			Huawei, HiSilicon

- R2-1908641	LS on NR mobility enhancement (R4-1907730; contact: Intel)	RAN4

- R2-1908853	CHO Activation	Sharp

- R2-1908908	Comparison of Mobility Interruption time reduction solutions	Qualcomm Incorporated

- R2-1908909	Optimizing mobility interruption time for MR-DC mobility scenarios	Qualcomm Incorporated

- R2-1908910	Control Plane handling and procedures to support Make-Before-Break (MBB) HO	Qualcomm Incorporated

- R2-1908911	Control Plane handling and procedures to support Dual Connectivity Role Switch (DCRS) based HO	Qualcomm Incorporated

- R2-1908912	User Plane changes to minimize the mobility interruption during Make-Before-Break (MBB) HO	Qualcomm Incorporated

- R2-1908913	User Plane changes to minimize the mobility interruption during Dual Connectivity Role Switch (DCRS) based HO	Qualcomm Incorporated

- R2-1908914	Clarifications on PUSCH switch during Make-Before-Break (MBB) HO	Qualcomm Incorporated

- R2-1908915	Lossless Make-Before-Break (MBB) HO support for Low latency, high reliability services	Qualcomm Incorporated

- R2-1908916	ROHC handling during Make-Before-Break (MBB) HO	Qualcomm Incorporated

- R2-1908917	Source connection SCell handling during MBB HO	Qualcomm Incorporated

- R2-1908922	PDCP aspects Impacts of non-DC based Solutions	CATT

- R2-1908923	Some Considerations on CP of non-DC based Solutions	CATT

- R2-1908924	Discussion on Multiple CHO Execution Conditions	CATT

- R2-1908925	Definition of CHO Execution	CATT

- R2-1908926	Further Discussion on Cell Evaluation for CHO Cell Selection	CATT

- R2-1908927	Disscussion on Signalling Structure of CHO Configuration Message	CATT

- R2-1908928	Consideration on The UE Capability of Supporting CHO	CATT

- R2-1908931	RACH-Less HO design considerations	Qualcomm Incorporated

- R2-1908932	MBB HO support for simultaneous Tx not feasible scenarios	Qualcomm Incorporated

- R2-1908933	Support Tx sharing during MBB HO	Qualcomm Incorporated

- R2-1908934	Conditional HO configuration handling	Qualcomm Incorporated

- R2-1908935	RLM and RLF handling during MBB HO	Qualcomm Incorporated

- R2-1908944	NR-specific Issues for Conditional Handover Procedures	MediaTek Inc.

- R2-1908945	Remaining Issues of Conditional Handover Execution	MediaTek Inc.

- R2-1908946	RRC Configurations Related to Conditional Handover in NR	MediaTek Inc.

- R2-1908947	Measurement Report for Conditional Handover Procedures	MediaTek Inc.

- R2-1908948	Mobility Failure Recovery in NR	MediaTek Inc.

- R2-1908956	2-step RACH vs RACH-less HO	CATT

- R2-1908957	On the support of simultaneous reception and transmission during handover	Ericsson

- R2-1908958	Characteristics of the handover interruption solution	Ericsson

- R2-1908959	Release of source cell in MBB handover	Ericsson

- R2-1908960	Data forwarding for reduced handover interruption	Ericsson

- R2-1908961	Uplink handling during handover with simultaneous connectivity	Ericsson

- R2-1908962	ROHC handling in MBB handover	Ericsson

- R2-1908963	Capability and TDM pattern negotiation for MBB	Ericsson

- R2-1908964	Reduced handover interruption time for services using RLC UM	Ericsson

- R2-1908965	Signalling aspects of SCG change with minimal interruption	Ericsson

- R2-1908966	On the need for RACH-less handover	Ericsson

- R2-1908967	Supported scenarios for RACH-less HO	Ericsson

- R2-1908968	RACH-less configuration in NR	Ericsson

- R2-1908969	Beam aspects of RACH-less handover	Ericsson

- R2-1909009	RRC configuration of CHO commands	Nokia, Nokia Shanghai Bell

- R2-1909010	UE capabilities for CHO	Nokia, Nokia Shanghai Bell

- R2-1909083	Measurement report enhancement in conditional handover	Spreadtrum Communications

- R2-1909084	RRC signaling size restriction in CHO	Spreadtrum Communications

- R2-1909142	Source cell signalling during CHO execution	NEC

- R2-1909143	CHO configurations for multiple candidate cells	NEC

- R2-1909144	Reuse of conditional handover for SCG change in NR-DC	NEC

- R2-1909148	On Supporting Make-Before-Break in NR	Samsung

- R2-1909149	Introducing Make-Before-Break in NR	Samsung

- R2-1909150	On Supporting RACH-less in NR RRC aspect	Samsung

- R2-1909151	On Supporting RACH-less in NR RRC aspect	Samsung

- R2-1909152	On Supporting RACH-less in NR MAC aspect	Samsung

- R2-1909186	User Plane Details in DC-based Solutions	Mediatek Inc.

- R2-1909187	User Plane Details in Non-DC-based Solutions	Mediatek Inc.

- R2-1909188	General Consideration on HO/SCG Interruption in NR	Mediatek Inc.

- R2-1909227	Further Details on the Bye Message	ETRI

- R2-1909228	Problems in Evaluation of CHO Execution Condition	ETRI

- R2-1909229	Signalling Overhead Reduction for CHO	ETRI

- R2-1909230	Faster Handover Failure Recovery	ETRI

- R2-1909232	Common Single and Dual Active eMBB Solution	ETRI

- R2-1909233	Data Forwarding Options and Interruption Time	ETRI

- R2-1909255	Discussion on the leaving conditions for NR CHO	PANASONIC R&D Center Germany

- R2-1909285	On switching PUSCH in split-bearer solutions	Nokia, Nokia Shanghai Bell

- R2-1909286	RACH-less HO in beam-based system	Nokia, Nokia Shanghai Bell

- R2-1909287	CFRA resources update for Conditional Handover	Nokia, Nokia Shanghai Bell

- R2-1909288	On keeping CHO commands after handover	Nokia, Nokia Shanghai Bell

- R2-1909289	On the actions upon RLF or HOF when CHO was configured	Nokia, Nokia Shanghai Bell

- R2-1909290	Time to say goodbye to bye message in CHO execution	Nokia, Nokia Shanghai Bell

- R2-1909291	Implicit versus explicit release of prepared cells	Nokia, Nokia Shanghai Bell

- R2-1909292	On informing the candidate target about CHO execution condition	Nokia, Nokia Shanghai Bell

- R2-1909293	On CHO configuration consistency and validity	Nokia, Nokia Shanghai Bell

- R2-1909294	Considerations for failure recovery in NR	Nokia, Nokia Shanghai Bell

- R2-1909295	Beam selection and consolidation enhancements	Nokia, Nokia Shanghai Bell

- R2-1909298	Split bearer solution for reducing the service interruption time during HO in NR	Nokia, Nokia Shanghai Bell

- R2-1909299	On Random Access resource selection for CHO	Nokia, Nokia Shanghai Bell

- R2-1909319	Conditional handover performance in FR2 band	Ericsson

- R2-1909330	Configuration of conditional handover in NR	Ericsson

- R2-1909331	Triggering of conditional handover in NR	Ericsson

- R2-1909332	Conditional handover execution in NR	Ericsson

- R2-1909333	Handling of a HO command while UE is monitoring CHO	Ericsson

- R2-1909334	Conditional handover failures in NR	Ericsson

- R2-1909335	Suspend while monitoring CHO in NR	Ericsson

- R2-1909336	Security implications of CHO	Ericsson

- R2-1909337	Further beam specific aspects on CHO	Ericsson

- R2-1909338	Draft LS on CHO modification	Ericsson

- R2-1909339	Draft LS on CHO execution	Ericsson

- R2-1909340	Comparison of solutions for handover robustness	Ericsson

- R2-1909341	Fast RLF triggering based on timer T312	Ericsson

- R2-1909342	Fast handover failure recovery	Ericsson

- R2-1909343	Repetition of RRC messages at handover	Ericsson

- R2-1909344	TP for 38.331 on CHO	Ericsson

- R2-1909415	Stage-2 procedure for make-before-break handover	Ericsson

- R2-1909465	Comparison between non-DC and DC-based solutions for NR	CMCC

- R2-1909466	Discussion on fast HO Failure Recovery in NR	CMCC

- R2-1909536	Report of EmailDisc-41_CHO execution	vivo

- R2-1909539	Fast handover failure recovery in NR	vivo

- R2-1909540	Conditional Handover without explict trigger condition	vivo

- R2-1909542	Remaining issues for cell level triggers for conditional handover	vivo

- R2-1909571	Running CR for the introduction of NR mobility enhancement	Intel Corporation

- R2-1909573	CHO execution condition	Intel Corporation

- R2-1909574	Failure handling on CHO	Intel Corporation

- R2-1909575	Consideration on the progress and status of NR mobility WI	Intel Corporation

- R2-1909576	User plane handling for non-split bearer solution	Intel Corporation

- R2-1909577	Control plane consideration for dual active protocol stack (DAPS)	Intel Corporation

- R2-1909579	Further consideration on CHO configuration	Intel Corporation

- R2-1909580	Comparison of DC based vs. MBB based Approaches	Futurewei

- R2-1909581	Call Flow Analysis for Handover Interruption Time Reduction	Futurewei

- R2-1909582	Complexity Analysis of Handover Interruption Time Reduction	Futurewei

- R2-1909583	Performance Analysis of Handover Interruption Time Reduction	Futurewei

- R2-1909584	Single/Dual Transmitter and Single/Dual Protocol Stacks	Futurewei

- R2-1909597	Configuration Details of Conditional HO	InterDigital

- R2-1909598	Further details on CHO execution	InterDigital

- R2-1909609	Report of Email Discussion [106#40][NR/Mob enh] Beam specific aspects of CHO	Qualcomm Incorporated

- R2-1909669	Discussion on RLM for NR mobility	OPPO

- R2-1909671	Discussion on eMBB-based handover for NR mobility	OPPO

- R2-1909672	Comparison of DC-based and eMBB-based handover for NR mobility	OPPO

- R2-1909676	Summary of [106#42] [NR/LTE/mob enh] CHO configuration	OPPO

- R2-1909677	Further discussion on CHO execution and failure handling	OPPO

- R2-1909679	Further consideration on CHO configuration update	OPPO

- R2-1909689	Discussion on CHO triggering condition	Huawei, HiSilicon

- R2-1909690	Discussion on target cell selection for CHO	Huawei, HiSilicon

- R2-1909691	Discussion on T304 timer for CHO	Huawei, HiSilicon

- R2-1909692	Discussion on failure handling for CHO	Huawei, HiSilicon

- R2-1909693	Discussion on configuration of execution condition for CHO	Huawei, HiSilicon

- R2-1909694	Discussion on CHO deconfiguration	Huawei, HiSilicon

- R2-1909695	Discussion on RACH resource reservation for CHO	Huawei, HiSilicon

- R2-1909696	Discussion on the maximum number of candidate cells in CHO	Huawei, HiSilicon

- R2-1909697	Considerations on modification of CHO configurations by RRC signalling	Huawei, HiSilicon

- R2-1909698	Discussion on CHO execution	Huawei, HiSilicon

- R2-1909755	Considerations on fast handover failure recovery for NR mobility enhancements	Potevio

- R2-1909757	Discussion on multiple target cells fulfil CHO	CMCC

- R2-1909758	Discussion on failure handling	CMCC

- R2-1909769	Further Consideration of Beams for NR CHO	CMCC

- R2-1909776	Discussion on RLM RLF and HOF for non-DC-based solution	vivo

- R2-1909779	2-step RACH in mobility enhancement	vivo

- R2-1909780	Enhanced RACH-less solution for NR	vivo

- R2-1909859	Enhancement for Single UL capable UE	Apple

- R2-1909860	Consideration on non-DC based HO	Apple

- R2-1909861	RACH-less HO	Apple

- R2-1909862	Consecutive Conditional HO	Apple

- R2-1909891	Discussion on PDCP impact for eMOB	LG Electronics Inc.

- R2-1909892	Support 0ms interruption handover for UM DRB	LG Electronics Inc.

- R2-1909893	Preventing the tranmission of the uplink data	LG Electronics Inc.

- R2-1909941	CHO configuration/ execution, general aspects still remaining	Samsung Telecommunications

- R2-1909942	CHO configuration, condition related aspects still remaining	Samsung Telecommunications

- R2-1909943	Introduction of CHO	Samsung Telecommunications

- R2-1909944	Introduction of CHO	Samsung Telecommunications

- R2-1910384	Non DC based solution for 0ms interruption time	Intel Corporation, Mediatek Inc, OPPO, Google Inc., vivo, ETRI, CATT, China Telecom, Xiaomi, Charter Communications, ASUSTeK,  LG Electronics, NEC, Ericsson, Apple, ITRI

- R2-1910514	Discussion on legacy handover handling during CHO procedure	Huawei, HiSilicon

- R2-1910515	Discussion on CHO preparation	Huawei, HiSilicon

- R2-1910516	Discussion on early data forwarding for CHO	Huawei, HiSilicon

- R2-1910517	Discussion on UE behaviors upon re-establishment	Huawei, HiSilicon

- R2-1910518	Discussion on bye message for CHO	Huawei, HiSilicon

- R2-1910519	Discussion on combination of simultaneous connectivity and CHO	Huawei, HiSilicon

- R2-1910520	Discussion on Conditional SN Change in MR-DC scenario	Huawei, HiSilicon

- R2-1910521	Consideration on fast HO failure recovery	Huawei, HiSilicon

- R2-1910545	Time to trigger for CHO execution condition	Fraunhofer HHI

- R2-1910561	Conditional HO execution	Lenovo, Motorola Mobility

- R2-1910594	Problems of ROHC handling in eMBB HO	Huawei, HiSilicon

- R2-1910595	Discussion on eMBB HO without packet duplication or simultaneous UL PUSCH transmission	Huawei, HiSilicon

- R2-1910596	Handover schemes comparison under DC Scenarios	Huawei, HiSilicon

- R2-1910597	Summary of the major differences of DC HO vs MBB HO	Huawei, HiSilicon

- R2-1910598	Handover based on DC for 0ms interruption	Huawei, HiSilicon

- R2-1910599	Procedure analysis of DC-based handover	Huawei, HiSilicon

- R2-1910600	Security key change without L2 reset	Huawei, HiSilicon

- R2-1910601	DC based solution for intra-frequency mobility in NR	Huawei, HiSilicon

- R2-1910602	Mobility enhancements under CU-DU architecture	Huawei, HiSilicon

- R2-1910603	Discussion on TDM Pattern for intra-frequency Handover	Huawei, HiSilicon

- R2-1910604	Discussion on BWP indication for intra-frequency Handover	Huawei, HiSilicon

- R2-1910605	Further consideration on reply LSs from RAN1 and RAN4	Huawei, HiSilicon

- R2-1910606	Considerations on 0ms interruption handover for single TRX UE	Huawei, HiSilicon

- R2-1910607	Discussion on RACH-less solution	Huawei, HiSilicon

- R2-1910608	Discussion on position based solution for RACH-less handover	Huawei, HiSilicon

- R2-1910609	Discussion on combination of simultaneous connectivity and CHO	Huawei, HiSilicon

- R2-1910615	Details on failure handling	Lenovo, Motorola Mobility

- R2-1910684	Conditional Handover without explict trigger condition	vivo

- R2-1910690	Enhanced inter-system mobility in RRC_INACTIVE in spotty NR coverage	Ericsson

- R2-1910691	Enhancements for Conditional Handover in FR2	Apple Inc.

- R2-1910738	Discussion on the DL UL handling for Non-DC based solutions	ZTE Corporation, Sanechips

- R2-1910739	Discussion on the PDCP aspects for Non-DC based solutions	ZTE Corporation, Sanechips

- R2-1910740	Discussion on the need of RACH-less HO in NR	ZTE Corporation, Sanechips

- R2-1910741	Further  issues for CHO execution	ZTE Corporation, Sanechips

- R2-1910742	Further  issues for  CHO configuration	ZTE Corporation, Sanechips

- R2-1910743	Discussion on the fast RLF recovery with the applying of CHO and DC	ZTE Corporation, Sanechips

- R2-1910744	Discussion on applying CHO based SCG mobility	ZTE Corporation, Sanechips

- R2-1910807	Security Configuration for Conditional Handover	SHARP Corporation

- R2-1910808	[DRAFT] LS on AS key derivation for conditional handover	SHARP Corporation

- R2-1910895	UE Capability for enhanced NR mobility	Qualcomm Incorporated

- R2-1911058	Discussion on conditional handover	SHARP Corporation

- R2-1911111	CHO Executing Loop Issue when HOF	LG Electronics Inc.

- R2-1911112	Legacy HO and Configuration Handling Issues	LG Electronics Inc.

- R2-1911113	RACH-less Handling when CHO Execution	LG Electronics Inc.

- R2-1911114	RLM Handling of Enhanced MBB HO	LG Electronics Inc.

- R2-1911133	Consideration of beam aspects in NR CHO	China Telecom

- R2-1911159	Discussion on mobility interuption solutions in NR	China Telecom

- R2-1911227	Discussion on RACH-less handover for NR	ASUSTeK

- R2-1911283	NR eMOB solutions to achieve 0ms interruption time	SHARP Corporation

- R2-1911290	CHO UE behaviour upon transitioning to RRC_INACTIVE	Samsung R&D Institute India

- R2-1911292	Support of RACHless HO in NR	Samsung R&D Institute India

- R2-1911309	Specification Impacts to specify Single Stack solution for NR	Samsung R&D Institute India

- R2-1911310	Down-selecting non-DC based solution for NR eMOB	Samsung R&D Institute India

- R2-1911331	Study on application of CHO mechanism to Conditional PSCell addition/change	NTT DOCOMO INC.

- R2-1911344	Support of Conditional PSCell addition/change	NTT DOCOMO INC., Nokia, Ericsson, Fujitsu, KT Corp, Media Tek Inc., NEC, Qualcomm Inc., SoftBank Corp.

- R2-1911392	Introduction of early RLF declaration or fast handover failure recovery for PSCell in MR-DC	Samsung Electronics Polska

- R2-1911396	Introduction of T312 for IRAT EUTRA measurement in NR	Samsung Electronics Polska

- R2-1911397	Introduction of T312 for IRAT NR measurement in EUTRA	Samsung Electronics Polska

- R2-1911398	Introduction of early RLF declaration for PSCell in (NG)EN-DC	Samsung Electronics Polska

- R2-1911399	Discussion on fast RLF recovery enhancements in NR	Samsung Electronics Polska

- R2-1911410	Discussion on Performing CHO instead of RRE in CHO	Samsung Electronics Polska, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

- R2-1911422	Discussion on Interruption Requirements in CHO	Samsung Electronics Polska

- R2-1911425	LS on Interruption Timer in Conditional Handover	Samsung Electronics Polska

- R2-1911426	Discussion on Multiple Triggering Condition for CHO Execution	Samsung Electronics Polska

- R2-1911444	Consideration of multiple candidate target cell for NR CHO	Samsung R&D Institute UK

- R2-1911445	Introduction of T312 in NR	Samsung R&D Institute UK, Qualcomm Incorporated

- R2-1911446	Further enhancement of T312 in NR	Samsung R&D Institute UK

- R2-1911448	Introducing T312 for Pcell in NR	Samsung R&D Institute UK

- R2-1911449	Applying T312 on serving frequency in NR	Samsung R&D Institute UK

- R2-1911476	Control plane during eMBB mobility for NR	LG Electronics France

- R2-1911477	DL packet delay due to inter-node forwarding_NR	LG Electronics France

- R2-1911500	Report of Email Discussion [106#40][NR/Mob enh] Beam specific aspects of CHO	Qualcomm Incorporated

- R2-1911640	Offline discussion report on 800	Intel Corporation

- R2-1911733	Summary of [106#42] [NR/LTE/mob enh] CHO configuration	OPPO

- R2-1911761	[DRAFT] LS on Conditional PSCell addition/change	NTT DOCOMO

- R2-1911766	Report of offline discussion 74	Intel Corporation

- R2-1911817	[DRAFT] LS on Conditional PSCell addition/change	NTT DOCOMO

- R2-1911845	LS on Conditional PSCell addition/change	RAN2

- R3-193359 Baseline CR for introducing Rel-16 NR mobility enhancement (Intel Corporation)

- R3-193361 Baseline CR for introducing Rel-16 NR mobility enhancement (Intel Corporation)

- R3-193360 Baseline CR for introducing Rel-16 NR mobility enhancement (Nokia, Nokia Shanghai Bell, Intel Corporation)

- R3-194372Minimisation of Glitches in Data Transmission (Ericsson)

- R3-193569PDCP SN Continuity for RLC-UM (Qualcomm Incorporated)

- R3-193570PDCP SN Continuity for NR  RLC-UM (Stage 2 CR NR) (Qualcomm Incorporated)

- R3-194375 Comparison of solutions for reduction of interruption time at HO (Ericsson)

- R3-193929 Discussion intra-CU eMBB Handover (CATT)

- R3-193930 (TP for [NR_Mob_enh] BL CR for TS 38.473): intra-CU eMBB Handover (CATT)

- R3-193573 NG based MBB Handover (Qualcomm Incorporated)

- R3-193379Consideration on Conditional Handover (NTT DOCOMO, INC.)

- R3-194164Conditional Handover - HO preparation (Ericsson)

- R3-193495Further discussion on support of conditional handover (LG Electronics)

- R3-193500Handling of changing conditions during CHO (Nokia, Nokia Shanghai Bell)

- R3-193501(TP for NR_Mob_enh BL CR for TS 38.423): Enabling efficient RRC reconfiguration and CHO cancel (Nokia, Nokia Shanghai Bell)

- R3-193665CHO for Mobility Enhancement (NEC)

- R3-193572NG based Conditional Handover (Qualcomm Incorporated)

- R3-193931Consideration on CHO Signaling Procedure (CATT)

- R3-193450Further Discussion on CHO Preparation with Multiple Candidate Cells in the Same Target Node (ZTE)

- R3-194107Discussion on the FFSes for CHO (Huawei)

- R3-194108(TP for NR_Mob_enh BL CR for TS 38.423): Discussion on the FFSes for CHO (Huawei)

- R3-193452Further Discussion on Cancel of Ongoing CHO per Target Node (ZTE)

- R3-194065Conditional Handover Cancel & Modification (Samsung)

- R3-193451 Further Discussion on Modification of Ongoing CHO (ZTE)

- R3-194305(TP for NR Mob BL CR for TS 38.420): Conditional Handover - HO modification (Ericsson)

- R3-194304(TP for NR Mob BL CR for TS 38.423): Conditional Handover - HO modification (Ericsson)

- R3-193497Consideration on overload due to CHO (Nokia, Nokia Shanghai Bell)

- R3-193498(TP for NR_Mob_enh BL CR for TS 38.423): Resource allocation indication for CHO (Nokia, Nokia Shanghai Bell)

- R3-193499(TP for LTE_feMob BL CR for TS 36.423): Resource allocation indication for CHO (Nokia, Nokia Shanghai Bell)

- R3-193453Further Discussion on Applicability of CHO in MR-DC SN Change Scenario (ZTE)

- R3-194116Discussion on Conditional SN Change in MR-DC (Huawei)

- R3-194117CR to 37.340 on Discussion on Conditional SN Change in MR-DC (Huawei)

- R3-194118(TP for NR_Mob_enh BL CR for TS 38.423):Discussion on Conditional SN Change in MR-DC (Huawei)

- R3-194113(TP for NR_Mob_enh BL CR for TS 38.300): CHO procedure (Huawei)

- R3-193479TP for BL CR for TS 38.423 Handover Success Procedure in MR-DC@5GC (ZTE)

- R3-193477TP for BL CR for TS 38.423 CHO Cancel from Potential Target Node (ZTE)

- R3-194114F1 impact in CHO (Huawei)

- R3-194115CR to 38.473 on F1 impact in CHO (Huawei)

- R3-193932Discussion intra-CU CHO (CATT)

- R3-193933(TP for [NR_Mob_enh] BL CR for TS 38.473): intra-CU CHO (CATT)

- R3-194067Introducing CHO indication to gNB-DU and gNB-CU-UP (Samsung)

- R3-194072(TP for NR_Mob_enh BL CR for TS 38.473): Introducing CHO indication over F1 (Samsung)

- R3-194071(TP for NR_Mob_enh BL CR for TS 38.463): Introducing CHO indication over E1 (Samsung R&D)

- R3-193454Some Other Issues with CHO (ZTE)

- R3-193934The Issues on Data Forwarding Specifics of CHO (CATT)

- R3-193503Avoiding unnecessary data duplication during enhanced HO (Nokia, Nokia Shanghai Bell)

- R3-193380Data forwarding timing on Conditional Handover (NTT DOCOMO, INC.)

- R3-194110Discussion on data forwarding for CHO (Huawei)

- R3-194111(TP for NR_Mob_enh BL CR for TS 38.300): Discussion on data forwarding for CHO (Huawei)

- R3-194378(TP for NR_Mob BL CR for TS 38.300): Conditional Handover - Data Forwarding (Ericsson)

- R3-194379(TP for NR_Mob BL CR for TS 38.423): Conditional Handover - Data Forwarding (Ericsson)

- R3-193481More Usage of Handover Success Procedure (ZTE)

- R4-1908030	Further considerations on the RACH-less handover for NR	Apple Inc.

- R4-1908233	RRM requirement for conditional handover	Intel Corporation

- R4-1908234	Wayforward on NR mobility enhancement	Intel Corporation

- R4-1908369	Discussion on requirement of conditional handover	MediaTek inc.

- R4-1908479	On Conditional HO in NR Mobility Enhancements	Qualcomm Incorporated

- R4-1908487	Enhanced MBB NR handover interruption requirements	Qualcomm Incorporated

- R4-1908670	UE Requirements for CHO in NR	Nokia, Nokia Shanghai Bell

- R4-1908796	RRM requirements for conditional handover	NEC

- R4-1908869	Handover delay and interruption time for conditional handover	NTT DOCOMO, INC.

- R4-1909550	Further discussion on RAN4 aspects of NR mobility enhancements	Ericsson

- R4-1909551	Further discussion on RACHless handover for NR	Ericsson

- R4-1909552	Reply LS on simultaneous RX/TX and RACHless handover for NR with non zero/non equal TA	Ericsson

- R4-1909628	Discussion on simultaneous connectivity handover of NR mobility enhancements	Huawei, HiSilicon

- R4-1909629	Discussion on RACH-less handover of NR mobility enhancements	Huawei, HiSilicon

- R4-1909630	CR on RACH-less handover requirements for NR mobility enhancement	Huawei, HiSilicon

- R4-1909631	Discussion on conditional handover of NR mobility enhancements	Huawei, HiSilicon

- R4-1909992	Reply LS on simultaneous RX/TX  for NR	Ericsson

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template