3GPP TSG RAN Meeting #85	RP-192292

Newport Beach, USA, September 16-20, 2019	(revision of RP-191970)

# Source:	NTT DOCOMO, INC.

# Title:	New SID on inter-gNB coordination for multi-carrier/TRP operations for NR

# Document for:	Approval

Agenda Item:	8.2.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Feasibility study on inter-gNB coordination for multi-carrier/TRP operations for NR

Acronym: FS_NR_MultiCarrier-TRP

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X |  |  |
| No | X | X |  | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

3	Justification

NG-RAN enables operators to deploy the logical entities flexibly in their network. The NG-RAN architecture can be extended for further to cover the promising functionalities, e.g. CA and MIMO over multiple TRPs, so that these functionalities can be provided via two gNBs.

[CA]

When the new spectrum is deployed, CA (Carrier Aggregation) can be used, only if the new carrier and the other existing carriers are served by the same gNBs/gNB-DUs. If not, DC (Dual Connectivity) is used to aggregate the carriers served by the different gNBs/gNB-DUs.

However, comparing CA and DC, CA would be better from service area perspective because DC requires multiple UL transmission in proportion to the number of nodes involved with the DC operation. (In other words, for CA, the UE would not have to support multiple UL transmissions.)

Thus, for CA, operators’ need to ensure future extension capability for possible new spectrum even for current equipment; CAPEX and/or OPEX would be increased.

Furthermore, DC with more than one SCGs  is not possible in current specification.

[Multi-TRP]

In Rel-16, RAN1 will specify various flavors of multi-TRP solutions. For example, one primary use case is Multi-TRP with Multi-DCI: Independent scheduling between 2 TRP with same or separate MAC over non-ideal backhaul as shown in the figure below.

Figure X. Multi-DCI/Multi-TRP

The information and mechanisms for coordination required to support this feature across different gNBs/gNB-DUs (e.g. timing coordination or resource allocation between TRPs) are not currently supported by existing interfaces and would need to be defined.

The following aspects should be considered as part of the study item:

- Backwards compatibility for UEs supporting Rel-15 CA and/or Rel-16 Multi-TRP features

- Architecture-aware enhancements for Rel-17 UEs

- Requirements on (non-ideal) backhaul latency

- Whether existing interfaces can support these use cases

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this Study Item is to investigate the feasibility on inter-gNB coordination to achieve Carrier Aggregation and multi-TRP between gNB. Study should be mainly conducted in RAN3 based on the existing design of physical layer and radio protocols, with required RAN1 and RAN2 consultation and other WGs if needed (e.g. on a basis of liaison exchange).

Note: For CU-DU split, both inter-gNB coordination between gNB-DUs connected to same gNB-CU and difference gNB-CU should be considered.

The study is to be carried out as follows:

- Identify requirements to achieve CA and multi-TRP between gNBs (e.g, time constrains of HARQ/CSI feedback/TRP coordination, information to be transferred)

- Identify possible interface options (e.g. reusing legacy interface or new one)

- Evaluate possible interface options from the requirement perspective and potentially down select options to consider for further study

- Conclude on the feasibility of defining an interface for the inter-gNB coordination.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.8xx | Feasibility study on inter-gNB coordination for CA and MIMO over multiple TRPs | RAN#88 June 2020 | RAN#89 September 2020 |  |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

7	Work item leadership

Primary: RAN3

Secondary: RAN1, RAN2

8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

NTT DOCOMO, INC.

AT&T

Deutsche Telekom

Verizon Wireless

Bell Canada

Telecom Italia