3GPP TSG RAN meeting #86										RP-192402

Sitges, Spain, December 9-12, 2019

## Status Report to TSG

Agenda item: 9.4.10

| WI / SI Name | Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | LTE_NR_DC_CA_enh-Core |  |  |  |
| Unique ID | 800088 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-192336 |  |  |  |
| Target Completion Date (indicate if changed) | Core part: March/2020 |  | Performance part: September/2020 |  |
| Overall Completion level | Core part: 75% |  | Performance Part: 35% |  |

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Oumer Teyeb |
|  | Company | Ericsson |
|  | Email | oumer.teyeb@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

##### Power control for dual connectivity

###### RAN1-AH-1901 (January 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

Agreements:

- For Rel. 16 UEs and asynchronous NN-DC operation, where MCG has serving cells only in FR1 and the SCG has serving cells only in FR2, the uplink power control is performed independently across cell groups

  - This is under the assumption that for NR Rel. 16, no joint power limit across FR1 and FR2 is defined by RAN4.

- RAN1 has not identified any use case to support the case where SCG is fully in FR1 and MCG is fully in FR2 for both synchronous & asynchronous NN-DC operation. At the same time, if supported, RAN1 has not identified other RAN1 specification impact other than the power control aspect listed below and UE capability

  - If supported, power control is performed independently across the two cell groups.

Sent an LS to RAN4 (cc RAN2) capturing the above in R1-1901402.

###### RAN1#96 (Feb/Mar 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

  - No agreements, but following proposal in email discussion:

Proposals:

- For Rel. 16 NR-DC with FR1+FR1 band combinations both the semi-static power sharing and dynamic power sharing operations are supported.

  - FFS whether each power sharing operation is associated with the UE capability.

  - Email discussion till 3/15 – Kianoush (QC) – email discussion did not provide an agreement, but collected PC schemes for further discussion in the next meeting.

###### RAN1#96bis (April 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

  - No agreements. Companies are encouraged to check the alternatives listed in R1-1905839, and use them as the reference to provide further analysis. Revisit in RAN1#97

###### RAN1#97 (May 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

  - No agreements. Feature lead summary on Tdocs submitted in R1-1907891

###### RAN1-98 (August 2019)

Agreements:

  - Aim to reuse the existing CA power determination for uplink transmissions on CC(s) in a same CG.

Agreements: Slide 3 of R1-1909864 is agreed:

- [Offline consensus] Considering the following two alternatives for semi-static power sharing with PMCG+ PSCG≤PTotalNN- DC

    - Alt.1: For the uplink transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa.

      - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCGin MCG such that pMCG≤PMCG;

      - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and  pSCG can be up to  PSCG'  .

        - Alt.1-1:  PMCG' and PSCG'  are configured by RRC signaling.

        - Alt.1-2:  PMCG' and PSCG'  are determined by RAN4 requirement.

    - Alt.2: For the uplink transmission in MCG and in SCG, UE limits its actual transmission power pMCG to be up to PMCGand  pSCG to be up to PSCG

###### RAN1-98bis (October 2019)

Agreements:

- Adopt Alt.1-2 and Alt.2 for semi-static power sharing for NR-NR DC.

  - Alt.1-2 is only subject to configured maximum transmission power defined by RAN4

  - Configuration between Alt.1-2 and Alt.2 is supported.

  - FFS: add more clarification

  - FFS: applied for synchronous DC only or applied for both synchronous and asynchronous DC (which may be the same or different for Alt.1-2 and Alt. 2)

Agreements:

- For semi-static power sharing for NR-NR DC, to down-select during this week:

  - Alt 1: no PHR is reported in a CG for the other CG

  - Alt 2: Virtual PHR for active CCs of another CG

(The above change is the updated during the meeting.)

Agreements:

For dynamic power sharing for NR-NR DC (if supported), to down-select during this week:

- Alt 1: When PHR for a CG is reported in another CG, reusing Rel-15 EN-DC framework to determine the PHR (actual/virtual) for active CCs of in the CG

- Alt 2: Virtual PHR for active CCs of another CG

- Alt 3: no PHR is reported in a CG for the other CG

- (The above change is the updated during the meeting)

Agreements:

- Support dynamic power sharing

  - If there is no overlapping transmission, maximum power on CG i is determined by RAN4 spec without considering P_CG_i.

  - If there is overlapping transmission, maximum power on CG i is limited to P_CG_i.

  - Note: “look-ahead” operation is included as a UE capability below

- In case of power limitation, MCG is prioritized over SCG and reuse CA rule within each CG

- Optional UE capability to indicate the support of dynamic power sharing operation

- Separate optional UE capability to indicate the support of ’look-ahead’ operation on condition that UE indicates support of dynamic power sharing operation.

###### RAN1-99 (November 2019)

Agreements:

- Alt.2 of semi-static power sharing can be configured for both synchronous and asynchronous NR-DC scenarios.

Conclusion:

- At least from UE power sharing perspective, the cases where one or both CGs have CC[s] over FR1 and FR2 are supported in Rel.16 NR-DC

Agreements:

- Support per FR configuration of parameter NR-DC-PC-mode for NR-DC

Agreements:

- Alt.1-2 of semi-static power sharing can be configured for synchronous DC scenario only.

  - It is up to UE to determine whether the overlapping with UL transmission on the SCG is possible, if/when factors other than the TDD UL-DL configurations of the serving cells in the SCG (e.g., timing difference, drift) need to be taken into account.

Agreements:

Offline consensus #2 in R1-1913407 is agreed.

[Offline consensus #2]: Make the following modification to the previous agreement:

- Alt.1: For the uplink transmission occasion in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa. - If such overlapping for any ongoing uplink transmission(s) withand UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCGin MCG such that pMCG≤PMCG; - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and pSCG can be up to PSCG' . - Alt.1-1: PMCG' and PSCG' are configured by RRC signaling. - Alt.1-2: PMCG' and PSCG' are determined by RAN4 requirement.

Agreements:

- Regarding whether semi-static power sharing or dynamic power sharing is explicitly configured by RRC signalling.

  - parameter NR-DC-PC-mode is extended to include dynamic power sharing configuration.

Agreements:

For NR-NR DC, w.r.t. handling deprioritized uplink transmission

- left for UE implementation to determine scaling down or dropping

Agreements:

- For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0,

- UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and

  - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG

  - Otherwise, pwr_SCG <= Ptotal;

- UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission.

  - (working assumption) No new RRC signaling is introduced for T_offset:

    - Alt.1: T_offset <= T_proc,2

    - Alt.2: T_offset <= 2*T_proc,2

    - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms

    - To be addressed in the CR stage

##### Enhancements to single Tx switched uplink solution for EN-DC

###### RAN1-AH-1901 (January 2019)

Agreements:

- It is concluded that it is beneficial to specify enhancements to single Tx switched uplink solution for EN-DC over the existing single Tx switched uplink solution, e.g. via introducing a new reference HARQ configuration using NR/LTE Rel-15 or LTE eIMTA solution, etc.

###### RAN1#96 (Feb/Mar 2019)

Agreements:

For single tx switched UL in EN-DC with TDD PCell, the LTE PCell can be configured with DL-reference UL/DL config

  - For DL HARQ timing, the DL-reference UL/DL configuration is applied

    - Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference

  - For UL HARQ timing is the same as without reference configuration for PCell’s UL/DL configuration other than TDD pattern 0/6,

    - FFS: whether/how to support TDD pattern 0/6 for LTE PCell

  - UE is not expected to transmit on the MCG and SCG simultaneously

  - For type 2 UE (i.e., UE without dynamic power sharing capability):

    - UE is allowed to transmit LTE PUSCH only in the UL subframes designated as UL in the DL-reference configuration

  - For type 1 UE (i.e., UE with dynamic power sharing capability):

    - The UE should not assume that LTE PUSCH is only scheduled in the UL subframes associated configured by the DL-reference configuration

    - The UE should not assume that NR PUSCH is only scheduled in the remaining UL subframes other than those configured by the DL-reference configuration

    - If there is a collision,

      - In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH

      - In other UL subframes, UE behaviour to be expected to specified with details FFS

        - E.g., drop LTE PUSCH, drop NR PUSCH, etc.

      - FFS for the case of NR SRS & NR PRACH

    - Note: the impact of switching time (if non-zero) will be further studied.

    - Note: the above does not assume any restriction between gNB and eNB (e.g., tight coordination)

  - FFS: whether/how to support HARQ-offset (similar as in SUO case 1 in EN-DC with LTE FDD PCell)

###### RAN1#96bis (April 2019)

Agreements:

For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following clarifications are agreed:

- For the LTE TDD PCell: use the PDSCH ACK timeline as in Table 10.1.3.1-1 in 36.213)

- For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell

  - For the LTE FDD SCell: use the PDSCH ACK timeline defined for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

  - FFS

    - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

    - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

Agreements:

For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration,

- Support PUCCH formats 3/4/5, PUCCH fallback operation based on implicit resource indication is not supported.

  - FFS PUCCH format and resource determination when the UE only receives DAI=1

  - FFS whether or not there are any additional issues related to SPS

###### RAN1#97 (May 2019)

  - No agreements.

###### RAN1-98 (August 2019)

Agreements:

- Regarding the FFS part (in change marks) in the agreement from RAN1 #96bis:

For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following is agreed:

- For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell (already agreed in RAN1 #96bis)

  - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

  - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

- Support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell

- Note: from UE perspective, it is expected that HARQ-offset value doesn’t violate the DL/UL configuration (in SIB1).

- For type 1 UE, the feature is optional. FFS for type 2 UE.

Agreements:

R15 specification on “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” (i.e., case1 HARQ timing in single UL), is applied to EN-DC UE capable of dual UL Tx in EN-DC with LTE FDD PCell to mitigate DL de-sensing due to Harmonics, at least including:

- UE behavior specified in 36.213 and 36.212

- FFS: all uplink subframes can be scheduled for LTE

Sent an LS to RAN2 notifying RAN2 on the above agreement in R1-1909880

Agreements:

When UE is configured with DL-reference config on the LTE PCell to support single UL in EN-DC with LTE TDD PCell, regarding the case of LTE PCell’s TDD pattern 0 & 6 (as configured in SIB1)

- Not supported in R16.

Proposal below under email approval until September 5th:

For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration, if an explicit PUCCH format 1a/1b resource is configured, option 2; otherwise, option 1

- Option 1: Support only PUCCH formats 3/4/5 when DAI=1, i.e., PUCCH fallback operation is not supported.

  - Default PUCCH resource corresponding ARI=00 is used when the UE only receives DAI=1 or SPS PDSCH.

- Option 2: Allocate an explicit PUCCH format 1a/1b resource for the fallback HARQ-ACK transmission corresponding to the PDCCH with DAI = 1 or SPS PDSCH.

Email discussion was inconclusive, to be continued in the next meeting

###### RAN1-98bis (October 2019)

Agreements:

For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration, Using only PUCCH formats 3/4/5.

- A default PUCCH format 3 resource is used when the UE only receives PDCCH with DAI=1 and/or SPS PDSCH.

  - The default one is corresponding to ARI=00

  - Note: the existing multiplexing rule for HARQ-ACK, SR, and CSI onto PUCCH format 3 remains unchanged

- No PUCCH resources are reserved for SPS PDSCH for the UE

Agreements:

If a UE is configured with Case1 HARQ timing in EN-DC, with LTE TDD Pcell and (as a working assumption) with LTE FDD PCell, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

Agreements:

- Introduce a new UE capability of supporting DL reference configuration for dual-Tx UL (e.g., to handle DL de-sensing from harmonics)

  - Agreements:

- For the single-Tx case, for FDD LTE Pcell,

  - All uplink subframes can be scheduled for LTE for type 1 UEs

    - In which case, NR transmission is dropped for when the LTE and NR transmissions collide

    - Note: there is no change of UL scheduling timing for LTE

  - Agreements:

- For the dual-Tx case, for FDD LTE Pcell,

  - All uplink subframes can be scheduled for LTE at least for type 1 UEs

    - Note: there is no change of UL scheduling timing for LTE

  - Agreements:

- Support new RRC signaling “tdm-PatternConfig-r16” to configure a UE with a DL-reference config to the MCG PCell (LTE FDD) according to Rel-16 behavior

- FFS: Introduce a new per-BC capability bit for UE to indicate that DL de-sensing from Harmonic need to be handled by configuring DL-reference config to the LTE PCell and limit LTE UL transmissions to a subset of the subframes

###### RAN1-99 (November 2019)

Agreements:

Confirm the working assumption part of the agreement from RAN1 #98bis below:

If a UE is configured with Case1 HARQ timing in EN-DC, with LTE TDD Pcell and (as a working assumption) with LTE FDD PCell, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

Agreements:

- For the single-Tx case, for TDD LTE Pcell,

  - All uplink subframes can be scheduled for LTE for type 1 UEs

    - In which case, NR transmission is dropped for when the LTE and NR transmissions collide

Agreements:

For a UE configured with DL-reference DL/UL configuration in Rel-16 (including single Tx with LTE TDD PCell or LTE FDD PCell, and dual Tx cases):

- For type 2 UE (i.e., UE without dynamic power sharing capability), any LTE UL transmissions should take place only in UL subframes designated for HARQ-ACK feedback.

- For type 1 UE (i.e., UE with dynamic power sharing capability),

  - Confirm that any LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated for HARQ-ACK feedback.

  - FFS UE is not expected to transmit semi-statically configured LTE UL transmissions in the UL subframes other than those designated as UL in the DL-reference configuration if such transmission collide with NR UL transmissions.

Agreements

For the FFS part in the agreement above,

- semi-statically configured LTE UL transmissions are allowed in all UL subframes.

  - Note: In case of collision, LTE transmission is prioritized

  - Note: this configuration is subject to UE capability

Agreements:

Clarify that “tdm-PatternConfig-r16” as agreed in RAN1 #98bis is applied in both single Tx case  (with LTE TDD PCell or LTE FDD PCell) and dual-Tx case.

Conclusion:

The value range of HARQ-offset is {0, 1, 2, 5, 6} in the DL-reference configuration for SUO of EN-DC with LTE TDD PCell, where the applicability of the values is shown below:

| Pcell Cell-specific TDD config | DL-Reference TDD config | Valid HARQ-offset value |
| --- | --- | --- |
| 1 | 2 | (0,1,5,6) |
|  | 4 | (0,5) |
|  | 5 | (0,1,5,6) |
| 2 | 5 | (0,5) |
| 3 | 4 | (0,1) |
|  | 5 | (0,1,2) |
| 4 | 5 | (0,1) |
| 5 | 5 | (0) |

- Note the above value range and the table can be derived from the previous agreement

- Note that the last row of the above table is included for completeness, which does not imply the necessity of a RRC configuration

Agreements:

Clarify the notes in the following two agreements from RAN1 #98bis (only high-lighted text is updated):

  - Agreements:

- For the single-Tx case, for FDD LTE Pcell,

  - All uplink subframes can be scheduled for LTE for type 1 UEs

    - In which case, NR transmission is dropped for when the LTE and NR transmissions collide

    - Note: there is no change of UL scheduling timing for LTE compared to R15 single-Tx with LTE FDD PCell

  - Agreements:

- For the dual-Tx case, for FDD LTE Pcell,

  - All uplink subframes can be scheduled for LTE at least for type 1 UEs

    - Note: there is no change of UL scheduling timing for LTE compared to R15 single-Tx with LTE FDD PCell

##### Efficient and low latency serving cell configuration/activation/setup

###### RAN1-AH-1901 (January 2019)

  - No agreements.

###### RAN1#96 (Feb/Mar 2019)

  - No agreements. R1-1903812 summarizes all the proposals and collects some companies views to them

###### RAN1#96bis (April 2019)

  - LS to RAN4 in R1-1905912 on the maximum allowed SCell activation delay for Rel16 CA agreed, asking RAN4 feedback on the components of the Rel-15 activation delay.

###### RAN1#97 (May 2019)

  - No agreements.

###### RAN1-98 (August 2019)

Conclusion:

Study further the following two options and aim to conclude in RAN1#98bis:

- Option 1:

  - NW can send activation command MAC CE and also independently use existing signalling to trigger aperiodic/semi-persistent CSI-RS when sending the activation command

  - Request RAN4 to consider specifying additional (tighter) maximum allowed activation delay requirements for following case

    - UE receives activation command MAC CE for Scell activation and also aperiodic/semi-persistent CSI-RS for the Scell ’at the same time’ as the activation command

      - FFS: ’at the same time’, i.e., exact timing between activation command and RS trigger

      - FFS: if separate requirement based on specific RS configuration

- Option 2 - Support enhancement for triggering a specific CSI measurement/reporting configuration during MAC-CE based Scell activation

  - CSI measurement/reporting is based on Rel-15/16 RS(s)

  - FFS RS will be A-TRS, SP CSI-RS, aperiodic CSI-RS, SP TRS, etc.

  - FFS between following candidates for enhanced triggering

    - Implicit (i.e., Scell activation command reception implies specific RS is triggered like in LTE)

    - Separate MAC CE linked to activation MAC CE

    - Enhanced activation command MAC CE

    - Other candidates not precluded

Note: companies (especially those interested in option 2) should bring ‘full proposals’ covering all design aspects for the next meeting.

Agreements:

- From RAN1 perspective, L1 based mechanism for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells can be supported

  - ‘dormancy-like’ => sparse/no PDCCH monitoring on activated Scell while maintaining CSI measurements/reporting

Email discussion/approval of the proposal below until September 5th

- Study further following candidates for L1 based mechanism and aim to conclude at RAN1#98bis

  - Option 1

    - Introduce dedicated DCI on Pcell (or cell x) for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scell(s) (or Scell(s) other than cell x)

    - For case when Scell is configured with multiple BWPs

      - Alt1: DCI indicates the BWP to be used on the Scell (FFS 1 or 2 bits per Scell)

      - Alt2: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of the Scell (1 bit per Scell)

    - For case when Scell is configured with 1 BWP

      - DCI indicates whether to monitor/not monitor PDCCH on the Scell (1 bit per Scell)

    - DCI is applicable for case when UE is not configured with CIF (i.e., no cross-carrier scheduling)

      - FFS for cross-carrier scheduling case

    - DCI details

      - At least 15 DCI bits (i.e., 1 bit per Scell)

      - Monitored in UESS along with other scheduling DCI

      - No extra BDs for monitoring this DCI (so same size or size matched to one of DCI 0-0/0-1/1-0/1-1)

        - Alt 1: DCI CRC scrambled with C-RNTI but some fields reserved in one of DCI 0-0/0-1/1-0/1-1, and other fields used for triggering the dormancy behavior

        - Alt 2: DCI CRC scrambled by another RNTI (i.e., similar to SPS PDCCH release)

    - Application delay

      - <= BWP switching delay;

    - The dedicated DCI is not expected to be used for data scheduling

  - Option 2

    - Introduce new RRC signaling based on which the Bandwidth part indicator field in DCI of cell x is re-interpreted by the UE to indicate the BWP used for cell x and also cell(s) other than cell x

    - For case when UE is configured with 1 BWP on cell x or on cell(s) other than cell x

      - Rel15 behaviour is applied (i.e., no CA enhancement for this case)

    - For case when UE is configured with multiple BWPs on cell x or on cell(s) other than cell x

      - RRC signalling for case when UE is not configured with cross-carrier scheduling

        - Details FFS

      - RRC signalling for case when UE is configured with cross-carrier scheduling

        - Details FFS

    - Grouping of multiple Scells/BWPs

      - Details FFS

    - Application delay

      - <= BWP switching delay;

  - Option 3

    - Introduce new RRC signaling for per-BWP configuration of cross-carrier vs. self scheduling and use the CIF/BWP indicator fields for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells.

      - For case when UE not configured with cross-carrier scheduling/CIF

        - Rel15 behaviour is applied (i.e., no CA enhancement for this case)

      - For case when UE is configured with 1 BWP on scheduled cells other than Pcell

        - Rel15 behaviour is applied (i.e., no CA enhancement for this case)

    - Application delay

      - <= BWP switching delay;

The email discussion agreed the following

For L1 based mechanism for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells, study further the candidates including the following and aim to conclude at RAN1#98bis

o   Option 1

  - Introduce dedicated DCI on Pcell (or cell x) for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scell(s) (or Scell(s) other than cell x)

  - For case when Scell is configured with multiple BWPs

i.      Alt1: DCI indicates the BWP to be used on the Scell (FFS 1 or 2 bits per Scell)

ii.      Alt2: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of the Scell (1 bit per Scell)

Note: It is also possible that the dedicated DCI is used to support both Alt. 1 and Alt. 2

  - For case when Scell is configured with 1 BWP

i.      Alt1: DCI indicates whether to monitor/not monitor PDCCH on the Scell (1 bit per Scell)

ii.      Alt2: DCI indicates switching between sparse PDCCH monitoring and frequent PDCCH monitoring (1bit per Scell)

  - DCI is applicable for case when UE is not configured with CIF (i.e., no cross-carrier scheduling)

i.      FFS for cross-carrier scheduling case

  - DCI details

i.      At least [15] DCI bits (i.e., 1 bit per Scell)

ii.      Monitored along with other scheduling DCI

iii.      No extra BDs for monitoring this DCI

      - Alt 1: DCI CRC scrambled with C-RNTI but some fields reserved in one of DCI 0-0/0-1/1-0/1-1, and other fields used for triggering the dormancy behavior

      - Alt 2: DCI CRC scrambled by another RNTI (i.e., similar to SPS PDCCH release)

      - Other alternatives not precluded (e.g. monitored in CSS with size similar to DCI 2-x)

  - Application delay

i.      <= BWP switching delay; BWP switching delay is starting point

ii.      FFS: if RF switching is applied for inter-band CA, and impact on application delay

  - The dedicated DCI is not expected to be used for data scheduling

  - FFS: If ACK should be transmitted in response to receiving the DCI (e.g. like SPS release PDCCH)

o    Option 2

  - Introduce new RRC signaling which allows to use the existing Bandwidth part indicator field in DCI of cell x/BWP timer expiration to indicate the BWP used for cell x and also cell(s) other than cell x

  - For case when UE is configured with 1 BWP on cell x or on cell(s) other than cell x

i.      Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: other alternatives not precluded

  - For case when UE is configured with multiple BWPs on cell x or on cell(s) other than cell x

i.      RRC signalling for case when UE is not configured with cross-carrier scheduling

      - Details FFS

ii.      RRC signalling for case when UE is configured with cross-carrier scheduling

      - Details FFS

  - Grouping of multiple Scells/BWPs

i.      Details FFS

  - Application delay

i.      <= BWP switching delay; BWP switching delay is starting point

ii.      FFS: if RF switching is applied for inter-band CA, and impact on application delay

iii.      FFS details: e.g. For UE indicating Type 2 capability, whether Type 1 BWP switching delay is used for some BWP switch cases (e.g. for switching between BWPs that differ only in SS configuration)

o    Option 3

  - Introduce new RRC signaling for per-BWP configuration of cross-carrier vs. self scheduling and use the CIF/BWP indicator fields for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells.

i.      For case when UE not configured with cross-carrier scheduling/CIF

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

ii.      For case when UE is configured with 1 BWP on scheduled cells other than Pcell

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

  - Application delay

i.      <= BWP switching delay BWP switching delay is starting point

ii.      FFS: if RF switching is applied for inter-band CA, and impact on application delay

o   Option 1a

- Introduce new RRC signalling to create a mapping of Scell/Scell group(s) to the indication field (x-bit) in the new and new dedicated DCI on Pcell (or cell x) for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on groups of activated Scell(s) (or groups of Scell(s) other than cell x)

i.      FFS details of RRC signaling to group the Scells

- For case when Scell is configured with multiple BWPs

i.      Alt1: DCI indicates the BWP to be used on the group of Scell(s)

ii.      Alt2: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of the group of Scell(s)

Note: It is also possible that the dedicated DCI is used to support both Alt. 1 and Alt. 2

- For case when Scell is configured with one or multiple BWPs

i.      Alt1: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of a the group of Scells (1 bit per group of Scells)

ii.      Alt2: DCI indicates switching between sparse PDCCH monitoring and frequent PDCCH monitoring for the group of Scell(s)

- DCI is applicable for case when UE is not configured with CIF (i.e., no cross-carrier scheduling)

- FFS for cross-carrier scheduling case

- DCI details FFS

- Application delay

- <= BWP switching delay; BWP switching delay is starting point

- FFS: if RF switching is applied for inter-band CA, and impact on application delay

- FFS between below alternatives

  - Alt1: The new DCI is dedicated DCI and is not expected to be used for data scheduling

  - Alt2: The new DCI is included along with scheduling DCI (i.e., enhanced field in scheduling DCI)

- FFS: If ACK should be transmitted in response to receiving the DCI (e.g. like SPS release PDCCH)

o    Option 3a

- When UE is configured with cross-carrier scheduling, use CIF/BWP indicator fields for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells

- Use per serving cell configuration of cross-carrier vs. self scheduling as in Rel-15

- For case when UE not configured with cross-carrier scheduling/CIF

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

- For case when UE is configured with 1 BWP on scheduled cells other than Pcell

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

- Application delay

- <= BWP switching delay BWP switching delay is starting point

- FFS: if RF switching is applied for inter-band CA, and impact on application delay

o    Option 3b

- Configure CIF to be present in DCI format in Pcell search-space even if cross-carrier scheduling is not configured

- Details FFS

- Application delay

- <= BWP switching delay BWP switching delay is starting point

- FFS: if RF switching is applied for inter-band CA, and impact on application delay

Note 1: FFS: If any additional UE behavior is needed when UE is also configured for receiving PDCCH based power saving signal/channel outside active time

Note 2: “BWP” => UE specific RRC configured BWP

Note 3: ‘dormancy like behavior’ may also include CSI measurement/reporting relaxation for some cases

Note 4: It is not precluded to further combine and modify above options

###### RAN1-98bis (October 2019)

Agreements:

- When the UE is outside Active Time, for the L1 based mechanism for transitioning from ’dormancy-like’ to ’non-dormancy like’ behaviour on activated Scells, an explicit information field for the UE is introduced to the PDCCH WUS

  - The explicit information field is configurable within a range of 0 to X1 bits

    - X1 <<15

    - FFS whether to use BWP framework for transitioning from dormany to non-dormancy

  - FFS the case of ’non-dormancy-like’ to ’dormancy like’ transition

- When the UE is in the Active Time, for the L1 based mechanism for transitioning between ’dormancy-like’ and ’non-dormancy like’ behaviour on activated Scells, an explicit information field is newly introduced to at least DCI formats 0-1 and 1-1 for the primary cell

  - The explicit information field is configurable within a range of 0 to X2 bits

    - X2 <<15

    - FFS whether to use BWP framework for transitioning from dormany to non-dormancy or vice versa

  - FFS The DCI formats may or may not schedule data (if supported w/o data, the value of X2 can be separately discussed)

  - FFS DCI formats 0-0 and 1-0

  - FFS the impact of CIF if any

Agreements:

- For the L1 based Scell dormancy indication sent on primary cell within active time

  - UE is configured with at least two BWPs for an Scell

    - The explicit information field in DCI indicates switching to/from dormant BWP configured for the Scell

      - FFS definition of dormant BWP

      - FFS whether or not to the same BWP switching delay to the non-dormant to dormant transition delay

  - Note: Rel15 behavior for case when 1BWP is configured for the Scell (i.e., no dormancy indication for that Scell)

R1-1911653

Agreements:

- For the L1 based Scell dormancy indication sent on primary cell outside active time in WUS PDCCH

  - The explicit information field in DCI is a bitmap with up to X1 bits and 1 bit per group of configured Scells

    - Each Scell group can have one/multiple Scells and up to X1 Scell groups are configured via RRC.

      - The Scell group configuration is independent from the Scell group configuration for dormancy indication within active time (if supported)

    - X1 = [5]

      - Note: X1 is upper bound.

  - Note: Number of bits used for explicit information field in WUS PDCCH is based on configuration

Proposals:

- For the L1 based Scell dormancy indication sent on primary cell within active time select one of the below alternatives

  - Alt 1

    - The PDCCH with dormancy indication is not used for scheduling data

      - Note: At least DCI formats 0-1 and 1-1 are used for the PDCCH (as already agreed)

    - The explicit information field in DCI is a bitmap with 1 bit per configured Scell

      - Note: No additional RRC configuration of Scell grouping

    - X2=15

      - Note: X2 is upper bound

    - FFS between below options

      - New RNTI to scramble the PDCCH DCI (i.e., similar to CS-RNTI)

      - Reuse C-RNTI but reserve the FDRA field to all 1s

      - Reuse C-RNTI but add a dedicated bit to differentiate from scheduling DCI

    - FFS: HARQ-ACK handling for the dedicated DCI

  - Alt 1a

    - The PDCCH with dormancy indication is also used for scheduling data on the primary cell

    - The explicit information field in DCI is a bitmap (appended to scheduling DCI) with up to X2 bits, and 1 bit per group of configured Scells

      - Each Scell group can have one/multiple Scells and up to X2 Scell groups are configured via RRC.

    - X2 = 5

      - Note: X2 is upper bound.

    - Number of bits used for explicit information field in DCI is variable

  - Email discussion/approval till 10/25 – Ravi (Ericsson)

###### RAN1-99 (November 2019)

Agreements:

- At least for the case when PDCCH schedules data for primary cell and also indicates dormancy for Scell(s) (i.e., Case 1),

  - N (0≤N≤X2) Scell groups are configured for the UE where each Scell group can have one or multiple Scells

    - Note: The Scell group configuration is independent from the agreed Scell group configuration for dormancy indication outside active time

    - Note: X2=5 per RAN1#98bis working assumption.

  - The explicit information field for SCell dormancy indication is a bitmap of length N with each bit corresponding to one Scell group.

  - The bitmap is appended to existing fields of DCI format 0-1,1-1 (i.e., size of DCI format 0-1, 1-1 is increased by N (0≤N≤X2) bits).

Agreements:

Fall back DCI formats (0_0 & 1_0) are not used for dormancy indication

Agreements:

- When PDCCH with DCI formats 1-1 is used for indicating dormancy for SCells,

  - UE expects that the PDCCH is not used for PDSCH scheduling

    - If FDRA field in PDCCH DCI format 1-1 is set to all 1s (when type 1 RA is used for UE) or

    - If FDRA field in PDCCH DCI format 1-1 is set to all 0s (when only type 0 RA is used for UE)

  - Note: Samsung is concerned with the above agreements since Samsung believes it is a duplicate solution (compared with the one using some bits in DCI scheduling PUSCH/PDSCH)

Agreements:

- For SCell dormancy indication outside active time, confirm X1=5

Agreements:

- For the case when PDCCH with DCI format 1-1 is used for indicating dormancy for SCells, and when UE is indicated that the PDCCH is not used for PDSCH scheduling (i.e., Case 2)

    - The explicit information field for SCell dormancy indication is a bitmap of length N1 where N1 is the number of configured Scells for the UE, and each bit in the bitmap corresponds to one configured SCell

    - The following fields are re-purposed in the PDCCH for dormancy indication– MCS (5), NDI (1), RV(2), HARQ process number(4), Antenna port(s) (at least 2 4), DMRS sequence initialization

      - Other fields are not re-purposed

        - FFS whether or not CIF, if present, can indicate a Scell or not

  - Note: the DCI format size is same as that of Case 1 (i.e., if RRC configures N (0≤N≤X2) SCell groups, N bits are added to the DCI)

Agreements:

- When UE is outside Active Time, for the L1 based mechanism for transitioning between ‘dormancy-like’ and ‘non-dormancy like’ behaviour, the same BWP framework as inside active time is used

Agreements:

- At least for case of dormancy indication within active time

  - If ‘0’ is indicated by DCI field

    - If ‘UE is in non-dormant BWP, UE switches to dormant BWP

    - If ‘UE is in dormant BWP, UE continues with dormant BWP

  - If ‘1’ is indicated by DCI field

    - If ‘UE is in non-dormant BWP, UE continues with the same non-dormant BWP

    - If UE is in dormant BWP, switch to a specific non-dormant BWP explicitly configured by RRC

Agreements

For dormancy indication outside active time, for interpreting ‘0’, ‘1’ in the SCell dormancy indication field,

- reuse same approach as that of inside active time

Agreements:

- DCI format 0-1 is not used for Case 2 dormancy indication

  - Conclusion:

- From RAN1 perspective,

  - Application delay for transitions between dormant BWP and non-dormant BWP will be specified by RAN4.

  - Until further RAN4 input is received, current DCI based BWP switching time that is supported by the UE is assumed as the application delay.

Agreements:

- For Type 2 codebook, ACK is transmitted by the UE in response to detection of Case 2 PDCCH with SCell dormancy indication

- For type 1 codebook, no HARQ response is supported in response to detection of Case 2 PDCCH with SCell dormancy indication

  - Agreements:

- If the default BWP is not the dormant BWP, BWP inactivity timer is not used for transitioning from dormant BWP to another BWP

##### Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers

###### RAN1-AH-1901 (January 2019)

  - No agreements

###### RAN1#96 (Feb/Mar 2019)

Agreements:

- At least for the case of lower SCS PDCCH scheduling a higher SCS PDSCH the earliest possible starting point for the PDSCH is defined by the end of the PDCCH +

  - >0. Detailed value(s) FFS

  - FFS other factor(s) impacting

- The limit of BDs/CCEs (per slot in the scheduling CC) for the scheduled CC is determined based on the numerology of the scheduling CC.

  - Change the definition of NcellsDL, NcellsDL,μto “the number of configured DL-CCs whose scheduling cell is with active DL BWP having SCS configuration ” as in Section 10.1 of 38.213

###### RAN1#96bis (April 2019)

Agreements:

- For case 1-1 scheduling (PDCCH in the beginning of the slot), when a lower SCS PDCCH schedules a higher SCS PDSCH:

  - The  is determined a number of symbols based on PDCCH SCS counting from the end of the last symbol of the received PDCCH symbol to the beginning of the first symbol of the corresponding received PDSCH, quantized (using the granularity of PDSCH slot duration) to the next PDSCH slot boundary

Agreements:

Regarding PDSCH-to-HARQ-ACK minimum allowed timing under cross-carrier scheduling with different numerologies:

- Use the Rel-15 specification without changes

  - Agreements:

Regarding PDCCH-to-PUSCH minimum allowed timing under cross-carrier scheduling with different numerologies:

- Use the Rel-15 specification without changes

  - Conclusion:

- The PDCCH monitoring occasion determination is based on the numerology of scheduling cell in cross-carrier scheduling with different numerologies

  - No spec change is intended

  - For the case of cross-carrier scheduling from lower SCS to higher SCS for PDSCH, companies are encouraged to provide sufficient details for the design alternative(s) for scheduling  – to finalize in RAN1#97.

  - Email discussion for the remaining details for CCS till next meeting – Karri (Nokia)

###### RAN1#97 (May 2019)

Agreements:

- Delta-values for lower SCS PDCCH to higher SCS PDSCH case 1-1 scheduling

  - 15 kHz: 4 symbols

  - 30 kHz: 4 symbols

  - 60 kHz: 8 symbols

- Case 1-2: use the same delta as the case 1-1 scheduling

  - With the quantization step

- Case 2:

  - Use the same delta values as the case 1-1 scheduling INCLUDING the quantization step

Agreements:

- For the case of higher SCS PDCCH scheduling lower SCS PDSCH, for the scheduling timing, use the same definition as with lower SCS PDCCH scheduling higher SCS PDSCH scheduling, without quantization to the next PDSCH slot

Agreements:

- For high-SCS to low-SCS scheduling, the delta for 120 kHz SCS PDCCH: [12] symbols

Agreements:

- For the case of a lower SCS PDCCH scheduling a higher SCS PDSCH (or PUSCH), support using M DCIs in one slot for PDCCH to schedule M respective PDSCH (or PUSCH) transmissions in N slots for PDSCH (or PUSCH), where M<=N (for slot aggregation, M<N) following at least one of the following rules (potential down-selection can be done during Rel-16 UE feature discussion):

  - Alt 1a: Define max number of unicast DCIs that the UE is expected to decode in each span of PDCCH symbols

  - Alt 1b: Define an increased number of valid unicast DCIs per PDCCH monitoring occasions

  - Alt 1c: based on Rel-15 FG 3-5/3-5b

    - Further discussion whether some additional clarification is needed or not  potential clarification can be discussed under UE features

Agreements:

- timeDurationForQCL threshold is determined based on the numerology of the scheduled cells.

Agreements:

When PDSCH and its scheduling PDCCH are in the different CCs, if the PDCCH-to-PDSCH delay < timeDurationForQCL or if the TCI information is absent from the DCI, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell

Conclusion:

If the number of valid unicast DCIs at the same monitoring occasions is increased in the UE feature discussion, further discuss whether/how to update the HARQ-ACK codebook and PUCCH resource determination.

Agreements:

- In Rel-16, support enabling HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group.

  - Note: vs. per cell group in Rel-15

Agreed an LS in R1-1907953 on the RRC parameters for cross-carrier scheduling with different numerologies

###### RAN1-98 (August 2019)

  - No agreements

###### RAN1-98bis (October 2019)

Agreements:

At least the following two UE capabilities will be introduced

- Scheduling cell of lower SCS and scheduled cell of higher SCS

- Scheduling cell of higher SCS and scheduled cell of lower SCS

###### RAN1-99 (November 2019)

Agreements:

- The same additional beam switching timing as agreed for A-CSI under 7.2.13.4 is used for PDSCH being cross-carrier scheduled with different numerologies

Agreements:

For the time offset between PDCCH and PDSCH under cross-carrier scheduling with different numerologies:

- The timing offsets 4,4,8,[12] for SCS 15, 30, 60, 120 kHz respectively are relaxed to 4,5,10,14

- The above-mentioned numbers assume that the minimum PDCCH-to-PDSCH timing in cross-carrier scheduling with different subcarrier spacings that from RAN1 spec perspective they are assumed to have zero time offset

  - Consider if there is a need to clarify the Rel-16 feature introduction CR on this aspect

  - Agreements:

For QCL other than TypeD:

- Clarify that the following rule in Rel-15 is also applicable to cross-carrier scheduling with different numerologies:

  - If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

Conclusion:

- Do not support cross-carrier PDCCH order in Rel-16

##### Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

This sub-feature was introduced to the WI in September RAN#85

###### RAN1-98bis (October 2019)

Agreements:

- Support cross-carrier A-CSI-RS triggering when µPDCCH < µCSI-RS

- Support cross-carrier A-CSI RS triggering when µPDCCH > µCSI-RS

- For the above two cases, separate UE capabilities are introduced, and the capabilities are NOT linked with the capabilities related to the cross-carrier scheduling with different numerologies

  - Agreements:

- To support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Definition of the slot index where A-CSI RS is transmitted

- n12CSI-RS/2PDCCH +Xn12μCSI-RS2μPDCCH+X, where the n1 is the PDCCH carrier slot with the DCI

- Note: The case when the frame boundaries of the two carriers are not aligned may require additional compensation when determining the actual slot number

New slot offset values

- Extend the X(≥0) values for cross-carrier A-CSI RS triggering when the PDCCH and A-CSI RS have different SCS

  - FFS X

Minimum A-CSI RS triggering offset for cross-carrier triggering of A-CSI RS when the PDCCH SCS and the A-CSI RS SCS are not the same

- Minimum delay from the end of the triggering PDCCH and the start of the CSI-RS in the CSI-RS carrier’s slots is defined as m

  - m = 4, 4, 8, [12] symbols for PDCCH SCS = 15, 30, 60, 120 kHz, respectively as defined for cross-carrier scheduling of PDSCH with different PDCCH and PDSCH SCS.

  - [Working assumption] When µPDCCH < µCSI-RS the minimum delay is quantized to the beginning of the next A-CSI RS carrier slot

- FFS impact, if any, due to beam switching timing

###### RAN1-99 (November 2019)

Agreements:

When µPDCCH < µCSI-RS, X{0, 1, …, 31}

Agreements:

For the minimum triggerind delay with 120 kHz SCS, revert the WA and adopt 14 symbols for 120 kHz SCS

In addition, m = 4, 4, 8 are updated to 4, 5, 10

Agreements:

To confirm the WA on quantization, when µPDCCH < µCSI-RS the minimum delay is quantized to the start of the next slot

Agreements:

For cross-carrier triggering of A-CSI RS when the PDCCH SCS and the A-CSI RS SCS are not the same,

  - The same definition of the slot index where CSI-RS is transmitted is assumed for CSI-IM transmission;

  - The same minimum triggering offset as for CSI RS is defined for CSI-IM.

Agreements:

For the case when µPDCCH < µCSI-RS:

- The additional delay d for the beam switching timing delay X+d is the same for A-CSI-RS triggering with different SCS and for cross-carrier scheduling with different SCS

  - FFS: potential effects of the UE power saving WIs cross-slot scheduling may need to be considered

- The additional delay is equal to d

  - d=8, 8, 14 PDCCH symbols for 15, 30, 60 kHz SCS respectively

Agreements

For cross carrier aperiodic CSI-RS with triggering time offset smaller the threshold, with A-CSI RS contains QCL-typeD information:

- If there is any other DL signal in the same symbols as the CSI-RS in the scheduled cell, the UE applies the QCL assumption of the other DL signal on the aperiodic CSI-RS.

  - The definition of “other DL signal” is same as Rel-15

- Otherwise,

  - if CORESET is configured in the A CSI-RS carrier, the UE follows existing spec text. I.e., A CSI-RS follows QCL assumption of the lowest ID CORESET in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

  - If CORESET is not configured in the A-CSI-RS carrier, apply the QCL assumption in the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the serving cell with the A-CSI-RS transmission

##### Support of unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA

This sub-feature was introduced to the WI in September RAN#85

###### RAN1-98bis (October 2019)

Agreements:

- Support explicit RRC signalling of slot offset to the UE in unaligned frame boundary with slot alignment and partial SFN alignment inter-band CA.

  - Where the slot offset for a CC is defined w.r.t. the Pcell/pScell timing, with slot granuality defined as (to down-select)

    - Alt 1: the maximum of Pcell/pScell lowest SCS among all the configured DL/UL BWPs and the CC’s lowest SCS among all the configured DL/UL BWPs

    - Alt 2:

      - If the CC is FR1, 15kHz; If the CC is FR2, 60kHz

    - Alt 3:

      - If the CC is FR1, 60kHz; If the CC is FR2, 120kHz

    - Alt4: 120kHz

    - Others?

- FFS: RAN1 Spec if any impact

- Note: Offset is always signalled if the offset is not zero for the UE indicating this capability

- Observation: One slot right-shift and one slot left-shift corresponds to different samples in the current spec description

Note: it is confirmed that the offset Range is to be limited to ±76800Ts as in the WID

Proposals:

In the above agreements, Alt 1 is adopted with the following details:

If SCS of PCell is smaller than [or equal to] SCS of SCell

q = -1

Otherwise

q = 1

      - For right shift and q = 1 or for left shift and q = -1: in the range of 2.5ms, offset is indicated e.g., by M, N and n,

- Offset equals to M*(16κ+NL) + nL (samples), where L is the length of the shorter slot of two type of slots of different length, M is multiple of 0.5ms, N is the number of the slots of each 0.5ms, and n is the number of slots in the fractional 0.5ms

- n is in the range of [0, N-1]

      - For left shift and q = 1 or for right shift and q = -1: in the range of 2.5ms, offset is indicated by M, N and n

- Offset equals to M*(16κ+NL) if n=0, otherwise equals to M*(16κ+NL) + (16κ +nL) (samples), where L is the length of the shorter slot of two type of slots of different length, M is multiple of 0.5ms, N is the number of the slots of each 0.5ms, and n is the number of slots in the fractional 0.5ms

- n is in the range of [0, N-1]

FFS： whether SSB SCS should also be taken into account

  - Email discussion/approval till 10/25 (including a potential RAN2 LS) – Xiaodong (CMCC)

###### RAN1-99 (November 2019)

Agreements:

Update and confirm the working assumption that Alt 1 modified as below

Alt 1: the granularity of the offset is determined by the maximum of Pcell/pScell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList and the CC’s lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList

Update and confirm the working assumption as below

  - For slot offset N, for CA case, the beginning of slot #0 of the CC with lower SCS (or PCell/PScell for equal SCS) coincides with the beginning of slot #(qN mod M) of the CC with higher SCS (or SCell for equal SCS)

    - Where

      - q = -1, if lowest SCS of PCell/PScell is smaller than or equal to lowest SCS of SCell

      - q = 1, otherwise

  - M is the number of slots per frame in the CC with higher SCS

  - Note: Other simple description is not precluded as long as it is aligned with above principle.

  - Here the lowest is same as the definition in Alt 1.

- Note: For the combinations of 60kHz/60kHz and 120kHz/120kHz for Pcell/Scell with a UE-specific Pcell configuration, the above agreements may not be fully desirable. Discuss further offline whether or not to further address these cases (without any additional RRC impact). If no additional consensus reached by Friday, the above agreements stay as is

Agreements:

At most single non-zero offset duration (independent on SCS) can be configured among CCs in the unaligned CA configuration.

Agreements:

For timing determination for cross-carrier scheduling of PDSCH in 5.1.2.1 and of PUSCH in 6.1.2.1 of TS38.214

- Update the scheduled slot determination equations with slot offset NSCS,PxSCH, and the slot offset is only applicable for cross-carrier scheduling

Note: Error case when NSCS,PDSCH + K0< minimum value of UE reported K0 capability.

Error case when NSCS,PUSCH + K2< minimum value of UE reported K2 capability.

Agreements:

Adding the slot offset for cross-carrier triggering aperiodic CSI report.

Agreements:

Adding the slot offset for cross-carrier triggering aperiodic SRS.

Agreements:

For the lowest SCS combinations of other than 60kHz/60kHz and 120kHz/120kHz

- For slot offset N, for CA case, the beginning of slot #0 of the CC with lower SCS (or PCell/PScell for equal SCS) coincides with the beginning of slot #(qN mod M) of the CC with higher SCS (or SCell for equal SCS)

    - Where

      - q = -1, if lowest SCS of PCell/PScell is smaller than or equal to lowest SCS of SCell

      - q = 1, otherwise

  - M is the number of slots per frame in the CC with higher SCS

  - Note: Other simple description is not precluded as long as it is aligned with above principle.

  - Here the lowest is same as the definition in Alt 1.

For the lowest SCS combinations of 60kHz/60kHz and 120kHz/120kHz

- For slot offset N, the beginning of slot #0 of the CC with lower subcarrier#0 of CRB#0 coincides with the beginning of slot #(qN mod M) of the CC with higher subcarrier#0 of CRB#0

    - Where

      - q = -1, if subcarrier#0 of CRB#0 of PCell/PScell is lower than subcarrier#0 of CRB#0 of SCell

      - q = 1, otherwise

  - M is the number of slots per frame in the CC

  - Here the lowest is same as the definition in Alt 1.

##### Other topics

###### RAN1-98 (August 2019)

Conclusion:

- There is no consensus in RAN1 on how to clarify what synchronous in synchronous NR-DC means in Rel-15, although it is commonly understood in RAN1 that synchronous means at least slot-level synchronization

#### 2.1.2	Remaining Open issues

- UE capabilities

## 2.2	RAN2

#### 2.2.1	Agreements

##### RAN2#105 (Feb/Mar 2019):

- Early Measurement reporting:

Agreements:

For IDLE/INACTIVE:

- Rel-16 early measurement configuration may contain both NR and LTE configuration, only NR configuration or only LTE configuration, to support various MR-DC and CA scenario. FFS on details.  IDLE mode and INACTIVE mode details will be discussed separately

- NR early measurement configuration should include NR specific measurement parameters configurations.

- Available beam and cell level measurement results can be included in early measurement reporting if configured.

- Two email discussions to be held to progress the work on early measurement reporting:

- [105#53][NR/eCA-DC] – Signaling  (Ericsson)

    - Discuss signalling for measurement reporting and identify options for:

    - When availability of measurements is indicated

    - When measurement results are provided

    - Deadline: Thursday 28/03/2019

- [105#54][NR/eCA-DC] – Measurement configuration (Qualcomm)

- Details of measurement configurations and measurement reporting

- Deadline: Thursday 28/03/2019

Agreements:

- The configured SCells (MCG and SCG) can be configured in deactivated or activated state by RRC upon addition or after a handover.  Timing requirements are up to RAN4.  FFS if this applies to resume.

- SCG Configuration in RRCResume message can be considered

- Sent an LS to RAN4 on RAN2 agreements and ask RAN4 to take the agreements into consideration and let us know if there are any concerns. LS endorsed in R2-1902734

- Fast MCG link Recovery:

Agreements:

- MCG failure can be indicated to the network via the SCG. FFS if via SCells.

- FFS how the failure is indicated, which SRBs, and which failure case the fast MCG failure recovery.

- We will aim to have a unified solution for the failure cases that we want to address.

- An email discussion to be held to progress the work on fast MCG recovery:

- [105#55][NR/eCA-DC] – MCG failure (Vivo)

  - Summarize the different options for 1) Failure indication message, 2) SRBs to use and 3) which failure cases to address

  - Deadline: Thursday 28/03/2019

##### RAN2#105bis (Apr 2019):

- Running CRs:

  - The following running CRs were endorsed:

    - 37.340: R2-1905430

    - 38.300: R2-1904474

- Early Measurement reporting:

Agreements:

- For NR IDLE mode, the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is setup will be supported.

- For both LTE and NR, sending full idle mode measurements before security activation shall not be allowed. FFS if some measurement information (detail TBD) related to idle mode measurements can be sent before security activation.

- SMC and SMC complete messages will not be modified to enable the signalling of early measurements.

- For both LTE and NR, RAN2 confirm that current specification allow that UEInformationRequest (or equivalent message to be specified in NR) can be sent by the network immediate after Security Mode Command without network having to wait for  Security Mode Complete (i.e. similar to sending of Reconfiguration after SMC)

- For NR INACTIVE mode, the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is resumed will be supported.

- Sending early measurement report is network controlled.

- For NR INACTIVE, the network can request early measurement report in RRCResume.

- For NR INACTIVE, early measurement reporting can be sent in RRCResumeComplete.

FFS Whether above two bullets should be applied to LTE RRCConnectionResume and RRCConnectionResumeComplete message.

- NR early measurements can be configured in both NR RRCRelease message and NR system information. FFS: Whether there are differences in the configuration that can be provided by RRCRelease and SI.

- Introduce some indication about the cell's early measurement support in NR system information.

- To control the duration of UE performing both IDLE and INACTIVE measurements, a single validity timer (similar to measIdleDuration in LTE euCA) is mandatory indicated only in NR RRCRelease message, i.e. not included in NR SIB.

- For both IDLE and INACTIVE early measurements, the following IEs can be optionally configured per NR frequency in both NR RRCRelease message and NR SIB:

  - A list of frequencies and optionally cells (similar to measCellList in LTE euCA) the UE is required to perform early measurements.

  - A cell quality threshold (similar to qualityThreshold in LTE euCA) the UE is required to report the measurement results only for the cells which met the configured thresholds.

FFS: A validity Area (similar to validityArea in LTE euCA) to indicate the list of cells within which UE is required to perform early measurements. If the UE reselects to a cell outside this list, the early measurements are no longer required (same as timer expiry).

If it is absent, the UE will not have area limitation of early measurements.

- For SSB based measurements:

  - For both IDLE and INACTIVE early measurements, SSB frequencies to be measured can be located out of sync raster

  - For both IDLE and INACTIVE early measurements, RSRP and RSRQ can be configured as cell and beam measurement quantity.

  - For both IDLE and INACTIVE early measurements, the configuration parameters provided per SSB frequency follow the same principles as those provided in SIB2/4 for the purposes of Idle/Inactive mobility. (Details differences can be discussed at stage 3 level)

  - As LTE euCA, cell / beam SINR is not introduced as measurement quantity in NR early measurement configuration in Rel-16.

- For SSB based beam level measurement configurations:

  - The UE is required to report the beam with the highest measurement quantity

FFS: Whether additional beams can be reported.

  - For both IDLE and INACTIVE early measurements, the UE can be configured with one of the 3 beam reporting types

    - No beam reporting;

    - Only beam identifier

    - Both beam identifier and quantity

FFS: Whether to support CSI-RS based NR early measurements

- LTE UE in IDLE mode, IDLE with suspended, and INACTIVE can be configured with NR early measurements to support fast setup of (NG)EN-DC (i.e. euCA is extended to support NR measurements). Details are FFS

- Fast MCG link Recovery:

Agreements:

- MCG fast recovery targets all MRDC architecture options

- When MCG failure occurs, UE follows SCG failure-like procedure:

  - UE does not trigger RRC connection re-establishment.

  - UE triggers an MCG failure procedure in which a failure information message is transmitted to the network via SCG.

- MCG fast recovery targets the following use cases MCG leg RLF

FFS: Other uses cases. Can consider in future whether the mechanism can be also be applied in the case of other MCG failures.

- MCG fast recovery can only be triggered after AS security has been activated and the SRB2 and at least one DRB have been setup

- MCG failure indication should include:

  - Available measurement results of MCG

  - MCG link failure cause

  - Available measurement results of SCG

  - Available measurement results of non-serving cells

- For MCG failure indication, new RRC message in introduced, e.g. MCGFailureInformation.

- SCG leg of the split SRB1 can be used for MCG fast recovery.

FFS: If configured, SRB3 can be used for MCG fast recovery. Priority is to complete the solution based on split SRB1

- New SRB is not introduced for MCG fast recovery.

##### RAN2#106 (May 2019):

- Running CRs:

The following running CRs were endorsed:

  - 37.340: R2-1905433

  - 36.300: R2-1905982

  - 38.300: R2-1908394

  - 36.331: R2-1905985

  - 38.331: R2-1905984

- Early Measurement reporting:

Agreements:

- RAN2 confirms that for both LTE and NR, sending cell RSRP/RSRQ of idle mode measurements before security activation shall not be allowed.

- RAN2 confirms that for both LTE and NR, sending cell PCI(s) with good quality and associated frequency of idle mode measurements before security activation shall not be allowed.

- How the UE applies filtering of beam measurements as part of early measurement reporting is left to UE implementation (Up to RAN4 to specify performance requirements for early measurement reporting)

- The UE can report more than one beam measurement. Network can configure whether it wants to receive more than just the best beam

FFS whether the network can configure max number of beams and a threshold above which beams are reported

- The early measurement configuration can be different between that in RRCRelease and in SIB. If the UE receives the early measurement configuration from RRCRelease, this overrides the early measurement configuration provided in SIB (if any).

FFS: Whether some other measurement related configuration in SI (e.g. smtc) outside of the early measurement configuration can still be used.

- A single early measurement configuration is provided in SI for idle and inactive

FFS: Whether the early measurement configuration can be kept when the UE receives the Release (to Inactive to Idle) in response to Resume Request.

- L3 filtering is not applied to early measurement reporting

- The UE performs the idle measurement for the frequencies in configured frequency list only when the UE support CA or MR-DC between the frequency and the serving frequency.

- FFS Whether the network can provide information on support of CA/DC between frequencies to assist the UE to determine which frequencies to provide measurement for.

- If UE reselects to a cell that does not support early measurements (as indicated by absence of an indicator in SI), the validity timer keeps running, but the UE is not required to performs measurements while camped on that cell (same as LTE euCA)

- Three email discussions to be held to progress the work on early measurement reporting:

- [106#35][NR/DCCA] Validity area (Vivo)

    - Whether it is needed or not

    - What is the content of the validity area

    - If configured, how does it affect UE idle measurement performing behaviour

      - What happens if the UE reselects to a cell that is not part of the validity area (for any of the configured frequencies/cells) while measIdleDuration is running

      - What happens if the UE reselects back to a cell that is part of the validity area while measIdleDuration is running

    - Intended outcome: Report to next meeting

    - Deadline:  Thursday 2019-08-08

- [106#36][NR/DCCA] Measurement and reporting configuration (Qualcomm)

    - What SIB(s) are used for idle mode meas configuration

    - If CSI-RS configuration is needed or not

    - Whether the network can configure max number of beams and a threshold above which beams are reported

    - Whether some other measurement related configuration in SI (e.g. smtc) outside of the early measurement configuration can still be used.

    - Whether the network can provide information on support of CA/DC between frequencies to assist the UE to determine which frequencies to provide measurement for.

    - How to prevent outdated measurement reporting

    - Any other scenarios where the UE removes/releases idle measurement configurations and/or results

    - Intended outcome: Report to next meeting

    - Deadline:  Thursday 2019-08-08

- [106#37][NR/DCCA] UE behaviour regarding idle measurement configurations and measurement results (Ericsson)

    - during state transitions, inter-RAT cell re-selection, etc, while measIdleDuration is running

    - 2 step resume/release

    - Inter-RAT cell re-selection

    - RRC rejection

    - autonomous transition to IDLE mode (e.g. reception of CN paging while in INACTIVE)

    - successful transition to CONNECTED mode

    - transition to IDLE/INACTIVE mode

    - when measIdleDuration expires or stops

    - clarifications regarding when the UE starts/stops/suspends/resumes idle measurements

    - Intended outcome: Report to next meeting

    - Deadline:  Thursday 2019-08-08

- Efficient and low latency configuration signaling:

Agreements:

- SCell dormant state like LTE euCA will not be introduced in NR.

- ‘dormancy’ behaviour will be studied as a solution for fast return to SCell utilisation for data transfer. The 'dormancy' behaviour implies that the UE stops monitoring PDCCH but continues other activities such as CSI measurements, AGC and beam management. RAN1/4 input required on feasibility and benefit.

- Temporary RS resources at SCell activation will be studied as a solution for fast SCell activation. RAN1/4 input required on feasibility and benefit.

- Sent an LS to RAN1/4 to request input for the dormancy behaviour and the denser RS approach (R2-1908483)

- An email discussion to be held to progress the work on SCG and MCG SCell configuration:

  - [106#38][NR/DCCA] SCG and MCG SCell Configuration with RRC Resume (Ericsson)

    - Progress the discussion of maintaining SCG and MCG SCell Configuration in RRC Resume and providing an SCG and MCG SCell configuration in RRC Resume for both LTE and NR. The discussion should aim to understand potential benefits.

    - Intended outcome: Report to next meeting

    - Deadline:  Thursday 2019-08-08

- Fast MCG link Recovery:

Agreements:

- Fast MCG recovery is not supported in case (intra and inter-RAT) handover failure

- Fast MCG recovery is not supported in case of integrity check failure

- Fast MCG recovery is not supported in case of RRC connection reconfiguration failure

- FFS Whether a guard timer is needed for the MCG failure indication message

- Once the MCG failure indication is triggered, the UE shall:

  - transmit the MCG failure indication;

  - suspend MCG transmission for all SRBs and DRBs;

  - reset MCG-MAC;

  - maintain the current measurement configurations from both the MN and the SN, and continue measurements based on configuration from the MN and the SN if possible.

FFS whether switch the primaryPath to SCG is needed

- If SCG failure is detected while MCG is suspended then initiate RRC re-establishment procedure

- Upon receiving the MCG failure indication, the MN sends reconfiguration with sync or RRC Release to the UE via SRB1.

- Upon reception of reconfig with sync the UE resumes MCG transmission if suspended

##### RAN2#107 (August 2019):

- Running CRs:

The following running CRs were endorsed:

  - 37.340: R2-1909192

  - 36.300: R2-1910240

  - 38.300: R2-1910241

  - 36.331: R2-1911764

  - 38.331: R2-1911763

Email discussions to be held to update the running CRs:

- [107#14][NR/DCCA] Stage 2 running CR (Ericsson/Vivo)

  - Intended outcome: Endorsed running CR

  - Deadline:  Thursday 2019-09-12

- [107#33][NR/DCCA] Stage 38.331 running CR (Ericsson)

  - Intended outcome: Running CR submitted to next meeting

  - Deadline:  Thursday 2019-10-03

- [107#34][NR/DCCA] Stage 36.331 running CR (Ericsson)

  - Intended outcome: Running CR submitted to next meeting

  - Deadline:  Thursday 2019-10-03

- Early Measurement reporting:

Agreements:

- For per-frequency SSB measurement configuration reuse the IE structure that is currently used in SIBs for cell reselection purposes.

- The legacy SSB measurement configurations in NR SIB2/4 and LTE SIB24 are reused for NR early measurements performed in frequencies which are candidates of cell selection/reselection, i.e. not introduce new measurement configurations in NR/LTE SIB for these SSBs.

- Same as LTE euCA, NR frequency list (not the SSB measurement configuration) can be different between RRC release and SIB. The frequency list, if provided, in RRC release message overrides the one provided in SIB.

- For per frequency SSB measurement configuration for purpose of only early measurements, it can be included in both RRC release message and SIB. If provided in RRC release message, it overrides the one provided in SIB in the cell where the RRC Release message is received.

FFS How UE manages the situation when an SSB measurement configuration for a given frequency is provided in SIB of the current cell and was also provided RRC Release (in an earlier cell).

- As in LTE euCA, the indication whether to report RSRP, RSRQ or both can be indicated in both RRC release message and SIB. If provided in RRC release, it overrides the one in SIB.

- Similar to LTE euCA, the indication of beam reporting type (i.e. whether to, not report beam results, report only the beam index, or report both beam index and results) can be indicated in both RRC release message and SIB. If provided in RRC release, it overrides the one in SIB.

- NR early measurement configuration is included in a new NR SIB.

- NR early measurement configuration is included in LTE SIB5 (i.e. the SIB including LTE early measurement configurations)

- It is not necessary to specify CSI-RS based early measurements for the case of SCell with SSB in Rel-16.

- It is not necessary to specify CSI-RS based early measurements for the case of SCell without SSB in Rel-16.

- In NR early measurement configuration, the UE can be configured with maximum number for beam reporting and only beams above configured threshold for cell quality derivation are required to be reported (as NR CONNECTED measurements).

- Do not support the network provide information on network’s support of CA/DC between frequencies to assist the UE to determine which frequencies to provide NR early measurement in Rel-16.

- Do not support a mechanism to prevent outdated early measurement reporting in Rel-16.

- Upon the reception of the RRCSetup message in response to RRCSetupRequest or RRCResumeRequest (while T331 is running), the UE stops T331, and deletes the dedicated idle mode measurement configuration, if any.

- Upon the reception of the RRCReject message in response to RRCSetupRequest or RRCResumeRequest (while T331 is running), the UE keeps performing the idle mode measurements.

- During a 2-step resume (i.e. RRCRelease in response to RRCResumeRequest), the network can release or reconfigure the idle mode measurements.

FFS whether this is delta or complete replace

- Upon the expiry of T331 while in IDLE or INACTIVE mode, the UE deletes the dedicated idle mode measurement configuration, if any.

- The UE deletes the early measurement results after it has successfully reported them to the network (i.e. in UEInformationResponse or RRCResumeComplete).

- An email discussion to be held to progress the remaining work on early measurements:

  - [107#35][NR] Remaining aspects of early measurement configuration (Vivo)

-To at least address the FFS points and the validity area

-Intended outcome: Report to next meeting

-Deadline:  Thursday 2019-10-03

- Efficient and low latency configuration signaling:

Agreements:

- The LTE RRCConnectionResume message (Inactive to Connected) can contain the MCG SCell configuration and the associated UE behaviour in handling the SCell configuration is the same as in the Rel-15 RRC connection reconfiguration procedure.

- In NR and LTE Rel-16, the UE maintains the MCG SCell configuration upon the initiation of the resume procedure.

- The RRC(Connection)Resume message contains an indication to restore/resume the MCG SCells (noting that behaviour in legacy eNBs that don't support this feature needs to be considered).

- The (LTE and NR) RRC(Connection)Resume (Inactive to Connected) message can contain the SCG configuration and the associated UE behaviour in handling the SCG configuration is the same as in the Rel-15 RRC (connection) reconfiguration procedure.

- In NR and LTE Rel-16, the UE maintains the SCG configuration upon the initiation of the resume procedure.

- The RRC(Connection)Resume message contains an indication to restore/resume the SCG (noting that behaviour in legacy e/gNBs that don't support this feature needs to be considered).

- An email discussion to be held to progress the work on SCell/SCG Resume:

  - [107#32][NR/DCCA] MCG SCell/SCG resume (Interdigital)

•	Possible enhancements to blind resume of MCG SCell and/or SCG (e.g. conditional restore of stored configuration on resume)

•	SCell activation at resume

•	Resume at SN

•	Suspended SCG (while in Connected mode)

-	The rapporteur can also add other items (keeping the total number limited)

Intended outcome: Report to next meeting

Deadline:  Thursday 2019-10-03

- Fast MCG recovery:

Agreements:

- Upon sending a MCG failure indication, UE starts a timer.

- Upon resumption of MCG, UE stops the timer.

- Upon expiry of the timer, UE initiates RRC connection re-establishment procedure.

- Network can configure the timer value (no infinite value)

- If a UE is configured with split SRB1 with PDCP duplication, there is no need to switch the primaryPath upon detection of MCG failure since MCG failure indication will be transmitted via SCG RLC bearer of split SRB1.

- If PDCP duplication is not activated, upon detection of MCG failure the primaryPath for split SRB1 is implicitly reconfigured to the SCG. The UE expects the network to explicitly reconfigure the primaryPath back to MCG in the MCG recovery or in a Re-establishment

- SRB3, if configured, can be used for MCG fast recovery.

- For MCG fast recovery via SRB3, MCG Failure Information message in UL (same message as for SRB1 case) is encapsulated by the UE into an SN RRC message.

- For MCG fast recovery via SRB3, the MN response message in DL (either a reconfiguration with sync or release message) is encapsulated by the SN in an SN RRC message.

FFS Transmission of the complete message

- An email discussion to be held to progress the work on MCG Fast recovery:

  - [107#31][NR/DCCA] MCG fast recovery (Ericsson)

•	Other recovery mechanisms:

o	(PCell) Recovery via SCell

o	Other means?

•	Issue of outstanding SRB1 PDCP packets upon MCG fast recovery

•	Any other issues related to NW control, configurations, etc.

-	The rapporteur can also add other items (keeping the total number limited)

Intended outcome: Report to next meeting

Deadline:  Thursday 2019-10-03

##### RAN2#107bis (October 2019):

- Running CRs:

  - 37.340: R2-1912268

  - 36.300: R2-1912540

  - 38.300: R2-1912541

  - 36.331: R2-1914189

  - 38.331: R2-1914188

- Early Measurement reporting:

Agreements:

- There is a validity area, and the action when the UE exits the validity area is that the UE stops all early measurements.

- Validity area is configured by means of dedicated RRC signaling

- Validity area can be configured by means of: Lists of PCIs; Lists of CellIdentity;

- When UE reselects to a cell that is not part of the validity area (for any of the configured frequencies/cells) while measIdleDuration is running, UE should stop measurement. UE stops the timer. WA that the UE also clears the entire early measurement configuration.

- If network uses broadcast signaling for the list of early measurements, it will provide all parameters by broadcast signaling with the only exception that dedicated signalling is used for the timer

- If network uses dedicated signaling for the list of early measurements, the following signalling options are allowed for each of the frequencies:

  - SSB measurement configuration (incl SMTC) and all other parameters are provided by dedicated signaling

  - SSB measurement configuration (incl SMTC) is broadcast and all other parameters are provided by dedicated signaling

- No UE requirements will be specified for what UE shall do upon reselection to a cell broadcasting for some frequency an SSB measurement configuration that differs from the values received in the RRC release message i.e. UE may stop early performing measurements for concerned frequency

- Efficient and low latency configuration/signaling:

Agreements:

- Based on RAN1/RAN4 reply LS, introduce ‘dormancy’ behaviour for NR SCell, i.e. the UE stops monitoring PDCCH on SCell but continue performing CSI measurements, AGC and beam management, if configured.

- RAN2 confirms that UE “dormancy” operation is part of SCell activated state (i.e. not as part of SCell deactivated state)

- Direct SCell activation (setting the SCell state to activated or deactivated) in resume message is supported, if R4 can confirm that there are no blocking issues from their point of view

- When the UE resumes to a cell included in the stored SCG, particular functionality for swapping of MCG and SCG configurations is not considered for Rel16

- New conditions/triggers for resuming directly to the SN are not considered in Rel16.

- R2 assumes the following (can be slightly modified due to progress on Scell dormancy):

  - The UE supports network-controlled suspension of the SCG in RRC_CONNECTED.

  - UE behaviour for a suspended SCG is FFS

  - The UE supports at most one SCG configuration, suspended or not suspended, in Rel16.

  - In RRC_CONNECTED upon addition of the SCG, the SCG can be either suspended or not suspended by configuration.

- LS sent to RAN4 regarding SCell state on resume is approved in R2-1914217

- Common Cell Configuration for Signalling Reduction will not be considered in Rel-16 in WI CADC enh.

- Agreed to have 2 email discussions to progress the work on efficient and low latency configuration/signalling:

  - [108#55][NR/DCCA] MCG SCell and SCG Configuration with RRC Resume (ZTE)

    - Scope: Identify and progress if possible any remaining issues (Main OI: How to handle the case if Resumed cells/scg or cells/scg to resume is in fact not present, smaller OI: gNB indicate what to resume/release to the UE).

    - Intended outcome: Report for the next meeting

    - Deadline:  2020-01-30

  - [108#56][NR/DCCA] SCell dormancy open issues(OPPO)

    - Intended outcome: Report for next meeting, paving the way for fruitful discussions.

    - Deadline:  2020-01-30

- Fast MCG recovery:

Agreements:

- Fast PCell recovery via SCell is not introduced in Rel-16.

- We add no functionality for optimized RRC re-establishment to SN RAT in Rel-16

- No further mechanisms are introduced to resolve outstanding UL/DL RRC deadlock messages situation upon the triggering of MCG failure recovery

- For MCG fast recovery via SRB3, the MCGFailureInformation message in UL is encapsulated in the ULInformationTransferMRDC message

- A new RRC message, i.e., DLInformationTransferMRDC, is introduced in order to allow the SN to encapsulate (for SRB3) the MN response (i.e., RRCReconfiguration or RRCRelease message) to be send to the UE

- The RRC procedure on these encapsulated messages are the same as if they had been received by SRB1

- When receiving a MN RRCRelease message encapsulated within an SN RRC message via SRB3, the UE does not send any complete message

- Split SRB1 is always used for the transmission of the MCGFailureInformation message. SRB3 is used only if split SRB1 is not configured

- MCG failure recovery can be configured by the network.

##### RAN2#108 (Nov 2019):

- Running CRs:

  - 37.340: R2-1914936

  - 36.300: R2-1915279

  - 38.300: R2-1915280

  - 36.331: R2-1915281

  - 38.331: R2-1915282

- Email discussions to be held to progress the stage 2/3 running CRs:

  - [108#13][DCCA] Stage 2 running CRs 36.300, 38.300 (Ericsson)

Scope: Final check on the stage 2 CRs

Intended outcome: Endorsed CRs

Deadline:  2019-12-13

  - [108#14][DCCA] Stage 2 running CRs 37.340 (vivo)

Scope: Final check on the stage 2 CR

Intended outcome: Endorsed CR

Deadline:  2019-12-13

  - [108#33][DCCA] RRC running CRs 36.331, 38.331 (Ericsson)

Scope: agreeable Draft CRs for next meeting, capturing agreements

Intended outcome: Agreeable Draft CR

Deadline:  2020-01-23

- NR DC:

Agreements:

- Agreed to have an email discussion to progress this work:

  - [108#15][DCCA] Power control for NR DC (Vivo)

Intended outcome: Endorsed TP, input to the running CR

Deadline: 2012-12-13

- Early Measurement reporting:

Agreements:

- Upon entering RRC CONNECTED mode, the UE stops validity timer T331 (if running) and deletes the dedicated idle mode measurement configuration (if configured).

- After moving to another RAT due to inter-RAT cell reselection, the UE stops validity timer T331 (if running) and deletes the dedicated idle mode measurement configuration (if configured)

- While transition from NR INACTIVE mode to NR IDLE mode, the UE keeps the validity timer T331 (if running) and the dedicated idle mode measurement configuration (if configured), i.e. just continue.

- While transition from LTE INACTIVE mode to LTE IDLE mode, the UE keeps the validity timer T331 (if running) and the dedicated idle mode measurement configuration (if configured), i.e. just continue.

- When UE reselects to a cell that is not part of the validity area, the UE stops the validity timer and also clears the entire early measurement configuration.

- For the early measurements during a 2-step resume:

  - if RRCConnectionRelease does not include idle/inactive measurement configuration, the UE keeps the configuration and T331 continues running (i.e. no action);

  - if RRCConnectionRelease includes idle/inactive measurement configuration, the new configuration completely replaces the old configuration (incl timer which would be started).

  - if RRCConnectionRelease includes an release indication, the UE releases the old configuration, stops timer.

- The validity area is defined as a carrier list (which could be different from the carriers to be measured during RRC_IDLE/INACTIVE) with optional PCI list per carrier.

- The early measurement results are sorted by RSRP unless only RSRQ is configured as reporting quantity.

- The validity area cannot include IRAT cells

- If, for a frequency for which SSB config was provided by broadcast @ initial configuration, reselected cell does not broadcast SSB config the UE is not required to measure concerned frequency while camping on concerned cell (but should re-attempt following another re-selection)

- Measurement for Cell reselection (304) and early measurements are independent.

- In case UE cannot find suitable cell to camp or in anycell selection does not trigger stopping T331 or deleting early measurement configuration (no need to capture in the TS).

- The NR Rel-16 early measurement reporting solution is introduced in LTE

  - The network can request (in RRCConnectionResume) the UE to send early measurements

  - The UE can include early measurements in RRCConnectionResumeComplete.

- If a UE is released by an eNB which only configures bcast LTE early measurements and then reselects to an eNB which broadcasts both LTE and NR idle/inactive measurement configurations, the UE shall apply these NR configurations

- A new indication is introduced in SIB2 to indicate that the UE can perform NR early measurements while camped on the cell.

- At least one indication is introduced in RRCConnectionResume to indicate that the UE shall include the LTE and/or NR early measurements in RRCConnectionResumeComplete.

- Efficient and low latency configuration/signaling:

Agreements:

- We use BWP model as agreed/assumed in R1.

- Upon entering dormancy, the UE clears/suspends any uplink grants (type 1 and type2) associated with the SCell.

- In dormancy SCell, the UE doesn’t perform RACH.

- In dormancy SCell, aperiodic CSI/SRS via self-carrier scheduling is not allowed.

- As dormant state in LTE euCA, SCell dormancy is not applicable to the PUCCH SCell.

- WA: If in dormancy SCell, aperiodic CSI via cross-carrier scheduling is not allowed, FFS for SRS

- Send LS to R1 cc R4 informing of agreements, stating that this is not finished and e.g. SRS transmissions on the dormancy SCell is still FFS (no action)  [R2-1916587]

- SCG suspension will not be pursued in Rel-16

- Fast MCG recovery:

Agreements:

- The guard timer for fast MCG link recovery should be configured via dedicated signalling, it is configured by the MN.

- The configuration of guard timer implicitly indicates that the feature of fast MCG link recovery is enabled by the network, and that the UE shall initiate the procedure.

- Cross-Carrier scheduling with different numerologies:

Agreements:

- It was agreed to use the baseline CR R2-1916583 to capture stage 3 of CCS with different numerologies.

- Other aspects:

Agreements:

- Async CA/DC:

  - Under async CA, clarify that the UE uses SFN of primary cell (i.e. PCell or PSCell) within the same cell group for the calculation of HARQ Process ID in SPS/CG, i.e. no change of rel-15 legacy UE behaviour.

  - Under async CA, clarify that the UE uses SFN of primary cell (i.e. PCell or PSCell) within the same cell group for calculation of downlink/uplink assignment occurrences of SPS/CG, i.e. no change of rel-15 legacy UE behaviour.

  - Under async CA, clarify that the UE uses SFN of primary cell (i.e. PCell or PSCell) within the same cell group for DRX on-duration determination, i.e. no change of rel-15 legacy UE behaviour.

  - R2 assumes that SFN from PCell is used for SI reception, thus no impact to SI reception

  - R2 assumes that SFN from PCell is used for Rel-15 DRX, thus no impact to Rel-15 DRX.

  - R2 assumes that SFN from PCell is used for UP operation (CG, DRX etc), thus no impact to UP.

  - Agreed to have an email discussion to progress the work on the RRC signaling aspects

- [108#57][DCCA] async CA (QC)

    - Scope: RRC signalling design for slot offset, taking into account R1 Nov outcome.

    - Intended outcome: agreeable TP for RRC

    - Deadline:  2020-01-30

- UE capabilities:

  - Agreed to have an email discussion to progress the work on UE capabilities for the CA/DC WI:

- [108#48][DCCA] DCCA R2 feature list (Huawei)

    - Scope: Identify features and dependencies (incl also e.g. TDD-FDD FRx applicability), and structure (per band per BC when applicable), for LTE and NR,

    - Intended outcome: Agreeable Input to R2 feature list(s), Agreeable TPs 38.331 38.306

    - Deadline:  2020-01-30

#### 2.2.2	Remaining Open issues

- Support of asynchronous and synchronous NR-NR Dual Connectivity:

  - RRC signaling aspects

- Early Measurement reporting:

  - Most important agreements in place and already captured in the running stage 3 CRs. Remaining issues being addressed via email discussion.

- Efficient and low latency serving cell configuration/activation/setup:

  - Some aspects of SCell dormancy behavior remain (e.g. UL SRS handling, temporary RS configuration for fast activation)

  - Some aspects of Resume of MCG SCells/SCG (e.g., How to handle the case if Resumed cells/scg or cells/scg to resume is in fact not present)

- Fast recovery:

  - All agreements in place, objective completed from RAN2 point of view (only minor updates to stage 3 running CRs remain)

- Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers:

  - Baseline CR agreed, only final checking/confirmation required.

- Other aspects:

  - UE capabilities

  - RRC signalling design for async CA/DC

## 2.3	RAN3

#### 2.3.1	Agreements

##### RAN3#103 (Feb/Mar 2019)

There were only high-level discussions and no agreements yet.

##### RAN3#103bis (Apr 2019)

No TUs allocated.

##### RAN3#104 (May 2019):

No TUs allocated.

##### RAN3#105 (August 2019):

No TUs allocated.

##### RAN3#105bis (October 2019)

- Support of efficient Cell Setup

- Discussions on the RAN2 agreements and how to design the network aspects in terms of X2 and Xn signalling.

- There were also discussions whether all possible scenarios (with MN change and/or SN change).

- First detailed discussions on stage 3.

- For reference see [3-1]

- Support of Fast MCG recovery:

- Basic agreement to include respective X2 and Xn signalling in the RRC TRANSFER message to carry the MN RRC messages MCGFailureInformation (addressed to MN) and RRC Reconfiguration or RRC Release (addressed to UE).

##### RAN3#106 (November 2019):

- Support of efficient Cell Setup

- CR for 37.340 was endorsed [3-2] excluding MN change and SN change scenarios due to ongoing discussions in RAN2.

- CR for 38.423 agreed in [3-3] and CR for 36.423 agreed in [3-4], containing new codepoints of the Lower Layer presence status change IE to allow “suspend lower layers” and “resume lower layer”, in addition to Rel-15 codepoints that (only) allow to release and re-establish lower layer resources. Remaining functionality from Rel-15 was maintained (keeping SDAP entities (in NG-RAN only), PDCP entities, Xn-U/X2-U bearer resources, NG-U/S1-U bearer resources and UE context information shall be kept).

- CR for 38.473 agreed in [3-5]. Impact to F1AP was not discussed explicitly at RAN3#105bis. It was however decided that more discussions on the UE context content which has to be maintained during lower layer suspension in the DU in order to specify minimum requirements upon receiving the Lower Layer Presence Status Change IE.

- The CR for F1AP [3-6] made it necessary to also provide stage 2 in TS 38.401. A CR was agreed in [3-5] and includes an example message flow for RRC_INACTIVE.

- Support of Fast MCG recovery:

- Continuation of stage 2/3 work based on agreements from RAN3#105bis

- CR for 37.340 was endorsed [3-7] introducing 2 example message flows, one for EN-DC and one for MR-DC with 5GC.

- CR for 38.423 agreed in [3-8] and CR for 36.423 agreed in [3-9], containing new IEs for inter-node RRC messages within the RRC TRANSFER message, allowing to relay RRC messages sent received over SRB3 via X2/Xn-C.
In addition, it included the possibility to allow the MN knowing whether this option was actually configured in the UE by the SN.

#### 2.3.2	Remaining Open issues

- Final discussions for specification in F1AP on the UE context content which has to be maintained during lower layer suspension in the DU in order to specify minimum requirements upon receiving the Lower Layer Presence Status Change IE

## 2.4	RAN4

#### 2.4.1	Agreements

##### RAN4#90 (Feb/Mar 2019)

No TUs allocated.

##### RAN4#90bis (Apr 2019)

No TUs allocated.

##### RAN4#91 (May 2019):

No TUs allocated.

##### RAN4#92 (August 2019):

A WF was agreed in R4-1909997.

- Early Measurement reporting:

Agreements:

- RAN4 defines measurement requirements for UE early measurement in IDLE and INACTIVE

- RAN4 defines UE requirements for NR Inter-RAT for early measurements

- RAN4 defines UE requirements for LTE Inter-RAT for early measurements

- The supported measurement quantities for early measurements are RSRP and RSRQ

- RAN4 defines measurement accuracy for early measurement reporting

- RAN4 defines measurement period for measurements for early reporting

- RAN4 captures requirements for early reporting based on SIB configuration

- In NR, supporting IDLE/INACTIVE mode CA measurement should not extend the measurement capability defined in Rel-15

- Discuss the definition and restriction on carriers for early measurement reporting.

- Discuss if the LTE way of using search thresholds for limiting the UE measurements for idle mode measurements for reporting.

- Analyze the necessary changes and proposed requirements

- Efficient and low latency serving cell configuration/activation/setup

Agreements:

Direct NR SCell activation:

- RAN4 defines delay requirements for directly activated configured SCell.

- RAN4 defines interruption requirements for directly activated configured SCell.

- At least as a starting point, time lines for direct NR SCell activation can be based on adapted time lines of corresponding E-UTRA requirements, where one adaptation is that additional time for CSI measurement and reporting is added.

- Provide input to RAN4#92bis on whether interruption window is to be adapted in NR requirements.

- Provide input to RAN4#92bis on values for parameters in the direct NR SCell activation time line.

- Provide analysis on necessary changes and proposed requirements

SCell activation delay:

- Two reply LSs were agreed as response to LSs received from RAN1 and RAN2:

  - On maximum allowed SCell activation delay for Rel16 CA (R4-1909995)

  - On NR fast SCell activation (R4-1910574)

##### RAN4#92-bis (October 2019):

A WF was agreed in R4-1912717.

- Early Measurement reporting:

Agreements:

- Requirements for early measurement reporting will be the same in Inactive mode and Idle mode using same principle as currently defined Inactive measurement requirements.

- For Idle measurements for early Measurement reporting companies are encouraged to provide input and views on following:

  - For NR,

    - Should RAN4 use the concept of UE idle mode measurement capability (total number of carrier) for defining requirements for early measurement reporting?

    - Should RAN4 adopt the concept of overlapping and non-overlapping carriers for defining requirements for early measurement reporting?

  - For NR, the number of carriers the UE should be able to report early measurements:

    - NR inter-frequency carriers

  - Use of measurement thresholds to decide when measurements for early reporting should be performed by the UE.

  - Impact on mobility measurement performance from measurements for early reporting.

- For Idle measurements for introducing Inter-RAT early measurements in 36.133 companies are encouraged to provide input and views on following:

  - For LTE,

    - Principle of introducing NR inter-RAT measurements for early reporting in LTE.

    - Number of NR carriers supported for early measurement reporting.

    - Detected cell status upon transitioning from connected to:

      - Idle mode, or

      - inactive mode

- For Idle measurements for UE behaviour, companies are encouraged to provide input and views on following, based on RAN2 outcome:

  - For NR,

    - At cell change, while the T331 timer is running, what is UE behaviour regarding measurements configured for EMR when:

      - One cell change occur

      - More than one cell change occur

    - At cell change, what is the UE behaviour related to reporting of EMR

      - RAN4 needs to define UE behaviour and the applicable requirements for EMR when a cell change occurs while the timer T331 is running

        - Option1: UE always stops

        - Option 2: UE always continues

        - Option 3: UE is either stops/restarts or continues the early measurement, e.g., based on the T331 timer and the time point of the cell change relative to the timer, measurement type (inter-frequency, inter-RAT, or intra-frequency), and/or the type of the change or no change in the cell/carrier relation caused by the cell change

          - For example, the UE could restart the measurement if the carrier relation or measurement type changes, say from overlapping carriers to non-overlapping carriers or from intra-frequency to inter-frequency, while the UE could continue if the carrier relation or measurement type do not change

        - If the stops the early measurements, will they be reported?

          - FFS: The stopped early measurement based on old samples (before the cell change) may still be reported to the network, at least when the remaining time to the expiry of the T331 timer is short or not sufficient for the new measurement (after the cell change)

        - If the UE continued, which requirement applies?

          - The early measurement, which was continued over a time period with one or more cell changes and was performed on a carrier which was a non-overlapping carrier with the serving carrier before and/or after the cell change, shall meet the requirement corresponding to that for the non-overlapping carrier

          - FFS for other scenarios

  - The concept of unknown cells considering early measurement reporting.

    - The UE shall not report or indicate the availability of measurements for EMR for unknown cells

    - If a cell measured for EMR purpose gets unknown while the T331 timer is running, the UE has to perform the cell identification prior to measuring that cell

  - Impact on early measurements after T331 stops and before reporting EMR

    - Observation: A UE may continue the measurements at least on the overlapping carriers (e.g., for mobility purpose) even after T3331 stops and report them for EMR purpose

    - If the UE continues a measurement after T331, e.g. on the overlapping carrier, RAN4 specifies applicable requirements if such measurements can be reported for EMR

##### RAN4#93 (November 2019):

- Direct SCell activation in RRC resume message (RAN2 LS):

A response LS to RAN2 was approved in R4-1915844, where RAN4 replies that it is RAN4 consensus that there is no blocking issue for allowing direct SCell activation in RRC resume before the UE sends early measurement results.

- Direct SCell activation:

The time lines and activation delays for direct SCell activation upon handover and upon SCell addition were agreed. The following issues are left for further studies:

- FFS for the direct activation requirements for multiple SCells.

- FFS during which time period of the activation delay interruption is allowed.

- Dormancy behaviour:

RAN4 agreed to await further progress in RAN1 before continuing work on requirements relating to dormancy behaviour.

- Early Measurement reporting:

A WF on early measurements was agreed in R4-1915842 with the following agreements.

Tentative agreement:

- An overlapping carrier is a carrier which the UE is actively measuring for mobility and EMR.

- An non-overlapping carrier is a carrier which the UE is actively measuring for EMR only.

Working assumption on EMR carriers:

- RAN4 EMR requirements for NR carriers while camped on NR only apply if EMR carriers are in CA combination supported by UE

- RAN4 EMR requirements for NR carriers while camped on LTE only apply if UE supports EN-DC and NR EMR carriers are in the ENDC band combination supported by UE

Agreements on NR measurements for UE camping on NR cell):

- Number of carriers to measure for EMR considering the use of overlapping and non-overlapping carriers:

  - Option 1: Re-use the LTE euCA requirements for number of carriers to be reported for early measurements (Qualcomm)

    - Limit the number of overlapping and number of non-overlapping carriers (i.e. LTE EMR approach)

  - Option 2: Specify the total number of carriers (MediaTek)

    - Limit the number of overlapping + non-overlapping carriers (i.e. total number of carriers for EMR)

  - Option 3: No limit on carriers that can be configured for EMR (as long as it does not exceed UE measurement capability) (Nokia, Huawei)

  - Option 4: Limit the number of non-overlapping carriers only (New, option 3 + limit on number of non-overlapping carriers)

- FFS: UE behavior when the above limits are exceeded

- Measurements for EMR and mobility:

  - Possible options:

    - S-measure threshold is assumed not applicable to carriers for EMR.

      - Overlapping EMR carriers are measured continuously.

      - Non-EMR carriers are measured according to s-Measure threshold.

    - Hence, beyond s-Measure threshold UE measures both EMR and non-EMR carriers.

    - To reduce overall measurement latency:

      - EMR carriers are measured when Serving cell RxLev is good

      - EMR carriers are not required to be measured or can be down selected when Serving cell RxLev is not good

      - Non-EMR carriers obey s-Measure threshold

  - RAN4 to discuss whether to apply threshold on serving cell condition for EMR:

    - Example:

      - EMR carriers will measured when Rxlev>threshold

      - EMR carriers are not required to be measured 
or can be down selected when Rxlev<threshold

- Early measurement and cell change, discuss following aspects :

  - Possible Agreement: Follow RAN2 agreements which are (for information)

    - Dedicated:

      - UE performs measurements for EMR using the RRCRelease configured MOs independently from PCell changes while T331 is running.

    - SIB Configured:

      - UE performs EMR measurements according to serving cell SIB configuration while T331 is running.

    - FFS whether the above also applies when the carrier changes from overlapping to non-overlapping or from non-overlapping to overlapping after the cell change

  - Discuss if any additional rules are necessary from RAN4 side

- Early measurement and cell change, discuss following aspects:

  - At cell change, what is the UE behaviour related to reporting of EMR:

    - Option 1: leave it to UE implementation

    - Option 2: RAN4 defines specific rules

  - RAN4 to discuss introduction of the concept of unknown cells considering early measurement reporting

    - Option 1: Introduce known/unknown cells for EMR

    - Option 2: Do not introduce known/unknown cells for EMR

  - Impact on early measurements after T331 stops and before reporting EMR

    - FFS if RAN4 specifies applicable requirements for the early measurements performed and used for EMR after the expiry of the T331 timer (at least for overlapping carriers)

    - FFS if the UE shall be capable of reporting the results of the early measurements at least up to T0 after the expiry of T331

- On detected cell and cell detection:

  - The existing requirements under ca-IdleModeMeasurements as baseline,

  - FFS whether to defines requirements for the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode in 36.133 section 4.9.2.1

- Possible options:

  - Option 1: Apply the same principles for NR inter-frequency cells.

  - Option 2: Do not apply the same principles for NR inter-frequency cells.

Agreements on (inter-RAT) NR measurements for UE camping on LTE cell:

- Different UE capabilities:

  - Option 1: UE supporting ca-IdleModeMeasurements also support measurement of NR inter-RAT for early reporting.

  - Option2: UE not supporting ca-IdleModeMeasurements support measurement of NR inter-RAT for early reporting.

  - Other options are not precluded

  - For all options: it is assumed that the UE under discussion is a UE capable of EN-DC, while a UE capable of existing ca-IdleModeMeasurements might not support EN-DC.

- Possible options: proceed the work in RAN4 defining requirements for all cases.

- Number of carriers:

  - Possible options:

    - Option 1: For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements existing number of LTE carriers as defined under ca-IdleModeMeasurements are kept unchanged

      - For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements shall support NR inter-RAT measurements on a number of NR carriers in addition to existing inter-frequency carriers as defined under ca-IdleModeMeasurements.

    - Option 2: For a UE supporting EMR of NR inter-RAT carriers in addition to ca-IdleModeMeasurements the UE support a number of EMR carriers which may be LTE or NR (total number of euCA EMR carriers remain unchanged)

    - Option 3: For a UE supporting EMR of NR inter-RAT carriers but not supporting ca-IdleModeMeasurements shall support measurements on a number of NR carriers.

    - Other options are not precluded

- Number of NR inter-RAT carriers:

  - FFS

- Detected cell state:

  - The existing requirements under ca-IdleModeMeasurements as baseline,

  - FFS whether to define requirements for the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode in 36.133 section 4.9.2.1

- Possible options:

  - Option 1: Apply the same principles for NR inter-RAT cells.

  - Option 2: Do not apply the same principles for NR inter-RAT cells.

#### 2.4.2	Remaining Open issues

- Support of asynchronous and synchronous NR-NR Dual Connectivity

  - Waiting progress from RAN1

- RRM requirements (core and performance) and test cases for early Measurement reporting

- Efficient and low latency serving cell configuration/activation/setup

- Fast recovery

- Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

[3-1]	R3-196154 "Summary of offline discussion for RRC Resume", report on offline discussions during RAN3#105bis

[3-2]	R3-197599 "Resuming SCG in RRC Resume", draft CR 37.340 Rel-16, endorsed at RAN3#106

[3-3]	R3-197600 "Resuming SCG in RRC Resume", CR0259r2 38.423 Rel-16, agreed at RAN3#106

[3-4]	R3-197779 "Resuming SCG in RRC Resume", CR1391r4 36.423 Rel-16, agreed at RAN3#106

[3-5]	R3-197778 "CR for TS 38.473 on supporting SN Resume during the RRCResume procedure", CR0514r2 38.473 Rel-16, agreed at RAN3#106

[3-6]	R3-197743 "Support of SN Resume during the RRCResume procedure", CR0112 38.401 Rel-16, agreed at RAN3#106.

[3-7]	R3-197780 "Support for MCG RLF information delivery over SRB3", draft CR 37.340 Rel-16, endorsed at RAN3#106

[3-8]	R3-197781 "Fast MCG link recovery via SRB3", CR1416r2 36.423 Rel-16, agreed at RAN3#106

[3-9]	R3-197782 "Fast MCG link Recovery with SRB3", CR02852 38.423 Rel-16, agreed at RAN3#106

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template