3GPP TSG RAN meeting #86										RP-192552

Sitges, Spain, December 9-12, 2019

Status Report to TSG

Agenda item:			10.2.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_DL-MIMO_EE |  |  |  |  |
| Unique ID | 800086 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182901 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2020 | Performance part: 09/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 90% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Yubo YANG |
|  | Company | Huawei |
|  | Email | yangyubo1@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Request RAN4 TU for the RAN4 core part objectives in the WID.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#98bis:

11 contributions [1-11] were submitted to RAN1#98bis meeting, and following were achieved:

Agreement

If intra-subframe repetition, frequency hopping and/or antenna switching are configured simultaneously, repetition is performed before frequency hopping and/or antenna switching.

Agreement

For the setting of transmit power of additional SRS:

Agreement

For DCI format of TPC for additional SRS, at least DCI formats 3/3A/3B is supported

- For 3/3A, the TPC index for the additional SRS is configured by higher layer signalling

- For 3B, the RRC parameter for the starting position of the additional SRS is configured by higher layer signalling

- FFS: UL grants, DL grants

Agreement

For configuration of additional SRS:

- Start position and number of symbols for additional SRS includes guard symbols if configured

Agreement

For the handling of collision of SRS and PUCCH/PUSCH/PRACH transmission for UEs not supporting sPUSCH/sPUCCH

- UE is not expected to be triggered with SRS and PUCCH/PUSCH/PRACH on colliding subframes.

Agreement

The existing power headroom (PH) type 3 is reused for selectively reporting additional SRS or PUSCH-less SCell SRS.

RAN1#99:

14 contributions [12-25] were submitted to RAN1#99 meeting, and following were achieved:

Agreement

The number of antenna switching is:

- for 1T2R is configured or the number of pairs is configured as 2 for 2T4R

- if the number of pairs is configured as 3 for 2T4R

- for 1T4R

Agreement

The number of frequency hops, N_FH (integer value), is determined according to the following formula

- N=R*NAS*NFH+(NAS*GSAS-1)*GSAS+(NFH*GSFH-1)*GSFH

R is the repetition, N_AS is the number of antenna switchings, N_FH is the number of frequency hops, GS_AS is the guard symbol configuration for antenna switching which is 0 or 1, GS_FH is the guard symbol configuration for frequency hopping, N is the duration in terms of OFDM symbols for the additional SRS symbols

- It is up to network to make sure that N_FH is an integer

- Note: It is up to the editor how to capture this in the specifications

Conclusion

No consensus in RAN1 for the introduction of sPUSCH and/or sPUCCH capability for Rel-16 UEs to enable sPUSCH and/or sPUCCH transmission in the SRS subframe.

Agreement (RRC impact)

- Independent configuration of guard symbol for antenna switching and/or frequency hopping is supported

Conclusion

For close loop power control of additional SRS, UL/DL grants are not used for close loop power control of additional SRS.

Agreement

- For PUSCH-less Scell, UE can be configured with either additional SRS or legacy SRS.

Agreement (MAC CE impact)

For PCell and SCell with PUSCH, support Type 3 PHR reporting for additional SRS

Agreement

The following working assumption is confirmed

When intra-subframe frequency hopping/repetition and intra-subframe antenna switching are concurrently configured, frequency hopping should be performed before antenna switching.

Note: Concerns were raised by Qualcomm in the above agreement introduces unnecessary specification impact

Agreement (RRC impact)

For RRC parameter P_{O_UE_SRS,c}, update the FFS on value range as “Same as p0-UE-PUSCH-r15 (-16,15)”

2.1.2	Remaining Open issues

No open issues.

2.2	RAN2

2.2.1	Agreements

RAN2#108:

6 contributions [26-31] were submitted to RAN2#108 meeting, and following were achieved:

R2-1916317	Introduction of DL MIMO efficiency enhancement	Huawei, HiSilicon	CR	Rel-16	36.306	15.6.0	1725	1	B	LTE_DL_MIMO_EE-Core

R2-1916318	Introduction of DL MIMO efficiency enhancement	Huawei, HiSilicon	CR	Rel-16	36.331	15.7.0	4175	1	B	LTE_DL_MIMO_EE-Core

- CRs are endorsed as baseline

- Updated CRs should be submitted to next meeting with latest RAN1 status.

2.2.2	Remaining Open issues

- Update the endorsed CRs on configuration and UE capabilities.

- Identify the impacts to PHR.

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#93:

6 contributions [32-37] were submitted to RAN4#93 meeting, and following were achieved:

Agreement:

For both SRS hopping and antenna switching, RAN4 can conclude if the transisent period is non-zero, same guard period is needed for some UE implementation.

RAN4 has not discussed the length of guard period yet

R4-1916030	Draft reply LS on UE capability of guard period for SRS with frequency hopping and antenna switching					Source: Huawei, HiSilicon

Decision: 		The document was Approved

2.4.2	Remaining Open issues

- Specify RF requirement of UE for additional SRS symbols and virtual cell ID for SRS.

- Specify the necessary UE performance requirements.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1910095	Remaining issues of additional SRS symbols in normal UL subframe	Huawei, HiSilicon

- R1-1910141	Discussion of additional SRS symbols	Lenovo, Motorola Mobility

- R1-1910195	Disucssion on remaining issues of additional SRS	vivo

- R1-1910288	Discussion on additional SRS symbols	ZTE

- R1-1910450	Discussion on additional SRS symbols	Samsung

- R1-1910580	Discussion on additional SRS symbols	LG Electronics

- R1-1910631	Remaining details for additional SRS	Intel Corporation

- R1-1910731	Additional SRS symbols	Qualcomm Incorporated

- R1-1910852	Discussion on introduction of additional SRS symbols	Nokia, Nokia Shanghai Bell

- R1-1910892	Remaining details for additional SRS symbols	Ericsson

- R1-1911357	Feature summary on LTE DL MIMO efficiency enhancement	Huawei, HiSilicon

- R1-1911916	Remaining issues of additional SRS symbols in normal UL subframe	Huawei, HiSilicon

- R1-1911934	Discussion on additional SRS symbols	ZTE

- R1-1912002	Remaining issues on additional SRS symbols..	vivo

- R1-1912190	Remaining details for additional SRS	Intel Corporation

- R1-1912267	Discussion on additional SRS symbols	LG Electronics

- R1-1912315	Discussion of additional SRS symbols	Lenovo, Motorola Mobility

- R1-1912440	Discussion on additional SRS symbols	Samsung

- R1-1912655	Discussion on introduction of additional SRS symbols	Nokia, Nokia Shanghai Bell

- R1-1912660	Remaining details for additional SRS symbols	Ericsson

- R1-1912689	Additional SRS symbols	Qualcomm Incorporated

- R1-1913167	Introduction of DL MIMO efficiency enhancements for LTE	Ericsson

- R1-1913168	Introduction of Rel-16 DL MIMO EE features in 36.212	Futurewei

- R1-1913169	Introduction of DL MIMO efficiency enhancements for LTE	Motorola Mobility

- R1-1913326	Feature summary on LTE DL MIMO efficiency enhancement	Huawei

- R2-1915878	Discussion on DL MIMO efficiency enhancement	Huawei, HiSilicon

- R2-1915644	Introduction of Additional SRS	Ericsson	CR	36.321

- R2-1915879	Introduction of DL MIMO efficiency enhancement	Huawei, HiSilicon	CR	36.306

- R2-1915880	Introduction of DL MIMO efficiency enhancement	Huawei, HiSilicon	CR	36.331

- R2-1916317	Introduction of DL MIMO efficiency enhancement	Huawei, HiSilicon	CR	36.306

- R2-1916318	Introduction of DL MIMO efficiency enhancement	Huawei, HiSilicon	CR	36.331

- R4-1915313	UE capability of requiring no guard period for SRS with frequency hopping and antenna switching Qualcomm Inc.

- R4-1915363	On new SRS requirements for LTE eMIMO Huawei, HiSilicon

- R4-1915314	Reply LS on UE capability of requiring no guard period for SRS with frequency hopping and antenna Qualcomm Inc.

- R4-1915362	Draft reply LS on UE capability of guard period for SRS with frequency hopping and antenna switching Huawei, HiSilicon

- R4-1916028	Draft reply LS on UE capability of guard period for SRS with frequency hopping and antenna switching Huawei, HiSilicon

- R4-1916030	Draft reply LS on UE capability of guard period for SRS with frequency hopping and antenna switching Huawei, HiSilicon

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template