3GPP TSG RAN meeting #86										RP-192580

Sitges, Spain, December 9-12, 2019

## Status Report to TSG

Agenda item:			9.4.7

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_POS |  |  |  |  |
| Unique ID | 830077 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191156 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2020 | Performance part: 09/2020 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 80% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  |  |
| --- | --- | --- |
| Rapporteur | Name | Alexey Khoryaev |
|  | Company | Intel Corporation |
|  | Email | alexey.khoryaev@intel.com |
| Rapporteur | Name | Florent Munier |
|  | Company | Ericsson |
|  | Email | florent.munier@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 endorsed the initial table of higher layer parameters for NR Positioning at the end of RAN1#98bis:

R1-1911564	Update to the list of higher layer parameters for NR positioning	Intel Corporation

The endorsed table was further updated based on agreements made at RAN1#99. The corresponding summary of the offline discussion is provided in R1-1913536.

#### Agreements on DL Reference Signals for NR Positioning

With respect to DL PRS signal design, the following set of agreements was made regarding BW, Start PRB and DL PRS Point A for DL PRS configurations:

Start PRB parameter for DL PRS configuration has granularity of one PRB with a minimum of 0 and a maximum of [2176] PRBs

4 PRB granularity is used for DL PRS BW configuration

Maximum BW for DL PRS in PRBs does not exceed 272 PRBs

Minimum BW for DL PRS in PRBs is not less than 24 PRBs

The DL-PRS-Point A can take values given by ARFCN-ValueNR

RAN1 further clarified definition of the frequency layer for NR Positioning:

Rename “frequency layer” to “positioning frequency layer” in previous agreements for positioning in Rel-16

In the agreements made in RAN1 related to NR positioning, a “positioning frequency layer” is a collection of DL PRS Resource Sets across one or more TRPs which have

  - the same SCS and CP type

  - the same centre frequency

  - the same point-A (already agreed)

  - FFS: details on configured BW

On resource element pattern the following patterns were initially agreed by RAN1 (final patterns were defined during RAN1#99):

At least the following DL PRS RE patterns, with comb size N equal to number of symbols M are supported (figures for information)

  - Comb-2: Symbols {0, 1} have relative RE offsets {0, 1}

  - Comb-4: Symbols {0, 1, 2, 3} have relative RE offsets {0, 2, 1, 3}

  - Comb-6: Symbols {0, 1, 2, 3, 4, 5} have relative RE offsets {0, 3, 1, 4, 2, 5}

FFS: other DL PRS RE patterns including patterns to coexist with LTE CRS/PRS

Furthermore RAN1 WG discussed different higher layer parameters required for configuration:

Parameter DL-PRS-ResourceRepetitionFactor is configured for a DL PRS Resource Set and controls how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set

  - Values: 1, 2, 4, 6, 8, 16, 32

Parameter DL-PRS-ResourceTimeGap is configured for a DL-PRS Resource Set

  - DL-PRS-ResourceTimeGap indicates offset in units of slots between two repeated instances of a DL PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL PRS Resource Set

  - DL-PRS-ResourceTimeGap is provided only if DL-PRS-ResourceRepetitionFactor is configured and is greater than 1

  - Values: 1, 2, 4, 8, 16, 32

The time duration spanned by one DL PRS Resource set containing repeated DL PRS Resources should not exceed DL-PRS-Periodicity

Note: UE RX beam sweeping is up to UE implementation

With respect to support of DL PRS muting operation, the following two options were agreed by RAN1 WG.

A bitmap for DL PRS muting is configured for a DL PRS Resource Set. The following options are supported for the applicability of the bitmap.

  - Option 1: Each bit in the bitmap corresponds to a configurable number of consecutive instances (in a periodic transmission of DL-PRS resource sets) of a DL-PRS Resource set

    - All DL-PRS Resources within a DL-PRS Resource Set instance are muted for a DL-PRS Resource Set instance that is indicated to be muted by the bitmap

  - Option 2:

    - Each bit in the bitmap corresponds to a single repetition index for each of the DL-PRS Resources within an instance of a DL-PRS Resource Set (The length of the bitmap is equal to DL-PRS-ResourceRepetitionFactor)

    - The above applies to all instances of the DL-PRS Resource Set that the above DL-PRS Resources are part of.

  - Bitmap size values: 2, 4, 8, 16, 32 bits

  - FFS: Configuration of bitmaps corresponding to both options at the same time to the UE

Considering various IDs: resource \ resource set \ TRP IDs, for NR Positioning operation, the following agreements were made by RAN1 WG to uniquely identify DL PRS Resource:

An ID is defined that can be associated with multiple DL PRS Resource Sets associated with a single TRP.

  - This ID can be used along with a DL PRS Resource Set ID and a DL PRS Resources ID to uniquely identify a DL PRS Resource

  - Name can be defined by RAN2

Each TRP should only be associated with one such ID

DL PRS Resource IDs are locally defined within DL PRS Resource Set

DL PRS Resource Set IDs are locally defined within TRP

In terms of DL PRS configuration, the following physical layer details were agreed by RAN1:

All DL PRS Resources of the DL PRS Resource Set have the same bandwidth

All DL PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL PRS Bandwidth and Start PRB

A higher layer parameter, DL-PRS-SFN0-Offset, is configured

  - Defines time offset of the SFN0 slot 0 for given TRP with respect to SFN0 slot 0 of FFS for RAN2 WG 1) serving TRP or 2) serving cell 3) etc.

A higher layer parameter, DL-PRS-ResourceSetSlotOffset, is configured

  - Defines slot offset with respect to SFN slot 0 for a TRP where DL PRS Resource Set is configured (i.e. slot where the first DL PRS Resource of DL PRS Resource Set occurs)

  - Values: {0,1,…, DL-PRS-Periodicity-1}

The previously defined higher layer parameter, DL-PRS-RstdReferenceInfo, is used as a reference to determine the higher layer parameters, DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty

RAN1 was not able to reach consensus on support of the DL-PRS-ResourceType parameter in Rel.16:

The parameter DL-PRS-ResourceType is not introduced in Rel-16

For generation of DL PRS signals, RAN1 has agreed on the following equation for initialization of PRBS sequence generator:

For DL PRS sequence initialization cinit is defined as follows:

  - where the parameter a = 2^22 and n_id has a range of 0~4095

RAN1 has also discussed multiplexing of DL PRS signals with other DL channel signals and has reached the following agreements:

For serving TRP, UE assumes that DL PRS is not mapped to any symbol that contains SS/PBCH

  - Note: In a slot in which SS/PBCH is transmitted on some symbols, DL PRS can be transmitted on other symbols

For neighbor TRPs, when time-frequency location for SSB transmissions on the neighbour TRP is provided, UE assumes that the DL-PRS is not mapped on symbols occupied by SSB transmissions of the neighbour TRP (i.e. DL PRS is not transmitted on these symbols)

Configured DL PRS are transmitted on DL symbols of a slot configured by higher layers

Configured DL PRS are transmitted on symbols of slot configured as flexible symbols by higher layers

If the UE is not provided with a measurement gap, the UE is not expected to process DL PRS Resources on serving or neighboring cells on symbols indicated as UL by the serving cell

Regarding, DL PRS muting, RAN1 has further clarified the possible options for muting operation:

UE can be configured with any of the following combinations of DL PRS muting options from the prior agreement

  - Option 1 only

  - Option 2 only

  - Option 1 and Option 2

    - If both options are configured, the logical AND operation is applied, i.e. for each bit from the Option 1 bitmap, the logical AND is applied to all bits of Option 2 bitmap

Finally, RAN1 also agreed on additional DL PRS patterns for NR Positioning:

Configuration of 12 symbols for DL PRS Resource is supported

Configuration of CombSize-12 for DL PRS Resource is supported

At most one pattern is to be supported for each agreed pair of number of symbols and combSize

The following relative RE offsets are supported for different combinations of number of symbols and combSize for DL PRS Resource:

|  | 2 symbols | 4 symbols | 6 symbols | 12 symbols |
| --- | --- | --- | --- | --- |
| Comb-2 | {0,1} | {0,1,0,1} | {0,1,0,1,0,1} | {0,1,0,1,0,1,0,1,0,1,0,1} |
| Comb-4 | NA | {0,2,1,3} | NA | {0,2,1,3,0,2,1,3,0,2,1,3}} |
| Comb-6 | NA | NA | {0,3,1,4,2,5} | {0,3,1,4,2,5,0,3,1,4,2,5} |
| Comb-12 | NA | NA | NA | Working assumption: {0,6,3,9,1,7,4,10,2,8,5,11} |

All DL PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of combSize. Parent IE for DL PRS combSize is up to RAN2

Regarding DL PRS periodicity, the common set of periodicity values was agreed for every numerology:

The following periodicity values of DL PRS resource allocation are supported depending on SCS

  - 2μ{4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots, µ = 0, 1, 2, 3 for SCS 15, 30, 60 and 120kHz respectively

In terms of DL PRS TX power, the following agreements have been made:

UE assumes constant EPRE for all REs of a given DL PRS Resource

DL PRS Resource TX power value range is the same as for SSB

Discussing UE capability, RAN1 made the following agreement:

Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability

#### Agreements on UL Reference Signals for NR Positioning

With respect to frequency hopping for SRS for positioning, the following agreement was made by RAN1 WG:

Frequency hopping of SRS for positioning is not supported in Rel-16

The following initial agreement was made with respect to SRS for positioning resource element pattern design (this agreement was further updated at RAN1#99):

The supported relative RE offsets for combinations of the number of symbols (1,2,4,8,12) and comb sizes (2,4,8) for SRS are given in the table below. At least one pattern for each pair are to be selected from the following options:

| Number of symbols / Comb size | 1 | 2 | 4 | 8 | 12 |
| --- | --- | --- | --- | --- | --- |
| 2 | {0} | {0, 1} | FFS: {0, 1, 0, 1} and/or {0, 0, 1, 1} | N/A | N/A |
| 4 | N/A | {0, 2} | FFS: {0,2,1,3} and/or {0,1,2,3} and or {0,2,3,1} | FFS: {0,2,1,3, 0,2,1,3} and/or {0,1,2,3, 0,1,2,3} and/or {0,0,1,1,2,2,3,3} | FFS: {0,2,1,3, 0,2,1,3, 0,2,1,3} and/or {0,1,2,3, 0,1,2,3,0,1,2,3} and/or {0,0,0,1,1,1,2,2,2,3,3,3} |
| 8 | N/A | N/A | N/A | FFS: {0,4,2,6,1,5,3,7} and/or {0,4,1,5,2,6,3,7} and/or {0,1,2,3,4,5,6,7} and/or {0,4,6,2,5,7,3,1} | N/A |

At RAN1#99, the above agreement was finalized as follows:

For each pair of comb size and number of symbols of SRS, there is one single RE pattern. The patterns in the table below are selected:

| Number of symbols / Comb size | 1 | 2 | 4 | 8 | 12 |
| --- | --- | --- | --- | --- | --- |
| 2 | {0} | {0, 1} | {0, 1, 0, 1} | N/A | N/A |
| 4 | N/A | {0, 2} | {0,2,1,3}, | {0,2,1,3, 0,2,1,3} | {0,2,1,3, 0,2,1,3, 0,2,1,3} |
| 8 | N/A | N/A | {0,4,2,6} | {0,4,2,6,1,5,3,7} | {0,4,2,6,1,5,3,7,0,4,2,6} |

Regarding periodic, semi-persistent and aperiodic transmission of SRS for positioning, RAN1 made the following agreement:

SRS for positioning supports semi persistent configuration with MAC CE activation/deactivation, with SRS for positioning to be received at the serving cell and neighbor cell

The aperiodic SRS for positioning is triggered by a DCI

  - There is no impact to Rel-15 DCI (reuse the triggers in place in rel-15)

  - The support of the reception of aperiodic SRS for positioning by the neighbor cell, is up to decision by RAN2 and RAN3 working groups

The Rel-15 aperiodic SRS framework is supported for SRS for positioning.  Transmission of aperiodic SRS for positioning is a UE capability. There is no impact on DCI design.

For new comSize 8, the additional number of cyclic shifts was agreed:

For UL SRS for positioning configured with comb size 8, the number of cyclic shifts is 6

For the purpose of SRS for positioning sequence generation, RAN1 reached the following agreement:

Reuse release 15 sequence initialization for the UL SRS for positioning. The number of bits for the RRC parameter sequence ID is increased to 16.

With respect to configuration of SRS for positioning, RAN1 made the following decision:

Only a single port is used for SRS for positioning in Rel-16

The maximum number of SRS resources per set for positioning is 16

The maximum number of supported SRS resource sets for positioning is a UE capability, with configurability of up to 16 resource sets per BWP.

  - Values for the UE capability include {1 ,16}. Other values FFS

During discussion on multiplexing of UL transmissions with SRS for positioning the following agreement was reached:

The SRS collision rules for Rel-15 are reused for Rel-16 for SRS for positioning

If the SRS for positioning collides with PUSCH, the SRS is dropped in the symbols where the collision occurs.

  - Note: The phase continuity between transmitted symbols on either side of a transmission gap may not be maintained

In addition, RAN1 agreed on RRC signaling for SRS for positioning and that configuration of SRS is per BWP:

RAN1 assumes that a new IE is configured for SRS for positioning in RRC signalling

Configuration of SRS for positioning is per UL BWP

The union of periodicities (and associated slot offsets) supported for NR SRS Rel-15 and the NR DL-PRS Rel-16 is supported for SRS for positioning in Rel-16.

Finally, during the discussion on the use of Rel.15 SRS for NR positioning, the following agreement was made:

Support reuse of Rel-15 SRS resource set for NR UL RTOA, AoA and gNB RSRP measurements for positioning in NR.

  - Note: There is no impact to specifications managed by RAN1

  - Note: There is no impact to specifications managed by RAN4 for UE requirements

  - Note: No new UE behaviour is expected

RAN1 has send LS on SRS for NR Positioning (approved in R1-1911634). The text of LS is provided below for convenience:

1. Overall Description: RAN1 would like to inform other RAN WGs on the following agreement: - SRS for positioning supports semi persistent configuration with MAC CE activation/deactivation, with SRS for positioning to be received at the serving cell and neighbor cell - The aperiodic SRS for positioning is triggered by a DCI - There is no impact to Rel-15 DCI (reuse the triggers in place in rel-15) - The support of the reception of aperiodic SRS for positioning by the neighbor cell, is up to decision by RAN2 and RAN3 working groups - RAN1 assumes that the SRS for positioning configuration will be included in the RRC configuration of a UE and that it is up to RAN2 and RAN3 WG’s scope to analyze further the system level aspects along with any further work on the design of higher layer signaling. 2. Actions: To RAN2, RAN3: ACTION: RAN1 respectfully asks RAN2 and RAN3 WGs to continue work on design of higher layer signalling for NR positioning taking into account corresponding agreements made by RAN1 WG for the NR Positioning work item.

#### Agreements on UE and gNB measurements for NR Positioning

RAN1 discussed the possibility of RSTD measurement b/w E-UTRA and NR cell and reached the following conclusion:

RSTD measured between an E-UTRA cell and an NR cell is not supported in Rel-16

Conclusion:

UE may select a subset of the DL PRS Resource/Resource sets configured by the network to perform and report requested measurements.

RAN1 discussed the necessity to support metric to characterize measurement quality and reached the following agreement:

Support the same set of resolutions for the quality metrics of all UE/gNB timing measurements (DL PRS RSTD, UE Rx-Tx time difference, UL RTOA, and gNB Rx-Tx time difference)

The quality metrics for all UE/gNB timing measurements include at least the following fields

  - Value: specifies the best estimate of the uncertainty of the measurement

  - Resolution: specifies the resolution levels used in the Value field

    - FFS: non-linear steps

    - FFS: scaling factor

FFS: NumSamples: specifies how many measurements used by UE/gNB to determine the MeasQuality

  - FFS: whether the number of samples is controlled and configured by network

The above agreements reached during RAN1#98, were further clarified during the RAN1#99 meeting with the following updates:

Angular measurements

  - The reporting granularity for the gNB angle measurements (AoA, ZOA) is defined as 0.1 degree.

  - RAN1 assumes that the details of the reporting granularity and ranges for the gNB angle measurements (AoA, ZoA) will be determined by RAN4.

  - The reporting granularity for the set of angles α (bearing angle), β (downtilt angle) and γ  (slant angle)  for the translation of the GCS to LCS is defined as 0.1 degree.

  - RAN1 assumes that the reporting granularity, the range of values and mapping for DL PRS RSRP and UL SRS RSRP measurements will be determined by RAN4

Timing measurement

  - The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0.

  - Note: RAN4 can determine if -1 can be a minimum value

  - RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth.

Include above information in an LS to RAN2/RAN3/RAN4

The following was agreed with respect to resolution and values for metrics characterizing quality of measurements during RAN1#99:

The quality metrics for both DL and UL timing based measurements have a resolution of {0.1m, 1m, 10m, 30m} and are reported with a 5-bit value field

The quality metrics for angular measurements (AoA and ZoA) in either LCS or GCS have a resolution of {0.1} deg and are reported with an 8-bit value field.

RAN1 has also discussed DL PRS configuration and UE capability parameters. The following was agreed at the RAN1#98bis meeting:

UE can be configured to measure DL PRS RSTD, UE Tx-Rx time difference, and DL PRS RSRP in up to X5 total number of DL PRS resources within

  - Up to X1 positioning frequency layers

    - FFS: X1

  - Up to X2 TRPs per positioning frequency layer

    - FFS: X2

  - Up to X3 DL PRS resource sets per TRP

    - FFS: X3

  - Up to X4 DL PRS resources per PRS resource set

    - FFS: X4

  - Up to max number of DL PRS Resources per UE

    - FFS: X5

A limit on the maximum number of DL PRS resources configured to the UE for all TRPs within a measurement window is defined.

  - This limit can be signalled as a UE capability.

  - FFS: the relationship of maximum numbers of DL PRS resources with the following factors

    - the type of the measurement window, e.g., sliding window

    - the maximum number of DL PRS OFDM symbols

    - the maximum measurment bandwidths

The above agreements were further clarified during the RAN1#99, as follows:

UE can be configured for DL PRS processing according to the following table:

| Description | Maximum numbers for DL PRS resources | Values that can be signaled as part of UE Capability |
| --- | --- | --- |
| Max number of frequency layers (X1) | X1=4 | Values = {1,4} FFS: other values |
| Max number of TRPs per frequency layer (X2) | X2=64 |  |
| Max number of PRS resource sets per TRP (X3) per frequency layer | X3=2 | Values = {1,2} |
| Max number of Resources per PRS resource set (X4) | X4=64 | FFS: values |
| Max number of DL PRS Resources per UE (X5) | NA | FFS: values |
| Max number of TRPs for all frequency layers (X6) per UE | 256 | FFS: values |
| Max number of Resources per frequency layer (X7) | NA | FFS: values |

Note: This does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signaled to be different for different features or feature sets.

RAN1 has also discussed configuration of SRS for positioning and UE capability parameters. The following was agreed at the RAN1#98bis meeting:

UE can be configured to transmit up to Y1 SRS resources for positioning per SRS resource Set

UE can be configured to transmit up to Y2 SRS resources for positioning across all SRS resource Sets

UE can be configured to transmit up to total Y3 SRS Resource Sets for positioning

FFS: Y1, Y2, Y3

FFS: whether Y1 and/or Y2 and/or Y3 are per BWP or all BWPs

The above agreement was further clarified during the RAN1#99, as follows:

The maximum number of SRS resources for positioning across all SRS resource sets per BWP is 64.

With respect to UE configuration for measurement reporting the following agreements were made:

UE can be configured to measure and report up to [M] DL PRS RSTD measurements with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability

  - All the RSTD measurements in a single report should have a single reference timing

  - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs.

  - M=[3]

UE can be configured to measure and report up to N (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP

  - N=[3]

The above agreements were further updated at RAN1#99 as follows:

UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability

  - All the RSTD measurements in a single report should have a single reference timing

  - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs.

  - M=[3] 4

UE can be configured to measure and report up to N=8  (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP

  - N=[3] FFS: N is a UE capability

The timestamp was agreed for UE measurement reporting:

A UE measurement can be associated with a time stamp.

  - For DL RSTD and RSRP measurements, the time stamp can include at least the SFN

    - FFS: SFN corresponds to the reference DL PRS resource or DL PRS resource sets (can be determined by the UE)

For Rx-Tx measurements, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing

  - FFS: these values correspond to the reference DL PRS resource or DL PRS resource set (can be determined by the UE)

The above agreement was modified as follows

A UE measurement can be associated with a time stamp. For UE RSTD, DL PRS RSRP and UE Rx-Tx time difference measurement report, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing. These values correspond to the reference provided by the DL-PRS-RstdReferenceInfo.

A gNB measurement can be associated with a time stamp. For UL RTOA, UL SRS RSRP, AoA and gNB Rx-Tx time difference measurement report, the time stamp can include the SFN, as well as the slot number for a reported subcarrier spacing.

RAN1 also agreed that ID of reference DL PRS resource or resource set should be provided in DL-PRS-RSTD-ReferenceInfo:

The ID (previously agreed for TRP) of the reference DL PRS Resource or DL PRS Resource set is provided in DL-PRS-RSTD-ReferenceInfo.

Association of DL PRS Resource with azimuth and elevation angles was discussed by RAN1 with the following agreement reached:

A DL PRS resource can be associated with one or more of the following in either GCS or LCS

  - azimuth angle

  - elevation angle

  - Note: The details of the message (e.g., resolution, range,  etc.) can be defined in RAN3.

At the RAN1#99, the definition of UL angle of arrival was updated as follows:

Modify the previous agreement on the definition of the AoA (ϕ) and ZoA (θ) as follows:

  - AoA (ϕ) and ZoA (θ) define the estimated angles of a user with respect to a reference direction which are determined at the TRP antenna for an UL channel corresponding to this UE.

  - The reference directions can be defined according to either one of the following options (both options are supported in specifications)

    - Option 1:

      - The reference directions are defined with respect to the Global Coordinate System (GCS) as follows:

        - For AoA, the geographical North, positive in a counter-clockwise direction.

        - For ZoA, the zenith/vertical

          - Note:  θ=0 pointing to the zenith and θ=90o pointing to the horizon.

    - Option 2:

      - For AoA and ZoA, tThe reference directions are defined is with respect to the Local Coordinate System (LCS) defined in TR 38.901 as follows:

        - For AoA, the x-axis, positive in a counter-clockwise direction.

        - For ZoA, the z-axis

          - Note: θ=0 pointing to the z-axis and θ=90o pointing to the x-y plane.

        - The translation of the GCS to LCS uses the set of angles α (bearing angle), β (downtilt angle),  γ  (slant angle), which are reported together with the AoA (ϕ) and ZoA (θ) in LCS.

With respect to gNB measurements the following was agreed:

When reporting a gNB measurement (UL RTOA measurements, gNB Rx-Tx time difference, SRS RSRP, or UL AoA/ZoA), gNB can be requested to report the ID of DL PRS resource and the ID of the DL PRS resource set, or SSB Index, which is used to identify the Rx beam that is used for determining  the gNB measurements.

RAN1 made the following working assumption with respect to UE Rx-Tx time difference measurements at RAN1#98bis, that was further confirmed at RAN1#99:

A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers

  - FFS: Reporting of SRS for positioning resource/resource set ID corresponding to a UE Rx-Tx time difference measurement

  - Note: This agreement does not introduce any new behavior for the transmission of SRS for positioning.

Confirm the working assumption from RAN1#98bis on reporting of multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning. The FFS item in the working assumption is removed.

RAN1 has also agreed to exclude SS-RSRPB measurement previously recommended for E-CID

Exclude SS-RSRPB from the list of UE RRM measurement for E-CID

Include this agreement in an LS to other RAN 2/4 WGs (R1-1913522)

#### Agreements on physical-layer procedures to support UE/gNB measurements

Open loop power control was agreed for SRS for positioning:

Only open loop power control mechanism is supported. If the UE is provided a pathloss reference from the serving or a neighbouring cell in the SRS for positioning configuration, but is not able to successfully measure the pathloss for the pathloss reference provided, use a RS resource obtained from the SSB that the UE uses to obtain MIB as the pathloss reference signal.

For the purpose of power control of the SRS for positioning purposes (for all positioning solutions using SRS when DL-PRS is available), support configuring a DL-PRS (in addition to SSB which has already been agreed) of a neighbouring cell to be used as DL path loss reference.

  - Note: UE support of DL-PRS as a DL pathloss reference can be discussed as part of UE capability discussions

RAN1 has discussed number of pathloss references for SRS for positioning. The following agreement was reached:

The number of distinct pathloss references that a UE can simultaneously maintain for SRS for positioning resource sets can be smaller than the number of configured SRS for positioning resource sets.

A UE does not expect to simultaneously maintain more than N additional pathloss estimates across all SRS resource sets for positioning, in addition to the up to four pathloss estimates per serving cell currently specified for PUSCH/PUCCH/SRS transmissions.

  - FFS: Value of N

There is at most one pathloss reference per SRS resource set configured.

Note: The value of N can be decided at RAN1#99 or as part of UE capability discussions.

The following related agreements were made at RAN1#99:

A UE does not expect to simultaneously maintain more than N distinct pathloss estimates across all SRS resource sets for positioning which are different from the up to four pathloss estimates per serving cell the UE maintains for PUSCH/PUCCH and other SRS transmissions, where N can be up to 16 per UE.

The maximum value of N supported by a UE can be indicated as a UE capability and can take the values N = {0,4,8,16}

The indication of DL PRS and SSB power was agreed to support power control for SRS for positioning:

For the purpose of power control of the SRS for positioning, if a DL reference signal is to be used as DL path loss reference, the following is provided to the UE by higher layers for each required TRP (e.g., serving and neighbours):

  - If the DL reference signal to be used as DL path loss reference is a SSB, provide the ss-PBCH-BlockPower.

    - Note: The ss-PBCH-BlockPower is currently defined as average EPRE of the resource elements that carry secondary synchronization signals in dBm that the TRP use for SSB transmission (see TS 38.213, clause 7; range currently defined as INTEGER (-60..50)).

  - If the DL reference signal to be used as DL path loss reference is a DL-PRS, provide the dl-PRS-Resource-Power.

    - The dl-PRS-Resource-Power is defined as average EPRE of the resources elements that carry DL-PRS Resource signals in dBm that the TRP uses for DL-PRS Resource transmission.

  - FFS: CSI-RS transmit power indication if CSI-RS is agreed as a pathloss reference.

RAN1 has discussed assistance information and made the following agreements:

If a DL reference signal is to be used as pathloss reference for the purpose of SRS for positioning power control, or if a DL reference signal is to be used as spatial relation info for SRS for positioning, the following is provided to the UE by higher layers:

  - If the DL RS to be used is a SSB, provide the time/frequency occupancy of the SSB.

  - If the DL RS to be used is a DL-PRS, provide the the time/frequency occupancy of the DL-PRS.

  - FFS: Overhead reduction for SSB which has already been detected.

  - FFS: CSI-RS configuration if CSI-RS is agreed as a pathloss reference or spatial relation info.

For DL-PRS processing, the following SSB assistance data can be provided for an indicated SSB:

  - PCI of the cell

  - ssbFrequency with values: ARFCN-ValueNR

  - halfFrameIndex with values: 0 or 1

  - SSB-periodicity with the values: ServingCellConfigCommon IE.

  - SSB-positionInBurst with values: of ServingCellConfigCommon IE.

  - ssbSubcarrierSpacing with values: SubcarrierSpacing IE

  - SFN-SSBoffset with values {0,1,2,…15}

  - Working assumption: Smtc per SSB frequency layer with values: SSB-MTC IE

  - SSB Index

The SSB index indicated for QCL Type D and QCL Type C for a DL-PRS resource should be the same

Note: SSB frequency layer is determined by ssbFrequency and ssbSubcarrierSpacing

Note: The SSB assistance data can be provided at least when DL-PRS-QCL-Info for a DL-PRS Resource of a TRP indicates ‘QCL Type-D’ or ‘QCL Type C’, or when DL-PRS is punctured by SSB.

Note: The SSB-positionInBurst parameter is needed only for SSB puncturing.

Note: The feasibility of the working assumption above can be assessed by RAN2 and RAN3.

With respect to DL PRS quasi-collocation (QCL), the following agreements were made by RAN1:

For a DL PRS resource, QCL-TypeC from an SSB from a TRP (Option 1 from previous related agreement in RAN1#98) is supported

For positioning purposes, to assist UE to perform Rx beamforming, the following can be used (option 1 from previous related agreement in RAN1#97):

  - The DL PRS can be configured to be QCL Type D with a DL Reference Signal from a serving or neighbouring cell. SSB or DL-PRS can be the QCL Type D source of DL-PRS.

  - FFS: CSI-RS for RRM

If the DL-PRS-QCL-Info for a DL-PRS Resource of a TRP indicates ‘QCL Type-D from a DL-PRS Resource’, the DL-PRS-ResourceSetId and the DL-PRS-ResourceId for the indicated source DL-PRS Resource can be provided.

QCL relation between two DL-PRS Resources can only be provided for DL-PRS Resources of the same TRP.

Conclusion:

For positioning purposes, use of CSI-RS for RRM as a source for QCL Type D for DL-PRS, is not supported

RAN1 has discussed spatial relation of DL RS and target SRS for positioning. The following was agreed by RAN1:

For positioning purposes, for UL Beam management/alignment towards serving cell, support configuration of a spatial relation between a reference DL RS from serving cell and the target SRS for positioning. The Reference DL RS that can be used are SSB, CSI-RS (NZP-CSI-RS-ResourceId), or DL-PRS for positioning.

For positioning purposes, for UL Beam management/alignment towards neighbouring cells, support configuration of a spatial relation between a reference DL RS from neighbouring cells and the target SRS for positioning. The Reference DL RS that can be used are SSB or DL-PRS.

  - FFS: CSI-RS for RRM as the reference DL RS

For each SRS resource for positioning, only a single RS resource for spatial relation can be provided per resource.

If the spatialRelationInfo or pathlossReferenceRS indicates a DL-PRS, the following parameters of the DL-PRS are provided in the UL-SRS for positioning configuration:

  - The ID (previously agreed) that can be associated with multiple DL-PRS Resource Sets associated with a single TRP

  - DL-PRS-ResourceSetId

  - DL-PRS-ResourceId

RAN1 has also discussed spatial relation of SRS resource for positioning. The following was agreed by RAN1:

Support configuration of a spatial relation between two SRS resources for positioning.

If the spatialRelationInfo indicates an SRS resource, the following parameters for the SRS for positioning can be provided:

  - Rel-15 SRS-ResourceId or Rel-16 Positioning SRS-ResourceId

  - UL BWP ID

  - Serving cell ID

RAN1 also agreed on assistance information for UE DL PRS measurements:

An expected RSTD value together with uncertainty (search window) is provided to the UE for the TRPs in the assistance data (analogous to LTE).

  - expectedRSTD is defined as the time difference with respect to the received DL subframe timings associated with the different TRPs the target device is expected to measure.

  - Note: It is assumed that precise TRP transmission time differences are provided to the UE for UE-based positioning (which is in the scope of RAN2).

  - Note: RAN2 can optimize the signalling to cover both UE-based and UE-assisted positioning in a single framework.

The expected RSTD value is a single value defined as the RSTD the UE is expected to measure (at the UE location).

The value range of the expected RSTD is +/- 500 us.

The value range for the uncertainty of the expected RSTD is

  - When any of the resources used for the DL positioning measurement are in FR1: +/- 32 us

  - When all of the resources used for the DL positioning measurement are in FR2: +/- 8 us

Regarding transmission of SRS for positioning the following was agreed:

The SRS for positioning purposes is transmitted within the active UL BWP of the UE.

RAN1 also agreed to support request for a measurement gap configuration, to measure DL PRS outside for active BWP

RRC signalling should be introduced for a UE to request a measurement gap configuration when the UE is expected to measure the DL PRS resource outside the active DL BWP.

For the case when DL PRS Resources are processed in the active BWP and there is no measurement gap configured to the UE:

In case DL PRS Resources are processed in the active BWP and there is no measurement gap configured to the UE, at least in FR2, the UE is not expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE. Behaviour in FR1 is up to RAN4 to decide.

  - Include this agreement in an LS to RAN4.

The following agreement was made for reporting of DL PRS-RSRP measurements

When the UE reports DL-PRS RSRP measurements on DL-PRS resources from one DL-PRS Resource Set, the UE may indicate in the measurement report for each TRP which DL-PRS RSRP measurements, if any, have been measured using the same Rx beam.

Note: As previously agreed, to support Option 3 of multi-beam operation, the NW may configure DL-PRS Resources as source RS for QCL Type D for a target DL-PRS Resource. That is, Option 3 can be achieved by Option 1 with a DL-PRS as source RS for QCL Type D (Options 3/1 from previous related agreement in RAN1#97).

For SRS based positioning, the following agreements were made by RAN1 WG:

For SRS for positioning, if the spatialRelationInfo or pathlossReferenceRS indicates an SSB, the following information can be provided for the indicated SSB:

  - PCI of the cell

  - ssbFrequency with values: ARFCN-ValueNR

  - halfFrameIndex with values: 0 or 1

  - SSB-periodicity with values: ServingCellConfigCommon IE.

  - ssbSubcarrierSpacing with values: SubcarrierSpacing IE

  - SFN-SSBoffset with values: {0,1,2,…15}

  - Smtc per SSB frequency layer with values : SSB-MTC

  - SFN0 Offset per physical cell ID: Time offset of the SFN0 slot0 of a given cell with respect to the serving Pcell.

  - SSB Index

  - SS-PBCH-BlockPower (at least when SSB is used as pathlossReferenceRS for an SRS)

Note: SSB frequency layer is determined by ssbFrequency and ssbSubcarrierSpacing

Note: RAN1 assumes that the above information is indicated in RRC

Based on the work at RAN1#99, the RAN1 WG has approved the following LS endorsed in R1-1913522 (it is provided below for convenience):

1. Overall Description: RAN1 has been discussing the UE behaviour when collision between the DL PRS and other DL signals or channels occurs. RAN1 would like to inform RAN4 of the below agreements that have been reached on this topic in RAN1. Agreement: For serving TRP, UE assumes that DL PRS is not mapped to any symbol that contains SS/PBCH - Note: In a slot in which SS/PBCH is transmitted on some symbols, DL PRS can be transmitted on other symbols For neighbor TRPs, when time-frequency location for SSB transmissions on the neighbour TRP is provided, UE assumes that the DL-PRS is not mapped on symbols occupied by SSB transmissions of the neighbour TRP (i.e. DL PRS is not transmitted on these symbols) Agreement: In case DL PRS Resources are processed in the active BWP and there is no measurement gap configured to the UE, at least in FR2, the UE is not expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE. Behaviour in FR1 is up to RAN4 to decide. Additionally, RAN1 previously sent a table with measurements and reference signals to RAN2 and RAN3. RAN1 has made an update with the below agreement: Agreement: - Exclude SS-RSRPB from the list of UE RRM measurement for E-CID RAN1 also made some progress on the reporting granularity for NR positioning measurements and reports. RAN1 would like to update the other working groups about this progress which is captured in the two agreements below. Agreement: The reporting granularity for the gNB angle measurements (AoA, ZOA) is defined as 0.1 degree. RAN1 assumes that the details of the reporting granularity and ranges for the gNB angle measurements (AoA, ZoA) will be determined by RAN4. The reporting granularity for the set of angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of the GCS to LCS is defined as 0.1 degree. RAN1 assumes that the reporting granularity, the range of values and mapping for DL PRS RSRP and UL SRS RSRP measurements will be determined by RAN4 Agreement: The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0. - Note: RAN4 can determine if -1 can be a minimum value RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth. Finally, RAN1 made an agreement on the SSB assistance data related to DL PRS processing as shown below. Agreement: For DL-PRS processing, the following SSB assistance data can be provided for an indicated SSB: - PCI of the cell - ssbFrequency with values: ARFCN-ValueNR - halfFrameIndex with values: 0 or 1 - SSB-periodicity with the values: ServingCellConfigCommon IE. - SSB-positionInBurst with values: of ServingCellConfigCommon IE. - ssbSubcarrierSpacing with values: SubcarrierSpacing IE - SFN-SSBoffset with values {0,1,2,…15} - Working assumption: Smtc per SSB frequency layer with values: SSB-MTC IE - SSB Index The SSB index indicated for QCL Type D and QCL Type C for a DL-PRS resource should be the same Note: SSB frequency layer is determined by ssbFrequency and ssbSubcarrierSpacing Note: The SSB assistance data can be provided at least when DL-PRS-QCL-Info for a DL-PRS Resource of a TRP indicates ‘QCL Type-D’ or ‘QCL Type C’, or when DL-PRS is punctured by SSB. Note: The SSB-positionInBurst parameter is needed only for SSB puncturing. Note: The feasibility of the working assumption above can be assessed by RAN2 and RAN3. 2. Actions: To RAN4: ACTION: RAN1 respectfully asks RAN4 to take the above agreements into account during their work on NR positioning. RAN1 also respectfully asks RAN4 to decide the UE behaviour in FR1 for DL PRS processing when other DL signals and channels are transmitted to the UE in the active BWP without a measurement gap. To RAN2, RAN3: ACTION: RAN1 respectfully asks RAN2 and RAN3 to take into account the removal of SS-RSRPB measurement from the previously agreed list. To RAN2, RAN3, RAN4: ACTION: RAN1 respectfully asks other working groups to take into account the agreements on reporting granularity and asks RAN4 to continue their work on the details. To RAN2, RAN3: ACTION: RAN1 respectfully RAN2 and RAN3 to take into account the agreement on SSB assistance data for DL PRS processing and to assess the feasibility of the working assumption included in that agreement.

#### 2.1.2	Remaining Open issues

RAN1 has completed the work on NR Positioning in the Rel.16. There is no open issues from physical layer perspective. The objectives of WID are met.

## 2.2	RAN2

#### 2.2.1	Agreements

CR endorsement

RAN2 WG has discussed stage 2 CR on NR Positioning based on draft CR in R2-1912703, that was endorsed as a running CR based on decisions made at RAN2#107bis (refer to [107bis#78][NR/Positioning] Running stage 2 CR on NR positioning).

RAN2 WG discussed Transmission Measurement Function in NG-RAN based on R2-1913396 providing TP for 38.305 Baseline CR that was agreed it to be included in the email discussion of the 38.305 CR.

In addition, the running CR for the introduction of NR positioning to 38.331 was discussed based on R2-1913412 with a decision to continue review and endorse it by email (see also [107bis#79][NR/Positioning] Running CR to 38.331 on NR positioning).

Procedures and protocol design for support of NR RAT-dependent positioning solutions

RAN2 has extensively discussed support of NR dependent positioning methods and has reached the following agreement:

Do not extend the LTE IEs for OTDOA or E-CID to include NR measurements/AD

The high-level LPP procedures (Request/Provide Assistance Data, Location Information, Capabilities) are extended (as already agreed)

There will be one or more new methods for NR RAT-dependent positioning

For stage 2, do not group the NR RAT-dependent techniques with the existing methods

For stage 2, capture the RAT-dependent measurements and RS types

For stage 2, capture the six RAT-dependent techniques described in the RAN1 LS (R2-1912011)

It was also agreed to generate a draft CR based on LS from RAN1 regarding their agreed positioning parameters as a part of e-mail discussion ([107bis#80][NR/Positioning] Capturing RAN1 parameters for positioning)

Support of SSR phase 2 (PPP-RTK)

RAN2 WG discussed GNSS SSR assistance data and agreed to update running CR as a part of e-mail discussion ([107bis#81][NR] Update of SSR phase 2 running CRs)

Based on discussion of TP for addition of SSR Atmospheric Correction Points Area Definition (R2-1912520) RAN2 based on outcome of the offline discussion # 402 (summarized in R2-1914073) agreed to implement both a list of coordinates and a grid definition for the correction points. The corresponding TP was agreed to be incorporated into the running CR for email discussion.

Support of broadcast assistance data

RAN2 discussed support of Broadcast of Location Assistance Data by NG-RAN based on R2-1913398 with no specific conclusions/agreements reached at the RAN2#107bis.

RAN2 e-mail discussions (107bis)

The following email discussions were organized after RAN2#107bis meeting:

[107bis#78][NR/Positioning] Running stage 2 CR on NR positioning (Intel). Intended outcome: Update of the running CR to reflect decisions of RAN2#107bis Deadline:  Thursday 2019-11-07

[107bis#79][NR/Positioning] Running CR to 38.331 on NR positioning (Ericsson). Intended outcome: Endorsable running CR. Deadline:  Thursday 2019-11-07

[107bis#80][NR/Positioning] Capturing RAN1 parameters for positioning (Intel). Generate a draft CR based on any LS from RAN1 regarding their agreed positioning parameters. Intended outcome: Draft CR to next meeting. Deadline:  Thursday 2019-11-07

[107bis#81][NR] Update of SSR phase 2 running CRs (Qualcomm). Intended outcome: Draft CRs for 36.355 and 36.331 for next meeting. Deadline:  Thursday 2019-11-07

CR Review (RAN2#108)

The running stage 2 CR on positioning was discussed with the latest revision in R2-1916472. It was agreed to continue revision by email to take into account decisions made during RAN2#108 meeting.

The running CR for the introduction of NR positioning to 38.331 (R2-1915690) was reviewed with a decision to further update the running CR during email discussion taking into account decisions made during RAN2#108 meeting.

RAN2 has also briefly discussed UE capability for NR Positioning without specific conclusion made.

Support of NR RAT-dependent positioning (RAN2#108)

The LPP CR capturing RAN1 parameters for positioning was discussed with a decision to continue revision as a part of e-mail discussion ([108#85][NR/Pos] Running CR to 36.355)

Protocol design (RAN2#108):

RAN2 WG organized multiple offline discussions to discuss several topics:

Offline discussion #501, on support of NR dependent positioning methods with an outcome to be reported in R2-1916401 as part of the LPP email discussion.

Offline discussion #502, on whether to include per-beam measurements with a report in R2-1916402 that was noted.

Offline discussion #503, on whether to have one method or follow the existing LTE structure with a report in R2-1916403. It was concluded to continue to work with the running CR based on multiple methods. Competing proposals for a single method can be seen at the next meeting.

Offline discussion #504, on whether to use the measurement structure as a baseline for further discussion with a report in R2-1916404.

Additional path reporting

RAN2 WG discussed DL/UL positioning methods and agreed to have an e-mail discussion on additional path reporting with a goal to develop a text proposal if the approach is agreeable ([108#87][NR/Pos] Additional path reporting).

Combined DL/UL and multi-RTT

The following agreements were reached by RAN2 WG with respect to DL/UL positionng methods:

For Multi-RTT positioning, the DL-PRS information for the candidate TRPs are provided by an LMF to the UE in an LPP Provide Assistance Data message.

The time/frequency occupancy of the DL-PRS required in the UL-PRS (SRS) information is provided as part of the DL-PRS assistance data for Multi-RTT positioning. UL-PRS (SRS) information includes an index/pointer to the relevant information in the DL-PRS assistance data (e.g., DL-PRS Resource Set ID/Resource ID).

The time/frequency occupancy of the SSBs required in both, DL-PRS and UL-PRS is grouped in a single IE, and a pointer/index is used to reference the required information.

UL-SRS (both Rel-15 and Rel-16) for positioning is configured by RRC.

FFS if we take steps to reduce the duplicate configuration between RRC and LPP for methods involving both DL and UL measurements.

Support of SSR phase 2 (PPP-RTK)

The text proposal for adding the phase bias indicator to the SSR phase bias message was discussed. The following agreement was made:

Add a per-satellite Phase Bias Indicator field to the SSR Satellite Phase Bias message to indicate which GNSS signals support Undifferenced Integer, Widelane Integer or Non-Integer positioning modes.

In addition, offline discussion #508 to converge on text suitable for inclusion in 38.305 and 36.305 was organized aiming to prepare draft CR in R2-1916409 [38.305] and R2-1916410 [36.305]. The R2-1916410 was not provided and corresponding material is to be handled as part of the general CR for SSR.

The following running CRs were endorsed by RAN2 WG:

R2-1915560, Running LPP CR for PPP-RTK support (SSR)

R2-1915561, Running LTE RRC CR for PPP-RTK support (SSR)

R2-1916412, GNSS Integer Ambiguity Level Indications

R2-1916409, Draft CR on Phase Bias Indicator [38.305]

Content and delivery of broadcast assistance data

RAN2 WG discussed open issues of broadcast positioning assistance data and made the following agreements:

The NR UE acquires posSI(s) based on the request from positioning upper layers.

The area scope mechanism in RRC is copied into the scheduling information for posSIBs.  FFS if there is a separate area ID for positioning.

The area scope of a posSIB and the corresponding SI validity area are part of the NRPPa metadata

It is RAN node to determine the SI broadcast status for posSI transmission (broadcasting vs. notBroadcasting)

posSIB change does not trigger SI change notification

It is not needed to configure valueTag for each posSIB in SIB1

Confirm that on-demand SI request should be supported for UEs in RRC_CONNECTED

On-demand SI request is supported for positioning system information for UEs in RRC_IDLE/RRC_INACTIVE

  - A) Msg3-based SI request mechanism should be extended to support positioning SI request for UEs in RRC_IDLE/RRC_INACTIVE

  - B) Working assumption: Msg1-based SI request mechanism should be extended to support posSIBs request. RACH resource for msg1-based request mechanism can be optionally configured.

On-demand system information in connected mode

RAN2 WG has discussed dedicated positioning SIB delivery for on-demand SI in connected state and reached the following agreement:

Support broadcast or dedicated delivery of system information carrying positioning assistance data upon on-demand SI request from UE in RRC_CONNECTED

UE-based positioning

The assistance data for UE based positioning node was discussed. The following agreements were made by RAN2 WG:

The positioning measurement assistance data and position calculation assistance data are defined in separate IEs.

Include spatial direction information of the DL-PRS Resources in the position calculation assistance data (e.g., azimuth, elevation).  FFS beamwidth.

Include a transmission reference location for each DL-PRS Resource ID.  FFS the exact terminology.

  - Provide a reference location for the transmitting antenna of the reference TRP

  - Provide relative locations for transmitting antennas of other TRPs

  - ASN.1 formulation to be further discussed

Split the position calculation assistance data into two separate posSIBs, one containing the TRP coordinates and one containing the RTDs.

E-mail discussions:

The following email discussions were organized after RAN2#108 meeting:

[108#84][NR/Pos] Running stage 2 CR on positioning, Intended outcome: Agreeable CR to be submitted to RAN2#109, Deadline: 2020-01-23

[108#41][NR/Pos] Running CR to 38.331 on positioning. Update the running CR.  Final version to include the whole RRC spec with the changes. Intended outcome: Agreeable CR for merge into the large RRC CR. Deadline:  2020-01-23

[108#85][NR/Pos] Running CR to 36.355. RAN1 parameters part to be complete by 2020-01-23.  Further aspects can be developed until the next meeting deadline. Intended outcome: Agreeable CR to next meeting. Deadline:  2020-02-13

[108#86][NR/Pos] Single positioning method approach in LPP. Develop a detailed proposal for a single positioning method to compare with the multiple methods in the running CR. Intended outcome: Deadline:  2020-02-13

[108#87][NR/Pos] Additional path reporting. Discuss the proposed additional path reporting and develop a text proposal if the approach is agreeable. Intended outcome: Agreeable TP for next meeting. Deadline:  2020-02-13

[108#88][NR/Pos] Remaining issues on broadcast assistance data. Clarify the remaining issues on broadcast:

  - Per-SIB vs. per-SI request in connected mode

  - Per-SIB vs. per-SI request in idle mode

  - Need of a separate SIB for posSIB scheduling

  - “Subscription” mechanism for posSIBs

  - FFS on separate area ID for posSI

  - Unicast scope for posSIBs in SI scheduling

  - Intended outcome: Report and TPs (RRC, stage 2, and LPP if needed) to next meeting

  - Deadline: 2020-02-13

[108#89][NR/Pos] UE-based downlink positioning assistance data. Intended outcome: Agreeable CR for merge into the general LPP CR. Deadline:  2020-01-23

#### 2.2.2	Remaining Open issues

Finalize work on definition of functional interfaces, signaling and procedures including UE reporting, to support NR RAT-dependent positioning

Finalize work on extensions of LPP protocol for NR RAT-dependent positioning

Finalize work on extensions of LPP protocol to support GNSS SSR (PPP-RTK support) based on the “Compact SSR” definitions specified for QZSS

Finalize work on specification of the DL-only UE based positioning

## 2.3	RAN3

#### 2.3.1	Agreements

The following BL CRs were endorsed by RAN3 WG at RAN3#105bis:

R3-196310 Transmission Measurement Function in NG-RAN

R3-194972 Transmission Measurement Function in NG-RAN

R3-196221 (TP for BL CR TS 38.305) Transmission Measurement Function in NG-RAN

R3-194935 NR Assistance Data Delivery over NRPPa

R3-194935 NR Assistance Data Delivery over NRPPa

In addition, RAN3 made the following agreement as a part of discussion on NRPPa functionality and procedures:

A TRP is identified by a {TRP ID, cell ID} pair, where the cell ID is optional. Absence of the cell ID indicates that the TRP is not associated with a cell.

General (RAN3#106)

RAN3 WG discussed Transmission Measurement Function in NG-RAN and endorsed draft CR (R3-196508) as a baseline.

NRPPa Extensions for RAT-Dependent Positioning (RAN3#106)

RAN3 WG discussed proposal to add support of NR positioning measurements in NRPPA and endorsed the following document R3-197688.

RAN3 organized offline discussion on TRP configuration. Summary of offline discussion on TRP configuration exchange was provided in R3-197788. The document was noted and agreed to be used as a basis for future discussions.

The NRPPa functionality and procedures were discussed by RAN3 WG and the draft LS to RAN2 was agreed in R3-197794. It was noticed that in the DL-AoD positioning technique, the UE position is estimated based on multiple Angles of Departure (AoD) of DL PRS from RAN node to UE, along with knowledge of the geographical coordinates of the measured RAN nodes. The UE measurements in DL-AoD should be combined with the RAN node information in order to derive AOD the UE location at LMF. From architecture perspective; there are two options for the measurements reporting and AOD calculation were identified by RAN3 WG with a preference for Option 1:

Option 1: LMF calculates the AOD based on the UE measurements and RAN node assistance information. LMF then further calculates UE location based on the AODs.

Option 2: The RAN nodes calculate the AoD with the RSRP information based on gNB configuration and send the AoD information back to LMF, who then calculates the UE position.

Broadcast Assistance Data Delivery

The CR on introduction of NR Positioning in NRPPa was endorsed as a baseline in R3-197691. The details of NR Assistance Data Delivery over F1AP were discussed based on submitted contributions. The merged versions were endorsed as a baseline in R3-197692 and R3-197693. It was agreed to use the same structure in F1AP and in NRPPa for assistance data.

#### 2.3.2	Remaining Open issues

Finalize work on extensions of NRPPa protocol for NR positioning.

Complete work on any open aspects of architecture, location of the transmission measurement function.

In cooperation with RAN2 WG, define IE details for broadcast assistance data delivery.

## 2.4	RAN4

#### 2.4.1	Agreements

Summary of agreements from RAN4#92bis

RAN4 WG discussed and agreed skeleton for the TS 38.133.

The system level simulation assumptions were discussed and approved in [R4-1912732]. Additionally, the following agreement on timeline for the system level simulation work was agreed:

Interested companies are to provide the system simulation results for RAN4#93 based on simulation assumptions to be approved in RAN4#92bis

Side conditions for input to the link simulations (based on the system simulation results) shall be finalized in RAN4#93

Ad-hoc minutes for NR Positioning RRM work were approved in R4-1912793, where agreements for NR positioning measurement were captured.  More detailed analysis on UE positioning requirements are expected to be discussed at the next meeting, e.g. intra-frequency and inter-frequency DL PRS RSTD/RSRP measurements.

RAN4 WG also discussed the need and how to define gNB requirements with the following open questions to be further studied:

Necessity of requirements

Applicability to different BS types

Requirements types (e.g. UL RTOA, UL SRS-RSRP, gNB Rx-Tx time difference, AoA and ZoA)

RAN4 WG discussed definition of reference point for FR2 UE and gNB timing related measurement without specific conclusion reached. Further discussion is expected at RAN4#93 to provide reply LS to RAN1.

Summary of agreements in RAN4#93

In RAN4#93, the draft CR of TS38.133 specification for NR Pos was approved in [R4-1915928], triggering the further work on preparation of corresponding CRs.

RAN4 WG endorsed ad-hoc meeting minutes for NR Positioning RRM in R4-1915848 and agreed on a way-forward for NR Positioning [R4-1915854] where all agreements and several important open issues for NR Positioning measurement requirements were discussed and addressed.

RAN4 WG analysed the system level simulation results provided by companies and reached the following agreements:

Side conditions for FR1 PRS RSTD measurements

Neighbor cells: PRS Es/Iot = [-13] dB

Reference cell: PRS Es/Iot

  - Option 1: -6 dB

  - Option 2: -3 dB

Side conditions for FR2 PRS RSTD measurements

Neighbor cells: PRS Es/Iot

  - Option 1: -13 dB

  - Option 2: -10 dB

Reference cell: PRS Es/Iot

  - Option 1: -6 dB

  - Option 2: -3 dB

Additionally, RAN4 WG also discussed the link level simulation assumption for PRS RSTD that were agreed in [R4-1915802]. For FR2, only the sync case will be applicable.

The RAN4 WG also agreed to define gNB requirements by either of approaches below [R4-1915854]:

Option 1: Define gNB measurement accuracy requirements for gNB measurement types in the Perf part

  - Rx-Tx timing difference

  - UL SRS-RSRP measurements

Option 2: Define gNB measurement accuracy requirements for gNB measurement types in the Perf part

  - Rx-Tx timing difference

  - UL SRS-RSRP measurements

  - AoA / ZoA measurements

  - UL RTOA measurements

Option 3: Do not define gNB requirements

Finally, RAN4 WG achieved consensus on the reference point for RSTD measurements in FR2 and agreed that for the reference point for UE and gNB timing measurements in FR2. The reply LS to RAN1 was approved in [R4-1915801].

#### 2.4.2  Remaining Open issues

Finalize work on the necessary core requirements as well as the necessary test cases for NR positioning. Resolve remaining open aspects of RAN4 core requirements for NR positioning measurements.

Beside the core part requirements, further define the necessary performance requirements and test cases for NR positioning.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson

- TS38.211, V15.4.0, “NR: Physical channels and modulation”

- R1-1910675, “Remaining details of UL PRS design for NR positioning”, Intel Corporation, Chongqing, China, October, 2019

- R1-1910676, “Remaining details of physical layer measurements for NR positioning”, Intel Corporation, Chongqing, China, October, 2019

- R1-1910677, “Remaining details of NR positioning physical layer procedures”, Intel Corporation, Chongqing, China, October, 2019

- R1-1910678, “Initial Discussion on UE Capabilities for NR Positioning WI”, Intel Corporation, Chongqing, China, October, 2019

- R1-1911633	Draft LS on SRS for NR Positioning	Intel Corporation

- R1-1911564	Update to the list of higher layer parameters for NR positioning	Intel Corporation

- R1-1910033	DL PRS design for NR positioning	Huawei, HiSilicon

- R1-1910121	Discussion on DL Reference Signals for NR Positioning	OPPO

- R1-1910176	Discussion on DL positioning reference signal design	CMCC

- R1-1910237	Discussion on DL RS for NR positioning	vivo

- R1-1910318	DL Reference Signals for NR Positioning	CATT

- R1-1910368	Discussion on DL PRS for NR positioning	ZTE Corporation

- R1-1910501	DL Reference Signals for NR Positioning	Samsung

- R1-1910586	Discussion on DL Reference Signals for NR Positioning	LG Electronics

- R1-1910674	Remaining details of DL PRS design for NR positioning	Intel Corporation

- R1-1910754	Considerations on DL Reference Signals for NR Positioning	Sony

- R1-1910887	Views on DL reference signals for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1910988	DL Reference Signals for NR Positioning	BUPT

- R1-1911073	Downlink RS design for NR Positioning	MediaTek Inc.

- R1-1911132	DL Reference Signals for NR Positioning	Qualcomm Incorporated

- R1-1911190	DL Reference Signals for NR Positioning	NTT DOCOMO, INC.

- R1-1911199	Views on DL reference signal designs for NR positioning	Mitsubishi Electric Co.

- R1-1911228	DL reference signals for NR positioning	Ericsson

- R1-1911432	Feature Lead Summary #1 on AI 7.2.10.1 - DL Reference Signals for NR Positioning	Intel Corporation

- R1-1911541	Feature Lead Summary #2 on AI 7.2.10.1 - DL Reference Signals for NR Positioning	Intel Corporation

- R1-1911343	SRS design for NR positioning	Huawei, HiSilicon

- R1-1910122	Discussion on UL Reference Signals for NR Positioning	OPPO

- R1-1910238	Discussion on UL RS for NR positioning	vivo

- R1-1910319	UL Reference Signals for NR Positioning	CATT

- R1-1910369	Discussion on UL SRS for positioning	ZTE Corporation,Sanechips

- R1-1910502	UL Reference Signals for NR Positioning	Samsung

- R1-1910587	Discussion on UL Reference Signals for NR Positioning	LG Electronics

- R1-1910675	Remaining details of UL PRS design for NR positioning	Intel Corporation

- R1-1910755	Considerations on UL Reference Signals for NR Positioning	Sony

- R1-1910888	Views on UL reference signals for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1911074	Uplink RS design for NR Positioning	MediaTek Inc.

- R1-1911133	UL Reference Signals for NR Positioning	Qualcomm Incorporated

- R1-1911201	Views on UL reference signal designs for NR positioning	Mitsubishi Electric Co.

- R1-1911207	Remaining Details on Uplink PRS Signal Design	Fraunhofer IIS, Fraunhofer HHI

- R1-1911229	UL reference signals for NR positioning	Ericsson

- R1-1911444	FL summary for 7.2.10.2 UL Reference Signals for NR Positioning	Ericsson

- R1-1911567	Offline notes #2 for 7.2.10.2 UL Reference Signals for NR Positioning	Ericsson

- R1-1911640	Offline notes #3 for 7.2.10.2 UL Reference Signals for NR Positioning	Ericsson

- R1-1910321	FL summary for 7.2.10.3 UE and gNB measurements for NR Positioning	CATT

- R1-1910035	NR positioning measurements	Huawei, HiSilicon

- R1-1910123	Discussion on Measurements for NR Positioning	OPPO

- R1-1910239	Discussion on UE and gNB measurements for NR positioning	vivo

- R1-1910320	UE and gNB measurements for NR Positioning	CATT

- R1-1910370	Discussion on measurements for NR positioning	ZTE Corporation

- R1-1910503	Discussion on UE and gNB measurements for NR Positioning	Samsung

- R1-1910588	Discussion on UE and gNB measurements for NR Positioning	LG Electronics

- R1-1910676	Remaining details of physical layer measurements for NR positioning	Intel Corporation

- R1-1910694	Remaining Issues for NR Positioning Measurements	Futurewei

- R1-1910756	UE measurements for NR Positioning	Sony

- R1-1910889	Measurements for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1911075	Measurement and reporting design for NR Positioning	MediaTek Inc.

- R1-1911134	UE and gNB measurements for NR Positioning	Qualcomm Incorporated

- R1-1911230	UE and gNB measurements for NR positioning	Ericsson

- R1-1911531	Summary #2 of UE and gNB measurements for NR Positioning	CATT

- R1-1910036	Physical layer procedure for NR positioning	Huawei, HiSilicon

- R1-1910124	Discussion on Physical-layer procedures for NR Positioning	OPPO

- R1-1910177	Discussion on physical-layer procedures to support NR positioning measurements	CMCC

- R1-1910240	Discussion on physical-layer procedures for UE/gNB measurements	vivo

- R1-1910322	Physical-layer procedures to support UE/gNB measurements	CATT

- R1-1910371	Discussion on physical layer procedure for NR positioning	ZTE Corporation

- R1-1910504	Discussion on necessity and details for physical-layer procedures to support UE/gNB measurements		Samsung

- R1-1910589	Discussion on necessity and details for physical-layer procedures to support UE/gNB measurements		LG Electronics

- R1-1910677	Remaining details of NR positioning physical layer procedures	Intel Corporation

- R1-1910695	Remaining Issues for NR Positioning Procedures	Futurewei

- R1-1910757	Physical-layer procedures to support NR positioning	Sony

- R1-1910890	Views on Physical Layer Procedures for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1911076	Procedure design for NR Positioning	MediaTek Inc.

- R1-1911135	Considerations on Phy-layer procedures for NR Positioning	Qualcomm Incorporated

- R1-1911200	Views on physical-layer procedures for NR positioning	Mitsubishi Electric Co.

- R1-1911231	Physical Layer procedures in NR positioning	Ericsson

- R1-1911416	Summary of 7.2.10.4: PHY procedures for positioning measurements	Qualcomm Incorporated

- R1-1911627	Summary #2 of 7.2.10.4: PHY procedures for positioning measurements	Qualcomm

- R1-1910310	Discussion on positioning in idle/inactive states	vivo

- R1-1910323	Network synchronization based on RAT-dependent signals	CATT

- R1-1910393	Angle resolution and beam configuration related procedures for NR positioning	Huawei, HiSilicon

- R1-1910678	Initial Discussion on UE Capabilities for NR Positioning WI	Intel Corporation

- R1-1911232	On reusing the TRS for positioning purpose	Ericsson

- R1-1913209	Introduction of NR positioning support	Ericsson

- R1-1913210	Introduction of NR positioning support	Nokia

- R1-1913211	Introduction of NR positioning support	Intel Corporation

- R1-1913492	[DRAFT] LS on DL PRS processing with other DL signals and channels	Nokia

- R1-1913522 with the following modifications

- R1-1911847	Discussion on DL Reference Signals for NR Positioning	OPPO

- R1-1911896	Remaining issues on DL PRS for NR positioning	Huawei, HiSilicon

- R1-1911936	Discussion on DL PRS for NR positioning	ZTE

- R1-1912044	Remaining issues on DL RS design for NR positioning	vivo

- R1-1912112	Downlink RS design for NR positioning	MediaTek Inc.

- R1-1912137	DL Reference Signals for NR Positioning	BUPT

- R1-1912144	DL Reference Signals for NR Positioning	CATT

- R1-1912228	Resolution of remaining opens for NR positioning DL PRS design	Intel Corporation

- R1-1912272	Discussion on DL Reference Signals for NR Positioning	LG Electronics

- R1-1912292	Views on DL reference signals for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1912360	Remaining issues on DL PRS design for NR Positioning	Sony

- R1-1912490	DL Reference Signals for NR Positioning	Samsung

- R1-1912548	Discussion on DL PRS design	CMCC

- R1-1912836	Views on DL reference signal designs for NR positioning	Mitsubishi Electric Corp

- R1-1912973	Remaining Details on DL Reference Signals for NR Positioning	Qualcomm Incorporated

- R1-1913135	DL Reference Signals for NR Positioning	Ericsson

- R1-1913285   Feature Lead Summary #1 on AI 7.2.10.1 - DL Reference Signals for NR Positioning	Intel Corporation

- R1-1913473   Feature Lead Summary #2 on AI 7.2.10.1 - DL Reference Signals for NR Positioning	Intel Corporation

- R1-1911848	Discussion on UL Reference Signals for NR Positioning	OPPO

- R1-1911897	Remaining issues on SRS for NR positioning	Huawei, HiSilicon

- R1-1911937	Discussion on UL SRS for positioning	ZTE

- R1-1912045	Remaining issues on  UL RS design for NR positioning	vivo

- R1-1912113	Uplink RS design for NR positioning	MediaTek Inc.

- R1-1912145	UL Reference Signals for NR Positioning	CATT

- R1-1912229	Resolution of remaining opens for NR positioning UL PRS design	Intel Corporation

- R1-1912273	Discussion on UL Reference Signals for NR Positioning	LG Electronics

- R1-1912293	Views on UL reference signals for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1912361	Remaining issues on UL PRS design for NR Positioning	Sony

- R1-1912491	UL Reference Signals for NR Positioning	Samsung

- R1-1912837	Views on UL reference signal designs for NR positioning		Mitsubishi Electric Corp

- R1-1912974	Remaining Details on UL Reference Signals for NR Positioning		Qualcomm Incorporated

- R1-1913101	Remaining details on SRS enhancements for positioning	Fraunhofer IIS, Fraunhofer HHI

- R1-1913136	UL Reference Signals for NR Positioning	Ericsson

- R1-1913435	Session notes on offline discussion 7.2.10.2 UL Reference Signals for NR Positioning	Ericsson

- R1-1913507	Session notes #2 on offline discussion 7.2.10.2 UL Reference Signals for NR Positioning	Ericsson

- R1-1912147	FL summary for 7.2.10.3 UE and gNB measurements for NR Positioning	CATT

- R1-1911849	Discussion on Measurements for NR Positioning	OPPO

- R1-1911898	Remaining issues on NR positioning measurements	Huawei, HiSilicon

- R1-1911938	Discussion on measurements for NR positioning	ZTE

- R1-1912046	Remaining issues on UE and gNB measurements for NR positioning	vivo

- R1-1912114	Measurement and reporting design for NR positioning	MediaTek Inc.

- R1-1912146	UE and gNB measurements for NR Positioning	CATT

- R1-1912230	Remaining details of physical layer measurements for NR positioning	Intel Corporation

- R1-1912274	Discussion on UE and gNB measurements for NR Positioning	LG Electronics

- R1-1912294	Measurements for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1912362	Remaining issues on UE measurements for NR Positioning	Sony

- R1-1912423	Remaining Issues for NR Positioning Measurements	Futurewei

- R1-1912492	Discussion on UE and gNB measurements for NR Positioning	Samsung

- R1-1912975	Remaining Details on UE and gNB measurements for NR Positioning	Qualcomm Incorporated

- R1-1913137	UE and gNB measurements for NR Positioning	Ericsson

- R1-1913447	Summary #2 of UE and gNB measurements for NR Positioning	CATT

- R1-1911850	Discussion on Physical-layer procedures for NR Positioning	OPPO

- R1-1911899	Remaining issues on physical layer procedures to support positioning measurements	Huawei, HiSilicon

- R1-1911939	Discussion on physical layer procedure for NR positioning	ZTE

- R1-1912047	Remaining issues on on physical-layer procedures for NR positioning	vivo

- R1-1912115	Procedure design for NR positioning	MediaTek Inc.

- R1-1912148	Physical-layer procedures to support UE/gNB measurements	CATT

- R1-1912231	Remaining opens of physical layer procedures for NR positioning	Intel Corporation

- R1-1912275	Discussion on necessity and details for physical-layer procedures to support UE/gNB measurements		LG Electronics

- R1-1912295	Views on Physical Layer Procedures for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1912363	Remaining issues on NR Positioning Physical Layer Procedures	Sony

- R1-1912424	Remaining Issues for NR Positioning Procedures	Futurewei

- R1-1912493	Discussion on necessity and details for physical-layer procedures to support UE/gNB measurements		Samsung

- R1-1912549	Discussion on physical-layer procedures to support NR positioning measurements	CMCC

- R1-1912976	Remaining Details on Phy-layer procedures for NR Positioning	Qualcomm Incorporated

- R1-1913138	Procedures for NR Positioning	Ericsson

- R1-1913325	Summary #2 of 7.2.10.4: PHY procedures for positioning measurements	Qualcomm Incorporated

- R1-1913504	Summary #3 of 7.2.10.4: PHY procedures for positioning measurements	Qualcomm Incorporated

- R1-1911900	Signaling between NG-RAN and 5GC to support NR positioning	Huawei, HiSilicon

- R1-1912048	Discussion on UE Capabilities for NR Positioning	vivo

- R1-1913139	DL PRS optimized for indoor industrial scenarios	Ericsson

- R2-1912011	LS on DL/UL Reference Signals and Measurements for NR Positioning (R1-1909796; Intel)

- R2-1912703	Running stage 2 CR on NR positioning 107#69	Intel Corporation, ESA

- R2-1913396	TP for 38.305 Baseline CR: Transmission Measurement Function in NG-RAN	Qualcomm Incorporated

- R2-1913412	Running CR for the introduction of NR positioning	    Ericsson

- R2-1912200	Discussion on procedures in RAT-Dependent positioning methods	CATT

- R2-1912704	Support of NR dependent positioning methods	Intel Corporation

- R2-1913423	RAT dependent positioning methods - architecture and protocol aspects	Ericsson

- R2-1913365	Discussion on RAT dependent NR Positioning techniques	Sony

- R2-1913395	DL and UL NR Positioning Procedures	Qualcomm Incorporated

- R2-1913037	Consideration on DL positioning method in NR	Huawei, HiSilicon

- R2-1912080	Discussion on DL-TDOA positioning	ZTE Corporation

- R2-1912534	Consideration of beam for NR OTDOA	Samsung R&D Institute UK

- R2-1913593	Angle of Departure UE positioning technique	Nokia, Nokia Shanghai Bell

- R2-1912081	Discussion on UL-TDOA positioning in NR	ZTE Corporation

- R2-1913039	Consideration on UL positioning method in NR	Huawei, HiSilicon

- R2-1913422	Stage-3 structure details for RAT-dependent NR positioning	Ericsson

- R2-1913041	Consideration on E-CID in NR	Huawei, HiSilicon

- R2-1912533	considerations for NR ECID	Samsung R&D Institute UK

- R2-1912082	Discussion on multi-cell E-CID in NR	ZTE Corporation

- R2-1913040	Consideration on Multi-RTT positioning in NR	Huawei, HiSilicon

- R2-1913278	On PRS Information for NR Positioning	LG Electronics

- R2-1913594	On-demand and dynamic PRS configuration for DL-TDOA	Nokia, Nokia Shanghai Bell R2-1907657

- R2-1913397	GNSS SSR Assistance Data	Qualcomm Incorporated

- R2-1912519	SSR Atmospheric Correction Points Area Definition 	u-blox AG

- R2-1912520	Text Proposal for addition of SSR Atmospheric Correction Points Area Definition	u-blox AG	discussion	Rel-16

- R2-1914073	[RAN2#107bis Offline-402] Converge on details of SSR Correction Points definition	u-blox AG

- R2-1912325	Adding the Phase Bias Indicator to the SSR Phase Bias message	Swift Navigation

- R2-1913424	GNSS Integer Ambiguity Level Indications	Ericsson

- R2-1913415	Introduction of additional posSIBs for QZSS PPP-RTK Phase 2	Ericsson

- R2-1913398	Broadcast of Location Assistance Data by NG-RAN	Qualcomm Incorporated

- R2-1913038	Discussion on broadcasting of positioning assistance data	Huawei, HiSilicon

- R2-1912201	Considerations on Broadcast Positioning Assistance Data	CATT

- R2-1913410	New SIB for hosting posSI Scheduling Information		Ericsson

- R2-1912705	Assistance data for UE based DL only positioning	Intel Corporation	R2-1909407

- R2-1913399	Assistance Data for DL-only UE-based mode	Qualcomm Incorporated

- R2-1913416	UE-based configuration options		Ericsson

- R2-1913228	NR UE-based Positioning Modes and Geographical Information	ITRI

- R2-1914310	LS on SRS for NR Positioning (R1-1911634; contact: Intel)	RAN1

- R2-1914727	Running stage 2 CR on positioning ([107bis#78][NR Pos] )	Intel Corporation, ESA 1916472

- R2-1916472	Running stage 2 CR on positioning ([107bis#78][NR Pos] )	Intel Corporation, ESA (R2-19147270

- R2-1915690	Running CR for the introduction of NR positioning	Ericsson

- R2-1914731	UE capability on positioning	Intel Corporation

- R2-1914729	Support of NR dependent positioning methods	Intel Corporation	R2-1916401	Report of offline #501	Intel

- R2-1915652	LPP, RRC and UE Rx-Tx Aspects of NR RAT dependent positioning	Ericsson

- R2-1916402	Report of offline #502	Ericsson

- R2-1916403	Report of offline #503	CATT

- R2-1916404	Report of offline #504	Ericsson

- R2-1914468	Signaling design for RAT-Dependent positioning methods	CATT

- R2-1915171	NR RAT-Dependent Positioning Procedures	LG Electronics Inc.

- R2-1915238	Discussion on different RAT dependent NR Positioning techniques	Sony

- R2-1914978	Discussion on sigalling design for NR DL positioning procedures	Huawei, HiSilico

- R2-1915651	Additional path reporting for downlink NR positioning measurements	Ericsson, T-Mobile, Nokia, Nokia Shanghai Bell, Deutsche Telekom	discussion

- R2-1916487	Additional path reporting for downlink NR positioning measurements	Ericsson, T-Mobile, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Polaris Wireless	discussion	R2-1915651

- R2-1916049	Consideration of beam for NR OTDOA	Samsung R&D Institute UK	discussion	R2-1912534

- R2-1916105	Angle of Departure UE positioning technique	Nokia, Nokia Shanghai Bell

- R2-1914980	Further considerations on DL procedures for NR positioning	Huawei, HiSilicon

- R2-1914979	Considerations on UL procedures for NR positioning	Huawei, HiSilicon

- R2-1914566	Modification to Multi-cell power control	ZTE Corporation

- R2-1915558	Stage 2 for Multi-RTT positioning	Qualcomm Incorporated

- R2-1915560	Running LPP CR for PPP-RTK support (SSR)	Qualcomm Incorporated

- R2-1915561	Running LTE RRC CR for PPP-RTK support (SSR)	Qualcomm Incorporated

- R2-1914989	Text Proposal for adding the Phase Bias Indicator to the SSR Phase Bias message	Swift Navigation, Ericsson

- R2-1916409	Draft CR on Phase Bias Indicator [38.305]	Swift Navigation

- R2-1916410	Draft CR on Phase Bias Indicator [36.305]	Swift Navigation

- R2-1915654	GNSS Integer Ambiguity Level Indications	Ericsson, Swift Navigation,ESA

- R2-1916411	GNSS Integer Ambiguity Level Indications	Ericsson

- R2-1916412	GNSS Integer Ambiguity Level Indications	Ericsson

- R2-1914470	Open Issues of Broadcast Positioning Assistance data	CATT

- R2-1914981	Discussion on broadcasting of positioning assistance data	Huawei, HiSilicon

- R2-1915647	New SIB for hosting posSI Scheduling Information	Ericsson

- R2-1915562	Remaining details on broadcast assistance data delivery	Qualcomm Incorporated

- R2-1916107	Dedicated positioning SIB delivery for on-demand SI in connected	Nokia, Nokia Shanghai Bell

- R2-1915780	On-demand system information and dedicated signalling	MediaTek Inc., Ericsson

- R2-1915936	Positioning SIB Acquisition in Connected Mode	Apple

- R2-1915656	On Demand Delivery of Positioning Assistance Data	Ericsson

- R2-1914469	Broadcast Positioning Procedure in Connected Mode	CATT Revised in R2-1916478

- R2-1916478	Broadcast Positioning Procedure in Connected Mode	CATT

- R2-1914982	Discussion on demand SI in connected mode for positioning	Huawei, HiSilicon

- R2-1915563	Assistance Data for DL-only UE-based mode	Qualcomm Incorporated

- R2-1915658	UE-based configuration options	Ericsson

- R2-1914730	Assistance data for UE based DL only positioning	Intel Corporation

- R2-1916091	Remaining issue on UE-based positioning in NR	ITRI

- R3-194935	NR Assistance Data Delivery over NRPPa	Ericsson

- R3-194972	Transmission Measurement Function in NG-RAN	Ericsson

- R3-195371	Discussion on TMF	ZTE Corporation

- R3-195391	Discussion on NRPPa procedures	CATT

- R3-195392	(TP for [NR_POS-Core] BL CR for TS 38.455): NRPPa procedures	CATT

- R3-195506	"(TP for NR_POS BL CR for TS 38.455) 	Discussion of NRPPa issues for 	Broadcast Assistance Data Delivery"	Qualcomm Incorporated

- R3-195507	Discussion of F1AP issues for Broadcast Assistance Data Delivery	Qualcomm Incorporated

- R3-195508	NRPPa functionality and procedures	Qualcomm Incorporated

- R3-195509	F1AP Functionality and Procedures	Qualcomm Incorporated

- R3-195593	Discussion on DL-AoD procedure	Huawei

- R3-195594	LS on DL-AOD procedure	Huawei

- R3-195595	(TP for BL CR TS 38.305) Transmission Measurement Function in NG-RAN	Huawei

- R3-195596	Positioning architecture	Huawei

- R3-195597	Discussion on Broadcast assistant data delivery	Huawei

- R3-195598	(TP BL CR 38.455) NR Assistance Data Delivery over NRPPa	Huawei

- R3-195599	Broadcast assistant data delivery	Huawei

- R3-195600	Broadcast assistant data delivery	Huawei

- R3-195601	F1 support for positioning	Huawei

- R3-195602	F1 support for positioning	Huawei

- R3-195603	F1 support for positioning	Huawei

- R3-195604	F1 support for positioning	Huawei

- R3-195659	Positioning Measurement Functions	Ericsson

- R3-195661	Support for Positioning Measurements in F1AP	Ericsson

- R3-195861	How to Support Positioning Measurements in F1AP	Ericsson

- R3-196221	(TP for BL CR TS 38.305) Transmission Measurement Function in NG-RAN	Huawei

- R3-196222	Summary of Offline on Broadcast Assistance Data over F1	Qualcomm

- R3-196310	Transmission Measurement Function in NG-RAN	Ericsson

- R3-196508 Transmission Measurement Function in NG-RAN (Ericsson)

- R3-196403 LS on SRS for NR Positioning (3GPP RAN1, Intel)

- R3-196613 Discussion on NR Rat-dependent positioning (ZTE Corporation)

- R3-196743 Discussion on NRPPa procedures (CATT, Huawei)

- R3-196744 (TP for [NR_POS-Core] BL CR for TS 38.455): NRPPa procedures (CATT, Huawei)

- R3-197118 Introducing NRPPa location procedures (Intel Corporation)

- R3-197145 BL CR for TS 38.455: introduction of NRPPa procedures (Intel Corporation)

- R3-197250 NRPPa functionality and procedures (Qualcomm Incorporated)

- R3-197251 Introduction of transmission and measurement functionality in NRPPa (Qualcomm Incorporated)

- R3-197278 Support for UL Positioning Measurements in NG-RAN (Ericsson LM)

- R3-196835 Discussion on NRPPa functionality and procedures view (with LS out) and update of BL CR (Huawei)

- R3-196475 NR Assistance Data Delivery over NRPPa (Ericsson)

- R3-196816 Discussion of F1AP issues for Broadcast Assistance Data Delivery (Qualcomm Incorporated)

- R3-196815 (TP for NR_POS BL CR for TS 38.455) 	Discussion of NRPPa issues for Broadcast Assistance Data Delivery (Qualcomm Incorporated)

- R3-196817 NR Assistance Data Delivery over F1AP (Qualcomm Incorporated)

- R3-196836 Discussion on broadcast assistance data delivery (with TP and CR) (Huawei)

- R3-196837 Positioning assistance information delivery (Huawei)

- R3-197315 NR positioning Assistance Data Delivery over F1AP (Ericsson)

- R3-197316 NR positioning Assistance Data Delivery over F1AP (Ericsson)

- R3-196838 Discussion on F1 support for positioning (Huawei)

- R3-197280 F1AP Functionality and Procedures (QUALCOMM Incorporated)

- R3-197283 How to Support Positioning Measurements in F1AP (Ericsson)

- R3-197285 Positioning Measurement Functions (Ericsson)

- R3-197287 Support for Positioning Measurements in F1AP (Ericsson)

- R4-1910922	System level Simulation Results for PRS-RSTD and PRS-RSRP Core Requirement	MediaTek inc.

- R4-1910923	Discussion on FR2 RSTD reference point	MediaTek inc.

- R4-1910924	Discussion on PRS measurement without gaps and differential RSRP consideration for test cases	MediaTek inc.

- R4-1910964	Initial system simulation results for RSTD measurement	CATT

- R4-1910965	Link-level simulation Assumptions for RSTD	CATT

- R4-1910966	Discussion on RSTD Measurement Report Mapping	CATT

- R4-1910967	Discussion on UE Rx-Tx Time Difference Measurement Report Mapping	CATT

- R4-1910968	Discussion on UE Rx-Tx Time Difference Measurement delay	CATT

- R4-1910975	Further discussion on reference point for timing related measurements in FR2	CATT

- R4-1910976	Response LS on Reference Point for Timing Related Measurements in FR2	CATT

- R4-1910977	Impact of positioning measurement on RRM measurement	CATT

- R4-1910978	Discussion on measurement requirements for RSTD	CATT

- R4-1911035	Discussion on specification structure for NR positioning RRM requirements	Intel Corporation

- R4-1911036	Draft CR on TS 38.133 structure for NR positioning RRM requirements	Intel Corporation

- R4-1911037	Discussion on NR DL PRS RSTD requirements for UE	Intel Corporation

- R4-1911038	Discussion on gNB measurement requirements for NR positioning	Intel Corporation

- R4-1911039	Updates on RAN4 work scope for NR positioning	Intel Corporation

- R4-1911040	On Reference Point for Timing Related Measurements	Intel Corporation

- R4-1911753	System-level evaluations for PRS-RSTD and PRS-RSRP	Intel Corporation

- R4-1911979	Discussion on PRS measurement	Huawei, HiSilicon

- R4-1911980	Discussion on reference point for timing related measurement	Huawei, HiSilicon

- R4-1911981	[draft] reply LS on Reference Point for Timing Related Measurements	Huawei, HiSilicon

- R4-1911982	Initial system level simulation results for NR positioning	Huawei, HiSilicon

- R4-1912075	On reference point for UE timing related measurements	Ericsson

- R4-1912076	On reference point for base station timing related measurements	Ericsson

- R4-1912077	Response LS on reference point for UE timing related measurements	Ericsson

- R4-1912078	Response LS on reference point for base station timing related measurements	Ericsson

- R4-1912079	System-level simulation results for NR RSTD	Ericsson

- R4-1912080	UE measurements for NR positioning	Ericsson

- R4-1912346	On DL PRS RSRP measurements for NR positioning	Qualcomm Incorporated

- R4-1912347	On DL PRS RSTD measurements in NR positioning	Qualcomm Incorporated

- R4-1912352	On necessity of gNB requirements for NR positioning	Qualcomm Incorporated

- R4-1912356	On reference point for timing related measurements in FR2	Qualcomm Incorporated

- R4-1912361	On UE Rx-Tx time difference measurements for NR positioning	Qualcomm Incorporated

- R4-1912362	On UE-based positioning performance requirements	Qualcomm Incorporated

- R4-1912365	system level simulation results for DL PRS side conditions	Qualcomm Incorporated

- R4-1912366	On impact of PRS on existing RRM requirements	Qualcomm Incorporated

- R4-1912415	On the scope of RRM requirements for NR positioning	Nokia, Nokia Shanghai Bell

- R4-1912416	System-level simulation results for RSTD in NR	Nokia, Nokia Shanghai Bell

- R4-1912452	Views on gNB measurement for NR positioning	NTT DOCOMO, INC.

- R4-1912731	WF on gNB RRM requirements for NR positioning	Intel, Qualcomm, NTT Docomo, Apple, MTK, CATT,CMCC

- R4-1912732	WF on simulation assumptions for system-level evaluations for PRS-RSTD and PRS-RSRP	Ericsson

- R4-1912733	Ad-hoc minutes for NR Positioning RRM	Ericsson

- R4-1912793	Ad-hoc minutes for NR Positioning RRM	Ericsson

- R4-1912836	WF on gNB RRM requirements for NR positioning	Intel, Qualcomm, NTT Docomo, Apple, MTK, CATT,CMCC

- R4-1913262	SLS results for PRS-RSTD & PRS-RSRP measurement	MediaTek inc.

- R4-1913263	Discussion on FR2 RSTD reference point	MediaTek inc.

- R4-1913264	Discussion on PRS-RSTD measurement	MediaTek inc.

- R4-1913265	Discussion on PRS-RSRP measurement	MediaTek inc.

- R4-1913266	Discussion on cell-phase synchronization between gNBs	MediaTek inc.

- R4-1913389	Discussion on reference point for UE timing related measurement	Huawei, HiSilicon

- R4-1913390	Discussion on reference point for gNB timing related measurement	Huawei, HiSilicon

- R4-1913391	[draft] reply LS on Reference Point for Timing Related Measurements	Huawei, HiSilicon

- R4-1913460	Discussion on NR UE PRS RSTD measurement requirements	Intel Corporation

- R4-1913461	Link-level simulation assumptions for PRS RSTD measurement	Intel Corporation

- R4-1913462	Discussion on NR UE RX-TX time difference measurement requirements	Intel Corporation

- R4-1913463	Considerations on gNB measurement requirements for NR positioning	Intel Corporation

- R4-1913464	WF on gNB requirements for NR positioning	Intel Corporation

- R4-1913465	On reference point for timing related measurements	Intel Corporation

- R4-1913487	System-level evaluations for PRS-RSTD and PRS-RSRP	Intel Corporation

- R4-1913705	Impact of positioning measurement on RRM measurement	CATT

- R4-1913706	Discussion on reference point for gNB timing related measurements	CATT

- R4-1913707	Response LS on Reference Point for Timing Related Measurements in FR2	CATT

- R4-1913717	Further discussion on reference point for timing related measurements in FR2	CATT

- R4-1913718	Response LS on Reference Point for UE Timing Measurements	CATT

- R4-1913719	Discussion on RSTD Measurement Report Mapping	CATT

- R4-1913720	Discussion on UE Rx-Tx time difference report mapping	CATT

- R4-1913721	Link-level simulation Assumptions for RSTD measurement	CATT

- R4-1913722	Discussion on frequency layer and measurement gap	CATT

- R4-1914104	Views on gNB measurement for NR positioning	NTT DOCOMO, INC.

- R4-1914953	Discussion on gNB requirements for NR positioning	Huawei, HiSilicon

- R4-1914954	Discussion on RSTD measurement	Huawei, HiSilicon

- R4-1914955	Discussion on PRS-RSRP measurement	Huawei, HiSilicon

- R4-1914956	Discussion on Rx-Tx time difference measurement	Huawei, HiSilicon

- R4-1914957	Further simulation results for NR positioning	Huawei, HiSilicon

- R4-1915171	On gNB measurement requirements for NR positioning	Nokia, Nokia Shanghai Bell

- R4-1915172	System-level simulation results for RSTD in NR	Nokia, Nokia Shanghai Bell

- R4-1915177	On DL PRS RSRP measurements for NR positioning	Qualcomm Incorporated

- R4-1915178	On DL PRS RSTD measurements in NR positioning	Qualcomm Incorporated

- R4-1915183	On gNB requirements for NR positioning	Qualcomm Incorporated

- R4-1915186	On reference point for timing related measurements in FR2	Qualcomm Incorporated

- R4-1915191	On UE Rx-Tx time difference measurements for NR positioning	Qualcomm Incorporated

- R4-1915192	On UE-based positioning performance requirements	Qualcomm Incorporated

- R4-1915193	system level simulation results for DL PRS side conditions	Qualcomm Incorporated

- R4-1915194	On impact of PRS on existing RRM requirements	Qualcomm Incorporated

- R4-1915254	On reference point for UE timing related measurements	Ericsson

- R4-1915255	Response LS on reference point for UE timing related measurements	Ericsson

- R4-1915256	Updated system-level simulation results for PRS RSTD and RSRP	Ericsson

- R4-1915257	Link-level simulation assumptions for PRS RSTD and RSRP	Ericsson

- R4-1915258	UE measurements for OTDOA	Ericsson

- R4-1915259	UE measurements for multi-RTT	Ericsson

- R4-1915260	UE measurements for E-CID	Ericsson

- R4-1915318	On reference point for base station timing related measurements	Ericsson LM

- R4-1915336	Draft Response LS on reference point for base station timing related measurements	Ericsson LM

- R4-1915420	System-level simulation results for RSTD in NR	Nokia, Nokia Shanghai Bell

- R4-1915801	Response LS on Reference Point for Timing Related Measurements in FR2	CATT

- R4-1915802	Link-level simulation assumptions for PRS RSTD and RSRP	Ericsson

- R4-1915848	Ad-hoc minutes for NR Positioning RRM	Ericsson

- R4-1915854	Way forward on NR Positioning RRM	Ericsson

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template22222