3GPP TSG-RAN WG1 Meeting #98b	R1-1910736

# Chongqing, China, October 14th – 20th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1274 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to postponing in subframes that are not BL/CE subframes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification states that downlink (uplink) transmissions are postponed in “non-BL/CE DL (UL)” subframes. The use of “non-BL/CE DL (UL)” subframes is unclear, and inconsistent in different parts of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Avoid usage of “non-BL/CE DL (UL) subframes”. Instead, use the terminology “subframes that are not BL/CE DL (UL) subframes” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.7.1, 8.6.1, 10.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.211 CR 0495 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.1.7.1	Modulation order and redundancy version determination

<Unchanged parts are omitted>

For BL/CE UEs, the same redundancy version is applied to PDSCH transmitted in a given block of  consecutive subframes, if the PDSCH is not carrying SystemInformationBlockType1-BR or SI message. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies , where  for FDD and  for TDD. Denote  as the subframe number of the first downlink subframe intended for PDSCH, given by n+x as defined in subclause 7.1.11. The PDSCH transmission spans  consecutive subframes including subframes that are not BL/CE DL subframes where the PDSCH transmission is postponed. For the  block of consecutive subframes within the set of  subframes, the redundancy version (rvidx) is determined according to Table 7.1.7.1-2 using , where  and . The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA,  and  is determined by the 'Redundancy version' field in DCI format 6-1A. For a BL/CE UE configured with CEModeB, or a BL/CE UE receiving PDSCH associated with P-RNTI,  for FDD and  for TDD, and .

<Unchanged parts are omitted>

### 8.6.1	Modulation order and redundancy version determination

<Unchanged parts are omitted>

For a BL/CE UE or for UEs configured with higher layer parameter PUSCH-EnhancementsConfig,

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment is using uplink resource allocation type 5, the redundancy version (rvidx) to use for the i-th BL/CE UL subframe in the physical uplink shared channel is determined according to Table 7.1.7.1-2 using  where i=0, 1, …, N-1, and N is the number of BL/CE UL subframes for the PUSCH transmission as determined in subclause 8.0. For a BL/CE UE configured in CEModeA,   is determined by the 'Redundancy version' field in DCI format 6-0A. For a BL/CE UE configured with CEModeB, .

-	otherwise, the same redundancy version is applied to PUSCH transmitted in a given block of  consecutive subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH. For BL/CE UEs, the PUSCH transmission spans  consecutive subframes including subframes that are notBL/CE UL subframes where the PUSCH transmission is postponed. For the  block of consecutive subframes within the set of  subframes, the redundancy version (rvidx) is determined according to Table 7.1.7.1-2 using , where , and . The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA,  and  is determined by the 'Redundancy version' field in DCI format 6-0A. For a BL/CE UE configured with CEModeB,  for FDD and  for TDD, and . For a UE configured with higher layer parameter PUSCH-EnhancementsConfig,  and  is determined by the 'Redundancy version' field in DCI format 0C. For UEs configured with higher layer parameter PUSCH-EnhancementsConfig, NabsPUSCH=NrepPUSCH.

<Unchanged parts are omitted>

### 10.1.5	Scheduling Request (SR) procedure

<Unchanged parts are omitted>

For a BL/CE UE, the scheduling request shall be transmitted on the PUCCH resource(s)  mapped to antenna port as defined in [3], where  is configured by higher layers. The SR configuration for SR transmission periodicity and SR subframe offset  is defined in Table 10.1.5-1 by the parameter sr-ConfigIndex  given by higher layers. The SR transmission instances are  consecutive BL/CE uplink subframes when >1, or one uplink subframe (which may or may not be a BL/CE UL subframe) when =1, where  is provided by higher layer parameter pucch-NumRepetitionCE-format1, starting from a subframe satisfying .

<Unchanged parts are omitted>