3GPP TSG RAN WG1 Meeting #98bis	R1-1911463

# Chongqing, China, 14th – 20th October, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.202 | CR | 0008 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on simultaneous transmission of UL channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The combination of simultaneous PUCCH and PUSCH transmissions is missing in TS38.202. In TS38.213, UE procedure for reporting uplink control information, including multiplexing/dropping of overlapped PUCCH(s) and PUSCH(s), is applied per PUCCH group. It implies that the UE is allowed to simultaneously transmit PUCCH in a PUCCH group and PUSCH(s) in another PUCCH group when multiple PUCCH groups are configured. Also, FG #4-1 (basic UL control channel, mandatory without capability signaling) in TR38.822 includes the following component which is aligned with above description in TS38.213. - 10) Single group of overlapping PUCCH/PUCCH and overlapping PUCCH/PUSCH s per slot per PUCCH cell group for control multiplexing However, this combination is currently not captured in Table 6.1-2 of TS38.202. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the combination of simultaneous PUCCH and PUSCH across different PUCCH groups in Table 6.1-2 and added Note 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not aligned with TS38.213 and TR38.822. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact analysis: No impact on existing implementation. |  |  |  |  |  |  |  |  |

<< Start of change>>

## 6.1	Uplink

The tables 6.1-1 and 6.1-2 describe the possible combinations of physical channels and SRS that can be sent in simultaneously in the uplink by one UE. Table 6.1-1 introduces notation for a "Transmission Type" which represents a physical channel or sounding reference signal, and any associated transport channel. Table 6.1-2 describes the combinations of these "Transmission Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be transmitted simultaneously.

Table 6.1-1: Uplink "Transmission Types"

| "Transmission Type" | Physical Channel or SRS | Associated Transport Channel | Comment |
| --- | --- | --- | --- |
| A | PRACH | RACH | Note 1 |
| B | PUCCH | N/A |  |
| C | PUSCH | UL-SCH | Note 2 |
| D | SRS | N/A |  |
| Note 1: RACH corresponds to contention based. Note 2: UCI on PUSCH without UL-SCH is possible. |  |  |  |

Table 6.1-2: Uplink "Transmission Type" combinations

| Supported Combinations | Comment |
| --- | --- |
| j x A | Note 1 |
| k x B | Note 2 |
| p x C | Note 3, Note 4 |
| p' x D | Note 3, Note 5 |
| j× A + k×B | Note 6 |
| j× A + p×C | Note 6 |
| j× A + p'×D | Note 6 |
| k× B + p×C | Note 8 |
| k× B + p'×D | Note 7 |
| p× C + p'×D | Note 7 |
| Note 1: The number of cell groups j in the supported combination is subject to UE capability. Note 2: The number of PUCCH groups k in the supported combination is subject to UE capability. Note 3: The number of carriers p, and p' in the supported combinations are subject to UE capability. Note 4: In the case there is one SUL carrier, then p-1 would be supported. Note 5: UE may be configured with p' but may also have capability to simultaneously sound less than this number. Note 6: Simultaneous PRACH with PUCCH (or PUSCH or SRS) is supported only in the case of inter-band CA, with j≤j, k≤k, p≤p, and p'≤p' depending on the configuration, and subject to UE capability for parallel transmission. Note 7: Simultaneous SRS with PUCCH (or PUSCH) is supported only in the case of inter-band CA, with k≤k, p≤p, and p'≤p' depending on the configuration, and subject to UE capability for parallel transmission. Note 8: Simultaneous PUCCH and PUSCH(s) is supported only in the case that multiple PUCCH groups are configured and the respective PUCCH and PUSCH(s) are transmitted in the different PUCCH groups, with k<k and p≤p. k and p are subject to UE capability for supported number of PUCCH groups and UL carriers, respectively. k and p depend on configuration. |  |

<< End of change>>