3GPP TSG-RAN WG1 Meeting # 98bis                                                            R1-1911530

Chongqing, China, October 14th – 20th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0062 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on beta_offset values for UCI reporting in PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specification, the beta offset values for DG PUSCH and Type 2 PUSCH transmission for the following two cases are not clear. - Case1: When the UE is provided betaOffsets =‘dynamic’ and a PUSCH is scheduled by DCI format 0_0. - Case2: When the UE is provided by CG-UCI-OnPUSCH =‘dynamic’ and a Type 2 PUSCH transmission with a configured grant scheduled by DCI format 0_0. This CR is to clarify the beta offset value is determined by the first value provided by betaOffsets =‘dynamic’ for Case 1 and the first value provided by CG-UCI-OnPUSCH =‘dynamic’ for Case 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the beta offset value is determined by the first value provided by betaOffsets =‘dynamic’ for Case 1 and the first value provided by CG-UCI-OnPUSCH =‘dynamic’ for Case 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined beta offset values in above two cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: If the network is implemented according to the CR and the UE is not, or UE is implemented according to this CR but the network is not, the network and UE may have a different understanding on the beta offsets used for UCI multiplexing in PUSCH. In a result, gNB may not be able to successfully decode the PUSCH and UCI in the PUSCH. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the second version of this CR. The first version is in R1-1911488. |  |  |  |  |  |  |  |  |

## 9.3	UCI reporting in physical uplink shared channel

Offset values are defined for a UE to determine a number of resources for multiplexing HARQ-ACK information and for multiplexing CSI reports in a PUSCH. The offset values are signalled to a UE either by a DCI format scheduling the PUSCH transmission or by higher layers.

If DCI format 0_0, or DCI format 0_1 that does not include a beta_offset indicator field, schedules the PUSCH transmission from the UE and the UE is provided betaOffsets = 'semiStatic', the UE applies the , , and  values that are provided by betaOffsets = 'semiStatic' for the corresponding HARQ-ACK information, Part 1 CSI reports and Part 2 CSI reports.

If the PUSCH is scheduled by DCI format 0_0 and the UE is provided betaOffsets = ‘dynamic’, the UE applies the  , , and  values that are determined from the first value of  betaOffsets = ‘dynamic’.

If the PUSCH is a configured grant Type 2 PUSCH activated by DCI format 0_0 and the UE is provided CG-UCI-OnPUSCH =‘dynamic’, the UE applies the  , , and  values that are determined from the first value of CG-UCI-OnPUSCH = ‘dynamic’.

HARQ-ACK information offsets  are configured to values according to Table 9.3-1. The betaOffsetACK-Index1, betaOffsetACK-Index2, and betaOffsetACK-Index3 respectively provide indexes , , and  for the UE to use if the UE multiplexes up to 2 HARQ-ACK information bits, more than 2 and up to 11 HARQ-ACK information bits, and more than 11 bits in the PUSCH, respectively.

Part 1 CSI report and Part 2 CSI report offsets  and , respectively, are configured to values according to Table 9.3-2. The betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part2-Index1 respectively provide indexes  and  for the UE to use if the UE multiplexes up to 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. The betaOffsetCSI-Part1-Index2 and betaOffsetCSI-Part2-Index2 respectively provide indexes  or  for the UE to use if the UE multiplexes more than 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH.

If a DCI format 0_1 schedules the PUSCH transmission from the UE and if DCI format 0_1 includes a beta_offset indicator field, as configured by uci-OnPUSCH, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3} a set of four  indexes, by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of four  indexes and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of four  indexes from Table 9.3-1 and 9.3-2, respectively, for multiplexing HARQ-ACK information, Part 1 CSI reports, and Part 2 CSI reports, respectively, in the PUSCH transmission. The beta_offset indicator field indicates a  value, a  value and a  value from the respective sets of values, with the mapping defined in Table 9.3-3.

Table 9.3-1: Mapping of beta_offset values for HARQ-ACK information and the index signalled by higher layers

| or or |  |
| --- | --- |
| 0 | 1.000 |
| 1 | 2.000 |
| 2 | 2.500 |
| 3 | 3.125 |
| 4 | 4.000 |
| 5 | 5.000 |
| 6 | 6.250 |
| 7 | 8.000 |
| 8 | 10.000 |
| 9 | 12.625 |
| 10 | 15.875 |
| 11 | 20.000 |
| 12 | 31.000 |
| 13 | 50.000 |
| 14 | 80.000 |
| 15 | 126.000 |
| 16 | Reserved |
| 17 | Reserved |
| 18 | Reserved |
| 19 | Reserved |
| 20 | Reserved |
| 21 | Reserved |
| 22 | Reserved |
| 23 | Reserved |
| 24 | Reserved |
| 25 | Reserved |
| 26 | Reserved |
| 27 | Reserved |
| 28 | Reserved |
| 29 | Reserved |
| 30 | Reserved |
| 31 | Reserved |

Table 9.3-2: Mapping of beta_offset values for CSI and the index signalled by higher layers

| or or |  |
| --- | --- |
| 0 | 1.125 |
| 1 | 1.250 |
| 2 | 1.375 |
| 3 | 1.625 |
| 4 | 1.750 |
| 5 | 2.000 |
| 6 | 2.250 |
| 7 | 2.500 |
| 8 | 2.875 |
| 9 | 3.125 |
| 10 | 3.500 |
| 11 | 4.000 |
| 12 | 5.000 |
| 13 | 6.250 |
| 14 | 8.000 |
| 15 | 10.000 |
| 16 | 12.625 |
| 17 | 15.875 |
| 18 | 20.000 |
| 19 | Reserved |
| 20 | Reserved |
| 21 | Reserved |
| 22 | Reserved |
| 23 | Reserved |
| 24 | Reserved |
| 25 | Reserved |
| 26 | Reserved |
| 27 | Reserved |
| 28 | Reserved |
| 29 | Reserved |
| 30 | Reserved |
| 31 | Reserved |

Table 9.3-3: Mapping of beta_offset indicator values to offset indexes

| beta_offset indicator | ( or or ), ( or ), ( or ) |
| --- | --- |
| '00' | 1st offset index provided by higher layers |
| '01' | 2nd offset index provided by higher layers |
| '10' | 3rd offset index provided by higher layers |
| '11' | 4th offset index provided by higher layers |