3GPP TSG-RAN WG1 Meeting #98bis	R1-1911646

# Chongqing, China, 14th – 20th October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0067 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on time gap definition for HARQ-ACK transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 10.2, the reference μ for the determiniation of N does not consider the SCS configuration of PUCCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the defintion of μ by taking SCS configuration of PUCCH into account and the condition of capability 1 and 2 for determination of N is clarified in section 10.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential insufficient processing time for HARQ-ACK feedback corresponding to SPS PDSCH release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The CR has isolated impact on timeline requirement between SPS PDSCH release and the corresponding HARQ-ACK in case the SCS configuration of PUCCH carrying the HARQ-ACK is smaller than SCS configuration of PDCCH carrying release DCI. If the network is implemented according to the CR and the UE is not, or the UE is implemented according to this CR but the network is not, there may be different understandings between gNB and UE on the required time gap between SPS PDSCH release and corresponding HARQ-ACK in case the SCS configuration of PUCCH carrying the HARQ-ACK is smaller than SCS configuration of PDCCH carrying release DCI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.2	PDCCH validation for DL SPS and UL grant Type 2

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field for the enabled transport block is set to '0'.

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation

|  | DCI format 0_0/0_1 | DCI format 1_0 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' | For the enabled transport block: set to '00' |

Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0 | DCI format 1_0 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '1's | set to all '1's |

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after  symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release, for ,  for , and  for , otherwise,  for ,  for ,  for , and  for , wherein  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

<unchanged part omitted>