3GPP TSG RAN WG1 Meeting #98bis	R1-1911533

# Chongqing, China, Oct. 14 – 20, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0010 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RSTD measurement for E-UTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The measurement of reference signal time difference (RSTD) defined in TS 38.215 is mistakenly applied to E-UTRAN cells, which should be E-UTRA (as part of NG-RAN) as - Captured in the WI description NR_newRAT-Core [RP-172115]. - Supported by TS 38.133, 38.305 and 38.455. The difference between using E-UTRA and E-UTRAN is that ng-eNB is not included in E-UTRAN since it is part of NG-RAN, namely - E-UTRA as an access technology, includes both eNB (connected to EPC) and ng-eNB (connected to 5GC) - E-UTRAN as the network of radio access technologoies includes only eNB (connnected to EPC) In summary, the RSTD measurement should be extended to all E-UTRA cells, instead of only the cells in E-UTRAN. Also the E-UTRAN used in UE GNSS timing of cell frames for UE positioning should also be changed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All instances of E-UTRAN in the definition of RSTD and UE GNSS timing of cell frames for UE positioning are changed to E-UTRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will limit the Rel-15 regulatory requirements to only being achieved by PRS transmitted from E-UTRAN, and makes EPC and interface between E-SMLC and LMF mandatorily present in architecture option 4, option 7, and option 5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The change can be treated as editorial and clarification. No impact from UE or gNB. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

### 5.1.9	UE GNSS Timing of Cell Frames for UE positioning for E-UTRA

| Definition | The timing between E-UTRA cell j and a GNSS-specific reference time for a given GNSS (e.g., GPS/Galileo/Glonass system time). TUE-GNSS is defined as the time of occurrence of a specified NG-RAN event according to GNSS time for a given GNSS Id. The specified NG-RAN event is the beginning of a particular frame (identified through its SFN) in the first detected path (in time) of the cell-specific reference signals of the cell j, where cell j is a cell chosen by the UE. The reference point for TUE-GNSSj shall be the antenna connector of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED inter-RAT |

<Unchanged parts are omitted>

### 5.1.13	Reference signal time difference (RSTD) for E-UTRA

| Definition | The relative timing difference between the E-UTRA neighbour cell j and the E-UTRA reference cell i, defined as TSubframeRxj – TSubframeRxi, where: TSubframeRxj is the time when the UE receives the start of one subframe from E-UTRA cell j TSubframeRxi is the time when the UE receives the corresponding start of one subframe from E-UTRA cell i that is closest in time to the subframe received from E-UTRA cell j. The reference point for the observed subframe time difference shall be the antenna connector of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED inter-RAT |

<Unchanged parts are omitted>