3GPP TSG-RAN WG1 Meeting #98bis 	R1-1911396

Chongqing, China, October 14-20, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1280 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SPDCCH monitoring for TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_sTTIandPT-Core |  |  |  |  |  | Date: |  |  | 2019-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current spec, collision between CSI-RS and SPDCCH search space makes the number of RE’s in each SREG to be 10 which is not a multiple of 4. This combined with SREG-interleaving per candidate creates a situation in which an RE in those SREG’s sometimes consists of port 0 and 2 and sometimes consists of port 1 and 3 when multiple candidates with different AL contain that same SREG. In the example below, two canddiates (one with AL1 and the other with AL2) are shown, and RE’s in SREG4 are shown to have different ports in these 2 candidates. The same thing happens for SREG’s 5, 6, 7. The situation like above incurs decoding failure at a UE and enforces a UE to double channel estimation and MIMO detection for SPDCCH in addition to complicated port mapping control for each SPDCCH candidate, whch results in violation of processing time. |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | A UE is not required to receive an SREG containing an RE having multiple different port mapping patterns. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE will not be able to fullfill required SPDCCH monitoring due to failed decoding on PDCCH candidates affected by the issues described in reasons for change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: An existing UE implementation will conform to the changes in the CR. There is no impact to UE and network implementation. |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

#### < Unchanged parts are omitted >

### 9.1.6	SPDCCH assignment procedure

If the UE is configured with shortTTI, and SPDCCH is monitored in a slot, the term 'slot/subslot' refers to a slot in this clause.

If the UE is configured with shortTTI, and SPDCCH is monitored in a subslot, the term 'slot/subslot' refers to a subslot in this clause.

For each serving cell, higher layer signalling can configure a UE with

-	one or two SPDCCH-PRB-sets for SPDCCH monitoring in a slot/subslot of a non-MBSFN subframe, and

-	one or two SPDCCH-PRB-sets for SPDCCH monitoring in a slot/subslot of an MBSFN subframe.

The PRBs corresponding to a SPDCCH-PRB-set are indicated by higher layers as described in Subclause 9.1.6.2. Each SPDCCH-PRB-set consists of a set of SCCEs numbered from 0 to where is the number of SCCEs in SPDCCH-PRB-set  in a subframe. Each SPDCCH-PRB-set can be configured for either localized SPDCCH transmission or distributed SPDCCH transmission.

The UE shall monitor a set of SPDCCH candidates on one or more activated serving cells as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the SPDCCHs in the set according to the monitored DCI formats.

A UE is not required to monitor CRS-based SPDCCH in an MBSFN subframe.

A UE is not required to monitor CRS-based SPDCCH and DMRS-based SPDCCH in a slot/subslot if the UE does not support differentRSType.

A UE is not expected to receive DMRS-based SPDCCH scheduling PDSCH in slots/subslots where the UE is configured with DL transmission modes 1-4, 6.

The UE is not expected to be configured to monitor SPDCCH with carrier indicator field in a given serving cell.

The UE is not expected to monitor

-	SPDCCH in the first subslot of a subframe

-	SPDCCH in the first slot of a subframe if higher layer parameters dl-STTI-Length is set to 'slot'.

For 4 port CRS-based SPDCCH, a UE is not required to receive an SREG belonging to multiple PDCCH candidates if any resource element in that SREG corresponds to different antenna ports for those multiple PDCCH candidates, where the precoding sub-matrix is described in the precoding operation from clause 6.3.4.3 of 3GPP TS 36.211 [3].

The set of SPDCCH candidates to monitor are defined in terms of SPDCCH UE-specific search spaces. An SPDCCH UE-specific search space  or slot/subslot number k at aggregation level  is defined by a set of SPDCCH candidates.

For a CRS-based SPDCCH-PRB-set or a DMRS-based SPDCCH-PRB-set configured with localized SPDCCH transmission in slot/subslot number , the SCCEs corresponding to SPDCCH candidate of the search space  at aggregation level are given by

< Unchanged parts are omitted >