3GPP TSG-RAN WG1 Meeting #99	R1-1913625

# Reno, NV, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.202 | CR | 0009 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of two-step RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2019-12-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of two-step RACH |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Scell, msgA PRACH and PUSCH is not supported |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity of whether msgA is supported in Scell |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	Simultaneous transmission and reception of physical channels and physical signals

This clause describes the requirements from the UE to send and receive multiple physical channels and physical signals simultaneously depending on the capabilities and service requirements. The following notation is used between both the uplink and downlink subclauses below.

-	p is the number of uplink carriers configured for the UE on which physical channels can be transmitted

-	p' is the number of uplink carriers configured for the UE on which SRS can be transmitted

-	q is the number of downlink carriers configured for the UE

-	j is the number of cell groups configured for the UE.

-	k is the number of PUCCH groups configured for the UE.

## 6.1	Uplink

The tables 6.1-1 and 6.1-2 describe the possible combinations of physical channels and SRS that can be sent in simultaneously in the uplink by one UE. Table 6.1-1 introduces notation for a "Transmission Type" which represents a physical channel or sounding reference signal, and any associated transport channel. Table 6.1-2 describes the combinations of these "Transmission Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be transmitted simultaneously.

Table 6.1-1: Uplink "Transmission Types"

| "Transmission Type" | Physical Channel or SRS | Associated Transport Channel | Comment |
| --- | --- | --- | --- |
| A | PRACH | RACH | Note 1, Note 8 |
| B | PUCCH | N/A |  |
| C | PUSCH | UL-SCH | Note 2, Note 8 |
| D | SRS | N/A |  |
| Note 1: RACH corresponds to contention based. Note 2: UCI on PUSCH without UL-SCH is possible. Note 8: For SCell, MsgA PRACH and MsgA PUSCH is not supported. |  |  |  |

Table 6.1-2: Uplink "Transmission Type" combinations

| Supported Combinations | Comment |
| --- | --- |
| j x A | Note 1 |
| k x B | Note 2 |
| p x C | Note 3, Note 4 |
| p' x D | Note 3, Note 5 |
| j× A + k×B | Note 6 |
| j× A + p×C | Note 6 |
| j× A + p'×D | Note 6 |
| k× B + p'×D | Note 7 |
| p× C + p'×D | Note 7 |
| Note 1: The number of cell groups j in the supported combination is subject to UE capability. Note 2: The number of PUCCH groups k in the supported combination is subject to UE capability. Note 3: The number of carriers p, and p' in the supported combinations are subject to UE capability. Note 4: In the case there is one SUL carrier, then p-1 would be supported. Note 5: UE may be configured with p' but may also have capability to simultaneously sound less than this number. Note 6: Simultaneous PRACH with PUCCH (or PUSCH or SRS) is supported only in the case of inter-band CA, with j≤j, k≤k, p≤p, and p'≤p' depending on the configuration, and subject to UE capability for parallel transmission. Note 7: Simultaneous SRS with PUCCH (or PUSCH) is supported only in the case of inter-band CA, with k≤k, p≤p, and p'≤p' depending on the configuration, and subject to UE capability for parallel transmission. |  |

## 6.2	Downlink

The tables 6.2-1, 6.2-2 describe the possible combinations of physical channels that can be received simultaneously in the downlink by one UE. Table 6.2-1 introduces notation for a "Reception Type" which represents a physical channel and any associated transport channel. Table 6.2-2 describes the combinations of these "Reception Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be received simultaneously. The UE shall be able to receive all TBs according to the indication on PDCCH. Any subset of the combinations specified in table 6.2-2 is also supported.

Table 6.2-1: Downlink "Reception Types"

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel | Comment |
| --- | --- | --- | --- | --- |
| A | PBCH | N/A | BCH |  |
| B | PDCCH+PDSCH | SI-RNTI | DL-SCH | Note 1 |
| C0 | PDCCH | P-RNTI | N/A | Note 1, Note 2 |
| C1 | PDCCH+PDSCH | P-RNTI | PCH | Note 1 |
| D0 | PDCCH+PDSCH | RA-RNTI or Temporary C-RNTI or MsgB-RNTI | DL-SCH | Note 3 |
| D1 | PDCCH+PDSCH | C-RNTI, CS-RNTI, MCS-C-RNTI | DL-SCH |  |
| D2 | PDCCH | C-RNTI, CS-RNTI, MCS-C-RNTI | DL-SCH |  |
| E | PDCCH | C-RNTI | N/A | Note 4 |
| F0 | PDCCH | Temporary C-RNTI | UL-SCH | Note 3 |
| F1 | PDCCH | C-RNTI, CS-RNTI, MCS-C-RNTI | UL-SCH |  |
| G | PDCCH | SFI-RNTI | N/A |  |
| H | PDCCH | INT-RNTI | N/A |  |
| J0 | PDCCH | TPC-PUSCH-RNTI | N/A |  |
| J1 | PDCCH | TPC-PUCCH-RNTI | N/A |  |
| J2 | PDCCH | TPC-SRS-RNTI | N/A |  |
| K | PDCCH | SP-CSI-RNTI | N/A |  |
| Note 1: These are received from PCell only. Note 2: In some cases UE is only required to monitor the short message within the DCI for P-RNTI. Note 3: These are received from PCell or PSCell. Note 4: This corresponds to PDCCH-ordered PRACH. |  |  |  |  |

Table 6.2-2: Downlink "Reception Type" combinations

| Supported Combinations |  |  | Comment |
| --- | --- | --- | --- |
| PCell | PSCell | SCell |  |
| 1. RRC_IDLE |  |  |  |
| A + (B and/or C1 and/or D0) + F0 |  |  | Note 1 |
| 2. RRC_INACTIVE |  |  |  |
| A + (B and/or C1 and/or D0) + F0 |  |  | Note 1 |
| 3. RRC_CONNECTED |  |  |  |
| A + C0 + (B and/or (D0 or (D1+m*D2))) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K | A + (D0 or (D1+m*D2)) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K | D1 + m*D2 + E + n*F1 + G + H + J0 + J1 + J2 + K | Note 2, Note 3, Note 4 |
| Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: For PCell, UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. Note 3: Supported combinations are subject to UE capabilities for dual connectivity, carrier aggregation, receiving of group TPC commands, pre-emption indication and dynamic SFI monitoring. Note 4: The values of m ≥ 0 and n≥ 0 in the supported combinations are subject to the UE capability. |  |  |  |