3GPP TSG-RAN WG1 Meeting #99	R1-1913629

# Reno, NV, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0005 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of channel access procedures to unlicensed spectrum for NR-based access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2019-12-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of NR-based access to unlicensed spectrum. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added functionality for channel access procedures required for operation in unlicensed spectrum. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for NR in unlicensed spectrum is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.0 (new), 4.1, 4.2, 4.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================<Unchanged text is omitted>========================================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[3]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[4]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[5]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[6]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[7]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[8]	3GPP TS 38.214: "NR; Physical layer procedures for data".

====================<Unchanged text is omitted>========================================

# 4	Channel access procedure

## 4.0	General

Unless otherwise noted, the definitions below are applicable for the following terminologies used in this specification:

-	A channel refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum.

-	A channel access procedure is a procedure based on sensing that evaluates the availability of a channel for performing transmissions. The basic unit for sensing is a sensing slot with a duration Tsl=9us. The sensing slot duration Tsl is considered to be idle if an eNB/gNB or a UE senses the channel during the sensing slot duration, and determines that the detected power for at least 4us within the sensing slot duration is less than energy detection threshold XThresh. Otherwise, the sensing slot duration Tsl is considered to be busy.

-	A channel occupancy refers to transmission(s) on channel(s) by eNB/gNB/UE(s) after performing the corresponding channel access procedures in this subclause.

-	A Channel Occupancy Time refers to the total time for which eNB/gNB/UE and any eNB/gNB/UE(s) sharing the channel occupancy perform transmission(s) on a channel after an eNB/gNB/UE performs the corresponding channel access procedures described in this clause. For determining a Channel Occupancy Time, if a transmission gap is less than or equal to 25us, the gap duration is counted in the channel occupancy time. A channel occupancy time can be shared for transmission between an eNB/gNB and the corresponding UE(s).

-	A DL transmission burst is defined as a set of transmissions from an eNB/gNB without any gaps greater than 16us. Transmissions from an eNB/gNB separated by a gap of more than 16us are considered as separate DL transmission bursts. An eNB/gNB can transmit transmission(s) after a gap within a DL transmission burst without sensing the corresponding channel(s) for availability.

-	A UL transmission burst is defined as a set of transmissions from a UE without any gaps greater than 16us. Transmissions from a UE separated by a gap of more than 16 us are considered as separate UL transmission bursts. A UE can transmit transmission(s) after a gap within a UL transmission burst without sensing the corresponding channel(s) for availability.

-    A discovery burst refers to a DL transmission burst including a set of signal(s) and/or channel(s) confined within a window and associated with a duty cycle. The discovery burst can be any of the following:

-    Transmission(s) initiated by an eNB that includes a primary synchronization signal (PSS), secondary synchronization signal (SSS) and cell-specific reference signal(s)(CRS) and may include non-zero power CSI reference signals (CSI-RS).

-    Transmission(s) initiated by a gNB that includes at least an SS/PBCH block consisting of a primary synchronization signal (PSS), secondary synchronization signal (SSS), physical broadcast channel (PBCH) with associated demodulation reference signal (DM-RS) and may also include CORESET for PDCCH scheduling PDSCH with SIB1, and PDSCH carrying SIB1 and/or non-zero power CSI reference signals (CSI-RS).

## 4.1	Downlink channel access procedures

An eNB operating LAA Scell(s) on channel(s) and a gNB performing transmission(s) on channel(s) shall perform the channel access procedures described in this subclause for accessing the channel(s) on which the transmission(s) are performed.

In this subclause, XThresh  for sensing is adjusted as described in subclause 4.1.5 when applicable.

A gNB performs channel access procedures in this subclause unless the higher layer parameter ChannelAccessMode-r16 is provided and ChannelAccessMode-r16 =’ semistatic’.

### 4.1.1	Type 1 DL channel access procedures

This subclause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by the sensing slots that are sensed to be idle before a downlink transmission(s) is random. The subclause is applicable to the following transmissions:

-	Transmission(s) initiated by an eNB including PDSCH/PDCCH/EPDCCH, or

-	Transmission(s) initiated by a gNB including unicast PDSCH with user plane data, or unicast PDSCH with user plane data and unicast PDCCH scheduling user plane data, or

-	Transmission(s) initiated by a gNB with only discovery burst or with discovery burst multiplexed with non-unicast information, where the transmission(s) duration is larger than 1ms  or the transmission causes the discovery burst duty cycle to exceed 1/20.

The eNB/gNB may transmit a transmission, after first sensing the channel to be idle during the sensing slot durations of a defer duration Td, and after the counter Nis zero in step 4. The counter N is adjusted by sensing the channel for additional sensing slot duration(s) according to the steps below:

1)	set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4;

2)	if N>0 and the eNB/gNB chooses to decrement the counter, set N=N-1;

3)	sense the channel for an additional sensing slot duration, and if the additional sensing slot duration is idle, go to step 4; else, go to step 5;

4)	if N=0, stop; else, go to step 2.

5)	sense the channel until either a busy sensing slot is detected within an additional defer duration Td or all the sensing slots of the additional defer duration Td are detected to be idle;

6)	if the channel is sensed to be idle during all the sensing slot durations of the additional defer duration Td, go to step 4; else, go to step 5;

If an eNB/gNB has not transmitted a transmission after step 4 in the procedure above, the eNB/gNB may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration Tsl when the eNB/gNB is ready to transmit and if the channel has been sensed to be idle during all the sensing slot durations of a defer duration Td immediately before this transmission. If the channel has not been sensed to be idle in a sensing slot duration Tsl when the eNB/gNB first senses the channel after it is ready to transmit or if the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration Td immediately before this intended transmission, the eNB/gNB proceeds to step 1 after sensing the channel to be idle during the sensing slot durations of a defer duration Td.

The defer duration Td consists of duration Tf=16us immediately followed by mp consecutive sensing slot durations Tsl, and Tf includes an idle sensing slot duration Tsl at start of Tf.

CWmin,p≤CWp≤CWmax,p

CWmin,p≤CWp≤CWmax,pis the contention window. CWp adjustment is described in subclause 4.1.4.

CWmin,p and CWmax,p are chosen before step 1 of the procedure above.

mp, CWmin,p, and CWmax,p are based on a channel access priority class p associated with the eNB/gNB transmission, as shown in Table 4.1.1-1.

XThresh

An eNB/gNB shall not transmit on a channel for a Channel Occupancy Time that exceeds Tmcot,p where the channel access procedures are performed based on a channel access priority class p associated with the eNB/gNB transmissions, as given in Table 4.1.1-1.

If an eNB/gNB transmits discovery burst(s) as described in subclause 4.1.2  when N>0 in the procedure above, the eNB/gNB shall not decrement N during the sensing slot duration(s) overlapping with discovery burst(s).

A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including discovery burst(s) satisfying the conditions described in this subclause.

A gNB shall use a channel access priority class applicable to the unicast user plane data multiplexed in PDSCH for performing the procedures above to transmit transmission(s) including unicast PDSCH with user plane data.

For p=3 and p=4 , if the absence of any other technology sharing the channel can be guaranteed on a long term basis (e.g. by level of regulation), Tmcot,p=10ms, otherwise, Tmcot,p=8ms.

Table 4.1.1-1: Channel Access Priority Class

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tmcot,p | allowed CWpsizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 7 | 2 ms | {3,7} |
| 2 | 1 | 7 | 15 | 3 ms | {7,15} |
| 3 | 3 | 15 | 63 | 8 or 10 ms | {15,31,63} |
| 4 | 7 | 15 | 1023 | 8 or 10 ms | {15,31,63,127,255,511,1023} |

#### 4.1.1.1	Regional limitations on channel occupancy time

In

Japan, if an eNB/gNB has transmitted a transmission after N=0 in step 4 of the procedure above, the eNB/gNB may transmit the next continuous transmission, for duration of maximum Tj=4ms, immediately after sensing the channel to be idle for at least a sensing interval of Tjs=34us and if the total sensing and transmission time is not more than 1000⋅Tmcot+TmcotTj-1⋅Tjs us. The sensing interval Tjs consists of duration Tf=16us immediately followed by two sensing slots and Tf includes an idle sensing slot at start of Tf. The channel is considered to be idle for Tjs if it is sensed to be idle during  the sensing slot durations of Tjs.

### 4.1.2	Type 2 DL channel access procedures

This subclause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by sensing slots that are sensed to be idle before a downlink transmission(s) is deterministic.

If an eNB performs Type 2 DL channel access procedures, it follows the procedures described in subclause 4.1.2.1.

Type 2A channel access procedures as described in subclause 4.1.2.1 are applicable to the following transmission(s) performed by an eNB/gNB:

-	Transmission(s) initiated by an eNB including discovery burst and not including PDSCH, or

-	Transmission(s) initiated by a gNB with only discovery burst or with discovery burst multiplexed with non-unicast information, where the transmission(s) duration is at most 1ms, and the discovery burst duty cycle is at most 120, or

-	Transmission(s) by an eNB/ gNB following transmission(s) by a UE after a gap of  25us in a shared channel occupancy as described in subclause 4.1.3.

Type 2B or Type 2C DL channel access procedures as described in subclause 4.1.2.2 and 4.1.2.3, respectively, are applicable to the transmission(s) performed by a gNB following transmission(s) by a UE after a gap of 16us or up to 16us, respectively, in a shared channel occupancy as described in subclause 4.1.3.

#### 4.1.2.1	Type 2A DL channel access procedures

An eNB/gNB may transmit a DL transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_dl=25us. Tshort_dl Tshort_dl consists of a duration Tf=16us immediately followed by one sensing slot and Tf includes a sensing slot at start of  Tf. The channel is considered to be idle for  Tshort_dl  if both sensing slots  of Tshort_dl are sensed to be idle.

XThresh

#### 4.1.2.2	Type 2B DL channel access procedures

A gNB may transmit a DL transmission immediately after sensing the channel to be idle within a duration of Tf=16us. Tf includes a sensing slot that occurs within the last 9us of Tf. The channel is considered to be idle within the duration Tf if the channel is sensed to be idle for a total of at least 5us with at least 4us of sensing occurring in the sensing slot.

#### 4.1.2.3	Type 2C DL channel access procedures

When a gNB follows the procedures in this subclause for transmission of a DL transmission, the gNB does not sense the channel before transmission of the DL transmission. The duration of the corresponding DL transmission is at most 584us.

### 4.1.3	DL channel access procedures in a shared channel occupancy

For the case where an eNB shares a channel occupancy initiated by a UE, the eNB may transmit a transmission that follows an autonomous PUSCH transmission by the UE as follows:

-	If 'COT sharing indication' in AUL-UCI in subframe n indicates '1', an eNB may transmit a transmission in subframe n+X, where X is subframeOffsetCOT-Sharing, including PDCCH but not including PDSCH on the same channel immediately after performing Type 2A DL channel access procedures in subclause 4.1.2.1, if the duration of the PDCCH is less than or equal to duration of two OFDM symbols and it shall contain at least AUL-DFI or UL grant to the UE from which the PUSCH transmission indicating COT sharing was received.

If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in subclause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a PUSCH transmission on scheduled or configured resources by the UE after a gap as follows:

-	The transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy.

-	If the higher layer parameters ULtoDL-CO-SharingED-Threshold-r16 is not provided, the transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively.

-	If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in subclause 4.1.2.3.

-	If the gap is 25us  or 16us, the gNB can transmit the transmission on the channel after performing Type 2A or Type 2B DL channel access procedures as described in subclause 4.1.2.1 and 4.1.2.2, respectively.

For the case where a gNB shares a channel occupancy initiated by a UE with configured grant PUSCH transmission, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE as follows:

-	If the higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 is provided, the UE is configured by COT-SharingList-r16 where COT-SharingList-r16 provides a table configured by higher layer. Each row of the provides a channel occupancy information given by higher layer parameter cg-COT-Sharing-r16. One row of the table is configured for indicating that the channel occupancy information is not available.

-	If the ‘COT sharing information’ in CG-UCI indicates a row index that corresponds to a cg-COT-Sharing-r16 that provides channel occupancy information, the gNB can share the UE channel occupancy assuming a channel access priority class p=CAPC and starting from O=offset slots from the end of the slot where CG-UCI is detected, for a duration of D=duration slots assuming a channel access priority class p=CAPC where duration, offset, CAPC are higher layer parameters provided by cg-COT-Sharing-r16 as cg-COT-Sharing-r16 ={duration, offset, CAPC}.

-    If the higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 is not provided, and if ‘COT sharing information’ in CG-UCI indicates ‘1’, the gNB can share the UE channel occupancy and start the DL transmission X= cg-COT-SharingOffset-r16 symbols from the end of the slot where CG-UCI is detected, where cg-COT-SharingOffset-r16 is provided by higher layer. The transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively.

For the case where a gNB uses channel access procedures as described in subclause 4.1.1 to initiate a transmission and shares the corresponding channel occupancy with a UE that transmits a transmission as described in subclause 4.2.1.2, the gNB may transmit a transmission within its channel occupancy that follows the UE’s transmission if any gap between any two transmissions in the gNB channel occupancy is at most 25us. In this case the following applies:

-

16us

-	If the gap is 25us or16us, the gNB can transmit the transmission on the channel after performing Type 2A or 2B DL channel access procedures as described in subclause 4.1.2.1 and 4.1.2.2, respectively.

-	If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in subclause 4.1.2.3.

### 4.1.4	Contention window adjustment procedures

If an eNB/gNB transmits transmissions including PDSCH that are associated with channel access priority class p on a channel, the eNB/gNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in subclause 4.1.1 for those transmissions as described in this subclause.

#### 4.1.4.1	Contention window adjustment procedures for transmissions by eNB

If an eNB transmits transmissions including PDSCH that are associated with channel access priority class p on a channel, the eNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in subclause 4.1.1 for those transmissions using the following steps:

1)	for every priority class p∈1,2,3,4, set CWp=CWmin,p;

2)	if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe k are determined as NACK, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value and remain in step 2; otherwise, go to step 1.

Reference subframe k is the starting subframe of the most recent transmission on the channel made by the eNB, for which at least some HARQ-ACK feedback is expected to be available.

The eNB shall adjust the value of CWp for every priority class p∈1,2,3,4 based on a given reference subframe k only once.

CWp=CWmax,pCWpCWmax,p

For determining Z,

-	if the eNB transmission(s) for which HARQ-ACK feedback is available start in the second slot of subframe k, HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k+1are also used in addition to the HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k.

-	if the HARQ-ACK values correspond to PDSCH transmission(s) on an LAA SCell that are assigned by (E)PDCCH transmitted on the same LAA SCell,

-	if no HARQ-ACK feedback is detected for a PDSCH transmission by the eNB, or if the eNB detects 'DTX', 'NACK/DTX' or 'any' state, it is counted as NACK.

-	if the HARQ-ACK values correspond to PDSCH transmission(s) on an LAA SCell that are assigned by (E)PDCCH transmitted on another serving cell,

-	if the HARQ-ACK feedback for a PDSCH transmission is detected by the eNB, 'NACK/DTX' or 'any' state is counted as NACK, and 'DTX' state is ignored.

-	if no HARQ-ACK feedback is detected for a PDSCH transmission by the eNB

-	if PUCCH format 1b with channel selection is expected to be used by the UE, 'NACK/DTX' state corresponding to 'no transmission' as described in Subclauses 10.1.2.2.1, 10.1.3.1 and 10.1.3.2.1 is counted as NACK, and 'DTX' state corresponding to 'no transmission' is ignored in [4].

-	Otherwise, the HARQ-ACK for the PDSCH transmission is ignored.

-	if a PDSCH transmission has two codewords, the HARQ-ACK value of each codeword is considered separately

-	bundled HARQ-ACK across M subframes is considered as M HARQ-ACK responses.

If the eNB transmits transmissions including PDCCH/EPDCCH with DCI format 0A/0B/4A/4B and not including PDSCH that are associated with channel access priority class p on a channel starting from time t0, the eNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in subclause 4.1.1 for those transmissions using the following steps:

1)	for every priority class p∈1,2,3,4 set CWp=CWmin,p

2)	if less than 10% of the UL transport blocks scheduled by the eNB using Type 2 channel access procedure (described in subclause 4.2.1.2) in the time interval between t0 and t0+TCO have been received successfully, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value and remain in step 2; otherwise, go to step 1.

TCO is computed as described in subclause 4.2.1.0.3.

#### 4.1.4.2 	Contention window adjustment procedures for DL transmissions by gNB

If a gNB transmits transmissions including PDSCH that are associated with channel access priority class p on a channel, the gNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in subclause 4.1.1 for those transmissions using the following steps:

- For every priority class p∈1,2,3,4, set CWp=CWmin,p.

- If HARQ-ACK feedback is available after the last update of CWp , go to step 3. Otherwise, if the gNB transmission after procedure described in subclause 4.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest DL transmission burst after the last update of CWp  transmitted after the procedures described in subclause 4.1.1, go to step 5; otherwise go to step 4.

- The HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL transmission burst for which HARQ-ACK feedback is available is used as follows:

  - If at least one HARQ-ACK feedback is ‘ACK’ for PDSCH(s) with transport block based transmissions or at least 10% of HARQ-ACK feedbacks is ‘ACK’ for PDSCH(s) with code block group based transmissions go to step 1; otherwise go to step 4.

- Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

- For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2.

The reference duration and duration Tw  in the procedure above are defined as follows:

-	The reference duration corresponding to a channel occupancy initiated by the gNB including transmission of PDSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PDSCH, then, the duration of the first transmission burst by the gNB within the channel occupancy that contains unicast PDSCH(s) is the reference duration for CWS adjustment.

-	Tw=max⁡(TA,TB+1ms) where TB is the duration of the transmission burst from start of the reference duration in  ms and TA=5ms  if the absence of any other technology sharing the channel can not be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms  otherwise.

If a gNB transmits transmissions using Type 1 channel access procedures associated with the channel access priority class p on a channel and the transmissions are not associated with explicit HARQ-ACK feedbacks by the corresponding UE(s), the gNB adjusts CWp before step 1 in the procedures described in subclase 4.1.1, using the latest CWp used for any DL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p  has not been used for any DL transmissions on the channel, CWp=CWmin,p is used.

#### 4.1.4.3	Common procedures for CWS adjustments for DL transmissions

The following applies to the procedures described in subclauses 4.1.4.1 and 4.1.4.2:

-	If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p.

-	If the CWp=CWmax,p is consecutively used K times for generation of Ninit, CWp  is reset to CWmin,p only for that priority class p for which CWp=CWmax,p is consecutively used K times for generation of Ninit. K  is selected by eNB/gNB from the set of values {1, 2, …,8} for each priority class p∈1,2,3,4.

### 4.1.5	Energy detection threshold adaptation procedures

An eNB/gNB accessing a channel on which transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold XThresh_max.

XThresh_max is determined as follows:

-	If the absence of any other technology sharing the channel can be guaranteed on a long-term basis (e.g. by level of regulation) then:

-	XThresh_max=min&Tmax+10dB,&Xr

-	Xr

is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB

-	Otherwise,

-	XThresh_max=max&-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX

-	Where:

-	TA=10dB for transmission(s) including PDSCH;

-	TA=	5dB for transmissions including discovery burst(s) as described in subclause 4.1.2;

-	PH=23dBm	 dBm;

-	PTX	 is the set maximum eNB/gNB output power in dBm for the channel;

-	eNB/gNB uses the set maximum transmission power over a single channel irrespective of whether single channel or multi-channel transmission is employed

-	Tmax(dBm)=10⋅log103.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz);

-	BWMHz

is the single channel bandwidth in MHz.

### 4.1.6	Channel access procedures for transmission(s) on multiple channels

An eNB/gNB can access multiple channels on which transmission(s) are performed, according to one of the Type A or Type B procedures described in this Subclause.

#### 4.1.6.1	Type A multi-channel access procedures

An eNB/gNB shall perform channel access on each channel ci∈C, according to the procedures described in subclause 4.1.1, where C is a set of channels on which the eNB/gNB intends to transmit, and i=0,1,…q-1, and q is the number of channels on which the eNB/gNB intends to transmit.

The counter N described in subclause 4.1.1 is determined for each channel ci and is denoted as Nci. Nci is maintained according to subclause 4.1.6.1.1 or 4.1.6.1.2.

##### 4.1.6.1.1	Type A1 multi-channel access procedures

Counter N as described in subclause 4.1.1 is independently determined for each channel ci and is denoted as Nci.

If the absence of any other technology sharing the channel cannot be guaranteed on a long term basis (e.g. by level of regulation), when the eNB/gNB ceases transmission on any one channel cj∈C, for each channel ci≠cj, the eNB/gNB can resume decrementing Nci when idle sensing slots are detected either after waiting for a duration of 4⋅Tsl, or after reinitialising Nci.

##### 4.1.6.1.2	Type A2 multi-channel access procedures

Counter N is determined as described in subclause 4.1.1 for channel cj∈C, and is denoted as Ncj, where cj is the channel that has the largest CWp value. For each channel ci, Nci=Ncj.

When the eNB/gNB ceases transmission on any one channel for which Nci is determined, the eNB/gNB shall reinitialise Nci for all channels.

#### 4.1.6.2	Type B multi-channel access procedure

A channel cj∈C is selected by the eNB/gNB as follows:

-	the eNB/gNB selects cj by uniformly randomly choosing cj from C before each transmission on multiple channels ci∈C, or

-	the eNB/gNB selects cj no more frequently than once every 1 second,

where C is a set of channels on which the eNB/gNB intends to transmit, i=0,1,…q-1, and q is the number of channels on which the eNB intends to transmit.

To transmit on channel cj

-	the eNB/gNB shall perform channel access on channel cj according to the procedures described in subclause 4.1.1 with the modifications described in subclause 4.1.6.2.1 or 4.1.6.2.2.

To transmit on channel ci≠cj, ci∈C

-	for each channel ci, the eNB/gNB shall sense the channel ci for at least a sensing interval Tmc=25us immediately before the transmitting on channel cj, and the eNB/gNB may transmit on carrier ci immediately after sensing the channel ci to be idle for at least the sensing interval Tmc. The channel ci is considered to be idle for Tmc if the channel is sensed to be idle during all the time durations in which such idle sensing is performed on the channel cj in given interval Tmc.

The eNB/gNB shall not transmit a transmission on a channel ci≠cj, ci∈C, for a period exceeding Tmcot,p as given in Table 4.1.1-1, where the value of Tmcot,p is determined using the channel access parameters used for channel cj.

For the procedures in this subclause, the channel frequencies of the set of channels C selected by gNB, is a subset of one of the sets of channel frequencies defined in [6].

##### 4.1.6.2.1	Type B1 multi-channel access procedure

A single CWpvalue is maintained for the set of channels C.

For determining CWp for channel access on channel cj, step 2 of the procedure described in subclause 4.1.4.1 is modified as follows

-	if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe k of all channels ci∈C are determined as NACK, increase CWp for each priority class p∈1,2,3,4 to the next higher allowed value; otherwise, go to step 1.

For determining CWp for a set of channels C, any PDSCH that fully or partially overlaps with any channel ci∈C, is used in the procedure described in subclause 4.1.4.2.

##### 4.1.6.2.2	Type B2 multi-channel access procedure

A CWp value is maintained independently for each channel ci∈C using the procedure described in subclause 4.1.4.

For determining CWp for channel ci, any PDSCH that fully or partially overlaps with channel  ci, is used in the procedure described in subclause 4.1.4.2.

For determining Ninit for channel cj, CWp value of channel cj1∈C is used, where cj1 is the channel with largest CWp among all channels in set C.

## 4.2	Uplink channel access procedures

A UE performing transmission(s) on LAA Scell(s), an eNB scheduling or configuring UL transmission(s) for a UE performing transmission(s) on LAA Scell(s), and a UE performing transmission(s) on channel(s) and a gNB scheduling or configuring UL transmission(s) for a UE performing transmissions on channel(s) shall perform the procedures described in this subclause for the UE to access the channel(s) on which the  transmission(s) are performed.

In this subclause, transmissions from a UE are considered as separate UL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in subclause 4.2.3 when applicable.

A UE performs channel access procedures in this subclause unless the higher layer parameter ChannelAccessMode-r16 is provided and ChannelAccessMode-r16 =’ semistatic’.

### 4.2.1	Channel access procedures for uplink transmission(s)

A UE can access a channel on which  UL transmission(s) are performed according to one of Type 1 or Type 2 UL channel access procedures. Type 1 channel access procedure is described in subclause 4.2.1.1. Type 2 channel access procedure is described in subclause 4.2.1.2.

If a UL grant scheduling a PUSCH transmission indicates Type 1 channel access procedures, the UE shall use Type 1 channel access procedures for transmitting transmissions including the PUSCH transmission unless stated otherwise in this subclause.

A UE shall use Type 1 channel access procedures for transmitting transmissions including the autonomous PUSCH transmission on configured UL resources unless stated otherwise in this subclause.

If a UL grant scheduling a PUSCH transmission indicates Type 2 channel access procedures, the UE shall use Type 2 channel access procedures for transmitting transmissions including the PUSCH transmission unless stated otherwise in this subclause.

A

UE shall use Type 1 channel access procedures for transmitting SRS transmissions not including a PUSCH transmission. UL channel access priority class p=1 in Table 4.2.1-1 is used for SRS transmissions not including a PUSCH.

If a UE is scheduled by an eNB/gNB to transmit PUSCH and SRS in contiguous transmissions without any gaps in between, and if the UE cannot access the channel for PUSCH transmission, the UE shall attempt to make SRS transmission  according to uplink channel access procedures specified for SRS transmission.

A UE shall use Type 1 channel access procedures for PUCCH transmissions unless stated otherwise in this subclause. If a DL grant scheduling a PUCCH transmission indicates Type 2 channel access procedures, the UE shall use Type 2 channel access procedures.

When a UE uses Type 1 channel access procedures for PUCCH transmissions or PUSCH only transmissions without UL-SCH, the UE shall use UL channel access priority class  p=1 in Table 4.2.1-1.

A UE shall use Type 1 channel access procedure for transmissions related to random access procedure that initiate a channel occupancy with UL channel access priority class p=1 in Table 4.2.1.

The total duration of autonomous uplink transmission(s) obtained by the channel access procedure in this subclause, including the following DL transmission if the UE sets ‘COT sharing indication’ in AUL-UCI to ‘1’ in a subframe within the autonomous uplink transmission(s), shall not exceed Tulmcot,p, where Tulmcot,p is given in Table 4.2.1-1.

Table 4.2.1-1: Channel Access Priority Class for UL

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tulmcot,p | allowed CWp sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For p=3,4, Tulmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided , otherwise, Tulmcot,p=6ms. NOTE 2: When Tulmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100us. The maximum duration before including any such gap shall be 6ms. |  |  |  |  |  |

#### 4.2.1.0	Channel access procedures and UL related signaling

##### 4.2.1.0.0	Channel access procedures upon detection of a common DCI

If a UE detects ‘UL duration and offset’ field in DCI Format 1C as described in subclause 5.3.3.1.4 of [5], the following are applicable:

-	If the UL duration and offset’ field indicates an ‘UL offset’ l and an ‘UL duration’ d for subframe n, then

the scheduled UE may use channel access procedures Type 2 for transmissions in subframes n+l+i where i=0,1,…d-1, irrespective of the channel access Type signalled in the UL grant for those subframes, if the end of UE transmission occurs in or before subframe n+l+d-1.

-	If the ‘UL duration and offset’ field indicates an ‘UL offset’ l and an ‘UL duration’ d for subframe n and the ‘COT sharing indication for AUL’ field is set to ‘1’, then a UE configured with autonomous UL may use channel access procedures Type 2 for autonomous UL transmissions assuming any priority class in subframes n+l+i where i=0,1,…d-1, if the end of UE autonomous UL transmission occurs in or before subframe n+l+d-1 and the autonomous UL transmission between n+l and n+l+d-1 shall be contiguous.

-	If the ‘UL duration and offset’ field indicates an ‘UL offset’ l and an ‘UL duration’d for subframe n and the ‘COT sharing indication for AUL’ field is set to ‘0’, then a UE configured with autonomous UL shall not transmit autonomous UL in subframes n+l+i where i=0,1,…d-1.

If a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0 as desctibed in subclause 11.1.1 of [7], the followings are applicable:

-	The UE may switch from Type 1 channel access procedures as described in subclause 4.2.1.1 to Type 2A channel access procedures as described in subclause 4.2.1.2.1 for its corresponding UL transmissions within the determined duration in time and location in frequency domain of the remaining channel occupancy. In this case, if the UL transmissions are PUSCH transmissions on configured resources, the UE may assume any priority class for the channel occupancy that shares with the gNB.

##### 4.2.1.0.1	Channel access procedures for consecutive UL transmission(s)

For contiguous UL transmission(s), the following are applicable:

-	If a UE is scheduled to transmit a set of  UL transmissions including PUSCH  using a UL grant , and if the UE cannot access the channel for a transmission in the set prior to the last transmission, the UE shall attempt to transmit the next transmission according to the channel access type indicated in the UL grant.

-	If a UE is scheduled to transmit a set of  consecutive UL transmissions without gaps including PUSCH  using one or more UL grant(s) and the UE transmits one of the scheduled UL transmissions in the set after accessing the channel according to one of Type 1 or Type 2 UL channel access procedures, the UE may continue transmission of the remaining UL transmissions in the set, if any.

-	A UE is not expected to be indicated with different channel access types for any consecutive UL transmissions without gaps in between the transmissions.

For UL transmission(s) with multiple starting positions scheduled by eNB, the following are applicable:

-	If a UE is scheduled by an eNB to transmit transmissions including PUSCH Mode 1 using the Type 1 channel access procedure indicated in DCI, and if the UE cannot access the channel for a transmission according to the PUSCH starting position indicated in the DCI, the UE shall attempt to make a transmission at symbol 7 in the same subframe according to Type 1 channel access procedure. There is no limit on the number of attempts the UE can make using Type 1 channel access procedure.

-	If a UE is scheduled by an eNB to transmit transmissions including PUSCH Mode 1 using the Type 2 channel access procedure indicated in DCI, and if the UE cannot access the channel for a transmission according to the PUSCH starting position indicated in the DCI, the UE may attempt to make a transmission at symbol 7 in the same subframe and according to Type 2 channel access procedure. The number of attempts the UE should make within the consecutively scheduled subframes including the transmission is limited to w+1, where w is the number of consecutively scheduled subframes using Type 2 channel access procedure.

For contiguous UL transmissions(s) including a transmission pause, the following are applicable:

-	If a UE is scheduled to transmit a set of  consecutive UL transmissions without gaps  using one or more UL grant(s), and if the UE has stopped transmitting during or before  one of these UL transmissions in the set and prior to the last UL transmission in the set, and if the channel is sensed by the UE to be continuously idle after the UE has stopped transmitting, the UE may transmit a later UL transmission in the set using Type 2 channel access procedure.

-	If a channel sensed by a UE is not continuously idle after the UE has stopped transmitting, the UE may transmit a later UL transmission in the set using Type 1 channel access procedure with the UL channel access priority class indicated in the DCI corresponding to the UL transmission.

For UL transmission(s) following autonomous UL transmission(s), the following are applicable:

-	If a UE is scheduled by an eNB to transmit on channel ci by a UL grant received on channel cj, i≠j, and if the UE is transmitting using autonomous UL on channel ci, the UE shall terminate the ongoing PUSCH transmissions using the autonomous UL at least one subframe before the UL transmission according to the received UL grant.

-	If a UE is scheduled by a UL grant received from an eNB on a channel to transmit a PUSCH transmission(s) starting from subframe n on the same channel using Type 1 channel access procedure and if at least for the first scheduled subframe occupies NRBUL resource blocks and the indicated ‘PUSCH starting position is OFDM symbol zero, and if the UE starts autonomous UL transmissions before subframe n using Type 1 channel access procedure on the same channel, the UE may transmit UL transmission(s) according to the received UL grant from subframe n without a gap, if the priority class value of the performed channel access procedure is larger than or equal to priority class value indicated in the UL grant, and the autonomous UL transmission in the subframe preceding subframe n shall end at the last OFDM symbol of the subframe regardless of the higher layer parameter endingSymbolAUL. The sum of the lengths of the autonomous UL transmission(s) and the scheduled UL transmission(s) shall not exceed the maximum channel occupancy time corresponding to the priority class value used to perform the autonomous uplink channel access procedure. Otherwise, the UE shall terminate the ongoing autonomous UL transmission at least one subframe before the start of the UL transmission according to the received UL grant on the same channel.

##### 4.2.1.0.2	Conditions for maintaining a Type 1 UL channel access procedures

If a UE receives a DCI indicating a UL grant scheduling a PUSCH transmission using Type 1 channel access procedures or indicating a DL grant scheduling a PUCCH transmission using Type 1 channel access procedures, and if the UE has an ongoing Type 1 channel access procedures before the PUSCH or PUCCH transmission starting time:

-	if the UL channel access priority class value p1 used for the ongoing Type 1 channel access procedures is same or larger than the UL channel access priority class value p2 indicated in the DCI, the UE may transmit the PUSCH transmission in response to the UL grant by accessing the channel by using the ongoing Type 1 channel access procedure.

-	if the UL channel access priority class value p1 used for the ongoing Type 1 channel access procedure is smaller than the UL channel access priority class value p2 indicated in the DCI, the UE shall terminate the ongoing channel access procedure.

-	the UE may transmit the PUCCH transmission in response to the DL grant by accessing the channel by using the ongoing Type 1 channel access procedures.

##### 4.2.1.0.3	Conditions for indicating Type 2 channel access procedures

An eNB/gNB may indicate Type 2 channel access procedures in the DCI of a UL grant or DL grant scheduling transmission(s) including PUSCH or PUCCH on a channel, respectively, as follows:

-	The eNB/gNB may indicate Type 2 channel access procedures in the DCI if the eNB/gNB has transmitted on the channel according to the channel access procedures described in subclause 4.1.1, or

-	 The eNB may indicate using the ‘UL duration and offset’ field that the UE may perform a Type 2 channel access procedure for transmissions(s) including PUSCH on a channel in a subframe n when the eNB has transmitted on the channel according to the channel access procedure described in subclause 4.1.1, or

-	The eNB may indicate using the ‘UL duration and offset’ field and ‘COT sharing indication for AUL’ field that a UE configured with autonomous UL may perform a Type 2 channel access procedure for autonomous UL transmissions(s) including PUSCH on a channel in subframe n when the eNB has transmitted on the channel according to the channel access procedure described in subclause 4.1.1 and acquired the channel using the largest priority class value and the eNB transmission includes PDSCH, or

-	The eNB/gNB may schedule UL transmissions on a channel, that follow a transmission by the eNB/gNB on that channel with Type 2A channel access procedures for the UL transmissions as described in subclause 4.2.1.2.1 if the UL transmissions occur within the time interval starting at t0 and ending at t0+TCO, where

-	TCO=Tmcot,p+Tg,

-	t0	 is the time instant when the eNB/gNB has started transmission,

-	Tmcot,p 	value is determined by the eNB/gNB as described in subclause 4.1.1,

-	Tg	 is the total duration of all gaps of duration greater than 25us that occur between the DL transmission of the eNB/gNB and UL transmissions scheduled by the eNB/gNB, and between any two UL transmissions scheduled by the eNB/gNB starting from t0.

The eNB/gNB shall schedule UL transmissions between t0 and t0+TCO without gaps between consecutive UL transmissions if they can be scheduled contiguously.

For a UL transmission on a channel that follows a transmission by the eNB/gNB on that channel using Type 2A channel access procedures as described in subclause 4.2.1.2.1, the UE may use Type 2A channel access procedure for the UL transmission.

If the eNB/gNB indicates Type 2 channel access procedure for the UE in the DCI, the eNB/gNB indicates the channel access priority class used to obtain access to the channel in the DCI.

For indicating a Type 2 channel access procedure, if the gap is at least 25us, or equal to 16us, or up to 16us, the gNB may indicate Type 2A, or Type 2B, or Type 2C UL channel procedures, respectively, as described in subclauses 4.2.1.2.

##### 4.2.1.0.4	Channel access procedures for UL multi-channel transmission(s)

If a UE

-	is scheduled to transmit on a set of channels Cn, and if Type 1 channel access procedure is indicated by the UL scheduling grants for the UL transmissions on the set of channels C, and if the UL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C , or

-	intends to perform an uplink transmission on configured resources on the set of channels C with Type 1 channel access procedure, and if UL transmissions are configured to start transmissions on the same time all channels in the set of channels C, and

if the channel frequencies of set of channels C is a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2]

-	the UE may transmit on channel ci∈C using Type 2 channel access procedure as described in subclause 4.2.1.2,

-	if Type 2 channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and

-	if the UE has accessed channel cj using Type 1 channel access procedure as described in subclause 4.2.1.1,

-	where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C.

- 	the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured by UL resources.

#### 4.2.1.1	Type 1 UL channel access procedure

This subclause describes channel access procedures by a UE where the time duration spanned by the sensing slots that are sensed to be idle before a UL transmission(s) is random. The subclause is applicable to the following transmissions:

-	PUSCH/SRS transmission(s) scheduled or configured by eNB/gNB, or

-	PUCCH transmission(s) scheduled or configured by gNB, or

-	Transmission(s) related to random access procedure.

A UE may transmit the transmission using Type 1 channel access procedure after first sensing the channel to be idle during the slot durations of a defer duration Td; and after the counter N is zero in step 4. The counter N is adjusted by sensing the channel for additional slot duration(s) according to the steps described below.

1)	set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4;

2)	if N>0 and the UE chooses to decrement the counter, set N=N-1;

3)	sense the channel for an additional slot duration, and if the additional slot duration is idle, go to step 4; else, go to step 5;

4)	if N=0, stop; else, go to step 2.

5)	sense the channel until either a busy slot is detected within an additional defer duration Td or all the slots of the additional defer duration Td are detected to be idle;

6)	if the channel is sensed to be idle during all the slot durations of the additional defer duration Td, go to step 4; else, go to step 5;

If a UE has not transmitted a UL transmission on a channel on which UL transmission(s) are performed after step 4 in the procedure above, the UE may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration Tsl when the UE is ready to transmit the transmission and if the channel has been sensed to be idle during all the slot durations of a defer duration Td immediately before the transmission. If the channel has not been sensed to be idle in a sensing slot duration Tsl when the UE first senses the channel after it is ready to transmit, or if the channel has not been sensed to be idle during any of the sensing slot durations of a defer duration Td immediately before the intended transmission, the UE proceeds to step 1 after sensing the channel to be idle during the slot durations of a defer duration Td.

The defer duration Td consists of duration Tf=16us immediately followed by mp consecutive slot durations where each slot duration is Tsl=9us, and Tf includes an idle slot duration Tsl at start of  Tf.

CWmin,p≤CWp≤CWmax,p

CWmin,p≤CWp≤CWmax,p is the contention window. CWp adjustment is described in subclause 4.2.2.

CWmin,p and CWmax,p are chosen before step 1 of the procedure above.

mp, CWmin,p, and CWmax,p are based on a channel access priority class p as shown in Table 4.2.1-1, that is signalled to the UE.

#### 4.2.1.2	Type 2 UL channel access procedure

This subclause describes channel access procedures by UE where the time duration spanned by the sensing slots that are sensed to be idle before a UL transmission(s) is deterministic.

If a UE is indicated by an eNB to perform Type 2 UL channel access procedures, the UE follows the procedures described in subclause 4.2.1.2.1.

#### 4.2.1.2.1	Type 2A UL channel access procedure

If a UE is indicated to perform Type 2A UL channel access procedures, the UE uses Type 2A UL channel access procedures for a UL transmission. The UE may transmit the transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_ul=25us. Tshort_ul consists of a duration Tf=16usimmediately followed by one slot sensing slotTsl=9us and Tfincludes a sensing slot Tslat start of Tf. The channel is considered to be idle for Tshort_ul if both sensing slots  of Tshort_ul.are sensed to be idle.

#### 4.2.1.2.2	Type 2B UL channel access procedure

If a UE is indicated to perform Type 2B UL channel access procedures, the UE uses Type 2B UL channel access procedure for a UL transmission. The UE may transmit the transmission immediately after sensing the channel to be idle within a duration of Tf=16us. Tf includes a sensing slot that occurs within the last 9us of Tf. The channel is considered to be idle within the duration Tf if the channel is sensed to be idle for total of at least 5us with at least 4us of sensing occurring in the sensing slot.

#### 4.2.1.2.3	Type 2C UL channel access procedure

If a UE is indicated to perform Type 2C UL channel access procedures for a UL transmission, the UE does not sense the channel before the transmission. The duration of the corresponding UL transmission is at most 584us.

### 4.2.2	Contention window adjustment procedures

If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, as described in this subclause.

#### 4.2.2.1 	Contention window adjustment procedures for UL transmissions scheduled/configured by eNB

If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, using the following procedure:

-	If the UE receives an UL grant or an AUL-DFI, the contention window size for all the priority classes is adjusted as following:

-	If the NDI value for at least one HARQ process associated with HARQ_ID_ref is toggled, or if the HARQ-ACK value(s) for at least one of the HARQ processes associated with HARQ_ID_ref received in the earliest AUL-DFI after nref+3 indicates ACK,

-	For every priority class p∈1,2,3,4, set CWp=CWmin,p;

-	Otherwise, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value;

-	If there exists one or more previous transmissions {T0, … , Tn} using Type 1 channel access procedure, from the start subframe(s) of the previous transmission(s) of which, N or more subframes have elapsed and neither UL grant nor AUL-DFI was received, where N = max (contentionWindowSizeTimer, Ti burst length+1) if contentionWindowSizeTimer > 0 and N = 0 otherwise, for each transmission Ti, CWp is adjusted as following:

-	increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value;

-	The CWp is adjusted once

-	Else if the UE transmits transmissions using Type 1 channel access procedure before N subframes have elapsed from the start of previous UL transmission burst using Type 1 channel access procedure and neither UL grant nor AUL-DFI is received,

-	the CWp is unchanged.

-	If the UE receives an UL grant or an AUL-DFI indicates feedback for one or more previous transmissions {T0, … , Tn} using Type 1 channel access procedure, from the start subframe(s) of the previous transmission(s) of which, N or more subframes have elapsed and neither UL grant nor AUL-DFI was received, where N = max (contentionWindowSizeTimer, Ti burst length+1) if contentionWindowSizeTimer > 0 and N = 0 otherwise, the UE may recompute CWp as follows:

-	The UE reverts CWp to the value used to transmit at nT0 using Type 1 channel access procedure.

-	The UE updates CWp sequentially in the order of the transmission {T0, … , Tn}

-	If the NDI value for at least one HARQ process associated with HARQ_ID_ref’ is toggled, or if the HARQ-ACK value(s) for at least one of the HARQ processes associated with HARQ_ID_ref’ received in the earliest AUL-DFI after nTi+3 indicates ACK.

-	For every priority class p∈1,2,3,4 set CWp=CWmin,p

-	Otherwise, increase CWp for every priority class p∈1,2,3,4to the next higher allowed value

-	If the UE transmits transmissions using Type 1 channel access procedure before N subframes have elapsed from the start of previous UL transmission burst using Type 1 channel access procedure and neither UL grant nor AUL-DFI is received,

-	CWp is unchanged.

HARQ_ID_ref is the HARQ process ID of UL-SCH in reference subframe nref. The reference subframe nref is determined as follows

-	If the UE receives an UL grant or an AUL-DFI in subframe ng, subframe nwis the most recent subframe before subframe ng-3 in which the UE has transmitted UL-SCH using Type 1 channel access procedure.

-	If the UE transmits transmissions including UL-SCH without gaps starting with subframe n0and in subframes n0,n1,⋯,nw and the UL-SCH in subframe n0 is not PUSCH Mode 1 that starts in the second slot of the subframe, reference subframe nref is subframe n0,

-	If the UE transmits transmissions including PUSCH Mode 1 without gaps starting with second slot of subframe n0 and in subframes n0,n1,⋯,nw and the, reference subframe nref is subframe n0 and n1,

-	otherwise, reference subframe nref is subframe nw,

HARQ_ID_ref’ is the HARQ process ID of UL-SCH in reference subframe nTi. The reference subframe nTi is determined as the start subframe of a transmission Ti using Type 1 channel access procedure and of which, N subframes have elapsed and neither UL grant nor AUL-DFI was received.

If the AUL-DFI with DCI format 0A is indicated to a UE that is activated with AUL transmission and transmission mode 2 is configured for the UE for grant-based uplink transmissions, the spatial HARQ-ACK bundling shall be performed by logical OR operation across multiple codewords for the HARQ process not configured for autonomous UL transmission.

If CWp changes during an ongoing channel access procedure, the UE shall draw a counter Ninit and applies it to the ongoing channel access procedure.

The UE may keep the value of CWp unchanged for every priority class p∈1,2,3,4, if the UE scheduled to transmit transmissions without gaps including PUSCH in a set subframes n0,n1,⋯,nw-1 using Type 1 channel access procedure, and if the UE is not able to transmit any transmission including PUSCH in the set of subframes.

The UE may keep the value of CWp for every priority class p∈1,2,3,4 the same as that for the last scheduled transmission including PUSCH using Type 1 channel access procedure, if the reference subframe for the last scheduled transmission is also nref.

#### 4.2.2.2 	Contention window adjustment procedures for UL transmissions scheduled/configured by gNB

If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, using the following steps:

- For every priority class p∈1,2,3,4, set CWp=CWmin,p;

- If HARQ-ACK feedback is available after the last update of CWp,  go to step 3. Otherwise, if the UE transmission after procedure described in subclause 4.2.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest UL transmission burst after the last update of CWp transmitted after the procedures described in subclause 4.1.1, go to step 5; otherwise go to step 4.

- The HARQ-ACK feedback(s) corresponding to PUSCH(s) in the reference duration for the latest UL transmission burst for which HARQ-ACK feedback is available is used as follows:

  - If at least one HARQ-ACK feedback is ‘ACK’ for PUSCH(s) with transport block (TB) based transmissions or at least 10% of HARQ-ACK feedbacks is ‘ACK’ for PUSCH(s) with code block group (CBG) based transmissions go to step 1; otherwise go to step 4.

- Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value;

- For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2.

The HARQ-ACK feedback, reference duration and duration Tw  in the procedure above are defined as the following:

-	HARQ-ACK feedback for PUSCH(s) transmissions are expected to be provided to UE(s) explicitly or implicitly where implicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause, is determined based on the indication for a new transmission or retransmission in the DCI scheduling PUSCH(s) as follows:

-	If a new transmission is indicated, ‘ACK’ is assumed for the transport blocks or code block groups in the corresponding PUSCH(s) for the TB-based and CBG-based transmission, respectively.

-	If a retransmission is indicated for TB-based transmissions, ‘NACK’ is assumed for the transport blocks in the corresponding PUSCH(s).

-	If a retransmission is indicated for CBG-based transmissions, if a bit value in the code block group transmission information (CBGTI) field is ‘0’ or ‘1’ as described in subclause 5.1.7.2 in [8], ‘ACK’ or ‘NACK’ is assumed for the corresponding CBG in the corresponding PUSCH(s), respectively.

-	The reference duration corresponding to a channel occupancy initiated by the UE including transmission of PUSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PUSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PUSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PUSCH, then, the duration of the first transmission burst by the UE within the channel occupancy that contains PUSCH(s) is the reference duration for CWS adjustment.

-	Tw=max⁡(TA,TB+1ms) where TB is the duraon of the transmission burst from start of the reference duration in  ms and TA=5ms  if the absence of any other technology sharing the channel cannot be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms otherwise.

If a UE transmits transmissions using Type 1 channel access procedures associated with the channel access priority class p on a channel and the transmissions are not associated with explicit or implicit HARQ-ACK feedbacks as described above in this subclause, the UE adjusts CWp before step 1 in the procedures described in subclause 4.2.1.1, using the latest CWp used for any UL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p has not been for any UL transmission on the channel, CWp=CWmin,p is used.

#### 4.2.2.3	Common procedures for CWS adjustments for UL transmissions

The following applies to the procedures described in subclauses 4.1.4.1 and 4.1.4.2:

-	If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p.

-	If the CWp=CWmax,p is consecutively used K times for generation of Ninit, CWp is reset to CWmin,p only for that priority class p for which CWp=CWmax,p is consecutively used K times for generation of Ninit. K is selected by UE from the set of values {1, 2, …,8} for each priority class p∈1,2,3,4.

### 4.2.3	Energy detection threshold adaptation procedure

A UE accessing a channel on which UL transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold XThresh_max.

XThresh_max is determined as follows:

-	If the UE is configured with higher layer parameter maxEnergyDetectionThreshold-r14 or maxEnergyDetectionThreshold-r16 ,

-	XThresh_max 	is set equal to the value signalled by the higher layer parameter.

-	otherwise

-	the UE shall determine X'Thresh_max according to the procedure described in subclause 4.2.3.1

-	if the UE is configured with higher layer parameter energyDetectionThresholdOffset-r14 or energyDetectionThresholdOffset-r16

-	XThresh_max 	is set by adjusting X'Thresh_max according to the offset value signalled by the higher layer parameter

-	otherwise

-	The UE shall set XThresh_max=X'Thresh_max

If the higher layer parameter absenceOfAnyOtherTechnology-r16 is not configured to a UE, and the higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 is configured to the UE, the gNB should use the gNB’s transmit power in determining the resulting energy detection threshold ULtoDL-CO-SharingED-Threshold-r16.

For the case where a UE performs channel access procedures as described in subclause 4.2.1.2.1 and shares its corresponding channel occupancy time with the gNB,  XThresh_max is set equal to the value provided by the higher layer parameter ULtoDL-CO-SharingED-Threshold-r16, if provided.

#### 4.2.3.1	Default maximum energy detection threshold computation procedure

If the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided

-	X'Thresh_max=min&Tmax+10dB&Xr where

-	Xr	 is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB

otherwise

-	X'Thresh_max=max&-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX

Where

-	TA=10dB

-	PH=23dBm;

-	PTX

is the set to the value of PCMAX_H,c as defined in [3].

-	Tmax(dBm)=10⋅log13.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz) ;

-	BWMHz

is the single channel bandwidth in MHz.

## 4.3	Channel access procedures for Semi-static channel occupancy

If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 =’semistatic’ by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Txwitihn every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where  Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1.

In the following procedures in this subclause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9us. The corresponding  XThresh adjustment for performing sensing by a gNB or a UE is described in subclauses 4.1.5 and 4.2.3, respectively.

A channel occupancy initiated by a gNB and shared with UE(s) shall satisfy the following:

-	The gNB shall transmit a DL transmission burst(s) starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current channel occupancy time.

-	The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16us.

-    The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most 16us.

-	A UE may transmit UL transmission burst(s) after DL transmission burst(s) within the channel occupancy time as follows:

-	If the gap between the UL and DL transmission bursts is at most 16us,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel.

-	If the gap between the UL and DL transmission bursts is more than 16us,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl=9us  before transmission.

-	The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100us before the start of the next channel occupancy time.