3GPP TSG-RAN WG1 Meeting #99	R1-1913631

# Reno, USA, November 18-22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0023 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR based access to unlicensed spectrum into 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2019-11-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inclusion of NR based access to unlicensed spectrum |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of NR based access to unlicensed spectrum |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR based access to unlicensed spectrum will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2.7, 6.3.2.1.3(new), 6.3.2.1.4(new), 6.3.2.4.1.2, 6.3.2.4.1.3, 6.3.2.4.1.4(new), 6.3.2.4.1.5(new), 6.3.2.4.2.4(new), 6.3.2.4.2.5(new), 7.1.1, 7.3.1, 7.3.1.1.1, 7.3.1.1.2, 7.3.1.2.1, 7.3.1.2.2, 7.3.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BCH	Broadcast channel

CBG	Code block group

CBGTI	Code block group transmission information

CG	Configured grant

CG-DFI	CG downlink feedback information

CG-UCI	CG uplink control information

CORESET	Control resource set

COT	Channel occupancy time

CQI	Channel quality indicator

CRC	Cyclic redundancy check

CRI	CSI-RS resource indicator

CSI	Channel state information

CSI-RS	CSI reference signal

DAI	Downlink assignment index

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

DMRS	Dedicated demodulation reference signal

HARQ	Hybrid automatic repeat request

HARQ-ACK	Hybrid automatic repeat request acknowledgement

LDPC	Low density parity check

LI	Layer indicator

MCS	Modulation and coding scheme

OFDM	Orthogonal frequency division multiplex

PBCH	Physical broadcast channel

PCH	Paging channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PMI	Precoding matrix indicator

PRB	Physical resource block

PRACH	Physical random access channel

PTRS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

RACH	Random access channel

RI	Rank indicator

RSRP	Reference signal received power

SFN	System frame number

SR	Scheduling request

SRS	Sounding reference signal

SS	Synchronisation signal

SUL	Supplementary uplink

TPC	Transmit power control

TrCH	Transport channel

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

VRB	Virtual resource block

ZP CSI-RS	Zero power CSI-RS

6.2.7	Data and control multiplexing

Denote the coded bits for UL-SCH as .

Denote the coded bits for HARQ-ACK or jointly coded bits for HARQ-ACK and CG-UCI when the high layer parameter cg-CG-UCI-Multiplexing is configured, if any, as .

Denote the coded bits for CSI part 1, if any, as .

Denote the coded bits for CSI part 2, if any, as .

Denote the coded bits for CG-UCI without HARQ-ACK, if any, as g0CG-UCI,  g1CG-UCI,  g2CG-UCI, g3CG-UCI, …, gGCG-UCI-1CG-UCI.

Denote the multiplexed data and control coded bit sequence as .

Denote  as the OFDM symbol index of the scheduled PUSCH, starting from 0 to , where  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS.

Denote  as the subcarrier index of the scheduled PUSCH, starting from 0 to , where  is expressed as a number of subcarriers.

Denote  as the set of resource elements, in ascending order of indices , available for transmission of data in OFDM symbol , for .

Denote  as the number of elements in set . Denote  as the -th element in .

Denote  as the set of resource elements, in ascending order of indices , available for transmission of UCI in OFDM symbol , for . Denote  as the number of elements in set . Denote  as the -th element in . For any OFDM symbol that carriers DMRS of the PUSCH, . For any OFDM symbol that does not carry DMRS of the PUSCH, .

If frequency hopping is configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the second hop.

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the second hop;

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, let

-	 and ;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK, let

- 	GCG-UCI1=NL∙Qm∙GCG-UCI2∙NL∙Qm and GCG-UCI2=NL∙Qm∙GCG-UCI2∙NL∙Qm

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2 or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH,; otherwise,

-	;

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise; and

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise;

-	if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK, let

-	GCSI-part11=minNL∙Qm∙GCSI-part12∙NL∙Qm,  M1∙NL∙Qm-GCG-UCI1 ;

-	GCSI-part12= GCSI-part1-GCSI-part11;

-	GCSI-part21=M1∙NL∙Qm-GCG-UCI1-GCSI-part11; and

-	GCSI-part22=M2∙NL∙Qm-GCG-UCI2-GCSI-part12;

-	if CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	let , and denote ,  as the number of OFDM symbols of the PUSCH in the first and second hop, respectively;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	;

-

-	.

If frequency hopping is not configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS;

-	if HARQ-ACK is present for transmission on the PUSCH or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, let ;

-	if CSI is present for transmission on the PUSCH, let  and ;

-	if CG-UCI is present for transmission on the PUSCH without HARQ-ACK, let GCG-UCI1=GCG-UCI;

-	let  and .

The multiplexed data and control coded bit sequence  is obtained according to the following:

Step 1:

Set  for ;

Set  for ;

Set  for ;

Set  for ;

if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits and without CG-UCI

the number of reserved resource elements for potential HARQ-ACK transmission is calculated according to Subclause 6.3.2.4.1.1, by setting ;

denote  as the number of coded bits for potential HARQ-ACK transmission using the reserved resource elements;

if frequency hopping is configured for the PUSCH, let  and ;

if frequency hopping is not configured for the PUSCH, let ;

denote  as the set of reserved resource elements for potential HARQ-ACK transmission, in OFDM symbol , for ;

Set ;

Set ;

for ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

end for

end if

;

end while

end for

else

for ;

end if

Denote  as the number of elements in .

Step 2:

if HARQ-ACK is present for transmission on the PUSCH and the number of HARQ-ACK information bits is more than 2 or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH,

Set ;

Set ;

Set ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

end if

Step 2A:

If CG-UCI is present for transmission on the PUSCH without HARQ-ACK,

Set  mcountCG-UCI1=0;

Set  mcountCG-UCI2=0;

Set  mcount,allCG-UCI=0;

for i=1 to NhopPUSCH

l=l(i);

while  mcountCG-UCIi<GCG-UCI(i)

if  MscUCIl>0

if  GCG-UCI(i) -mcountCG-UCI1≥MscUCIl.NL.Qm

d=1;

mcountRE=MscUCIl;

end if

if GCG-UCI(i) -mcountCG-UCI1<MscUCIl.NL.Qm

d=MscUCIl.NL.QmGCG-UCI(i) -mcountCG-UCIi;

mcountRE=GCG-UCI(i) -mcountCG-UCIiNL.Qm;

end if

for  j=0 to  mcountRE-1

k=ΦlUCI(j.d);

for v=0 to NL.Qm-1

gl,k,v=gmcount,allCG-UCICG-UCI;

mcount,allCG-UCI=mcount,allCG-UCI+1;

mcountCG-UCIi=mcountCG-UCIi+1;

end for

end for

Φl,tmpUCI=∅;

for j=0 to mcountRE-1

Φl,tmpUCI=Φl,tmpUCI∪ΦlUCI(j.d);

end for

ΦlUCI=ΦlUCI\Φl,tmpUCI;

ΦlUL-SCH=ΦlUL-SCH\Φl,tmpUCI;

MscUCIl=ΦlUCI;

MscUL-SCHl=ΦlUL-SCH;

end if

l=l+1;

end while

end for

end if

Step 3:

if CSI is present for transmission on the PUSCH,

Set ;

Set ;

Set ;

for  to

;

while

;

end while

while

if

if

;

;

end if

if

;

;

end if

;

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

Set ;

Set ;

Set ;

for  to

;

while

;

end while

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

end if

Step 4:

if UL-SCH is present for transmission on the PUSCH,

Set ;

for  to

if

for  to

;

for  to

;

;

end for

end for

end if

end for

end if

Step 5:

if HARQ-ACK is present for transmission on the PUSCH without CG-UCI and the number of HARQ-ACK information bits is no more than 2,

Set ;

Set ;

Set ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

end if

;

end while

end for

end if

Step 6:

Set ;

for  to

for  to

;

for  to

;

;

end for

end for

end for

6.3.2.1.3	CG-UCI

For CG-UCI bits transmitted on a CG PUSCH, the CG-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiCG-UCI  for i=0,1, …, OCG-UCI-1 and A=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.

Table 6.3.2.1.3-1: Mapping order of CG-UCI fields

| Field | Bitwidth |
| --- | --- |
| HARQ process number | 4 |
| Redundancy version | 2 |
| New data indicator | 1 |
| Channel Occupancy Time (COT) sharing information | log2C if both higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 and higher layer parameter cg-COT-SharingList-r16 are configured, where C is the number of combinations configured in cg-COT-SharingList-r16; 1 if higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 is not configured and higher layer parameter cg-COT-SharingOffset-r16 is configured; 0 otherwise; |

6.3.2.1.4	HARQ-ACK and CG-UCI

When higher layer parameter cg-CG-UCI-Multiplexing is configured, the UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows, where A=OCG-UCI+OACK.

-	The CG-UCI bits are mapped to the UCI bit sequencea0, a1, a2, a3, …,aOCG-UCI-1 , where ai=oiCG-UCI for i=0,1, …, OCG-UCI-1. The CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part, and OCG-UCI is number of CG-UCI bits;

- 	The HARQ-ACK bits are mapped to the UCI bit sequence aOCG-UCI, aOCG-UCI+1,  …,aOCG-UCI+OACK-1, where ai+OCG-UCI=oiACK for i=0,1, …, OACK-1. The HARQ-ACK bit sequence o0ACK, o1ACK, …, oOACK-1ACK is given by Subclause 9.1 of [5, TS38.213], and OACK is number of HARQ-ACK bits.

6.3.2.4.1.2	CSI part 1

For CSI part 1 transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined as follows:

QCSI-1'=minOCSI-1+LCSI-1∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'

where

-	 is the number of bits for CSI part 1;

-	if , ; otherwise  is the number of CRC bits for CSI part 1 determined according to Subclause 6.3.1.2.1;

-	;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise, is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	QACK/CG-UCI'=QACK' if HARQ-ACK is present for transmission on the same PUSCH with UL-SCH and without CG-UCI, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH as defined in section 6.3.2.4.1.1 if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is no more than 2 bits, where  is the number of reserved resource elements for potential HARQ-ACK transmission in OFDM symbol , for , in the PUSCH transmission, defined in Subclause 6.2.7; or

QACK/CG-UCI'=QACK' if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmitted on the PUSCH as defined in section 6.3.2.4.1.5; or

QACK/CG-UCI'=QCG-UCI' if CG-UCI is present on the same PUSCH with UL-SCH and without HARQ-ACK, where QCG-UCI' is the number of coded modulation symbols per layer for CG-UCI transmitted on the PUSCH as defined in section 6.3.2.4.1.4;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is configured by higher layer parameter scaling.

For CSI part 1 transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined as follows:

if there is CSI part 2 to be transmitted on the PUSCH,

else

end if

where

-	 is the number of bits for CSI part 1;

-	if , ; otherwise  is the number of CRC bits for CSI part 1 determined according to Subclause 6.3.1.2.1;

-	;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is no more than 2 bits, where  is the number of reserved resource elements for potential HARQ-ACK transmission in OFDM symbol , for , in the PUSCH transmission, defined in Subclause 6.2.7;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is the code rate of the PUSCH, determined according to Subclause 6.1.4.1 of [6, TS38.214];

-	 is the modulation order of the PUSCH.

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

Rate matching is performed according to Subclause 5.4.1 by setting  and the rate matching output sequence length to , where

-	 is the number of code blocks for UCI determined according to Subclause 5.2.1;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

6.3.2.4.1.3	CSI part 2

For CSI part 2 transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as , is determined as follows:

QCSI-2'=minOCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1'

where

-	 is the number of bits for CSI part 2;

-	if , ; otherwise  is the number of CRC bits for CSI part 2 determined according to Subclause 6.3.1.2.1;

-	;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise, is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	QACK/CG-UCI'=QACK' if HARQ-ACK is present for transmission on the same PUSCH with UL-SCH and without CG-UCI, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH as defined in section 6.3.2.4.1.1 if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is 1 or 2 bits;or

QACK/CG-UCI'=QACK' if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, where QACK' is the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmitted on the PUSCH as defined in section 6.3.2.4.1.5; or

QACK/CG-UCI'=QCG-UCI' if CG-UCI is present on the same PUSCH with UL-SCH and without HARQ-ACK, where QCG-UCI' is the number of coded modulation symbols per layer for CG-UCI transmitted on the PUSCH as defined in section 6.3.2.4.1.4;

-	 is the number of coded modulation symbols per layer for CSI part 1 transmitted on the PUSCH;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, .

-	 is configured by higher layer parameter scaling.

For CSI part 2 transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as , is determined as follows:

where

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is 1 or 2 bits;

-	 is the number of coded modulation symbols per layer for CSI part 1 transmitted on the PUSCH;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, .

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

Rate matching is performed according to Subclause 5.4.1 by setting  and the rate matching output sequence length to , where

-	 is the number of code blocks for UCI determined according to Subclause 5.2.1;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

6.3.2.4.1.4	CG-UCI

For CG-UCI transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for CG-UCI transmission, denoted as QCG-UCI', is determined as follows:

QCG-UCI'=minOCG-UCI+LCG-UCI∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=l0Nsymb,allPUSCH-1MscUCIl

where

-    OCG-UCI is the number of CG-UCI bits;

-	LCG-UCI is the number of CRC bits for CG-UCI determined according to Subclause 6.3.1.2.1;

-	βoffsetPUSCH=βoffsetCG-UCI;

-	CUL-SCH is the number of code blocks for UL-SCH of the PUSCH transmission;

-	Kr is the r-th code block size for UL-SCH of the PUSCH transmission;

-	MscPUSCH is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	MscPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the PUSCH transmission;

-	MscUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l, for l=0,1,2,…, Nsymb,allPUSCH-1, in the PUSCH transmission and Nsymb,allPUSCH is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, MscUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, MscUCIl=MscPUSCH- MscPT-RSl;

-	α is configured by higher layer parameter scaling;

-	l0 is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission.

The input bit sequence to rate matching is dr0, dr1, dr2, dr3, …, drNr-1 where r is the code block number, and Nr is the number of coded bits in code block number r.

Rate matching is performed according to Subclause 5.4.1 by setting IBIL=1 and the rate matching output sequence length to Er=EUCICUCI, where

-	CUCI is the number of code blocks for UCI determined according to Subclause 5.2.1;

-	NL is the number of transmission layers of the PUSCH;

-	Qm is the modulation order of the PUSCH;

-	EUCI=NL∙QCG-UCI'∙Qm.

The output bit sequence after rate matching is denoted as fr0, fr1,fr2, …, frEr-1 where Er is the length of rate matching output sequence in code block number r.

6.3.2.4.1.5	HARQ-ACK and CG-UCI

For HARQ-ACK and CG-UCI transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmission, denoted as QACK', is determined as follows:

QACK'=minOACK+OCG-UCI+LACK∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=l0Nsymb,allPUSCH-1MscUCIl

where

-	OACK is the number of HARQ-ACK bits;

-    OCG-UCI is the number of CG-UCI bits;

-	if OACK+OCG-UCI>360, LACK=11; otherwise LACK is the number of CRC bits for HARQ-ACK and CG-UCI determined according to Subclause 6.3.1.2.1;

-	[ βoffsetPUSCH=βoffsetHARQ-ACK; ]

-	CUL-SCH is the number of code blocks for UL-SCH of the PUSCH transmission;

-	Kr is the r-th code block size for UL-SCH of the PUSCH transmission;

-	MscPUSCH is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	MscPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the PUSCH transmission;

-	MscUCIl is the number of resource elements that can be used for transmission of UCI in OFDM symbol l, for l=0,1,2,…, Nsymb,allPUSCH-1, in the PUSCH transmission and Nsymb,allPUSCH is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, MscUCIl=0;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, MscUCIl=MscPUSCH- MscPT-RSl;

-	α is configured by higher layer parameter scaling;

-	l0 is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission.

The input bit sequence to rate matching is dr0, dr1, dr2, dr3, …, drNr-1 where r is the code block number, and Nr is the number of coded bits in code block number r.

Rate matching is performed according to Subclause 5.4.1 by setting IBIL=1 and the rate matching output sequence length to Er=EUCICUCI, where

-	CUCI is the number of code blocks for UCI determined according to Subclause 5.2.1;

-	NL is the number of transmission layers of the PUSCH;

-	Qm is the modulation order of the PUSCH;

-	EUCI=NL∙QACK'∙Qm.

The output bit sequence after rate matching is denoted as fr0, fr1,fr2, …, frEr-1 where Er is the length of rate matching output sequence in code block number r.

6.3.2.4.2.4	CG-UCI

For CG-UCI transmission on PUSCH, the number of coded modulation symbols per layer for CG-UCI transmission, denoted as QCG-UCI', is determined according to Subclause 6.3.2.4.1.4, by setting the number of CRC bits LCG-UCI=0.

The input bit sequence to rate matching is d0, d1, d2, …, dN-1.

Rate matching is performed according to Subclause 5.4.3, by setting the rate matching output sequence length

E=NL∙QCG-UCI'∙Qm, where

-	NL is the number of transmission layers of the PUSCH;

-	Qm is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as f0, f1,f2, …, fE-1.

6.3.2.4.2.5	HARQ-ACK and CG-UCI

For HARQ-ACK and CG-UCI transmission on PUSCH, the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmission, denoted as QACK', is determined according to Subclause 6.3.2.4.1.5, by setting the number of CRC bits LACK=0.

The input bit sequence to rate matching is d0, d1, d2, …, dN-1.

Rate matching is performed according to Subclause 5.4.3, by setting the rate matching output sequence length E=NL∙QACK'∙Qm, where

-	NL is the number of transmission layers of the PUSCH;

-	Qm is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as f0, f1,f2, …, fE-1.

7.1.1	PBCH payload generation

Denote the bits in a transport block delivered to layer 1 by , where  is the payload size generated by higher layers. The lowest order information bit  is mapped to the most significant bit of the transport block as defined in Subclause [6.1.4] of [8, TS 38.321].

Generate the following additional timing related PBCH payload bits , where:

-	 are the 4th, 3rd, 2nd, and 1st LSB of SFN, respectively;

-	 is the half frame bit ;

- if Lmax=10 as defined in Subclause 4.1 of [5, TS38.213],

aA+5 is the MSB of kSSB as defined in Subclause 7.4.3.1 of [4, TS 38.211].

aA+6 is reserved.

aA+7 is the MSB of candidate SS/PBCH block index.

else if Lmax=20 as defined in Subclause 4.1 of [5, TS38.213],

aA+5 is the MSB of kSSB  as defined in Subclause 7.4.3.1 of [4, TS 38.211].

aA+6, aA+7 are the 5th and 4th bits of the candidate SS/PBCH block index, respectively.

end if

-	if  Lmax=64 as defined in Subclause 4.1 of [5, TS38.213],

are the 6th, 5th, and 4th bits of SS/PBCH block index, respectively.

else

is the MSB of  as defined in Subclause 7.4.3.1 of [4, TS 38.211].

are reserved.

end if

Let ; ; ; ; ;

for  to

if  is an SFN bit

;

;

elseif  is the half radio frame bit

elseif

;

;

else

;

;

end if

end for

where Lmax is the number of candidate SS/PBCH blocks in a half frame according to Subclause 4.1 of [5, TS38.213], and the value of  is given by Table 7.1.1-1.

Table 7.1.1-1: Value of PBCH payload interleaver pattern

| 0 | 16 | 4 | 8 | 8 | 24 | 12 | 3 | 16 | 9 | 20 | 14 | 24 | 21 | 28 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 5 | 30 | 9 | 7 | 13 | 2 | 17 | 11 | 21 | 15 | 25 | 22 | 29 | 28 |
| 2 | 18 | 6 | 10 | 10 | 0 | 14 | 1 | 18 | 12 | 22 | 19 | 26 | 25 | 30 | 29 |
| 3 | 17 | 7 | 6 | 11 | 5 | 15 | 4 | 19 | 13 | 23 | 20 | 27 | 26 | 31 | 31 |

7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

Table 7.3.1-1: DCI formats

| DCI format | Usage |
| --- | --- |
| 0_0 | Scheduling of PUSCH in one cell |
| 0_1 | Scheduling of one or multiple PUSCH in one cell, or indicating downlink feedback information for configured grant PUSCH (CG-DFI) |
| 1_0 | Scheduling of PDSCH in one cell |
| 1_1 | Scheduling of PDSCH in one cell, and/or triggering one shot HARQ-ACK codebook feedback |
| 2_0 | Notifying a group of UEs of the slot format, available RB sets, COT duration and search space set group switching |
| 2_1 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE |
| 2_2 | Transmission of TPC commands for PUCCH and PUSCH |
| 2_3 | Transmission of a group of TPC commands for SRS transmissions by one or more UEs |

The fields defined in the DCI formats below are mapped to the information bits  to  as follows.

Each field is mapped in the order in which it appears in the description, including the zero-padding bit(s), if any, with the first field mapped to the lowest order information bit  and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to .

If the number of information bits in a DCI format is less than 12 bits, zeros shall be appended to the DCI format until the payload size equals 12.

The size of each DCI format is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary.

7.3.1.1	DCI formats for scheduling of PUSCH

7.3.1.1.1	Format 0_0

DCI format 0_0 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment –  bits  where   is defined in subclause 7.3.1.0-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

  - bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Subclause 6.3 of [6, TS 38.214]

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured;

-	If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured.

-	If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted.

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment – number of bits determined by the following:

-	bits if the higher layer parameter useInterlacePUSCH-Common-r16 is not configured, where

-	 is the size of the initial UL bandwidth part.

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Subclause 8.3 of [5, TS 38.213], where  if  and  otherwise

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

- if the higher layer parameter useInterlacePUSCH-Common-r16 is configured

  - 5 bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz

  - 6 bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz

-	Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Subclause 6.3 of [6, TS 38.214]

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit, reserved

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits, reserved

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise

Table 7.3.1.1.1-1: UL/SUL indicator

| Value of UL/SUL indicator | Uplink |
| --- | --- |
| 0 | The non-supplementary uplink |
| 1 | The supplementary uplink |

Table 7.3.1.1.1-2: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 00 | 0 |
| 01 | 1 |
| 10 | 2 |
| 11 | 3 |

Table 7.3.1.1.1-3: Frequency hopping indication

| Bit field mapped to index | PUSCH frequency hopping |
| --- | --- |
| 0 | Disabled |
| 1 | Enabled |

Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0

| Bit field mapped to index | Channel Access Type | CP extension |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA |
| 1 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | C3*symbol length – 25 us – TA |
| 2 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | C1*symbol length – 25 us |
| 3 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 0 |

7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback informatin (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Subclause 10.1 of [5, TS38.213].

-	DFI flag – 0 or 1 bit

-	1 bit if the UE is configured to monitor DCI format 0_1 with CRC scrambled by CS-RNTI and for operation in a cell with shared spectrum channel access. For a DCI format 0_1 with CRC scrambled by CS-RNTI, the bit value of 0 indicates activating type 2 CG transmission and the bit value of 1 indicates CG-DFI. For a DCI format 0_1 with CRC scrambled by C-RNTI/SP-CSI-RNTI/MCS-C-RNTI, the bit is reserved.

-	0 bit otherwise;

If DCI format 0_1 is used for indicating CG-DFI, all the remaining fileds are set as follows:

- HARQ-ACK bitmap – [16] bits , where the order of the bitmap to HARQ process index mapping is such that HARQ process indices are mapped in ascending order from MSB to LSB of the bitmap. For each bit of the bitmap, value 1 indicates ACK, and value 0 indicates NACK.

- TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS38.213]

- All the remaining bits in format 0_1 are set to zero.

If DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell or activating type 2 CG transmission, all the remaining fileds are set as follows:

-	UL/SUL indicator – 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only PUCCH carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active UL bandwidth part:

-	if higher layer parameter useInterlacePUSCH-Dedicated-r16 is not configured

-	 bits if only resource allocation type 0 is configured, where  is defined in Subclause 6.1.2.2.1 of [6, TS 38.214],

-	bits if only resource allocation type 1 is configured, or  bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the  LSBs provide the resource allocation as defined in Subclause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

- If the higher layer parameter useInterlacePUSCH-Dedicated-r16 is configured

  - 5 + Y bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz

  - 6 + Y bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz

The value of Y is determined by log2NN+12 where N is the number of RB sets contained in the BWP as defined in subclause x of [x].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment"  field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Subclause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList or pusch-TimeDomainAllocationList-r16 if the higher layer parameter is configured; otherwise I is the number of entries in the default table.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured, or if the higher layer parameter frequencyHopping is not configured, or if only resource allocation type 2 is configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Subclause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214]

-	New data indicator –1 bit if the number of scheduled PUSCH indicated by indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationList-r16

-	Redundancy version –– number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationList-r16, where each bit corresponds to one scheduled PUSCH as defined in subclause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

-	HARQ process number – 4 bits

-	1st downlink assignment index – 1, 2 or 4 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included-r16 configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included-r16 = “enable”.

-	2nd downlink assignment index – 0, 2 or 4 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included-r16 configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included-r16 = “enable”;

-	0 bit otherwise.

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS38.213]

-	SRS resource indicator – or  bits, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook',

-	 bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'nonCodeBook' and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, Lmax is given by that parameter

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset.

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214].

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize.

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PDSCH is not configured or if the number of scheduled PUSCH indicated by the Time domain resource assignment field is largen than 1; otherwise, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

-	UL-SCH indicator – 0 or 1 bit as follows

0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1;

1 bit otherwise. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. Except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s).

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits.  The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16 for operation in a cell with shared spectrum channel access and ChannelAccessMode-r16 = “dynamic”; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16.

A UE does not expect that the bit width of a field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_1 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_1 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell.

If the number of information bits in DCI format 0_1 scheduling a single PUSCH prior to padding is not equal to the number of information bits in DCI format 0_1 scheduling multiple PUSCHs for the same serving cell, zeros shall be appended to the DCI format 0_1 with smaller size until the payload size is the same for scheduling a single PUSCH and multiple PUSCHs.

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field | Bandwidth part |
| --- | --- |
| 2 bits |  |
| 00 | Configured BWP with BWP-Id = 1 |
| 01 | Configured BWP with BWP-Id = 2 |
| 10 | Configured BWP with BWP-Id = 3 |
| 11 | Configured BWP with BWP-Id = 4 |

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transform precoder is disabled and maxRank = 2 or 3 or 4

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled and maxRank = 2

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

Table 7.3.1.1.2-6: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |

Table 7.3.1.1.2-7: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 1 |
| 1 | 2 | 1 | 1 |
| 2 | 2 | 2 | 1 |
| 3 | 2 | 3 | 1 |
| 4 | 2 | 0 | 2 |
| 5 | 2 | 1 | 2 |
| 6 | 2 | 2 | 2 |
| 7 | 2 | 3 | 2 |
| 8 | 2 | 4 | 2 |
| 9 | 2 | 5 | 2 |
| 10 | 2 | 6 | 2 |
| 11 | 2 | 7 | 2 |
| 12-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-8: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6-7 | Reserved | Reserved |

Table 7.3.1.1.2-9: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4-7 | Reserved | Reserved |

Table 7.3.1.1.2-10: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 2-7 | Reserved | Reserved |

Table 7.3.1.1.2-11: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 2-7 | Reserved | Reserved |

Table 7.3.1.1.2-12: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-17: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7-15 | Reserved | Reserved |

Table 7.3.1.1.2-18: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3-15 | Reserved | Reserved |

Table 7.3.1.1.2-19: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank =4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2-15 | Reserved | Reserved |

Table 7.3.1.1.2-20: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 1_1, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerLis set to 3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |

Table 7.3.1.1.2-26: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port |  | Value of LSB | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |  | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 |  | 1 | 2nd DMRS port which shares PTRS port 1 |

Table 7.3.1.1.2-27: void

Table 7.3.1.1.2-28: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 2 | 2 | 2 | 2 |
|  |  | 3 | reserved | 3 | 3 |

Table 7.3.1.1.2-29: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6-7 | reserved | 6 | 0,3 |
|  |  |  |  | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10-15 | reserved |

Table 7.3.1.1.2-30: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14-15 | reserved |

Table 7.3.1.1.2-31: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14 | 0,1,2,3 |
|  |  |  |  | 15 | reserved |

Table 7.3.1.1.2-32: SRI indication for codebook based PUSCH transmission

| Bit field mapped to index | SRI(s), |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

Table 7.3.1.1.2-33: Void

Table 7.3.1.1.2-34: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | [2 or 3] |

Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1, configured by high layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16

| Entry index | Channel Access Type | CP extension | CAPC |
| --- | --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 1 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 2 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 3 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 4 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 1 |
| 5 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 2 |
| 6 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 3 |
| 7 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 4 |
| 8 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 9 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 10 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 11 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 12 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 1 |
| 13 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 2 |
| 14 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 3 |
| 15 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA | 4 |
| 16 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 0 | 1 |
| 17 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 0 | 2 |
| 18 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 0 | 3 |
| 19 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 0 | 4 |
| 20 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 1*symbol length – 25 us | 1 |
| 21 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 1*symbol length – 25 us | 2 |
| 22 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 1*symbol length – 25 us | 3 |
| 23 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 1*symbol length – 25 us | 4 |
| 24 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | C3*symbol length – 25 us – TA | 1 |
| 25 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | C3*symbol length – 25 us – TA | 2 |
| 26 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | C3*symbol length – 25 us – TA | 3 |
| 27 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | C3*symbol length – 25 us – TA | 4 |
| 28 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 0 | 1 |
| 29 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 0 | 2 |
| 30 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 0 | 3 |
| 31 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 0 | 4 |
| 32 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 1*symbol length – 25 us | 1 |
| 33 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 1*symbol length – 25 us | 2 |
| 34 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 1*symbol length – 25 us | 3 |
| 35 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 1*symbol length – 25 us | 4 |
| 36 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C2*symbol length – 16 us – TA | 1 |
| 37 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C2*symbol length – 16 us – TA | 2 |
| 38 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C2*symbol length – 16 us – TA | 3 |
| 39 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C2*symbol length – 16 us – TA | 4 |
| 40 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C3*symbol length – 25 us – TA | 1 |
| 41 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C3*symbol length – 25 us – TA | 2 |
| 42 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C3*symbol length – 25 us – TA | 3 |
| 43 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C3*symbol length – 25 us – TA | 4 |

7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment –  bits where  is given by subclause 7.3.1.0

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index – 6 bits according to ra-PreambleIndex in Subclause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator – 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved

-	SS/PBCH index – 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index – 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Subclause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Reserved bits – 12 bits for operation in a cell with shared spectrum channel access; otherwise 10 bits

Otherwise, all remaining fields are set as follows:

-	Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS 38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits as defined in Subclause 9.1.3 of [5, TS 38.213], as counter DAI

-	TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bits otherwise

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator – 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages – 8 bits, according to Subclause 6.5 of [9, TS38.331]. If only the scheduling information for Paging is carried, this bit field is reserved.

-	Frequency domain resource assignment – bits.  If only the short message is carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214]. If only the short message is carried, this bit field is reserved.

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5. If only the short message is carried, this bit field is reserved.

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message is carried, this bit field is reserved.

-	TB scaling – 2 bits as defined in Subclause 5.1.3.2 of [6, TS38.214]. If only the short message is carried, this bit field is reserved.

-	Reserved bits – 8 bits for operation in a cell with shared spectrum channel access; otherwise 6 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by SI-RNTI:

-	Frequency domain resource assignment – bits

-	 is the size of CORESET 0

-	Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	System information indicator – 1 bit as defined in Table 7.3.1.2.1-2

-	Reserved bits – 17 bits for operation in a cell with shared spectrum channel access; otherwise [15] bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by RA-RNTI:

-	Frequency domain resource assignment – bits

-	 is the size of CORESET 0 if CORESET 0 is configured for the cell and  is the size of initial DL bandwidth part if CORESET 0 is not configured for the cell

-	Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	TB scaling – 2 bits as defined in Subclause 5.1.3.2 of [6, TS38.214]

-	LSBs of SFN – 2 bits as defined in Subclause 8 of [5, TS 38.213] for operation in a cell with shared spectrum channel access; otherwise 0 bit

-	Reserved bits – 14 bits for operation in a cell with shared spectrum channel access; otherwise 16 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment – bits

-	 is the size of CORESET 0

-	Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits, reserved

-	TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; otherwise 0 bit

Table 7.3.1.2.1-1: Short Message indicator

| Bit field | Short Message indicator |
| --- | --- |
| 00 | Reserved |
| 01 | Only scheduling information for Paging is present in the DCI |
| 10 | Only short message is present in the DCI |
| 11 | Both scheduling information for Paging and short message are present in the DCI |

Table 7.3.1.2.1-2: System information indicator

| Bit field | System information indicator |
| --- | --- |
| 0 | SIB1 [9, TS38.331, Subclause 5.2.1] |
| 1 | SI message [9, TS38.331, Subclause 5.2.1] |

7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator – 0 or 3 bits as defined in Subclause 10.1 of [5, TS 38.213].

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active DL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Subclause 5.1.2.2.1 of [6, TS38.214],

-	bits if only resource allocation type 1 is configured, or

-	 bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the LSBs provide the resource allocation as defined in Subclause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as defined in Subclause 5.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Subclause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter is configured; otherwise I is the number of entries in the default table.

-	VRB-to-PRB mapping – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if interleaved VRB-to-PRB mapping is not configured by high layers;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Subclause 7.3.1.6  of [4, TS 38.211].

-	PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType is not configured or is set to 'static', or 1 bit if the higher layer parameter prb-BundlingType is set to 'dynamic' according to Subclause 5.1.2.3 of [6, TS 38.214].

-	Rate matching indicator – 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Subclause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where  is the number of aperiodic ZP CSI-RS resource sets configured by higher layer.

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2):

-	Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Subclause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number – 4 bits

-	Downlink assignment index – number of bits as defined in the following

-	6 bits if more than one serving cell are configured in the DL and the higher layer parameter NFI-TotalDAI-Included-r16 = enable. The 4 MSB bits are the counter DAI and the total DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group.

-	4 bits if only one serving cell are configured in the DL and the higher layer parameter NFI-TotalDAI-Included-r16 = enable. The 2 MSB bits are the counter DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group;

-	4 bits if more than one serving cell are configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook=enhancedDynamic-r16, and NFI-TotalDAI-Included-r16 is not configured, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook=enhancedDynamic-r16, and NFI-TotalDAI-Included-r16 is not configured, where the 2 bits are the counter DAI;

-	0 bits otherwise.

-	TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Subclause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

-	One-shot HARQ-ACK request – 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedback-r16 is configured;

-	0 bit otherwise.

-	PDSCH group index – 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16;

-	0 bit otherwise.

-	New feedback indicator – 0, 1 or 2 bits.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 and the higher layer parameter NFI-TotalDAI-Included-r16 is not configured;

-	2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 and the higher layer parameter NFI-TotalDAI-Included-r16 = enable;

- 	0 bit otherwise.

-	Number of requested PDSCH group(s) – 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16;

-	0 bit otherwise.

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Subclause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_1,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214].

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PDSCH is not configured, otherwise, 2, 4, 6, or 8 bits as defined in Subclause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and maxNrofCodeWordsScheduledByDCI for the PDSCH.

-	CBG flushing out information (CBGFI) – 1 bit if higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE", 0 bit otherwise.

-	DMRS sequence initialization – 1 bit.

-	ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-List-r16 for operation in a cell with shared spectrum channel access and ChannelAccessMode-r16 = “dynamic”; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 are configured by the higher layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16.

If DCI formats 1_1 are monitored in multiple search spaces associated with multiple CORESETs in a BWP for scheduling the same serving cell, zeros shall be appended until the payload size of the DCI formats 1_1 monitored in the multiple search spaces equal to the maximum payload size of the DCI format 1_1 monitored in the multiple search spaces.

Table 7.3.1.2.2-1: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 1 | 0,1 |
| 3 | 2 | 0 |
| 4 | 2 | 1 |
| 5 | 2 | 2 |
| 6 | 2 | 3 |
| 7 | 2 | 0,1 |
| 8 | 2 | 2,3 |
| 9 | 2 | 0-2 |
| 10 | 2 | 0-3 |
| 11 | 2 | 0,2 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4-31 | reserved | reserved | reserved |
| 5 | 2 | 2 | 1 |  |  |  |  |
| 6 | 2 | 3 | 1 |  |  |  |  |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 2 | 0,2 | 1 |  |  |  |  |
| 12 | 2 | 0 | 2 |  |  |  |  |
| 13 | 2 | 1 | 2 |  |  |  |  |
| 14 | 2 | 2 | 2 |  |  |  |  |
| 15 | 2 | 3 | 2 |  |  |  |  |
| 16 | 2 | 4 | 2 |  |  |  |  |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2-31 | reserved | reserved |
| 3 | 2 | 0 |  |  |  |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6-63 | Reserved | Reserved | Reserved |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 3 | 0 | 1 |  |  |  |  |
| 12 | 3 | 1 | 1 |  |  |  |  |
| 13 | 3 | 2 | 1 |  |  |  |  |
| 14 | 3 | 3 | 1 |  |  |  |  |
| 15 | 3 | 4 | 1 |  |  |  |  |
| 16 | 3 | 5 | 1 |  |  |  |  |
| 17 | 3 | 0,1 | 1 |  |  |  |  |
| 18 | 3 | 2,3 | 1 |  |  |  |  |
| 19 | 3 | 4,5 | 1 |  |  |  |  |
| 20 | 3 | 0-2 | 1 |  |  |  |  |
| 21 | 3 | 3-5 | 1 |  |  |  |  |
| 22 | 3 | 0-3 | 1 |  |  |  |  |
| 23 | 2 | 0,2 | 1 |  |  |  |  |
| 24 | 3 | 0 | 2 |  |  |  |  |
| 25 | 3 | 1 | 2 |  |  |  |  |
| 26 | 3 | 2 | 2 |  |  |  |  |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58-63 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-5: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

Table 7.3.1.2.2-6: Allowed entries for DCI format 1_1, configured by high layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16

| Entry index | Channel Access Type | CP extension |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 |
| 1 | Type2C-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA |
| 2 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | 0 |
| 3 | Type2B-ULChannelAccess defined in [subclause 4.2.1.2.3 in 37.213] | C2*symbol length – 16 us – TA |
| 4 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 0 |
| 5 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | 1*symbol length – 25 us |
| 6 | Type2A-ULChannelAccess defined in [subclause 4.2.1.2.1 in 37.213] | C3*symbol length – 25 us – TA |
| 7 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 0 |
| 8 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | 1*symbol length – 25 us |
| 9 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C2*symbol length – 16 us – TA |
| 10 | Type1-ULChannelAccess defined in [subclause 4.2.1.1 in 37.213] | C3*symbol length – 25 us – TA |

7.3.1.3.1	Format 2_0

DCI format 2_0 is used for notifying the slot format, COT duration, available RB set, and search space group switching.

The following information is transmitted by means of the DCI format 2_0 with CRC scrambled by SFI-RNTI:

-	Slot format indicator 1, Slot format indicator 2, …, Slot format indicator N.

-    If the higher layer parameter availableRB-SetPerCell-r16 is configured,

-	Available RB set Indicator 1, Available RB set Indicator 2, …, Available RB set Indicator N1,

-   If the higher layer parameter CO-DurationPerCell-r16 is configured

-	 COT duration indicator 1, COT duration indicator 2, …, COT duration indicator N2.

-   If the higher layer parameter searchSpaceSwitching-r16 = “explicit”

-	Monitoring group flag 1, Monitoring group flag 2, …, Monitoring group flag [M].

The size of DCI format 2_0 is configurable by higher layers up to 128 bits, according to Subclause 11.1.1 of [5, TS 38.213].