3GPP TSG-RAN WG1 Meeting #99	R1-1913685

# Reno, NV, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0032 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of cross-carrier scheduling with different numerologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-12-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of cross-carrier scheduling with different numerologies |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of slot offset. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for cross-carrier scheduling with different numerologies |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.5	Carrier aggregation

Transmissions in multiple cells can be aggregated. Unless otherwise noted, the description in this specification applies to each of the serving cells.

For carrier aggregation of cells with non-aligned frames, the slot offset Nslot, offsetCA between a PCell/PScell A and an SCell B is determined by the difference between the higher-layer parameter CA-slot-offset for cell A and higher-layer parameter CA-slot-offset for cell B. The quantity μoffset is defined as the maximum of the lowest subcarrier spacing configuration among the subcarrier spacings given by the higher-layer parameters SCS-SpecificCarrierList configured for each cell in the cell pair. The slot offset Nslot, offsetCA fulfills

-	when the lowest subcarrier spacing configuration among the subcarrier spacings configured for the cell is μ=2 for both cells or μ=3 for both cells, the start of slot 0 for the cell with point A with the lowest frequency coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell where  q=-1 if the lowest subcarreier spacing configuration given by SCS-SpecificCarrierList of the PCell/PSCell is smaller than the lowest subcarrier spacing given by SCS-SpecificCarrierList for the SCell, otherwise q=1;

-	otherwise, the start of slot 0 for the cell with the lower subcarrier spacing among two cells, or the PCell or PSCell if both cells have the same subcarrier spacing, coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell where  q=-1 if the lowest subcarreier spacing configuration given by SCS-SpecificCarrierList of the PCell/PSCell is smaller than the lowest subcarrier spacing given by SCS-SpecificCarrierList for the SCell, otherwise q=1;