3GPP TSG-RAN WG1 Meeting #99	R1-1913610

# Reno, NV, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0501 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | X | Radio Access Network |  |  |  |  | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Introduction of additional MTC enhancements for LTE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2019-12-05 |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-16 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | Introduction of additional MTC enhancements for LTE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates according to RAN1 agreements for the following new features: - UE-group wake-up signals (6.11B.1) - MPDCCH performance improvement (6.8B.5) - LTE-MTC transmission in LTE control region (6.4.1, 6.8B.5) - Downlink subcarrier puncturing for NR coexistence performance improvement (6.3.5, 6.4.1) - Resource reservation for NR coexistence performance improvement (5.3.4, 5.4.3, 5.5.2.1.2, 5.5.2.1A.4, 5.5.2.2.2, 6.4.1, 6.8B.5, 6.10.3.2, 6.10.3A.2) - Preconfigured uplink resources (5.3.4, 5.5.2.1.1, 6.4.1, 6.8B.5) - Multi-TB scheduling (5.3.4, 5.6A.2, 6.4.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of additional MTC enhancements for LTE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4, 5.4.3, 5.5.2.1.1, 5.5.2.1.2, 5.5.2.1A.4, 5.5.2.2.2, 5.6A.2, 6.3.4, 6.3.5, 6.4.1, 6.8B.4, 6.8B.5, 6.10.3.2, 6.10.3A.2, 6.11B.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  |  | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  |  | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 5	Uplink

## 5.3	Physical uplink shared channel

The baseband signal representing the physical uplink shared channel is defined in terms of the following steps:

-	scrambling

-	modulation of scrambled bits to generate complex-valued symbols

-	mapping of the complex-valued modulation symbols onto one or several transmission layers

-	transform precoding to generate complex-valued symbols

-	precoding of the complex-valued symbols

-	mapping of precoded complex-valued symbols to resource elements

-	generation of complex-valued time-domain SC-FDMA signal for each antenna port

Figure 5.3-1: Overview of uplink physical channel processing

### 5.3.1	Scrambling

For each codeword , the block of bits , where  is the number of bits transmitted in codeword  on the physical uplink shared channel in one subframe/slot/subslot, shall be scrambled with a UE-specific scrambling sequence prior to modulation, resulting in a block of scrambled bits  according to the following pseudo code

Set i = 0

while

if 	// ACK/NACK or Rank Indication placeholder bits

else

if 	// ACK/NACK or Rank Indication repetition placeholder bits

else	// Data or channel quality coded bits, Rank Indication coded bits or ACK/NACK coded bits

end if

end if

i = i + 1

end while

where x and y are tags defined in 3GPP TS 36.212 [3] clause 5.2.2.6 and where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  corresponds to the RNTI associated with the PUSCH transmission as described in clause 8 in 3GPP TS 36.213 [4]. For AUL PUSCH, nRNTI=0.

For BL/CE UEs, the same scrambling sequence is applied per subframe to PUSCH for a given block of  subframes. The subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies . For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first uplink subframe intended for PUSCH. The PUSCH transmission spans  consecutive subframes including non-BL/CE UL subframes where the UE postpones the PUSCH transmission. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2.

For PUSCH with a subframe duration, up to two codewords can be transmitted in one subframe, i.e., . In the case of single-codeword transmission, .

### 5.3.2	Modulation

For each codeword, the block of scrambled bits  shall be modulated as described in clause 7.1, resulting in a block of complex-valued symbols . Table 5.3.2-1 specifies the modulation mappings applicable for the physical uplink shared channel. For sub-PRB allocations only π/2 BPSK and QPSK are supported.

Table 5.3.2-1: Uplink modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PUSCH | π/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM |

### 5.3.2A	Layer mapping

The complex-valued modulation symbols for each of the codewords to be transmitted are mapped onto one or two layers. Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is the number of layers and  is the number of modulation symbols per layer.

#### 5.3.2A.1	Layer mapping for transmission on a single antenna port

For transmission on a single antenna port, a single layer is used, , and the mapping is defined by

with .

#### 5.3.2A.2	Layer mapping for spatial multiplexing

For spatial multiplexing, the layer mapping shall be done according to Table 5.3.2A.2-1. The number of layers  is less than or equal to the number of antenna ports  used for transmission of the physical uplink shared channel. 
The case of a single codeword mapped to multiple layers is only applicable when the number of antenna ports used for PUSCH is four, except for slot-PUSCH and subslot-PUSCH transmission where a single codeword is used irrespective of the number of layers.

Table 5.3.2A.2-1: Codeword-to-layer mapping for spatial multiplexing

| Number of layers | Number of codewords | Codeword-to-layer mapping |  |
| --- | --- | --- | --- |
| 1 | 1 |  |  |
| 2 | 1 |  |  |
| 2 | 2 |  |  |
| 3 | 2 |  |  |
| 4 | 2 |  |  |
| 41 | 11 |  |  |
| NOTE 1: Only used for slot-PUSCH and subslot-PUSCH |  |  |  |

### 5.3.3	Transform precoding

For each layer  the block of complex-valued symbols  is divided into  sets, each corresponding to one SC-FDMA symbol. Transform precoding shall be applied according to

resulting in a block of complex-valued symbols . The variable, where  represents the bandwidth of the PUSCH in terms of resource blocks, and shall fulfil

where  is a set of non-negative integers.

In case of PUSCH transmissions using sub-PRB allocations for BL/CE UEs, the variable .

### 5.3.3A	Precoding

The precoder takes as input a block of vectors ,  from the transform precoder and generates a block of vectors ,  to be mapped onto resource elements.

#### 5.3.3A.1	Precoding for transmission on a single antenna port

For transmission on a single antenna port, precoding is defined by

where , .

#### 5.3.3A.2	Precoding for spatial multiplexing

Precoding for spatial multiplexing is only used in combination with layer mapping for spatial multiplexing as described in clause 5.3.2A.2. Spatial multiplexing supports  or  antenna ports where the set of antenna ports used for spatial multiplexing is  and , respectively.

Precoding for spatial multiplexing is defined by

where , .

The precoding matrix  of size  is given by one of the entries in Table 5.3.3A.2-1 for  and by Tables 5.3.3A.2-2 through 5.3.3A.2-5 for  where the entries in each row are ordered from left to right in increasing order of codebook indices.

Table 5.3.3A.2-1: Codebook for transmission on antenna ports

| Codebook index | Number of layers |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  | - |
| 2 |  | - |
| 3 |  | - |
| 4 |  | - |
| 5 |  | - |

Table 5.3.3A.2-2: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 7 |  |  |  |  |  |  |  |  |
| 8 – 15 |  |  |  |  |  |  |  |  |
| 16 – 23 |  |  |  |  |  |  |  |  |

Table 5.3.3A.2-3: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |
| 8 – 11 |  |  |  |  |
| 12 – 15 |  |  |  |  |

Table 5.3.3A.2-4: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |
| 8 – 11 |  |  |  |  |

Table 5.3.3A.2-5: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |
| --- | --- |
| 0 |  |

### 5.3.4	Mapping to physical resources

For each antenna port  used for transmission of the PUSCH in a subframe the block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in clause 5.1.1.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to physical resource blocks on antenna port  and assigned for transmission of PUSCH. The relation between the index  and the antenna port number  is given by Table 5.2.1-1. The mapping to resource elements  corresponding to the physical resource blocks assigned for transmission shall fulfil the following criteria:

-	not used for transmission of reference signals, and

-	not part of the last SC-FDMA symbol in a subframe, if the UE transmits SRS in the same subframe in the same serving cell, and

-	not part of the last SC-FDMA symbol in a subframe configured with cell-specific SRS for non-BL/CE UEs and BL/CE UEs in CEModeA, if the PUSCH transmission partly or fully overlaps with the cell-specific SRS bandwidth, and

-	not part of an SC-FDMA symbol reserved for possible SRS transmission in a UE-specific aperiodic SRS subframe in the same serving cell, and

-	not part of an SC-FDMA symbol reserved for possible SRS transmission in a UE-specific periodic SRS subframe in the same serving cell when the UE is configured with multiple TAGs

-	not part of the first SC-FDMA symbol in a subframe if the associated DCI indicates PUSCH starting position '01', '10', or '11' and does not indicate PUSCH mode 2.

-	not part of the first SC-FDMA symbol in the second slot in a subframe if the associated DCI indicates PUSCH starting position '01', '10', or '11' and PUSCH mode 2.

-	not part of the last SC-FDMA symbol in a subframe if the associated DCI indicates PUSCH ending symbol '1' and does not indicate PUSCH mode 3.

-	not part of the second slot in a subframe if the associated DCI indicates PUSCH ending symbol '0' and PUSCH mode 3.

-	not part of SC-FDMA symbols 5 to 13 in a subframe if the associated DCI indicates PUSCH ending symbol '1' and PUSCH mode 3.

The mapping to resource elements  shall be in increasing order of first the index , then the index . The mapping starts with the first slot in an uplink subframe, except for slot-PUSCH, subslot-PUSCH transmission, or PUSCH mode 2.

In case of PUSCH transmissions using sub-PRB allocations for BL/CE UEs, the mapping starts over in every valid uplink subframe composing an UL resource unit.

In case of slot-PUSCH, the mapping shall start at  in the slot assigned for transmission.

In case of PUSCH mode 2, the mapping shall start at  in the second slot of the subframe assigned for transmission.

In case of subslot-PUSCH, the mapping shall start at symbol  where the start of the mapping is dependent on the uplink subslot number in the subframe assigned for transmission and the DMRS-pattern field in the related uplink DCI format [3] according to Table 5.3.4-1 where starting symbol index "4" for subslot #5 is applied if the UE has indicated the capability ul-pattern-ddd-r15.

Table 5.3.4-1: Starting symbol index for subslot-PUSCH transmission

| DMRS-pattern field in uplink-related DCI format [3] | Uplink subslot number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 |
| 00 | 1 | 4 | 6 | 1 | 3 | 5 |
| 01 | 0 | 3 | 5 | 0 | 2 | 4 |
| 10 | – | 3 | – | 0 | 2 | – |
| 11 | – | 3 | – | – | 2 | – |

In case of a semi-persistently scheduled subslot-PUSCH, and semi-persistent scheduling (i.e. higher layer parameter sps-ConfigUL-STTI is configured, see 3GPP TS 36.331 [9]) with a configured periodicity of 1 subslot (i.e. semiPersistSchedIntervalUL-STTI set to sTTI1), the mapping shall start at symbol  depending on the DMRS-pattern field in the related uplink DCI format [3] according to Table 5.3.4-2.

In case of a semi-persistently scheduled subslot-PUSCH and semi-persistent scheduling (the higher layer parameter sps-ConfigUL-sTTI-r15 is configured, see 3GPP TS 36.331 [9]) with repetitions enabled (the higher layer parameter totalNumberPUSCH-SPS-STTI-UL-Repetitions is configured), the mapping shall start at symbol  depending on the DMRS-pattern field in the related uplink DCI format [3] according to Table 5.3.4-2.

Table 5.3.4-2: Starting symbol index for subslot-PUSCH transmission in case of semi-persistent scheduling with a configured periodicity of 1 subslot

| DMRS-pattern field in uplink-related DCI format [3] | Uplink subslot number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 |
| 00 | 1 | 4 | 6 | 1 | 3 | 5 |
| 10 | 1 | 3 | 6 | 0 | 3 | 5 |

In case of subslot-PUSCH and semi-persistent scheduling with a configured periodicity longer than 1 subslot the mapping shall start at symbol  according to the first row of Table 5.3.4-2 (i.e. equivalent to a signalling of DMRS-pattern field set to '00').

For the UpPTS, the mapping shall start at symbol  and if dmrsLess-UpPts is set to true the mapping shall end at symbol  in the second slot of a special subframe, otherwise, the mapping shall end at symbol  in the second slot of a special subframe.

For BL/CE UEs, the PUSCH transmission is restricted as follows:

-	For CEModeA, if the PUSCH is associated with C-RNTI or SPS C-RNTI and the higher layer parameter ce-pusch-maxBandwidth-config is set to 5 MHz, the maximum number of allocatable PRBs for PUSCH is 24 PRBs. The allocatable PRBs include the PRBs belonging to the narrowbands defined in clause 5.2.4 and the odd PRB at the center of the uplink system bandwidth in case of odd total number of uplink PRBs. If a resource assignment or frequency hopping would result in a PUSCH resource allocation outside the allocatable PRBs then the PUSCH transmission in that subframe is dropped.

-	For all other cases, the maximum number of allocatable PRBs for PUSCH is 6 PRBs restricted to one of the narrowbands defined in clause 5.2.4.

For BL/CE UEs in CEModeB, resource elements in the last SC-FDMA symbol in a subframe configured with cell-specific SRS shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

For BL/CE UEs, if one or more SC-FDMA symbol(s) are left empty due to guard period for narrowband or wideband retuning, the affected SC-FDMA symbol(s) shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

For a UE configured with SRS carrier switching, if the first symbol in a subframe overlaps with an SRS transmission (including any interruption due to uplink or downlink RF retuning time) in a carrier without PUSCH/PUCCH, the resource elements in the first SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

For a UE configured with SRS carrier switching, if the last symbol in a subframe is counted in the PUSCH mapping and the last symbol in the subframe overlaps with an SRS transmission (including any interruption due to uplink or downlink RF retuning time) in a carrier without PUSCH/PUCCH, the resource elements in the last SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

For a UE configured with SRS carrier switching, if the last symbol in a subframe is not counted in the PUSCH mapping and the second-to-last symbol in the subframe overlaps with an SRS transmission (including any interruption due to uplink or downlink RF retuning time) in a carrier without PUSCH/PUCCH, the resource elements in the second-to-last SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

For a UE configured with PUSCH Mode 1, if DCI indicates PUSCH mode 1 enabled and the corresponding transmission of PUSCH starts in the second slot of a subframe, the resource elements in the first slot of the subframe shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

For a UE configured with autonomous uplink,

-	if the UE indicates PUSCH ending symbol '1' in uplink control information, or endingSymbolAUL is set to '12', the resource elements in the last SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH;

-	if the UE indicates PUSCH starting symbol '1' in uplink control information, the resource elements in the first SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

If uplink frequency-hopping is disabled or the resource blocks allocated for PUSCH transmission are not contiguous in frequency, the set of physical resource blocks to be used for transmission is given by  where  is obtained from the uplink scheduling grant as described in clause 8.1 in 3GPP TS 36.213 [4].

If uplink frequency-hopping with type 1 PUSCH hopping is enabled, the set of physical resource blocks to be used for transmission is given by clause 8.4.1 in 3GPP TS 36.213 [4].

If uplink frequency-hopping with predefined hopping pattern is enabled, the set of physical resource blocks to be used for transmission in slot  is given by the scheduling grant together with a predefined pattern according to

where  is obtained from the scheduling grant as described in clause 8.1 in 3GPP TS 36.213 [4]. The parameter pusch-HoppingOffset,, is provided by higher layers. The size  of each sub-band is given by,

where the number of sub-bands  is given by higher layers. The function  determines whether mirroring is used or not. The parameter Hopping-mode provided by higher layers determines if hopping is "inter-subframe" or "intra and inter-subframe".

The hopping function and the function are given by

where  and the pseudo-random sequence  is given by clause 7.2 and CURRENT_TX_NB indicates the transmission number for the transport block transmitted in slot as defined in [8]. The pseudo-random sequence generator shall be initialised with  for frame structure type 1 and  for frame structure type 2 at the start of each frame.

For BL/CE UEs, the PRB resources for PUSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.10 and 5.3.3.1.11 in [3]. Each of the NTB≥1 PUSCH codewords is transmitted with repetitions, where NTB is the number of transport blocks defined in clause 8.0 of 3GPP TS 36.213 [4]. The PUSCH transmission spans NabsPUSCH≥NTBNrepPUSCH consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUSCH transmission if .

-	If higher layer parameter ce-reserved-resource-UL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PUSCH transmission with  associated with C-RNTI or SPS C-RNTI,

-	In a subframe that is fully reserved, the PUSCH transmission is postponed until the next BL/CE uplink subframe that is not fully reserved.

-	In a subframe that is partially reserved, the reserved SC-FDMA symbols shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

-	In case the UE is a BL/CE UE configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, the PUSCH transmission spans  consecutive subframes including non-BL/CE UL subframes where the UE postpones the PUSCH transmission.

-	For BL/CE UE in CEModeA, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set and the frequency hopping flag in DCI format 6-0A indicates frequency hopping, otherwise frequency hopping is disabled.

-	For BL/CE UE in CEModeB, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set, otherwise frequency hopping is disabled.

-	If frequency hopping is not enabled for PUSCH, all PUSCH repetitions are located at the same PRB resources.

-	If a BL/CE UE is configured with CEMode A and higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig, the UE is not expected to have the frequency hopping enabled for PUSCH with the resource allocation including the center PRB not belonging to any narrowband.

-	If frequency hopping is enabled for PUSCH and the UE is not configured with CEModeA and higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig,

-	PUSCH is transmitted in uplink subframe  within the  consecutive subframes using the same number of consecutive PRBs as in the previous subframe starting from the PRB resources of the narrowband  with the same RIV as that of narrowband . The narrowband  is defined as

where  is the absolute subframe number of the first UL subframe intended for carrying the PUSCH and  and  are cell-specific higher-layer parameters. For the  consecutive subframes, the UE shall not transmit PUSCH in subframe  if it is not a BL/CE UL subframe.

-	If frequency hopping is enabled for PUSCH and the UE is configured with CEModeA and higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig,

-	Except when the PUSCH resource allocation includes the center PRB not belonging to any narrowband, PUSCH is transmitted in uplink subframe  within the  consecutive subframes using the same number of consecutive PRBs as in the previous subframe, where nNB(i0) is the narrowband index that starting PRB located in the absolute subframe number of the first UL subframe i0, defined as

-	If NRBUL mod 2=0 or NRBUL mod 2=1 with RBSTART<NRBUL2,  nNB(i0) =RBSTART-le6

-	If NRBUL mod 2=1 with RBSTART>NRBUL2,  nNB(i0) =RBSTART-le-16

where le=NRBUL2-6NNBUL2 is the number of edge PRB(s) not belonging to narrowbands in one side of system bandwidth NRBUL, NNBUL is the number of narrowbands, the starting PRB index RBSTART and the length  LCRBs of the allocated resources are defined in clause 8.1.1 of [4]. After hopping, the narrowband  in subframe  is defined as

where  and  are cell-specific higher-layer parameters. For the   consecutive subframes, the UE shall not transmit PUSCH in subframe  if it is not a BL/CE UL subframe. After hopping, the resource blocks have the same relative location of starting PRB in nNB(i) as in narrowband nNB(i0).

-	If a frequency hopping leads to a split resource allocation, where some PRB(s) is (are) on one edge and some PRB(s) is (are) on the other edge of the system bandwidth, the PUSCH transmission is dropped in that subframe.

-	If a frequency hopping leads to a resource allocation, where some PRB(s) is (are) not belonging to any narrowband, the PUSCH transmission is dropped in that subframe.

For BL/CE UEs, for PUSCH transmission corresponding to the random access response grant and its retransmission, frequency hopping of the PUSCH is enabled when higher layer parameter rar-HoppingConfig is set. Further

-	if PRACH CE level 0 or 1 is used for the last PRACH attempt,  is set to the higher layer parameter interval-UlHoppingConfigCommonModeA;

-	if PRACH CE level 2 or 3 is used for the last PRACH attempt,  is set to the higher layer parameter interval-UlHoppingConfigCommonModeB.

For BL/CE UEs in CEModeB, for PUSCH transmission not associated with Temporary C-RNTI, for frame structure type 1, after a transmission duration of  time units (which may include non-BL/CE UL subframes), a gap of  time units shall be inserted, according to the UE capability ue-CE-NeedULGaps, as specified in TS 36.331 [9]. BL/CE UL subframes within the gap of  time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

For BL/CE UEs, for PUSCH transmission associated with Temporary C-RNTI for frame structure type 1, and if PRACH CE level 2 or 3 is used for the last PRACH attempt, after a transmission duration of  time units (which may include non-BL/CE UL subframes), a gap of  time units shall be inserted. BL/CE UL subframes within the gap of  time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

For UEs configured with PUSCH-EnhancementsConfig, the number of PUSCH subframe repetitions  and the PRB resources for PUSCH transmission in the first subframe are obtained from the DCI as described in clause 5.3.3.1.1C in [3]. The PUSCH transmission spans  consecutive subframes, including DL subframes where the UE postpones the PUSCH transmission in the case of frame structure type 2. PUSCH frequency hopping is enabled when the higher-layer parameters pusch-HoppingOffsetPUSCH-Enh and interval-ULHoppingPUSCH-Enh are set and the frequency hopping flag in DCI format 0C indicates frequency hopping, otherwise frequency hopping is disabled. If frequency hopping is not enabled for PUSCH, the PUSCH repetitions are located at the same PRB resources as in the first subframe. If frequency hopping is enabled for PUSCH, PUSCH is transmitted in uplink subframe  within the  consecutive subframes using the PRB resources starting at PRB index

nPRBi=nPRBi0if iNPRB,hopPUSCH-j0mod 2=0 nPRBi0+fPRB,hopPUSCHmod NPRBULif iNPRB,hopPUSCH-j0mod 2=1

j0=i0NPRB,hopPUSCH, i0≤i≤i0+ NabsPUSCH-1

where  is the absolute subframe number of the first UL subframe carrying the PUSCH and  is given by the higher-layer parameter interval-ULHoppingPUSCH-Enh and  is given by the higher-layer parameter pusch-HoppingOffsetPUSCH-Enh.

## 5.4	Physical uplink control channel

The physical uplink control channel, PUCCH, carries uplink control information. Simultaneous transmission of PUCCH and PUSCH from the same UE is supported if enabled by higher layers. For frame structure type 2, the PUCCH is not transmitted in the UpPTS field.

The physical uplink control channel supports multiple formats as shown in Table 5.4-1 with different number of bits per subframe, where  represents the bandwidth of the PUCCH format 4 as defined by clause 5.4.2B, and  and  are defined in Table 5.4.2C-1.

Formats 2a and 2b are supported for normal cyclic prefix only.

Table 5.4-1: Supported PUCCH formats

| PUCCH format | Modulation scheme | Number of bits per subframe, |
| --- | --- | --- |
| 1 | N/A | N/A |
| 1a | BPSK | 1 |
| 1b | QPSK | 2 |
| 2 | QPSK | 20 |
| 2a | QPSK+BPSK | 21 |
| 2b | QPSK+QPSK | 22 |
| 3 | QPSK | 48 |
| 4 | QPSK |  |
| 5 | QPSK |  |

All PUCCH formats use a cyclic shift, , which varies with the symbol number  and the slot number  according to

where the pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with , where  is given by clause 5.5.1.5 with  corresponding to the primary cell, at the beginning of each radio frame.

The physical resources used for PUCCH format 1/1a/1b and PUCCH format 2/2a/2b depends on two parameters,  and , given by higher layers. 
The variable  denotes the bandwidth in terms of resource blocks that are available for use by PUCCH formats 2/2a/2b transmission in each slot. The variable  denotes the number of cyclic shift used for PUCCH formats 1/1a/1b in a resource block used for a mix of formats 1/1a/1b and 2/2a/2b. The value of  is an integer multiple of  within the range of {0, 1, …, 7}, where  is provided by higher layers. No mixed resource block is present if . At most one resource block in each slot supports a mix of formats 1/1a/1b and 2/2a/2b. 
Resources used for transmission of PUCCH formats 1/1a/1b, 2/2a/2b, 3, 4, and 5 are represented by the non-negative indices , , ,  and , respectively.

### 5.4.1	PUCCH formats 1, 1a and 1b

For PUCCH format 1, information is carried by the presence/absence of transmission of PUCCH from the UE. 
In the remainder of this clause,  shall be assumed for PUCCH format 1.

For PUCCH formats 1a and 1b, one or two explicit bits are transmitted, respectively. The block of bits  shall be modulated as described in Table 5.4.1-1, resulting in a complex-valued symbol. 
The modulation schemes for the different PUCCH formats are given by Table 5.4-1.

The complex-valued symbol  shall be multiplied with a cyclically shifted length  sequence  for each of the  antenna ports used for PUCCH transmission according to

where  is defined by clause 5.5.1 with and . The antenna-port specific cyclic shift  varies between symbols and slots as defined below.

The block of complex-valued symbols  shall be scrambled by  and block-wise spread with the antenna-port specific orthogonal sequence  according to

where

and

with  for the two slots in a subframe given by Table 5.4.1-1a. The sequence  is given by Table 5.4.1-2 and Table 5.4.1-3 and  is defined below.

Resources used for transmission of PUCCH format 1, 1a and 1b are identified by a resource index  from which the orthogonal sequence index  and the cyclic shift  are determined according to

where

The resource indices within the two resource blocks in the two slots of a subframe to which the PUCCH is mapped are given by

for  and by

for , where , with for normal CP and for extended CP.

The parameter deltaPUCCH-Shift  is provided by higher layers.

Table 5.4.1-1: Modulation symbol  for PUCCH formats 1a and 1b

| PUCCH format |  |  |
| --- | --- | --- |
| 1a | 0 |  |
|  | 1 |  |
| 1b | 00 |  |
|  | 01 |  |
|  | 10 |  |
|  | 11 |  |

Table 5.4.1-1a: The quantity  for PUCCH formats 1a and 1b

| PUCCH format |  |  |
| --- | --- | --- |
|  | first slot | second slot |
| normal 1/1a/1b | 4 | 4 |
| shortened 1/1a/1b | 4 | 3 |

Table 5.4.1-2: Orthogonal sequences  for

| Sequence index | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

Table 5.4.1-3: Orthogonal sequences  for

| Sequence index | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

### 5.4.2	PUCCH formats 2, 2a and 2b

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols .

Each complex-valued symbol  shall be multiplied with a cyclically shifted length  sequence  for each of the  antenna ports used for PUCCH transmission according to

where  is defined by clause 5.5.1 with and .

Resources used for transmission of PUCCH formats 2/2a/2b are identified by a resource index  from which the cyclic shift  is determined according to

where

and

for  and by

for .

For PUCCH formats 2a and 2b, supported for normal cyclic prefix only, the bit(s)  shall be modulated as described in Table 5.4.2-1 resulting in a single modulation symbol  used in the generation of the reference-signal for PUCCH format 2a and 2b as described in clause 5.5.2.2.1.

Table 5.4.2-1: Modulation symbol  for PUCCH formats 2a and 2b

| PUCCH format |  |  |
| --- | --- | --- |
| 2a | 0 |  |
|  | 1 |  |
| 2b | 00 |  |
|  | 01 |  |
|  | 10 |  |
|  | 11 |  |

### 5.4.2A	PUCCH format 3

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The complex-valued symbols  shall be block-wise spread with the orthogonal sequences  and  resulting in  sets of  values each according to

where  for both slots in a subframe using normal PUCCH format 3 and , holds for the first and second slot, respectively, in a subframe using shortened PUCCH format 3. The orthogonal sequences  and  are given by Table 5.4.2A-1. Resources used for transmission of PUCCH format 3 are identified by a resource index  from which the quantities  and  are derived according to

Each set of complex-valued symbols shall be cyclically shifted according to

where  is given by Subclause 5.4,  is the slot number within a radio frame and  is the SC-FDMA symbol number within a slot.

The shifted sets of complex-valued symbols shall be transform precoded according to

where  is the number of antenna ports used for PUCCH transmission, resulting in a block of complex-valued symbols .

Table 5.4.2A-1: The orthogonal sequence

| Sequence index | Orthogonal sequence |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  | - |

### 5.4.2B	PUCCH format 4

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The block of complex-valued symbols  is divided into  sets, each corresponding to one SC-FDMA symbol. Transform precoding shall be applied according to

where ,  and  are given by Table 5.4.2C-1 for normal PUCCH format 4 and shortened PUCCH format 4, resulting in a block of complex-valued symbols . The variable, where  represents the bandwidth of the PUCCH format 4 in terms of resource blocks, shall fulfil

where  is a set of non-negative integers.

### 5.4.2C	PUCCH format 5

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The complex-valued symbols  shall be divided into  sets, each corresponding to one SC-FDMA symbol. Block-wise spreading shall be applied according to

where ,  and  are given by Table 5.4.2C-1 for normal PUCCH format 5 and shortened PUCCH format 5, and  is given by Table 5.4.2C-2 with  provided by higher layers.

The block-wise spread complex-valued symbols shall be transform precoded according to

where , resulting in a block of complex-valued symbols .

Table 5.4.2C-1: The quantities  and

| PUCCH format type | Normal cyclic prefix |  | Extended cyclic prefix |  |
| --- | --- | --- | --- | --- |
| Normal PUCCH format | 6 | 6 | 5 | 5 |
| Shortened PUCCH format | 6 | 5 | 5 | 4 |

Table 5.4.2C-2: Orthogonal sequences

|  | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |

### 5.4.3	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in Subclause 5.1.2.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to resource elements. PUCCH uses one or more resource block in each of the two slots in a subframe. Within the physical resource block(s) used for transmission, the mapping of  to resource elements  on antenna port  and not used for transmission of reference signals shall be in increasing order of first , then  and finally the slot number, starting with the first slot in the subframe. The relation between the index  and the antenna port number  is given by Table 5.2.1-1.

For non-BL/CE UEs, except for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

For BL/CE UEs, PUCCH is transmitted with  repetitions. The PUCCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUCCH transmission if .

-	The quantity  is given

-	by the higher layer parameter pucch-NumRepetitionCE-Format1 for PUCCH format 1/1a and pucch-NumRepetitionCE-Format2 for PUCCH format 2/2a/2b, if configured. Otherwise

-	by the higher-layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13.

-	If higher layer parameter ce-reserved-resource-UL-time is configured, then in case of PUCCH transmission with ,

-	In a subframe that is fully reserved, the PUCCH transmission is postponed until the next BL/CE uplink subframe that is not fully reserved.

-	In a subframe that is partially reserved, the reserved SC-FDMA symbols shall be counted in the PUCCH mapping but not used for transmission of the PUCCH.

The physical resource blocks to be used for transmission of PUCCH in subframe  within the  consecutive subframes are given by

where  is the absolute subframe number of the first uplink subframe intended for PUCCH.

The variable  depends on the PUCCH format.

-	Formats 1, 1a and 1b:

-	Formats 2, 2a and 2b:

-	Format 3:

-	Format 5 (non-BL/CE UEs only):

For non-BL/CE UEs, for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

where  is obtained from [4].

Mapping of modulation symbols for the physical uplink control channel for PUCCH formats 1 – 3 is illustrated in Figure 5.4.3-1.

In case of simultaneous transmission of sounding reference signal and PUCCH format 1, 1a, 1b, 3, 4 or 5 when there is one serving cell configured, the shortened PUCCH format shall be used where the last SC-FDMA symbol in the second slot of a subframe shall be left empty.

In case of guard period for narrowband or wideband retuning for BL/CE UEs, if an SC-FDMA symbol is left empty due to guard period, the SC-FDMA symbol shall be counted in the PUCCH mapping but not used for transmission of the PUCCH. The SC-FDMA symbol affected by the guard period can be the first SC-FDMA symbol in the first slot of a subframe and/or the last SC-FDMA symbol in the second slot of a subframe.

Figure 5.4.3-1: Mapping to physical resource blocks for PUCCH formats 1 – 3 for non-BL/CE UEs.

## 5.5	Reference signals

Two types of uplink reference signals are supported:

-	Demodulation reference signal, associated with transmission of PUSCH or (S)PUCCH

-	Sounding reference signal, not associated with transmission of PUSCH or (S)PUCCH

The same set of base sequences is used for demodulation and sounding reference signals.

### 5.5.1	Generation of the reference signal sequence

Reference signal sequence  is defined by a cyclic shift  of a base sequence  according to

where

-	 is the length of the reference signal sequence, ,  is defined in subclause 5.5.2.1.2, and,

-	 when either

-	the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which is set to 1 to indicate the use of Table 5.5.2.1.1-3, or,

-	the Cyclic Shift Field mapping table for DMRS bit is set to 1 in the most recent uplink-related DCI format 7 which indicates the use of Table 5.5.2.1.1-4, and

-	 otherwise.

Multiple reference signal sequences are defined from a single base sequence through different values of .

Base sequences  are divided into groups, where  is the group number and  is the base sequence number within the group, such that each group contains one base sequence () of each length ,  and two base sequences () of each length , . The sequence group number  and the number  within the group may vary in time as described in clauses 5.5.1.3 and 5.5.1.4, respectively. The definition of the base sequence  depends on the sequence length.

#### 5.5.1.1	Base sequences of length  or larger

For, the base sequence  is given by

where the  root Zadoff-Chu sequence is defined by

with  given by

The length  of the Zadoff-Chu sequence is given by the largest prime number such that.

#### 5.5.1.2	Base sequences of length less than

For , , , and , the base sequence is given by

where the value of  is given by Table 5.5.1.2-1, Table 5.5.1.2-2, Table 5.5.1.2-3, and Table 5.5.1.2-4 for , , , and , respectively. For , the base sequence  is given by

Table 5.5.1.2-1: Definition of  for .

| 0 | -1 | 1 | 3 | -3 | 3 | 3 | 1 | 1 | 3 | 1 | -3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 3 | 3 | 3 | -1 | 1 | -3 | -3 | 1 | -3 | 3 |
| 2 | 1 | 1 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -3 | 1 | -1 |
| 3 | -1 | 1 | 1 | 1 | 1 | -1 | -3 | -3 | 1 | -3 | 3 | -1 |
| 4 | -1 | 3 | 1 | -1 | 1 | -1 | -3 | -1 | 1 | -1 | 1 | 3 |
| 5 | 1 | -3 | 3 | -1 | -1 | 1 | 1 | -1 | -1 | 3 | -3 | 1 |
| 6 | -1 | 3 | -3 | -3 | -3 | 3 | 1 | -1 | 3 | 3 | -3 | 1 |
| 7 | -3 | -1 | -1 | -1 | 1 | -3 | 3 | -1 | 1 | -3 | 3 | 1 |
| 8 | 1 | -3 | 3 | 1 | -1 | -1 | -1 | 1 | 1 | 3 | -1 | 1 |
| 9 | 1 | -3 | -1 | 3 | 3 | -1 | -3 | 1 | 1 | 1 | 1 | 1 |
| 10 | -1 | 3 | -1 | 1 | 1 | -3 | -3 | -1 | -3 | -3 | 3 | -1 |
| 11 | 3 | 1 | -1 | -1 | 3 | 3 | -3 | 1 | 3 | 1 | 3 | 3 |
| 12 | 1 | -3 | 1 | 1 | -3 | 1 | 1 | 1 | -3 | -3 | -3 | 1 |
| 13 | 3 | 3 | -3 | 3 | -3 | 1 | 1 | 3 | -1 | -3 | 3 | 3 |
| 14 | -3 | 1 | -1 | -3 | -1 | 3 | 1 | 3 | 3 | 3 | -1 | 1 |
| 15 | 3 | -1 | 1 | -3 | -1 | -1 | 1 | 1 | 3 | 1 | -1 | -3 |
| 16 | 1 | 3 | 1 | -1 | 1 | 3 | 3 | 3 | -1 | -1 | 3 | -1 |
| 17 | -3 | 1 | 1 | 3 | -3 | 3 | -3 | -3 | 3 | 1 | 3 | -1 |
| 18 | -3 | 3 | 1 | 1 | -3 | 1 | -3 | -3 | -1 | -1 | 1 | -3 |
| 19 | -1 | 3 | 1 | 3 | 1 | -1 | -1 | 3 | -3 | -1 | -3 | -1 |
| 20 | -1 | -3 | 1 | 1 | 1 | 1 | 3 | 1 | -1 | 1 | -3 | -1 |
| 21 | -1 | 3 | -1 | 1 | -3 | -3 | -3 | -3 | -3 | 1 | -1 | -3 |
| 22 | 1 | 1 | -3 | -3 | -3 | -3 | -1 | 3 | -3 | 1 | -3 | 3 |
| 23 | 1 | 1 | -1 | -3 | -1 | -3 | 1 | -1 | 1 | 3 | -1 | 1 |
| 24 | 1 | 1 | 3 | 1 | 3 | 3 | -1 | 1 | -1 | -3 | -3 | 1 |
| 25 | 1 | -3 | 3 | 3 | 1 | 3 | 3 | 1 | -3 | -1 | -1 | 3 |
| 26 | 1 | 3 | -3 | -3 | 3 | -3 | 1 | -1 | -1 | 3 | -1 | -3 |
| 27 | -3 | -1 | -3 | -1 | -3 | 3 | 1 | -1 | 1 | 3 | -3 | -3 |
| 28 | -1 | 3 | -3 | 3 | -1 | 3 | 3 | -3 | 3 | 3 | -1 | -1 |
| 29 | 3 | -3 | -3 | -1 | -1 | -3 | -1 | 3 | -3 | 3 | 1 | -1 |

Table 5.5.1.2-2: Definition of  for

| 0 | -1 | 3 | 1 | -3 | 3 | -1 | 1 | 3 | -3 | 3 | 1 | 3 | -3 | 3 | 1 | 1 | -1 | 1 | 3 | -3 | 3 | -3 | -1 | -3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -3 | 3 | -3 | -3 | -3 | 1 | -3 | -3 | 3 | -1 | 1 | 1 | 1 | 3 | 1 | -1 | 3 | -3 | -3 | 1 | 3 | 1 | 1 | -3 |
| 2 | 3 | -1 | 3 | 3 | 1 | 1 | -3 | 3 | 3 | 3 | 3 | 1 | -1 | 3 | -1 | 1 | 1 | -1 | -3 | -1 | -1 | 1 | 3 | 3 |
| 3 | -1 | -3 | 1 | 1 | 3 | -3 | 1 | 1 | -3 | -1 | -1 | 1 | 3 | 1 | 3 | 1 | -1 | 3 | 1 | 1 | -3 | -1 | -3 | -1 |
| 4 | -1 | -1 | -1 | -3 | -3 | -1 | 1 | 1 | 3 | 3 | -1 | 3 | -1 | 1 | -1 | -3 | 1 | -1 | -3 | -3 | 1 | -3 | -1 | -1 |
| 5 | -3 | 1 | 1 | 3 | -1 | 1 | 3 | 1 | -3 | 1 | -3 | 1 | 1 | -1 | -1 | 3 | -1 | -3 | 3 | -3 | -3 | -3 | 1 | 1 |
| 6 | 1 | 1 | -1 | -1 | 3 | -3 | -3 | 3 | -3 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -3 | -1 | 1 | -1 | 3 | -1 | -3 |
| 7 | -3 | 3 | 3 | -1 | -1 | -3 | -1 | 3 | 1 | 3 | 1 | 3 | 1 | 1 | -1 | 3 | 1 | -1 | 1 | 3 | -3 | -1 | -1 | 1 |
| 8 | -3 | 1 | 3 | -3 | 1 | -1 | -3 | 3 | -3 | 3 | -1 | -1 | -1 | -1 | 1 | -3 | -3 | -3 | 1 | -3 | -3 | -3 | 1 | -3 |
| 9 | 1 | 1 | -3 | 3 | 3 | -1 | -3 | -1 | 3 | -3 | 3 | 3 | 3 | -1 | 1 | 1 | -3 | 1 | -1 | 1 | 1 | -3 | 1 | 1 |
| 10 | -1 | 1 | -3 | -3 | 3 | -1 | 3 | -1 | -1 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -1 | 1 | 3 | 3 | -1 | 1 | -1 | 3 |
| 11 | 1 | 3 | 3 | -3 | -3 | 1 | 3 | 1 | -1 | -3 | -3 | -3 | 3 | 3 | -3 | 3 | 3 | -1 | -3 | 3 | -1 | 1 | -3 | 1 |
| 12 | 1 | 3 | 3 | 1 | 1 | 1 | -1 | -1 | 1 | -3 | 3 | -1 | 1 | 1 | -3 | 3 | 3 | -1 | -3 | 3 | -3 | -1 | -3 | -1 |
| 13 | 3 | -1 | -1 | -1 | -1 | -3 | -1 | 3 | 3 | 1 | -1 | 1 | 3 | 3 | 3 | -1 | 1 | 1 | -3 | 1 | 3 | -1 | -3 | 3 |
| 14 | -3 | -3 | 3 | 1 | 3 | 1 | -3 | 3 | 1 | 3 | 1 | 1 | 3 | 3 | -1 | -1 | -3 | 1 | -3 | -1 | 3 | 1 | 1 | 3 |
| 15 | -1 | -1 | 1 | -3 | 1 | 3 | -3 | 1 | -1 | -3 | -1 | 3 | 1 | 3 | 1 | -1 | -3 | -3 | -1 | -1 | -3 | -3 | -3 | -1 |
| 16 | -1 | -3 | 3 | -1 | -1 | -1 | -1 | 1 | 1 | -3 | 3 | 1 | 3 | 3 | 1 | -1 | 1 | -3 | 1 | -3 | 1 | 1 | -3 | -1 |
| 17 | 1 | 3 | -1 | 3 | 3 | -1 | -3 | 1 | -1 | -3 | 3 | 3 | 3 | -1 | 1 | 1 | 3 | -1 | -3 | -1 | 3 | -1 | -1 | -1 |
| 18 | 1 | 1 | 1 | 1 | 1 | -1 | 3 | -1 | -3 | 1 | 1 | 3 | -3 | 1 | -3 | -1 | 1 | 1 | -3 | -3 | 3 | 1 | 1 | -3 |
| 19 | 1 | 3 | 3 | 1 | -1 | -3 | 3 | -1 | 3 | 3 | 3 | -3 | 1 | -1 | 1 | -1 | -3 | -1 | 1 | 3 | -1 | 3 | -3 | -3 |
| 20 | -1 | -3 | 3 | -3 | -3 | -3 | -1 | -1 | -3 | -1 | -3 | 3 | 1 | 3 | -3 | -1 | 3 | -1 | 1 | -1 | 3 | -3 | 1 | -1 |
| 21 | -3 | -3 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 3 | 1 | -3 | -1 | 1 | -1 | 1 | -1 | -1 | 3 | 3 | -3 | -1 | 1 | -3 |
| 22 | -3 | -1 | -3 | 3 | 1 | -1 | -3 | -1 | -3 | -3 | 3 | -3 | 3 | -3 | -1 | 1 | 3 | 1 | -3 | 1 | 3 | 3 | -1 | -3 |
| 23 | -1 | -1 | -1 | -1 | 3 | 3 | 3 | 1 | 3 | 3 | -3 | 1 | 3 | -1 | 3 | -1 | 3 | 3 | -3 | 3 | 1 | -1 | 3 | 3 |
| 24 | 1 | -1 | 3 | 3 | -1 | -3 | 3 | -3 | -1 | -1 | 3 | -1 | 3 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -3 | -1 | 3 |
| 25 | 1 | -1 | 1 | -1 | 3 | -1 | 3 | 1 | 1 | -1 | -1 | -3 | 1 | 1 | -3 | 1 | 3 | -3 | 1 | 1 | -3 | -3 | -1 | -1 |
| 26 | -3 | -1 | 1 | 3 | 1 | 1 | -3 | -1 | -1 | -3 | 3 | -3 | 3 | 1 | -3 | 3 | -3 | 1 | -1 | 1 | -3 | 1 | 1 | 1 |
| 27 | -1 | -3 | 3 | 3 | 1 | 1 | 3 | -1 | -3 | -1 | -1 | -1 | 3 | 1 | -3 | -3 | -1 | 3 | -3 | -1 | -3 | -1 | -3 | -1 |
| 28 | -1 | -3 | -1 | -1 | 1 | -3 | -1 | -1 | 1 | -1 | -3 | 1 | 1 | -3 | 1 | -3 | -3 | 3 | 1 | 1 | -1 | 3 | -1 | -1 |
| 29 | 1 | 1 | -1 | -1 | -3 | -1 | 3 | -1 | 3 | -1 | 1 | 3 | 1 | -1 | 3 | 1 | 3 | -3 | -3 | 1 | -1 | -1 | 1 | 3 |

Table 5.5.1.2-3: Definition of  for

| 0 | -1 | -3 | 3 | -3 | 3 | -3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -1 | 3 | -1 | 1 | 1 | 1 |
| 2 | 3 | -1 | -3 | -3 | 1 | 3 |
| 3 | 3 | -1 | -1 | 1 | -1 | -1 |
| 4 | -1 | -1 | -3 | 1 | -3 | -1 |
| 5 | 1 | 3 | -3 | -1 | -3 | 3 |
| 6 | -3 | 3 | -1 | -1 | 1 | -3 |
| 7 | -1 | -3 | -3 | 1 | 3 | 3 |
| 8 | 3 | -1 | -1 | 3 | 1 | 3 |
| 9 | 3 | -3 | 3 | 1 | -1 | 1 |
| 10 | -3 | 1 | -3 | -3 | -3 | -3 |
| 11 | -3 | -3 | -3 | 1 | -3 | -3 |
| 12 | 3 | -3 | 1 | -1 | -3 | -3 |
| 13 | 3 | -3 | 3 | -1 | -1 | -3 |
| 14 | 3 | -1 | 1 | 3 | 3 | 1 |
| 15 | -1 | 1 | -1 | -3 | 1 | 1 |
| 16 | -3 | -1 | -3 | -1 | 3 | 3 |
| 17 | 1 | -1 | 3 | -3 | 3 | 3 |
| 18 | 1 | 3 | 1 | 1 | -3 | 3 |
| 19 | -1 | -3 | -1 | -1 | 3 | -3 |
| 20 | 3 | -1 | -3 | -1 | -1 | -3 |
| 21 | 3 | 1 | 3 | -3 | -3 | 1 |
| 22 | 1 | 3 | -1 | -1 | 1 | -1 |
| 23 | -3 | 1 | -3 | 3 | 3 | 3 |
| 24 | 1 | 3 | -3 | 3 | -3 | 3 |
| 25 | -1 | -1 | 1 | -3 | 1 | -1 |
| 26 | 1 | -3 | -1 | -1 | 3 | 1 |
| 27 | -3 | -1 | -1 | 3 | 1 | 1 |
| 28 | -1 | 3 | -3 | -3 | -3 | 3 |
| 29 | 3 | 1 | -1 | 1 | 3 | 1 |

Table 5.5.1.2-4: Definition of  for

| 0 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | -1 | -3 | 3 | -1 | 3 | -1 | 3 | -3 | -1 | -1 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | -1 | 1 | -3 | -3 | -3 | 1 | -1 | 3 | -3 | -3 | 1 |
| 2 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | 1 | 3 | -3 | 1 | 1 | -3 | 1 | -3 | 3 | 1 | -1 |
| 3 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | 3 | -3 | -1 | 3 | -1 | 3 | 1 | -1 | -3 | 3 | -3 |
| 4 | -3 | -3 | -3 | -3 | -3 | -1 | 3 | -3 | -1 | 1 | -1 | -3 | 3 | 3 | 1 | -3 | 1 | -1 |
| 5 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | -3 | -3 | 1 | 1 | 1 | -3 | 1 | 1 | -3 | -3 |
| 6 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | 1 | 1 | -3 | -3 | -3 | 1 | -1 | 3 | -1 | 3 |
| 7 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -1 | 3 | -1 | 3 | 3 | -1 | -1 | 1 | 3 | 3 | -1 |
| 8 | -3 | -3 | -3 | -3 | -3 | 1 | -1 | -1 | -1 | -3 | 3 | -1 | 3 | -3 | 3 | -1 | 1 | 3 |
| 9 | -3 | -3 | -3 | -3 | -3 | 3 | -3 | 1 | -1 | 3 | -3 | 3 | 3 | -1 | -3 | 1 | 1 | -3 |
| 10 | -3 | -3 | -3 | -3 | -3 | 3 | -1 | -3 | -3 | 1 | 1 | 3 | -3 | -1 | 3 | -1 | 3 | 1 |
| 11 | -3 | -3 | -3 | -3 | -3 | 3 | 3 | -1 | -1 | -1 | 3 | 1 | -3 | 3 | -1 | 1 | -3 | 1 |
| 12 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | -3 | 1 | 3 | 1 | -1 | 3 | -3 | -1 | -3 | 1 | 1 |
| 13 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -1 | -1 | 3 | -3 | -3 | 1 | 3 | 1 | -3 | 1 |
| 14 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | 3 | -3 | -1 | 3 | 1 | 3 | -1 | 3 | -1 | -3 |
| 15 | -3 | -3 | -3 | -3 | -1 | -3 | -1 | 3 | -3 | 1 | -3 | 1 | -1 | -3 | -3 | 1 | 1 | 3 |
| 16 | -3 | -3 | -3 | -3 | -1 | -1 | 3 | -3 | 3 | -1 | -3 | 1 | 1 | -1 | -3 | -1 | 3 | -3 |
| 17 | -3 | -3 | -3 | -3 | -1 | -1 | 3 | -1 | -3 | 1 | 3 | -1 | -3 | -3 | 1 | 3 | -1 | 1 |
| 18 | -3 | -3 | -3 | -3 | -1 | 3 | -1 | -1 | 3 | 3 | -1 | -3 | 1 | 1 | 1 | -1 | -3 | -1 |
| 19 | -3 | -3 | -3 | -3 | -1 | 3 | 1 | -3 | -1 | -3 | 3 | 1 | -1 | 3 | -1 | 1 | 3 | -1 |
| 20 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | 3 | 1 | 1 | -3 | -1 | 1 | 3 | 3 | -1 | 3 | -1 |
| 21 | -3 | -3 | -3 | -3 | 1 | -3 | 1 | 3 | 1 | -1 | -1 | 3 | 3 | -1 | 1 | 1 | -3 | 3 |
| 22 | -3 | -3 | -3 | -3 | 1 | -3 | 3 | -3 | -1 | 3 | 1 | 1 | -1 | -1 | 3 | 3 | -1 | 3 |
| 23 | -3 | -3 | -3 | -3 | 1 | -3 | 3 | -1 | 3 | -3 | -1 | -1 | -1 | 1 | -3 | -3 | 3 | 1 |
| 24 | -3 | -3 | -3 | -3 | 1 | 1 | 3 | 1 | 1 | -1 | 3 | 1 | 1 | 3 | -1 | -3 | 1 | 3 |
| 25 | -3 | -3 | -3 | -3 | 1 | 3 | 3 | 3 | 1 | -3 | 1 | -3 | -3 | 3 | -3 | 1 | -1 | -3 |
| 26 | -3 | -3 | -3 | -3 | 3 | 1 | 3 | 3 | -1 | 3 | -3 | -3 | -1 | 3 | -1 | -1 | -3 | 1 |
| 27 | -3 | -3 | -3 | -1 | -3 | -3 | -1 | -1 | -3 | 3 | 3 | 1 | -3 | -1 | -1 | 3 | 1 | -3 |
| 28 | -3 | -3 | -3 | -1 | -3 | 1 | -1 | 1 | -3 | 3 | 1 | -3 | -1 | 1 | 3 | 1 | -1 | -1 |
| 29 | -3 | -3 | -3 | -1 | -3 | 3 | 1 | 1 | -1 | -1 | 1 | 3 | 1 | -3 | 1 | -3 | -1 | 1 |

#### 5.5.1.3	Group hopping

The sequence-group number  in slot  is defined by a group hopping pattern  and a sequence-shift pattern  according to

There are 17 different hopping patterns and 30 different sequence-shift patterns. Sequence-group hopping can be enabled or disabled by means of the cell-specific parameter Group-hopping-enabled provided by higher layers. Sequence-group hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

The group-hopping pattern  may be different for PUSCH, (S)PUCCH and SRS and is given by

where the pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5.

The sequence-shift pattern  definition differs between PUCCH, PUSCH and SRS.

For SPUCCH/PUCCH, the sequence-shift pattern  is given by  where  is given by clause 5.5.1.5.

For PUSCH, the sequence-shift pattern  is given by , where  is configured by higher layers, if no value for  is provided by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, otherwise it is given by  with  given by clause 5.5.1.5.

For SRS, the sequence-shift pattern  is given by where  is given by clause 5.5.1.5.

#### 5.5.1.4	Sequence hopping

Sequence hopping only applies for reference-signals of length .

For reference-signals of length , the base sequence number  within the base sequence group is given by .

For reference-signals of length , the base sequence number  within the base sequence group in slot  is defined by

where the pseudo-random sequence  is given by clause 7.2. The parameter Sequence-hopping-enabled provided by higher layers determines if sequence hopping is enabled or not. Sequence hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

For PUSCH or SPUCCH/PUCCH format 4 transmission with ≥ 6 RBs, the pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5.

For SRS, the pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5 and  is given by clause 5.5.1.3.

#### 5.5.1.5	Determining virtual cell identity for sequence generation

The definition of  depends on the type of transmission.

Transmissions associated with PUSCH:

-	 if no value for  is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure,

-	 otherwise.

Transmissions associated with SPUCCH/PUCCH:

-	 if no value for  is configured by higher layers,

-	 otherwise.

Sounding reference signals:

-	.

### 5.5.2	Demodulation reference signal

#### 5.5.2.1	Demodulation reference signal for PUSCH

##### 5.5.2.1.1	Reference signal sequence

The PUSCH demodulation reference signal sequence  associated with layer  is defined by

where

and  if

-	the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which is set to 1 to indicate the use of Table 5.5.2.1.1-3, or,

-	the Cyclic Shift Field mapping table for DMRS bit field is set to 1 in the most recent uplink-related DCI format 7 which indicates the use of Table 5.5.2.1.1-4, or,

-	subslot-PUSCH/slot-PUSCH for the transport block is semi-persistently scheduled (i.e. higher layer parameter sps-ConfigUL-STTI is configured, see 3GPP TS 36.331 [9]), and ifdma-Config-SPS is set.

In all other cases, .

Subclause 5.5.1 defines the sequence  where, for PUSCH demodulation reference signal sequence,  when

-	the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which is set to 1 to indicate the use of Table 5.5.2.1.1-3, or,

-	the Cyclic Shift Field mapping table for DMRS bit field is set to 1 in the most recent uplink-related DCI format 7 which indicates the use of Table 5.5.2.1.1-4, or,

-	subslot-PUSCH/slot-PUSCH for the transport block is semi-persistently scheduled (i.e. higher layer parameter sps-ConfigUL-STTI is configured, see 3GPP TS 36.331 [9]), and ifdma-Config-SPS is set.

In all other cases, .

The orthogonal sequence  is given by  for subslot-PUSCH/slot-PUSCH. In all other cases, it is given by  for DCI format 0 if the higher-layer parameter Activate-DMRS-with OCC is not set or if the temporary C-RNTI was used to transmit the most recent uplink-related DCI for the transport block associated with the corresponding PUSCH transmission. Otherwise,

-	if higher-layer parameter ul-DMRS-IFDMA is not set,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI [3],

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is not present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field is set to 0, and

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field is set to 1.

The cyclic shift  in a slot  is given as if the ul-V-SPS-RNTI-r14 was used to transmit the most recent uplink-related DCI for the transport block associated with the corresponding PUSCH transmission. If pur-PUSCH-cyclic-shift is configured it provides the value of ncs,λ and the cyclic shift  in a slot  is given as .

Otherwise, the cyclic shift  in a slot  is given as  with

where the value of  is given by Table 5.5.2.1.1-2 according to the parameter cyclicShift provided by higher layers. For non-BL/CE UEs  is given using the most recent uplink-related DCI 3GPP TS 36.212 [3] for the transport block associated with the corresponding PUSCH transmission, except for subslot-PUSCH/slot-PUSCH, as follows:

-	if the higher-layer parameter ul-DMRS-IFDMA is not set,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is not present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field is set to 0, and

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field is set to 1.

For subslot-PUSCH/slot-PUSCH for non-BL/CE UEs, is given by Table 5.5.2.1.1-4, using the cyclic shift field in the most recent uplink-related DCI. If the Cyclic Shift Field mapping table for DMRS bit field is set to 0, in Table 5.5.2.1.1-4 is ignored. If the Cyclic Shift Field mapping table for DMRS bit field is set to 1, both  and are given by Table 5.5.2.1.1-4.

For BL/CE UEs, a cyclic shift field of '000' shall be assumed when determining  from Table 5.5.2.1.1-1.

For subframe-based PUSCH transmission, the first row of Table 5.5.2.1.1-1 shall be used to obtain  and  if there is no uplink-related DCI for the same transport block associated with the corresponding PUSCH transmission, and

-	if the initial PUSCH for the same transport block is semi-persistently scheduled and cyclicShiftSPS is not configured, or

-	if the initial PUSCH for the same transport block is scheduled by the random-access response grant.

An exception applies if subframe-based PUSCH for the transport block is semi-persistently scheduled and the higher-layer parameter cyclicShiftSPS is configured. In this case, the value of  is given by Table 5.5.2.1.1-1 according to the higher-layer parameter cyclicShiftSPS.

An exception applies if subslot-PUSCH/slot-PUSCH for the transport block is semi-persistently scheduled (see 3GPP TS 36.331, sps-ConfigUL-sTTI). In this case:

-	 is given by Table 5.5.2.1.1-1 according to the higher-layer parameter cyclicShiftSPS-STTI if the higher layer parameter ifdma-Config-SPS is not set, and,

-	 and are given by Table 5.5.2.1.1-3 according to the higher-layer parameter cyclicShiftSPS-STTI if the higher layer parameter ifdma-Config-SPS is set.

The quantity  is given by

where the pseudo-random sequence  is defined by clause 7.2. The application of  is cell-specific. The pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame. The quantity  is given by  if no value for  is configured by higher layers for PUSCH/(S)PUCCH format 4/PUCCH format 5 or the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, otherwise it is given by .

The vector of reference signals shall be precoded according to

where  is the number of antenna ports used for PUSCH transmission.

For PUSCH transmission using a single antenna port, ,  and .

For spatial multiplexing,  or  and the precoding matrix  shall be identical to the precoding matrix used in clause 5.3.3A.2 for precoding of the PUSCH in the same subframe.

Table 5.5.2.1.1-1: Mapping of Cyclic Shift Field in uplink-related DCI format to  and

| Cyclic Shift Field in uplink-related DCI format [3] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 0 | 6 | 3 | 9 |  |  |  |  |
| 001 | 6 | 0 | 9 | 3 |  |  |  |  |
| 010 | 3 | 9 | 6 | 0 |  |  |  |  |
| 011 | 4 | 10 | 7 | 1 |  |  |  |  |
| 100 | 2 | 8 | 5 | 11 |  |  |  |  |
| 101 | 8 | 2 | 11 | 5 |  |  |  |  |
| 110 | 10 | 4 | 1 | 7 |  |  |  |  |
| 111 | 9 | 3 | 0 | 6 |  |  |  |  |

Table 5.5.2.1.1-2: Mapping of cyclicShift to values

| cyclicShift |  |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 6 |
| 5 | 8 |
| 6 | 9 |
| 7 | 10 |

Table 5.5.2.1.1-3: Mapping of Cyclic Shift Field in uplink-related DCI format to , , and

| Cyclic Shift Field in uplink-related DCI format [3] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 1 | 0 | 6 | 3 | 9 |  |  |  |  |
| 001 | 1 | 6 | 0 | 9 | 3 |  |  |  |  |
| 010 | 1 | 3 | 9 | 6 | 0 |  |  |  |  |
| 011 | 0 | 4 | 10 | 7 | 1 |  |  |  |  |
| 100 | 0 | 2 | 8 | 5 | 11 |  |  |  |  |
| 101 | 0 | 8 | 2 | 11 | 5 |  |  |  |  |
| 110 | 0 | 10 | 4 | 1 | 7 |  |  |  |  |
| 111 | 1 | 9 | 3 | 0 | 6 |  |  |  |  |

Table 5.5.2.1.1-4:  for subslot-PUSCH/slot-PUSCH

| Cyclic Shift Field in uplink-related DCI format [3] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 6 | 3 | 9 | 0 | 0 | 1 | 1 |
| 1 | 6 | 0 | 9 | 3 | 1 | 1 | 0 | 0 |

##### 5.5.2.1.2	Mapping to physical resources

For each antenna port used for transmission of the PUSCH, the sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to the resource blocks.

-	 when either

-	the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which is set to 1 to indicate the use of Table 5.5.2.1.1-3, or

-	the Cyclic Shift Field mapping table for DMRS bit field is set to 1 in the most recent uplink-related DCI format 7 which indicates the use of Table 5.5.2.1.1-4, and

-	 otherwise.

If higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which is set to 1 to indicate the use of Table 5.5.2.1.1-3, the mapping to resource elements , with  for normal cyclic prefix and  for extended cyclic prefix, in the subframe shall be in increasing order of first  for all values of  satisfying , then the slot number. The quantity  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI.

In case of slot-PUSCH, the mapping to resource elements , with  for normal cyclic prefix, in the slot of the subframe where slot-PUSCH is transmitted shall be in increasing order of first  for all values of , except if the Cyclic Shift Field mapping table for DMRS bit field is set to 1 in the most recent uplink-related DCI format 7, which indicates the use of Table 5.5.2.1.1-4. In this case the mapping to resource element shall be in increasing order of first  only for values of  satisfying .

In case of subslot-PUSCH, the mapping to resource elements , in the subframe shall be in increasing order of first  for all values of , except if the Cyclic Shift Field mapping table for DMRS bit field is set to 1 in the most recent uplink-related DCI format 7, which indicates the use of Table 5.5.2.1.1-4. In this case the mapping to resource element shall be in increasing order of first  only for values of  satisfying . The value of  depends on the uplink subslot number and the DMRS-pattern field in the most recent uplink-related DCI, according to Table 5.5.2.1.2-1, or according to Table 5.5.2.1.2-2 in case of semi-persistent scheduling of subslot-PUSCH (i.e. higher layer patameter sps-ConfigUL-sTTI-r15 is configured, se 3GPP TS 36.331 [9]) and with a configured periodicity of 1 subslot (i.e. semiPersistSchedIntervalUL-STTI-r15 set to sTTI1). In case of subslot-PUSCH and semi-persistent scheduling with a configured periodicity longer than 1 subslot, the mapping shall start at symbol  according to the first row of Table 5.5.2.1.2-2 (i.e. equivalent to a signalling of DMRS-pattern field set to '00'). In case no value of  is defined for the uplink subslot number, and in case no valid starting symbol index (see table 5.3.4-1), no reference signal is transmitted associated with the uplink-related DCI format.

Table 5.5.2.1.2-1: The quantity  for subslot-PUSCH

| DMRS-pattern field in uplink-related DCI format [3] | Uplink subslot number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 |
| 00 | 0 | 3 | 5 | 0 | 2 | 4 |
| 01 | 2 | 4 | - | 1 | 3 | - |
| 10 | - | - | - | 2 | - | - |
| 11 | - | 5 | - | - | 4 | - |

Table 5.5.2.1.2-2: The quantity  for subslot-PUSCH for semi-persistent scheduling

| DMRS-pattern field in uplink-related DCI format [3] | Uplink subslot number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 |
| 00 | 0 | 3 | 5 | 0 | 2 | 4 |
| 10 | 0 | 5 | 5 | 2 | 2 | 4 |

For all other cases, the set of physical resource blocks used in the mapping process and the relation between the index  and the antenna port number  shall be identical to the corresponding PUSCH transmission as defined in clause 5.3.4.

The mapping to resource elements , with , or with  according to Table 5.5.2.1.2-1 for subslot-PUSCH, for normal cyclic prefix and  for extended cyclic prefix, in the subframe shall be in increasing order of first, then the slot number, except for slot-PUSCH and subslot-PUSCH where the reference signal is only mapped to the slot where the slot-PUSCH/subslot-PUSCH is transmitted). No DM-RS shall be transmitted in UpPTS if dmrsLess-UpPts is set to true.

For BL/CE UEs, if higher layer parameter ce-reserved-resource-UL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PUSCH transmission associated with C-RNTI or SPS C-RNTI,

-	In a slot that is fully reserved, the demodulation reference signal transmission is dropped.

-	In a SC-FDMA symbol that is reserved, the demodulation reference signal transmission is dropped.

#### 5.5.2.1A	Demodulation reference signal for PUSCH with sub-PRB allocations

##### 5.5.2.1A.1	Reference signal sequence using modulation schemes other than π/2-BPSK

The reference signal sequence  for  is defined by a cyclic shift  of a base sequence according to

,

where  is given by Tables 5.5.2.1A.1-1 and 5.5.2.1A.1-2 for and , respectively. The cyclic shift  is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 5.5.2.1A.1-3.

If group hopping is enabled, the base sequence index  is given by clause 5.5.2.1A.3.

If group hopping is not enabled, the base sequence index  is given by

-	 for

-	 for

Table 5.5.2.1A.1-1: Definition of  for

|  | ∅0, … ,∅2 |  |  |
| --- | --- | --- | --- |
| 0 | 1 | -3 | -3 |
| 1 | 1 | -3 | -1 |
| 2 | 1 | -3 | 3 |
| 3 | 1 | -1 | -1 |
| 4 | 1 | -1 | 1 |
| 5 | 1 | -1 | 3 |
| 6 | 1 | 1 | -3 |
| 7 | 1 | 1 | -1 |
| 8 | 1 | 1 | 3 |
| 9 | 1 | 3 | -1 |
| 10 | 1 | 3 | 1 |
| 11 | 1 | 3 | 3 |

Table 5.5.2.1A.1-2: Definition of  for

| 0 | 1 | 1 | 1 | 1 | 3 | -3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 3 | 1 | -3 | 3 |
| 2 | 1 | -1 | -1 | -1 | 1 | -3 |
| 3 | 1 | -1 | 3 | -3 | -1 | -1 |
| 4 | 1 | 3 | 1 | -1 | -1 | 3 |
| 5 | 1 | -3 | -3 | 1 | 3 | 1 |
| 6 | -1 | -1 | 1 | -3 | -3 | -1 |
| 7 | -1 | -1 | -1 | 3 | -3 | -1 |
| 8 | 3 | -1 | 1 | -3 | -3 | 3 |
| 9 | 3 | -1 | 3 | -3 | -1 | 1 |
| 10 | 3 | -3 | 3 | -1 | 3 | 3 |
| 11 | -3 | 1 | 3 | 1 | -3 | -1 |
| 12 | -3 | 1 | -3 | 3 | -3 | -1 |
| 13 | -3 | 3 | -3 | 1 | 1 | -3 |

Table 5.5.2.1A.1-3: Definition of

| threeTone-CyclicShift |  | sixTone-CyclicShift |  |
| --- | --- | --- | --- |
| 0 |  | 0 |  |
| 1 |  | 1 |  |
| 2 |  | 2 |  |
| - | - | 3 |  |

##### 5.5.2.1A.2	Reference signal sequence using π/2-BPSK modulation scheme

For  using π/2-BPSK modulation scheme,  NIDcellmod2 is used to determine which 2 of 3 subcarriers will be used:

-	0 indicates that the two subcarriers having the lowest indices among the three allocated are utilized.

-	1 indicates that the two subcarriers having the highest indices among the three allocated are utilized.

The reference signal sequences ru1(n) and ru2(n) for   using 2 out of 3 subcarriers are defined by

ru1n=121+j1-2cnwn mod 16,       0≤n<NrepPUSCHMslotsULMRU

ru2n=(-1)n121+j1-2cnwn mod 16,       0≤n<NrepPUSCHMslotsULMRU

where the binary sequence  is defined by clause 7.2 and shall be initialised with  at the start of the PUSCH transmission using sub-PRB allocations for BL/CE UEs. The quantity  is given by Table 5.5.2.1A.2-1 where u=NIDcellmod16  if group hopping is not enabled, and by clause 5.5.2.1A.3 if group hopping is enabled for PUSCH using sub-PRB allocations for BL/CE UEs.

Table 5.5.2.1A.2-1: Definition of

| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 2 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 3 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 |
| 4 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 5 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 6 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 |
| 7 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 9 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 |
| 10 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 |
| 11 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 12 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 |
| 13 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 14 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 15 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 |

The reference signal sequences for PUSCH using sub-PRB allocations for BL/CE UEs is given by clause 5.3.3, where  and  correspond to the complex-valued symbols at the input of the transform precoding. The resulting complex-valued symbols at the output of the transform precoding correspond to the sequence  which is mapped to physical resources as described in clause 5.5.2.1A.4.

##### 5.5.2.1A.3	Group hopping

For the reference signal for PUSCH transmission using sub-PRB allocations for BL/CE UEs, sequence-group hopping can be enabled where the sequence-group number  in slot  of a radio frame nf is defined by a group hopping pattern fghn' and a sequence-shift pattern  according to

u=fghn'+fssmodMseqRU

where the number of reference signal sequences available for each resource unit size, MseqRU is given by Table 5.5.2.1A.3-1.

Table 5.5.2.1A.3-1: Definition of MseqRU

| Modulation Scheme | MscRU | MseqRU |
| --- | --- | --- |
| π/2-BPSK | 3 | 16 |
| QPSK | 3 | 12 |
|  | 6 | 14 |

Sequence-group hopping can be enabled or disabled as described in clause 5.5.1.3.

The group-hopping pattern fghn' is given by

fghn'=i=07c(8n'+i)∙2imodMseqRU

where n'=ns for  using QPSK modulation scheme. When using π/2-BPSK modulation scheme, for frame structure type 1, n' is the slot number ns of the first slot of the resource unit, and for frame structure type 2, n' is the frame number nf of the first slot of the resource unit. The pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with  cinit=NIDcellMseqRU at the beginning of the resource unit for using π/2-BPSK modulation scheme and in every even slot for  using QPSK modulation scheme.

The sequence-shift pattern  is given by

fssns=NIDcell+∆ssmodMseqRU

where .

##### 5.5.2.1A.4	Mapping to physical resources

The sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to the sub-carriers.

The set of sub-carriers used in the mapping process shall be identical to the corresponding PUSCH transmissions using sub-PRB allocations for BL/CE UEs as defined in clause 5.3.4.

The mapping to resource elements  shall be in increasing order of first, then , and finally the slot number. The value of the symbol index  in a slot is 3.

For BL/CE UEs, if higher layer parameter ce-reserved-resource-UL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PUSCH transmission associated with C-RNTI or SPS C-RNTI,

-	In a slot that is fully reserved, the demodulation reference signal transmission is dropped.

-	In a SC-FDMA symbol that is reserved, the demodulation reference signal transmission is dropped.

#### 5.5.2.2	Demodulation reference signal for PUCCH

##### 5.5.2.2.1	Reference signal sequence

The PUCCH demodulation reference signal sequence  for PUCCH formats 1, 1a, 1b, 2, 2a, 2b, and 3 is defined by

where

and  is the number of antenna ports used for PUCCH transmission. For PUCCH formats 2a and 2b,  equals  for , where  is defined in clause 5.4.2. For all other cases,

The sequence is given by clause 5.5.1 with  and  where the expression for the cyclic shift  is determined by the PUCCH format.

For PUCCH formats 1, 1a and 1b,  is given by

where , ,  and  are defined by clause 5.4.1. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-2, respectively.

For PUCCH formats 2, 2a and 2b,  is defined by clause 5.4.2. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-3, respectively.

For PUCCH format 3,  is given by

where  is given by Table 5.5.2.2.1-4 and  and  for the first and second slot in a subframe, respectively, are obtained from clause 5.4.2A. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-3, respectively.

Table 5.5.2.2.1-1: Number of PUCCH demodulation reference symbols per slot

| PUCCH format | Normal cyclic prefix | Extended cyclic prefix |
| --- | --- | --- |
| 1, 1a, 1b | 3 | 2 |
| 2, 3 | 2 | 1 |
| 2a, 2b | 2 | N/A |

Table 5.5.2.2.1-2: Orthogonal sequences  for PUCCH formats 1, 1a and 1b

| Sequence index | Normal cyclic prefix | Extended cyclic prefix |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  | N/A |

Table 5.5.2.2.1-3: Orthogonal sequences  for PUCCH formats 2, 2a, 2b and 3.

| Normal cyclic prefix | Extended cyclic prefix |
| --- | --- |

Table 5.5.2.2.1-4: Relation between  and  for PUCCH format 3.

| 0 | 0 | 0 |
| --- | --- | --- |
| 1 | 3 | 3 |
| 2 | 6 | 6 |
| 3 | 8 | 9 |
| 4 | 10 | N/A |

The PUCCH demodulation reference signal sequence  for PUCCH formats 4 and 5 is defined by

where

and

Subclause 5.5.1 defines the sequence  where .

The cyclic shift  in a slot  is given as  with

where the values of and  are given by Subclause 5.5.2.1.1 and

with  obtained as described in clause 5.4.2C.

##### 5.5.2.2.2	Mapping to physical resources

The sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to resource elements on antenna port . The mapping shall be in increasing order of first, then  and finally the slot number. The set of values for  and the relation between the index  and the antenna port number  shall be identical to the values used for the corresponding PUCCH transmission. The values of the symbol index  in a slot are given by Table 5.5.2.2.2-1.

Table 5.5.2.2.2-1: Demodulation reference signal location for different PUCCH formats.

| PUCCH format | Set of values for |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 1, 1a, 1b | 2, 3, 4 | 2, 3 |
| 2, 3 | 1, 5 | 3 |
| 2a, 2b | 1, 5 | N/A |
| 4,5 | 3 | 2 |

For BL/CE UEs, if higher layer parameter ce-reserved-resource-UL-time is configured, then in case of PUCCH transmission associated with C-RNTI or SPS C-RNTI,

-	In a slot that is fully reserved, the demodulation reference signal transmission is dropped.

-	In a SC-FDMA symbol that is reserved, the demodulation reference signal transmission is dropped.

#### 5.5.2.3	Demodulation reference signal for SPUCCH

##### 5.5.2.3.1	Reference signal sequence

The SPUCCH demodulation reference signal sequence  for subslot-SPUCCH format 4, and, slot-SPUCCH formats 1, 1a, 1b, 3 and 4 is as defined for  in subclause 5.5.2.2.1 for PUCCH format 1, 1a, 1b, 2, 2a, 2b and 3, using the parameter settings in Table 5.5.2.3.1-1, and with the number of reference symbols  replaced by  and given by Table 5.5.2.3.1-2.

NOTE: Subslot-SPUCCH format 1/1a/1b does not employ a reference signal based design.

The sequence is given by clause 5.5.1 with , where the expression for the cyclic shift is determined depending on the SPUCCH format, see table 5.5.2.3.1-3.

Table 5.5.2.3.1-1: Parameters for SPUCCH demodulation reference signal

| SPUCCH format |  | Frequency hopping |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Slot | 1, 1a, 1b | Disabled | 0 | 12 | See Table 5.5.2.2.1-2 for normal cyclic prefix | 1 |
|  |  | Enabled | 0 | 12 | 1 | 1 |
|  | 3 | Disabled | 0 | 12 | See subclause 5.5.2.2.2 | 1 |
|  | 4 | Enabled | 0 |  | 1 | 1 |
| Subslot | 4 | Disabled | 0 |  | 1 | 1 |

Table 5.5.2.3.1-2: Number of SPUCCH demodulation reference symbols  per slot or per subslot

| SPUCCH format |  | Frequency hopping |  |
| --- | --- | --- | --- |
| Slot | 1, 1a, 1b | Enabled or disabled | 3 |
|  | 3 | Disabled | 2 |
|  | 4 | Enabled | 2 |
| Subslot | 4 | Disabled | 1 |

Table 5.5.2.3.1-3:

| SPUCCH format |  | Frequency hopping |  |
| --- | --- | --- | --- |
| Slot | 1, 1a, 1b | Enabled or disabled | see in subclause 5.4A.2 |
|  | 3 | Disabled | see for PUCCH format 3 in subclause 5.5.2.2.1 and determining and in subclause 5.4A.3.1 |
|  | 4 | Enabled | see for PUCCH format 4 in subclause 5.5.2.2.1 |
| Subslot | 4 | Disabled | see for PUCCH format 4 in subclause 5.5.2.2.1 |

##### 5.5.2.3.2	Mapping to physical resources

The sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to resource elements  on antenna port . The mapping shall be in increasing order of first, then . The set of values for  and the relation between the index  and the antenna port number  shall be identical to the values used for the corresponding SPUCCH transmission. The values of the symbol index  in a slot and a subslot are given by Table 5.5.2.3.2-1 and Table 5.5.2.3.2-2 respectively.

Table 5.5.2.3.2-1: Demodulation reference signal location for different slot-SPUCCH formats

| SPUCCH format | Frequency hopping | Slot | Set of values for |
| --- | --- | --- | --- |
| 1, 1a, 1b | Enabled | 1st | 1, 4, 5 |
|  |  | 2nd | 1, 2, 5 |
|  | Disabled | 1st and 2nd | 2, 3, 4 |
| 3 | Disabled | 1st and 2nd | 1, 5 |
| 4 | Enabled | 1st and 2nd | 1, 5 |

Table 5.5.2.3.2-2: Demodulation reference signal location for different subslot-SPUCCH formats

| SPUCCH format | Subslot number in subframe | Slot |  |
| --- | --- | --- | --- |
| 4 | 0 | 1st | 0 |
|  | 1 | 1st | 3 |
|  | 2 | 1st | 5 |
|  | 3 | 2nd | 0 |
|  | 4 | 2nd | 2 |
|  | 5 | 2nd | 4 |

### 5.5.3	Sounding reference signal

#### 5.5.3.1	Sequence generation

The sounding reference signal sequence  is defined by clause 5.5.1, where  is the sequence-group number defined in clause 5.5.1.3,  is the base sequence number defined in clause 5.5.1.4, and . The cyclic shift  of the sounding reference signal is given as

,

where  is configured separately for periodic and each configuration of aperiodic sounding by the higher-layer parameters cyclicShift and cyclicShift-ap, respectively, for each UE and  is the number of antenna ports used for sounding reference signal transmission. The parameter  if  , otherwise . The parameter  is given by the higher layer parameter transmissionCombNum if configured, otherwise .

#### 5.5.3.2	Mapping to physical resources

The sequence shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in clause 5.1.3.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to resource elements  on antenna port  according to

where  is the number of antenna ports used for sounding reference signal transmission and the relation between the index  and the antenna port  is given by Table 5.2.1-1. The set of antenna ports used for sounding reference signal transmission is configured independently for periodic and each configuration of aperiodic sounding. The quantity  is the frequency-domain starting position of the sounding reference signal and for  and  is the length of the sounding reference signal sequence defined as

where is given by Table 5.5.3.2-1 through Table 5.5.3.2-4 for each uplink bandwidth . The cell-specific parameter srs-BandwidthConfig,  and the UE-specific parameter srs-Bandwidth, are given by higher layers. For UpPTS,  shall be reconfigured to  if this reconfiguration is enabled by the cell-specific parameter srsMaxUpPts given by higher layers, otherwise if the reconfiguration is disabled ,where  is a SRS BW configuration and  is the set of SRS BW configurations from the Tables 5.5.3.2-1 to 5.5.3.2-4 for each uplink bandwidth ,  is the number of format 4 PRACH in the addressed UpPTS and derived from Table 5.7.1-4.

The frequency-domain starting position  is defined by

where for normal uplink subframes  is defined by

and for UpPTS by

The quantity  is given by

where the relation between the index  and the antenna port  is given by Table 5.2.1-1,  is given by the UE-specific parameter transmissionComb or transmissionComb-ap for periodic and each configuration of aperiodic transmission, respectively, provided by higher layers for the UE, and  is frequency position index. The variable  is equal to 0 for UpPTS in the first half frame and equal to 1 for UpPTS in the second half frame of a radio frame.

The frequency hopping of the sounding reference signal is configured by the parameter , provided by higher-layer parameter srs-HoppingBandwidth. Frequency hopping is not supported for aperiodic transmission. If frequency hopping of the sounding reference signal is not enabled (i.e., ), the frequency position index  remains constant (unless re-configured) and is defined by  where the parameter  is given by higher-layer parameters freqDomainPosition and freqDomainPosition-ap for periodic and each configuration of aperiodic transmission, respectively. If frequency hopping of the sounding reference signal is enabled (i.e., ), the frequency position indexes  are defined by

where  is given by Table 5.5.3.2-1 through Table 5.5.3.2-4 for each uplink bandwidth ,

where  regardless of the  value on Table 5.5.3.2-1 through Table 5.5.3.2-4, and

counts the number of UE-specific SRS transmissions, where  is UE-specific periodicity of SRS transmission defined in clause 8.2 of 3GPP TS 36.213 [4],  is SRS subframe offset defined in Table 8.2-2 of 3GPP TS 36.213 [4] and  is the maximum value of  for a certain configuration of SRS subframe offset.

The sounding reference signal shall be transmitted in the last symbol of the uplink subframe.

Table 5.5.3.2-1:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 1 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 2 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 3 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 4 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |
| 5 | 12 | 1 | 4 | 3 | 4 | 1 | 4 | 1 |
| 6 | 8 | 1 | 4 | 2 | 4 | 1 | 4 | 1 |
| 7 | 4 | 1 | 4 | 1 | 4 | 1 | 4 | 1 |

Table 5.5.3.2-2:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 1 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 2 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 3 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 4 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 5 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 6 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 7 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |

Table 5.5.3.2-3:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 72 | 1 | 24 | 3 | 12 | 2 | 4 | 3 |
| 1 | 64 | 1 | 32 | 2 | 16 | 2 | 4 | 4 |
| 2 | 60 | 1 | 20 | 3 | 4 | 5 | 4 | 1 |
| 3 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 4 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 5 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 6 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 7 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |

Table 5.5.3.2-4: and, , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 96 | 1 | 48 | 2 | 24 | 2 | 4 | 6 |
| 1 | 96 | 1 | 32 | 3 | 16 | 2 | 4 | 4 |
| 2 | 80 | 1 | 40 | 2 | 20 | 2 | 4 | 5 |
| 3 | 72 | 1 | 24 | 3 | 12 | 2 | 4 | 3 |
| 4 | 64 | 1 | 32 | 2 | 16 | 2 | 4 | 4 |
| 5 | 60 | 1 | 20 | 3 | 4 | 5 | 4 | 1 |
| 6 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 7 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |

#### 5.5.3.3	Sounding reference signal subframe configuration

The cell-specific subframe configuration period  and the cell-specific subframe offset  for the transmission of sounding reference signals are listed in Tables 5.5.3.3-1 and 5.5.3.3-2, for frame structures type 1 and 2 respectively, where the parameter srs-SubframeConfig is provided by higher layers. Sounding reference signal subframes are the subframes satisfying. For frame structure type 2, a sounding reference signal is transmitted only in uplink subframes or UpPTS.

Table 5.5.3.3-1: Frame structure type 1 sounding reference signal subframe configuration

| srs-SubframeConfig | Binary | Configuration Period (subframes) | Transmission offset (subframes) |
| --- | --- | --- | --- |
| 0 | 0000 | 1 | {0} |
| 1 | 0001 | 2 | {0} |
| 2 | 0010 | 2 | {1} |
| 3 | 0011 | 5 | {0} |
| 4 | 0100 | 5 | {1} |
| 5 | 0101 | 5 | {2} |
| 6 | 0110 | 5 | {3} |
| 7 | 0111 | 5 | {0,1} |
| 8 | 1000 | 5 | {2,3} |
| 9 | 1001 | 10 | {0} |
| 10 | 1010 | 10 | {1} |
| 11 | 1011 | 10 | {2} |
| 12 | 1100 | 10 | {3} |
| 13 | 1101 | 10 | {0,1,2,3,4,6,8} |
| 14 | 1110 | 10 | {0,1,2,3,4,5,6,8} |
| 15 | 1111 | reserved | reserved |

Table 5.5.3.3-2: Frame structure type 2 sounding reference signal subframe configuration

| srs-SubframeConfig | Binary | Configuration Period (subframes) | Transmission offset (subframes) |
| --- | --- | --- | --- |
| 0 | 0000 | 5 | {1} |
| 1 | 0001 | 5 | {1, 2} |
| 2 | 0010 | 5 | {1, 3} |
| 3 | 0011 | 5 | {1, 4} |
| 4 | 0100 | 5 | {1, 2, 3} |
| 5 | 0101 | 5 | {1, 2, 4} |
| 6 | 0110 | 5 | {1, 3, 4} |
| 7 | 0111 | 5 | {1, 2, 3, 4} |
| 8 | 1000 | 10 | {1, 2, 6} |
| 9 | 1001 | 10 | {1, 3, 6} |
| 10 | 1010 | 10 | {1, 6, 7} |
| 11 | 1011 | 10 | {1, 2, 6, 8} |
| 12 | 1100 | 10 | {1, 3, 6, 9} |
| 13 | 1101 | 10 | {1, 4, 6, 7} |
| 14 | 1110 | reserved | reserved |
| 15 | 1111 | reserved | reserved |

## 5.6	SC-FDMA baseband signal generation

This clause applies to all uplink physical signals and uplink physical channels except the physical random access channel and PUSCH using sub-PRB allocations for BL/CE UEs.

The time-continuous signal  for antenna port  in SC-FDMA symbol  in an uplink slot is defined by

for where , ,  and  is the content of resource element  on antenna port .

For frame structure type 3, if the associated DCI indicates PUSCH starting position other than '00' or if 'autonomous PUSCH' is configured,  is given by

where

and were  is given by TS36.213 [4] if 'autonomous PUSCH' is configured.

The quantity  is given by clause 8.1. The UE behaviour if  is undefined.

The SC-FDMA symbols in a slot shall be transmitted in increasing order of , starting with , where SC-FDMA symbol starts at time  within the slot.

Table 5.6-1 lists the values of that shall be used.

Table 5.6-1: SC-FDMA parameters

| Configuration | Cyclic prefix length |
| --- | --- |
| Normal cyclic prefix |  |
| Extended cyclic prefix |  |

## 5.6A	SC-FDMA baseband signal generation for PUSCH using sub-PRB allocations

### 5.6A.1	Modulation schemes other than π/2-BPSK

For , the time-continuous signal  for antenna port  in SC-FDMA symbol  in an uplink slot is defined by clause 5.6 with  replaced by .

### 5.6A.2	Modulation scheme π/2-BPSK

For MscRU=3 and π/2-BPSK modulation only 2-of-3 adjacent subcarriers are selected as described in 5.5.2.1A.2. The time-continuous signal  in SC-FDMA symbol  in an uplink slot is defined by

for  where , ,  is given by Table 5.6-1, and ak,l and  are respectively the modulation value for subcarrier index  and k+1 for symbol , and the values of  used on  and  are respectively obtained by subtracting  from the resulting set of allocated subcarriers as described in Table 8.1.6-1 of [4], and  represents the lower subcarrier index among the selected subcarriers and  is the subcarrier index adjacent to it. The phase rotation  is given by

ϕ=π2l mod 2+φavgkl

φavgkl=φavgkl-1+2πΔfk+1N+NCP,lTs  when  l>0

φavgk0=0

l=0,1,…,NTBNrepPUSCHMRUMslotsULMsymbUL-1

l= l mod MsymbUL

where NTB  is the number of transport blocks defined in clause 8.0 of 3GPP TS 36.213 [4]. If NTB >1 and interleaving between codewords is applied according to clause 8.0 of 3GPP TS 36.213 [4], then the symbol counter  is reset at the start of the transmission and incremented for each symbol during the transmission. For other cases, the symbol counter  is reset at the start of each PUSCH codeword transmission and incremented for each symbol during the transmission of the PUSCH codeword.

The SC-FDMA symbols in a slot shall be transmitted in increasing order of , starting with , where SC-FDMA symbol  starts at time  within the slot.

# 6	Downlink

## 6.3	General structure for downlink physical channels

This clause describes a general structure, applicable to more than one physical channel.

The baseband signal representing a downlink physical channel is defined in terms of the following steps:

-	scrambling of coded bits in each of the codewords to be transmitted on a physical channel

-	modulation of scrambled bits to generate complex-valued modulation symbols

-	mapping of the complex-valued modulation symbols onto one or several transmission layers

-	precoding of the complex-valued modulation symbols on each layer for transmission on the antenna ports

-	mapping of complex-valued modulation symbols for each antenna port to resource elements

-	generation of complex-valued time-domain OFDM signal for each antenna port

Figure 6.3-1: Overview of physical channel processing

### 6.3.1	Scrambling

For each codeword , the block of bits , where  is the number of bits in codeword  transmitted on the physical channel in one subframe/slot/subslot, shall be scrambled prior to modulation, resulting in a block of scrambled bits according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised at the start of each subframe, where the initialisation value of  depends on the transport channel type according to

where  corresponds to the RNTI associated with the PDSCH transmission as described in clause 7.1 3GPP TS 36.213 [4].

For BL/CE UEs, the same scrambling sequence is applied per subframe to PDSCH for a given block of  subframes. The subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies . For the  block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for PDSCH. The PDSCH transmission spans  consecutive subframes including non-BL/CE DL subframes where the PDSCH transmission is postponed.

For BL/CE UEs,

-	if the PDSCH is carrying SIB1-BR

-

-	else if the PDSCH is carrying SI message (except for SIB1-BR) or if the PDSCH transmission is associated with P-RNTI or SC-RNTI:

-	 for frame structure type 1 and  for frame structure type 2

-	otherwise

-	 for UEs assuming CEModeA (according to the definition in Clause 12 of [4]) or configured with CEModeA

-	 for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in Clause 12 of [4]) or configured with CEModeB

For PDSCH with a subframe duration, up to two codewords can be transmitted in one subframe, i.e., . In the case of single codeword transmission,  is equal to zero.

### 6.3.2	Modulation

For each codeword , the block of scrambled bits shall be modulated as described in clause 7.1 using one of the modulation schemes in Table 6.3.2-1, resulting in a block of complex-valued modulation symbols .

Table 6.3.2-1: Modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PDSCH | QPSK, 16QAM, 64QAM, 256QAM |
| PMCH | QPSK, 16QAM, 64QAM, 256QAM |

### 6.3.3	Layer mapping

The complex-valued modulation symbols for each of the codewords to be transmitted are mapped onto one or several layers. Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is the number of layers and  is the number of modulation symbols per layer, unless  and "MUST interference presence and power ratio (MUSTIdx)" signalled in the associated DCI is '00' for only one codeword in which case , where  for the layer  for which MUSTIdx is '00', and  for the layer  for which MUSTIdx is not '00'. The value of is determined from Table 6.3.3-1 using MUSTIdx and the modulation order of the codeword for which MUSTIdx is not '00'.

Table 6.3.3-1: Values for

| MUSTIdx | Modulation order |  |  |
| --- | --- | --- | --- |
|  | QPSK | 16QAM | 64QAM |
| 01 | 8/10 | 32/42 | 128/170 |
| 10 | 50/58 | 144.5/167 | 40.5/51 |
| 11 | 264.5/289 | 128/138 | 288/330 |

#### 6.3.3.1	Layer mapping for transmission on a single antenna port

For transmission on a single antenna port, a single layer is used, , and the mapping is defined by

with .

#### 6.3.3.2	Layer mapping for spatial multiplexing

For spatial multiplexing, the layer mapping shall be done according to Table 6.3.3.2-1. The number of layers  is less than or equal to the number of antenna ports  used for transmission of the physical channel. The case of a single codeword mapped to multiple layers is only applicable when the number of cell-specific reference signals is four or when the number of UE-specific reference signals is two or larger. For subslot/slot-PDSCH, the number of codewords is always one.

Table 6.3.3.2-1: Codeword-to-layer mapping for spatial multiplexing

| Number of layers | Number of codewords | Codeword-to-layer mapping |  |
| --- | --- | --- | --- |
| 1 | 1 |  |  |
| 2 | 1 |  |  |
| 2 | 2 |  |  |
| 3 | 1 |  |  |
| 3 | 2 |  |  |
| 4 | 1 |  |  |
| 4 | 2 |  |  |
| 5 | 2 |  |  |
| 6 | 2 |  |  |
| 7 | 2 |  |  |
| 8 | 2 |  |  |

#### 6.3.3.3	Layer mapping for transmit diversity

For transmit diversity, the layer mapping shall be done according to Table 6.3.3.3-1. There is only one codeword and the number of layers  is equal to the number of antenna ports  used for transmission of the physical channel.

Table 6.3.3.3-1: Codeword-to-layer mapping for transmit diversity

| Number of layers | Number of codewords | Codeword-to-layer mapping |  |
| --- | --- | --- | --- |
| 2 | 1 |  |  |
| 4 | 1 |  | If two null symbols shall be appended to |

### 6.3.4	Precoding

The precoder takes as input a block of vectors,  from the layer mapping and generates a block of vectors,  to be mapped onto resources on each of the antenna ports, where  represents the signal for antenna port.

#### 6.3.4.1	Precoding for transmission on a single antenna port

For transmission on a single antenna port, precoding is defined by

where  is the number of the single antenna port used for transmission of the physical channel and , .

#### 6.3.4.2	Precoding for spatial multiplexing using antenna ports with cell-specific reference signals

Precoding for spatial multiplexing using antenna ports with cell-specific reference signals is only used in combination with layer mapping for spatial multiplexing as described in clause 6.3.3.2. Spatial multiplexing supports two or four antenna ports and the set of antenna ports used is or, respectively.

##### 6.3.4.2.1	Precoding without CDD

Without Cyclic Delay Diversity (CDD), precoding for spatial multiplexing is defined by

where the precoding matrix  is of size  and , .

For spatial multiplexing, the values of  shall be selected among the precoder elements in the codebook configured in the eNodeB and the UE. The eNodeB can further confine the precoder selection in the UE to a subset of the elements in the codebook using codebook subset restrictions. The configured codebook shall be selected from Table 6.3.4.2.3-1 or 6.3.4.2.3-2.

##### 6.3.4.2.2	Precoding for large delay CDD

For large-delay CDD, precoding for spatial multiplexing is defined by

where the precoding matrixis of size  and , . The diagonal size-matrix  supporting cyclic delay diversity and the size- matrix  are both given by Table 6.3.4.2.2-1 for different numbers of layers .

The values of the precoding matrix  shall be selected among the precoder elements in the codebook configured in the eNodeB and the UE. The eNodeB can further confine the precoder selection in the UE to a subset of the elements in the codebook using codebook subset restriction. The configured codebook shall be selected from Table 6.3.4.2.3-1 or 6.3.4.2.3-2.

For 2 antenna ports, the precoder is selected according to  where  denotes the precoding matrix corresponding to precoder index 0 in Table 6.3.4.2.3-1.

For 4 antenna ports, the UE may assume that the eNodeB cyclically assigns different precoders to different vectors on the physical downlink shared channel as follows. A different precoder is used every vectors, where  denotes the number of transmission layers in the case of spatial multiplexing. In particular, the precoder is selected according to , where  is the precoder index given by  and denote precoder matrices corresponding to precoder indices 12,13,14 and 15, respectively, in Table 6.3.4.2.3-2.

Table 6.3.4.2.2-1: Large-delay cyclic delay diversity

| Number of layers |  |  |
| --- | --- | --- |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

##### 6.3.4.2.3	Codebook for precoding and CSI reporting

For transmission on two antenna ports, , and for the purpose of CSI reporting based on two antenna ports  or , the precoding matrix  shall be selected from Table 6.3.4.2.3-1 or a subset thereof. For the closed-loop spatial multiplexing transmission mode defined in 3GPP TS 36.213 [4], the codebook index 0 is not used when the number of layers is .

Table 6.3.4.2.3-1: Codebook for transmission on antenna ports  and for CSI reporting based on antenna ports  or

| Codebook index | Number of layers |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  | - |

For transmission on four antenna ports, , the precoding matrix  shall be selected from Table 6.3.4.2.3-2 or a subset thereof. For the purpose of CSI reporting based on four antenna ports  or , the precoding matrix  shall be selected from Table 6.3.4.2.3-2 or a subset thereof except for alternativeCodeBookEnabledFor4TX-r12 =TRUE in which case the precoding matrix  shall be selected from Tables 7.2.4-0A, 7.2.4-0B, 7.2.4-0C, 7.2.4-0D in [4] or a subset thereof, and except for advancedCodebookEnabled = TRUE in which case the precoding matrix  shall be selected from Table 7.2.4-17C in [4] or a subset thereof. The quantity  denotes the matrix defined by the columns given by the set  from the expression  where  is the  identity matrix and the vector  is given by Table 6.3.4.2.3-2.

Table 6.3.4.2.3-2: Codebook for transmission on antenna ports and for CSI reporting based on antenna ports  or

| Codebook index |  | Number of layers |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 |
| 0 |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |

For the purpose of CSI reporting for 8, 12, 16, 20, 24, 28, and 32 CSI reference signals the codebooks are given in clause 7.2.4 of 3GPP TS 36.213 [4].

#### 6.3.4.3	Precoding for transmit diversity

Precoding for transmit diversity is only used in combination with layer mapping for transmit diversity as described in clause 6.3.3.3. The precoding operation for transmit diversity is defined for two and four antenna ports.

For transmission on two antenna ports, , the output ,  of the precoding operation is defined by

for with .

For rank=1 transmission on two antenna ports, , the output ,  of the precoding operation is defined by

where .

For transmission on four antenna ports,, the output ,  of the precoding operation is defined by

for  with .

#### 6.3.4.4	Precoding for spatial multiplexing using antenna ports with UE-specific reference signals

Precoding for spatial multiplexing using antenna ports with UE-specific reference signals is only used in combination with layer mapping for spatial multiplexing as described in clause 6.3.3.2. Spatial multiplexing using antenna ports with UE-specific reference signals supports up to eight antenna ports.

If the higher-layer parameter dmrs-tableAlt is set to TRUE and the set of antenna ports  is used for two layers transmission, the precoding operation for transmission on the two antenna ports is defined by

where , .

If the higher-layer parameter semiOpenLoop is set to TRUE for subframe PDSCH or the higher-layer parameter semiOpenLoop-STTI is set to TRUE for slot/subslot PDSCH and the set of antenna ports  is used for rank=2 transmission, the precoding operation for transmission on the two antenna ports is defined by

where  and .

If the number of codewords is two and the DCI associated with the scheduled PDSCH is of Format 2D and the 'PDSCH RE Mapping and Quasi-Co-Location indicator' field in the DCI indicates a higher-layer configured PDSCH-RE-MappingQCL containing two sets of parameters, the precoding operation for transmission on  antenna ports is defined by

where ,  and set of antenna ports  used is given by Table 6.3.4.4-1.

Table 6.3.4.4-1: Layer-to-port mapping for two-codeword transmission when PDSCH-RE-MappingQCL contains two sets of parameters

| Number of layers | Layer-to-port mapping |
| --- | --- |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |

Otherwise, the set of antenna ports used is and the precoding operation for transmission on  antenna ports is defined by

where , .

### 6.3.5	Mapping to resource elements

For each of the antenna ports used for transmission of the physical channel, the block of complex-valued symbols  shall conform to the downlink power allocation specified in clause 5.2 in 3GPP TS 36.213 [4] and be mapped in sequence starting with  to resource elements  which meet all of the following criteria in the current subframe:

-	they are in the physical resource blocks corresponding to the virtual resource blocks assigned for transmission, and

-	they are not used for transmission of the core part of PBCH, synchronization signals, and

-	they are assumed by the UE not to be used for cell-specific reference signals, where the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports for and the frequency shift of cell-specific reference signals derived as described in clause 6.10.1.2 4, and

The mapping to resource elements  on antenna port  not reserved for other purposes shall be in increasing order of first the index  over the assigned physical resource blocks and then the index, starting with the first slot in a subframe.

For BL/CE UEs, if the higher layer parameter ce-punctured-subcarriers-DL is configured, then in case of MPDCCH or PDSCH transmission associated with C-RNTI or SPS C-RNTI,

-	The parameter ce-punctured-subcarriers-DL indicates the number of subcarriers (1 or 2) and their position (lower or higher edge) to puncture at the downlink narrowband edges:

-	If the value is ‘00’, then the number of punctured subcarriers on the higher edge of narrowbands above the DC subcarrier is 2 and the number of punctured subcarriers on the higher edge of narrowbands below the DC subcarrier is 1.

-	If the value is ‘01’, then the number of punctured subcarriers on the higher edge of narrowbands above the DC subcarrier is 1 and the number of punctured subcarriers on the higher edge of narrowbands below the DC subcarrier is 0.

-	If the value is ‘10’, then the number of punctured subcarriers on the lower edge of narrowbands above the DC subcarrier is 0 and the number of punctured subcarriers on the lower edge of narrowbands below the DC subcarrier is 1.

-	If the value is ‘11’, then the number of punctured subcarriers on the lower edge of narrowbands above the DC subcarrier is 1 and the number of punctured subcarriers on the lower edge of narrowbands below the DC subcarrier is 2.

-	The subcarrier puncturing is applied to transmission of the following physical signals and channels when the transmission is aligned with a narrowband edge.

-	MPDCCH

-	PDSCH

-	CSI reference signals

-	No subcarrier puncturing is applied to transmissions that are not aligned with a narrowband edge.

## 6.4	Physical downlink shared channel

The physical downlink shared channel shall be processed and mapped to resource elements as described in clause 6.3 with the following additions and exceptions:

-	In resource blocks in which UE-specific reference signals are not transmitted, the PDSCH shall be transmitted on the same set of antenna ports as the PBCH, which is one of , , or .

-	In resource blocks in which UE-specific reference signals are transmitted, the PDSCH shall be transmitted on antenna port(s) ,,, , , , , or the antenna ports listed in Table 6.3.4.4-1, where  is the number of layers used for transmission of the PDSCH.

-	If PDSCH is transmitted in MBSFN subframes as defined in 3GPP TS 36.213 [4], the PDSCH shall be transmitted on one or several of antenna port(s)  or on the antenna ports indicated in Table 6.3.4.4-1, where  is the number of layers used for transmission of the PDSCH.

-	PDSCH is not mapped to resource elements used for UE-specific reference signals associated with PDSCH

-	In mapping to resource elements, the same positions of the cell-specifc reference shall be assumed for all antenna ports on which the PDSCH is transmitted and the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports and the frequency shift of the cell-specific reference signals derived as described in clause 6.10.1.2, unless indicated otherwise in clause 7.1.9 in 3GPP TS 36.213 [4],

-	if one set of parameters for cell-specific reference signal positions are provided by clause 7.1.9 in 3GPP TS 36.213 [4], the values of these parameters are used for all antenna ports whereon the PDSCH is transmitted in the resource blocks indicated by the relevant DCI.

-	if two sets of parameters for cell-specific reference signal positions are provided by clause 7.1.9 in 3GPP TS 36.213 [4], the first set of parameters are used for the set of antenna ports associated with PDSCH codeword  while the second set of parameters are used for the set of antenna ports associated with PDSCH codeword , according to codeword-to-layer mapping and layer-to-port mapping in Subclause 6.3.3.2 and Subclause 6.3.4.4, respectively.

-	If the DCI associated with the PDSCH uses the C-RNTI or semi-persistent C-RNTI, PDSCH modulation symbols on the indicated antenna ports are not mapped to resource elements assumed by the UE to be used for transmission of:

-	zero-power CSI reference signals, where the positions of the CSI reference signals assumed for each antenna port are given by clause 6.10.5.2. The configuration for zero power CSI reference signals is

-	obtained as described in clause 6.10.5.2 and used for all antenna ports whereon the PDSCH is transmitted, unless indicated otherwise in clause 7.1.9 in 3GPP TS 36.213 [4]

-	if one set of parameters for zero power CSI reference signal configuration is provided by clause 7.1.9 in 3GPP TS 36.213 [4], the values of these parameters are used for all antenna ports on which the PDSCH is transmitted in the resource blocks indicated by the corresponding DCI scheduling the PDSCH, and

-	if two sets of parameters for zero power CSI reference signal configuration are provided by clause 7.1.9 in 3GPP TS 36.213 [4], the first set of parameters are used for the set of antenna ports associated with PDSCH codeword  while the second set of parameters are used for the set of antenna ports associated with PDSCH codeword  , according to codeword-to-layer mapping and layer-to-port mapping in Subclause 6.3.3.2 and Subclause 6.3.4.4, respectively.

-	obtained by higher-layer configuration of up to five reserved CSI-RS resources as part of the discovery signal configuration following the procedure for zero-power CSI-RS in clause 6.10.5.2.

-	non-zero-power CSI reference signals for CSI reporting, except for non-zero power CSI reference signals configured by csi-RS-ConfigNZP-ApList, where the positions of the non-zero-power CSI reference signals for CSI reporting are given by clause 6.10.5.2. The configuration for non-zero power CSI reference signals is obtained as described in clause 6.10.5.2.

-	PDSCH is not mapped to any physical resource-block pair(s) carrying an EPDCCH associated with the PDSCH.

-	PDSCH with subframe duration on antenna port 7, 8, 9, 10, 11, 12, 13 or 14 is not mapped to any physical resource-block pair(s) carrying PBCH or synchronization signals.

-	Frame structure type 1, PDSCH on antenna port 5 is not mapped to any physical resource-block pair(s) carrying PBCH or synchronization signals.

-	Frame structure type 2, PDSCH on antenna port 5 is not mapped to any physical resource-block pair(s) carrying PBCH.

-	For frame structure type 1 and 2, the index  in the first slot in a subframe fulfils  where  is given by clause 7.1.6.4 of 3GPP TS 36.213 [4].

-	For frame structure type 3,

-	if the higher layer parameter subframeStartPosition indicates 's07' and the downlink transmission starts in the second slot of a subframe

-	the index  in the second slot in a subframe fulfils  where  is given by clause 7.1.6.4 of 3GPP TS 36.213 [4],

-	otherwise

-	the index  in the first slot in a subframe fulfils  where  is given by clause 7.1.6.4  of 3GPP TS 36.213 [4],

-	In mapping to resource elements, if the DCI associated with the PDSCH uses the C-RNTI or semi-persistent C-RNTI, and transmit diversity according to clause 6.3.4.3 is used, and if the higher-layer parameter semiOpenLoop is not set and if the DCI associated with the PDSCH is not of format 7, resource elements in an OFDM symbol assumed by the UE to contain CSI-RS shall be used in the mapping if and only if all of the following criteria are fulfilled:

-	there is an even number of resource elements for the OFDM symbol in each resource block assigned for transmission, and

-	the complex-valued symbols  and , where  is an even number, can be mapped to resource elements  and  in the same OFDM symbol with .

-	In mapping to resource elements, if the DCI associated with the PDSCH uses C-RNTI or semi-persistent C-RNTI and if the higher-layer parameter semiOpenLoop is set for subframe PDSCH or the higher-layer parameter semiOpenLoop-STTI is set for slot/subslot PDSCH or if the DCI associated with the PDSCH is of format 7 and transmit diversity according to subclause 6.3.4.3 is used, a pair of resource elements ,  shall be used in the mapping if and only if

-	the complex-valued symbols  and  can be mapped to resource elements  and  in the same OFDM symbol and the same PRB with , where  is an even number and  starts from 0 at the lowest subcarrier of the PRB.

### 6.4.1	Physical downlink shared channel for BL/CE UEs

For BL/CE UEs, the following additions and exceptions hold in addition to those in clause 6.4:

-	The maximum number of allocatable PRBs for PDSCH is restricted as follows:

-	If the PDSCH is associated with C-RNTI or SPS C-RNTI and the higher layer parameter ce-pdsch-maxBandwidth-config is set,

-	if the higher layer parameter ce-pdsch-maxBandwidth-config is set to 20 MHz, the maximum number of allocatable PRBs for PDSCH is 96 PRBs restricted to the narrowbands defined in clause 6.2.7;

-	if the higher layer parameter ce-pdsch-maxBandwidth-config is set to 5 MHz, the maximum number of allocatable PRBs for PDSCH is 24 PRBs restricted to no more than four of the narrowbands defined in clause 6.2.7.

- 	If the PDSCH is associated with G-RNTI and the higher layer parameter pdsch-MaxBandwidth-SC-MTCH is set to 24 PRBs, the maximum number of allocatable PRBs for PDSCH is 24 PRBs restricted to no more than four of the narrowbands defined in clause 6.2.7.

-	For all other cases, the maximum number of allocatable PRBs for PDSCH is 6 PRBs restricted to one of the narrowbands defined in clause 6.2.7.

-	Resource elements occupied by CSI reference signals shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

-	Resource elements belonging to synchronization signals, the core part of PBCH, PBCH repetitions, or resource elements reserved for reference signals in the mapping operation of PBCH but not used for transmission of reference signals, shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

-	PRB pairs occupied by RSS shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

-	For BL/CE UEs in CEModeB configured in transmission mode 9, in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals as in subframe #0 shall not be counted in the PDSCH mapping and not used for transmission of the PDSCH.

-	Resource elements belonging to PRBs in which PRS is transmitted (including PRS muted subframes) shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

-	If the higher layer parameter ce-punctured-subcarriers-DL is configured, and the DCI associated with the PDSCH uses C-RNTI or SPS C-RNTI, and transmit diversity according to clause 6.3.4.3 is used,

-	In the mapping to resource elements, when the complex-valued symbols  and , where  is an even number, are mapped to resource elements  and  in the same OFDM symbol with , then if ce-punctured-subcarriers-DL indicates that any of  and  shall be counted but not used for transmission, the UE shall assume that both  and  are counted but not used for transmission.

-	If PDSCH transmission in the LTE control region is configured by higher layer parameter ce-dl-lte-control-region-config, the mapping to resource elements (k,l) not reserved for other purposes shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l, starting from l=lDataStart in the first slot to the last OFDM symbol available for downlink transmission in the subframe, then from l=0 in the first slot to l=lDataStart-1 in the first slot, where lDataStart  is given by clause 7.1.6.4 of 3GPP TS 36.213 [4].

For BL/CE UEs, if the PDSCH is not carrying SIB1-BR the PRB resources for PDSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.12, 5.3.3.1.13, and 5.5.1.3.14 in [3], or provided by higher layers. Each of the NTB≥1 PDSCH codewords is transmitted with  repetitions, where NTB is the number of transport blocks defined in clause 7.1.11 of 3GPP TS 36.213 [4]. The PDSCH transmission spans NabsPDSCH≥NTBNrepPDSCH consecutive subframes, including non-BL/CE DL subframes where the the PDSCH transmission is postponed.

-	If higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PDSCH transmission associated with C-RNTI or SPS C-RNTI,

-	In a subframe that is fully reserved, meaning that all OFDM symbols of all PRBs of the PDSCH transmission are reserved in that subframe, the PDSCH transmission is postponed until the next BL/CE downlink subframe that is not fully reserved.

-	In a subframe that is partially reserved, the reserved resource elements shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

-	If frequency hopping is not enabled for PDSCH, all PDSCH repetitions are located at the same PRB resources, and

-	if frequency hopping is enabled for PDSCH, the PDSCH shall be transmitted in subframe  within the  consecutive downlink subframes using the PRB resources of the narrowband  with the same RIV as that of narrowband nNB(i0). The narrowband nNB(i) is defined as

where  is the absolute subframe number of the first downlink subframe intended for PDSCH and ,  and  are cell-specific higher-layer parameters. For PDSCH carrying SI other than SIB1-BR and for PDSCH associated with P-RNTI, if interval-DlHoppingConfigCommonModeB is signalled in SIB1-BR, then the frequency hopping granularity  is set to interval-DlHoppingConfigCommonModeB; otherwise,  is set to interval-DlHoppingConfigCommonModeA signalled in SIB1-BR.

For BL/CE UE in CEModeA, frequency hopping of PDSCH associated with C-RNTI or SPS C-RNTI is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig is set and the frequency hopping flag in DCI format 6-1A indicates frequency hopping, otherwise, frequency hopping of is not enabled. For BL/CE UE in CEModeB, frequency hopping of PDSCH associated with C-RNTI or SPS C-RNTI is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig is set, otherwise, frequency hopping of is not enabled.

The UE shall not expect PDSCH in subframe  if it is not a BL/CE DL subframe.

For BL/CE UEs, if the PDSCH carries SIB1-BR, the PDSCH transmission is repeated periodically in every period of 8 radio frames, where a period starts with a radio frame with  where  is the system frame number. The PDSCH is transmitted  times in each period of 8 frames, Let  be the set of narrowbands, excluding narrowbands overlapping with the 72 center subcarriers for , and ordered in increasing order of narrowband index. The PDSCH transmission cycles through the set  of narrowbands in increasing order of , starting with  for the first subframe, according to

where  is the number of narrowbands in the set .

The set of frames and subframes used for SIB1-BR transmission in each period are given by Tables 6.4.1-1 and 6.4.1-2.

Table 6.4.1-1: The set of frames and subframes for SIB1-BR for .

|  |  | Frame structure type 1 |  | Frame structure type 2 |  |
| --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 4 | 1 | 5 |
|  | 1 | 1 | 4 | 1 | 5 |

Table 6.4.1-2: The set of frames and subframes for SIB1-BR for .

|  |  | Frame structure type 1 |  | Frame structure type 2 |  |
| --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 4 | 1 | 5 |
|  | 1 | 1 | 4 | 1 | 0 |
| 8 | 0 | 0, 1 | 4 | 0, 1 | 5 |
|  | 1 | 0, 1 | 9 | 0, 1 | 0 |
| 16 | 0 | 0, 1 | 4, 9 | 0, 1 | 0, 5 |
|  | 1 | 0, 1 | 0, 9 | 0, 1 | 0, 5 |

BL/CE UEs may assume the same precoding matrix being used for a PRB across a block of  consecutive subframes when UE-specific reference signals are transmitted together with the PDSCH, where the subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies .

For PDSCH transmission associated with SI-RNTI or P-RNTI to BL/CE UEs, frequency hopping of the PDSCH is enabled when higher layer parameter si-HoppingConfigCommon is set.

For PDSCH transmission associated with PUR-RNTI to BL/CE UEs using UE-specific MPDCCH search space, frequency hopping of the PDSCH is enabled when higher layer parameter pur-PDSCH-frequency-hopping is set

For PDSCH transmission associated with RA-RNTI or temporary C-RNTI to BL/CE UEs, frequency hopping of the PDSCH is enabled when higher layer parameter rar-HoppingConfig is set. Further

-	if PRACH CE level 0 or 1 is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA;

-	if PRACH CE level 2 or 3 is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB.

For PDSCH transmission associated with SC-RNTI to BL/CE UEs, frequency hopping of the PDSCH is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig-SC-MCCH is set. Further

-	if mpdcch-pdsch-HoppingConfig-SC-MCCH is set to CEModeA,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA;

-	if mpdcch-pdsch-HoppingConfig-SC-MCCH is set to CEModeB,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB.

For PDSCH transmission associated with G-RNTI to BL/CE UEs,

-	if the higher layer parameter mpdcch-pdsch-CEmodeConfig-SC-MTCH is set to CEModeA,

-	if the higher layer parameter mpdcch-pdsch-HoppingConfig-SC-MTCH is set and the frequency hopping flag in DCI format 6-1A indicates frequency hopping, then frequency hopping of the PDSCH is enabled and  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA, otherwise frequency hopping is not enabled;

-	if the higher layer parameter mpdcch-pdsch-CEmodeConfig-SC-MTCH is set to CEModeB,

-	if the higher layer parameter mpdcch-pdsch-HoppingConfig-SC-MTCH is set, then frequency hopping of the PDSCH is enabled and  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB, otherwise frequency hopping is not enabled.

### 6.4.2	Slot/subslot-based physical downlink shared channel

For slot or subslot-based PDSCH, in this specification referred to as slot-PDSCH and subslot-PDSCH respectively, the following additions and exceptions hold in addition to those in clause 6.4:

-	PDSCH is not mapped to resource elements of SCCEs used by the associated SPDCCH, or resource elements used for UE-specific reference signals associated with SPDCCH

-	In case of slot-PDSCH:

-	the mapping to resource elements  on antenna port  not reserved for other purposes shall be in increasing order of first the index  over the assigned physical resource blocks and then the index, for the slot of the assigned physical resources in the subframe, and

-	in case of UE-specific reference signals, the PDSCH is not mapped to any physical resource blocks carrying PBCH.

-	In case of subslot-PDSCH:

-	the mapping to resource elements  on antenna port  not reserved for other purposes shall be in increasing order of first the index  over the assigned physical resource blocks and then the index, starting from given in Table 6.4.2-1. The starting value and the value range of  depends on the number of symbols used for PDCCH and the subslot number in the subframe, according to Table 6.4.2-1, and

-	in case of UE-specific reference signals,

-	the PDSCH is not mapped to any physical resource blocks in frequency domain carrying PBCH or synchronization signals for the OFDM symbols of the given subslot.

-	and in case the DCI associated with the subslot-PDSCH indicates the absence of the UE-specific reference signals (see DMRS position indicator field in 3GPP TS 36.212 [3]), the PDSCH is not mapped to any physical resource blocks in frequency domain that carried PBCH or synchronization signals for the OFDM symbols of the previous subslot.

Table 6.4.2-1: Starting value of index , i.e. , for subslot PDSCH

| Number of symbols used for PDCCH | Downlink subslot index |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 |
| 1 | 1 | 3 | 5 | 0 | 2 | 4 |
| 2 | - | 2 | 5 | 0 | 2 | 4 |
| 3 | - | 3 | 5 | 0 | 2 | 4 |

-	For PDSCH associated with UE-specific reference signals,

-	the PDSCH shall only be mapped to physical resource blocks in frequency domain assigned for PDSCH transmission where the assignment maps to both physical resource blocks of a PRG.

-	the subslot-PDSCH shall not be mapped to the physical resource blocks of a PRG in case the resource elements of the associated SPDCCH are mapped to those physical resource blocks.

-	In addition, the following additions and exceptions related to L1 signaling and/or higher layer configuration controlling rate-matching around SPDCCH resources hold:

-	PDSCH is not mapped to resource elements belonging to a SPDCCH resource set configured with higher layer parameter rateMatchingMode indicating ‘m2’ if either this SPDCCH resource set is not configured with higher layer parameter spdcch-L1-ReuseIndication or it is configured with higher layer parameter spdcch-L1-ReuseIndication ‘n1’ or ‘n2’ indicating ‘0’ for the SPDCCH resource set.

-	PDSCH is not mapped to resource elements belonging to a SPDCCH resource set configured with higher layer parameter rateMatchingMode indicating ‘m3’ if the SPDCCH associated with PDSCH is found in this SPDCCH resource set and if either this SPDCCH resource set is not configured with higher layer parameter spdcch-L1-ReuseIndication or it is configured with higher layer parameter spdcch-L1-ReuseIndication ‘n1’ or ‘n2’ indicating ‘0’ for the SPDCCH resource set. If the SPDCCH associated with PDSCH is found on a candidate belonging to two SPDCCH resource sets, the SPDCCH is assumed to be found in both SPDCCH resource sets.

-	PDSCH is not mapped to resource elements belonging to a SPDCCH resource set configured with higher layer parameter rateMatchingMode indicating ‘m4’ if the SPDCCH associated with PDSCH is not found in this SPDCCH resource set and if either this SPDCCH resource set is not configured with higher layer parameter spdcch-L1-ReuseIndication or it is configured with higher layer parameter spdcch-L1-ReuseIndication ‘n1’ or ‘n2’ indicating ‘0’ for the SPDCCH resource set.

-	PDSCH is not mapped to resource elements belonging to a SPDCCH resource set configured with higher layer parameter spdcch-L1-ReuseIndication ‘n0’ if the bit of the Used/Unused SPDCCH resource indication field corresponding to this SPDCCH resource set in the DCI format 7 associated with PDSCH is set to 1.

-	PDSCH is not mapped to resource elements belonging to SCCE#0 to SCCE#of a SPDCCH resource set configured with higher layer parameter spdcch-L1-ReuseIndication ‘n1’ or ‘n2’ indicating ‘2’ for the SPDCCH resource set if the most significant bit of the Used/Unused SPDCCH resource indication field in the DCI format 7 associated with PDSCH is set to 1.

-	PDSCH is not mapped to resource elements belonging to SCCE# to SCCE# of a SPDCCH resource set configured with higher layer parameter spdcch-L1-ReuseIndication ‘n1’ or ‘n2’ indicating ‘2’ for the SPDCCH resource set if the least significant bit of the Used/Unused SPDCCH resource indication field in the DCI format 7 associated with PDSCH is set to 1.

-	It should be noted that not mapping PDSCH to resource elements belonging to a SPDCCH resource set holds irrespective of other indications (spdcch-L1-ReuseIndication or rateMatchingMode) associated with other SPDCCH resource sets (if configured).

-	For a UE with the higher-layer parameter blindSlotSubslotPDSCH-Repetitions set to TRUE and PDSCH associated with a downlink assignment sent on PDCCH/SPDCCH with DCI format 7 indicating k transmissions, the rate-matching around SPDCCH resources if applicable for PDSCH in the k-1 valid slots/subslots following the slot/subslot containing the downlink assignment follows

-	the rate-matching around SPDCCH resources of the PDSCH in the slot/subslot containing the downlink assignment, if the DCI format 7 indicating k transmissions is received on SPDCCH.

-	the rate-matching around SPDCCH resources of the PDSCH in the SPDCCH resource set according to rateMatchingMode indicating ‘m2’, if configured by higher layers, if the DCI format 7 indicating  transmissions is received on PDCCH. For other configurations of rateMatchingMode, no rate-matching around SPDCCH resources for PDSCH in the k-1 valid slots/subslots following the PDCCH with DCI format 7 indicating k PDSCH transmissions is applied..

## 6.8B	MTC physical downlink control channel

### 6.8B.1	MPDCCH formats

The MPDCCH formats are defined as in Clause 6.8A.1 with the following exceptions:

-	The term EPDCCH is replaced by MPDCCH.

-	The MTC physical downlink control channel carries downlink control information and is transmitted across  consecutive BL/CE DL subframes. Within each of the  BL/CE DL subframes an MPDCCH is transmitted using an aggregation of one or several consecutive enhanced control channel elements (ECCEs) where each ECCE consists of multiple enhanced resource element groups (EREGs), defined in clause 6.2.4A.

-	For frame structure type 2,

-	If repetition is not configured for the MPDCCH, the number of EREGs per ECCE is given by Table 6.8A.1-1. If repetition is configured for the MPDCCH, the number of EREGs per ECCE is given by Table 6.8B.1-1.

-	For those special subframes where the MPDCCH is not supported, these special subframes are considered BL/CE DL subframes for both MPDCCH and PDSCH transmission, only if they are indicated as BL/CE DL subframe by higher layer signalling.

-	For an MPDCCH associated with 2 or 4 PRBs, if repetition is not configured for the MPDCCH, the supported MPDCCH formats are given by Table 6.8A.1-2. Otherwise, the supported MPDCCH formats are given by Table 6.8B.1-2. However, for MPDCCH format 5, the equation defining the relation between ECCE index and EREG index does not apply and the number of ECCEs refers to the MPDCCH mapping to the REs of the 2+4 PRB set as defined in Subclause 6.8B.5.

Table 6.8B.1-1: Number of EREGs per ECCE, , for frame structure type 2.

| Normal cyclic prefix |  | Extended cyclic prefix |  |
| --- | --- | --- | --- |
| Normal subframe | Special subframe, configuration 3, 4, 8 | Normal subframe | Special subframe, configuration 1, 2, 3, 5, 6 |
| 4 |  | 8 |  |

Table 6.8B.1-2: Supported MPDCCH formats

| MPDCCH format | Number of ECCEs in a subframe for one MPDCCH, |  |  |  |
| --- | --- | --- | --- | --- |
|  | =4 |  | =8 |  |
|  | Localized transmission | Distributed transmission | Localized transmission | Distributed transmission |
| 0 | 2 | 2 | 1 | 1 |
| 1 | 4 | 4 | 2 | 2 |
| 2 | 8 | 8 | 4 | 4 |
| 3 | 16 | 16 | 8 | 8 |
| 4 | - | - | - | - |
| 5 | 24 | 24 | 12 | 12 |

### 6.8B.2	Scrambling

Scrambling shall be perfomed according to Clause 6.8A.2 with EPDCCH replaced by MPDCCH except that the same scrambling sequence is applied per subframe to MPDCCH for a given block of  subframes and  is the MPDCCH set number. For an MPDCCH associated with a 2+4 PRB set as defined in [4],  is used to generate the scrambling sequence for mapping to REs in 6 PRBs as well as 2 PRBs and 4 PRBs.

The subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies . For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for the MPDCCH. The MPDCCH transmission spans  consecutive subframes, including non-BL/CE DL subframes where the MPDCCH transmission is postponed.

For BL/CE UEs,

-	if the MPDCCH transmission is associated with P-RNTI or SC-RNTI:

-	 for frame structure type 1 and  for frame structure type 2

-	otherwise

-	 for UEs assuming CEModeA (according to the definition in Clause 12 of [4]) or configured with CEModeA:

-	 for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in Clause 12 of [4]) or configured with CEModeB.

### 6.8B.3	Modulation

Modulation shall be performed according to 6.8A.3 with EPDCCH replaced by MPDCCH.

### 6.8B.4	Layer mapping and precoding

Layer mapping and precoding shall be done according to Clause 6.8A.4 with EPDCCH replaced by MPDCCH.

### 6.8B.5	Mapping to resource elements

Mapping to resource elements shall be done according to Clause 6.8A.5with the following exceptions:

-	The term EPDCCH shall be replaced by MPDCCH.

-	The mapping shall be repeated across each of the  BL/CE DL subframes.

-	 is the number of ECCEs used for this MPDCCH in the first of the  subframes.

-	For an MPDCCH associated with a 2+4 PRB set as defined in [4], the mapping to resource elements  on antenna port  shall be in increasing order of first the index  and then the index over the 6 PRBs for MPDCCH format 5 and over the 2 or 4 PRBs for the other MPDCCH formats.

-	For localized transmission and MPDCCH format 5, the single antenna port  to use is given by Table 6.8A.5-1 with

where  equals the C-RNTI.

-	Resource elements occupied by CSI reference signals shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

-	PRB pairs occupied by RSS shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

-	Resource elements belonging to PRBs in which PRS is transmitted (including PRS muted subframes) shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

-	A BL/CE UE not configured with higher layer parameter ce-pdsch-maxBandwidth-config and not configured with higher layer parameter ce-PDSCH-FlexibleStartPRB-AllocConfig may assume there is no MPDCCH transmission which uses overlapping sets of subframes as PDSCH transmissions to that UE, where the MPDCCH is located at a different narrowband than the PDSCH.

-	A BL/CE UE configured with higher layer parameter ce-pdsch-maxBandwidth-config may assume that there is no MPDCCH transmission which uses overlapping sets of subframes as PDSCH transmissions to that UE, where the MPDCCH transmission and PDSCH transmission in any of the overlapping subframes span a PRB region larger than X contiguous PRBs where X=25 if ce-pdsch-maxBandwidth-config is set to 5 MHz and X=100 if ce-pdsch-maxBandwidth-config is set to 20 MHz.

-	A BL/CE UE configured with higher layer parameter ce-PDSCH-FlexibleStartPRB-AllocConfig may assume there is no MPDCCH transmission in MPDCCH candidates not fully contained within the tuning narrowband defined for PDSCH in Clause 6.2.8.

-	For BL/CE UEs in CEModeB, in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals as in subframe #0 shall not be counted in the MPDCCH mapping and not used for transmission of the MPDCCH.

-	Resource elements belonging to synchronization signals, the core part of PBCH, PBCH repetitions, or resource elements reserved for reference signals in the mapping operation of PBCH but not used for transmission of reference signals, shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

-	If MPDCCH transmission in the LTE control region is configured by the higher layer parameter ce-dl-lte-control-region-config,

-	For frame structure type 1 and frame structure type 2 except special subframe configuration 9 or 10,

-	Symbols used for transmission of MPDCCH or demodulation signals associated with MPDCCH and mapped to resource element k,l in the second slot, where l∈{0..lMPDCCHStart-1}, shall additionally be mapped to resource element (k,l) in the first slot.

- 	For frame structure type 2 and special subframe configuration 9 or 10,

- 	Symbols used for transmission of MPDCCH or demodulation signals associated with MPDCCH and mapped to resource element (k,l+4) in the first slot, where l∈{0..lMPDCCHStart-1}, shall additionally be mapped to resource element (k,l) in the first slot, if resource element (k,l) in the first slot is not used for cell-specific reference signals.

-	In the subframes where an MPDCCH or its associated PDSCH is transmitted in response to a physical random access transmission initiated by a PDCCH order, the UE shall receive the MPDCCH or its associated PDSCH, and assume no other UE-specific reception is needed.

-	For MPDCCH transmission associated with C-RNTI or TPC-PUCCH-RNTI or TPC-PUSCH-RNTI or SPS C-RNTI that are not configured to use the Type2-MPDCCH common search space, frequency hopping of the MPDCCH is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig is set.

-	For MPDCCH transmission associated with PUR-RNTI using UE-specific MPDCCH search space, frequency hopping of the MPDCCH is enabled when higher layer parameter pur-MPDCCH-frequency-hopping is set.

-	For MPDCCH transmission associated with Type2-MPDCCH common search space, frequency hopping of the MPDCCH is enabled when higher layer parameter rar-HoppingConfig is set. Further

-	if PRACH CE level 0 or 1 is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA;

-	if PRACH CE level 2 or 3 is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB.

-	For MPDCCH transmission associated with SC-RNTI, frequency hopping of the MPDCCH is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig-SC-MCCH is set. Further

-	if mpdcch-pdsch-HoppingConfig-SC-MCCH is set to CEModeA,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA;

-	if mpdcch-pdsch-HoppingConfig-SC-MCCH is set to CEModeB,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB.

-	For MPDCCH transmission associated with G-RNTI, frequency hopping of the MPDCCH is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig-SC-MTCH is set. Further

-	if mpdcch-pdsch-CEmodeConfig-SC-MTCH is set to CEModeA,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA;

-	if mpdcch-pdsch-CEmodeConfig-SC-MTCH is set to CEModeB,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB.

-	The narrowband  for MPDCCH transmission in the first subframe of MPDCCH search space is provided by higher layers. Starting subframe configuration of a search space where UE monitors an MPDCCH is also provided by higher layers. The MPDCCH search space uses  subframes, spanning  consecutive subframes, including non-BL/CE DL subframes where the MPDCCH transmission is postponed.

-	If higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, then in case of MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space or Type0-MPDCCH common search space,

-	In a subframe that is fully reserved, meaning that all OFDM symbols of all PRB pairs of the MPDCCH search space are reserved in that subframe, the MPDCCH transmission is postponed until the next BL/CE downlink subframe that is not fully reserved.

-	In a subframe that is partially reserved, the reserved resource elements shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

-	If frequency hopping is not enabled for MPDCCH, the repetitions of an MPDCCH candidate are located at the same PRB resources in the same narrowband , and

-	if frequency hopping is enabled for MPDCCH, an MPDCCH candidate shall be transmitted in absolute subframe  using the same PRB resources within each narrowband

where  is the absolute subframe number of the first downlink subframe of MPDCCH search space, and ,  and  are cell-specific higher-layer parameters. The UE shall not expect MPDCCH transmission in absolute subframe  if it is not a BL/CE DL subframe.

-	The UE may assume the same precoding matrix being used for a PRB across a block of  consecutive subframes for MPDCCH, where the subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies .

-	If mpdcch-crs-idle-config or mpdcch-crs-config is configured by higher layers, the relation between the MPDCCH and CRS antenna ports is defined as follows:

-	For distributed transmission and when two CRS ports are configured by the eNB, the relation between the symbols transmitted on the antenna ports used for MPDCCH transmission and CRS ports 0 – 1 is defined by the precoder matrix for single-layer transmission in Table 6.3.4.2.3-1 using codebook index 0 for antenna port 107 and codebook index 1 for antenna port 109.

-	For distributed transmission and when four CRS ports are configured by the eNB, in absolute subframe nabs and resource block index nPRB within the PRB in which MPDCCH is transmitted, the relation between the symbols transmitted on the antenna ports used for MPDCCH transmission and CRS ports 0 – 3 is defined by the precoder matrix for single-layer transmission in Table 6.3.4.2.3-2 using codebook index i for antenna port 107 and codebook index i+1 for antenna port 109, where
i=12+2nabs+iΔnNBch,DL+nPRB mod 2

-	For localized transmission, when two CRS ports are configured by the eNB and predefined mapping type is used, in absolute subframe nabs and resource block index nPRB within the PRB set in which MPDCCH is transmitted, the relation between the symbols transmitted on the antenna port used for MPDCCH transmission and CRS ports 0 – 1 is defined by the precoder matrix for single-layer transmission in Table 6.3.4.2.3-1, with codebook index i, where
i=nabs+iΔnNBch,DL+nPRB mod 2

-	For localized transmission, when four CRS ports are configured by the eNB and predefined mapping type is used, in absolute subframe nabs and resource block index nPRB within the PRB set in which MPDCCH is transmitted, the relation between the symbols transmitted on the antenna port used for MPDCCH transmission and CRS ports 0 – 3 is given by the precoder matrix for single-layer transmission in Table 6.3.4.2.3-2 using codebook index i where
i=12+nabs+iΔnNBch,DL+ΔPRBnPRB mod 4 mod 4

ΔPRB0   ΔPRB1   ΔPRB2   ΔPRB3 =[0 2 1 3]

-	For localized transmission and when CSI-based or reciprocity-based mapping type is used, the relation between the symbols transmitted on the antenna port used for MPDCCH transmission and the CRS ports is given in [4]. When it is indicated in [4] that the antenna port is changed for an MPDCCH candidate with aggregation level 2, the antenna port shall be replaced by the antenna port determined for an MPDCCH candidate with aggregation level 4 in the same search space.

The UE may assume that an MPDCCH associated with the P-RNTI is transmitted on the set  of narrowbands where  is defined in Subclause 6.4.1. For a UE monitoring an MPDCCH associated with the P-RNTI, the first MPDCCH narrowband is given by  where ,  is the Paging Narrowband (PN) obtained according to [10], and  is the higher-layer parameter paging-narrowBands.

-	If the higher-layer parameter si-HoppingConfigCommon disables frequency hopping for an MPDCCH associated with P-RNTI, each MPDCCH candidate shall be located in the same PRB in narrowband  where .

-	If the higher-layer parameter si-HoppingConfigCommon enables frequency hopping for an MPDCCH with P-RNTI, an MPDCCH candidate shall be located in narrowband  in absolute subframe  using the same PRB resources within each narrowband  where

where  is the absolute subframe number of the first downlink subframe of MPDCCH search space according to locations of paging opportunity subframes, and ,  and  are cell-specific higher-layer parameters. For MPDCCH associated with P-RNTI, if interval-DlHoppingConfigCommonModeB is signalled in SIB1-BR, then the frequency hopping granularity  is set to interval-DlHoppingConfigCommonModeB; otherwise,  is set to interval-DlHoppingConfigCommonModeA signalled in SIB1-BR.

The UE shall not expect MPDCCH transmission in absolute subframe  if it is not a BL/CE DL subframe.

## 6.10	Reference signals

Six types of downlink reference signals are defined:

-	Cell-specific Reference Signal (CRS)

-	MBSFN reference signal

-	UE-specific Reference Signal (DM-RS) associated with PDSCH

-	DeModulation Reference Signal (DM-RS) associated with EPDCCH or MPDCCH

-	Positioning Reference Signal (PRS)

-	CSI Reference Signal (CSI-RS)

There is one reference signal transmitted per downlink antenna port.

### 6.10.1	Cell-specific Reference Signal (CRS)

The UE may assume cell-specific reference signals are, unless otherwise stated in [4, clause 12], transmitted in

-	all downlink subframes for frame structure type 1,

-	all downlink subframes and DwPTS for frame structure type 2,

-	non-empty subframes for frame structure type 3

in a cell supporting PDSCH transmission.

If special subframe configuration 10 is configured by the higher layer signalling ssp10-CRS-LessDwPTS, the UE cannot assume that cell specific reference signals are transmitted in the 5th OFDM symbol of the special subframe.

Cell-specific reference signals are transmitted on one or several of antenna ports 0 to 3.

Cell-specific reference signals are transmitted in subframes where  only.

#### 6.10.1.1	Sequence generation

The reference-signal sequence  is defined by

where  is the slot number within a radio frame and  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol where

#### 6.10.1.2	Mapping to resource elements

The reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference symbols for antenna port  in slot  according to

where

The variables  and  define the position in the frequency domain for the different reference signals where  is given by

The cell-specific frequency shift is given by .

Resource elements  used for transmission of cell-specific reference signals on any of the antenna ports in a slot shall not be used for any transmission on any other antenna port in the same slot and set to zero.

In an MBSFN subframe, cell-specific reference signals shall only be transmitted in the non-MBSFN region of the MBSFN subframe.

Figures 6.10.1.2-1 and 6.10.1.2-2 illustrate the resource elements used for reference signal transmission according to the above definition. The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 6.10.1.2-1. Mapping of downlink reference signals (normal cyclic prefix)

Figure 6.10.1.2-2. Mapping of downlink reference signals (extended cyclic prefix)

### 6.10.2	MBSFN reference signals

MBSFN reference signals shall be transmitted in the MBSFN region of MBSFN subframes only when the PMCH is transmitted. MBSFN reference signals are transmitted on antenna port 4.

MBSFN reference signals are defined for extended cyclic prefix only.

#### 6.10.2.1	Sequence generation

##### 6.10.2.1.1	Sequence generation for 15 kHz and 7.5 kHz subcarrier spacing

The MBSFN reference-signal sequence  is defined by

where  is the slot number within a radio frame and  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol.

##### 6.10.2.1.2	Sequence generation for 1.25 kHz subcarrier spacing

The MBSFN reference-signal sequence  is defined by

where  is the subframe number within a radio frame and  is the OFDM symbol number within the subframe. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol.

#### 6.10.2.2	Mapping to resource elements

##### 6.10.2.2.1	Mapping to resource elements for 15 kHz and 7.5 kHz subcarrier spacing

The reference-signal sequence  in OFDM symbol  shall be mapped to complex-valued modulation symbols  with  according to

where

Figure 6.10.2.2-1 illustrates the resource elements used for MBSFN reference signal transmission in case of . In case of , the MBSFN reference signal shall be mapped to resource elements according to Figure 6.10.2.2-3. The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 6.10.2.2-1: Mapping of MBSFN reference signals (extended cyclic prefix, )

Figure 6.10.2.2-3: Mapping of MBSFN reference signals (extended cyclic prefix, )

##### 6.10.2.2.1	Mapping to resource elements for 1.25 kHz

The reference-signal sequence  in OFDM symbol  shall be mapped to complex-valued modulation symbols  with  according to

where

### 6.10.3	UE-specific reference signals associated with PDSCH

UE-specific reference signals associated with PDSCH

-	are transmitted on antenna port(s) , , ,  ,, , ,  or on the antenna ports indicated in Table 6.3.4.4-1, where  is the number of layers used for transmission of the PDSCH;

-	are present and are a valid reference for PDSCH demodulation only if the PDSCH transmission is associated with the corresponding antenna port according to clause 7.1 of 3GPP TS 36.213 [4];

-	are transmitted only on the physical resource blocks upon which the corresponding PDSCH is mapped.

A UE-specific reference signal associated with PDSCH is not transmitted in resource elements  in which one of the physical channels or physical signals other than the UE-specific reference signals defined in 6.1 are transmitted using resource elements with the same index pair  regardless of their antenna port .

A UE-specific reference signal associated with subslot-PDSCH or slot-PDSCH is only transmitted in physical resource blocks in frequency domain assigned for PDSCH transmission where

-	the assignment maps to both physical resource blocks of a given PRG (see subclause 6.4.2);

-	in case of subslot-PDSCH, the associated SPDCCH is not mapped to resource elements of a given PRG assigned for PDSCH transmission (see subclause 6.4.2)..

For frame structure type 3, for PDSCH in a subframe with the same duration as the DwPTS duration of a special subframe configuration, the UE-specific reference signals are defined the same as that for the corresponding special subframe configuration.

#### 6.10.3.1	Sequence generation

For antenna port 5, the UE-specific reference-signal sequence  is defined by

where  denotes the assigned bandwidth in resource blocks of the corresponding PDSCH transmission. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each subframe where  is as described in clause 7.1 3GPP TS 36.213 [4].

For any of the antenna ports , the reference-signal sequence  is defined by

.

The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each subframe.

For BL/CE UEs, the same scrambling sequence is applied per subframe to the UE-specific reference-signal sequence for a given block of  subframes. The subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies . For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for PDSCH. The PDSCH transmissions span  consecutive subframes, including non-BL/CE DL subframes where the PDSCH transmission is postponed. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2.

The quantities , , are given by

-	 if no value for  is provided by higher layers or if DCI format 1A, 2B or 2C is used for the DCI associated with the PDSCH transmission

-	 otherwise

The value of  is zero unless specified otherwise. For a PDSCH transmission on ports 7 or 8,  is given by the DCI format 2B, 2C, 2D, 6-1A, 7-1E, 7-1F and 7-1G  in 3GPP TS 36.212 [3] associated with the PDSCH transmission. 
In the case of DCI format 2B or 7-1E,  is indicated by the scrambling identity field according to Table 6.10.3.1-1. In the case of DCI format 2C or 2D,  is given by Table 5.3.3.1.5C-1, Table 5.3.3.1.5C-2 or Table 5.3.3.1.5C-6 in 3GPP TS 36.212 [3]. In the case of DCI format 7-1F or 7-1G,  is given by Table 5.3.3.1.22-1, Table  5.3.3.1.22-2, Table 5.3.3.1.22-3 or Table 5.3.3.1.5C-6 in 3GPP TS 36.212 [3].  For a PDSCH transmission on ports 11 or 13,  is given by the DCI format 2C or 2D in 3GPP TS 36.212 [3] associated with the PDSCH transmission where  is given by Table 5.3.3.1.5C-2 in 3GPP TS 36.212 [3].

Table 6.10.3.1-1: Mapping of scrambling identity field in DCI format 2B to values for antenna ports 7 and 8

| Scrambling identity field in DCI format 2B (3GPP TS 36.212 [3]) |  |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

#### 6.10.3.2	Mapping to resource elements

For antenna port 5, in a physical resource block with frequency-domain index  assigned for the corresponding PDSCH transmission, the reference signal sequence  shall be mapped to complex-valued modulation symbols  with  in a subframe according to:

Normal cyclic prefix:

Extended cyclic prefix:

where  is the counter of UE-specific reference signal resource elements within a respective OFDM symbol of the PDSCH transmission.

The cell-specific frequency shift is given by .

The mapping shall be in increasing order of the frequency-domain index  of the physical resource blocks assigned for the corresponding PDSCH transmission. The quantity  denotes the assigned bandwidth in resource blocks of the corresponding PDSCH transmission.

Figure 6.10.3.2-1 illustrates the resource elements used for UE-specific reference signals for normal cyclic prefix for antenna port 5.

Figure 6.10.3.2-2 illustrates the resource elements used for UE-specific reference signals for extended cyclic prefix for antenna port 5.

The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 6.10.3.2-1: Mapping of UE-specific reference signals, antenna port 5 (normal cyclic prefix)

Figure 6.10.3.2-2: Mapping of UE-specific reference signals, antenna port 5 (extended cyclic prefix)

For antenna ports , , , , , , or  the antenna ports indicated in Table 6.3.4.4-1 in a physical resource block with frequency-domain index  assigned for the corresponding PDSCH transmission, a part of the reference signal sequence  shall be mapped to complex-valued modulation symbols  in a subframe according to

Normal cyclic prefix:

where

The sequence  is given by Table 6.10.3.2-1.

Table 6.10.3.2-1: The sequence  for normal cyclic prefix

| Antenna port |  |
| --- | --- |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |
| 11 |  |
| 12 |  |
| 13 |  |
| 14 |  |

Extended cyclic prefix:

where

The sequence  is given by Table 6.10.3.2-2.

Table 6.10.3.2-2: The sequence  for extended cyclic prefix and for slot/subslot-PDSCH

| Antenna port |  |
| --- | --- |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |

For extended cyclic prefix, UE-specific reference signals are not supported on antenna ports 9 to 14.

For slot-PDSCH transmission, the baseline pattern (see ‘Baseline’ in Figure 6.10.3.2-2A) of UE-specific reference signals is defined as follows. It is applied in MBSFN subframes.

where

-

-

-

-	l=l0+l'

-

-

and

-	l0=3 if the slot where the PDSCH is transmitted in () fulfils

-	l0=2 if the slot where the PDSCH is transmitted in () fulfils

The sequence  is given by Table 6.10.3.2-2.

For slot-PDSCH transmission in normal subframes,is generated as for the baseline slot-PDSCH UE-specific reference signal pattern for the same values of , while  is given by and depends on the cell-specific frequency shift as follows (see ‘v0’, ‘v1’ and ‘v2’ in Figure 6.10.3.2-2A for vshift mod 3=0, vshift mod 3=1, and vshift mod 3=2, respectively):

-	For vshift mod 3=0, k'=2, 7, 11p∈7,81, 5, 10p∈9,10,

-	For vshift mod 3=1, k'=2, 6, 11p∈7,80, 5, 9p∈9,10,

-	For vshift mod 3=2, k'=1, 6, 10p∈7,80, 4, 9p∈9,10.

Figure 6.10.3.2-2A: Mapping of UE-specific reference signals for slot-PDSCH, antenna ports 7, 8, 9 and 10 (normal cyclic prefix)

For subslot-PDSCH transmission, the baseline pattern (see ‘Baseline’ in Figure 6.10.3.2-2B) of UE-specific reference signals is defined as follows. It is applied if the presence of UE-specific reference signals is indicated in the DCI associated with the subslot-PDSCH (see DMRS position indicator field in 3GPP TS 36.212 [3]), and in downlink subslots where the baseline pattern, including all the REs associated with  if the parameter maxLayersMIMO-STTI  is configured with 2 layers, or  if the parameter maxLayersMIMO-STTI  is configured with 4 layers, has no overlapping resource element with CRS and no overlapping resource element with configured zero-power and non-zero-power CSI reference signals:

where

The sequence  is given by Table 6.10.3.2-2.For subslot-PDSCH transmission in normal subframes, in downlink subslots where the baseline pattern, including all the REs associated with  if the parameter maxLayersMIMO-STTI  is configured with 2 layers, or  if the parameter maxLayersMIMO-STTI  is configured with 4 layers, has overlapping resource elements with configured zero-power or non-zero-power CSI reference signals or has overlapping resource elements with CRS, if the presence of UE-specific reference signals is indicated in the DCI associated (see DMRS position indicator field in 3GPP TS 36.212 [3]) with the subslot-PDSCH, a shifted pattern of UE-specific reference signals is applied. In the shifted pattern,is generated as for the baseline subslot-PDSCH UE-specific reference signal pattern for the same value of , while  is given by and depends on the cell-specific frequency shift as follows (see also ‘v0’,’v1’ and ‘v2’ in Figure 6.10.3.2-2B for vshift mod 3=0, vshift mod 3=1, and vshift mod 3=2, respectively):

-	For vshift mod 3=0, k'=2, 8p∈7, 8 and nPRB mod 2=04, 11p∈7, 8 and nPRB mod 2=11, 7p∈9, 10 and nPRB mod 2=02, 10p∈9, 10 and nPRB mod 2=1,

-	For vshift mod 3=1, k'=2, 8p∈7, 8 and nPRB mod 2=03, 11p∈7, 8 and nPRB mod 2=10, 6p∈9, 10 and nPRB mod 2=02, 9p∈9, 10 and nPRB mod 2=1,

-	For vshift mod 3=2,  k'=1, 9p∈7, 8 and nPRB mod 2=03, 10p∈7, 8 and nPRB mod 2=10, 7p∈9, 10 and nPRB mod 2=01, 9p∈9, 10 and nPRB mod 2=1,

For subslot-PDSCH transmission in MBSFN subframes, in downlink subslots where the baseline pattern, including all the REs associated with  if the parameter maxLayersMIMO-STTI  is configured with 2 layers, or  if the parameter maxLayersMIMO-STTI  is configured with 4 layers,  has overlapping resource elements with configured zero-power or non-zero-power CSI reference signals, if the presence of UE-specific reference signals is indicated in the DCI associated (see DMRS position indicator field in 3GPP TS 36.212 [3]) with the subslot-PDSCH, the shifted pattern of UE-specific reference signals for , as defined above, is applied (see ‘v0’ in Figure 6.10.3.2-2B for vshift mod 3=0).

Figure 6.10.3.2-2B: Mapping of UE-specific reference signals for subslot-PDSCH, antenna ports 7, 8, 9 and 10 (normal cyclic prefix)

Resource elements  used for transmission of UE-specific reference signals to one UE on any of the antenna ports in the set , where  or  shall

-	not be used for transmission of PDSCH on any antenna port in the same slot, and

-	not be used for UE-specific reference signals to the same UE on any antenna port other than those in  in the same slot.

Figure 6.10.3.2-3 illustrates the resource elements used for UE-specific reference signals for normal cyclic prefix for antenna ports 7, 8, 9 and 10. Figure 6.10.3.2-4 illustrates the resource elements used for UE-specific reference signals for extended cyclic prefix for antenna ports 7, 8.

For BL/CE UEs, if higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PDSCH transmission associated with C-RNTI or SPS C-RNTI,

-	If all OFDM symbols in a PRB are reserved, the demodulation reference signal transmission in that PRB is dropped.

Figure 6.10.3.2-3: Mapping of UE-specific reference signals, antenna ports 7, 8, 9 and 10 (normal cyclic prefix)

Figure 6.10.3.2-4: Mapping of UE-specific reference signals, antenna ports 7 and 8 (extended cyclic prefix)

### 6.10.3A	Demodulation reference signals associated with EPDCCH, MPDCCH, or SPDCCH

The demodulation reference signal associated with EPDCCH/MPDCCH/SPDCCH

-	is transmitted on the same antenna port  as the associated EPDCCH/MPDCCH/SPDCCH physical resource;

-	is present and is a valid reference for EPDCCH/MPDCCH/SPDCCH demodulation only if the EPDCCH/MPDCCH/SPDCCH transmission is associated with the corresponding antenna port;

-	is transmitted only on the physical resource blocks upon which the corresponding EPDCCH/MPDCCH/SPDCCH is mapped.

A demodulation reference signal associated with EPDCCH/MPDCCH/SPDCCH is not transmitted in resource elements  in which one of the physical channels or physical signals other than the demodulation reference signals defined in 6.1 are transmitted using resource elements with the same index pair  regardless of their antenna port .

#### 6.10.3A.1	Sequence generation

For any of the antenna ports , the reference-signal sequence  is defined by

.

For non-BL/CE UEs, the pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each subframe where

-	 and  shall be replaced by  and , respectively, for the EPDCCH

-	 and  shall be replaced by  and , respectively, for the SPDCCH

-	, , and

-	 is configured by higher layers.

The EPDCCH/SPDCCH set to which the EPDCCH/SPDCCH associated with the demodulation reference signal belong is denoted .

For BL/CE UEs, the same scrambling sequence is applied per subframe to the demodulation reference signal associated with MPDCCH for a given block of  subframes. The subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies . For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for MPDCCH. The MPDCCH transmissions span  consecutive subframes, including non-BL/CE DL subframes where the MPDCCH transmission is postponed.

For BL/CE UEs,

-	if the MPDCCH transmission is associated with P-RNTI or SC-RNTI:

-	 for frame structure type 1 and  for frame structure type 2

-	otherwise

-	for UEs assuming CEModeA (according to the definition in Clause 12 of [4]) or configured with CEModeA.

-	 for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in Clause 12 of [4]) or configured with CEModeB.

The quantities  and  are configured by higher layers. The MPDCCH set to which the MPDCCH associated with the demodulation reference signal belong is denoted . For an MPDCCH associated with a 2+4 PRB set as defined in [4],  is used to generate the scrambling sequence for the 6 PRBs as well as for the 2 PRBs and 4 PRBs.

#### 6.10.3A.2	Mapping to resource elements

For the antenna port  in a physical resource block  assigned for the associated EPDCCH/MPDCCH, a part of the reference signal sequence  shall be mapped to complex-valued modulation symbols  in a subframe according to

Normal cyclic prefix:

where

The sequence  is given by Table 6.10.3A.2-1.

Table 6.10.3A.2-1: The sequence  for normal cyclic prefix

| Antenna port |  |
| --- | --- |
| 107 |  |
| 108 |  |
| 109 |  |
| 110 |  |

Extended cyclic prefix:

where

The sequence  is given by Table 6.10.3A.2-2.

Table 6.10.3A.2-2: The sequence  for extended cyclic prefix

| Antenna port |  |
| --- | --- |
| 107 |  |
| 108 |  |

For extended cyclic prefix, demodulation reference signals are not supported on antenna ports 109 to 110.

For the antenna port  in a physical resource block  assigned for the SPDCCH, a part of the reference signal sequence  shall be mapped to complex-valued modulation symbols  in a subframe according to the procedure used for UE-specific reference signals associated with subslot-PDSCH on antenna port described in section 6.10.3.2 with the following amendments:

-	for slot-SPDCCH, ,

-	for slot-SPDCCH in MBSFN subframes, the procedure used for the baseline pattern of UE-specific reference signals associated with subslot-PDSCH is applied

-	for slot-SPDCCH in normal subframes, the procedure used for the shifted pattern of UE-specific reference signals associated with subslot-PDSCH depending on the cell-specific frequency shift is applied.

Resource elements  used for transmission of demodulation reference signals to one UE on any of the antenna ports in the set , where  or  shall

-	not be used for transmission of EPDCCH/MPDCCH on any antenna port in the same slot, and

-	not be used for demodulation reference signals to the same UE on any antenna port other than those in  in the same slot.

Replacing antenna port numbers 7 – 10 by 107 – 110 in Figure 6.10.3.2-3 provides an illustration of the resource elements used for demodulation reference signals associated with EPDCCH/MPDCCH for normal cyclic prefix. Replacing antenna port numbers 7 – 8 by 107 – 108 in Figure 6.10.3.2-4 provides an illustration of the resource elements used for demodulation reference signals associated with EPDCCH/MPDCCH for extended cyclic prefix.

For frame structure type 3, for EPDCCH in a subframe with the same duration as the DwPTS duration of a special subframe configuration, the mapping of the demodulation reference signals to the resource elements is the same as that for the corresponding special subframe configuration.

For BL/CE UEs, if higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, then in case of MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space or Type0-MPDCCH common search space,

-	If all OFDM symbols in a PRB are reserved, the demodulation reference signal transmission in that PRB is dropped.

### 6.10.4	Positioning reference signals

Positioning reference signals shall only be transmitted in resource blocks in downlink subframes configured for positioning reference signal transmission. If both normal and MBSFN subframes are configured as positioning subframes within a cell, the OFDM symbols in a MBSFN subframe configured for positioning reference signal transmission shall use the same cyclic prefix as used for subframe #0. If only MBSFN subframes are configured as positioning subframes within a cell, the OFDM symbols configured for positioning reference signals in the MBSFN region of these subframes shall use extended cyclic prefix length. In a subframe configured for positioning reference signal transmission, the starting positions of the OFDM symbols configured for positioning reference signal transmission shall be identical to those in a subframe in which all OFDM symbols have the same cyclic prefix length as the OFDM symbols configured for positioning reference signal transmission.

Positioning reference signals are transmitted on antenna port 6.

The positioning reference signals shall not be mapped to resource elements  allocated to the core part of the PBCH, PSS or SSS regardless of their antenna port .

Positioning reference signals are defined for  only.

#### 6.10.4.1	Sequence generation

The reference-signal sequence  is defined by

where  is the slot number within a radio frame,  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol where  equals  unless configured by higher layers and where

#### 6.10.4.2	Mapping to resource elements

If PRS frequency hopping is not configured by higher layers, the reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference signal for antenna port  in slot  according to

where

Normal cyclic prefix:

Extended cyclic prefix:

The bandwidth for positioning reference signals  is configured by higher layers and the cell-specific frequency shift is given by  where  if no value for  is configured by higher layers.

If PRS frequency hopping is configured by higher layers, a PRS frequency hopping configuration provided by higher layers contains the following:

-	The length of the PRS occasion group,

-	Number of PRS frequency hopping bands,

-	 defined as twice the starting PRB index of PRS frequency hopping band  where

-	 if ,

-	 where  is the index of the first PRB in the PRS frequency hopping narrowband configured by higher layers if

If PRS frequency hopping is configured by higher layers, the reference signal sequence in the PRS occasion , , in the PRS occasion group shall be mapped to complex-valued modulation symbols  used as reference signal for antenna port  in slot  according to

where

-	for normal cyclic prefix

-	for extended cyclic prefix

Figure 6.10.4.2-1: Mapping of positioning reference signals (normal cyclic prefix)

Figure 6.10.4.2-2: Mapping of positioning reference signals (extended cyclic prefix)

#### 6.10.4.3	Positioning reference signal subframe configuration

The subframe configuration period  and the subframe offset  for the transmission of positioning reference signals are listed in Table 6.10.4.3-1. The PRS configuration index  is configured by higher layers. Positioning reference signals are transmitted only in configured DL subframes. Positioning reference signals shall not be transmitted in DwPTS. Positioning reference signals shall be transmitted in  consecutive downlink subframes, where  is configured by higher layers.

The positioning reference signal instances, for the first subframe of the  downlink subframes, shall satisfy .

Table 6.10.4.3-1: Positioning reference signal subframe configuration

| PRS configuration Index | PRS periodicity (subframes) | PRS subframe offset (subframes) |
| --- | --- | --- |
| 0 – 159 | 160 |  |
| 160 – 479 | 320 |  |
| 480 – 1119 | 640 |  |
| 1120 – 2399 | 1280 |  |
| 2400 – 2404 | 5 |  |
| 2405 – 2414 | 10 |  |
| 2415 – 2434 | 20 |  |
| 2435 – 2474 | 40 |  |
| 2475 – 2554 | 80 |  |
| 2555-4095 | Reserved |  |

### 6.10.5	CSI reference signals

CSI reference signals are transmitted on 1, 2, 4, 8, 12, 16, 20, 24, 28, or 32 antenna ports using , , , , , ,, ,  and, respectively.

For CSI reference signals using more than eight antenna ports,  CSI-RS configurations in the same subframe, numbered from 0 to , where value 0 corresponds to the configured resourceConfig-r11 or resourceConfig-r10 and value k (k>0) corresponds to the configured k-th entry of NZP-ResourceConfig-r13 from an aggregated list consisting of nzp-resourceConfigList-r13 followed by nzp-resourceConfigListExt-r14 (if configured), are aggregated to obtain  antenna ports in total. Each CSI-RS configuration in such an aggregation corresponds to  antenna ports and one of the configurations in the range 0-19 in Table 6.10.5.2-1 for normal cyclic prefix, and one of the configurations in the range 0-15 in Table 6.10.5.2-2 for extended cyclic prefix. The supported configurations of aggregated CSI-RS configurations are shown in Table 6.10.5-1. If the higher layer parameter NZP-TransmissionComb is not configured,  unique CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix and from Table 6.10.5.2-2 for extended cyclic prefix are aggregated to form 12, 16, 20, 24, 28, or 32 antenna ports.

For CSI reference signals using more than sixteen antenna ports, when higher layer parameter NZP-TransmissionComb is configured, the number of unique CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix and from Table 6.10.5.2-2 for extended cyclic prefix that are aggregated to form 20, 24, 28, or 32 antenna ports can be less than or equal to . The number of antenna ports within each such unique CSI-RS resource configuration is an integer multiple of .

CSI reference signals are defined for  only.

Table 6.10.5-1: Aggregation of CSI-RS configurations.

| Total number of antenna ports | Number of antenna ports per CSI-RS configuration | Number of CSI-RS configurations |
| --- | --- | --- |
| 12 | 4 | 3 |
| 16 | 8 | 2 |
| 20 | 4 | 5 |
| 24 | 8 | 3 |
| 28 | 4 | 7 |
| 32 | 8 | 4 |

#### 6.10.5.1	Sequence generation

The reference-signal sequence  is defined by

where  is the slot number within a radio frame and  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol where

The quantity  equals  unless configured by higher layers.

#### 6.10.5.2	Mapping to resource elements

In subframes configured for CSI reference signal transmission, the reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference symbols on antenna port  . The mapping depends on the higher-layer parameter CDMType.

For the case of CDMType is not configured or is configured to CDM2:

where

For the case of CDMType equal to CDM4:

where

and where  is given by Table 6.10.5.2-0.

Table 6.10.5.2-0: The sequence  for CDM4.

| 15 | 15,17 |  |
| --- | --- | --- |
| 16 | 16,18 |  |
| 17 | 19,21 |  |
| 18 | 20,22 |  |

If neither of the higher-layer parameters NZP-FrequencyDensity and NZP-TransmissionComb are configured, .

If the UE is configured with one or more of the parameters NZP-FrequencyDensity and NZP-TransmissionComb,

-	if either NZP-FrequencyDensity equals 1,

-	if NZP-FrequencyDensity equals 1/2 and NZP-TransmissionComb equals 0,

-	if NZP-FrequencyDensity equals 1/2 and NZP-TransmissionComb equals 1,

-	if NZP-FrequencyDensity equals 1/3 and NZP-TransmissionComb equals 0,

-	if NZP-FrequencyDensity equals 1/3 and NZP-TransmissionComb equals 1,

-	if NZP-FrequencyDensity equals 1/3 and NZP-TransmissionComb equals 2,

The quantity  and the necessary conditions on  are given by Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively.

The relation between the antenna port number  and the quantity  depends on the number of CSI-RS antenna ports:

-	for CSI reference signals using up to eight antenna ports,

-	for CSI reference signals using more than eight antenna ports when the higher-layer parameter CDMType equals CDM2

where  is the CSI-RS resource number.

-	for CSI reference signals using more than eight antenna ports when the higher-layer parameter CDMType equals CDM4, antenna port number  where  for CSI-RS resource number .

For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 32:

where

The resource elements for the  CDM8 pattern, where , are determined by aggregating pairs of resource elements  satisfying  from the  aggregated CSI-RS configurations, where at most one pair of resource elements is drawn from each of the  aggregated CSI-RS configurations. For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 32, the aggregated CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix and from Table 6.10.5.2-2 for extended cyclic prefix are restricted to one of , , or . Antenna port number  where  for CSI-RS resource number . The sequence is given by Table 6.10.5.2-0A, where .

Table 6.10.5.2-0A: The sequence  for CDM8 with 32 CSI-RS antenna ports.

| 15, 17, 19, 21 |  |
| --- | --- |
| 16, 18, 20, 22 |  |
| 23, 25, 27, 29 |  |
| 24, 26, 28, 30 |  |
| 31, 33, 35, 37 |  |
| 32, 34, 36, 38 |  |
| 39, 41, 43, 45 |  |
| 40, 42, 44, 46 |  |

For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 24:

where

For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 24, the aggregated CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix are restricted to  in that order. Resource elements for CDM8 patterns are determined as follows:

-	Aggregating resource element quadruplet  satisfying  from CSI-RS configuration 1 with resource element quadruplet  satisfying  from CSI-RS configuration 2

-	Aggregating resource element quadruplet  satisfying  from CSI-RS configuration 3 with resource element quadruplet  satisfying  from CSI-RS configuration 1

-	Aggregating resource element quadruplet  satisfying  from CSI-RS configuration 2 with resource element quadruplet  satisfying  from CSI-RS configuration 3

Antenna port number  where  for CSI-RS resource number . The sequence is given by Table 6.10.5.2-0B. The sequence index  is determined as follows:

-	For resource element quadruplet  satisfying  from CSI-RS configuration 1, resource element quadruplet  satisfying  from CSI-RS configuration 2, or resource element quadruplet  satisfying  from CSI-RS configuration 3, .

-	For resource element quadruplet  satisfying  from CSI-RS configuration 1, resource element quadruplet  satisfying  from CSI-RS configuration 2, or resource element quadruplet  satisfying  from CSI-RS configuration 3, .

Table 6.10.5.2-0B: The sequence  for CDM8 with 24 CSI-RS antenna ports.

| 15, 25, 31 |  |
| --- | --- |
| 16, 26, 32 |  |
| 19, 29, 35 |  |
| 20, 30, 36 |  |
| 17, 23, 33 |  |
| 18, 24, 34 |  |
| 21, 27, 37 |  |
| 22, 28, 38 |  |

Multiple CSI reference signal configurations can be used in a given cell. A UE can be configured with multiple sets of CSI reference signals,

-	one or more configurations for CSI reporting for which the UE shall assume non-zero transmission power for the CSI-RS, and

-	zero or more configurations for which the UE shall assume zero transmission power, and

-	zero or more configurations valid across the system downlink bandwidth as part of the discovery signals for which the UE shall assume non-zero transmission power for the CSI-RS.

The CSI-RS configurations for which the UE shall assume non-zero transmission power are provided by higher layers.

The CSI-RS configurations for which the UE shall assume zero transmission power in a subframe are given by a bitmap derived according to clause 7.2.7 in 3GPP TS 36.213 [4]. For each bit set to one in the 16-bit bitmap, the UE shall assume zero transmission power for the resource elements corresponding to the four CSI reference signal column in Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively, except for resource elements that overlap with those for which the UE shall assume non-zero transmission power CSI-RS as configured by higher layers. The most significant bit corresponds to the lowest CSI reference signal configuration index and subsequent bits in the bitmap correspond to configurations with indices in increasing order.

CSI reference signals not corresponding to higher layer configured parameters csi-RS-ConfigNZP-ApList or csi-RS-ConfigZP-ApList can only occur in

-	downlink slots where  fulfils the condition in Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively, and

-	where the subframe number fulfils the conditions in clause 6.10.5.3.

CSI reference signals corresponding to either higher layer configured parameter csi-RS-ConfigNZP-ApList or csi-RS-ConfigZP-ApList can only occur in

-	downlink slots where  fulfils the condition in Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively.

The UE shall assume that CSI reference signals are not transmitted

-	in the DwPTS for special subframe configuration 0, 5, 9 and 10 for normal cyclic prefix and special subframe configuration 0, 4 and 7 for extended cyclic prefix, in case of frame structure type 2,

- 	in the DwPTS for normal CP for the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 24,

-	in subframes where PDSCH/EPDCCH transmission starts in the second slot of a subframe for frame structure type 3,

-	in subframes where PDSCH/EPDCCH transmission ends prior to the end of a subframe for frame structure type 3,

-	in an empty subframe where there is no PDSCH or discovery signal transmission for frame structure type 3,

-	in subframes where transmission of a CSI-RS would collide with SystemInformationBlockType1 messages,

-	in the primary cell in subframes configured for transmission of paging messages in the primary cell for any UE with the cell-specific paging configuration.

For special subframe configuration {1, 2, 6, or 7}, a UE does not expect to be configured with one of CSI-RS configurations {1, 2, 3, 4, 6, 7, 8, 9, 12, 13, 14, 15, 16, 17} in DwPTS for normal CP.

The UE shall assume that none of the CSI reference signals corresponding to a CSI reference signal configuration are transmitted in subframes where transmission of any of those CSI reference signals would collide with transmission of synchronization signals or the core part of PBCH.

Resource elements  used for transmission of CSI reference signals on any of the antenna ports in the set , where , , , , , , , , , , , , , , ,  or  shall not be used for transmission of PDSCH on any antenna port in the same slot if higher layer parameter CDMType is not configured, or is configured to CDM2.

Resource elements  used for transmission of CSI reference signals on any of the antenna ports in the set , where

-	,  or  for CSI reference signals on 12 ports, or

-	, ,  or  for CSI reference signals on 16 ports, or

-	, , ,  or for CSI reference signals on 20 ports, or

-	, , , ,  or  for CSI reference signals on 24 ports, or

-	, , , ,,  or  for CSI reference signals on 28 ports, or

-	, , , , , ,  or  for CSI reference signals on 32 ports

shall not be used for transmission of PDSCH on any antenna port in the same slot if higher layer parameter CDMType is configured to CDM4.

Resource elements  used for transmission of CSI reference signals on any of the antenna ports in the set , where

-	,  or  for CSI reference signals on 24 ports, or

-	, ,  or  for CSI reference signals on 32 ports

shall not be used for transmission of PDSCH on any antenna port in the same slot if higher layer parameter CDMType is configured to CDM8.

The mapping for CSI reference signal configuration 0 is illustrated in Figures 6.10.5.2-1 and 6.10.5.2-2.

Table 6.10.5.2-1: Mapping from CSI reference signal configuration to  for normal cyclic prefix

| CSI-RS config. | Number of CSI reference signals configured |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 or 2 |  |  |  | 4 |  |  |  | 8 |  |  |  |
|  | Normal subframe |  | Special subframe |  | Normal subframe |  | Special subframe |  | Normal subframe |  | Special subframe |  |
| 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 |
| 1 | (11,2) | 1 | (11,5) | 0 | (11,2) | 1 | (11,5) | 0 | (11,2) | 1 | (11,5) | 0 |
| 2 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 |
| 3 | (7,2) | 1 | (7,5) | 0 | (7,2) | 1 | (7,5) | 0 | (7,2) | 1 | (7,5) | 0 |
| 4 | (9,5) | 1 |  |  | (9,5) | 1 |  |  | (9,5) | 1 |  |  |
| 5 | (8,5) | 0 | (8,5) | 0 | (8,5) | 0 | (8,5) | 0 |  |  |  |  |
| 6 | (10,2) | 1 | (10,5) | 0 | (10,2) | 1 | (10,5) | 0 |  |  |  |  |
| 7 | (8,2) | 1 | (8,2) | 1 | (8,2) | 1 | (8,2) | 1 |  |  |  |  |
| 8 | (6,2) | 1 | (6,5) | 0 | (6,2) | 1 | (6,5) | 0 |  |  |  |  |
| 9 | (8,5) | 1 |  |  | (8,5) | 1 |  |  |  |  |  |  |
| 10 | (3,5) | 0 | (3,5) | 0 |  |  |  |  |  |  |  |  |
| 11 | (2,5) | 0 | (2,5) | 0 |  |  |  |  |  |  |  |  |
| 12 | (5,2) | 1 | (5,5) | 0 |  |  |  |  |  |  |  |  |
| 13 | (4,2) | 1 | (4,5) | 0 |  |  |  |  |  |  |  |  |
| 14 | (3,2) | 1 | (3,2) | 1 |  |  |  |  |  |  |  |  |
| 15 | (2,2) | 1 | (2,2) | 1 |  |  |  |  |  |  |  |  |
| 16 | (1,2) | 1 | (1,5) | 0 |  |  |  |  |  |  |  |  |
| 17 | (0,2) | 1 | (0,5) | 0 |  |  |  |  |  |  |  |  |
| 18 | (3,5) | 1 |  |  |  |  |  |  |  |  |  |  |
| 19 | (2,5) | 1 |  |  |  |  |  |  |  |  |  |  |
| 20 | (11,1) | 1 |  |  | (11,1) | 1 |  |  | (11,1) | 1 |  |  |
| 21 | (9,1) | 1 |  |  | (9,1) | 1 |  |  | (9,1) | 1 |  |  |
| 22 | (7,1) | 1 |  |  | (7,1) | 1 |  |  | (7,1) | 1 |  |  |
| 23 | (10,1) | 1 |  |  | (10,1) | 1 |  |  |  |  |  |  |
| 24 | (8,1) | 1 |  |  | (8,1) | 1 |  |  |  |  |  |  |
| 25 | (6,1) | 1 |  |  | (6,1) | 1 |  |  |  |  |  |  |
| 26 | (5,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 27 | (4,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 28 | (3,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 29 | (2,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 30 | (1,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 31 | (0,1) | 1 |  |  |  |  |  |  |  |  |  |  |

Note:	. Configurations 0 – 19 for normal subframes are available for frame structure types 1, 2 and 3. Configurations 20 – 31 and configurations for special subframes are available for frame structure type 2 only.

Table 6.10.5.2-2: Mapping from CSI reference signal configuration to  for extended cyclic prefix.

| CSI-RS config. | Number of CSI reference signals configured |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 or 2 |  |  |  | 4 |  |  |  | 8 |  |  |  |
|  | Normal subframe |  | Special subframe |  | Normal subframe |  | Special subframe |  | Normal Subframe |  | Special subframe |  |
| 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 |
| 1 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 |
| 2 | (10,4) | 1 |  |  | (10,4) | 1 |  |  | (10,4) | 1 |  |  |
| 3 | (9,4) | 1 |  |  | (9,4) | 1 |  |  | (9,4) | 1 |  |  |
| 4 | (5,4) | 0 | (5,4) | 0 | (5,4) | 0 | (5,4) | 0 |  |  |  |  |
| 5 | (3,4) | 0 | (3,4) | 0 | (3,4) | 0 | (3,4) | 0 |  |  |  |  |
| 6 | (4,4) | 1 |  |  | (4,4) | 1 |  |  |  |  |  |  |
| 7 | (3,4) | 1 |  |  | (3,4) | 1 |  |  |  |  |  |  |
| 8 | (8,4) | 0 | (8,4) | 0 |  |  |  |  |  |  |  |  |
| 9 | (6,4) | 0 | (6,4) | 0 |  |  |  |  |  |  |  |  |
| 10 | (2,4) | 0 | (2,4) | 0 |  |  |  |  |  |  |  |  |
| 11 | (0,4) | 0 | (0,4) | 0 |  |  |  |  |  |  |  |  |
| 12 | (7,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 13 | (6,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 14 | (1,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 15 | (0,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 16 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 |
| 17 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 |
| 18 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 |
| 19 | (5,1) | 1 | (5,1) | 1 | (5,1) | 1 | (5,1) | 1 |  |  |  |  |
| 20 | (4,1) | 1 | (4,1) | 1 | (4,1) | 1 | (4,1) | 1 |  |  |  |  |
| 21 | (3,1) | 1 | (3,1) | 1 | (3,1) | 1 | (3,1) | 1 |  |  |  |  |
| 22 | (8,1) | 1 | (8,1) | 1 |  |  |  |  |  |  |  |  |
| 23 | (7,1) | 1 | (7,1) | 1 |  |  |  |  |  |  |  |  |
| 24 | (6,1) | 1 | (6,1) | 1 |  |  |  |  |  |  |  |  |
| 25 | (2,1) | 1 | (2,1) | 1 |  |  |  |  |  |  |  |  |
| 26 | (1,1) | 1 | (1,1) | 1 |  |  |  |  |  |  |  |  |
| 27 | (0,1) | 1 | (0,1) | 1 |  |  |  |  |  |  |  |  |

Note:	. Configurations 0 – 15 for normal subframes are available for both frame structure type 1 and type 2. Configurations 16 – 27 and configurations for special subframes are available for frame structure type 2 only.

Figure 6.10.5.2-1: Mapping of CSI reference signals (CSI configuration 0, normal cyclic prefix)

Figure 6.10.5.2-2: Mapping of CSI reference signals (CSI configuration 0, extended cyclic prefix)

#### 6.10.5.3	CSI reference signal subframe configuration

The subframe configuration period  and the subframe offset  for the occurence of CSI reference signals are listed in Table 6.10.5.3-1. The parameter  can be configured separately for CSI reference signals for which the UE shall assume non-zero and zero transmission power. Subframes containing CSI reference signals that do not correspond to either higher layer configured parameter csi-RS-ConfigNZP-ApList or csi-RS-ConfigZP-ApList shall satisfy .

Table 6.10.5.3-1: CSI reference signal subframe configuration

| CSI-RS-SubframeConfig | CSI-RS periodicity (subframes) | CSI-RS subframe offset (subframes) |
| --- | --- | --- |
| 0 – 4 | 5 |  |
| 5 – 14 | 10 |  |
| 15 – 34 | 20 |  |
| 35 – 74 | 40 |  |
| 75 – 154 | 80 |  |

## 6.11B	MTC wake-up signal (MWUS)

### 6.11B.1	Sequence generation

The MWUS sequence  in subframe x=0, 1, …, M-1 is defined by

wm= θnf, nsm'e-jπunn+1131ej2πgm132

m=0, 1, …, 131

m'=m+132x

n=m mod 132

θnf,nsm'=1 if cnf,ns2m'=0 and cnf,ns2m'+1=0 -1 if cnf,ns2m'=0 and cnf,ns2m'+1=1j if cnf,ns2m'=1 and cnf,ns2m'+1=0-1 if cnf,ns2m'=1 and cnf,ns2m'+1=1

u=NIDcell mod 126+3

where  is the actual duration of MWUS as defined in [4]. For a UE not configured with group MWUS, g=0. For a UE configured with group MWUS, g=14NgroupWUS+1 for 0≤NgroupWUS≤7, where NgroupWUS is determined by the UE group to which the UE is associated as determined by higher layers [10]. The common MWUS sequence shall be determined by  g=126 unless the resource is shared with non-group MWUS and common MWUS is configured to be non-group MWUS in which case g=0.

The scrambling sequence cnf,nsi, i=0, 1, …, 2∙132M-1 is given by clause 7.2, and shall be initialized at the start of the MWUS with

cinit_WUS=(NIDcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDcell+NIDresource∙229

where nf_start_PO is the first frame of the first PO to which the MWUS is associated, ns_start_PO is the first slot of the first PO to which the MWUS is associated and NIDresource indicates the group MWUS resource to which the UE is associated. For a UE not configured with group MWUS, NIDresource=0, whereas for a UE configured with group MWUS, NIDresource is determined by higher layers [10].

### 6.11B.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the MWUS within a subframe. The UE shall not assume that the MWUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. If only one CRS port is configured by the eNB, the UE may assume the transmission of all MWUS subframes is using the same antenna port; otherwise, the UE may assume the same antenna port is used for MWUS transmission in downlink subframes w0 + 2n and w0 + 2n + 1, where w0 is the first downlink subframe of the MWUS transmission as specified in [4], and n=0, 1.

The MWUS bandwidth is 2 consecutive PRBs, the frequency location of the lowermost PRB signaled by higher layers. For both PRB pairs in the frequency domain, for which MWUS is defined, the MWUS sequence wm shall be mapped to resource elements (k,l) in sequence, starting with w(0) in increasing order of first the index k=0, 1, …, NscRB-1, over the 12 assigned subcarriers and then the index l=3, 4, …, 2NsymbDL-1 in each subframe in which MWUS is transmitted.

The MWUS sequence is mapped to the set of subframes in the actual MWUS duration as defined in [4], where in a subframe in which an MWUS PRB pair overlaps with any PRB pair carrying PSS, SSS, RSS, PBCH or PDSCH associated with SI-RNTI is transmitted, the subframe is counted in the MWUS mapping but not used for transmission of MWUS.

A resource element (k,l) overlapping with resource elements where cell-specific reference signals according to clause 6.10 are transmitted shall not be used for MWUS transmission but is counted in the mapping process.