# 3GPP TSG RAN WG Meeting #RP-86	RP-192675

# Sitges, Spain, 09-12 December 2019

# Source:	MediaTek Inc., LG Electronics Inc., Sony Inc., Fujitsu Inc.

# Title:	WID: Higher Layer Protocol Enhancements

# Document for:	Approval

Agenda Item:	9.1.2

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	Higher layer protocol enhancements for NR

## Acronym: HLPEnh

## Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | x |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 670041 | Study on Latency reduction techniques for LTE | Study on how to reduce latency for LTE |
| 710180 | L2 latency reduction techniques for LTE | Work Item on defining instant uplink access |
| 720091 | Shortened TTI and processing time for LTE | Work Item on defining shorter transmission duration and reduced processing time |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

## 3	Justification

TCP is the preeminent transport protocol for broadband applications, but in a high-speed wireless environment like NR, it has some known performance bottlenecks:

- Head-of-line blocking and additional delays: Bidirectional links where UL (or DL) TCP ACKs are likely to be delayed behind regular DL (or UL) TCP data packets and this in turn increases the RTT (Round Trip Time) resulting in a poor TCP performance.The current BSR/SR mechanism requires several handshakes between UE and network, and thus delays the TCP ACK transmission in radio interface.In the receiver side, the PDCP entity performs packet reordering which causes delayed delivery of TCP ACK packets.

- With reduced RTT, receiver can acknowledge TCP packets faster, thus enabling a faster increase in TCP window size during the Slow-Start phase. Hence, as mentioned in R2-1909118, a reduced latency can significantly improve TCP performance. But as observed in Latency Reduction [RP-150465] [TR 36.881], a shorter latency also incur a huge overhead, thereby making the overall effect detrimental to the system.

Current TCP implementation is designed and optimized for stable networks, and does not adapt well upon sudden bandwidth changes. Without a chance to adapt its retransmission timer and TCP window to such a large bandwidth decrement, TCP will assume packet loss and trigger un-necessary congestion control triggering TCP retransmissions. These bottlenecks can significantly impair the performance of TCP in the real-world settings, but can be mitigated within RAN2:

- ACK Prioiritisation for head-of-line blockage: Ensuring UL TCP ACKs are not held up by DL TCP data packets in the transmitter side or as a result of reordering in the receiver side.

- Low latency ACK transmission: Ensuring low latency scheduling and resource allocation for UL TCP ACK transmission.

- Scheduling delays: Exploring shorter latency through a flexible mapping of configured resources and neumerologies.

This work is intended to enhance the NR radio system in order to:

- Significantly increase the data transfer over the NR Uu air interface for an active UE;

- Significantly reduce the packet data transport round trip latency for UE;

- Reduce RTT through flexible mapping of configured resources and numerologies.

The industry wide assumption is that QUIC will eventually replace TCP. The QUIC standardization process is on-going within IETF, however as part of this work it will be possible to investigate some of the claimed benefits of QUIC over TCP:

- Improved congestion control

- Decrease head-of-line blocking

- Reduced connection establishment latency

- Forward error correction

- Connection migration (WLAN and 3GPP RAN)

- Faster development at the user space of the kernel

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The work is expected to proceed as follows:

- Study, and if agreed, specify enhancements to TCP acknowledgement handling:

- TCP ACK prioritisation (for both transmitter and receiver sides);

- Low latency TCP ACK transmission

- Reduce delay by exploring shorter RTT by using flexible resource mapping

- Study the benefits of QUIC over TCP regarding:

  - Recovery from packet loss and/or blockages

  - Throughput improvement

  - Latency reduction

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Rapporteur:	TBD

Company: 	TBD

Email:    	TBD

## 7	Work item leadership

RAN WG2

## 8	Aspects that involve other WGs

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name