3GPP TSG RAN meeting #86										RP-192687

## Sitges, Spain, December 9-12, 2019

## Status Report to TSG

Agenda item:			10.2.4

| WI / SI Name | LTE-based 5G terrestrial broadcast |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_terr_bcast |  |  |  |  |
| Unique ID | 830076 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191924 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2020 | Performance part: 06/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 90% | Performance Part: 0% | Testing part: |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Alberto RICO ALVARINO |
|  | Company | Qualcomm Incorporated |
|  | Email | albertor@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

For the new numerology for rooftop reception, the following was agreed:

RAN1#98b:

Agreement:

For the numerology with 300us CP, the scheduling granularity is 1 OFDM symbol of 3ms

Agreement:

For the TBS determination for the 300us CP numerology, the procedure for calculating the TBS is as follows:

- Reuse the modulation and TBS index table in TS 36.213 and according to the signalled IMCS, obtain ITBS

  - FFS whether modifications to the legacy modulation and TBS index table are needed (to be checked once the RS pattern is agreed)

- Reuse the transport block size table in TS 36.213 and according to the obtained ITBS, obtain the corresponding TBS

- Calculate the used TBS’ as TBS'=round{TBS ×α}, where the round operation maps TBS ×α to the closest TBS in the legacy transport block size tables

  - In case two TBSs are at the same distance of TBS ×α, the larger TBS is chosen

  - The value of α is fixed in the specification to α=3

Working assumption:

For the RS pattern, T_d=4

- FFS: T_d = 2 as a configurable option

RAN1#99:

Agreement:

The working assumption on D_t = 4 and D_f = 3 is confirmed with the following clarifications:

- The pattern is per the figure below:

- Additionally, support D_t=2 and D_f=3. The pattern is per the figure below:

- No additional RS (e.g. RS for frequency tracking) is introduced.

- The network configures between D_t=2 and D_t = 4.

Agreement:

The following tables are agreed for MCS and TBS index for D_t = 4.

Modulation and TBS index table for QPSK, 16QAM, 64QAM:

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 4 | 10 |
| 12 | 4 | 11 |
| 13 | 4 | 12 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 4 | 16 |
| 18 | 4 | 17 |
| 19 | 4 | 18 |
| 20 | 4 | 19 |
| 21 | 6 | 19 |
| 22 | 6 | 20 |
| 23 | 6 | 21 |
| 24 | 6 | 22 |
| 25 | 6 | 23 |
| 26 | 6 | 24 |
| 27 | 6 | 25 |
| 28 | 6 | 26/26A |
| 29 | 2 | reserved |
| 30 | 4 |  |
| 31 | 6 |  |

Modulation and TBS index table for QPSK, 16QAM, 64QAM and 256QAM:

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 2 |
| 2 | 2 | 4 |
| 3 | 2 | 6 |
| 4 | 2 | 8 |
| 5 | 2 | 10 |
| 6 | 4 | 11 |
| 7 | 4 | 12 |
| 8 | 4 | 13 |
| 9 | 4 | 14 |
| 10 | 4 | 15 |
| 11 | 4 | 16 |
| 12 | 4 | 17 |
| 13 | 4 | 18 |
| 14 | 4 | 19 |
| 15 | 6 | 20 |
| 16 | 6 | 21 |
| 17 | 6 | 22 |
| 18 | 6 | 23 |
| 19 | 6 | 24 |
| 20 | 6 | 25 |
| 21 | 8 | 27 |
| 22 | 8 | 28 |
| 23 | 8 | 29 |
| 24 | 8 | 30 |
| 25 | 8 | 31 |
| 26 | 8 | 32 |
| 27 | 8 | 33/33A/33B |
| 28 | 2 | reserved |
| 29 | 4 |  |
| 30 | 6 |  |
| 31 | 8 |  |

Agreement:

The values for A^0370 are {12, 24, FFS, FFS} for baseband capability scaling to support the numerology of 0.370 KHz.

- FFS values to be filled during Rel-16 UE feature discussion

For CAS enhancements, the following was agreed:

RAN1#98b:

Agreement:

For the CFI indication in MIB, use 2 bits to indicate CFI among the following: {notConfigured, 1, 2, 3}

Agreement:

Repetition of legacy PBCH symbols should be defined

- FFS how many repetitions are needed.

- FFS detailed mapping (e.g. whether RE-level rotation, different mapping for different PCI are necessary)

- FFS whether interference randomization techniques are necessary until R1#99.

RAN1#99:

Agreement:

- PBCH repetition is specified as following

  - For downlink bandwidth configuration , PBCH is repeated in subframe #0 of SFN satisfying SFN mod 4 = 0

  - For downlink bandwidth configuration , PBCH is repeated in subframe #0 of SFN satisfying SFN mod 8 = 4.

  - For downlink bandwidth configuration , PBCH is not repeated.

- For CAS subframe where PBCH is repeated, PBCH repetition mapping is the same as defined for LTE-MTC in subframe #0 in FDD system

- The copied PBCH symbols are multiplied with   at OFDM symbol  slot  and subcarrier , where the length-144 Gold sequence at each OFDM symbol  within slot used to determine  is initialized by   , and  is given by:

- NOTE:  is the symbol index within one slot, which can take values from 0 to 6 for normal CP, and from 0 to 5 for ECP

Agreement:

AL16 is introduced, where the lowest CCE of AL16 is aligned with the lowest CCE of a legacy AL8 candidate.

For the new numerology for support of mobility up to 250 km/h, the following was agreed

RAN1#98b:

Agreement:

For the new numerology with 100us CP, the legacy modulation and TBS index table and the legacy transport block size tables are reused.

RAN1#99:

Agreement:

The values for  are {2, 4, 6, 8} for baseband capability scaling to support the numerology of 2.5 KHz.

- FFS: different values (if needed) for the case of time interleaving (if supported)

Additionally, the following agreements were reached:

RAN1#98b:

Agreement:

For the support of numerologies with 100 us and 300 us CP, the baseline for “baseband capability” is the one defined in TS 36.213 subclause 11.1

- FFS: The value range of parameters for the scaling factor for new numerologies

RAN1#99:

Agreement:

R1-1913483 is endorsed in R1-1913542 with the following modifications:

- In Section 4.3, “MIB-MBMS with a 40ms periodicity” is replaced with “MIB-MBMS is transmitted with a 40ms periodicity and updated every 160 ms”

- In Section 4.3, “SIB1-MBMS with a 80ms periodicity” is replaced with “SIB1-MBMS is transmitted with an 80ms periodicity and updated every 160 ms”

#### 2.1.2	Remaining Open issues

- None

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

#### 2.4.1	Agreements

###### RAN4 #92bis (Oct.)

- MBSFN measurement period

  - Long numerology: MAX(640 ms, period during which the UE decodes 5 symbols containing PMCH transmissions)

  - Short numerology: follow legacy MBFSN measurement period

- The current measurement requirements on FeMBMS/Unicast-mixed carrier in R14 can be reused in R16 for LTE-based 5G terrestrial broadcast

- MCH BLER report period

  - Follow legacy MBSFN

- Phase synchronization accuracy

  - 100us CP numerology follows legacy requirement of <= 5us

  - Companies are encouraged to provide analysis or simulation results to determine the 300us CP length requirement

- Numerology switch interruption requirement

  - Option 1(HW): The current numerology switching delay requirements and interruption requirements due to numerology switching in R14 can be reused in R16 for LTE-based 5G terrestrial broadcast

  - Other options are not precluded

- Cell identification and detection requirement

  - No additional requirement is needed, legacy requirement applies to Rel-16 for mixed cells

- RSRP/RSRQ accuracy requirement

  - Follow legacy MBSFN accuracy requirement

- RSRP report mapping

  - Long numerology (300us CP)

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| MBSFN_RSRP_00 | MBSFN_RSRP -156 | dBm / 370Hz |
| MBSFN_RSRP_01 | -156 MBSFN_RSRP < -155 | dBm / 370Hz |
| MBSFN_RSRP_02 | -155 MBSFN_RSRP < -154 | dBm / 370Hz |
| … | … | … |
| MBSFN_RSRP_95 | -62 MBSFN_RSRP < -61 | dBm / 370Hz |
| MBSFN_RSRP_96 | -61 MBSFN_RSRP < -60 | dBm / 370Hz |
| MBSFN_RSRP_97 | -60 MBSFN_RSRP | dBm / 370Hz |

  - Short numerology (100us CP)

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| MBSFN_RSRP_00 | MBSFN_RSRP -148 | dBm / 2.5 kHz |
| MBSFN_RSRP_01 | -148 MBSFN_RSRP < -147 | dBm / 2.5 kHz |
| MBSFN_RSRP_02 | -147 MBSFN_RSRP < -146 | dBm / 2.5 kHz |
| … | … | … |
| MBSFN_RSRP_95 | -54 MBSFN_RSRP < -53 | dBm / 2.5 kHz |
| MBSFN_RSRP_96 | -53 MBSFN_RSRP < -52 | dBm / 2.5 kHz |
| MBSFN_RSRP_97 | -52 MBSFN_RSRP | dBm / 2.5 kHz |

- RSRQ report mapping

  - Long numerology (300us CP)

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| MBSFN_RSRQ_00 | MBSFN_RSRQ -38 | dB |
| MBSFN_RSRQ_01 | -39.0 MBSFN_RSRQ < -38.3 | dB |
| MBSFN_RSRQ_02 | -38.3 MBSFN_RSRQ < -37.6 | dB |
| … | … | … |
| MBSFN_RSRQ_30 | -17.7 MBSFN_RSRQ < -17 | dB |
| MBSFN_RSRQ_31 | -16 MBSFN_RSRQ | dB |

  - Short numerology (100us CP)

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| MBSFN_RSRQ_00 | MBSFN_RSRQ -38 | dB |
| MBSFN_RSRQ_01 | -39.0 MBSFN_RSRQ < -38.3 | dB |
| MBSFN_RSRQ_02 | -38.3 MBSFN_RSRQ < -37.6 | dB |
| … | … | … |
| MBSFN_RSRQ_30 | -17.7 MBSFN_RSRQ < -17 | dB |
| MBSFN_RSRQ_31 | -16 MBSFN_RSRQ | dB |

###### RAN4 #93 (Nov.)

- Reuse Rel-14 numerology change interruption requirement

- Phase synchronization accuracy requirement is <=[5]us for CP length 300us numerology

#### 2.4.2	Remaining Open issues

- Channel delay profile for both long (cp length 300us) and short (cp length 100us) numerologies demod requirement

- Demod performance requirement

  - PMCH decoding performance

  - CAS detection performance

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 4.	References

R1-1911377	Summary of offline discussion on LTE-based 5G Terrestrial Broadcast	Qualcomm Incorporated

R1-1910096	On RS pattern and TBS  for the numerology to support rooftop	Huawei, HiSilicon

R1-1910435	Support for new numerology of rooftop reception	Shanghai Jiao Tong University

R1-1911360	Support of longer numerologies for rooftop reception	Qualcomm Incorporated

R1-1911355	Time and Frequency Interleaver for LTE-based 5G Terrestrial Broadcast	EBU, BBC, IRT

R1-1910733	Analysis of CAS reception	Qualcomm Incorporated

R1-1910098	On MCS and TBS tables for the numerology to support mobility	Huawei, HiSilicon

R1-1910734	Numerology with 100us CP	Qualcomm Incorporated

R1-1910099	Additional simulation results for CAS evaluation	Huawei, HiSilicon

R1-1911240	Echo Delay Profile for Testing Receiver Performance in MBSFN	EBU

R1-1912693	Updated TR 36.276 - Overall description of LTE-based 5G broadcast	Qualcomm Incorporated

R1-1913483	Updated TR 36.276 - Overall description of LTE-based 5G broadcast	Qualcomm Incorporated

R1-1913317	Proposals for terrestrial broadcast	Qualcomm Incorporated

R1-1913412	Proposals for terrestrial broadcast (v2)	Qualcomm Incorporated

R1-1913484	Proposals for terrestrial broadcast (v3)	Qualcomm Incorporated

R1-1913439	On time and frequency interleaving for terrestrial broadcast	EBU, BBC, IRT, NHK, ABS, DISH, Eutelsat, Shanghai Jiao Tong University, Fraunhofer HHI, Fraunhofer IIS, Enensys, Qualcomm, Rohde&Schwarz, Nokia, Nokia Shanghai Bell, ATEME, Nomor Research, Sony

R1-1911913	On RS pattern(s), MCS and UE capabilities for the numerology to support rooftop	Huawei, HiSilicon

R1-1911962	Support for the new numerology of rooftop reception	Shanghai Jiao Tong University

R1-1912331	CRBI-based interleaver for 5G terrestrial broadcasting	ABS,SARI-CAS,Shanghai Jiao Tong University

R1-1913247	Support of longer numerologies for rooftop reception	Qualcomm Incorporated

R1-1913079	Reference Symbol Overheads	EBU/BBC/IRT

R1-1913095	Information on Time Interleaving in State-of-the-Art Terrestrial Broadcast Systems				EBU/BBC/IRT/ABS/NHK

R1-1911914	Potential enhancement for CAS	Huawei, HiSilicon

R1-1911961	PBCH repetition for enhancement in CAS	Shanghai Jiao Tong University

R1-1912691	Analysis of CAS reception	Qualcomm Incorporated

R1-1911915	On UE capabilities and the remaining issues for the numerology to support mobility	Huawei, HiSilicon

R1-1912692	Numerology with 100us CP	Qualcomm Incorporated

R1-1913259	On the performance evaluation of HARQ-based time-interleaving for LTE-based 5G terrestrial broadcast	EBU, BBC, IRT

R1-1912925	Additional simulation results for CAS evaluation	Huawei, HiSilicon

R4-1911284	Work plan for LTE-based 5G terrestrial broadcast Qualcomm Incorporated

R4-1912790	Way forward on RRM requirements for LTE-based 5G terrestrial broadcast Qualcomm Incorporated

R4-1912849	Way forward on RRM requirements for LTE-based 5G terrestrial broadcast Qualcomm Incorporated

R4-1911283	On LTE-based 5G terrestrial broadcast Qualcomm Incorporated

R4-1911912	Discussion on RRM impacts for LTE-based 5G Terrestrial Broadcast Huawei, HiSilicon

R4-1913816	On LTE-based 5G terrestrial broadcast	Qualcomm Incorporated

R4-1913817	CR: RRM core requirement for 5G broadcast	Qualcomm Incorporated

R4-1914740	Echo Delay Profile for Testing UE Performance in MBSFN	EBU/BBC/IRT

R4-1914806	CR on numerology switching delay and interruptions requirements with FeMBMS for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1914807	CR on cell phase synchronization accuracy for MBMS services for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1914808	CR on MBSFN RSRP report mapping with new numerology for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1914809	CR on MBSFN RSRQ report mapping with new numerology for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1914810	CR on MBSFN Measurements in idle mode for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1914811	CR on MBSFN Measurements in connected mode for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1915506	CR: RRM core requirement for 5G broadcast	Qualcomm Incorporated

R4-1915592	CR on numerology switching delay and interruptions requirements with FeMBMS for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1915597	CR on MBSFN Measurements in connected mode for LTE-based 5G broadcast	Huawei, HiSilicon

R4-1915945	CR: RRM core requirement for 5G broadcast	Qualcomm Incorporated