3GPP TSG RAN Meeting #86									  RP-192718

Sitges, Spain, 9th – 12th December 2019

Agenda Item:	9.1.2

Source:	Apple Inc.

Title:	FR2 Mobility in MR-DC

Document for:	Discussion/Decision

# Introduction

The NR Mobility Enhancement Work Item [1] was approved in 3GPP RAN #80 meeting. An objective is “to study solution(s) to improve HO/SCG change reliability and robustness especially considering challenges in high/med frequency”. RAN2 made some progress on conditional HO to improve the mobility robustness in general; however, the challenges specific to high/med frequency still have not been clearly addressed. In addition, for the feature DAPS introduced in Rel-16, it was agreed in RAN2#108 that “DAPS HO for FR2 to FR2 case is not supported in Rel-16.”. The main reason is RAN1#98bis meeting has concluded the following on FR2-FR2 HO scenario.

Recently, two RAN email discussions relate to FR2 mobility [12][13]. In the drafted summary of [12], proposal 2 suggests to study handover interruption reduction in FR2 under MR-DC enhancements WI. And in the draft summary of [13], similar suggestion is also proposed to support 0ms interruption in FR2 in MR-DC WI.

In this contribution, we focused to highlight the importance on the robustness aspect in FR2 (high/med frequency) mobility, and propose to also include it into the scope of Rel-17 MR-DC WI, as one sub-objective of FR2 mobility enhancement.

# Discussion

Motivation for FR2 specific enhancement

NR is expected to work in the frequency range higher than 24 GHz known as FR2 [2]. UE beams are directional in FR2 and therefore, when a UE moves or rotates it may experience rapid signal degradation. Such rapid degradation will result in increased signalling, a high frequency of HO and associated failure rates, and finally, more ping-pong between cells. Therefore, HO is considerably more challenging in NR at FR2 than in LTE.

We measured the number of A4 measurement reports sent by each UE within 5 seconds. Figure 1 shows the CDF of the number of A4 reports. We see that 50% of users send at least 7 A4 reports within 5 seconds while a significant fraction of UEs also send tens of reports.

Figure 2 compares the CDF of the total number of attempted handovers to the best neighbors prepared through the CHO method as well as the number of handovers for which the best neighbor is not prepared through CHO. We observe that the number of attempted handovers is high in FR2 due to fast changes in RSRP. Furthermore, we observe that in most cases the UE can reliably find a suitable neighbor prepared through CHO.

Figure 1: CDF of number of A4 reports    Figure 2: CDF of number of HO attempts

Observation 1: UEs may send a large number of measurement reports due to fast changes in RSRP in FR2 stemming from signal directionality.

Observation 2: Number of attempted HO is large in FR2 due to rapid rate of change in RSRP on account of signal directionality.

We also studied the number of handover failures (HOF) due to RLF when CHO is implemented. We assumed that a UE can attempt to connect to a different cell from the list of prepared cells fulfilling CHO conditions if the HO to the first target cell fails. In order to measure HOF, we counted the number of RLF happening from time HO command from serving cell is received by UE to time that UE accesses to the target cell. Table 1 shows the simulation results and shows that HoF rate due to RLF using Rel. 15 NR mobility solutions is high in FR2. Furthermore, we observed that CHO decreases HOF in FR2 but that HOF is still significantly high. Even with CHO there is a chance that a UE experiences rapid RSRP degradation during HO to the otherwise suitable cell, resulting in RLF.

Table 1: HOF rate due to RLF

| HOF (NR Rel. 15) | 8% |
| --- | --- |
| HOF (CHO) | 1% |

Observation 3: The HOF rate due to RLF using NR release 15 mobility solutions is high in FR2.

We showed some advantages and also issues of deploying CHO in NR at FR2.

Observation 4: CHO decreases HOF rate in FR2 but that HOF rate is still significantly high.

RAN2 progress/schedule in Rel-16 discussion

In RAN2#105bis, some agreements on CHO were made including the followings as captured in meeting report, however no real progress was made on the aspect of high frequency mobility:

Agreements:

0:	CHO is introduced in NR to solve robustness/reliability issue.

2	Cell level quality is used as baseline for CHO execution condition;

FFS: on whether beam quality is used as input for CHO execution condition.

3	 RS type SSB can be used

FFS: CSI-RS, use of more than one RS type

FFS: Enhancements to the above CHO framework to specifically address usage in FR2 (e.g. address high number of handovers, RLFs, etc)

Agreements:

0:	CHO is introduced in NR to solve robustness/reliability issue.

2	Cell level quality is used as baseline for CHO execution condition;

FFS: on whether beam quality is used as input for CHO execution condition.

3	 RS type SSB can be used

FFS: CSI-RS, use of more than one RS type

FFS: Enhancements to the above CHO framework to specifically address usage in FR2 (e.g. address high number of handovers, RLFs, etc)

In RAN2#106 some progress were made for general CHO design, but there is no time for the FR2/high frequency mobility discussion, even though there were quite some contributions submitted to RAN2#106 in [6] - [11]. In RAN2#107, RAN2 agreed that for the scenario of multiple CHO cells UE selects the cell based on beams and beam quality, which still did not address the particular issues in FR2 summarized above.

Observation 5: RAN2 has no time to discusses FR2 specific handover robustness enhancement (CHO).

Proposal in Rel-17

We believe in order to facilitate the progress on this specific issue of mobility enhancement in high/med frequency (FR2), a dedicated objective focusing on FR2 mobility should be included in the WID of Rel-17 MR-DC. Besides the objective of 0ms interruption time in FR2, the other aspect is to further improve the HO robustness in FR2.

Proposal 1: To include the following objective into Rel-17 Multi-Radio Dual-Connectivity enhancements WID.

- Support of FR2 mobility enhancement, aiming at further enhancement on HO robustness. [RAN2, RAN3, RAN4]

# Conclusion

In this contribution, we further highlight the importance of FR2 (high/med frequency) mobility, and propose followings:

Proposal 1: To include the following objective into Rel-17 Multi-Radio Dual-Connectivity enhancements WID.

- Support of FR2 mobility enhancement, aiming at further enhancement on HO robustness. [RAN2, RAN3, RAN4]

# References

- RP-181433, “New WID: NR mobility enhancements,” Intel Corporation, 3GPP RAN #80, June 2018

- TS38.101-2 v15.2.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, June 2018

- R2-1901546, “On NR Mobility in FR2”, Apple Inc., 3GPP RAN2#105, February 2019

- R2-1904980, “Performance of Conditional HO in FR2”, Apple Inc., 3GPP RAN2 #105bis, April 2019

- R2-1904344, “Consideration of Beamforming for NR Conditional Handover”, CMCC, 3GPP RAN2#105bis, April 2019

- R2-1907216, “Enhancements for Conditional Handover”, Apple Inc., 3GPP RAN2#106, May 2019

- R2-1905949, “Consideration of Beamforming for NR Conditional Handover”, CMCC, 3GPP RAN2#106, May 2019

- R2-1906092, “Beam level trigger for conditional handover”, vivo, 3GPP RAN2#106, May 2019

- R2-1906218, “On FR2 impact on CHO”, Ericsson, 3GPP RAN2#106, May 2019

- R2-1908030, “Further consideration of beam quality on NR CHO execution”, Samsung, 3GPP RAN2#106, May 2019

- R2-1907266, “Beam selection and consolidation enhancements”, Nokia, Nokia Shanghai Bell, 3GPP RAN2#106, May 2019

- RP-192437 Summary of email Discussion for FR2 specific enhancements

- RP-192967 Summary of Rel-17 email discussion on MR-DC enhancements