3GPP TSG RAN meeting #86										       RP-192818

Sitges, Spain, December 9-12, 2019

Status Report to TSG

Agenda item:			9.5.14

| WI / SI Name | Introduction of NR FDD bands with variable duplex and corresponding framework |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FDD_bands_varduplex |  |  |  |  |
| Unique ID | 841001 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-192285 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2019 | Performance part: 12/2019 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 90% | Performance Part: 99% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Meng Zhang |
|  | Company | Huawei |
|  | Email | zhangmeng62@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#92bis

Contributions [1] – [20] were submitted to RAN4#92bis meeting. It was agreed in RAN4#92bis meeting that for the introduction of new FDD bands with variable duplex, to enable the variable duplex FDD mode and uplink sharing operation:

For duplex mode,

- Specify n91, n92, n93, n94 as FDD bands with variable duplex operation

- Duplex mode of the bands is FDD

- Specify in the RAN4 spec(s) to clarify variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band

For Tx-Rx separation

- RAN4 will specify the confinement Tx-Rx separation for n91, n92, n93 and n94 as minimum value within the range

For 7.5kHz shift

- 7.5KHz uplink shift requirement is applied for band n91, n92, n93, n94

  - Specify in 38.101-1 this applicability

  - Specify in 38.104 this applicability

- There is no specification impact

For channel bandwidth and SCS

- Specify channel bandwidth and SCS for band n91, n92, n93, n94 in both 38.101-1 and 38.104

  - Clarify which BW is applied for uplink and which is for downlink by notes in the table

  - Asymmetric channel bandwidths will be defined as agreed in RAN4#92

For REFSENS

- Specify REFSENS for n91, n92, n93, n94

  - REFSENS follows the values defined for n75 and n76 in Table X.X correspondingly for each channel bandwidth and SCS

  - REFSENS is based on minimum Tx-Rx separation

For spurious emission

- Specify spurious emission requirements for n91, n92, n93 and n94

  - Specify in 38.104 the additional spurious requirements and colocation requirements with other base stations

  - Specify in 38.141-1 the conformance test requirements for additional spurious and colocation with other base stations

  - Note that n91 and n93 are only for local area BS operation

For mandatory or optional support

- Support for ULSUP is optional for UEs which support FDD bands with variable duplex n91, n92, n93 and n94

RAN4#93

Contributions [21] – [32] were submitted to RAN4#93 meeting.

BS CRs including core and conformance test are technically endorsed in R4-1916058 and R4-1916059. It is agreed in the meeting that, it is common understanding in RAN4 that we do not need new capability signalling for FDD bands with uplink sharing.

Chairman proposal:

- Timing mask requirement is generic requirements for all the bands including New FDD bands with variable duplex and other bands with uplink sharing

- RAN4 is going to introduce the generic timing requirements in

  - either separated WI (new WI or existing WI)

  - SUL band combination basket WI

- WI for FDD band with variable duplex will proceed based on Rel-15 requirements without any new signalling in Rel-16

- If all the band specific requirements for new FDD band with variable duplex are completed, this WI can be closed

2.4.2	Remaining Open issues

- Core part:

  - Specify UE RF requirements for the new NR frequency bands n91 n92 n93 and n94, with Release independence from Rel-15

  - Remaining BS RF requirements to be introduced in 36.xxx and 37.xxx specs

  - Discussion on timing requirements for UL sharing (amended time masks)

- Perf. part:

  - Remaining BS RF requirements to be introduced in 36.xxx and 37.xxx specs

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R4-1911183 System parameters for new FDD bands with variable duplex ZTE

- R4-1913028 WF on UE and BS RF requirements for introducation of FDD bands with variable duplex Huawei

- R4-1913075 WF on UE and BS RF requirements for introducation of FDD bands with variable duplex Huawei

- R4-1911550	On the timing requiremens for FDD bands with variable duplex Ericsson

- R4-1911646	UE capability aspects for new NR FDD bands with variable duplex Nokia, Nokia Shanghai Bell

- R4-1912392	Variable duplexer spacing Qualcomm Incorporated

- R4-1911290	CR to 38.101-1 adding band n91 n92 n93 and n94 Huawei, HiSilicon, Vodafone

- R4-1911291	CR to 38.101-1 for channel raster and sync raster for new variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911292	CR to 38.101-1 on 7.5kHz shift for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911293	CR to 38.101-1 on channel bandwidth and SCS for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911294	CR to 38.101-1 on the minimum Tx-Rx separation for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911295	CR to 38.101-1 on the asymmetric channel BW for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911296	Discussion on the system parameters for the variable duplex FDD bands Huawei, HiSilicon

- R4-1912456	On BS requirements for NR FDD bands with variable duplex Nokia, Nokia Shanghai Bell

- R4-1911297	CR to 38.104 adding band n91 n92 n93 and n94 Huawei, HiSilicon, Vodafone

- R4-1911298	CR to 38.104 for channel raster and sync raster for new variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911299	CR to 38.104 on 7.5kHz shift for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911300	CR to 38.104 on channel bandwidth and SCS for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911301	CR to 38.141-1 on OBUE test requirements for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1911302	CR to 38.141-1 on spurious emissions test requirements for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1913911	UE reference architecture for NR FDD bands with variable duplex MediaTek Inc.

- R4-1914145	Introduction of timing requirements for FDD bands with variable duplex Ericsson

- R4-1914158	Discussion on the deviation confinement of Tx-Rx separation for asymmetric bandwidth under variable duplex operation Huawei, HiSilicon

- R4-1914270	UE capability aspects for new NR FDD bands with variable duplex Nokia, Nokia Shanghai Bell

- R4-1916180 LS to RAN2 on FDD band capability signalling for uplink sharing Nokia, Nokia Shanghai Bell

- R4-1914146	Introduction of variable-duplex FDD bands Ericsson

- R4-1914167	CR to 38.101-1 for FDD bands with variable duplex Huawei, HiSilicon, Vodafone

- R4-1916057	CR to 38.101-1 for FDD bands with variable duplex Huawei, HiSilicon, Vodafone

- R4-1914168	CR to 38.104 for FDD bands with variable duplex Huawei, HiSilicon, Vodafone

- R4-1916058	CR to 38.104 for FDD bands with variable duplex Huawei, HiSilicon, Vodafone

- R4-1914169	CR to 38.141-1 for variable duplex FDD bands Huawei, HiSilicon, Vodafone

- R4-1916059	CR to 38.141-1 for variable duplex FDD bands Huawei, HiSilicon, Vodafone

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template