3GPP TSG-RAN WG3 Meeting #106 	R3-197556

# Reno, NV, USA, 18. - 22.11.2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0236 | rev | 2 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SN Status Transfer for bearer reconfiguration during HO with DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 25.10.2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During a HO, the target node may reconfigure some SN-terminated bearers into MN-terminated ones. Currently, however, the source may not provide the SN status for the transferred DRBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A list of DRBs transferred to MN is added to the HO REQUEST ACKNOWLEDGE and S-NODE RELEASE REQUEST messages. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only clarifies existing functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some bearers may have to be dropped during a HO with DC activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 9.2.1.2, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: The flag is changed into a list of affected DRBs; the list is also added to the Release procedure. |  |  |  |  |  |  |  |  |

*** First change, skipped text not changed ***

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

For each E-RAB ID IE included in the QoS Flow To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the IE in the UE context and use it for subsequent inter-system handover.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

At reception of the HANDOVER REQUEST message the target NG-RAN node shall prepare the configuration of the AS security relation between the UE and the target NG-RAN node by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE, as specified in TS 33.501 [28].

Upon reception of the PDU Session Resource Setup List IE, contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as specified in TS 38.413 [5] for the PDU Session Resource Setup procedure. The target NG-RAN node shall report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU session resources. When the target NG-RAN node reports the unsuccessful establishment of a PDU session resource, the cause value should be precise enough to enable the source NG-RAN node to know the reason for the unsuccessful establishment.

For each PDU session if the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store the received PDU Session Aggregate Maximum Bit Rate in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [7].

For each QoS flow for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. For each PDU session that the target NG-RAN node decides to admit the data forwarding for at least one QoS flow, the target NG-RAN node includes the PDU Session level DL data forwarding GTP-U Tunnel Endpoint IE within the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resource Admitted Info IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

For each QoS flow for which the source NG-RAN node has not yet received the SDAP end marker packet if QoS flow re-mapping happened before handover, the source NG-RAN node shall include the UL Forwarding IE within the Data Forwarding and Offloading Info from source NG-RAN node IE in the HANDOVER REQUEST message, and if the target NG-RAN node decides to admit uplink data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted Item IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the uplink data forwarding.

For each PDU session resource successfully setup at the target NG-RAN, the target NG-RAN node may allocate resources for additional Xn-U PDU session resource GTP-U tunnels, indicated in the Secondary Data Forwarding Info from target NG-RAN node List IE.

For each DRB for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DRB ID IE and the mapped QoS Flows List IE within the Source DRB to QoS Flow Mapping List IE contained in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node may include the QoS Flow Mapping Indication IE in the Source DRB to QoS Flow Mapping List IE to indicate that only the uplink or downlink QoS flow is mapped to the DRB. If the target NG-RAN node decides to use the same DRB configuration and to map the same QoS flows as the source NG-RAN node, the target NG-RAN node includes the DL Forwarding GTP Tunnel Endpoint IE within the Data Forwarding Response DRB List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this DRB.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding GTP Tunnel Endpoint IE for a given DRB in the Data Forwarding Response DRB List IE within Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted List IE and the source NG-RAN node accepts the data forwarding proposed by the target NG-RAN node, the source NG-RAN node shall perform forwarding of uplink data for the DRB.

If the HANDOVER REQUEST includes PDU session resources for PDU sessions associated to S-NSSAIs not supported by target NG-RAN, the target NG-RAN shall reject such PDU session resources. In this case, and if at least one PDU Session Resource To Be Setup Item IE is admitted, the target NG-RAN shall send the HANDOVER REQUEST ACKNOWLEDGE message including the PDU Session Resources Not Admitted List IE listing corresponding PDU sessions rejected at the target NG-RAN.

If the Mobility Restriction List IE is

-	contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	store the information received in the Mobility Restriction List IE in the UE context;

-	use this information to determine a target for the UE during subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE, except when one of the PDU sessions has a particular ARP value (TS 23.501 [7]) in which case the information shall not apply;

-	use this information to select a proper SCG during dual connectivity operation.

-	use this information to select proper RNA(s) for the UE when moving the UE to RRC_INACTIVE.

-	not contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	consider that no roaming and no access restriction apply to the UE.

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Index to RAT/Frequency Selection Priority IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the UE Context Reference at the S-NG-RAN IE is contained in the HANDOVER REQUEST message the target NG-RAN node may use it as specified in TS 37.340 [8]. In this case, the source NG-RAN node may expect the target NG-RAN node to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [8].

If the UE Context Kept Indicator IE set to "True" is included, then if the DRBs transferred to MN IE is included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource To Be Setup List IE and the Common Network Instance IE is not present, the target NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", the target NG-RAN node shall perform user plane integrity protection or ciphering, respectively. If the NG-RAN node is not able to perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required".

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or the Confidentiality Protection Indication IE is set to "preferred", the target NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively and shall notify the SMF whether it succeeded the user plane integrity protection or ciphering or not for the concerned security policy.

For each PDU session for which the Maximum Integrity Protected Data Rate IE is included in the Security Indication IE in the PDU Session Resources To Be Setup List IE, the NG-RAN node shall store the respective information and, if integrity protection is to be performed for the PDU session, it shall enforce the traffic corresponding to the received Maximum Integrity Protected Data Rate IE, for the concerned PDU session and concerned UE, as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "not needed", the target NG-RAN node shall not perform user plane integrity protection or ciphering, respectively, for the concerned PDU session.

For each PDU session, if the Additional UL NG-U UP TNL Information List IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the target NG-RAN node may forward the UP transport layer information to the target S-NG-RAN node as the uplink termination point for the user plane data for this PDU session split in different tunnel.

If the Location Reporting Information IE is included in the HANDOVER REQUEST message, then the target NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

Upon reception of UE History Information IE in the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

*** Next change, skipped text not changed ***

### 8.3.6	M-NG-RAN node initiated S-NG-RAN node Release

#### 8.3.6.1	General

The M-NG-RAN node initiated S-NG-RAN node Release procedure is triggered by the M-NG-RAN node to initiate the release of the resources for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: M-NG-RAN node initiated S-NG-RAN node Release, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUEST message. Upon reception of the S-NODE RELEASE REQUEST message the S-NG-RAN node shall stop providing user data to the UE.

The S-NG-RAN node UE XnAP ID IE shall be included if it has been obtained from the S-NG-RAN node. The M-NG-RAN node shall provide appropriate information within the Cause IE. The M-NG-RAN node may also provide appropriate information per PDU session resource within the Cause IE of the PDU Session Resources To Be Released List IE.

Upon reception of the S-NODE RELEASE REQUEST message containing UE Context Kept Indicator IE set to "True", the S-NG-RAN node shall, if supported, only initiate the release of the resources related to the UE-associated signalling connection between the M-NG-RAN node and the S-NG-RAN node.

If the UE Context Kept Indicator IE set to "True" is included, then if the DRBs transferred to MN IE is included in the S-NODE RELEASE REQUEST message, the S-NG-RAN node shall, if supported, provide the uplink/downlink PDCP SN and HFN status for the listed DRBs, as specified in TS 37.340 [8].

If the S-NG-RAN node confirms the request to release S-NG-RAN node resources, it shall send the S-NODE RELEASE REQUEST ACKNOWLEDGE message to the M-NG-RAN node.

If the S-NODE RELEASE REQUEST message contains a PDU session resource to be released which is configured with the SCG bearer option within the PDU Session Resources To Be Released List IE, the S-NG-RAN node shall include the RLC Mode IE within the DRBs To Be Released List IE in the S-NODE RELEASE REQUEST ACKNOWLEDGE message. The RLC Mode IE indicates the RLC mode used in the S-NG-RAN node for the DRB.

Interaction with the Xn-U Address Indication procedure

If the S-NG-RAN node provides data forwarding related information in the S-NODE RELEASE REQUEST ACKNOWLEDGE message for QoS flows mapped to DRBs configured with an SN terminated bearer option in the PDU Sessions To Be Released List - SN terminated IE, the M-NG-RAN node may decide to provide data forwarding addresses to the S-NG-RAN node and trigger the Xn-U Address Indication procedure as specified in TS 37.340 [8].

*** Next change, skipped text not changed ***

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |

*** Next change, skipped text not changed ***

#### 9.1.2.14	S-NODE RELEASE REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the release of resources.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session Resources To Be Released List | O |  | PDU session List with Cause 9.2.1.26 |  | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | Indicates that the target M-NG-RAN node reconfigured the listed DRBs as MN-terminated bearers. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

*** Next change, skipped text not changed ***

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport	,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

handoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

retrieveUEContext	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

rANPaging	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANPaging

PROCEDURE CODE			id-rANPaging

CRITICALITY				reject

}

xnUAddressIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnUAddressIndication

PROCEDURE CODE			id-xnUAddressIndication

CRITICALITY				reject

}

uEContextRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				reject

}

sNGRANnodeAdditionPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeAdditionRequest

SUCCESSFUL OUTCOME		SNodeAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeAdditionRequestReject

PROCEDURE CODE			id-sNGRANnodeAdditionPreparation

CRITICALITY				reject

}

sNGRANnodeReconfigurationCompletion	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReconfigurationComplete

PROCEDURE CODE			id-sNGRANnodeReconfigurationCompletion

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequest

SUCCESSFUL OUTCOME		SNodeModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeModificationRequestReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequired

SUCCESSFUL OUTCOME		SNodeModificationConfirm

UNSUCCESSFUL OUTCOME	SNodeModificationRefuse

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequest

SUCCESSFUL OUTCOME		SNodeReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeReleaseReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequired

SUCCESSFUL OUTCOME		SNodeReleaseConfirm

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeCounterCheck	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeCounterCheckRequest

PROCEDURE CODE			id-sNGRANnodeCounterCheck

CRITICALITY				reject

}

sNGRANnodeChange		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeChangeRequired

SUCCESSFUL OUTCOME		SNodeChangeConfirm

UNSUCCESSFUL OUTCOME	SNodeChangeRefuse

PROCEDURE CODE			id-sNGRANnodeChange

CRITICALITY				reject

}

rRCTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

xnRemoval	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnRemovalRequest

SUCCESSFUL OUTCOME		XnRemovalResponse

UNSUCCESSFUL OUTCOME		XnRemovalFailure

PROCEDURE CODE			id-xnRemoval

CRITICALITY				reject

}

xnSetup	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnSetupRequest

SUCCESSFUL OUTCOME		XnSetupResponse

UNSUCCESSFUL OUTCOME		XnSetupFailure

PROCEDURE CODE			id-xnSetup

CRITICALITY				reject

}

nGRANnodeConfigurationUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRANNodeConfigurationUpdate

SUCCESSFUL OUTCOME		NGRANNodeConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	NGRANNodeConfigurationUpdateFailure

PROCEDURE CODE			id-nGRANnodeConfigurationUpdate

CRITICALITY				reject

}

e-UTRA-NR-CellResourceCoordination	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-UTRA-NR-CellResourceCoordinationRequest

SUCCESSFUL OUTCOME		E-UTRA-NR-CellResourceCoordinationResponse

PROCEDURE CODE			id-e-UTRA-NR-CellResourceCoordination

CRITICALITY				reject

}

cellActivation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

reset	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

notificationControl			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NotificationControlIndication

PROCEDURE CODE			id-notificationControl

CRITICALITY				ignore

}

activityNotification		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ActivityNotification

PROCEDURE CODE			id-activityNotification

CRITICALITY				ignore

}

privateMessage			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

secondaryRATDataUsageReport	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

END

-- ASN1STOP

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAssistanceInfo-NR,

CPTransportLayerInformation,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

CriticalityDiagnostics,

XnUAddressInfoperPDUSession-List,

DataTrafficResourceIndication,

DeliveryStatus,

DesiredActNotificationLevel,

DRB-ID,

DRB-List,

DRB-Number,

DRBsSubjectToStatusTransfer-List,

DRBToQoSFlowMapping-List,

E-UTRA-CGI,

ExpectedUEBehaviour,

GlobalNG-RANNode-ID,

GlobalNG-RANCell-ID,

GUAMI,

InterfaceInstanceIndication,

I-RNTI,

LocationInformationSNReporting,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

MR-DC-ResourceCoordinationInfo,

ServedCells-E-UTRA,

ServedCells-NR,

ServedCellsToUpdate-E-UTRA,

ServedCellsToUpdate-NR,

MAC-I,

MaskedIMEISV,

MobilityRestrictionList,

NG-RAN-Cell-Identity,

NG-RANnodeUEXnAPID,

NR-CGI,

NE-DC-TDM-Pattern,

PagingDRX,

PagingPriority,

PLMN-Identity,

PDCPChangeIndication,

PDUSessionAggregateMaximumBitRate,

PDUSession-ID,

PDUSession-List,

PDUSession-List-withCause,

PDUSession-List-withDataForwardingFromTarget,

PDUSession-List-withDataForwardingRequest,

PDUSessionResourcesAdmitted-List,

PDUSessionResourcesNotAdmitted-List,

PDUSessionResourcesToBeSetup-List,

PDUSessionResourceChangeRequiredInfo-SNterminated,

PDUSessionResourceChangeRequiredInfo-MNterminated,

PDUSessionResourceChangeConfirmInfo-SNterminated,

PDUSessionResourceChangeConfirmInfo-MNterminated,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupInfo-SNterminated,

PDUSessionResourceSetupInfo-MNterminated,

PDUSessionResourceSetupResponseInfo-SNterminated,

PDUSessionResourceSetupResponseInfo-MNterminated,

PDUSessionResourceModificationInfo-SNterminated,

PDUSessionResourceModificationInfo-MNterminated,

PDUSessionResourceModificationResponseInfo-SNterminated,

PDUSessionResourceModificationResponseInfo-MNterminated,

PDUSessionResourceModConfirmInfo-SNterminated,

PDUSessionResourceModConfirmInfo-MNterminated,

PDUSessionResourceModRqdInfo-SNterminated,

PDUSessionResourceModRqdInfo-MNterminated,

PDUSessionType,

QoSFlowIdentifier,

QoSFlowNotificationControlIndicationInfo,

QoSFlows-List,

RANPagingArea,

ResetRequestTypeInfo,

ResetResponseTypeInfo,

RFSP-Index,

RRCConfigIndication,

RRCResumeCause,

SCGConfigurationQuery,

SecurityIndication,

S-NG-RANnode-SecurityKey,

SpectrumSharingGroupID,

SplitSRBsTypes,

S-NG-RANnode-Addition-Trigger-Ind,

S-NSSAI,

TAISupport-List,

Target-CGI,

TimeToWait,

TraceActivation,

UEAggregateMaximumBitRate,

UEContextID,

UEContextInfoRetrUECtxtResp,

UEContextKeptIndicator,

UEHistoryInformation,

UEIdentityIndexValue,

UERadioCapabilityForPaging,

UERANPagingIdentity,

UESecurityCapabilities,

UPTransportLayerInformation,

UserPlaneTrafficActivityReport,

XnBenefitValue,

RANPagingFailure

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

id-AMF-Region-Information-To-Delete,

id-AssistanceDataForRANPaging,

id-AvailableDRBIDs,

id-Cause,

id-cellAssistanceInfo-NR,

id-ConfigurationUpdateInitiatingNodeChoice,

id-UEContextID,

id-CriticalityDiagnostics,

id-XnUAddressInfoperPDUSession-List,

id-DesiredActNotificationLevel,

id-DRBsSubjectToStatusTransfer-List,

id-ExpectedUEBehaviour,

id-GlobalNG-RAN-node-ID,

id-GUAMI,

id-indexToRatFrequSelectionPriority,

id-List-of-served-cells-E-UTRA,

id-List-of-served-cells-NR,

id-LocationInformationSN,

id-LocationInformationSNReporting,

id-LocationReportingInformation,

id-MAC-I,

id-MaskedIMEISV,

id-MN-to-SN-Container,

id-MobilityRestrictionList,

id-M-NG-RANnodeUEXnAPID,

id-new-NG-RAN-Cell-Identity,

id-newNG-RANnodeUEXnAPID,

id-oldNG-RANnodeUEXnAPID,

id-OldtoNewNG-RANnodeResumeContainer,

id-PagingDRX,

id-PagingPriority,

id-PCellID,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourcesActivityNotifyList,

id-PDUSessionResourcesAdmitted-List,

id-PDUSessionResourcesNotAdmitted-List,

id-PDUSessionResourcesNotifyList,

id-PDUSessionToBeAddedAddReq,

id-PDUSessionToBeReleased-RelReqAck,

id-RANPagingArea,

id-requestedSplitSRB,

id-RequiredNumberOfDRBIDs,

id-ResetRequestTypeInfo,

id-ResetResponseTypeInfo,

id-RespondingNodeTypeConfigUpdateAck,

id-RRCResumeCause,

id-selectedPLMN,

id-ServedCellsToActivate,

id-servedCellsToUpdate-E-UTRA,

id-ServedCellsToUpdateInitiatingNodeChoice,

id-servedCellsToUpdate-NR,

id-sourceNG-RANnodeUEXnAPID,

id-SpareDRBIDs,

id-S-NG-RANnodeMaxIPDataRate-UL,

id-S-NG-RANnodeMaxIPDataRate-DL,

id-S-NG-RANnodeUEXnAPID,

id-TAISupport-list,

id-Target2SourceNG-RANnodeTranspContainer,

id-targetCellGlobalID,

id-targetNG-RANnodeUEXnAPID,

id-TimeToWait,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-TraceActivation,

id-UEContextInfoHORequest,

id-UEContextInfoRetrUECtxtResp,

id-UEContextKeptIndicator,

id-UEContextRefAtSN-HORequest,

id-UEHistoryInformation,

id-UEIdentityIndexValue,

id-UERANPagingIdentity,

id-UESecurityCapabilities,

id-UserPlaneTrafficActivityReport,

id-XnRemovalThreshold,

id-PDUSessionAdmittedAddedAddReqAck,

id-PDUSessionNotAdmittedAddReqAck,

id-SN-to-MN-Container,

id-RRCConfigIndication,

id-SplitSRB-RRCTransfer,

id-UEReportRRCTransfer,

id-PDUSessionReleasedList-RelConf,

id-BearersSubjectToCounterCheck,

id-PDUSessionToBeReleasedList-RelRqd,

id-ResponseInfo-ReconfCompl,

id-initiatingNodeType-ResourceCoordRequest,

id-respondingNodeType-ResourceCoordResponse,

id-PDUSessionToBeReleased-RelReq,

id-PDUSession-SNChangeRequired-List,

id-PDUSession-SNChangeConfirm-List,

id-PDCPChangeIndication,

id-SCGConfigurationQuery,

id-UEContextInfo-SNModRequest,

id-requestedSplitSRBrelease,

id-PDUSessionAdmitted-SNModResponse,

id-PDUSessionNotAdmitted-SNModResponse,

id-admittedSplitSRB,

id-admittedSplitSRBrelease,

id-PDUSessionAdmittedModSNModConfirm,

id-PDUSessionReleasedSNModConfirm,

id-s-ng-RANnode-SecurityKey,

id-PDUSessionToBeModifiedSNModRequired,

id-S-NG-RANnodeUE-AMBR,

id-PDUSessionToBeReleasedSNModRequired,

id-target-S-NG-RANnodeID,

id-S-NSSAI,

id-MR-DC-ResourceCoordinationInfo,

id-RANPagingFailure,

id-UERadioCapabilityForPaging,

id-PDUSessionDataForwarding-SNModResponse,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-NE-DC-TDM-Pattern,

id-InterfaceInstanceIndication,

id-S-NG-RANnode-Addition-Trigger-Ind,

id-DRBs-transferred-to-MN,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional },

...

}

UEContextInfoHORequest ::= SEQUENCE {

ng-c-UE-reference						AMF-UE-NGAP-ID,

cp-TNL-info-source						CPTransportLayerInformation,

ueSecurityCapabilities					UESecurityCapabilities,

securityInformation						AS-SecurityInformation,

indexToRatFrequencySelectionPriority	RFSP-Index															OPTIONAL,

ue-AMBR									UEAggregateMaximumBitRate,

pduSessionResourcesToBeSetup-List		PDUSessionResourcesToBeSetup-List,

rrc-Context								OCTET STRING,

locationReportingInformation			LocationReportingInformation										OPTIONAL,

mrl										MobilityRestrictionList												OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {UEContextInfoHORequest-ExtIEs} }	OPTIONAL,

...

}

UEContextInfoHORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

UEContextRefAtSN-HORequest ::= SEQUENCE {

globalNG-RANNode-ID				GlobalNG-RANNode-ID,

sN-NG-RANnodeUEXnAPID			NG-RANnodeUEXnAPID,

iE-Extensions					ProtocolExtensionContainer { {UEContextRefAtSN-HORequest-ExtIEs} }	OPTIONAL,

...

}

UEContextRefAtSN-HORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

-- **************************************************************

--

#### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequestAcknowledge-IEs}},

...

}

HandoverRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionResourcesAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesAdmitted-List			PRESENCE mandatory}|

{ ID id-PDUSessionResourcesNotAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesNotAdmitted-List		PRESENCE optional }|

{ ID id-Target2SourceNG-RANnodeTranspContainer	CRITICALITY ignore	TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore	TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore	TYPE DRB-List										PRESENCE optional },

...

}

-- **************************************************************

--

#### -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverPreparationFailure-IEs}},

...

}

HandoverPreparationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional },

...

}

-- **************************************************************

--

#### -- SN STATUS TRANSFER

--

-- **************************************************************

SNStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{SNStatusTransfer-IEs}},

...

}

SNStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-DRBsSubjectToStatusTransfer-List		CRITICALITY ignore		TYPE DRBsSubjectToStatusTransfer-List		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- UE CONTEXT RELEASE

--

-- **************************************************************

UEContextRelease ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{UEContextRelease-IEs}},

...

}

UEContextRelease-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverCancel-IEs}},

...

}

HandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue				CRITICALITY reject		TYPE UEIdentityIndexValue							PRESENCE mandatory}|

{ ID id-UERANPagingIdentity					CRITICALITY ignore		TYPE UERANPagingIdentity							PRESENCE mandatory}|

{ ID id-PagingDRX							CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea						CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-PagingPriority						CRITICALITY ignore		TYPE PagingPriority									PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging			CRITICALITY ignore		TYPE AssistanceDataForRANPaging					PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging			CRITICALITY ignore		TYPE UERadioCapabilityForPaging					PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UEContextID								CRITICALITY reject		TYPE UEContextID									PRESENCE mandatory}|

{ ID id-MAC-I									CRITICALITY reject		TYPE MAC-I											PRESENCE mandatory}|

{ ID id-new-NG-RAN-Cell-Identity				CRITICALITY reject		TYPE NG-RAN-Cell-Identity						PRESENCE mandatory}|

{ ID id-RRCResumeCause							CRITICALITY ignore		TYPE RRCResumeCause									PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-GUAMI									CRITICALITY reject		TYPE GUAMI											PRESENCE mandatory}|

{ ID id-UEContextInfoRetrUECtxtResp				CRITICALITY reject		TYPE UEContextInfoRetrUECtxtResp				PRESENCE mandatory}|

{ ID id-TraceActivation							CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-MaskedIMEISV							CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-LocationReportingInformation			CRITICALITY ignore		TYPE LocationReportingInformation				PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT FAILURE

--

-- **************************************************************

RetrieveUEContextFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextFailure-IEs}},

...

}

RetrieveUEContextFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-OldtoNewNG-RANnodeResumeContainer		CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN-U ADDRESS INDICATION

--

-- **************************************************************

XnUAddressIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnUAddressIndication-IEs}},

...

}

XnUAddressIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-XnUAddressInfoperPDUSession-List		CRITICALITY reject		TYPE XnUAddressInfoperPDUSession-List		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional}|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional}|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional}|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional},

...

}

PDUSessionToBeAddedAddReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionToBeAddedAddReq-Item

PDUSessionToBeAddedAddReq-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeAddedAddReq-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeAddedAddReq-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestAcknowledge-IEs}},

...

}

SNodeAdditionRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedAddedAddReqAck	CRITICALITY ignore		TYPE PDUSessionAdmittedAddedAddReqAck		PRESENCE mandatory}|

{ ID id-PDUSessionNotAdmittedAddReqAck		CRITICALITY ignore		TYPE PDUSessionNotAdmittedAddReqAck		PRESENCE optional }|

{ ID id-SN-to-MN-Container					CRITICALITY reject		TYPE OCTET STRING								PRESENCE mandatory}|

{ ID id-admittedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes								PRESENCE optional }|

{ ID id-RRCConfigIndication					CRITICALITY reject		TYPE RRCConfigIndication						PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-LocationInformationSN				CRITICALITY ignore		TYPE Target-CGI									PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo		PRESENCE optional },

...

}

PDUSessionAdmittedAddedAddReqAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedAddedAddReqAck-Item

PDUSessionAdmittedAddedAddReqAck-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

sn-terminated			PDUSessionResourceSetupResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE

-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionNotAdmittedAddReqAck ::= SEQUENCE {

pduSessionResourcesNotAdmitted-SNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

pduSessionResourcesNotAdmitted-MNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionNotAdmittedAddReqAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionNotAdmittedAddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST REJECT

--

-- **************************************************************

SNodeAdditionRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestReject-IEs}},

...

}

SNodeAdditionRequestReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RECONFIGURATION COMPLETE

--

-- **************************************************************

SNodeReconfigurationComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReconfigurationComplete-IEs}},

...

}

SNodeReconfigurationComplete-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-ResponseInfo-ReconfCompl				CRITICALITY ignore		TYPE ResponseInfo-ReconfCompl					PRESENCE mandatory},

...

}

ResponseInfo-ReconfCompl ::= SEQUENCE {

responseType-ReconfComplete		ResponseType-ReconfComplete,

iE-Extensions						ProtocolExtensionContainer { {ResponseInfo-ReconfCompl-ExtIEs} } OPTIONAL,

...

}

ResponseInfo-ReconfCompl-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResponseType-ReconfComplete ::= CHOICE {

configuration-successfully-applied			Configuration-successfully-applied,

configuration-rejected-by-M-NG-RANNode		Configuration-rejected-by-M-NG-RANNode,

choice-extension					ProtocolIE-Single-Container { {ResponseType-ReconfComplete-ExtIEs} }

}

ResponseType-ReconfComplete-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Configuration-successfully-applied ::= SEQUENCE {

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-successfully-applied-ExtIEs} } OPTIONAL,

...

}

Configuration-successfully-applied-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Configuration-rejected-by-M-NG-RANNode ::= SEQUENCE {

cause											Cause,

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-rejected-by-M-NG-RANNode-ExtIEs} } OPTIONAL,

...

}

Configuration-rejected-by-M-NG-RANNode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST

--

-- **************************************************************

SNodeModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequest-IEs}},

...

}

SNodeModificationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication						PRESENCE optional }|

{ ID id-selectedPLMN							CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList					CRITICALITY ignore		TYPE MobilityRestrictionList					PRESENCE optional }|

{ ID id-SCGConfigurationQuery					CRITICALITY ignore		TYPE SCGConfigurationQuery						PRESENCE optional }|

{ ID id-UEContextInfo-SNModRequest				CRITICALITY reject		TYPE UEContextInfo-SNModRequest				PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-requestedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-requestedSplitSRBrelease				CRITICALITY ignore		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL			CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL			CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting			CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional}|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-PCellID									CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern						CRITICALITY ignore		TYPE NE-DC-TDM-Pattern							PRESENCE optional},

...

}

UEContextInfo-SNModRequest ::= SEQUENCE {

ueSecurityCapabilities							UESecurityCapabilities								OPTIONAL,

s-ng-RANnode-SecurityKey						S-NG-RANnode-SecurityKey							OPTIONAL,

s-ng-RANnodeUE-AMBR								UEAggregateMaximumBitRate							OPTIONAL,

indexToRatFrequencySelectionPriority			RFSP-Index											OPTIONAL,

lowerLayerPresenceStatusChange					LowerLayerPresenceStatusChange						OPTIONAL,

pduSessionResourceToBeAdded						PDUSessionsToBeAdded-SNModRequest-List				OPTIONAL,

pduSessionResourceToBeModified					PDUSessionsToBeModified-SNModRequest-List			OPTIONAL,

pduSessionResourceToBeReleased					PDUSessionsToBeReleased-SNModRequest-List			OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {UEContextInfo-SNModRequest-ExtIEs} }			OPTIONAL,

...

}

UEContextInfo-SNModRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionsToBeAdded-SNModRequest-List ::= SEQUENCE (SIZE(maxnoofPDUSessions)) OF PDUSessionsToBeAdded-SNModRequest-Item

PDUSessionsToBeAdded-SNModRequest-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionsToBeModified-SNModRequest-List ::= SEQUENCE (SIZE(maxnoofPDUSessions)) OF PDUSessionsToBeModified-SNModRequest-Item

PDUSessionsToBeModified-SNModRequest-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate				OPTIONAL,

sn-terminated			PDUSessionResourceModificationInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModificationInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE

-- nor the PDU Session Resource Modification Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeModified-SNModRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeModified-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-S-NSSAI		CRITICALITY reject	EXTENSION S-NSSAI		PRESENCE optional},

...

}

PDUSessionsToBeReleased-SNModRequest-List ::= SEQUENCE {

pdu-session-list		PDUSession-List-withCause				OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeReleased-SNModRequest-List-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeReleased-SNModRequest-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestAcknowledge-IEs}},

...

}

SNodeModificationRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionAdmitted-SNModResponse		CRITICALITY ignore		TYPE PDUSessionAdmitted-SNModResponse			PRESENCE optional }|

{ ID id-PDUSessionNotAdmitted-SNModResponse		CRITICALITY ignore		TYPE PDUSessionNotAdmitted-SNModResponse		PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING										PRESENCE optional }|

{ ID id-admittedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-admittedSplitSRBrelease					CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics							PRESENCE optional }|

{ ID id-LocationInformationSN					CRITICALITY ignore		TYPE Target-CGI											PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo				PRESENCE optional }|

{ ID id-PDUSessionDataForwarding-SNModResponse	CRITICALITY ignore		TYPE PDUSessionDataForwarding-SNModResponse	PRESENCE optional }|

{ ID id-RRCConfigIndication						CRITICALITY reject		TYPE RRCConfigIndication								PRESENCE optional },

...

}

PDUSessionAdmitted-SNModResponse ::= SEQUENCE {

pduSessionResourcesAdmittedToBeAdded			PDUSessionAdmittedToBeAddedSNModResponse 			OPTIONAL,

pduSessionResourcesAdmittedToBeModified			PDUSessionAdmittedToBeModifiedSNModResponse 		OPTIONAL,

pduSessionResourcesAdmittedToBeReleased			PDUSessionAdmittedToBeReleasedSNModResponse			OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmitted-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeAddedSNModResponse ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeAddedSNModResponse-Item

PDUSessionAdmittedToBeAddedSNModResponse-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sn-terminated			PDUSessionResourceSetupResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE

-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeModifiedSNModResponse::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeModifiedSNModResponse-Item

PDUSessionAdmittedToBeModifiedSNModResponse-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sn-terminated			PDUSessionResourceModificationResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModificationResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Response Info – SN terminated IE

-- nor the PDU Session Resource Modification Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeReleasedSNModResponse ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingRequest		OPTIONAL,

mn-terminated			PDUSession-List-withCause						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionNotAdmitted-SNModResponse ::= SEQUENCE {

pdu-Session-List		PDUSession-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionNotAdmitted-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionNotAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionDataForwarding-SNModResponse ::= SEQUENCE {

sn-terminated		PDUSession-List-withDataForwardingRequest,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionDataForwarding-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionDataForwarding-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST REJECT

--

-- **************************************************************

SNodeModificationRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestReject-IEs}},

...

}

SNodeModificationRequestReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUIRED

--

-- **************************************************************

SNodeModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequired-IEs}},

...

}

SNodeModificationRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication						PRESENCE optional }|

{ ID id-PDUSessionToBeModifiedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeModifiedSNModRequired	PRESENCE optional }|

{ ID id-PDUSessionToBeReleasedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeReleasedSNModRequired	PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-SpareDRBIDs								CRITICALITY ignore		TYPE DRB-List										PRESENCE optional }|

{ ID id-RequiredNumberOfDRBIDs					CRITICALITY ignore		TYPE DRB-Number										PRESENCE optional }|

{ ID id-LocationInformationSN					CRITICALITY ignore		TYPE Target-CGI										PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-RRCConfigIndication						CRITICALITY reject		TYPE RRCConfigIndication							PRESENCE optional },

...

}

PDUSessionToBeModifiedSNModRequired::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF 	PDUSessionToBeModifiedSNModRequired-Item

PDUSessionToBeModifiedSNModRequired-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

sn-terminated			PDUSessionResourceModRqdInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModRqdInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Required Info – SN terminated IE

-- nor the PDU Session Resource Modification Required Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.4.4 apply.

iE-Extension		ProtocolExtensionContainer { {PDUSessionToBeModifiedSNModRequired-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeModifiedSNModRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionToBeReleasedSNModRequired ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingRequest		OPTIONAL,

mn-terminated			PDUSession-List-withCause						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeReleasedSNModRequired-ExtIEs} } OPTIONAL,

...

}

PDUSessionToBeReleasedSNModRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION CONFIRM

--

-- **************************************************************

SNodeModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationConfirm-IEs}},

...

}

SNodeModificationConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedModSNModConfirm		CRITICALITY ignore		TYPE PDUSessionAdmittedModSNModConfirm		PRESENCE optional }|

{ ID id-PDUSessionReleasedSNModConfirm			CRITICALITY ignore		TYPE PDUSessionReleasedSNModConfirm			PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional },

...

}

PDUSessionAdmittedModSNModConfirm ::= SEQUENCE (SIZE(maxnoofPDUSessions)) OF PDUSessionAdmittedModSNModConfirm-Item

PDUSessionAdmittedModSNModConfirm-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceModConfirmInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModConfirmInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Confirm Info – SN terminated IE

-- nor the PDU Session Resource Modification Confirm Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.4.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedModSNModConfirm-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedModSNModConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionReleasedSNModConfirm ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingFromTarget						OPTIONAL,

mn-terminated			PDUSession-List														OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REFUSE

--

-- **************************************************************

SNodeModificationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRefuse-IEs}},

...

}

SNodeModificationRefuse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST

--

-- **************************************************************

SNodeReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequest-IEs}},

...

}

SNodeReleaseRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleased-RelReq			CRITICALITY ignore		TYPE PDUSession-List-withCause					PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore		TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore		TYPE DRB-List										PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeReleaseRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequestAcknowledge-IEs}},

...

}

SNodeReleaseRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID						CRITICALITY reject		TYPE NG-RANnodeUEXnAPID									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID						CRITICALITY reject		TYPE NG-RANnodeUEXnAPID									PRESENCE optional }|

{ ID id-PDUSessionToBeReleased-RelReqAck			CRITICALITY ignore		TYPE PDUSessionToBeReleasedList-RelReqAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore		TYPE CriticalityDiagnostics							PRESENCE optional },

...

}

PDUSessionToBeReleasedList-RelReqAck ::= SEQUENCE {

pduSessionsToBeReleasedList-SNterminated		PDUSession-List-withDataForwardingRequest												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelReqAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeReleasedList-RelReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REJECT

--

-- **************************************************************

SNodeReleaseReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseReject-IEs}},

...

}

SNodeReleaseReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUIRED

--

-- **************************************************************

SNodeReleaseRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequired-IEs}},

...

}

SNodeReleaseRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleasedList-RelRqd		CRITICALITY ignore		TYPE PDUSessionToBeReleasedList-RelRqd		PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory},

...

}

PDUSessionToBeReleasedList-RelRqd ::= SEQUENCE {

pduSessionsToBeReleasedList-SNterminated		PDUSession-List-withDataForwardingRequest						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelRqd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeReleasedList-RelRqd-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE CONFIRM

--

-- **************************************************************

SNodeReleaseConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseConfirm-IEs}},

...

}

SNodeReleaseConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionReleasedList-RelConf			CRITICALITY ignore		TYPE PDUSessionReleasedList-RelConf			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

PDUSessionReleasedList-RelConf ::= SEQUENCE {

pduSessionsReleasedList-SNterminated		PDUSession-List-withDataForwardingFromTarget					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionReleasedList-RelConf-ExtIEs} }	OPTIONAL,

...

}

PDUSessionReleasedList-RelConf-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE COUNTER CHECK REQUEST

--

-- **************************************************************

SNodeCounterCheckRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeCounterCheckRequest-IEs}},

...

}

SNodeCounterCheckRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-BearersSubjectToCounterCheck			CRITICALITY ignore		TYPE BearersSubjectToCounterCheck-List		PRESENCE mandatory},

...

}

BearersSubjectToCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF BearersSubjectToCounterCheck-Item

BearersSubjectToCounterCheck-Item ::= SEQUENCE {

drb-ID							DRB-ID,

ul-count						INTEGER (0.. 4294967295),

dl-count						INTEGER (0.. 4294967295),

iE-Extensions					ProtocolExtensionContainer { {BearersSubjectToCounterCheck-Item-ExtIEs} }	OPTIONAL,

...

}

BearersSubjectToCounterCheck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE REQUIRED

--

-- **************************************************************

SNodeChangeRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRequired-IEs}},

...

}

SNodeChangeRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-target-S-NG-RANnodeID					CRITICALITY reject		TYPE GlobalNG-RANNode-ID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSession-SNChangeRequired-List		CRITICALITY ignore		TYPE PDUSession-SNChangeRequired-List		PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory},

...

}

PDUSession-SNChangeRequired-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeRequired-Item

PDUSession-SNChangeRequired-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceChangeRequiredInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceChangeRequiredInfo-MNterminated	OPTIONAL,

-- NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present,

-- abnormal conditions as specified in clause 8.3.5.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSession-SNChangeRequired-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-SNChangeRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE CONFIRM

--

-- **************************************************************

SNodeChangeConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeConfirm-IEs}},

...

}

SNodeChangeConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSession-SNChangeConfirm-List			CRITICALITY ignore		TYPE PDUSession-SNChangeConfirm-List			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

PDUSession-SNChangeConfirm-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeConfirm-Item

PDUSession-SNChangeConfirm-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceChangeConfirmInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceChangeConfirmInfo-MNterminated	OPTIONAL,

-- NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present,

-- abnormal conditions as specified in clause 8.3.5.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSession-SNChangeConfirm-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-SNChangeConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE REFUSE

--

-- **************************************************************

SNodeChangeRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRefuse-IEs}},

...

}

SNodeChangeRefuse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RRC TRANSFER

--

-- **************************************************************

RRCTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RRCTransfer-IEs}},

...

}

RRCTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-SplitSRB-RRCTransfer					CRITICALITY reject		TYPE SplitSRB-RRCTransfer						PRESENCE optional }|

{ ID id-UEReportRRCTransfer		CRITICALITY reject		TYPE UEReportRRCTransfer		PRESENCE optional },

...

}

SplitSRB-RRCTransfer ::= SEQUENCE {

rrcContainer					OCTET STRING						OPTIONAL,

srbType							ENUMERATED {srb1, srb2, ...},

deliveryStatus					DeliveryStatus						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {SplitSRB-RRCTransfer-ExtIEs} } OPTIONAL,

...

}

SplitSRB-RRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEReportRRCTransfer::= SEQUENCE {

rrcContainer					OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { {UEReportRRCTransfer-ExtIEs} } OPTIONAL,

...

}

UEReportRRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- NOTIFICATION CONTROL INDICATION

--

-- **************************************************************

NotificationControlIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{NotificationControlIndication-IEs}},

...

}

NotificationControlIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionResourcesNotifyList		CRITICALITY reject		TYPE PDUSessionResourcesNotifyList			PRESENCE optional },

...

}

PDUSessionResourcesNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesNotify-Item

PDUSessionResourcesNotify-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

qosFlowsNotificationContrIndInfo	QoSFlowNotificationControlIndicationInfo,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesNotify-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- ACTIVITY NOTIFICATION

--

-- **************************************************************

ActivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ActivityNotification-IEs}},

...

}

ActivityNotification-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UserPlaneTrafficActivityReport			CRITICALITY ignore		TYPE UserPlaneTrafficActivityReport			PRESENCE optional }|

{ ID id-PDUSessionResourcesActivityNotifyList	CRITICALITY ignore		TYPE PDUSessionResourcesActivityNotifyList	PRESENCE optional }|

{ ID id-RANPagingFailure						CRITICALITY ignore		TYPE RANPagingFailure								PRESENCE optional },

...

}

PDUSessionResourcesActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesActivityNotify-Item

PDUSessionResourcesActivityNotify-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

pduSessionLevelUPactivityreport		UserPlaneTrafficActivityReport											OPTIONAL,

qosFlowsActivityNotifyList			QoSFlowsActivityNotifyList												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesActivityNotify-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesActivityNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsActivityNotifyItem

QoSFlowsActivityNotifyItem ::= SEQUENCE {

qosFlowIdentifier					QoSFlowIdentifier,

pduSessionLevelUPactivityreport		UserPlaneTrafficActivityReport,

iE-Extensions						ProtocolExtensionContainer { {QoSFlowsActivityNotifyItem-ExtIEs} } OPTIONAL,

...

}

QoSFlowsActivityNotifyItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- XN SETUP REQUEST

--

-- **************************************************************

XnSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupRequest-IEs}},

...

}

XnSetupRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-TAISupport-list				CRITICALITY reject	TYPE TAISupport-List				PRESENCE mandatory}|

{ ID id-AMF-Region-Information		CRITICALITY reject	TYPE AMF-Region-Information			PRESENCE mandatory}|

{ ID id-List-of-served-cells-NR		CRITICALITY reject	TYPE ServedCells-NR					PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA	CRITICALITY reject	TYPE ServedCells-E-UTRA				PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN SETUP RESPONSE

--

-- **************************************************************

XnSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupResponse-IEs}},

...

}

XnSetupResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-TAISupport-list				CRITICALITY reject	TYPE TAISupport-List				PRESENCE optional }|

{ ID id-List-of-served-cells-NR		CRITICALITY reject	TYPE ServedCells-NR					PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA	CRITICALITY reject	TYPE ServedCells-E-UTRA				PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-AMF-Region-Information		CRITICALITY reject	TYPE AMF-Region-Information			PRESENCE optional}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN SETUP FAILURE

--

-- **************************************************************

XnSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupFailure-IEs}},

...

}

XnSetupFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE

--

-- **************************************************************

NGRANNodeConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdate-IEs}},

...

}

NGRANNodeConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TAISupport-list							CRITICALITY reject	TYPE TAISupport-List									PRESENCE optional }|

{ ID id-ConfigurationUpdateInitiatingNodeChoice	CRITICALITY ignore	TYPE ConfigurationUpdateInitiatingNodeChoice	PRESENCE mandatory}|

{ ID id-TNLA-To-Add-List						CRITICALITY ignore	TYPE TNLA-To-Add-List									PRESENCE optional }|

{ ID id-TNLA-To-Remove-List						CRITICALITY ignore	TYPE TNLA-To-Remove-List								PRESENCE optional }|

{ ID id-TNLA-To-Update-List						CRITICALITY ignore	TYPE TNLA-To-Update-List								PRESENCE optional }|

{ ID id-GlobalNG-RAN-node-ID					CRITICALITY reject	TYPE GlobalNG-RANNode-ID								PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Add			CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Delete		CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication					PRESENCE optional },

...

}

ConfigurationUpdateInitiatingNodeChoice ::= CHOICE {

gNB									ProtocolIE-Container	{ {ConfigurationUpdate-gNB} },

ng-eNB								ProtocolIE-Container	{ {ConfigurationUpdate-ng-eNB} },

choice-extension					ProtocolIE-Single-Container { {ServedCellsToUpdateInitiatingNodeChoice-ExtIEs} }

}

ServedCellsToUpdateInitiatingNodeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ConfigurationUpdate-gNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-NR			CRITICALITY ignore TYPE	ServedCellsToUpdate-NR					PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR			CRITICALITY ignore TYPE	CellAssistanceInfo-NR					PRESENCE optional },

...

}

ConfigurationUpdate-ng-eNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-E-UTRA	CRITICALITY ignore TYPE	ServedCellsToUpdate-E-UTRA				PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR		CRITICALITY ignore TYPE	CellAssistanceInfo-NR			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

NGRANNodeConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdateAcknowledge-IEs}},

...

}

NGRANNodeConfigurationUpdateAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeTypeConfigUpdateAck	CRITICALITY ignore	TYPE RespondingNodeTypeConfigUpdateAck		PRESENCE mandatory}|

{ ID id-TNLA-Setup-List						CRITICALITY ignore	TYPE TNLA-Setup-List							PRESENCE optional }|

{ ID id-TNLA-Failed-To-Setup-List			CRITICALITY ignore	TYPE TNLA-Failed-To-Setup-List				PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject	TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

RespondingNodeTypeConfigUpdateAck ::= CHOICE {

ng-eNB					RespondingNodeTypeConfigUpdateAck-ng-eNB,

gNB						RespondingNodeTypeConfigUpdateAck-gNB,

choice-extension		ProtocolIE-Single-Container { {RespondingNodeTypeConfigUpdateAck-ExtIEs} }

}

RespondingNodeTypeConfigUpdateAck-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RespondingNodeTypeConfigUpdateAck-ng-eNB ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs} }	OPTIONAL,

...

}

RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RespondingNodeTypeConfigUpdateAck-gNB ::= SEQUENCE {

served-NR-Cells		ServedCells-NR																OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs} }	OPTIONAL,

...

}

RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

NGRANNodeConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{NGRANNodeConfigurationUpdateFailure-IEs}},

...

}

NGRANNodeConfigurationUpdateFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- E-UTRA NR CELL RESOURCE COORDINATION REQUEST

--

-- **************************************************************

E-UTRA-NR-CellResourceCoordinationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{E-UTRA-NR-CellResourceCoordinationRequest-IEs}},

...

}

E-UTRA-NR-CellResourceCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-initiatingNodeType-ResourceCoordRequest	CRITICALITY reject	TYPE InitiatingNodeType-ResourceCoordRequest		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication						PRESENCE optional },

...

}

InitiatingNodeType-ResourceCoordRequest ::= CHOICE {

ng-eNB								ResourceCoordRequest-ng-eNB-initiated,

gNB									ResourceCoordRequest-gNB-initiated,

choice-extension					ProtocolIE-Single-Container { {InitiatingNodeType-ResourceCoordRequest-ExtIEs} }

}

InitiatingNodeType-ResourceCoordRequest-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResourceCoordRequest-ng-eNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI							OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordRequest-ng-eNB-initiated-ExtIEs} }	OPTIONAL,

...

}

ResourceCoordRequest-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordRequest-gNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI						OPTIONAL,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofNRCells						SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordRequest-gNB-initiated-ExtIEs} }	OPTIONAL,

...

}

ResourceCoordRequest-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- E-UTRA NR CELL RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

E-UTRA-NR-CellResourceCoordinationResponse::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{E-UTRA-NR-CellResourceCoordinationResponse-IEs}},

...

}

E-UTRA-NR-CellResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-respondingNodeType-ResourceCoordResponse  CRITICALITY reject  	TYPE RespondingNodeType-ResourceCoordResponse 	PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication						PRESENCE optional },

...

}

RespondingNodeType-ResourceCoordResponse ::= CHOICE {

ng-eNB								ResourceCoordResponse-ng-eNB-initiated,

gNB									ResourceCoordResponse-gNB-initiated,

choice-extension					ProtocolIE-Single-Container { {RespondingNodeType-ResourceCoordResponse-ExtIEs} }

}

RespondingNodeType-ResourceCoordResponse-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResourceCoordResponse-ng-eNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-ng-eNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordResponse-gNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofNRCells						SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-gNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SECONDARY RAT DATA USAGE REPORT

--

-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SecondaryRATDataUsageReport-IEs}},

...

}

SecondaryRATDataUsageReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionResourceSecondaryRATUsageList	CRITICALITY reject		TYPE PDUSessionResourceSecondaryRATUsageList	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- XN REMOVAL REQUEST

--

-- **************************************************************

XnRemovalRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalRequest-IEs}},

...

}

XnRemovalRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-XnRemovalThreshold			CRITICALITY reject	TYPE XnBenefitValue					PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN REMOVAL RESPONSE

--

-- **************************************************************

XnRemovalResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalResponse-IEs}},

...

}

XnRemovalResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN REMOVAL FAILURE

--

-- **************************************************************

XnRemovalFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalFailure-IEs}},

...

}

XnRemovalFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION REQUEST

--

-- **************************************************************

CellActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ CellActivationRequest-IEs}},

...

}

CellActivationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToActivate					CRITICALITY reject		TYPE ServedCellsToActivate						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

ServedCellsToActivate ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolIE-Single-Container { {ServedCellsToActivate-ExtIEs} }

}

ServedCellsToActivate-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION RESPONSE

--

-- **************************************************************

CellActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationResponse-IEs}},

...

}

CellActivationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivatedServedCells					CRITICALITY reject		TYPE ActivatedServedCells						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

ActivatedServedCells ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolIE-Single-Container { {ActivatedServedCells-ExtIEs} }

}

ActivatedServedCells-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION FAILURE

--

-- **************************************************************

CellActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationFailure-IEs}},

...

}

CellActivationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESET REQUEST

--

-- **************************************************************

ResetRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetRequest-IEs}},

...

}

ResetRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResetRequestTypeInfo					CRITICALITY reject		TYPE ResetRequestTypeInfo						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESET RESPONSE

--

-- **************************************************************

ResetResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetResponse-IEs}},

...

}

ResetResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResetResponseTypeInfo					CRITICALITY reject		TYPE ResetResponseTypeInfo						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

-- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs XNAP-PRIVATE-IES ::= {

...

}

END

-- ASN1STOP

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-Additional-UL-NG-U-TNLatUPF-List,

id-DefaultDRB-Allowed,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods

FROM XnAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM XnAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES

FROM XnAP-Containers;

#### -- A

Additional-UL-NG-U-TNLatUPF-Item ::= SEQUENCE {

additional-UL-NG-U-TNLatUPF				UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { { Additional-UL-NG-U-TNLatUPF-Item-ExtIEs} }	OPTIONAL,

...

}

Additional-UL-NG-U-TNLatUPF-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Additional-UL-NG-U-TNLatUPF-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF Additional-UL-NG-U-TNLatUPF-Item

ActivationIDforCellActivation	::= INTEGER (0..255)

AllocationandRetentionPriority ::= SEQUENCE {

priorityLevel					INTEGER (0..15,...),

pre-emption-capability			ENUMERATED {shall-not-trigger-preemptdatDion, may-trigger-preemption, ...},

pre-emption-vulnerability		ENUMERATED {not-preemptable, preemptable, ...},

iE-Extensions					ProtocolExtensionContainer { {AllocationandRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationandRetentionPriority-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ActivationSFN ::= INTEGER (0..1023)

AMF-Region-Information ::= SEQUENCE (SIZE (1..maxnoofAMFRegions)) OF GlobalAMF-Region-Information

GlobalAMF-Region-Information ::= SEQUENCE {

plmn-ID				PLMN-Identity,

amf-region-id		BIT STRING (SIZE (8)),

iE-Extensions					ProtocolExtensionContainer { {GlobalAMF-Region-Information-ExtIEs} } OPTIONAL,

...

}

GlobalAMF-Region-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-UE-NGAP-ID ::= INTEGER (0..1099511627775)

AreaOfInterestInformation ::= SEQUENCE (SIZE(1..maxnoofAoIs)) OF AreaOfInterest-Item

AreaOfInterest-Item ::= SEQUENCE {

listOfTAIsinAoI					ListOfTAIsinAoI												OPTIONAL,

listOfCellsinAoI				ListOfCells													OPTIONAL,

listOfRANNodesinAoI				ListOfRANNodesinAoI											OPTIONAL,

requestReferenceID	RequestReferenceID,

iE-Extensions					ProtocolExtensionContainer { {AreaOfInterest-Item-ExtIEs} } OPTIONAL,

...

}

AreaOfInterest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AS-SecurityInformation ::= SEQUENCE {

key-NG-RAN-Star					BIT STRING (SIZE(256)),

ncc								INTEGER (0..7),

iE-Extensions					ProtocolExtensionContainer { {AS-SecurityInformation-ExtIEs} } OPTIONAL,

...

}

AS-SecurityInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForRANPaging ::= SEQUENCE {

ran-paging-attempt-info			RANPagingAttemptInfo	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {AssistanceDataForRANPaging-ExtIEs} } OPTIONAL,

...

}

AssistanceDataForRANPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow ::= INTEGER (0..4095, ...)

#### -- B

BPLMN-ID-Info-EUTRA ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF BPLMN-ID-Info-EUTRA-Item

BPLMN-ID-Info-EUTRA-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastEUTRAPLMNs,

tac								TAC,

e-utraCI						E-UTRA-Cell-Identity,

ranac							RANAC OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-EUTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BPLMN-ID-Info-NR ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BPLMN-ID-Info-NR-Item

BPLMN-ID-Info-NR-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastPLMNs,

tac								TAC,

nr-CI							NR-Cell-Identity,

ranac							RANAC OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-NR-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BitRate	::= INTEGER (0..4000000000000,...)

BroadcastPLMNs ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BroadcastEUTRAPLMNs ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF PLMN-Identity

BroadcastPLMNinTAISupport-Item ::= SEQUENCE {

plmn-id							PLMN-Identity,

tAISliceSupport-List			SliceSupport-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastPLMNinTAISupport-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNinTAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetworkLayer,

transport			CauseTransportLayer,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-Single-Container { {Cause-ExtIEs} }

}

Cause-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CauseRadioNetworkLayer ::= ENUMERATED {

cell-not-available,

handover-desirable-for-radio-reasons,

handover-target-not-allowed,

invalid-AMF-Set-ID,

no-radio-resources-available-in-target-cell,

partial-handover,

reduce-load-in-serving-cell,

resource-optimisation-handover,

time-critical-handover,

tXnRELOCoverall-expiry,

tTXnRELOCprep-expiry,

unknown-GUAMI-ID,

unknown-local-NG-RAN-node-UE-XnAP-ID,

inconsistent-remote-NG-RAN-node-UE-XnAP-ID,

encryption-and-or-integrity-protection-algorithms-not-supported,

protection-algorithms-not-supported,

multiple-PDU-session-ID-instances,

unknown-PDU-session-ID,

unknown-QoS-Flow-ID,

multiple-QoS-Flow-ID-instances,

switch-off-ongoing,

not-supported-5QI-value,

tXnDCoverall-expiry,

tXnDCprep-expiry,

action-desirable-for-radio-reasons,

reduce-load,

resource-optimisation,

time-critical-action,

target-not-allowed,

no-radio-resources-available,

invalid-QoS-combination,

encryption-algorithms-not-supported,

procedure-cancelled,

rRM-purpose,

improve-user-bit-rate,

user-inactivity,

radio-connection-with-UE-lost,

failure-in-the-radio-interface-procedure,

bearer-option-not-supported,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

resources-not-available-for-the-slice-s,

ue-max-IP-data-rate-reason,

cP-integrity-protection-failure,

uP-integrity-protection-failure,

slice-not-supported-by-NG-RAN,

mN-Mobility,

sN-Mobility,

count-reaches-max-value,

unknown-old-en-gNB-UE-X2AP-ID,

pDCP-Overload,

drb-id-not-available,

unspecified,

...,

ue-context-id-not-known,

non-relocation-of-context

}

CauseTransportLayer ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

hardware-failure,

o-and-M-intervention,

not-enough-user-plane-processing-resources,

unspecified,

...

}

CellAssistanceInfo-NR	::= CHOICE {

limitedNR-List				SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

full-List					ENUMERATED {all-served-cells-NR, ...},

choice-extension			ProtocolIE-Single-Container { {CellAssistanceInfo-NR-ExtIEs} }

}

CellAssistanceInfo-NR-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CellGroupID ::= INTEGER (0..maxnoofSCellGroups)

Connectivity-Support		::= SEQUENCE {

eNDC-Support			ENUMERATED {supported, not-supported, ...},

iE-Extensions			ProtocolExtensionContainer { {Connectivity-Support-ExtIEs} }	OPTIONAL,

...

}

Connectivity-Support-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

COUNT-PDCP-SN12 ::= SEQUENCE {

pdcp-SN12						INTEGER (0..4095),

hfn-PDCP-SN12					INTEGER (0..1048575),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN12-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

COUNT-PDCP-SN18 ::= SEQUENCE {

pdcp-SN18						INTEGER (0..262143),

hfn-PDCP-SN18					INTEGER (0..16383),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN18-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPTransportLayerInformation ::= CHOICE {

endpointIPAddress			TransportLayerAddress,

choice-extension			ProtocolIE-Single-Container { {CPTransportLayerInformation-ExtIEs} }

}

CPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= BIT STRING (SIZE(16))

CyclicPrefix-E-UTRA-DL ::= ENUMERATED {

normal,

extended,

...

}

CyclicPrefix-E-UTRA-UL ::= ENUMERATED {

normal,

extended,

...

}

#### -- D

XnUAddressInfoperPDUSession-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF XnUAddressInfoperPDUSession-Item

XnUAddressInfoperPDUSession-Item ::= SEQUENCE {

pduSession-ID			PDUSession-ID,

dataForwardingInfoFromTargetNGRANnode		DataForwardingInfoFromTargetNGRANnode										OPTIONAL,

pduSessionResourceSetupCompleteInfo-SNterm			PDUSessionResourceBearerSetupCompleteInfo-SNterminated		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { XnUAddressInfoperPDUSession-Item-ExtIEs} }					OPTIONAL,

...

}

XnUAddressInfoperPDUSession-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondarydataForwardingInfoFromTarget-List	CRITICALITY ignore	EXTENSION SecondarydataForwardingInfoFromTarget-List	PRESENCE optional},

...

}

DataForwardingAccepted	::= ENUMERATED {data-forwarding-accepted, ...}

DataForwardingInfoFromTargetNGRANnode ::= SEQUENCE {

qosFlowsAcceptedForDataForwarding-List			QoSFLowsAcceptedToBeForwarded-List,

pduSessionLevelDLDataForwardingInfo				UPTransportLayerInformation								OPTIONAL,

pduSessionLevelULDataForwardingInfo				UPTransportLayerInformation								OPTIONAL,

dataForwardingResponseDRBItemList				DataForwardingResponseDRBItemList						OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataForwardingInfoFromTargetNGRANnode-ExtIEs} }	OPTIONAL,

...

}

DataForwardingInfoFromTargetNGRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFLowsAcceptedToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsAcceptedToBeForwarded-Item

QoSFLowsAcceptedToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

iE-Extension				ProtocolExtensionContainer { {QoSFLowsAcceptedToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsAcceptedToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataforwardingandOffloadingInfofromSource ::= SEQUENCE {

qosFlowsToBeForwarded			QoSFLowsToBeForwarded-List,

sourceDRBtoQoSFlowMapping		DRBToQoSFlowMapping-List													OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataforwardingandOffloadingInfofromSource-ExtIEs} }	OPTIONAL,

...

}

DataforwardingandOffloadingInfofromSource-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFLowsToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsToBeForwarded-Item

QoSFLowsToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

dl-dataforwarding			DLForwarding,

ul-dataforwarding			ULForwarding,

iE-Extension		ProtocolExtensionContainer { {QoSFLowsToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingResponseDRBItemList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {

drb-ID				DRB-ID,

dlForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

ulForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataForwardingResponseDRBItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataTrafficResources ::= BIT STRING (SIZE(6..17600))

DataTrafficResourceIndication ::= SEQUENCE {

activationSFN					ActivationSFN,

sharedResourceType				SharedResourceType,

reservedSubframePattern			ReservedSubframePattern										OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataTrafficResourceIndication-ExtIEs} }	OPTIONAL,

...

}

DataTrafficResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DeliveryStatus	::= INTEGER (0..4095, ...)

DesiredActNotificationLevel	::= ENUMERATED {none, qos-flow, pdu-session, ue-level, ...}

DefaultDRB-Allowed ::= ENUMERATED {true, false, ...}

DLForwarding	::= ENUMERATED {dl-forwarding-proposed, ...}

DRB-ID	::= INTEGER (1..32, ...)

DRB-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-ID

DRB-List-withCause ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-List-withCause-Item

DRB-List-withCause-Item ::= SEQUENCE {

drb-id		DRB-ID,

cause		Cause,

rLC-Mode	RLCMode								OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRB-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

DRB-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Number ::= INTEGER (1..32, ...)

DRBsSubjectToStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToStatusTransfer-Item

DRBsSubjectToStatusTransfer-Item ::= SEQUENCE {

drbID				DRB-ID,

pdcpStatusTransfer-UL	DRBBStatusTransferChoice,

pdcpStatusTransfer-DL	DRBBStatusTransferChoice,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject		EXTENSION QoSFlows-List					PRESENCE optional },

...

}

DRBBStatusTransferChoice ::= CHOICE {

pdcp-sn-12bits		DRBBStatusTransfer12bitsSN,

pdcp-sn-18bits		DRBBStatusTransfer18bitsSN,

choice-extension		ProtocolIE-Single-Container { {DRBBStatusTransferChoice-ExtIEs} }

}

DRBBStatusTransferChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

DRBBStatusTransfer12bitsSN ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..2048))											OPTIONAL,

cOUNTValue				COUNT-PDCP-SN12,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer12bitsSN-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer12bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBBStatusTransfer18bitsSN ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..131072))										OPTIONAL,

cOUNTValue				COUNT-PDCP-SN18,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer18bitsSN-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer18bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBToQoSFlowMapping-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBToQoSFlowMapping-Item

DRBToQoSFlowMapping-Item ::= SEQUENCE {

drb-ID							DRB-ID,

qosFlows-List					QoSFlows-List,

rLC-Mode						RLCMode								OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRBToQoSFlowMapping-Item-ExtIEs} }	OPTIONAL,

...

}

DRBToQoSFlowMapping-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DuplicationActivation ::= ENUMERATED {active, inactive, ...}

Dynamic5QIDescriptor ::= SEQUENCE {

priorityLevelQoS			PriorityLevelQoS,

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

fiveQI						FiveQI													OPTIONAL,

delayCritical				ENUMERATED {delay-critical, non-delay-critical, ...}	OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow				AveragingWindow											OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

E-RAB-ID		::= INTEGER (0..15, ...)

E-UTRAARFCN ::= INTEGER (0..maxEARFCN)

E-UTRA-Cell-Identity			::= BIT STRING (SIZE(28))

E-UTRA-CGI ::= SEQUENCE {

plmn-id				PLMN-Identity,

e-utra-CI			E-UTRA-Cell-Identity,

iE-Extension		ProtocolExtensionContainer { {E-UTRA-CGI-ExtIEs} } 	OPTIONAL,

...

}

E-UTRA-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRAFrequencyBandIndicator ::= INTEGER (1..256, ...)

E-UTRAMultibandInfoList ::= SEQUENCE (SIZE(1..maxnoofEUTRABands)) OF E-UTRAFrequencyBandIndicator

E-UTRAPCI ::= INTEGER (0..503, ...)

E-UTRAPRACHConfiguration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							ENUMERATED {true, false, ...},

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)								OPTIONAL,

-- C-ifTDD: This IE shall be present if the EUTRA-Mode-Info IE in the Served Cell Information IE is set to the value “TDD” --

iE-Extensions							ProtocolExtensionContainer { {E-UTRAPRACHConfiguration-ExtIEs} }	OPTIONAL,

...

}

E-UTRAPRACHConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRATransmissionBandwidth ::= ENUMERATED {bw6, bw15, bw25, bw50, bw75, bw100, ..., bw1}

EventType ::= ENUMERATED {

report-upon-change-of-serving-cell,

report-UE-moving-presence-into-or-out-of-the-Area-of-Interest,

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod						ExpectedActivityPeriod						OPTIONAL,

expectedIdlePeriod							ExpectedIdlePeriod							OPTIONAL,

sourceOfUEActivityBehaviourInformation		SourceOfUEActivityBehaviourInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEBehaviour	::= SEQUENCE {

expectedUEActivityBehaviour		ExpectedUEActivityBehaviour 					OPTIONAL,

expectedHOInterval				ExpectedHOInterval		 						OPTIONAL,

expectedUEMobility				ExpectedUEMobility								OPTIONAL,

expectedUEMovingTrajectory		ExpectedUEMovingTrajectory						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEMobility ::= ENUMERATED {

stationary,

mobile,

...

}

ExpectedUEMovingTrajectory ::= SEQUENCE (SIZE(1..maxnoofCellsUEMovingTrajectory)) OF ExpectedUEMovingTrajectoryItem

ExpectedUEMovingTrajectoryItem ::= SEQUENCE {

nGRAN-CGI				GlobalNG-RANCell-ID,

timeStayedInCell		INTEGER (0..4095)													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEMovingTrajectoryItem-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEMovingTrajectoryItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

#### -- F

FiveQI ::= INTEGER (0..255, ...)

#### -- G

GBRQoSFlowInfo ::= SEQUENCE {

maxFlowBitRateDL			BitRate,

maxFlowBitRateUL			BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			ENUMERATED {notification-requested, ...}				OPTIONAL,

maxPacketLossRateDL			PacketLossRate											OPTIONAL,

maxPacketLossRateUL			PacketLossRate											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} }	OPTIONAL,

...

}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalgNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

gnb-id			GNB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobalgNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalgNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID-Choice ::= CHOICE {

gnb-ID					BIT STRING (SIZE(22..32)),

choice-extension		ProtocolIE-Single-Container { {GNB-ID-Choice-ExtIEs} }

}

GNB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GlobalngeNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

enb-id			ENB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobaleNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobaleNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID-Choice ::= CHOICE {

enb-ID-macro			BIT STRING (SIZE(20)),

enb-ID-shortmacro		BIT STRING (SIZE(18)),

enb-ID-longmacro		BIT STRING (SIZE(21)),

choice-extension		ProtocolIE-Single-Container { {ENB-ID-Choice-ExtIEs} }

}

ENB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GlobalNG-RANCell-ID	::= SEQUENCE {

plmn-id					PLMN-Identity,

ng-RAN-Cell-id			NG-RAN-Cell-Identity,

iE-Extensions		ProtocolExtensionContainer { {GlobalNG-RANCell-ID-ExtIEs} } OPTIONAL,

...

}

GlobalNG-RANCell-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalNG-RANNode-ID ::= CHOICE {

gNB						GlobalgNB-ID,

ng-eNB					GlobalngeNB-ID,

choice-extension		ProtocolIE-Single-Container { {GlobalNG-RANNode-ID-ExtIEs} }

}

GlobalNG-RANNode-ID-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GTP-TEID	::= OCTET STRING (SIZE(4))

GTPtunnelTransportLayerInformation ::= SEQUENCE {

tnl-address			TransportLayerAddress,

gtp-teid			GTP-TEID,

iE-Extensions		ProtocolExtensionContainer { {GTPtunnelTransportLayerInformation-ExtIEs} } OPTIONAL,

...

}

GTPtunnelTransportLayerInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMI ::= SEQUENCE {

plmn-ID				PLMN-Identity,

amf-region-id		BIT STRING (SIZE (8)),

amf-set-id			BIT STRING (SIZE (10)),

amf-pointer			BIT STRING (SIZE (6)),

iE-Extensions		ProtocolExtensionContainer { {GUAMI-ExtIEs} } OPTIONAL,

...

}

GUAMI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- H

#### -- I

InterfaceInstanceIndication ::= INTEGER (0..255, ...)

I-RNTI ::= CHOICE {

i-RNTI-full			BIT STRING (SIZE(40)),

i-RNTI-short		BIT STRING (SIZE(24)),

choice-extension	ProtocolIE-Single-Container { {I-RNTI-ExtIEs} }

}

I-RNTI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

#### -- J

#### -- K

#### -- L

LastVisitedCell-Item ::= CHOICE {

nG-RAN-Cell						LastVisitedNGRANCellInformation,

e-UTRAN-Cell					LastVisitedEUTRANCellInformation,

uTRAN-Cell						LastVisitedUTRANCellInformation,

gERAN-Cell						LastVisitedGERANCellInformation,

choice-extension				ProtocolIE-Single-Container { { LastVisitedCell-Item-ExtIEs} }

}

LastVisitedCell-Item-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation	::= OCTET STRING

LastVisitedNGRANCellInformation	::= OCTET STRING

LastVisitedUTRANCellInformation	::= OCTET STRING

LCID ::= INTEGER (1..32, ...)

ListOfCells ::= SEQUENCE (SIZE(1..maxnoofCellsinAoI)) OF CellsinAoI-Item

CellsinAoI-Item ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

ng-ran-cell-id			NG-RAN-Cell-Identity,

iE-Extensions			ProtocolExtensionContainer { {CellsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

CellsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfRANNodesinAoI ::= SEQUENCE (SIZE(1.. maxnoofRANNodesinAoI)) OF GlobalNG-RANNodesinAoI-Item

GlobalNG-RANNodesinAoI-Item ::= SEQUENCE {

global-NG-RAN-Node-ID		GlobalNG-RANNode-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalNG-RANNodesinAoI-Item-ExtIEs} } OPTIONAL,

...

}

GlobalNG-RANNodesinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfTAIsinAoI ::= SEQUENCE (SIZE(1..maxnoofTAIsinAoI)) OF TAIsinAoI-Item

TAIsinAoI-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAIsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

TAIsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LocationInformationSNReporting ::= ENUMERATED {

pSCell,

...

}

LocationReportingInformation ::= SEQUENCE {

eventType			EventType,

reportArea			ReportArea,

areaOfInterest		AreaOfInterestInformation			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...

}

#### -- M

MAC-I ::= BIT STRING (SIZE(16))

MaskedIMEISV	::= BIT STRING (SIZE(64))

MaximumDataBurstVolume ::= INTEGER (0..4095, ...)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate-UL			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaxIPrate-DL	CRITICALITY ignore	EXTENSION MaxIPrate	PRESENCE optional},

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MBSFNControlRegionLength ::= INTEGER (0..3)

MBSFNSubframeAllocation-E-UTRA ::= CHOICE {

oneframe				BIT STRING (SIZE(6)),

fourframes				BIT STRING (SIZE(24)),

choice-extension		ProtocolIE-Single-Container { {MBSFNSubframeAllocation-E-UTRA-ExtIEs} }

}

MBSFNSubframeAllocation-E-UTRA-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

MBSFNSubframeInfo-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofMBSFNEUTRA)) OF MBSFNSubframeInfo-E-UTRA-Item

MBSFNSubframeInfo-E-UTRA-Item ::= SEQUENCE {

radioframeAllocationPeriod		ENUMERATED{n1,n2,n4,n8,n16,n32,...},

radioframeAllocationOffset		INTEGER (0..7, ...),

subframeAllocation				MBSFNSubframeAllocation-E-UTRA,

iE-Extensions					ProtocolExtensionContainer { {MBSFNSubframeInfo-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

MBSFNSubframeInfo-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

MobilityRestrictionList ::= SEQUENCE {

serving-PLMN						PLMN-Identity,

equivalent-PLMNs					SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity			OPTIONAL,

rat-Restrictions					RAT-RestrictionsList										OPTIONAL,

forbiddenAreaInformation			ForbiddenAreaList											OPTIONAL,

serviceAreaInformation				ServiceAreaList												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MobilityRestrictionList-ExtIEs} }				OPTIONAL,

...

}

MobilityRestrictionList-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-LastE-UTRANPLMNIdentity					CRITICALITY ignore	EXTENSION PLMN-Identity									PRESENCE optional		},

...

}

RAT-RestrictionsList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF RAT-RestrictionsItem

RAT-RestrictionsItem ::= SEQUENCE {

plmn-Identity					PLMN-Identity,

rat-RestrictionInformation		RAT-RestrictionInformation,

iE-Extensions		ProtocolExtensionContainer { {RAT-RestrictionsItem-ExtIEs} } OPTIONAL,

...

}

RAT-RestrictionsItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

RAT-RestrictionInformation ::= BIT STRING {e-UTRA (0),nR (1)} (SIZE(8, ...))

ForbiddenAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ForbiddenAreaItem

ForbiddenAreaItem ::= SEQUENCE {

plmn-Identity		PLMN-Identity,

forbidden-TACs		SEQUENCE (SIZE(1..maxnoofForbiddenTACs)) OF TAC,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenAreaItem-ExtIEs} } OPTIONAL,

...

}

ForbiddenAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

ServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ServiceAreaItem

ServiceAreaItem ::= SEQUENCE {

plmn-Identity						PLMN-Identity,

allowed-TACs-ServiceArea			SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC		OPTIONAL,

not-allowed-TACs-ServiceArea		SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServiceAreaItem-ExtIEs} }				OPTIONAL,

...

}

ServiceAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

MR-DC-ResourceCoordinationInfo ::= SEQUENCE {

ng-RAN-Node-ResourceCoordinationInfo			NG-RAN-Node-ResourceCoordinationInfo,

iE-Extension											ProtocolExtensionContainer { {MR-DC-ResourceCoordinationInfo-ExtIEs}}	OPTIONAL,

...

}

MR-DC-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-Node-ResourceCoordinationInfo ::= CHOICE {

eutra-resource-coordination-info					E-UTRA-ResourceCoordinationInfo,

nr-resource-coordination-info						NR-ResourceCoordinationInfo

}

E-UTRA-ResourceCoordinationInfo ::= SEQUENCE {

e-utra-cell												E-UTRA-CGI,

ul-coordination-info									BIT STRING (SIZE (6..4400)),

dl-coordination-info									BIT STRING (SIZE (6..4400))	OPTIONAL,

nr-cell													NR-CGI	OPTIONAL,

e-utra-coordination-assistance-info				E-UTRA-CoordinationAssistanceInfo	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {E-UTRA-ResourceCoordinationInfo-ExtIEs} } 	OPTIONAL,

...

}

E-UTRA-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRA-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

NR-ResourceCoordinationInfo ::= SEQUENCE {

nr-cell													NR-CGI,

ul-coordination-info									BIT STRING (SIZE (6..4400)),

dl-coordination-info									BIT STRING (SIZE (6..4400))	OPTIONAL,

e-utra-cell												E-UTRA-CGI	OPTIONAL,

nr-coordination-assistance-info					NR-CoordinationAssistanceInfo		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {NR-ResourceCoordinationInfo-ExtIEs} } 	OPTIONAL,

...

}

NR-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

#### -- N

NE-DC-TDM-Pattern ::= SEQUENCE {

subframeAssignment			ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6},

harqOffset					INTEGER (0..9),

iE-Extension				ProtocolExtensionContainer { {NE-DC-TDM-Pattern-ExtIEs}}	OPTIONAL,

...

}

NE-DC-TDM-Pattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourInformation-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-E-UTRA-Item

NeighbourInformation-E-UTRA-Item ::= SEQUENCE {

e-utra-PCI			E-UTRAPCI,

e-utra-cgi			E-UTRA-CGI,

earfcn				E-UTRAARFCN,

tac					TAC,

ranac				RANAC																		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NeighbourInformation-E-UTRA-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

NeighbourInformation-NR ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-NR-Item

NeighbourInformation-NR-Item ::= SEQUENCE {

nr-PCI								NRPCI,

nr-cgi								NR-CGI,

tac									TAC,

ranac								RANAC														OPTIONAL,

nr-mode-info						NeighbourInformation-NR-ModeInfo,

connectivitySupport					Connectivity-Support,

measurementTimingConfiguration		OCTET STRING,

iE-Extensions			ProtocolExtensionContainer { {NeighbourInformation-NR-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

NeighbourInformation-NR-ModeInfo ::= CHOICE {

fdd-info				NeighbourInformation-NR-ModeFDDInfo,

tdd-info				NeighbourInformation-NR-ModeTDDInfo,

choice-extension		ProtocolIE-Single-Container { {NeighbourInformation-NR-ModeInfo-ExtIEs} }

}

NeighbourInformation-NR-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NeighbourInformation-NR-ModeFDDInfo ::= SEQUENCE {

ul-NR-FreqInfo		NRFrequencyInfo,

dl-NR-FequInfo		NRFrequencyInfo,

ie-Extensions		ProtocolExtensionContainer { {NeighbourInformation-NR-ModeFDDInfo-ExtIEs} } OPTIONAL,

...

}

NeighbourInformation-NR-ModeFDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourInformation-NR-ModeTDDInfo ::= SEQUENCE {

nr-FreqInfo			NRFrequencyInfo,

ie-Extensions		ProtocolExtensionContainer { {NeighbourInformation-NR-ModeTDDInfo-ExtIEs} } OPTIONAL,

...

}

NeighbourInformation-NR-ModeTDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-Cell-Identity ::= CHOICE {

nr						NR-Cell-Identity,

e-utra					E-UTRA-Cell-Identity,

choice-extension		ProtocolIE-Single-Container { {NG-RAN-Cell-Identity-ExtIEs} }

}

NG-RAN-Cell-Identity-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NG-RAN-CellPCI ::= CHOICE {

nr					NRPCI,

e-utra				E-UTRAPCI,

choice-extension	ProtocolIE-Single-Container { {NG-RAN-CellPCI-ExtIEs} }

}

NG-RAN-CellPCI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NG-RANnodeUEXnAPID ::= INTEGER (0.. 4294967295)

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI,

priorityLevelQoS			PriorityLevelQoS													OPTIONAL,

averagingWindow				AveragingWindow														OPTIONAL,

maximumDataBurstVolume		MaximumDataBurstVolume 												OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

NonDynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRARFCN	::= INTEGER (0.. maxNRARFCN)

NR-Cell-Identity		::= BIT STRING (SIZE (36))

NG-RAN-Cell-Identity-ListinRANPagingArea ::= SEQUENCE (SIZE (1..maxnoofCellsinRNA)) OF NG-RAN-Cell-Identity

NR-CGI ::= SEQUENCE {

plmn-id				PLMN-Identity,

nr-CI				NR-Cell-Identity,

iE-Extension		ProtocolExtensionContainer { {NR-CGI-ExtIEs} } 	OPTIONAL,

...

}

NR-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRFrequencyBand ::= INTEGER (1..1024, ...)

NRFrequencyBand-List ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF NRFrequencyBandItem

NRFrequencyBandItem ::= SEQUENCE {

nr-frequency-band			NRFrequencyBand,

supported-SUL-Band-List		SupportedSULBandList											OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NRFrequencyBandItem-ExtIEs} } 	OPTIONAL,

...

}

NRFrequencyBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRFrequencyInfo ::= SEQUENCE {

nrARFCN				NRARFCN,

sul-information		SUL-Information												OPTIONAL,

frequencyBand-List		NRFrequencyBand-List,

iE-Extension		ProtocolExtensionContainer { {NRFrequencyInfo-ExtIEs} }		OPTIONAL,

...

}

NRFrequencyInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRModeInfo ::= CHOICE {

fdd							NRModeInfoFDD,

tdd							NRModeInfoTDD,

choice-extension			ProtocolIE-Single-Container { {NRModeInfo-ExtIEs} }

}

NRModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NRModeInfoFDD ::= SEQUENCE {

ulNRFrequencyInfo			NRFrequencyInfo,

dlNRFrequencyInfo			NRFrequencyInfo,

ulNRTransmissonBandwidth	NRTransmissionBandwidth,

dlNRTransmissonBandwidth	NRTransmissionBandwidth,

iE-Extension		ProtocolExtensionContainer { {NRModeInfoFDD-ExtIEs} } 	OPTIONAL,

...

}

NRModeInfoFDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRModeInfoTDD ::= SEQUENCE {

nrFrequencyInfo			NRFrequencyInfo,

nrTransmissonBandwidth	NRTransmissionBandwidth,

iE-Extension			ProtocolExtensionContainer { {NRModeInfoTDD-ExtIEs} } 	OPTIONAL,

...

}

NRModeInfoTDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ...}

NRPCI ::= INTEGER (0..1007, ...)

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ...}

NRTransmissionBandwidth	::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { {NRTransmissionBandwidth-ExtIEs} } OPTIONAL,

...

}

NRTransmissionBandwidth-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NumberOfAntennaPorts-E-UTRA ::= ENUMERATED {an1, an2, an4, ...}

#### -- O

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PacketLossRate ::= INTEGER (0..1000, ...)

PagingDRX	::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PDCPChangeIndication ::= CHOICE {

from-S-NG-RAN-node				ENUMERATED {s-ng-ran-node-key-update-required, pdcp-data-recovery-required, ...},

from-M-NG-RAN-node				ENUMERATED {pdcp-data-recovery-required, ...},

choice-extension				ProtocolIE-Single-Container { {PDCPChangeIndication-ExtIEs} }

}

PDCPChangeIndication-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

PDCPDuplicationConfiguration ::= ENUMERATED {

configured,

de-configured,

...

}

PDCPSNLength ::= SEQUENCE {

ulPDCPSNLength			ENUMERATED {v12bits, v18bits, ...},

dlPDCPSNLength			ENUMERATED {v12bits, v18bits, ...},

iE-Extension			ProtocolExtensionContainer { {PDCPSNLength-ExtIEs} }		OPTIONAL,

...

}

PDCPSNLength-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAggregateMaximumBitRate ::= SEQUENCE {

downlink-session-AMBR				BitRate,

uplink-session-AMBR					BitRate,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionAggregateMaximumBitRate-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-ID

PDUSession-List-withCause ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-List-withCause-Item

PDUSession-List-withCause-Item ::= SEQUENCE {

pduSessionId		PDUSession-ID,

cause				Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List-withDataForwardingFromTarget ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingFromTarget-Item

PDUSession-List-withDataForwardingFromTarget-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

dataforwardinginfoTarget			DataForwardingInfoFromTargetNGRANnode,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List-withDataForwardingRequest ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingRequest-Item

PDUSession-List-withDataForwardingRequest-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

dataforwardingInfofromSource			DataforwardingandOffloadingInfofromSource					OPTIONAL,

dRBtoBeReleasedList						DRBToQoSFlowMapping-List									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

##### -- PDU Session related message level IEs BEGIN

--

-- **************************************************************

-- **************************************************************

--

###### -- PDU Session Resources Admitted List

--

-- **************************************************************

PDUSessionResourcesAdmitted-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesAdmitted-Item

PDUSessionResourcesAdmitted-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

pduSessionResourceAdmittedInfo		PDUSessionResourceAdmittedInfo,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesAdmitted-Item-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourcesAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceAdmittedInfo ::= SEQUENCE {

dL-NG-U-TNL-Information-Unchanged		ENUMERATED {true, ...}														OPTIONAL,

qosFlowsAdmitted-List					QoSFlowsAdmitted-List,

qosFlowsNotAdmitted-List				QoSFlows-List-withCause														OPTIONAL,

dataForwardingInfoFromTarget			DataForwardingInfoFromTargetNGRANnode										OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourceAdmittedInfo-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceAdmittedInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondarydataForwardingInfoFromTarget-List	CRITICALITY ignore	EXTENSION SecondarydataForwardingInfoFromTarget-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resources Not Admitted List

--

-- **************************************************************

PDUSessionResourcesNotAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofPDUSessions)) OF PDUSessionResourcesNotAdmitted-Item

PDUSessionResourcesNotAdmitted-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

cause						Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resources To Be Setup List

--

-- **************************************************************

PDUSessionResourcesToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesToBeSetup-Item

PDUSessionResourcesToBeSetup-Item ::= SEQUENCE {

pduSessionId					PDUSession-ID,

s-NSSAI							S-NSSAI,

pduSessionAMBR					PDUSessionAggregateMaximumBitRate													OPTIONAL,

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

source-DL-NG-U-TNL-Information  UPTransportLayerInformation															OPTIONAL,

securityIndication				SecurityIndication																	OPTIONAL,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance															OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourcesToBeSetup-Item-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourcesToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-UL-NG-U-TNLatUPF-List	CRITICALITY ignore	EXTENSION Additional-UL-NG-U-TNLatUPF-List 	PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance																	OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource													OPTIONAL,

securityIndication				SecurityIndication																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-SNterminated-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecurityResult						CRITICALITY reject	EXTENSION SecurityResult							PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Setup-SNterminated-Item

QoSFlowsToBeSetup-List-Setup-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated 	OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode			OPTIONAL,

qosFlowsNotAdmittedList			QoSFlows-List-withCause							OPTIONAL,

securityResult					SecurityResult									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-SetupResponse-SNterminated-Item

DRBsToBeSetupList-SetupResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

rLC-Mode												RLCMode,

uL-Configuration									ULConfiguration						OPTIONAL,

secondary-SN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo		GBRQoSFlowInfo														OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-Setup-MNterminated-Item

DRBsToBeSetupList-Setup-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

rLC-Mode												RLCMode,

uL-Configuration										ULConfiguration						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated		QoSFlowsMappedtoDRB-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-Setup-MNterminated-Item

QoSFlowsMappedtoDRB-Setup-MNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

qosFlowMappingIndication		QoSFlowMappingIndication		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList				DRBsAdmittedList-SetupResponse-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-DRBsNotAdmittedSetupModifyList	CRITICALITY ignore	EXTENSION DRB-List-withCause		PRESENCE optional},

...

}

DRBsAdmittedList-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-SetupResponse-MNterminated-Item

DRBsAdmittedList-SetupResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation						OPTIONAL,

pduSessionNetworkInstance		PDUSessionNetworkInstance						OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated		OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource		OPTIONAL,

qosFlowsToBeModified-List		QoSFlowsToBeSetup-List-Modified-SNterminated	OPTIONAL,

qoSFlowsToBeReleased-List		QoSFlows-List-withCause							OPTIONAL,

drbsToBeModifiedList			DRBsToBeModified-List-Modified-SNterminated		OPTIONAL,

dRBsToBeReleased				DRB-List-withCause								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Modified-SNterminated-Item

QoSFlowsToBeSetup-List-Modified-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters							OPTIONAL,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo										OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication 							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModified-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-Modified-SNterminated-Item

DRBsToBeModified-List-Modified-SNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

mN-DL-SCG-UP-TNLInfo					UPTransportParameters		OPTIONAL,

secondary-MN-DL-SCG-UP-TNLInfo			UPTransportParameters		OPTIONAL,

lCID									LCID						OPTIONAL,

rlc-status								RLC-Status					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation								OPTIONAL,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated			OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode					OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-ModificationResponse-SNterminated	OPTIONAL,

dRBsToBeReleased				DRB-List-withCause										OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource				OPTIONAL,

qosFlowsNotAdmittedTBAdded		QoSFlows-List-withCause									OPTIONAL,

qosFlowsReleased				QoSFlows-List-withCause									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters									OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters								OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated						OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-Modification-MNterminated				OPTIONAL,

dRBsToBeReleased				DRB-List-withCause											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModifiedList-Modification-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-Modification-MNterminated-Item

DRBsToBeModifiedList-Modification-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters					OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters						OPTIONAL,

uL-Configuration										ULConfiguration								OPTIONAL,

pdcpDuplicationConfiguration							PDCPDuplicationConfiguration 				OPTIONAL,

duplicationActivation									DuplicationActivation						OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated					QoSFlowsMappedtoDRB-Setup-MNterminated	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList						DRBsAdmittedList-ModificationResponse-MNterminated,

dRBsReleasedList						DRB-List																		OPTIONAL,

dRBsNotAdmittedSetupModifyList			DRB-List-withCause																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsAdmittedList-ModificationResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-ModificationResponse-MNterminated-Item

DRBsAdmittedList-ModificationResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters				OPTIONAL,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Required Info - SN terminated

--

-- **************************************************************

PDUSessionResourceChangeRequiredInfo-SNterminated ::= SEQUENCE {

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Confirm Info - SN terminated

--

-- **************************************************************

PDUSessionResourceChangeConfirmInfo-SNterminated ::= SEQUENCE {

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Required Info - MN terminated

--

-- **************************************************************

PDUSessionResourceChangeRequiredInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Confirm Info - MN terminated

--

-- **************************************************************

PDUSessionResourceChangeConfirmInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Required Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModRqdInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation						OPTIONAL,

qoSFlowsToBeReleased-List		QoSFlows-List-withCause							OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource		OPTIONAL,

drbsToBeSetupList				DRBsToBeSetup-List-ModRqd-SNterminated			OPTIONAL,

drbsToBeModifiedList			DRBsToBeModified-List-ModRqd-SNterminated		OPTIONAL,

dRBsToBeReleased				DRB-List-withCause								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModRqdInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetup-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetup-List-ModRqd-SNterminated-Item

DRBsToBeSetup-List-ModRqd-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

pDCP-SNLength									PDCPSNLength											OPTIONAL,

sn-UL-PDCP-UPTNLinfo							UPTransportParameters,

dRB-QoS											QoSFlowLevelQoSParameters,

secondary-SN-UL-PDCP-UP-TNLInfo					UPTransportParameters								OPTIONAL,

duplicationActivation							DuplicationActivation								OPTIONAL,

uL-Configuration								ULConfiguration										OPTIONAL,

qoSFlowsMappedtoDRB-ModRqd-SNterminated			QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated,

rLC-Mode										RLCMode,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo		GBRQoSFlowInfo														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModified-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-SNterminated-Item

DRBsToBeModified-List-ModRqd-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

sN-UL-PDCP-UP-TNLInfo							UPTransportParameters									OPTIONAL,

dRB-QoS											QoSFlowLevelQoSParameters								OPTIONAL,

secondary-SN-UL-PDCP-UP-TNLInfo					UPTransportParameters									OPTIONAL,

uL-Configuration								ULConfiguration											OPTIONAL,

pdcpDuplicationConfiguration					PDCPDuplicationConfiguration							OPTIONAL,

duplicationActivation							DuplicationActivation									OPTIONAL,

qoSFlowsMappedtoDRB-ModRqd-SNterminated		QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier					QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo			GBRQoSFlowInfo														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Confirm Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModConfirmInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF						UPTransportLayerInformation								OPTIONAL,

dRBsAdmittedList						DRBsAdmittedList-ModConfirm-SNterminated,

dRBsNotAdmittedSetupModifyList			DRB-List-withCause										OPTIONAL,

dataforwardinginfoTarget				DataForwardingInfoFromTargetNGRANnode					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsAdmittedList-ModConfirm-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsAdmittedList-ModConfirm-SNterminated-Item

DRBsAdmittedList-ModConfirm-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

mN-DL-CG-UP-TNLInfo								UPTransportParameters							OPTIONAL,

secondary-MN-DL-CG-UP-TNLInfo					UPTransportParameters							OPTIONAL,

lCID											LCID											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Required Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModRqdInfo-MNterminated ::= SEQUENCE {

dRBsToBeModified				DRBsToBeModified-List-ModRqd-MNterminated				OPTIONAL,

dRBsToBeReleased				DRB-List-withCause											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModRqdInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModified-List-ModRqd-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-MNterminated-Item

DRBsToBeModified-List-ModRqd-MNterminated-Item ::= SEQUENCE {

drb-ID								DRB-ID,

sN-DL-SCG-UP-TNLInfo				UPTransportLayerInformation,

secondary-SN-DL-SCG-UP-TNLInfo		UPTransportLayerInformation		OPTIONAL,

lCID								LCID							OPTIONAL,

rlc-status							RLC-Status						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs} } OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Confirm Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModConfirmInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- PDU Session Resource Setup Complete Info - SN terminated

--

-- **************************************************************

PDUSessionResourceBearerSetupCompleteInfo-SNterminated ::= SEQUENCE {

dRBsToBeSetupList 			SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item,

iE-Extensions				ProtocolExtensionContainer { {PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item ::= SEQUENCE {

dRB-ID						DRB-ID,

mN-Xn-U-TNLInfoatM			UPTransportLayerInformation,

iE-Extensions				ProtocolExtensionContainer { {DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-Secondary-MN-Xn-U-TNLInfoatM	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PDU Session related message level IEs END

--

-- **************************************************************

PDUSessionResourceSecondaryRATUsageList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSecondaryRATUsageItem

PDUSessionResourceSecondaryRATUsageItem ::= SEQUENCE {

pDUSessionID								PDUSession-ID,

secondaryRATUsageInformation						SecondaryRATUsageInformation,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSecondaryRATUsageItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSecondaryRATUsageItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionUsageReport ::= SEQUENCE {

rATType								ENUMERATED {nr, eutra, ...},

pDUSessionTimedReportList			VolumeTimedReportList,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionUsageReport-ExtIEs} } OPTIONAL,

...

}

PDUSessionUsageReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionType ::= ENUMERATED {ipv4, ipv6, ipv4v6, ethernet, unstructured, ...}

PDUSession-ID	::= INTEGER (0..255)

PDUSessionNetworkInstance	::= INTEGER (1..256, ...)

PDUSessionCommonNetworkInstance	::= OCTET STRING

PLMN-Identity ::= OCTET STRING (SIZE(3))

PriorityLevelQoS ::= INTEGER (1..127, ...)

ProtectedE-UTRAResourceIndication ::= SEQUENCE {

activationSFN					ActivationSFN,

protectedResourceList			ProtectedE-UTRAResourceList,

mbsfnControlRegionLength		MBSFNControlRegionLength					OPTIONAL,

pDCCHRegionLength				INTEGER (1..3),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResourceIndication-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedE-UTRAResourceList ::= SEQUENCE (SIZE (1.. maxnoofProtectedResourcePatterns)) OF ProtectedE-UTRAResource-Item

ProtectedE-UTRAResource-Item ::= SEQUENCE {

resourceType							ENUMERATED {downlinknonCRS, cRS, uplink, ...},

intra-PRBProtectedResourceFootprint		BIT STRING (SIZE(84, ...)),

protectedFootprintFrequencyPattern		BIT STRING (SIZE(6..110, ...)),

protectedFootprintTimePattern			ProtectedE-UTRAFootprintTimePattern,

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResource-Item-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAResource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedE-UTRAFootprintTimePattern ::= SEQUENCE {

protectedFootprintTimeperiodicity			INTEGER (1..320, ...),

protectedFootrpintStartTime					INTEGER (1..20, ...),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAFootprintTimePattern-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAFootprintTimePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- Q

QoSCharacteristics ::= CHOICE {

non-dynamic						NonDynamic5QIDescriptor,

dynamic							Dynamic5QIDescriptor,

choice-extension				ProtocolIE-Single-Container { {QoSCharacteristics-ExtIEs} }

}

QoSCharacteristics-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

QoSFlowIdentifier	::= INTEGER (0..63, ...)

QoSFlowLevelQoSParameters ::= SEQUENCE {

qos-characteristics			QoSCharacteristics,

allocationAndRetentionPrio	AllocationandRetentionPriority,

gBRQoSFlowInfo				GBRQoSFlowInfo														OPTIONAL,

relectiveQoS				ReflectiveQoSAttribute												OPTIONAL,

additionalQoSflowInfo		ENUMERATED {more-likely, ...}										OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {QoSFlowLevelQoSParameters-ExtIEs} } OPTIONAL,

...

}

QoSFlowLevelQoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowMappingIndication ::= ENUMERATED {

ul,

dl,

...

}

QoSFlowNotificationControlIndicationInfo ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowNotify-Item

QoSFlowNotify-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

notificationInformation		ENUMERATED {fulfilled, not-fulfilled, ...},

iE-Extensions				ProtocolExtensionContainer { {QoSFlowNotificationControlIndicationInfo-ExtIEs} } OPTIONAL,

...

}

QoSFlowNotificationControlIndicationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlows-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlow-Item

QoSFlow-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowMappingIndication		QoSFlowMappingIndication					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlow-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlow-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlows-List-withCause ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowwithCause-Item

QoSFlowwithCause-Item ::= SEQUENCE {

qfi					QoSFlowIdentifier,

cause				Cause					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlowwithCause-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowwithCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsAdmitted-Item

QoSFlowsAdmitted-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

iE-Extension		ProtocolExtensionContainer { {QoSFlowsAdmitted-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsToBeSetup-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-Item

QoSFlowsToBeSetup-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

e-RAB-ID						E-RAB-ID											OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlowsToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsUsageReportList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsUsageReport-Item

QoSFlowsUsageReport-Item ::= SEQUENCE {

qosFlowIdentifier					QoSFlowIdentifier,

rATType								ENUMERATED {nr, eutra, ...},

qoSFlowsTimedReportList				VolumeTimedReportList,

iE-Extensions						ProtocolExtensionContainer { {QoSFlowsUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

QoSFlowsUsageReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- R

RANAC ::= INTEGER (0..255)

RANAreaID ::= SEQUENCE {

tAC					TAC,

rANAC				RANAC												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RANAreaID-ExtIEs} } 	OPTIONAL,

...

}

RANAreaID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANAreaID-List ::= SEQUENCE (SIZE(1..maxnoofRANAreasinRNA)) OF RANAreaID

RANPagingArea ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

rANPagingAreaChoice		RANPagingAreaChoice,

iE-Extensions			ProtocolExtensionContainer { {RANPagingArea-ExtIEs} } OPTIONAL,

...

}

RANPagingArea-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingAreaChoice ::= CHOICE {

cell-List			NG-RAN-Cell-Identity-ListinRANPagingArea,

rANAreaID-List		RANAreaID-List,

choice-extension	ProtocolIE-Single-Container { {RANPagingAreaChoice-ExtIEs} }

}

RANPagingAreaChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RANPagingAttemptInfo ::= SEQUENCE {

pagingAttemptCount					INTEGER (1..16, ...),

intendedNumberOfPagingAttempts		INTEGER (1..16, ...),

nextPagingAreaScope					ENUMERATED {same, changed, ...}	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {RANPagingAttemptInfo-ExtIEs} } OPTIONAL,

...

}

RANPagingAttemptInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingFailure		::= 	ENUMERATED {

true,

...

}

ReferenceID ::= INTEGER (1..64, ...) -- This IE may need to be refined.

ReflectiveQoSAttribute ::= ENUMERATED {subject-to-reflective-QoS, ...}

ReportArea ::= ENUMERATED {

cell,

...

}

RequestReferenceID ::= INTEGER (1..64, ...)

ReservedSubframePattern ::= SEQUENCE {

subframeType					ENUMERATED {mbsfn, non-mbsfn, ...},

reservedSubframePattern			BIT STRING (SIZE(10..160)),

mbsfnControlRegionLength		MBSFNControlRegionLength					OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {ReservedSubframePattern-ExtIEs} } OPTIONAL,

...

}

ReservedSubframePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestTypeInfo ::= CHOICE {

fullReset			ResetRequestTypeInfo-Full,

partialReset		ResetRequestTypeInfo-Partial,

choice-extension	ProtocolIE-Single-Container { {ResetRequestTypeInfo-ExtIEs} }

}

ResetRequestTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResetRequestTypeInfo-Full ::= SEQUENCE {

iE-Extension					ProtocolExtensionContainer { {ResetRequestTypeInfo-Full-ExtIEs} } OPTIONAL,

...

}

ResetRequestTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestTypeInfo-Partial ::= SEQUENCE {

ue-contexts-ToBeReleasedList	ResetRequestPartialReleaseList,

iE-Extension					ProtocolExtensionContainer { {ResetRequestTypeInfo-Partial-ExtIEs} } OPTIONAL,

...

}

ResetRequestTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestPartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetRequestPartialReleaseItem

ResetRequestPartialReleaseItem ::= SEQUENCE {

ng-ran-node1UEXnAPID						NG-RANnodeUEXnAPID			OPTIONAL,

ng-ran-node2UEXnAPID						NG-RANnodeUEXnAPID			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ResetRequestPartialReleaseItem-ExtIEs} } OPTIONAL,

...

}

ResetRequestPartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponseTypeInfo ::= CHOICE {

fullReset			ResetResponseTypeInfo-Full,

partialReset		ResetResponseTypeInfo-Partial,

choice-extension	ProtocolIE-Single-Container { {ResetResponseTypeInfo-ExtIEs} }

}

ResetResponseTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResetResponseTypeInfo-Full ::= SEQUENCE {

iE-Extension					ProtocolExtensionContainer { {ResetResponseTypeInfo-Full-ExtIEs} } OPTIONAL,

...

}

ResetResponseTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponseTypeInfo-Partial ::= SEQUENCE {

ue-contexts-AdmittedToBeReleasedList	ResetResponsePartialReleaseList,

iE-Extension					ProtocolExtensionContainer { {ResetResponseTypeInfo-Partial-ExtIEs} } OPTIONAL,

...

}

ResetResponseTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponsePartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetResponsePartialReleaseItem

ResetResponsePartialReleaseItem ::= SEQUENCE {

ng-ran-node1UEXnAPID						NG-RANnodeUEXnAPID		OPTIONAL,

ng-ran-node2UEXnAPID						NG-RANnodeUEXnAPID		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ResetResponsePartialReleaseItem-ExtIEs} } OPTIONAL,

...

}

ResetResponsePartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLCMode	::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication 	Reestablishment-Indication,

iE-Extensions					ProtocolExtensionContainer { {RLC-Status-ExtIEs} } OPTIONAL,

...

}

RLC-Status-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Reestablishment-Indication ::= ENUMERATED {

reestablished,

...

}

RFSP-Index ::= INTEGER (1..256)

RRCConfigIndication ::= ENUMERATED {

full-config,

delta-config,

...

}

RRCResumeCause ::= ENUMERATED {

rna-Update,

...

}

#### -- S

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {

secondarydataForwardingInfoFromTarget				DataForwardingInfoFromTargetNGRANnode,

iE-Extensions		ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecondarydataForwardingInfoFromTarget-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF SecondarydataForwardingInfoFromTarget-Item

SCGConfigurationQuery	::= ENUMERATED {true, ...}

SecondaryRATUsageInformation ::= SEQUENCE {

pDUSessionUsageReport		PDUSessionUsageReport				OPTIONAL,

qosFlowsUsageReportList		QoSFlowsUsageReportList				OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {SecondaryRATUsageInformation-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATUsageInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			ENUMERATED {required, preferred, not-needed, ...},

confidentialityProtectionIndication		ENUMERATED {required, preferred, not-needed, ...},

maximumIPdatarate						MaximumIPdatarate												OPTIONAL,

-- This IE shall be present if the Integrity Protection IE within the Security Indication IE is present and set to "required" or "preferred". --

iE-Extensions							ProtocolExtensionContainer { {SecurityIndication-ExtIEs} } OPTIONAL,

...

}

SecurityIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityResult ::= SEQUENCE {

integrityProtectionResult				ENUMERATED {performed, not-performed, ...},

confidentialityProtectionResult			ENUMERATED {performed, not-performed, ...},

iE-Extensions							ProtocolExtensionContainer { {SecurityResult-ExtIEs} } OPTIONAL,

...

}

SecurityResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

##### -- Served Cells E-UTRA IEs

ServedCellInformation-E-UTRA ::= SEQUENCE {

e-utra-pci								E-UTRAPCI,

e-utra-cgi								E-UTRA-CGI,

tac										TAC,

ranac									RANAC																					OPTIONAL,

broadcastPLMNs							SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedCellInformation-E-UTRA-perBPLMN,

e-utra-mode-info						ServedCellInformation-E-UTRA-ModeInfo,

numberofAntennaPorts					NumberOfAntennaPorts-E-UTRA																OPTIONAL,

prach-configuration						E-UTRAPRACHConfiguration																OPTIONAL,

mBSFNsubframeInfo						MBSFNSubframeInfo-E-UTRA																OPTIONAL,

multibandInfo							E-UTRAMultibandInfoList																	OPTIONAL,

freqBandIndicatorPriority				ENUMERATED {not-broadcast, broadcast, ...} 												OPTIONAL,

bandwidthReducedSI						ENUMERATED {scheduled, ...}																OPTIONAL,

protectedE-UTRAResourceIndication		ProtectedE-UTRAResourceIndication														OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-ExtIEs} }					OPTIONAL,

...

}

ServedCellInformation-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-EUTRA		CRITICALITY ignore	EXTENSION BPLMN-ID-Info-EUTRA		PRESENCE optional },

...

}

ServedCellInformation-E-UTRA-perBPLMN ::= SEQUENCE {

plmn-id					PLMN-Identity,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-perBPLMN-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-perBPLMN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellInformation-E-UTRA-ModeInfo ::= CHOICE {

fdd					ServedCellInformation-E-UTRA-FDDInfo,

tdd					ServedCellInformation-E-UTRA-TDDInfo,

choice-extension	ProtocolIE-Single-Container{ {ServedCellInformation-E-UTRA-ModeInfo-ExtIEs} }

}

ServedCellInformation-E-UTRA-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ServedCellInformation-E-UTRA-FDDInfo ::= SEQUENCE {

ul-earfcn			E-UTRAARFCN,

dl-earfcn			E-UTRAARFCN,

ul-e-utraTxBW		E-UTRATransmissionBandwidth,

dl-e-utraTxBW		E-UTRATransmissionBandwidth,

iE-Extensions		ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-FDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-FDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellInformation-E-UTRA-TDDInfo ::= SEQUENCE {

earfcn					E-UTRAARFCN,

e-utraTxBW				E-UTRATransmissionBandwidth,

subframeAssignmnet		ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6,...},

specialSubframeInfo		SpecialSubframeInfo-E-UTRA,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-TDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-TDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-E-UTRA-Item

ServedCells-E-UTRA-Item ::= SEQUENCE {

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR									OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellsToUpdate-E-UTRA ::= SEQUENCE {

served-Cells-ToAdd-E-UTRA		ServedCells-E-UTRA																OPTIONAL,

served-Cells-ToModify-E-UTRA	ServedCells-ToModify-E-UTRA														OPTIONAL,

served-Cells-ToDelete-E-UTRA	SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI 					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToUpdate-E-UTRA-ExtIEs} }	OPTIONAL,

...

}

ServedCellsToUpdate-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-ToModify-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-E-UTRA-Item

ServedCells-ToModify-E-UTRA-Item ::= SEQUENCE {

old-ECGI					E-UTRA-CGI,

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR											OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA										OPTIONAL,

deactivation-indication		ENUMERATED {deactivated, ...}									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-cells-ToModify-E-UTRA-Item-ExtIEs} }	OPTIONAL,

...

}

Served-cells-ToModify-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

##### -- Served Cells NR IEs

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-NR		CRITICALITY ignore	EXTENSION BPLMN-ID-Info-NR		PRESENCE optional },

...

}

ServedCells-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-NR-Item

ServedCells-NR-Item ::= SEQUENCE {

served-cell-info-NR			ServedCellInformation-NR,

neighbour-info-NR			NeighbourInformation-NR				OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-NR-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-ToModify-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-NR-Item

ServedCells-ToModify-NR-Item ::= SEQUENCE {

old-NR-CGI					NR-CGI,

served-cell-info-NR			ServedCellInformation-NR,

neighbour-info-NR			NeighbourInformation-NR				 							OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA										OPTIONAL,

deactivation-indication		ENUMERATED {deactivated, ...}									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-cells-ToModify-NR-Item-ExtIEs} } 	OPTIONAL,

...

}

Served-cells-ToModify-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellsToUpdate-NR ::= SEQUENCE {

served-Cells-ToAdd-NR		ServedCells-NR															OPTIONAL,

served-Cells-ToModify-NR	ServedCells-ToModify-NR													OPTIONAL,

served-Cells-ToDelete-NR	SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF NR-CGI 				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCellsToUpdate-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellsToUpdate-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType ::= CHOICE {

ul-onlySharing				SharedResourceType-UL-OnlySharing,

ul-and-dl-Sharing			SharedResourceType-ULDL-Sharing,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ExtIEs} }

}

SharedResourceType-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-UL-OnlySharing ::= SEQUENCE {

ul-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-UL-OnlySharing-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-UL-OnlySharing-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType-ULDL-Sharing ::= CHOICE {

ul-resources				SharedResourceType-ULDL-Sharing-UL-Resources,

dl-resources				SharedResourceType-ULDL-Sharing-DL-Resources,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-UL-Resources ::= CHOICE {

unchanged					NULL,

changed						SharedResourceType-ULDL-Sharing-UL-ResourcesChanged,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged ::= SEQUENCE {

ul-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType-ULDL-Sharing-DL-Resources ::= CHOICE {

unchanged					NULL,

changed						SharedResourceType-ULDL-Sharing-DL-ResourcesChanged,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged ::= SEQUENCE {

dl-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupport-List	::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAI

S-NG-RANnode-SecurityKey ::= BIT STRING (SIZE(256))

S-NG-RANnode-Addition-Trigger-Ind ::= ENUMERATED {

sn-change,

inter-MN-HO,

intra-MN-HO,

...

}

S-NSSAI ::= SEQUENCE {

sst						OCTET STRING (SIZE(1)),

sd						OCTET STRING (SIZE(3))							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {S-NSSAI-ExtIEs} } OPTIONAL,

...

}

S-NSSAI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SpecialSubframeInfo-E-UTRA ::= SEQUENCE {

specialSubframePattern	SpecialSubframePatterns-E-UTRA,

cyclicPrefixDL			CyclicPrefix-E-UTRA-DL,

cyclicPrefixUL			CyclicPrefix-E-UTRA-UL,

iE-Extensions			ProtocolExtensionContainer { {SpecialSubframeInfo-E-UTRA-ExtIEs} } OPTIONAL,

...

}

SpecialSubframeInfo-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SpecialSubframePatterns-E-UTRA ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

ssp10,

...

}

SpectrumSharingGroupID ::= INTEGER (1..maxnoofCellsinNG-RANnode)

SplitSRBsTypes ::= ENUMERATED {srb1, srb2, srb1and2, ...}

SUL-FrequencyBand ::= INTEGER (1..1024)

SUL-Information ::= SEQUENCE {

sulFrequencyInfo			NRARFCN,

sulTransmissionBandwidth	NRTransmissionBandwidth,

iE-Extensions				ProtocolExtensionContainer { {SUL-Information-ExtIEs} } OPTIONAL,

...

}

SUL-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SupportedSULBandList ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF SupportedSULBandItem

SupportedSULBandItem ::= SEQUENCE {

sulBandItem					SUL-FrequencyBand,

iE-Extensions				ProtocolExtensionContainer { {SupportedSULBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TAC ::= OCTET STRING (SIZE (3))

TAISupport-List	::= SEQUENCE (SIZE(1..maxnoofsupportedTACs)) OF TAISupport-Item

TAISupport-Item ::= SEQUENCE {

tac								TAC,

broadcastPLMNs					SEQUENCE (SIZE(1..maxnoofsupportedPLMNs)) OF BroadcastPLMNinTAISupport-Item,

iE-Extensions					ProtocolExtensionContainer { {TAISupport-Item-ExtIEs} } OPTIONAL,

...

}

TAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Target-CGI ::= CHOICE {

nr							NR-CGI,

e-utra						E-UTRA-CGI,

choice-extension			ProtocolIE-Single-Container { {TargetCGI-ExtIEs} }

}

TargetCGI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

TimeToWait ::= ENUMERATED {

v1s,

v2s,

v5s,

v10s,

v20s,

v60s,

...

}

TNLA-To-Add-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Add-Item

TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Add-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Update-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Update-Item

TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage 	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Update-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Remove-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Remove-Item

TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Remove-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Setup-Item

TNLA-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

TNLA-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Failed-To-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Failed-To-Setup-Item

TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

TNLA-Failed-To-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))

TraceActivation ::= SEQUENCE {

ng-ran-TraceID			OCTET STRING (SIZE (8)),

interfaces-to-trace 	BIT STRING { ng-c (0), x-nc (1), uu (2), f1-c (3), e1 (4)} (SIZE(8)),

trace-depth 			Trace-Depth,

trace-coll-address		TransportLayerAddress,

ie-Extension 			ProtocolExtensionContainer { {TraceActivation-ExtIEs} } OPTIONAL,

...

}

TraceActivation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Trace-Depth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

dl-UE-AMBR				BitRate,

ul-UE-AMBR				BitRate,

iE-Extension			ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UEAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextKeptIndicator ::= ENUMERATED {true, ...}

UEContextID ::= CHOICE {

rRCResume				UEContextIDforRRCResume,

rRRCReestablishment		UEContextIDforRRCReestablishment,

choice-extension		ProtocolIE-Single-Container { {UEContextID-ExtIEs} }

}

UEContextID-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UEContextIDforRRCResume ::= SEQUENCE {

i-rnti					I-RNTI,

allocated-c-rnti			C-RNTI,

accessPCI				NG-RAN-CellPCI,

iE-Extension			ProtocolExtensionContainer { {UEContextIDforRRCResume-ExtIEs} } OPTIONAL,

...

}

UEContextIDforRRCResume-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextIDforRRCReestablishment ::= SEQUENCE {

c-rnti					C-RNTI,

failureCellPCI			NG-RAN-CellPCI,

iE-Extension			ProtocolExtensionContainer { {UEContextIDforRRCReestablishment-ExtIEs} } OPTIONAL,

...

}

UEContextIDforRRCReestablishment-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextInfoRetrUECtxtResp ::= SEQUENCE {

ng-c-UE-signalling-ref					AMF-UE-NGAP-ID,

signalling-TNL-at-source				CPTransportLayerInformation,

ueSecurityCapabilities					UESecurityCapabilities,

securityInformation						AS-SecurityInformation,

ue-AMBR									UEAggregateMaximumBitRate,

pduSessionResourcesToBeSetup-List		PDUSessionResourcesToBeSetup-List,

rrc-Context								OCTET STRING,

mobilityRestrictionList					MobilityRestrictionList									OPTIONAL,

indexToRatFrequencySelectionPriority	RFSP-Index												OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {UEContextInfoRetrUECtxtResp-ExtIEs} } 	OPTIONAL,

...

}

UEContextInfoRetrUECtxtResp-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEHistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCellsinUEHistoryInfo)) OF LastVisitedCell-Item

UEIdentityIndexValue ::= CHOICE {

indexLength10				BIT STRING (SIZE(10)),

choice-extension			ProtocolIE-Single-Container { {UEIdentityIndexValue-ExtIEs} }

}

UEIdentityIndexValue-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERadioCapabilityForPaging ::= SEQUENCE {

uERadioCapabilityForPagingOfNR			UERadioCapabilityForPagingOfNR			OPTIONAL,

uERadioCapabilityForPagingOfEUTRA		UERadioCapabilityForPagingOfEUTRA		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UERadioCapabilityForPaging-ExtIEs} }	OPTIONAL,

...

}

UERadioCapabilityForPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UERadioCapabilityForPagingOfNR ::= OCTET STRING

UERadioCapabilityForPagingOfEUTRA ::= OCTET STRING

UERANPagingIdentity ::= CHOICE {

i-RNTI-full			BIT STRING ( SIZE (40)),

choice-extension	ProtocolIE-Single-Container { {UERANPagingIdentity-ExtIEs} }

}

UERANPagingIdentity-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UESecurityCapabilities ::= SEQUENCE {

nr-EncyptionAlgorithms					BIT STRING {nea1-128(1),

nea2-128(2),

nea3-128(3)} (SIZE(16, ...)),

nr-IntegrityProtectionAlgorithms		BIT STRING {nia1-128(1),

nia2-128(2),

nia3-128(3)} (SIZE(16, ...)),

e-utra-EncyptionAlgorithms				BIT STRING {eea1-128(1),

eea2-128(2),

eea3-128(3)} (SIZE(16, ...)),

e-utra-IntegrityProtectionAlgorithms	BIT STRING {eia1-128(1),

eia2-128(2),

eia3-128(3)} (SIZE(16, ...)),

iE-Extension			ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} } OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ULConfiguration::= SEQUENCE {

uL-PDCP							UL-UE-Configuration,

iE-Extensions					ProtocolExtensionContainer { {ULConfiguration-ExtIEs} } OPTIONAL,

...

}

ULConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UL-UE-Configuration::= ENUMERATED {no-data, shared, only, ...}

ULForwarding	::= ENUMERATED {ul-forwarding-proposed, ...}

UPTransportLayerInformation ::= CHOICE {

gtpTunnel					GTPtunnelTransportLayerInformation,

choice-extension			ProtocolIE-Single-Container { {UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UPTransportParameters ::= SEQUENCE (SIZE(1..maxnoofSCellGroupsplus1)) OF UPTransportParametersItem

UPTransportParametersItem ::= SEQUENCE {

upTNLInfo		UPTransportLayerInformation,

cellGroupID		CellGroupID,

iE-Extension	ProtocolExtensionContainer { {UPTransportParametersItem-ExtIEs} } OPTIONAL,

...

}

UPTransportParametersItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UserPlaneTrafficActivityReport ::= ENUMERATED {inactive, re-activated, ...}

#### -- V

VolumeTimedReportList ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF VolumeTimedReport-Item

VolumeTimedReport-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { {VolumeTimedReport-Item-ExtIEs} } OPTIONAL,

...

}

VolumeTimedReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- W

#### -- X

XnBenefitValue ::= INTEGER (1..8, ...)

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

XnAP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofForbiddenTACs							INTEGER ::= 4096

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMultiConnectivityMinusOne            INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSliceItems							INTEGER ::= 1024

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																	ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= AAA

END

-- ASN1STOP

### 9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

XnAP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

XNAP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 			UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol IE pairs

--

-- **************************************************************

XNAP-PROTOCOL-IES-PAIR ::= CLASS {

&id						ProtocolIE-ID 		UNIQUE,

&firstCriticality		Criticality,

&FirstValue,

&secondCriticality		Criticality,

&SecondValue,

&presence				Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY 		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY 		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

XNAP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID 		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

XNAP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::= 	ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {XNAP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PROTOCOL-IES.&id					({IEsSetParam}),

criticality		XNAP-PROTOCOL-IES.&criticality			({IEsSetParam}{@id}),

value			XNAP-PROTOCOL-IES.&Value				({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	XNAP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			XNAP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	XNAP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			XNAP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=	SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			XNAP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		XNAP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {XNAP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {XNAP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PRIVATE-IES.&id			({IEsSetParam}),

criticality		XNAP-PRIVATE-IES.&criticality	({IEsSetParam}{@id}),

value			XNAP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

*** Remaining text not changed ***