# 3GPP TSG-RAN WG4 Meeting #92bis															R4-1916044

# Chongqing , China, 14th – 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0053 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.141-2: Annex L.5 Resource element TX power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This is formal CR for agreed draft CR R4-1912964 during RAN4#92bis meeting. This CR introduces changes that are discussed and proposed in contribution R4-1912332 to update of number of symbols that are used for OFDM symbol power from 4th symbol to all PDSCH symbols. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As discussed in [1] it is proposed to use all OFDM symbols that carry PDSCH and not contain RS or SSB to calculate OFDM power symbol. [1] R4-1912332 On Total power dynamic range conformance test for NR BS, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current design of test models in NR test specification would cause unintentional tightening of the core requirement compared to real-life operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex L.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.141-1 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Cover page corrected. |  |  |  |  |  |  |  |  |

# L.5	Resource element TX power

Perform FFT on z'ν with the FFT window timing ∆c.

The result is called Z't,f. The RE TX (RETP) power is then defined as:

RETP=Z't,f2SCS

where SCS is the subcarrier spacing in Hz.

From RETP, the OFDM symbol TX power (OSTP) is derived as follows:

OSTP=RETP

Where the summation accumulates NRBNscRB values of all OFDM symbols that carry PDSCH and not containing PDCCH, RS or SSB within a slot.

NscRB=12.

From the acquired samples Ndl values for each OSTP can be obtained and averaged where Ndl is the number of slots in a 10 ms measurement interval for FDD. For TDD, Ndl is the number of slots with downlink symbols in a 10 ms measurement interval and is computed according to the values in table 4.9.2.2-1.

For the example used in the annex, Ndl=64 and NRB=264.