3GPP TSG-RAN WG4 Meeting #93	R4-1914114

# Reno, USA, 18th – 22th Nov., 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0170 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for SA SS-SINR tests with PCell in FR1 (Section A.6.7.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Modify the test cases to reduce the Io value (below -50dBm) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the Noc and corresponding value to reduce the Io to below -50dBm. - Delete the square bracket. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Io value exceeds the upper limit that the core reuqirement can cover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the formal CR of endorsed CR R4-1910912 in RAN4#92b meeting |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

### A.6.7.3	SS-SINR

#### A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.1.1.

##### A.6.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in Table A.6.7.3.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |
| SMTC configruation |  |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/15kHz | -93 |  | -116 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -115.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -115 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -114.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -112.5 |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | -93 |  | Same as Noc for 15 kHz |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | -90 |  | -113 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -112.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -111.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -110 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -109.5 |  |
|  |  |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  |  | dB | 4.54 | 2.66 | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/SCS | -88.46 | -90.34 | -120 | -120 |
|  |  |  | NR_FDD_FR1_B |  |  |  | -119.5 | -119.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  | -119 | -119 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 | -118.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 | -118 |
|  |  |  | NR_FDD_FR1_G |  |  |  | -117 | -117 |
|  |  |  | NR_FDD_FR1_H |  |  |  | -116.5 | -116.5 |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | -85.46 | -87.34 | -117 | -117 |
|  |  |  | NR_FDD_FR1_B |  |  |  | -116.5 | -116.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  | -116 | -116 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 | -115.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 | -115 |
|  |  |  | NR_FDD_FR1_G |  |  |  | -114 | -114 |
|  |  |  | NR_FDD_FR1_H |  |  |  | -113.5 | -113.5 |
| SS-SINR Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |
| IoNote3 |  | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/ 9.36MHz | -57.5 |  | -85.51 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -85.01 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -84.51 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.01 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -83.51 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -82.51 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -82.01 |  |
|  |  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/ 38.16MHz | -51.41 |  | -79.41 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -78.91 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -78.41 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -77.91 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.41 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -76.41 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -75.91 |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

##### A.6.7.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.1.1.

#### A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.14.1.1 and 10.1.14.1.2.

##### A.6.7.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.6.7.3.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.2.2-2: SS-SINR Inter frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |  |
| Duplex mode |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  |  |  | Config 2 |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  |  | Config 3 |  |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  |  | CR.1.1 FDD | - | R.1.1 FDD | - | CR.1.1 FDD |  |  |
|  |  |  | Config 2 |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  |  | Config 3 |  |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  |  |  | Config 2 |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  |  | Config 3 |  |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| SMTC configuration |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | kHz | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A | dBm/15kHz | -88 |  | -108.5 |  | -119.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -119 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -118.5 |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -118 |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -117.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -116.5 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -116 |  |  |
| Note2 | Config 1,2 N |  |  |  | dBm/15kHz | -88 |  | -108.5 |  | Same as Noc for 15kHz T |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A |  | -85 |  | -105.5 |  | -116.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -116 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -115.5 |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -115 |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -114.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -114.5 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -113 |  |  |
|  |  |  |  |  | dB | -1.75 | -1.75 | 20 | 20 | -4.0 |  | -4.0 |
|  |  |  |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |  |
| SS-RSRP Note3 | Config 1,2 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A | dBm/SCS | -89.75 |  | -88.5 |  | -123.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -123 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -122.5 |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -122 |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -121.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -120.5 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -120 |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A |  | -86.75 |  | -85.5 |  | -120.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -120 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -119.5 |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -119 |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -118.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -117.5 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -117 |  |  |
| SS-SINRNote3 |  |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A | dB | -1.75 |  | 20 |  | -4.0 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -90.09 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -89.59 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -89.09 |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -88.59 |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -88.09 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -87.09 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -86.59 |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NR_SDL_FR1_A | dBm/ 38.16MHz | -51.73 |  | -54.41 |  | -84 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -83.5 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -83 |  |  |
|  |  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -82.5 |  |  |
|  |  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -82 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -81 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -80.5 |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.3.2.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.14.1.1 and 10.1.14.1.2.

----------------------------------------------------------- End of Change ----------------------------------------------------------------