3GPP TSG-RAN WG4 Meeting #93	R4-1915218

# Reno, NV, USA,  18th – 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR | 0009 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.810: Quiet Zone validation for RRM testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_test_methods |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RRM FR2 testing requires only EIRP/EIS and no TRP measurement. Thus the time consiming TRP related procedures for quiet zone validation are not required. This information is missing in the TR, which for validation purposes refers only to Annex D, impying also the TRP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the quiet zone validation procedure fo the RRM methodology in section 6.2.1.1, added a clarification (red font): The quality of quiet zone validation defined in Annex D only needs to be performed with the reference probe P0 and only to assess the single-directional EIRP and EIS metrics.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Time consuming TRP related procedures for quiet zone validation will be unnecessary required for RRM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.508 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# 6	UE RRM testing methodology

## 6.1	General

Testability aspects of the UE have been considered. Unless otherwise indicated below, device under test (DUT) refers to UE nodes. The exact list of RRM tests for UE can only be determined once the core requirements are settled.

For frequency bands in FR2 (e.g. mm-wave), conducted antenna connectors are assumed not to be available at DUT and the OTA testing is considered as the baseline approach for NR UE RRM testing methodology.

The possibility of performing conducted tests using an intermediate frequency (IF) were evaluated. It was decided that this approach would be challenging to standardise for various reasons since IF is an internal interface in the DUT and using a standardised IF (signal level, number of IF ports, IF frequency etc.) would preclude many different DUT implementations including direct conversion receivers. In addition, IF testing excludes all components which operate at the radio frequency such as RF filters, duplexers, transmit receive switch, low noise amplifier (LNA), power amplifier (PA), analogue beamforming phase shifting elements etc., and the algorithms which control such components from the test.

Further details of a suitable OTA test environment are to be discussed in the work item, and may have impact to the core requirements which are defined. For example, side conditions for the applicability of core requirements should be defined in a way in which they can be ensured in an OTA environment.

## 6.2	Measurement setup

### 6.2.1	Baseline setup

#### 6.2.1.1	Description

The baseline measurement setup of UE RRM characteristics for frequency bands in FR2 is capable of establishing an OTA link between the DUT and a number of emulated gNB sources as shown in Figure 6.2.1.1-1.

Figure 6.2.1.1-1: Baseline measurement setup of RRM characteristics

The absolute position of the probes is left up to implementation. The relative angular relationship between two AoAs can be achieved using any combination of probes. The relative orientation of the coordinate system is defined with respect to one probe, referred to as the reference probe P0 along which the z-axis is defined. The test conditions in Annex C.2 and the positioning guidelines in C.3 apply to the RRM baseline setup. The quality of quiet zone validation defined in Annex D only needs to be performed with the reference probe P0 and only to assess the single-directional EIRP and EIS metrics.

< End of changes >