3GPP TSG-RAN4 Meeting #93	R4-1915433

# Reno, USA, 18th – 22nd November, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5552 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.101 to revise LTE CA_NS_04 AMPR for BW class D |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CA_NS_04 total back-off has less back off than NS_04 total back-off for the reasons mentioned in R4-1913129. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise QPSK AMPR for PC3 CA_NS_04 in Table 6.2.4A.4-1A from 3.5dB to 4dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE emissions will exceed additional spurious limit over ETC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

#### 6.2.4A.4	A-MPR for CA_NS_04

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmission on two component carriers for contiguously aggregated signals is specified in Table 6.2.4A.4-1 and Table 6.2.4A.4-1A for UE power class 3 and in Table 6.2.4A.4-2 for UE power class 2.

Table 6.2.4A.4-1: Contiguous Allocation A-MPR for CA_NS_04 (power class 3), Bandwidth Class C

| CA Bandwidth Class C | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 25 RB / 100 RB | 0 – 34 and 90 – 124 | >0 | N/A | ≤3dB | ≤3.5dB |
|  | 35 – 89 | N/A | >90 | ≤1dB | ≤2.5dB |
| 50RB / 100 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤3dB | ≤4dB |
| 75 RB / 75 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤4dB | ≤4dB |
| 100 RB / 75 RB | 0 – 49 and 125 – 174 | >0 | N/A | ≤4dB | ≤4dB |
|  | 50 - 124 | N/A | >125 | ≤3dB | ≤4dB |
| 100 RB / 100 RB | 0 – 59 and 140 – 199 | >0 | N/A | ≤3dB | ≤4dB |
|  | 60– 139 | N/A | >140 | ≤3dB | ≤4dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis.. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4A.4-1A: Contiguous Allocation A-MPR for CA_NS_04 (power class 3), Bandwidth Class D

| CA Bandwidth Class D | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 50 RB / 75 RB/ 100 RB | 0 – 64 and 161 – 224 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 65 – 160 | N/A | >161 | ≤3dB | ≤3.5dB |
| 50 RB / 100 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 75 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 100 RB / 100 RB | 0 – 78 and 197 – 274 | >0 | N/A | ≤3.5dB | ≤4dB |
|  | 79 - 196 | N/A | >197 | ≤2.5dB | ≤3dB |
| 100 RB / 100 RB / 100 RB | 0 – 96 and 204 – 300 | >0 | N/A | ≤4dB | ≤4dB |
|  | 97– 203 | N/A | >204 | ≤2.5dB | ≤3dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis.. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4A.4-2: Contiguous Allocation A-MPR for CA_NS_04 (power class 2)

| CA Bandwidth Class C | Lower edge cutoff frequency [MHz]5 | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR per modulation [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 25 RB / 100 RB | 2513.5 | 0 – 42 | >0 | N/A | ≤3.5 | ≤4 | ≤4 | FFS |
|  |  | 43 – 81 | N/A | >82 | ≤1 | ≤1.5 | ≤1.5 | FFS |
|  |  | 82 – 124 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | FFS |
| 50 RB / 100 RB | 2518.4 | 0 – 52 | >0 | N/A | ≤4 | ≤4 | ≤4 | FFS |
|  |  | 53 – 94 | N/A | >95 | ≤1 | ≤1.5 | ≤1.5 | FFS |
|  |  | 95 – 149 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | FFS |
| 75 RB / 75 RB | 2519.0 | 0 – 54 | >0 | N/A | ≤4 | ≤4 | ≤4 | FFS |
|  |  | 55 – 94 | N/A | >95 | ≤2 | ≤2.5 | ≤2.5 | FFS |
|  |  | 95 – 149 | >0 | N/A | ≤1.5 | ≤2 | ≤2 | FFS |
| 75 RB / 100 RB | 2523.4 | 0 – 64 | >0 | N/A | ≤3.5 | ≤4 | ≤4 | FFS |
|  |  | 65 – 114 | N/A | >115 | ≤2 | ≤2.5 | ≤2.5 | FFS |
|  |  | 115 – 174 | >0 | N/A | ≤1 | ≤1.5 | ≤2 | FFS |
| 100 RB / 100 RB | 2528.3 | 0 – 69 | >0 | N/A | ≤3.5 | ≤4 | ≤4 | FFS |
|  |  | 70 – 129 | N/A | >130 | ≤2 | ≤2.5 | ≤2.5 | FFS |
|  |  | 130 – 199 | >0 | N/A | ≤1.5 | ≤1.5 | ≤2 | FFS |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe NOTE 5: The A-MPR values in this table shall apply when the lower edge of the aggregated channel bandwidth (Figure 5.6A-1) is less than or equal to the lower edge cutoff frequency specified in this table for the corresponding CA bandwidth combination. When the lower edge of the aggregated channel bandwidth exceeds the lower edge cutoff frequency, then the A-MPR shall be equal to the MPR specified in Table 6.2.3A-1a. |  |  |  |  |  |  |  |  |

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 3

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA 	= 	11,	 				0≤ A < 0.05

= -55.0A + 13.75,		0.05≤ A < 0.15

= -4.0A + 6.10, 			0.15≤ A < 0.40

= -0.83A + 4.83,		0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 2

A-MPR = CEIL {MA, 0.5}

If the UE is configured to CA_41D or any uplink inter-band CA configuration containing CA_41D and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 3

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA 	= 	11.5,	 				0≤ A < 0.05

= -55.0A + 14.25,		0.05≤ A < 0.15

= -4.0A + 6.60, 			0.15≤ A < 0.40

= -0.833A + 5.333,		0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

Where MA is defined as follows when the lower edge of the aggregated channel bandwidth (Table 5.6A-1) is less than or equal to the lower edge cutoff frequency specified in Table 6.2.4A.4-2 for the corresponding CA bandwidth combination

MA 	= 13.0,		 				0 ≤ A < 0.05

= 15.33 – 46.67A,		0.05 ≤ A < 0.20

= -7.0 – 5.0A, 			0.20 ≤ A < 0.50

= 4.5,						0.50 ≤ A ≤ 1

And MA is defined as follows when the lower edge of the aggregated channel bandwidth exceeds the lower edge cutoff frequency specified in Table 6.2.4A.4-2 for the corresponding CA bandwidth combination

MA 	= 8.2,		 				0 ≤ A < 0.04

= 9.8 – 40.0A,			0.04 ≤ A < 0.075

= 8.0 – 16.0A, 			0.075 ≤ A < 0.25

= 4.83 – 3.33A,			0.25 ≤ A < 0.40

= 3.83 – 0.83A,			0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

## <End of Changes>