3GPP TSG-RAN WG4 Meeting # 93	R4-1913767

# Reno, Nevada, USA, 18 – 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0896 | rev | - | Current version: | 13.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.141: Correction of references for NB-IoT testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incorrect references for NB-IoT testing have been identified: 1) In subclause 6.5.1.4.2, the whole TS 36.141 subclause 6.5.2.4.2 step 1 to 2 should be referenced instead of just step 2. 2) In subclause 6.6.4.4.2, subclause 6.6.4.5.5 should be referenced for NB-IoT stand-alone operation instead of subclause 6.6.4.5.1. 3) In subclauses 7.4.4.1, 7.5.4.1, 7.7.4.1, Annex A.14 in TS 36.141 should be referenced for NB-IoT instead of A.1. Endorsed draft CR: R4-1911211 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the identified errors of references for NB-IoT testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors of references for NB-IoT testing remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.4.2, 6.6.4.4.2, 7.4.4.1, 7.5.4.1, 7.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

##### 6.5.1.4.2	Procedure

1)	Set the BS to transmit according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.9.2.

2)	Perform the measurement on each carrier as follows:

-	For measurement on an E-UTRA carrier, see TS 36.141 [9] subclause 6.5.2.4.2 step 2 to 4.

-	For EVM measurement on a UTRA FDD carrier, see TS 25.141 [10] subclause 6.7.1.4.2 step 2 to 5 using a filter defined in subclause 6.7.1.1.

-	For PCDE measurement on a UTRA FDD carrier, see TS 25.141 [10] subclause 6.7.2.4.2 using TM3.

-	For RCDE measurement on a UTRA FDD carrier, see TS 25.141 [10] subclause 6.7.4.4.2 using TM6.

-	For EVM measurement on a UTRA TDD carrier, see TS 25.142 [12] subclause 6.8.1.4.2.2 using a filter defined in subclause 6.8.1.1.

-	For PCDE measurement on a UTRA TDD carrier, see TS 25.142 [12] subclause 6.8.2.4.1.2.

-	For RCDE measurement on a UTRA TDD carrier, see TS 25.142 [12] subclause 6.8.3.4.1.1.

-	For measurement on a GSM/EDGE carrier, see TS 51.021 [11] subclause 6.2.2.

-	For measurement on an NB-IoT carrier, see TS 36.141 [9] subclause 6.5.2.4.2 step 1 to 2.

In addition, for a multi-band capable BS, the following step shall apply:

3)	For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band. For multi-band capable BS with separate antenna connector, the antenna connector not being under test in case of SBT or MBT shall be terminated.

<Next change>

##### 6.6.4.4.2	Procedure

1)	Set the Base Station to transmit at maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.9.2.

2)	For E-UTRA with NB-IoT (in-band and/or guard band operation), measure ACLR outside the Base Station RF Bandwidth edges and ACLR inside sub-block gap or Inter RF Bandwidth gap , in addition, for non-contiguous spectrum operation as specified in subclause 6.6.4.5.1. For NB-IoT stand-alone operation, measure ACLR as specified in subclause 6.6.4.5.5.

3)	For UTRA FDD, measure ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in subclause 6.6.4.5.2.

4)	Measure Cumulative Adjacent Channel Leakage power Ratio (CACLR) inside sub-block gap or the Inter RF Bandwidth gap as specified in subclause 6.6.4.5.4.

In addition, for a multi-band capable BS, the following step shall apply:

5)	For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band. For multi-band capable BS with separate antenna connector, the antenna connector not being under test in case of SBT or MBT shall be terminated.

<Next change>

#### 7.4.4.1	Initial conditions

Test environment: 			Normal; see Annex B.2.

Base Station RF Bandwidth positions to be tested: 	MRFBW in single-band operation, see subclause 4.9.1, BRFBW_T’RFBW and B’RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

1)	Set up the equipment as shown in Annex D.2.1.

2)	Generate the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the BS under test as follows:

-	For E-UTRA see Annex A.1 in TS 36.141 [9].

-	For UTRA FDD see Annex A.2 in TS 25.141 [10].

-	For UTRA TDD see Annex A.2.1 in TS 25.142 [12].

-	For GSM see subclause 7.6.2 in TS 51.021 [11] and Annex P in TS 45.005 [6] for reference channels to test.

-	For NB-IoT see Annex A.14 in TS 36.141 [9].

<Next change>

#### 7.5.4.1	Initial conditions

Test environment: 			normal; see Annex B.2.

Base Station RF Bandwidth positions to be tested: 	MRFBW in single-band operation, see subclause 4.9.1, BRFBW_T’RFBW and B’RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

In addition, in multi-band operation:

-	For BRFBW_T’RFBW, out-of-band blocking testing above the highest operating band may be omitted

-	For B’RFBW_TRFBW, out-of-band blocking testing below the lowest operating band may be omitted

1)	Set up the equipment as shown in Annex D.2.1.

2)	Generate the wanted signal according to the applicable test configuration (see clause 5) using reference measurement channel to the BS under test as follows:

-	For E-UTRA see Annex A.1 in TS 36.141 [9].

-	For UTRA FDD see Annex A.2 in TS 25.141 [10].

-	For UTRA TDD see Annex A.2.1 in TS 25.142 [12].

-	For GSM see subclause 7.6.2 in TS 51.021 [11] and Annex P in TS 45.005 [6] for reference channels to test.

-	For NB-IoT see Annex A.14 in TS 36.141 [9].

<Next change>

#### 7.7.4.1	Initial conditions

Test environment: Normal; see Annex B.2.

Base Station RF Bandwidth positions to be tested: 	In single-band operation: MRFBW if TC6 is applicable; BRFBW and TRFBW for other TC, see subclause 4.9.1, Table 5.1-1 and Table 5.2-1. In multi- band operation: BRFBW_T’RFBW and B’RFBW_TRFBW, see subclause 4.9.1.

1)	Set-up the measurement system as shown in Annex D.2.3.

2) 	Generate the wanted signal according to the applicable test configuration (see clause 5) using reference measurement channel to the BS under test as follows:

-	For E-UTRA see Annex A.1 in TS 36.141 [9].

-	For UTRA FDD see Annex A.2 in TS 25.141 [10].

-	For UTRA TDD see Annex A.2.1 in TS 25.142 [12].

-	For GSM see subclause 7.7.2 in TS 51.021 [11] and Annex P in TS 45.005 [6] for reference channels to test.

-	For NB-IoT see Annex A.14 in TS 36.141 [9].

<End of change>