3GPP TSG-RAN4 Meeting #93	R4-1913382

# Reno, US, 18th  – 22th Novermber 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5570 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for correction on Rel-14 eFDMIMO PMI test cases with advanced codebook for TS 36.101 (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eFDMIMO-Perf |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For Rel-14 eFDMIMO, beamsteering model with dual-cluster beam directions as specified in B.2.3B.4A was introduced for PMI test cases with advanced codebook. The value of p (power ratio among these two beams) not specified in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As agreed in WF R4-1708725 slide 4, relative power ratio (p) is fixed as 1 for PMI test cases with advanced codebook. Clarify the value of relative power ratio for these PMI test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bring ambiguity to the configuration of PMI test cases with advanced codebook due to unspecified p value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.2.3.9, 9.4.2.3.10, B.2.3B.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< ---------------------------------------------------------Start of change ---------------------------------------------------------------->

9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)

For the parameters specified in Table 9.4.2.3.9-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.9-2.

Table 9.4.2.3.9-1: PMI test for dual-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | Medium 2D XP 16(2,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,30 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1} |
| eMIMO-Type |  |  | Class A |
| advancedCodebookEnabled |  |  | TRUE |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 8 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 000000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 |
|  |  |  | Pc | dB | -6 |
|  |  |  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 8 |
| Measurement channel |  |  | R.78 FDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 4: Randomization of the dual-cluster beam directions shall be used as specified in B.2.3B.4A. The value of relative power ratio (p) shall be fixed as 1 during the test. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.9-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | [1.03] |
| UE Category | ≥2 |

9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)

For the parameters specified in Table 9.4.2.3.10-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.10-2.

Table 9.4.2.3.10-1: PMI test for dual-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | Medium 2D XP 16(2,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,26 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1} |
| eMIMO-Type |  |  | Class A |
| advancedCodebookEnabled |  |  | TRUE |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 8 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 000000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Measurement channel |  |  | R.78 TDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the dual-cluster beam directions shall be used as specified in B.2.3B.4A. The value of relative power ratio (p) shall be fixed as 1 during the test. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.10-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | [1.03] |
| UE Category | ≥2 |

< ---------------------------------------------------------Start of next change ----------------------------------------------------------- >

B.2.3B.4A	Beam steering approach with dual cluster beams

Given the channel spatial correlation matrix in B.2.3B.1, the corresponding random channel matrix H can be calculated. The signal model for the k-th subframe is denoted as

And the steering matrix is further expressed as following:

where

-	,are independent channels for  the first beam and second beam with the Nr xNt channel matrix per subcarrier.

-	, are the steering matrix for first beam and second beam

-	 is the steering matrix in first dimension with same polarization,

-	 is the steering matrix in second dimension with same polarization,

-	 is the number of antenna elements infirst dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization,

-	 is the relative power ratio of the second beam to the first beam, the value of is specific to a test case,

For 1 antenna element of the same polarization in one direction, .

For 2 antenna elements of the same polarization in one direction, .

For 3 antenna elements of the same polarization in one direction,.

For 4 antenna elements of the same polarization in one direction, .

where the index  stands for first dimension and second dimension respectively.

-	 controls the phase variation in first dimension and second dimension respectively, and the phase for k-th subframe is denoted by, where is the random start value with the uniform distribution, i.e., ,  is the step of phase variation, which is defined in Table B.2.3B.4-1, and k is the linear increment of 1 for every subframe throughout the simulation, the index  stands for first dimension and second dimension respectively.

-	 is the precoding matrix for Nt transmission antennas,

-	 is the received signal,  is the transmitted signal, and is AWGN.

Table B.2.3B.4-1: The step of phase variation

| Variation Step | Value (rad/subframe) |
| --- | --- |
|  | 1.2566×10-3 |
|  | 2.5132×10-3 |

< ---------------------------------------------------------End of change -----------------------------------------------------------